| Project | IEEE 802.21 MIHS <http://www.ieee802.org/21/> |  |
| --- | --- | --- |
| Title | Suggestion about MIH configure: LB Issue- #49: Comment 622 |  |
| DCN | 21-06-0xxx-00-0000 |  |
| Date Submitted | September 7, 2006 |  |
| Source(s) | Guo Junxiang |  |
| Re: | IEEE 802.21 Session #16 in Melbourne, Australia |  |
| Abstract | This contribution is a response to LB Issue #49 in comment assignment and provides amendments about the MIH Configure command and suggests add Link Configure command. |  |
| Purpose |  |  |
| Notice | This document has been prepared to assist the IEEE 802.21 Working Group. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s). The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that IEEE 802.2 may make this contribution public. |  |
| Patent Policy | The contributor is familiar with IEEE patent policy, as outlined in Section 6.3 of the IEEE-SA Standards Board Operations Manual <http://standards.ieee.org/guides/opman/sect6.html#6.3> and in Understanding Patent Issues During IEEE Standards Development <http://standards.ieee.org/board/pat/guide.html>. |  |

According to the suggestion of comment 622 and the resolution results of related comments 623 and 624, we suggest that make the following amendments to the MIH Configure commands and add the Link Configure commands into current draft.

# 1.Amendments to MIH_Configure.request

We suggest to adopt the following changed text to section 7.4.8.1:

    - MIH Configure

MIH Configure may be issued by upper layer entities to control the behavior of lower layer, for example, to set some feature of the driver of a specific link. When MIH receives MIH Configure, it issues the Link Configure command to corresponding link. The configuration parameter are passed on to Link Configure command.

      - MIH_Configure.request

        - Function

This primitive is issued by uppers to control the behavior of a lower layer link.

        - Semantics of the service primitive

The parameters of the primitive are as follows:

MIH_Configure.request 			(

SourceIdentifier,

DestinationIdentifier,

LinkIdentifier,

ConfigurationRequestsSets

)

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| Source Identi fier | Identifier | Any valid individual or group identifier | The identifier of entity where the request is initiated. This field may be optionally left empty if the com mand is local. |
| Destination Identifier | Identifier | MIH_LOCAL, MIH_REMOTE | The destination identifier of request or response. This is the identifier of local or peer MIH Function. |
| LinkIdentifier | Identifier | N/A | Identifier of the link to configure. |
| Configuration Request Sets | Set of configuration parameters for corre sponding interfaces | N/A | Containing a set of configuration parameters. |

        - When generated

This primitive is generated when L3 or upper layers attempt to control the behaviors of lower layer link, for example, set some features in the drivers.

        - Effect of receipt

A local configure command causes the MIH to issue a Link Configure command to set the behaviors of lower layer link according to the specified configuration parameters. A remote configure command causes the MIH in the peer entity to issue a Link Configure command.

Table 18- Link configuration parameters

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| NETWORK_TYPES_IN_USE | INTEGER | 0x01 802.3 0x02 802.11 0x03 802.16 0x04 3GPP 0x05 3GPP2 | Set the network type that should be used for the lower layer entity, e.g., a driver. |
| OPERATION_MODE | INTEGER | 0x00 Normal Mode 0x01 Power Saving Mode 0x02 Power Down | Change the device’s power mode |
| DISABLE_TRANSMITTER | Boolean | N/A | Enable/disable the transmitter of the inter face. |
| NETWORK_ID | STRING | N/A | Change to the specified network of an inter face. |
| CURRENT_ADDRESS | STRING | N/A | Change the current address to the value specified |
| SUSPEND_DRIVER | Boolean | N/A | Suspend or resume of the specified interface. |

# 2.Amendments to MIH_Configure.confirm

As the LinkIdentifier  had been added to MIH_Configure.request, the corresponding response command should also add this parameter, so suggest the following changes to MIH_Configure.confirm:

      - MIH_Configure.confirm

        - Function

This primitive is issued by the MIH entity to the upper layer to report the result of the MIH Configure command.

        - Semantics of the service primitive

The parameters of the primitive are as follows:

MIH_Configure.confirm (

SourceIdentifier,

DestinationIdentifier,

LinkIdentifier,

ConfigurationResponseSets

)

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| Source Identifier | Identifier | Any valid individual or group identifier | The identifier of entity where the request is initiated. This field may be optionally left empty if the command is local. |
| Destination Identifier | Identifier | MIH_LOCAL, MIH_REMOTE | The destination identifier of request or response. This is the identifier of local or peer MIH Function. |
| LinkIdentifier | Identifier | N/A | Identifier of the link to configure. |
| Configuration-ResponseSets | Set of configuration parameters and their configuration results | N/A | Containing a set of configuration parameters and their corresponding configuration results: ConfigurationParameter ResultCode |

        - When generated

This primitive is generated when MIH receive the results of the Link Command.

        - Effect of receipt

Upon receipt of the result code, the upper layers make evaluations and take actions.

# 3. Add new commands - Link Configure

The MIH Configure Command should have corresponding link layer command as above text show, so we suggest to add the following text as chapter 7.3.22 and 7.3.23 into draft P802-21-D01-08.pdf.

    - Link_Configure.request

      - Function

This primitive is issued by MIH function to control the behavior of a lower layer link.

      - Semantics of the service primitive

The parameters of the primitive are as follows:

Link_Configure.request 			(

ConfigurationRequestsSets

)

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| Configuration Request Sets | Set of configuration parameters for corresponding interfaces | N/A | Containing a set of configuration parameters. See table 18 below. |

7.3.22.3   When generated

This primitive is generated by MIHF when it receives the MIH_Configure.request command from the upper layer and attempt to control the behaviors of lower layer link, for example, set some features in the drivers.

7.3.22.4  Effect of receipt

The recipient responds with Link_Configure.confirm primitive.

Table 18- Link configuration parameters

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| NETWORK_TYPES_IN_USE | INTEGER | 0x01 802.3 0x02 802.11 0x03 802.16 0x04 3GPP 0x05 3GPP2 | Set the network type that should be used for the lower layer entity, e.g., a driver. |
| OPERATION_MODE | INTEGER | 0x00 Normal Mode 0x01 Power Saving Mode 0x02 Power Down | Change the device’s power mode |
| DISABLE_TRANSMITTER | Boolean | N/A | Enable/disable the transmitter of the inter face. |
| NETWORK_ID | STRING | N/A | Change to the specified network of an inter face. |
| CURRENT_ADDRESS | STRING | N/A | Change the current address to the value specified |
| SUSPEND_DRIVER | Boolean | N/A | Suspend or resume of the specified interface. |

    - Link_Configure.confirm

      - Function

This primitive is sent in response to the Link_Configure.request primitive . This primitive is used to report the result of the Link Configure command

      - Semantics of the service primitive

The parameters of the primitive are as follows:

Link_Configure.confirm (

ConfigurationResponseSets

)

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| Configuration-ResponseSets | Set of configuration parameters and their configuration results | N/A | Containing a set of configuration parameters and their corresponding configuration results: ConfigurationParameter ResultCode |

      - When generated

This primitive is sent in response to the Link_Configure.request command operation.

      - Effect of receipt

The recipient will report the Link Configure results to its upper layer.