| Project | IEEE 802.21 Media Independent Handover Services <http://www.ieee802.org/21/> |  |
| --- | --- | --- |
| Title | IP Configuration Methods (addressing LB comments , 386, 480, 496) |  |
| Date Submitted | July, 2006 |  |
| Source(s) | Ronny Kim (LG Electronics), Soohong Daniel Park (Samsung), Qiaobing Xie (Motorola) |  |
| Re: |  |  |
| Abstract | IP configuration methods |  |
| Purpose |  |  |
| Notice | This document has been prepared to assist the IEEE 802.21 Working Group. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s). The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.21. |  |
| Patent Policy | The contributor is familiar with IEEE patent policy, as outlined in Section 6.3 of the IEEE-SA Standards Board Operations Manual <http://standards.ieee.org/guides/opman/sect6.html#6.3> and in Understanding Patent Issues During IEEE Standards Development <http://standards.ieee.org/board/pat/guide.html>. |  |

Proposed resolution to LB comments #386, #480, and #496:

[Modify the text as suggested below: Page 103, line 25]

### MIH_Network_Address_Information

#### 7.4.14.1 MIH_Network_Address_Information.request

7.4.14.1.1 Function

This primitive is used to request an IP address related information from a potential handover target PoS before a handover. The request can be delivered from the MIH Function on the MS to the MIH Function on the old PoS and frowarded by the the MIH Function on the old PoS to the potential handover target MIH Function on a new PoS.. This primitive can be delivered from MIH Function of MS to MIH Function on new PoS when R2 communication in the MIH communication model is possible. Upon reception of this message, the MIH Function on the new PoS may either interact with upper layer to achieve enough information or further deliver this message to the MIH Function in the network, e.g Access Router with MIH Function which may have enough information on this message. This may also trigger the network entity, e.g, Access Router or Foreign Agent to send out Mobility related messages directly to the mobile node.

##### 7.4.14.1.2 Semantics of service primitive

The parameters of the primitive are as follows:

MIH_Network_Address_Information.request 	(

SourceIdentifier,

DestinationIdentifier,

CurrentLinkIdentifier,

MacMobileNode,

TargetPoAIdentifier

Current IP Configuration Method

Current DHCP Server Address

CurrentAccessRouterAddress,

CurrentFAAddress

)

Local or Remote: Both

MIHF(Terminal) <> MIHF (Network) <> MIHF (Network)

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| Source Identifier | Identifier | Any valid individ ual or group iden tifier | The identifier of entity where the request is initiated. This field may be optionally left empty if the command is local. |
| Destination Identifier | Identifier | MIH_LOCAL, MIH_REMOTE | The destination identifier of request or response. This is the identifier of local or peer MIH Function. |
| CurrentLinkIdentifier | NetworkIdenti fier. May be one of different 802 and Cellular networks. | Identifier | This identifies the current access net work over which the command needs to be sent. This is valid only for remote commands which need to be sent to remote MIHF. The command is then sent either at L2 or at L3. |
| MacMobileNode | MAC Address | N/A | MAC Address of Mobile Node |
| TargetPoAIdentifier | MAC Address |  | This is the candidate target Point of Attachment (AP/BS) on new network. |
| Current IP Configuration Method | Bitmap | 0~31 | Bit 0: IPv4 static configuration Bit 1: IPv4 dynamic configuration (DHCPv4) Bit 2: Mobile IPv4 with FA (FA-CoA) Bit 3: Mobile IPv4 without FA (Co-located CoA) Bits 4 -10: reserved for IPv4 address configurations Bit 11: IPv6 stateless address configuration Bit 12: IPv6 stateful address configuration (DHCPv6) Bit 13: IPv6 manual configuration Bits 14-31: Reserved. |
| Current DHCP Server Address | IP Address | N/A | IP address of currnet DHCP Server. This parameter is only included when mobile node is using dynamic address configuration. |
| Current FA Address | IP Address | N/A | IP address of Current Foreign Agent. This parameter is only included when mobile node is using Mobile IPv4, |
| Current Access Router Address | IP Address | N/A | IP address of Current Access Router This parameter is only included when mobile node is using IPv6 |

7.4.14.1.3 When generated

When the mobile terminal is reparing for handover, the mobile terminal initiates Network Address information acquisition procedure by generating this primitive.

7.4.14.1.3 Effect on receipt

Upon reception of this primitive, the MIHF on the new PoS may either interact with upper layer to collect the requested information or further forward this reqeust to the another MIHF in the network, e.g Access Router with MIHF which may have enough information on this message. When this primitive is delivered to the upper layer of the MIHF in the network, e.g, Access Router with MIHF, this may trigger to send network address related information, e.g, Router Advertisement directly to the mobile node.

#### 7.4.14.2 MIH_Network_Address_Information.response

7.4.14.2.2 Function

This primitive is used by an MIH Function to respond an MIH_Network_Address_Infomration.request from a peer MIH Function.

7.4.14.2.3 Semantics of service primitive

The parameters of the primitive are as follows:

MIH_Network_Address_Information.response (

SourceIdentifier,

DestinationIdentifier,

CurrentLinkIdentifier,

MacMobileNode,

NewPoAIdentifier

IP Configuration Method

DHCP Server Address,

FAAddress,

AccessRouterAddress,

Result Code

)

Local or Remote: Both

MIHF (Network) <> MIHF (Network) <> MIHF(Terminal)

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| Source Identifier | Identifier | Any valid individ ual or group iden tifier | The identifier of entity where the request is initiated. This field may be optionally left empty if the command is local. |
| Destination Identifier | Identifier | MIH_LOCAL, MIH_REMOTE | The destination identifier of request or response. This is the identifier of local or peer MIH Function. |
| CurrentLinkIdentifier | NetworkIdenti fier. May be one of different 802 and Cellular networks. | Identifier | This identifies the current access net work over which the command needs to be sent. This is valid only for remote commands which need to be sent to remote MIHF. The command is then sent either at L2 or at L3. |
| MacMobileNode | MAC Address | N/A | MAC Address of Mobile Node |
| NewPoAIdentifer | MAC Address | N/A | This is the target Point of Attachment (AP/BS) on new network. |
| IP Configuration Method | Bitmap | 0~31 | Bit 0: IPv4 static configuration Bit 1: IPv4 dynamic configuration (DHCPv4) Bit 2: Mobile IPv4 with FA (FA-CoA) Bit 3: Mobile IPv4 without FA (Co-located CoA) Bits 4 -10: reserved for IPv4 address configurations Bit 11: IPv6 stateless address configuration Bit 12: IPv6 stateful address configuration (DHCPv6) Bit 13: IPv6 manual configuration Bits 14-31: Reserved. |
|  | IP Address |  |  |
| DHCP Server Address | IP Address | N/A | IP address of DHCP Server. This parameter is only included when mobile node is using dynamic address configuration |
| FA Address | IP Address | N/A | IP address of Foreign Agent. This parameter is only included when mobile node is using Mobile IPv4, |
| Access Router Address | IP Address | N/A | IP address of Access Router This parameter is only included when mobile node is using IPv6 |
| Result Code | Bitmap | 0~8 | Bit 0: IP configuration Method is not available Bit 1: DHCP Server address is not available Bit 2: FA Address is not available Bit 3: Access Router Address is not available Bit 4: No information is provided due to accessibility of same entity (FA, Access Router, DHCP Server, etc) Bit 5~7: Reserved |

[Modify the text as suggested below: Page 41, line 49, Table 4]

| 10 | MIH Network Address Information | L, R | Client -> Network, Net work-> Client | Sent from MIHF entity of Moble Node to either current MIHF entity or tar get MIHF entity to obtain IP configuration related information. |
| --- | --- | --- | --- | --- |

[Modify the text as suggested below: Page 121, line 38, Table 21]

| Type | Parameter Name |
| --- | --- |
| 1 | IP Renewal Flag |
| 2 | IP Address Configuration Methods List |
| 3 | Link Down Reason Code |

[Modify the text as suggested below: Page 122, line 37]

#### 8.4.2.2 IP Address Configuration Methods List

This parameter specifies the list of IP Address Configuration Methods that may be supported by a PoA.

| Type | Length | Value |
| --- | --- | --- |
| 2 | 1 | Indicates type of IP Address Configuration Methods supported by the new PoA 0: IPv4 static configuration 1: IPv4 dynamic configuration (DHCPv4) 2. Mobile IPv4 with FA (FA-CoA) 3: Mobile IPv4 without FA (Co-located CoA) 4~10: reserved for IPv4 address configuration 11: IPv6 stateless address configuration 12: IPv6 stateful address configuration (DHCPv6) 13: IPv6 manual configuration 14~255: Reserved |

[Modify the text as suggested below: Page 130, line 65]

| Name | Type |
| --- | --- |
| MacMobileNode | Mobile Node MAC Address (251) |
| MacNewPoA | PoA MAC Address (250) |
| MacOldAccessRouter | Access Router MAC Address (249) |
| MacNewAccessRouter | Access Router MAC Address (249) |
| IP_Renewal_Indicator | IP Renewal Flag (1) |
| IPConfigurationMethods | IP Address Configuration Methods List (2) |

[Add new TLV: Page 122, line 23]

| 25 | Link Resource Parameters Values |
| --- | --- |
| 26 | IP Address Information Status |

[Add new subsection 8.4.2.26 for added new TLV: Page 129, line 10]

#### 8.4.2.26 IP Address Information Status

This parameter specifies the status of IP address information request.

| Type | Length | Value |
| --- | --- | --- |
| 26 | 1 | 0: IP configuration Method is not available 1: DHCP Server address is not available 2: FA Address is not available 3: Access Router Address is not available 4: No information is provided due to accessibility of same entity (FA, Access Router, DHCP Server, etc) 5~255: Reserved |

[Modify the text as suggested below: Page 137, line 16]

#### MIH_Network_Address_Information Request

The ‘MIH_Network_Address_Information.request’ is delivered from one MIHF to another MIHF in order to discover mobile node’s network address related information before handover. Upon reception of this message, the receiving remote MIHF may either interact with upper layer to achieve enough information or further deliver this message to the MIHF which may have enough information on this message.

Serving FA Address / Serving Access Router Address/ DHCP Server Address parameter is chosen depending on what kind of IP address configuration methods is set. E.g., if IP Address configuration method is IPv4 dynamic configuration (DHCPv4), then IP address included in “Serving FA Address / Serving Access Router Address/ DHCP Server Address” is DHCP Server address (IPv4).

| Name | Type |  |
| --- | --- | --- |
| MacMobileNode | Mobile Node MAC Address (251) |  |
| IPConfigurationMethods | IP Address Configuration Methods List (2) |  |
| Serving FA Address / Serving Access Router Address / DHCP Server Address | IP Address (247) |  |

#### MIH_Network_Address_Information Response

This is sent as a response to MIH_Network_Address_Information.request.

Serving FA Address / Serving Access Router Address/ DHCP Server Address parameter is chosen depending on what kind of IP address configuration methods is set. E.g., if IP Address configuration method is IPv4 dynamic configuration (DHCPv4), then IP address included in “Serving FA Address / Serving Access Router Address/ DHCP Server Address” is DHCP Server address (IPv4). When more than two IP address configuration methods are supported, “Serving FA Address / Serving Access Router Address/ DHCP Server Address” parameter is repeated up to number of supporting IP configuration methods.

| Name | Type |  |
| --- | --- | --- |
| MacMobileNode | Mobile Node MAC Address (251) |  |
| IPConfigurationMethods | IP Address Configuration Methods List (2) |  |
| FA Address / Access Router Address/ DHCP Server Address | IP Address (247) |  |
| IP Address Information Status | IP Address Information Status Code (26) |  |