| Project | IEEE 802.21 MIHO <http://www.ieee802.org/21/> |  |
| --- | --- | --- |
| Title | Handover Procedures Example |  |
| Date Submitted | September 20, 2006 |  |
| Source(s) | Ulises Olvera |  |
| Re: | IEEE 802.21 Session #16 in September 2006 |  |
| Abstract | This document proposes text for LB1 comment # 872, 874, 875 (Issue 61). |  |
| Purpose | LB#1 comment resolution |  |
| Notice | This document has been prepared to assist the IEEE 802.21 Working Group. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s). The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.21. |  |
| Patent Policy | The contributor is familiar with IEEE patent policy, as outlined in Section 6.3 of the IEEE-SA Standards Board Operations Manual <http://standards.ieee.org/guides/opman/sect6.html#6.3> and in Understanding Patent Issues During IEEE Standards Development <http://standards.ieee.org/board/pat/guide.html>. |  |

- Proposal for D0109

The following steps describe the flow depicted in Figure  .

- The UE is associated to an 802.11 AP and there is currently a session established through the corresponding Home Agent

- An IEEE 802.21 Link Parameter change event is triggered due to e.g., undesirable radio conditions. The MIH function notifies the MIH User through a corresponding MIH Link Parameter Change message. The MIH might also notify the local MIH User. This is illustrated by the dotted arrow.

- The MIHF notifies the MIH Peer at the network (which previously registered for the event service) indicating that a Link Parameter Change has been triggered. The MIH User may optionally contact the MIH Information Server through an MIH Capability Discovery Request to query about the available network for the user that generated the event. Note that this information could have been previously obtained or it could be readily available within the network entity where the MIH User resides.

- The MIH User might use a Handover Prepare message in order to request resource from alternate networks.

- The MIH User selects a candidate based on the information received in through the relevant MIH event indication. How this is done is out of scope of the IEEE 802.21 standard.

- The MIH User generates a Handover Initiate Request towards its local MIHF. In turn, the local MIHF contacts its peer at the STA/UE to transfer the MIH Handover Initiate message. E.g., over a layer 3 transport using the MIH Protocol. The remote MIHF at the UE relays the MIH Handover Initiate Request, using an MIH Handover.Indication primitive.

- The MIH User issues an MIH Switch.Request towards its local MIHF to initiate the establishment of a new radio connection according to the information received in the MIH Handover Initiate Message.

- The MIHF issues an Link Switch request command ordering the establishment of a link layer connection.

- The 3GPP2 network interface establishes a new radio connection

- and a new PPP session.

- The cellular interface  confirms the establishment of the link layer connection.

- An MIH Handover Initiate.Response message is sent to the requesting MIH User indicating the successful establishment of the new radio connection. Note that the MIH User might have direct access to the relevant cellular technology. This is however out of the scope of this standard. Note that the establishment of the PPP session might trigger a Link Up event at the network side.

- Mobile IP registration might be triggered now.

- An MIH Handover Commit message is sent to MIH User indicating the successful establishment of the new radio connection.

- The MIH User may tear down the connection using an MIH Handover Switch.Request command.

- A Link_Switch. request is used to release resources.

- A Link_Switch.confirmresponse is issued indicating success release of resources.

- An MIH Switch.Confirm is issued notifying the user of successful release of resources.

- An MIH Commit Response may be issued towards the peer.

- The session is now transferred towards the new link layer and a MIH Handover_Complete.Indication is used to alert the source PoS that resource might be released

- The session is now flowing throught the new link layer technlogy (i.e. 3GPP2)

- The UE is disassociated from the previous PoA