| Project | IEEE 802.21 MIHO <http://www.ieee802.org/21/> |  |
| --- | --- | --- |
| Title | Network Standards IE |  |
| Date Submitted | November 15, 2006 |  |
| Source(s) | Yoshihiro Ohba, Subir Das, Srinivas Sreemanthula and Ronny Kim |  |
| Re: | IEEE 802.21 Session #17 in November 2006 |  |
| Abstract | This document proposes text for Network Standards IE |  |
| Purpose | LB#1a comment resolution |  |
| Notice | This document has been prepared to assist the IEEE 802.21 Working Group. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s). The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.21. |  |
| Patent Policy | The contributor is familiar with IEEE patent policy, as outlined in Section 6.3 of the IEEE-SA Standards Board Operations Manual <http://standards.ieee.org/guides/opman/sect6.html#6.3> and in Understanding Patent Issues During IEEE Standards Development <http://standards.ieee.org/board/pat/guide.html>. |  |

- Proposal for D02-00

Change Section 6.3.5.2.4:

| Type | Description | Length | Value |
| --- | --- | --- | --- |
| TYPE_IE_NETWORK_STA NDARDS | Access Network specific applicable revisions to the base access network standard | 8 octets | This is the list of revisions applicable for the access network standard. For 802 networks this is the list of specific revisions applicable for that access network. For cellular networks this represents the release version of the standrad that is appliacble. This field contains a bitmap in which each bit corresponds to a standard of the specific Network Type. The bitmap is defined as follows. For Network Type = Wireless – IEEE 802.11: Bit 0: 802.11a Bit 1: 802.11b Bit 2: 802.11c Bit 3: 802.11d Bit 4: 802.11e Bit 5: 802.11f Bit 6: 802.11g Bit 7: 802.11h Bit 8: 802.11i Bit 9: 802.11j Bit 10: 802.11k Bit 11: Reserved Bit 12: Reserved Bit 13: 802.11n Bit 14: Reserved Bit 15: 802.11p Bit 16: Reserved Bit 17: 802.11r Bit 18: 802.11s Bit 19: 802.11T Bit 20: 802.11u Bit 21: 802.11v Bit 22: 802.11w Bit 23: Reserved Bit 24: 802.11y Bit 25-63: Reserved For Network Type = Wireless – IEEE 802.16: Bit 0: 802.16-2001 Bit 1: 802.16c-2002 Bit 2: 802.16a-2003 Bit 3: 802.16-2004 Bit 4: 802.16e-2005 Bit 5: 802.16g-2007 Bit 6-63: Reserved For Newtork Type = CDMA2000: Bit 0: CDMA-2000-Rev-0 Bit 1: Rev-A Bit 2: Rev-B Bit 3: Rev-C Bit 4: Rev-D (?) Bit 5-63: Reserved For Network Type = UMTS Bit 0: GSM Bit 1: GPRS Bit 2: EDGE Bit 3: Rel-99 Bit 4: Rel-4 Bit 5: Rel-5 (w/ HSDPA) Bit 6: Rel-6 (w/ HSUPA) Bit 7: Rel-7 (MIMO/OFDM) Bit 8: Rel-8 Bit 9-63: Reserved |

[2] Add one optional Network Standards IE to TYPE_CONTAINER_POA TLV.  (This is because different PoAs in the same network type may support different set of standards.)