| Project | IEEE 802.21 MIHS <http://www.ieee802.org/21/> |  |
| --- | --- | --- |
| Title | Additional primitives for MIH_NET_SAP |  |
| DCN | 21-06-0785-00-0000 |  |
| Date Submitted | November 2006 |  |
| Source(s) | Kentaro Ishizu, Yoshia Saito, Masahiro Kuroda, Ryoichi Komiya | NICT |
| Re: | IEEE 802.21 session #17 in November 2006 |  |
| Abstract | This document defines additional primitives of MIH_TP_DATA.{request, confirm} for MIH_NET_SAP to achieve acknowledged connection-oriented data transfer. |  |
| Purpose | Discuss and adopt in the draft |  |
| Notice | This document has been prepared to assist the IEEE 802.21 Working Group. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s). The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that IEEE 802.2 may make this contribution public. |  |
| Patent Policy | The contributor is familiar with IEEE patent policy, as outlined in Section 6.3 of the IEEE-SA Standards Board Operations Manual <http://standards.ieee.org/guides/opman/sect6.html#6.3> and in Understanding Patent Issues During IEEE Standards Development <http://standards.ieee.org/board/pat/guide.html>. |  |

This document describes proposals to the IEEE 802.21 standard draft D02.00.

We propose to define additional primitives of MIH_TP_Data.{response,confirm} for MIH_NET_SAP. Without these primitives, MIHF can not accomplish the acknowledged connection-oriented data transfer which enables retransmission, state synchronization, sequential execution and so on. Also, we found mistakes in the text of the existing primitives for MIH_NET_SAP.

We propose to replace 7.6.2.3.4, 7.6.2.4.3 and 7.6.2.4.4 with the following text specified in blue.

- Effect on Receipt

The receipt of this primitive causes the selected transport service provider to issue an MIH_TP_Connect.confirm to a local MIHF.

- When Generated

The transport service provider responds with this service primitive in response to a MIH_TP_Connect.request from a local MIHF. It is passed from the transport service provider to the MIHF to confirm a data transport over an acknowledged connection-oriented transport session.

- Effect on Receipt

The receiving MIHF instance may stop its retry timer for connecting to a remote MIHF.

Next, we propose to add new following text specified in blue.

      - MIH_TP_Data.response

        - Function

This primitive is used to notify the transport service provider of the receipt of an MIH PDU.

        - Semantics

The primitive shall provide parameters as follows:

MIH_TP_Data.response 				(

TransportType,

SourceAddress,

DestinationAddress

)

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| TransportType | Enumerate | N/A | Identifies the protocol layer specific transport option |
| SourceAddress | Address | N/A | Protocol layer specific Transport Address of entity which has the Source MIHF |
| DestinationAddress | Address | N/A | Protocol layer specific Transport Address of entity which has the Destination MIHF |

        - When Generated

The MIHF responds with this service primitive when a request to transfer a MIH PDU has been received from a remote MIHF.

        - Effect on Receipt

The receipt of this primitive causes the selected transport service provider to issue an MIH_TP_Data.confirm to a local MIHF.

      - MIH_TP_Data.confirm

        - Function

This primitive is used to confirm an acknowledged connection-oriented mode transfer.

        - Semantics

The primitive shall provide parameters as follows:

MIH_TP_Data.confirm 				(

TransportType,

SourceAddress,

DestinationAddress

)

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| TransportType | Enumerate | N/A | Identifies the protocol layer specific transport option |
| SourceAddress | Address | N/A | Protocol layer specific Transport Address of entity which has the Source MIHF |
| DestinationAddress | Address | N/A | Protocol layer specific Transport Address of entity which has the Destination MIHF |

        - When Generated

This primitive is passed from the transport service provider to MIHF to confirm that a request to transfer an MIH PDU succeeded.

        - Effect on Receipt

Upon receipt of this primitive, the receiving MIHF may stop its retransmission timer for the corresponding request. When the MIHF does not receive this primitive for a pre-defined time after transmitting a MIH_TP_Data.request, the MIHF may try to retransmit the MIH_TP_Data.request..