| Project | IEEE 802.21 MIHS <http://www.ieee802.org/21/> |  |
| --- | --- | --- |
| Title | Clarification of MIH_NMS_SAP |  |
| DCN | 21-06-0786-00-0000 |  |
| Date Submitted | November 2006 |  |
| Source(s) | Masahiro Kuroda, Yoshia Saito, Kentaro Ishizu, Ryoichi Komiya | NICT |
| Re: | IEEE 802.21 session #18 in Dallas |  |
| Abstract | This document adds the definition of MIH_NMS_SAP which is missing. |  |
| Purpose | Discuss and adopt in the draft |  |
| Notice | This document has been prepared to assist the IEEE 802.21 Working Group. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s). The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that IEEE 802.2 may make this contribution public. |  |
| Patent Policy | The contributor is familiar with IEEE patent policy, as outlined in Section 6.3 of the IEEE-SA Standards Board Operations Manual <http://standards.ieee.org/guides/opman/sect6.html#6.3> and in Understanding Patent Issues During IEEE Standards Development <http://standards.ieee.org/board/pat/guide.html>. |  |

This document describes proposals to the IEEE 802.21 standard draft D02.00.

We clarify the definition of “Network Management System” and “MIH_NMS_SAP” in this document.

We propose to add the following definition of Network Management System to Section 3.

Network Management System – A combination of hardware and software used to monitor, control, and administer a data communication network. Individual network entities (NEs) in a network are managed by an entity management system.

The "Network Management Entity" in Figure 12 should be replaced with "Network Management System".

We also propose to replace 5.6.2.2, 7.1.2 and 7.2.3.1 with the following text specified in blue and to add the description of MIH_NMS_SAP primitives to 7.7.

- MIH_NMS_SAP

The MIH_NMS_SAP provides the interface between the MIHF and the Network Management System (NMS).  The SAP defines the interface to get state information from MIHF and initialize MIHF to recover from failure so that NMS can manage MIHF.

- Media Independent SAPs

- MIH_SAP: This SAP defines the interface between MIHF and MIH Users.

- MIH_NET_SAP: This SAP defines the transport services that can be used by MIHF.

- MIH_NMS_SAP: This SAP defines the interface between MIHF and Network Management System in the access networks such as 802 and 3GPP/3GPP2 networks.

7.2.3.1 MIH_NMS_SAP

The key primitives defined as part of MIH_NMS_SAP are described in Error: Reference source not found.

| - MIH_NMS_SAP Primitives |  |  |  |
| --- | --- | --- | --- |
| Primitives | Service Category | Description |  |
| Initialize | Network Management | This primitive brings in MIHF to initial state in the state transition. |  |
| Get_State | Network Management | Get state information of MIHF indicating working status. |  |

7.7 MIH_NMS_SAP primitives

MIH_NMS_SAP provides minimum sets of primitives for NMS to monitor and configure MIHF. Primitives are “Initialize” and “Get_state”.  The NMS relevant information and control flow are shown in Figure xx.

Figure xx - NMS relevant information and control flow

7.7.1 MIH_NMS_Initialize

The primitives associated with the initialization of MIHF are as follows:

- MIH_NMS_Initialize.request

- MIH_NMS_Initialize.confirm

NMS sends the MIH_NMS_Initialize.request to set MIHF to initial working state. MIHF returns the MIH_NMS_Initialize.confirm to NMS.

7.7.1.1 MIH_NMS_Initialize.request

7.7.1.1.1 Function

This primitive is used by NMS to initialize MIHF.

7.7.1.1.2 Semantics

The parameters of the primitive are as follows:

MIH_NMS_Initialize.request     (

)

7.7.1.1.3 When Generated

This primitive is generated by NMS when it needs to initialize MIHF.

7.7.1.1.4 Effect on Receipt

MIHF executes initialization.

7.7.1.2 MIH_NMS_Initialize.confirm

7.7.1.2.1 Function

This primitive returns the result of MIHF initialization.

7.7.1.2.2 Semantics

The parameters of the primitive are as follows:

MIH_NMS_Initialize.confirm   (

ResultCode

)

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| ResultCode | Enumerate | Success; Failure; Rejected | Result of MIHF initialization. |

7.7.1.2.3 When Generated

This primitive is generated by MIHF when MIHF has processed MIH_NMS_Initialize.request from NMS.

7.7.1.2.4 Effect on Receipt

NMS may confirm whether MIHF works or not.

7.7.2.1 MIH_NMS_Get_State

The primitives associated with retrieval of MIHF state information are as follows:

- MIH_NMS_Get_State.request

- MIH_NMS_Get_State.confirm

NMS uses the MIH_NMS_Get_State.request primitive to get the state information collected by MIHF through MIH_SAP, MIH_NET_SAP and MIH_LINK_SAP. The MIHF uses the MIH_NMS_Get_State.confirm primitive to send the state information to NMS.

The state information includes the working state defined in the MIHF state transition graph.

The state information may include information of

- MIH_LINK_SAP primitives

- Subscribed link layer events

- Link Configuration Thresholds (Link Parameter Type, Initiate Action Threshold, Rollback Action Threshold, Execute Action Threshold)

- Link status (Up/Going Down/Down)

- Link Handover status (Imminent/Complete)

- Link Capability (Supported ES/CS List)

- Link Parameters (RSSI, NDR, C/I, BER, PER, Link Data Rate)

- MIH_SAP primitives

- MIH Capability (Supported ES/CS/IS List)

- MIH Status (Device Info, Operation Mode, Link ID, Channel ID, Battery Level, Link QoS Parameter List)

- MIH Handover status

- MIH_NET_SAP primitives

- Connectivity of  data transfer session (Connect/Disconnect)

7.7.2.1 MIH_NMS_Get_State.request

7.7.2.1.1 Function

This primitive is used by NMS to get the state information.

7.7.2.1.2 Semantics

The parameters of the primitive are as follows:

MIH_NMS_Get_State.request   (

StateInformationRequestList

)

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| StateInformationRequestList | List | N/A | List of the state information to be queried |

7.7.2.1.3 When Generated

This primitive is generated by NMS when it needs to get the state information from MIHF.

7.7.2.1.4 Effect on Receipt

MIHF collects the requested information.

7.7.2.2 MIH_NMS_Get_State.confirm

7.7.2.2.1 Function

This primitive returns the state information to NMS.

7.7.2.2.2 Semantics

The parameters of the primitive are as follows:

MIH_NMS_Get_State.request   (

StateInformationResponseList

)

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| StateInformationResponseList | List | N/A | List of the state information |

7.7.2.2.3 When Generated

This primitive is generated by MIHF when the MIHF receives MIH_NMS_Get_State.request from NMS.

7.7.2.2.4 Effect on Receipt

NMS may manage MIHF based on the state information.