| Project | IEEE 802.21 MIHS <http://www.ieee802.org/21/> |  |
| --- | --- | --- |
| Title | Clarification of QoS relevant description |  |
| DCN | 21-06-0787-00-0000 |  |
| Date Submitted | November 2006 |  |
| Source(s) | Kentaro Ishizu, Yoshia Saito, Masahiro Kuroda, Ryoichi Komiya | NICT |
| Re: | IEEE 802.21 session #17 in November 2006 |  |
| Abstract | This document restructures section 5.1.3. |  |
| Purpose | Discuss and adopt in the draft |  |
| Notice | This document has been prepared to assist the IEEE 802.21 Working Group. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s). The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that IEEE 802.2 may make this contribution public. |  |
| Patent Policy | The contributor is familiar with IEEE patent policy, as outlined in Section 6.3 of the IEEE-SA Standards Board Operations Manual <http://standards.ieee.org/guides/opman/sect6.html#6.3> and in Understanding Patent Issues During IEEE Standards Development <http://standards.ieee.org/board/pat/guide.html>. |  |

We propose to clarify the QoS relevant description by restructuring section 5.1.3.

- Update the section 5.1.3 with the following text.

- Quality of Service

The quality of the service (QoS) experienced by an application depends on the accuracy, speed  and dependability of information transfer of the communication channel.  The IEEE 802.21 standard provides information to check whether application QOS requirements are satisfied in the presence of handover and defines parameters that may be used in specifying and assessing characteristics of data transfers between a source and its destinations. The communication channel is considered to be composed of several connected segments, each under a possibly different but cooperative administrative authority. Examples for such channels, such as for IP traffic have been detailed in ITU normative Y.1540.

- Create new section “5.2.1 MIH Design Principle” and move the text under section 5.2 to section 5.2.1

- Create new section “5.2.2 QoS Design Principle” and put the text as the following text in blue, which is a part of text in section 5.1.3 with some modification

    - MIHF Design Principle

5.2.2	QoS Design Principle

It is assumed that applications communicate via a packetized communication channel over which a transmitted packet can experience the following outcomes:

- Be received with no errors at its intended destination

- Be received with errors at its intended destination

- Not be received in which case it is said that the packet is lost.

The maximum attainable speed of data transfer over a given communication channel can be constant, as it is usually the case with network segments involving only wired links, or it can be time varying at different scales, at is the case for segments involving wireless links. This measure will be called channel capacity, for the purposes of this document.

The ability of the channel to provide accurate information transfer can be described via a statistical model characterized by the following parameters:

- Minimum Packet Transfer Delay: is defined as the minimum delay over a population of interest

- Average Packet Transfer Delay: is defined as the arithmetic mean of the delay over a population of interest

- Maximum Packet Transfer Delay: is defined as the maximum delay over a population of interest

- Jitter: is defined as the standard deviation of the delay over a population of interest

- Packet loss rate: is defined as the ratio between the number of frames that are transmitted but not received and the total number of frames transmitted over a population of interest.

- Packer error rate: is defined as the ratio between the number of packets that have been received with errors and the total number of packets present in a population of interest.

It is generally accepted that based on the required accuracy of data transfer applications can be grouped into a small number of behavioral sets [Y.1541] called Classes of Service (CoS). Support for differentiation via Classes of Service is pervasive in many of the IEEE 802 based standards (802.11, 802.1q, 802.16, etc.).

For a link that supports CoS differentiation, per CoS traffic accuracy parameters need to be maintained in order to provide insights on how individual traffic classes are faring.

In summary, the following set of parameters characterize the speed and accuracy of the data transfer a multi-CoS traffic link supports:

- Link capacity, representing the maximum information transfer rate achievable. This value is determined by the physical characteristics of the link. While for wired links is it usually a constant, for wireless links it is time varying.

- Link Packet Error Rate: representing the ratio between the number of frames received in error and the total number of frames transmitted in a link  population of interest

- Supported classes of service: represents the maximum number of differentiable classes of service supported by this link.

- Class of Service Parameters List. For each of the supported classes of service the following parameters are defined:

- Class Minimum Packet Transfer Delay: is defined as the minimum delay over a class population of interest

- Class Average Packet Transfer Delay: is defined as the arithmetic mean of the delay over a class population of interest

- Class Maximum Packet Transfer Delay: is defined as the maximum delay over a class  population of interest

- Class Packet Delay Jitter: is defined as the standard deviation of the delay over a class population of interest

- Class Packet loss rate: is defined as the ratio between the number of frames that are transmitted but not received and the total number of frames transmitted over a class population of interest.

The performance implications to consider from the MIH perspective include both the transient network performance achieved during a handover as well as the continuous monitoring of current network conditions.

The 802.21 specifications provide mechanisms to support seamless mobility as an enabler; however the MIHF alone cannot guarantee seamless mobility. Depending on the QOS requirements of the end-to-end application, seamless mobility implies minimizing the latency, and potential loss incurred during a handover so as to minimize the end-to-end delay and loss perceived by the application. Seamless mobility also implies the timely assessment of network conditions, such as the monitoring of the signal strength, packet loss, on the link, on both current and target networks in order to optimize the handover decision and its execution.

The MIH services defined by the IEEE 802.21, including event, command, and information service, need to consider network traffic performance objectives and their potential fulfillment as well as their relation to application traffic quality of service requirements.