| Project | IEEE 802.21 Media Independent Handover Services <http://www.ieee802.org/21/> |  |
| --- | --- | --- |
| Title | NAI primitives clean up |  |
| Date Submitted | November, 2006 |  |
| Source(s) | Ronny Kim, Jin Lee (LG Electronics) |  |
| Re: |  |  |
| Abstract | When the contribution 21-06-0765-00-000_Issue 19 was accepted, there were request and response primitives only. After Letter ballot # 1 comment resolution, we have mechanism of request, indication and response, confirmation pairs. In order to align with request, indication and response, confirmation pairs, clearing text is submitted. |  |
| Purpose |  |  |
| Notice | This document has been prepared to assist the IEEE 802.21 Working Group. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s). The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.21. |  |
| Patent Policy | The contributor is familiar with IEEE patent policy, as outlined in Section 6.3 of the IEEE-SA Standards Board Operations Manual <http://standards.ieee.org/guides/opman/sect6.html#6.3> and in Understanding Patent Issues During IEEE Standards Development <http://standards.ieee.org/board/pat/guide.html>. |  |

[Modify 7.4.14.1.3  with the text as suggested below: Page 131, line 38]

7.4.14.1.3 Effect on receipt

Upon reception of this primitive, the MIHF on the new PoS may either interact with upper layer to collect the requested information or further forward this reqeust to the another MIHF in the network, e.g Access Router with MIHF which may have enough information on this message. A corresponding indication (MIH_Network_Address_Information.indication) maybe triggered to send to all MIH users in the local stack. The associated parameters for the generated indication are same as those used in command request.

[Replace 7.4.14 with the text as suggested below: Page 129, line 55]

#### MIH_Network_Address_Information.indication

- Function

This primitive is generated by MIH Function on new PoS to indicate that MIH_Network_Address_Information Request message is received from a peer MIH Function on old PoS.

- Semantics of service primitive

The parameters of the primitive are as follows:

MIH_Network_Address_Information.request 	(

SourceIdentifier,

DestinationIdentifier,

CurrentLinkIdentifier,

MacMobileNode,

TargetPoAIdentifier

Current IP Configuration Method

Current DHCP Server Address

CurrentAccessRouterAddress,

CurrentFAAddress

)

Local or Remote: Both

MIHF(Terminal) <> MIHF (Network) <> MIHF (Network)

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| Source Identifier | Identifier | Any valid individual or group identifier | The identifier of entity where the request is initiated. This field may be optionally left empty if the command is local. |
| Destination Identifier | Identifier | MIH_LOCAL, MIH_REMOTE | The destination identifier of request or response. This is the identifier of local or peer MIH Function. |
| CurrentLinkIdentifier | NetworkIdentifier. May be one of different 802 and Cellular networks. | Identifier | This identifies the current access net work over which the command needs to be sent. This is valid only for remote commands which need to be sent to remote MIHF. The command is then sent either at L2 or at L3. |
| MacMobileNode | MAC Address | N/A | MAC Address of Mobile Node |
| TargetPoAIdentifier | MAC Address |  | This is the candidate target Point of Attachment (AP/BS) on new network. |
| Current IP Configuration Method | Bitmap | 0~31 | Bit 0: IPv4 static configuration Bit 1: IPv4 dynamic configuration (DHCPv4) Bit 2: Mobile IPv4 with FA (FA-CoA) Bit 3: Mobile IPv4 without FA (Co-located CoA) Bits 4 -10: reserved for IPv4 address configurations Bit 11: IPv6 stateless address configuration Bit 12: IPv6 stateful address configuration (DHCPv6) Bit 13: IPv6 manual configuration Bits 14-31: Reserved. |
| Current DHCP Server Address | IP Address | N/A | IP address of currnet DHCP Server. This parameter is only included when mobile node is using dynamic address configuration. |
| Current FA Address | IP Address | N/A | IP address of Current Foreign Agent. This parameter is only included when mobile node is using Mobile IPv4, |
| Current Access Router Address | IP Address | N/A | IP address of Current Access Router This parameter is only included when mobile node is using IPv6 |

- When generated

This primitive is generated by MIH Function on receiving MIH_Network_Address_Information Request message from a peer MIH Function.

- Effect on receipt

Upon reception of this primitive, The MIH user on new PoS may generate another MIH_Network_Address_information.request primitive further to deliver the corresponding message to the MIHF in the network, e.g Access Router with MIHF which may have enough information on this message or replies with MIH_Network_Address_Information.response primitive. When this primitive is delivered to the upper layer of the MIHF in the network, e.g, Access Router with MIHF, this may trigger to send network address related information, e.g, Router Advertisement either directly to the mobile node or to MIHF.

[Modify the text as suggested below: Page 134, line 38]

- When generated

The MIHF responds with this primitive in response to a MIH_Network_Address_Information.request command from a peer MIHF entity.

- Effect on receipt

When the MIHF in the old PoS receives this message from the new PoS, the MIHF in the new PoS further delivers this message to the MIHF in the Mobile Node.

[Replace 7.4.14 with the text as suggested below: Page 129, line 55]

#### MIH_Network_Address_Information.confirm

- Function

This primitive is generated by MIH Function to confirm that MIH_Network_Address_Information Response message is received from a peer MIH Function.

- Semantics of service primitive

The parameters of the primitive are as follows:

MIH_Network_Address_Information.response (

SourceIdentifier,

DestinationIdentifier,

CurrentLinkIdentifier,

MacMobileNode,

NewPoAIdentifier

IP Configuration Method

DHCP Server Address,

FAAddress,

AccessRouterAddress,

Result Code

)

Local or Remote: Both

MIHF (Network) <> MIHF (Network) <> MIHF(Terminal)

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| Source Identifier | Identifier | Any valid individ ual or group iden tifier | The identifier of entity where the request is initiated. This field may be optionally left empty if the command is local. |
| Destination Identifier | Identifier | MIH_LOCAL, MIH_REMOTE | The destination identifier of request or response. This is the identifier of local or peer MIH Function. |
| CurrentLinkIdentifier | NetworkIdenti fier. May be one of different 802 and Cellular networks. | Identifier | This identifies the current access net work over which the command needs to be sent. This is valid only for remote commands which need to be sent to remote MIHF. The command is then sent either at L2 or at L3. |
| MacMobileNode | MAC Address | N/A | MAC Address of Mobile Node |
| NewPoAIdentifer | MAC Address | N/A | This is the target Point of Attachment (AP/BS) on new network. |
| IP Configuration Method | Bitmap | 0~31 | Bit 0: IPv4 static configuration Bit 1: IPv4 dynamic configuration (DHCPv4) Bit 2: Mobile IPv4 with FA (FA-CoA) Bit 3: Mobile IPv4 without FA (Co-located CoA) Bits 4 -10: reserved for IPv4 address configurations Bit 11: IPv6 stateless address configuration Bit 12: IPv6 stateful address configuration (DHCPv6) Bit 13: IPv6 manual configuration Bits 14-31: Reserved. |
| DHCP Server Address | IP Address | N/A | IP address of DHCP Server. This parameter is only included when mobile node is using dynamic address configuration |
| FA Address | IP Address | N/A | IP address of Foreign Agent. This parameter is only included when mobile node is using Mobile IPv4, |
| Access Router Address | IP Address | N/A | IP address of Access Router This parameter is only included when mobile node is using IPv6 |
| Result Code | Bitmap | 0~8 | Bit 0: IP configuration Method is not available Bit 1: DHCP Server address is not available Bit 2: FA Address is not available Bit 3: Access Router Address is not available Bit 4: No information is provided due to accessibility of same entity (FA, Access Router, DHCP Server, etc) Bit 5~7: Reserved |

- When generated

This primitive is generated by MIH Function on receiving MIH_Network_Address_Information Response message from a peer MIH Function.

- Effect on receipt

The MIH user may use received network address related information to configure network address.