| Project | IEEE 802.21 Media Independent Handover Services <http://www.ieee802.org/21/> |  |
| --- | --- | --- |
| Title | 21-06-0805-00-0000-MediaIndependentPaging.doc |  |
| Date Submitted | November 8, 2006 |  |
| Source(s) | Alice Qin, Behcet Sarikaya Phil Barber Muthaiah Venkatachalam | Huawei Technologies alice.Q@huawei.com, bsarikaya@huawei.com, pbarber@huawei.com Intel Corporation muthaiah.venkatachalam@intel.com |
| Re: | P802-21-D02.00.pdf |  |
| Abstract | Media Independent Paging extension to 802.21 |  |
| Purpose | Propose new text in current draft. |  |
| Notice | This document has been prepared to assist the IEEE 802.21 Working Group. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s). The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.21. |  |
| Patent Policy | The contributor is familiar with IEEE patent policy, as outlined in Section 6.3 of the IEEE-SA Standards Board Operations Manual <http://standards.ieee.org/guides/opman/sect6.html#6.3> and in Understanding Patent Issues During IEEE Standards Development <http://standards.ieee.org/board/pat/guide.html>. |  |

Media Independent Paging

1	Abstract	1

2	Introduction	2

3	Terminology	3

4	Motivation	3

5	Media Independent Paging Reference Framework	3

5.1	Media Independent Paging Logical Reference Model	3

6	Media Independent Paging Reference Model and SAPs	4

6.1	General PMIHF Reference Model	4

6.2	Primitives of Command Service Category	6

6.2.1	MI_P_EXII.Request (MI Exit Dormant Mode Request)	6

6.2.2	MI_P_EXII.response (MI Exit Dormant Mode Response)	7

6.2.3	MI_P_ENTI.Request (MI Enter Dormant Mode Request)	7

6.2.4	MI_P_ENTI.response (MI Enter Dormant Mode response)	9

6.3	Primitives of Event Service Category	10

6.3.1	MI_P_LU.Request (MI Location Update Request)	10

6.3.2	MI_P_LU.response (MI Location Update Response)	11

6.3.3	MI_P_QUERY.request	12

6.3.4	MI_P_QUERY.response	13

6.3.5	MI_PM_NOTIFY	14

6.3.6	P_PM_NOTIFY	15

6.3.7	P_PA_NOTIFY	15

7	Media Independent Paging Key Procedures and Operations	16

7.1	Paging-elimination	16

7.2	Paging-aggregation	18

7.3	Location Update via "connected" radio	18

7.4	Aggregated Location Update	19

7.5	Dormant Mode Entry	21

8	References	22

# Abstract

This document introduces the Media Independent Paging and Dormant Mode, which we propose as an extension to Media Independent Handover service in order to support heterogeneous Dormant Mode and Paging mechanism. The new primitives and architectural entities are described and several usage scenarios are detailed.

# Introduction

Media Independent Paging mechanism is used by the network to page a multi-mode mobile station/node (MN) in dormant mode on the selected technical interface. Paging may be initiated and executed in different technical interfaces, or even the interfaces could be heterogeneous. The Media Independent Paging related Events and Commands may identify the initiator of paging and the destination where the paging procedure may be operated.

In addition, Media Independent Paging mechanism may obtain location update and network entry via other selected active technical interface from an MN in dormant mode in one technical interface.

Media Independent Paging Application (MIPA) is one of the users of the media independent handover function (MIHF) [1] services where MIHF is paging enabled. Figure 1 shows the location of the MIPA in MIH service architecture.

Figure 1 Paging Enabled MIH Service

Media Independent Paging messages may be transferred over Media Independent Handover protocol and reuse the Media Independent Handover service reference model. Media Independent Paging events and commands may be triggered by media specific paging events or the upper layer user, for example, MIPA. Media Independent Paging events and commands may be mapped into the media specific paging operation and procedure to finish the whole paging function across multiple heterogeneous networks.

This document introduces the Media Independent Paging reference framework in Section 5. In Section 6, the primitives are presented as an extension to MIHF to support paging. The paging key procedures and usage scenarios are described in Section 7.

# Terminology

PMIHF – A Media Independent Handover Function (MIHF) that is paging enabled.

Dormant Mode Controller (DMC) — It is a Media Independent enabled functional entity that administers the activity of Dormant mode MN across multiple heterogeneous networks via MIH service. It is identified by MIHF ID defined in IEEE 802.21.  There shall be a single DMC (called Anchor DMC) that contains the location information of the MN on the media specific link.

Media Independent enabled Paging Agent (MI-P PA) – It is a Media Independent enabled functional entity that handles the interaction between MI-Paging DMC and media specific paging agent related functionality implemented in the Base Station.

Paging Area – It may be interpreted as comprising of one or more Paging Agents. Paging Area identifier (Paging Area ID) identifies the Paging Area. The combination of Paging Area ID, Network Identifier and network type is a unique identifier for the paging area in which the MN located.

Data Path Function (DPF) — The Data Path function is used to setup the bearer plane between Base-Stations or between other entities such as Gateways or between gateways and base-stations.

# Motivation

Media Independent Paging and Dormant Mode mechanism is advantageous to an MN and the network for the following reasons:

- Media Independent Paging and Dormant Mode mechanism allows paging on multiple technology interfaces with a single selected technology interface. This architecture supports a unified heterogeneous paging and dormant mode mechanism.

- A host in dormant mode is expecting to be paged on an optimal interface, and not to be paged on only one. For example, a paging request from WLAN could be executed via 3GPP network.

- To extend the battery life of a multi-mode MN.

# Media Independent Paging Reference Framework

## Media Independent Paging Logical Reference Model

Media independent paging enabled MIH functionality called PMIHF may be implemented either on the MN or in the network. MIHP network model is shown in Figure 2.

The model includes an PMIHF capable mobile station (MI-P MN) that supports multiple access technology options and has multiple radio interfaces. The MIH capability advertisement, subscription and query need to provide extensions to support MI-Paging. The model assumes that the provisioning service provider allows its user to be paged across multiple access technologies. When provisioned, the MN may use MIIS to obtain pertinent information such as the network node that provides media independent paging function capability.

Figure 2 A Network Model for Media Independent Paging Services

The DMC maintains the location entry of the MN.  Whenever MN enters Dormant Mode, it registers with DMC.  MN registers Paging Area ID of its visited network, the network type and the network identifier with DMC, which are obtained from the media specific Layer 2.  Usually, the DMC is integrated with DPF.  As soon as DMC detects paging requirement for MN, DMC sends paging request to the paging agent in homogeneous network or the anchor DMC in heterogeneous network.  MN in dormant mode is paged.  As MN comes back to active mode and sends exiting Dormant Mode event primitive to DMC to indicate that it has quit from Dormant Mode on the selected link interface, and then the anchor DMC on the selected link may mark the location entry of the MN by a status flag. If the anchor DMC which originates the paging action is not in the homogeneous network, the exiting Dormant Mode event primitive is relayed to the anchor DMC.

Whenever the MN changes Paging Area, MN may obtain the new Paging Area ID via media specific Layer 2.  Location update procedure may be initiated by MN to update the information about the Paging Area in which the MN is located, as long as the MN is still in Dormant Mode.  The media specific location update message may be carried in an information element over the event service of MIHF.  The location update procedure may be executed over the selected link instead of the current link.

# Media Independent Paging Reference Model and SAPs

## General PMIHF Reference Model

MI-Paging Function provides Media Independent Paging and Dormant Mode service via Media Independent Service Access Point (SAP) to the upper layers.  The MI-Paging Function exchanges primitives with Media-specific Paging Function using media specific paging SAP. Via the MI-Paging SAP, the MI-Paging function executes the heterogeneous paging operations and procedures.

The MI-Paging SAP consists of a set of service primitives that specify the information to be exchanged and the format of the information exchanges. These primitives have mappings to the media specific paging primitives.

Figure 3 General PMIHF Reference Model and SAPs

The following Table 1 lists the MI-Paging primitives. Several media dependent primitives shall be appended in media specific technologies. For example, two triggers are presented in Table 2.

Table 1 MI-Paging SAP Primitives

| No. | Primitives | Service Category | Description |
| --- | --- | --- | --- |
| 1 | MI_P_LU | Event Service | Indicate that the MN intends to update its location. |
| 2 | MI_P_ENTI | Command Service | Indicate the MN enter Dormant Mode |
| 3 | MI_P_EXII | Command Service | Indicate the MN exit Dormant Mode |
| 4 | MI_PM_NOTIFY | Event Service | Inform remote PMIHF the paging mode changes. |
| 5 | MI_PM_QUERY | Event Service | Query about the connection mode in which the MN is in selected network. |

Table 2 Paging SAPs Primitives

| No. | Primitives | Service Category | Description |
| --- | --- | --- | --- |
| 1 | P_PM_NOTIFY | Event service | New paging mode trigger. |
| 2 | P_PA_NOTIFY | Event service | Paging Area trigger. |

## Primitives of Command Service Category

### MI_P_EXII.Request (MI Exit Dormant Mode Request)

- Function

This primitive is used by a peer PMIHF to communicate with the local user as well as the remote PMIHF on network. The primitive is used to communicate intent of exiting Dormant Mode. This primitive may be initiated by local user or the remote PMIHF.

- Semantics

The parameters of the primitive are as follows:

MI_P_EXII.request(

Source Identifier,

Destination Identifier,

MN ID,

Interface IDs list

)

Table 3

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| Source Identifier | Identifier | Any valid MIHF identifier | Source of this message |
| Destination Identifier | Identifier | Any valid MIHF identifier | Destination of this message |
| MN ID | Identifier | Any valid individual identifier | The identifier of MN, (eg. FQDN or NAI) |
| Interface IDs list | list | Any valid interface identifiers | Lists of interface Identifier |

- When generated

This primitive may be as a result of media independent paging procedure and initiated by the remote PMIHF or the local Paging function.

- Effect on receipt

On receiving the request the MN may exit Dormant Mode in selected links indicated by interface IDs list.

### MI_P_EXII.response (MI Exit Dormant Mode Response)

- Function

This primitive is used by a peer PMIHF to communicate with the PMIHF that sent out a MI_P_EXII.request. The primitive is used to communicate the response of Dormant Mode exiting Dormant Mode request.

- Semantics

The parameters of the primitive are as follows:

MI_P_EXII.response(

Source Identifier,

Destination Identifier,

MN ID,

Interface IDs list

)

Table 4

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| Source Identifier | Identifier | Any valid MIHF identifier | Source of this message |
| Destination Identifier | Identifier | Any valid MIHF identifier | Destination of this message |
| MN ID | Identifier | Any valid individual identifier | The identifier of MN, (e.g. FQDN or NAI) |
| Interface IDs list | List | Any valid interface identifiers | Lists of interface Identifier |

- When generated

The PMIHF responds with this primitive in response to an MI_P_EXII.request command from a peer PMIHF entity.

- Effect on receipt

On receiving the response the remote PMIHF may have started the media specific exiting Dormant Mode procedure.

### MI_P_ENTI.Request (MI Enter Dormant Mode Request)

- Function

This primitive is used by a peer PMIHF to communicate with the local user as well as the remote PMIHF on network. The primitive is used to communicate intent of entering Dormant Mode. The operation may be initiated either by the MN or by the network.

- Semantics

The parameters of the primitive are as follows:

MI_P_ENTI.request(

Source Identifier,

Destination Identifier,

MN ID,

Groups [Interface ID,

Anchor DMC ID]

)

Table 5

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| Source Identifier | Identifier | Any valid MIHF identifier | Source of this message |
| Destination Identifier | Identifier | Any valid MIHF identifier | Destination of this message |
| MN ID | Identifier | Any valid individual identifier | The identifier of MN, (e.g. FQDN or NAI) |
| Interface ID | Interface Identifier | Any valid interface identifier | Lists of interface Identifier |
| Anchor DMC ID | Identifier | Any valid BS/AP Identifier | A logical network identifier for the serving BS administering paging activity for the MN while in Idle Mode. |

- When generated

This primitive is generated by remote PMIHF or local user that may want the terminal to enter Dormant Mode in the current selected link or all links. This command may be sent either to just the local selected link or it may be sent to the remote PMIHF in the network as well. The remote PMIHF may initiate this primitive to command the terminal to enter Dormant Mode in selected several or all links.

- Effect on receipt

The remote PMIHF responds with MI_P_ENTI.response primitive. If the recipient of the command is a network PMIHF, it may perform a media specific network initiated entering Dormant Mode operation message exchange with the Paging Function in the target link. If the primitive originated from local user to local PMIHF, it may perform a media specific MN initiated entering Dormant Mode operation.

### MI_P_ENTI.response (MI Enter Dormant Mode response)

- Function

This primitive is used by a peer PMIHF to communicate with the PMIHF that sent out a MI_P_ENTI.request. The primitive is used to communicate the response of Dormant Mode entering request. The response may be sent either by the client or the network depending on where the request was initiated.

- Semantics

The parameters of the primitive are as follows:

MI_P_ENTI.Response (

Source Identifier,

Destination Identifier,

MN ID,

Group [Interface ID,

Anchor DMC ID]

)

Table 6

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| Source Identifier | Identifier | Any valid MIHF identifier | Source of this message |
| Destination Identifier | Identifier | Any valid MIHF identifier | Destination of this message |
| MN ID | Identifier | Any valid individual identifier | The identifier of MN, (e.g. FQDN or NAI) |
| Interface ID | Interface Identifier | Any valid interface identifier | Lists of interface Identifier |
| Anchor DMC ID | Identifier | Any valid BS/AP Identifier | A logical network identifier for the serving BS administering paging activity for the MN while in Idle Mode. |

- When generated

The PMIHF responds with this primitive in response to a MI_P_ENTI.request command from a peer MIHF entity.

- Effect on receipt

On receiving the response the remote PMIHF may have started the media specific entering Dormant Mode procedure.

## Primitives of Event Service Category

### MI_P_LU.Request (MI Location Update Request)

- Function

This primitive is generated by local media specific paging function or local user that may want the terminal to update location in the current selected link or all links. This command may be sent either to just the local selected link or it may be sent to the remote PMIHF in the network as well. The remote PMIHF may initiate this primitive to update the location in the other remote PMIHF in other selected links.

- Semantics

The parameters of the primitive are as follows:

MI_P_LU.request (

Source Identifier,

Destination Identifier,

MN ID,

Group [Interface ID,

Anchor DMC ID,

Target BSID,

Paging Area ID,

Power Down Indicator]

)

Table 7

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| Source Identifier | Identifier | Any valid MIHF identifier | Source of this message |
| Destination Identifier | Identifier | Any valid MIHF identifier | Destination of this message |
| MN ID | Identifier | Any valid individual identifier | The identifier of MN, (eg. FQDN or NAI) |
| Interface ID | Interface Identifier | Any valid interface identifier | Lists of interface Identifier |
| Anchor DMC ID | Identifier | Any valid BS/AP Identifier | A logical network identifier for the serving BS administering paging activity for the MN while in Idle Mode. |
| Target BSID | Identifier | Identifier of Target BS |  |
| Paging Area ID | Identifier | One or more logical affiliation groupings of BS | Identifier of the paging area the MN is assigned to |
| Power Down Indicator | Boolean | False/True | Indicates the MN is currently attempting to perform Location Update due to power down. |

- When generated

When the media specific paging function needs to send location request, it may indicate the local PMIHF to initiate this primitive. The remote PMIHF may initiate this and send it to the other selected PMIHF.

- Effect on receipt

On receiving the request the receipt may relay this or re-construct this and send it to other remote MIHF to finish the location update operation in selected link or all links.

### MI_P_LU.response (MI Location Update Response)

- Function

This primitive is used by a peer PMIHF to communicate with the PMIHF that sent out a MI_P_LU.request. The primitive is used to communicate the response of location update request.

- Semantics

The parameters of the primitive are as follows:

MI_P_LU.Response (

Source Identifier,

Destination Identifier,

MN ID,

Group [Interface ID,

Anchor DMC ID,

Result,

Paging Information]

)

Table 8

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| Source Identifier | Identifier | Any valid MIHF identifier | Source of this message |
| Destination Identifier | Identifier | Any valid MIHF identifier | Destination of this message |
| MN ID | Identifier | Any valid individual identifier | The identifier of MN, (eg. FQDN or NAI) |
| Interface ID | Interface Identifier | Any valid interface identifier | Lists of interface Identifier |
| Anchor DMC ID | Identifier | Any valid BS/AP Identifier | A logical network identifier for the serving BS administering paging activity for the MN while in Idle Mode. |
| Result | Enumerate | 0:Success 1:Failure | Identify the result of location procedure |
| Paging Information | List | N/A | This specifies the parameters on location update and paging. |

- When generated

The PMIHF responds with this primitive in response to an MI_P_LU.request command from a peer MIHF entity.

- Effect on receipt

On receiving the response the remote PMIHF may have started and finished the media specific location update procedure.

### MI_P_QUERY.request

- Function

This primitive is used by the PMIHF in network node to query about the connection mode information of the distinctive MN in selected network.

- Semantics

The parameters of the primitive are as follows:

MI_P_QUERY.request (

Source Identifier,

Destination Identifier,

MN ID,

Interface ID,

Anchor DMC ID

)

Table 9

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| Source Identifier | Identifier | Any valid MIHF identifier | Source of this message |
| Destination Identifier | Identifier | Any valid MIHF identifier | Destination of this message |
| MN ID | Identifier | Any valid individual identifier | The identifier of MN, (eg. FQDN or NAI) |
| Interface ID | Interface Identifier | Any valid interface identifier | Lists of interface Identifier |
| Anchor DMC ID | Identifier | Any valid BS/AP Identifier | A logical network identifier for the serving BS administering paging activity for the MN while in Idle Mode. |

- When generated

As the PMIHF in the DMC is triggered by paging requirement events, such as when downlink data is needed to be transmitted to MN in the network, the PMIHF shall send this primitive to the remote PMIHF to query the connection mode to determine which radio interface in MN is appreciate to transport the MI_P_ANN primitives to page the MN in the selected network..

- Effect on receipt

On receiving this primitive, the remote PMIHF in anchor DMCs shall send back the connection mode information of the interface of MN indicated by interface id.

### MI_P_QUERY.response

- Function

This primitive is used by the PMIHF in network node to response to the MI_P_QUERY.request message to inform the sender the connection mode information of the distinctive MN in selected network.

- Semantics

The parameters of the primitive are as follows:

MI_P_QUERY.response (

Source Identifier,

Destination Identifier,

MN ID,

Interface ID,

Anchor DMC ID,

Connection mode

)

Table 10

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| Source Identifier | Identifier | Any valid MIHF identifier | Source of this message |
| Destination Identifier | Identifier | Any valid MIHF identifier | Destination of this message |
| MN ID | Identifier | Any valid individual identifier | The identifier of MN, (eg. FQDN or NAI) |
| Interface ID | Interface Identifier | Any valid interface identifier | Lists of interface Identifier |
| Anchor DMC ID | Identifier | Any valid BS/AP Identifier | A logical network identifier for the serving BS administering paging activity for the MN while in Idle Mode. |
| Connection Mode | Enumerate | 0: Active mode 1: Idle mode | This specifies the status the MN is in. |

- When generated

As the PMIHF in the DMC receives the MI_P_QUERY.request, the PMIHF shall send this primitive to the remote PMIHF to indicate the connection mode for the peer.

- Effect on receipt

On receiving this primitive, the remote PMIHF in anchor DMCs determines which radio interface in MN is appreciate to transport the MI_P_EXII primitives to wake up the MN in the selected network.

### MI_PM_NOTIFY

- Function

This primitive is used by the PMIHF to inform the remote PMIHF that the link layer changes its paging mode.

- Semantics

The parameters of the primitive are as follows:

MI_PM_NOTIFY (

Source Identifier,

Destination Identifier,

MN ID,

Interface ID,

Connection Mode

)

Table 11

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| Source Identifier | Identifier | Any valid MIHF identifier | Source of this message |
| Destination Identifier | Identifier | Any valid MIHF identifier | Destination of this message |
| MN ID | Identifier | Any valid individual identifier | The identifier of MN, (eg. FQDN or NAI) |
| Interface ID | Interface Identifier | Any valid interface identifier | Lists of interface Identifier |
| Connection Mode | Enumerate | 0: Active mode 1: Idle mode | This specifies the status the MN is in. |

- When generated

As the PMIHF receives P_PM_NOTIFY, the PMIHF shall send this primitive to the remote PMIHF.

- Effect on receipt

On receiving this primitive, the remote PMIHF in anchor DMCs shall update the location entry status.

### P_PM_NOTIFY

- Function

This primitive notifies the PMIHF that MN enters into the dormant mode or MN exits from dormant mode and come back to active mode on layer 2.

- Semantics

P_PM_NOTIFY (

Interface ID,

Connection Mode

)

Table 12

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| Interface ID | Interface Identifier | Any valid interface identifier | Lists of interface Identifier |
| Connection Mode | Enumerate | 0: Active mode 1: Idle mode | This specifies the status the MN is in. |

- When generated

Whenever MN changes mode from active to dormant mode, and vice versa, this primitive is sent to local PMIHF.

- Effect on receipt

MI_PM_NOTIFY primitive is triggered to be sent to the remote PMIHF in pertinent anchor DMCs, which maintains the location entry status of the MN.

### P_PA_NOTIFY

- Function

The primitive triggers the location update procedure.

- Semantics

P_PA_NOTIFY (

Interface ID,

Target BSID,

Paging Area ID

)

Table 13

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| Interface ID | Interface Identifier | Any valid interface identifier | Lists of interface Identifier |
| Target BSID | Identifier | Identifier of Target BS |  |
| Paging Area ID | Identifier | One or more logical affiliation groupings of BS | Identifier of the paging area the MN is assigned to |

- When generated

Whenever MN changes Paging Area, this primitive is sent up to PMIHF.  This primitive is also sent when the MN is powered on.

- Effect on receipt

The PMIHF located in the MN may generate the MI_P_LU.request primitive and send it up to remote PMIHF located in anchor DMC.  This primitive helps DMC to have updated information about the location of the MN.

# Media Independent Paging Key Procedures and Operations

In this section we show the use of media independent paging primitives in five scenarios. These scenarios are taken from [2].

## Paging-elimination

When the MN has multiple radio interfaces and accesses to multiple radio networks, one of radio interfaces is in active mode, others are in dormant mode, as shown in Fig 4.

Radio j network needs to page the user. Instead of paging the user on the Radio j airlink and spending considerable resources, the radio j network determines if any other radio is currently on for the user, and then eliminates paging by sending a unicast message on radio k network to turn on the radio j network.

The Radio j network may make an application of the procedure in Fig 4.

MN

DMC of Radio k

DMC of Radio j

DMC of Radio i

PA of Radio k

Trigger Paging

1)MI_P_QUERY.Request

2)MI_P_QUERY.Request

MI_P_QUERY.Response

MI_P_QUERY.response

3)MI_P_EXII.Request

MI_P_EXII.Request

5)Network re-entry procedure

MI_P_EXII.response

4)MI_P_EXII.response

6)MI_PM_NOTIFY

Figure 4 Paging Elimination Procedure

- DMC of radio j network receives downlink data to the MN, Radio j network needs to page the MN. The PMIHF in DMC of radio j network sends MI_P_QUERY.request message to DMC of radio i network to retrieve the connection mode of MN. The PMIHF in DMC of radio i network send MI_P_QUERY.response message to inform the radio j network that the MN is not in active mode currently.

- The MN has access to three kinds of radio networks.  As the DMC of radio j network determines that it is not anticipating to page the user in radio i network, the PMIHF in DMC of radio j network sends MI_P_QUERY.request message to DMC of radio k network to retrieve the connection mode of MN.  Interface ID is used to differentiate which radio interface the sender indicates should be involved in the forthcoming network re-entry procedure.  The PMIHF in DMC of radio k network send MI_P_QUERY.response message to inform the radio j network that the MN is actually in active mode at the moment.

- The DMC of radio j makes determination that the MN shall be paged in radio k network.  The PMIHF of the DMC of radio j sends MI_P_EXII.request message to the PMIHF in DMC of radio k network

- The MI_P_ EXII .request message is forwarded to the MN. The MN sends back MI_P_ EXII.response message as a response to the MI_P_ EXII.request message.

- As the indication in MI_P_ EXII message, the MN initiates the network re-entry procedure in radio j network.

- After the MN exits the dormant mode in radio j network, the DMC of radio j informs the DMC of radio k that the MN comes back into active mode via sending MI_PM_NOTIFY message.

## Paging-aggregation

When the MN has multiple radio interfaces and accesses to multiple radio networks, all radio interfaces could be triggered to exit dormant mode by sending one unicast message via one of radio interfaces which is in active mode, as in Fig 5.

The Radio k network may make an application of the procedure in Fig 5.

MN

DMC of Radio k

DMC of Radio j

DMC of Radio i

PA of Radio k

1)Trigger both r_i and r_j to exit Dormant Mode

4) Network re-entry procedure

2)MI_P_EXII.Request

3)MI_P_EXII.Response

4) Network re-entry procedure

5)MI_PM_NOTIFY

Figure 5 Paging Aggregation Procedure

- The DMC of radio k needs to awake the MN back into active mode on radio i and j network.

- The DMC of radio k sends MI_P_EXII.request message to the MN to indicate that radio i and j network exit dormant mode by the interface IDs list.

- The MI_P_EXII.response is sent back as a response for this request.

- The MN re-enters the network at same time on multiple radio networks which appear in the interface IDs list in MI_P_EXII.response message.

- When the radio interface of the MN comes back into active mode successfully, the MI_PM_NOTIFY primitive is sent to the action initiator.

## Location Update via "connected" radio

When the MN has multiple radio interfaces and accesses to multiple radio networks, the location update procedure on one interface in dormant mode is implemented by sending MI_P_LU primitives via other radio interface which is in active mode, as in Fig 6.

Radio k network may make an application of the procedure in Fig 6.

MN

DMC of Radio k

DMC of Radio j

PA of Radio k

1) Location update is triggered by r_j， P_PA_NOTIFY is triggered from lower layer to MIPF in MN.

2) MI_P_LU.Request

3) MI_P_LU.Response

Figure 6 Location update via “connected” link

The radio interface j of the MN is in dormant mode, its location update information is transmitted over radio k network, instead of radio j network.

- When the MN is spanning from one paging area to another paging area, P_PA_NOTIFY is triggered from lower layer to PMIHF, the MN needs to update location on radio j network.

- The PMIHF in the MN sends MI_P_LU.request primitive in which the interface ID indicates the selected interface by location update procedure. This primitive is forwarded to the anchor DMC of radio j network by PA and anchor DMC of radio k network.

- The PMIHF in the DMC of radio j network sends MI_P_LU.response primitive as the response to the MI_P_LU.request primitive from the MN. The response is forwarded to the MN by the DMC of radio k network.

## Aggregated Location Update

The MN has multiple radio interfaces and accesses to multiple radio networks.  The MN traverses the different paging areas for multiple radio interfaces simultaneously, since the paging areas may be overlapping with each other even though paging area is irrelevant for distinctive radio networks.  In such cases, several location update procedures shall be triggered at the same time on respective radio networks.

The multiple location update information can be aggregated and transmitted over a single radio network by sending MI_P_LU primitives, and propagated by the anchor DMC in this network via MIH service, as in Fig 7.

Radio k network may make an application of the procedure in Fig 7.

MN

DMC of Radio k

DMC of Radio j

PA of Radio k

1) Location update is triggered on r_i, r_i and r_k simultaneously

2) MI_P_LU.Request (r_i,r_j,r_k)

5) MI_P_LU.Response

DMC of Radio i

3) MI_P_LU.Request (r_j)

4) MI_P_LU.Request (r_i)

MI_P_LU.Response (r_i)

MI_P_LU.Response (r_i)

Figure 7 Aggregated Location Update Procedure

- The MN needs to update location on radio j network, radio i network and radio k network simultaneously.

- The PMIHF in the MN sends MI_P_LU.request primitive to the anchor DMC of radio k network, in which the interface IDs indicate the related interface with location update procedures.

- The PMIHF in the DMC of radio k network forwards MI_P_LU.request primitive to the anchor DMC of the radio j network in which it includes location information of the MN on radio j network.  The anchor DMC of radio j network sends back MI_P_LU.response primitive as the response to the MI_P_LU.request primitive from the anchor DMC of radio k network.

- As in step 3, the PMIHF in the DMC of radio k network forwards MI_P_LU.request primitive to the anchor DMC of the radio i network in which it includes location information of the MN on radio i network.  The anchor DMC of radio i network sends back MI_P_LU.response primitive as the response to the MI_P_LU.request primitive from the anchor DMC of radio k network.

- When the PMIHF in anchor DMC of radio k network received all required MI_P_LU.response primitive messages, it sends the MI_P_LU.response primitive to the initiator to indicate that the aggregated location update procedure is finished.

## Dormant Mode Entry

The MN has multiple radio interfaces and accesses to multiple radio networks.  The MN may initiate multiple radio interfaces to enter into dormant mode simultaneously for some reason.  In such cases, this command shall be transmitted from one radio network to those target networks, then, the dormant mode entry procedure may be initiated by the network.

Radio k network may make an application of the procedure in Fig 8.

MN

DMC of Radio k

DMC of Radio j

PA of Radio k

2) MI_P_ENTI.Request (r_i,r_j,r_k)

5) MI_P_ENTI.Response

DMC of Radio i

3) MI_P_ENTI.Request (r_j)

MI_P_ENTI.Request (r_i)

MI_P_ENTI.Response (r_i)

4) MI_P_ENTI.Response (r_i)

Dormant Mode entry

Dormant Mode entry

Figure 8 Dormant Mode Entry Procedure

- The MN needs to enter into dormant mode on radio j network, radio i network and radio k network simultaneously.

- The PMIHF in the MN sends MI_P_ENTI.request primitive to the anchor DMC of radio k network, in which the interface IDs indicates the related interface with dormant mode entry procedures.

- The PMIHF in the DMC of radio k network forwards MI_P_ ENTI.request primitive to the anchor DMC of the radio j network, in which it includes paging information of the MN on radio j network.  Also it forwards MI_P_ENTI.request primitive to the anchor DMC of the radio i network.

- The anchor DMC of radio j network sends back MI_P_ ENTI.response primitive as the response to the MI_P_ ENTI.request primitive from the anchor DMC of radio k network. It is the same message sent to the anchor DMC of radio i network.

- When the PMIHF in anchor DMC of radio k network received all required MI_P_ENTI.response primitive messages, it sends the MI_P_ENTI.response primitive to the initiator to indicate that the dormant mode entry procedure is finished.

# References

      - IEEE 802.21, "Draft IEEE Standard for Local and Metropolitan Area Networks: Media Independent Handover Services", D02, September 2006.

      - Muthiah Venkatachalam, “Media Independent Idle Mode and Paging”, IEEE 802.21 Intel Contribution 21-06-0701-00-0000, July 2006.