| Project | IEEE 802.21 Media Independent Handover Services <http://www.ieee802.org/21/> |  |
| --- | --- | --- |
| Title | MIH-Configure updates |  |
| Date Submitted | 3/14/07 |  |
| Source(s) | Srinivas Sreemanthula |  |
| Ref: |  |  |
| Abstract | This contribution provides detailed resolutions for the comment |  |
| Purpose | Discuss and adopt in the draft. |  |
| Notice | This document has been prepared to assist the IEEE 802.21 Working Group. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s). The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.21. |  |
| Patent Policy | The contributor is familiar with IEEE patent policy, as outlined in Section 6.3 of the IEEE-SA Standards Board Operations Manual <http://standards.ieee.org/guides/opman/sect6.html#6.3> and in Understanding Patent Issues During IEEE Standards Development <http://standards.ieee.org/board/pat/guide.html>. |  |

7.6.17 MIH_Configure_Link

The MIH_Configure_Link may be issued by upper layer entities to control the behavior of link layers. When the MIHF receives MIH_Configure_Link, it may issue the Link_Configure_Thresholds or other commands to configure or manage the corresponding link.

7.6.17.1 MIH_Configure_Link.request

7.6.17.1.1 Function

This primitive is issued by MIH Users to control the behavior of the link layer.

7.6.17.1.2 Semantics of the service primitive

The parameters of the primitive are as follows:

MIH_Configure_Link.request (

DestinationIdentifier,

LinkIdentifier,

LinkAction,

QoSParameterConfiguration

)

Local or Remote: Both

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| Destination Identifier | Identifier | Any valid individual or group identifier | Local or remote destination identifier |
| Link Identifier | Link ID | N/A | Identifier of the link |
| Link Action | Enumerated | 0-255 | Link Action: 0: POWER-UP 1: POWER-DOWN 2: POWER-SAVE 3-255: reserved |
| QoS Parameter Configuration | LIST | N/A | A list of QoS Parameters and their corresponding values as specified in Table 13. |
| QoS Parameter Configuration | LIST | N/A | A list of QoS Parameters and their corresponding values as specified in Table 13. |

|  |  |  | A list of QoS Parameters and their corresponding values as specified in Table 13. |
| --- | --- | --- | --- |

7.6.17.1.3 When generated

This primitive is generated when MIH Users attempt to control the behavior of the lower layer links,.

7.6.17.1.4 Effect of receipt

A local MIH configure command causes the MIHF to issue either the indicated action on the link layer or a Link Configure Threshold command to set the

thresholds for the lower layers according to the specified configuration parameters. A MIH remote configure

command causes the MIHF to create a MIH message and send to the destination node identified in the destination identifier.

7.6.17.2 MIH_Configure_Link.indication

7.6.17.2.1 Function

This primitive is issued by MIHF when a remote MIH Configure request message is received.

7.6.17.2.2 Semantics of the service primitive

The parameters of the primitive are as follows:

MIH_Configure_Link.indication (

SourceIdentifier,

LinkIdentifier,

LinkAction,

QoSParameterConfiguration

)

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| Source Identifier | Identifier | Any valid individual or group identifier | Local or remote identifier |
| Link Identifier | Link ID | N/A | Identifier of the link |
| Link Action | Enumerated | 0-255 | Link Action: 0: POWER-UP 1: POWER-DOWN 2: POWER-SAVE 3-255: reserved |
| QoS Parameter Configuration | LIST | N/A | A list of QoS Parameters and their corresponding values as specified in Table 13. |

7.6.17.2.3 When generated

This primitive is generated when MIHF receives a corresponding request message from a peer entity.

7.6.17.2.4 Effect of receipt

MIH users can issue local MIH configure commands to take the appropriate link action or set the thresholds for QoS parameter reporting.

7.6.17.3 MIH_Configure_Link.response

7.6.17.3.1 Function

This primitive is issued by the MIH users to report the result of the MIH_Configure_Link request.

7.6.17.3.2 Semantics of the service primitive

The parameters of the primitive are as follows:

MIH_Configure_Link.response (

DestinationIdentifier,

LinkIdentifier,

LinkActionStatus

ConfigurationResponseList

)

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| Source Identifier | Identifier | Any valid individual or group identifier | Local or remote destination identifier |
| LinkActionStatus | Enumerated | Success/Error | Status of LinkAction |
| Configuration Response List | LIST | N/A | List of Configuration Response |

Table 41—Elements of Configuration Response

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| Configuration Parameter Type | Integer | 0-255 | Parameter for which configuration has been requested. |
| QoSParameterStatus | Enumerated | Success/Error | Status of QoS Parameter setting |

7.6.17.3.3 When generated

This primitive is generated when the MIH users receive a confirmation due to a local MIH configure request with the results.

7.6.17.3.4 Effect of receipt

Upon receipt the MIHF generates a corresponding MIH packet towards the destination identifier.

7.6.17.4 MIH_Configure_Link.confirm

7.6.17.4.1 Function

This primitive is issued by the MIH entity to report the result of the MIH_Configure_Link request to the MIH users.

7.6.17.4.2 Semantics of the service primitive

The parameters of the primitive are as follows:

MIH_Configure_Link.confirm (

SourceIdentifier,

LinkIdentifier,

LinkActionStatus,

ConfigurationResponseList

)

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| Source Identifier | Identifier | Any valid individual or group identifier | Local or remote destination identifier |
| LinkActionStatus | Enumerated | Success/Error | Status of LinkAction |
| Configuration Response List | LIST | N/A | List of Configuration Response |

Table 41—Elements of Configuration Response

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| Configuration Parameter Type | Integer | 0-255 | Parameter for which configuration has been requested. |
| QoSParameterStatus | Enumerated | Success/Error | Status of QoS Parameter setting |

7.6.17.4.3 When generated

This primitive is generated when the MIHF receives the results of MIH Configure request locally or when the MIH Configure response is received from a remote entity.

7.6.17.4.4 Effect of receipt

Upon receipt of the result code, the MIH Users are aware of the results..