| Project | IEEE 802.21 Media Independent Handover Services <http://www.ieee802.org/21/> |  |  |
| --- | --- | --- | --- |
| Title | LB #1b Issue 19 |  |  |
| Date Submitted | March 12, 2007 |  |  |
| Source(s) | Ronny Kim, Jin Lee LG Electronics, Inc. 533,Hogye-1dong,Dongan-gu, Anyang-shi, Kyongki-do, Korea | Voice: +82-31-450-1856 Fax: +82-31-450-7912 [mailto: ronnykim, jins978@lge.com] |  |
| Re: | IEEE 802.21 Session #19 in Orlando |  |  |
| Abstract |  |  |  |
| Purpose | Proposed normative text for comment #4363 |  |  |
| Notice | This document has been prepared to assist the IEEE 802.21 Working Group. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s). The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein. |  |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.21. |  |  |
| Patent Policy | The contributor is familiar with IEEE patent policy, as outlined in Section 6.3 of the IEEE-SA Standards Board Operations Manual <http://standards.ieee.org/guides/opman/sect6.html#6.3> and in Understanding Patent Issues During IEEE Standards Development <http://standards.ieee.org/board/pat/guide.html>. |  |  |

- LB #1b Issue 19

- (MIH_Network_Address_Information)

[Insert in 7.6.20 MIH_MN_HO_Candidate_Query as following, page128, line24]

7.6.20 MIH_MN_HO_Candidate_Query

7.6.20.1 MIH_MN_HO_Candidate_Query.request

7.6.20.1.1 Function

This primitive is used by MIH users on a mobile node to inform MIHF to perform candidates query for a possible handover initiation and query if the QoS resources of the ongoing data sessions can be supported in the candidate networks. It also enables to request IP address related information in the candidate networks and included current IP address related information may help to have IP Address Information Status.MIHF may forward the message remotely to the peer MIHF in the network.

7.6.20.1.2 Semantics of service primitive

MIH_MN_HO_Candidate_Query.request (

…………………..

IP Configuration Method,

DHCP Server Address,

FA Address,

Access Router Address

)

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| ………………….. |  |  |  |
| IP Configuration Method | Bitmap | 0-31 | Current IP configuration method is included. Bit 0:IPv4 static configuration Bit 1:IPv4 dynamic configuration (DHCPv4) Bit 2:Mobile IPv4 with FA Bit 3: Mobile IPv4 without FA Bit 4-10:reserved for IPv4 address configuration Bit 11:IPv6 stateless address configuration Bit 12: IPv6 stateful address configuration Bit 13: IPv6 manual configuration Bit 14-31 : (Reserved) |
| DHCP Server Address | IP Address | N/A | IP address of current DHCP Server. It is only included when mobile node is using dynamic address configuration |
| FA Address | IP Address | N/A | IP address of current Foreign Agent. It is only included when mobile node is using Mobile IPv4 |
| Access Router Address | IP Address | N/A | IP address of current Access Router. It is only included when mobile node is using IPv6 |

7.6.20.1.3 When generated

This primitive is generated by an upper layer entity such as a Handover Policy engine, L3 Mobility protocol like MIP, application or some other entity in the mobile node that may want to query other candidate networks for a possible handover. This command may be sent either to just the local MIH Function or it may be sent to the remote MIH Function as well. MN uses the QueryResourceList parameter to notify the Serving PoS of the resource list which it is interested in checking at the candidate networks as part of this query. MN’s upper layer entity may generate this primitive when it wants to know IP address related information in candidate networks before handover.

7.6.20.2 MIH_MN_HO_Candidate_Query.indication

7.6.20.2.2 Semantics of service primitive

MIH_MN_HO_Candidate_Query.indication (

…………………..  ,

IP Configuration Method,

DHCP Server Address,

FA Address,

Access Router Address

)

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| ………………….. |  |  |  |
| IP Configuration Method | Bitmap | 0-31 | Current IP configuration method is included. Bit 0:IPv4 static configuration Bit 1:IPv4 dynamic configuration (DHCPv4) Bit 2:Mobile IPv4 with FA Bit 3: Mobile IPv4 without FA Bit 4-10:reserved for IPv4 address configuration Bit 11:IPv6 stateless address configuration Bit 12: IPv6 stateful address configuration Bit 13: IPv6 manual configuration Bit 14-31 : (Reserved) |
| DHCP Server Address | IP Address | N/A | IP address of current DHCP Server. It is only included when mobile node is using dynamic address configuration |
| FA Address | IP Address | N/A | IP address of current Foreign Agent. It is only included when mobile node is using Mobile IPv4 |
| Access Router Address | IP Address | N/A | IP address of current Access Router. It is only included when mobile node is using IPv6 |

7.6.20.2.4 Effect on receipt

The MIH user receiving this may generate a MIH_N2N_HO_Query_Resources.request primitives to get requested query resource list and/or IP address related information exchange MIH_N2N_HO_Query_Resources message with MIH Functions in one or more candidate networks under consideration before sending a corresponding response message.

7.6.20.3 MIH_MN_HO_Candidate_Query.response

7.6.20.3.1 Function

This primitive is used by MIH users to inform MIHF of the results of the candidate query request and IP address related information request. MIHF may forward the message to the remote MIHF on the mobile node.

7.6.20.3.2 Semantics of service primitive

MIH_MN_HO_Candidate_Query.response (

…………………..  ,

IP Configuration Method,

DHCP Server Address,

FA Address,

Access Router Address,

IP Address Information Status

)

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| ………………….. |  |  |  |
| IP Configuration Method | Bitmap | 0-31 | IP configuration Method on PreferredCandidatePoA List Bit 0:IPv4 static configuration Bit 1:IPv4 dynamic configuration (DHCPv4) Bit 2:Mobile IPv4 with FA Bit 3: Mobile IPv4 without FA Bit 4-10:reserved for IPv4 address configuration Bit 11:IPv6 stateless address configuration Bit 12: IPv6 stateful address configuration Bit 13: IPv6 manual configuration Bit 14-31 : (Reserved) |
| DHCP Server Address | IP Address | N/A | IP address of candidate DHCP Server. It is only included when dynamic address configuration is supported |
| FA Address | IP Address | N/A | IP address of candidate Foreign Agent. It is only included Mobile IPv4 is supported |
| Access Router Address | IP Address | N/A | IP address of candidate Access Router. It is only included IPv6 Stateless configuration is supported |
| IP Address Information Status | Bitmap | 0-8 | Bit 0 : IP configuration Method is not available Bit1 : DHCP Server address is not available Bit2 : FA address is not available Bit3 : Access Router Address is not available Bit4 : No information is provided due to accessibility of same entity (FA, Access Router, DHCP Server, etc) Bit5-7: Reserved |

7.6.20.3.4 Effect on receipt

On receiving the response the entity which originally initiated the candidate query request decides the suitability of the candidates based on the list,  and available resource, and achieves the IP address related information on each network.

7.6.20.4 MIH_MN_HO_Candidate_Query.confirm

7.6.20.4.1 Function

This primitive is used by MIHF to inform MIH Uusers of the receipt of candidate query and IP address related information response

7.6.20.4.2 Semantics of service primitive

MIH_MN_HO_Candidate_Query.confirm (

…………………..  ,

IP Configuration Method,

DHCP Server Address,

FA Address,

Access Router Address,

IP Address Information Status

)

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| ………………….. |  |  |  |
| IP Configuration Method | Bitmap | 0-31 | IP configuration Method on PreferredCandidatePoA List Bit 0:IPv4 static configuration Bit 1:IPv4 dynamic configuration (DHCPv4) Bit 2:Mobile IPv4 with FA Bit 3: Mobile IPv4 without FA Bit 4-10:reserved for IPv4 address configuration Bit 11:IPv6 stateless address configuration Bit 12: IPv6 stateful address configuration Bit 13: IPv6 manual configuration Bit 14-31 : (Reserved) |
| DHCP Server Address | IP Address | N/A | IP address of candidate DHCP Server. It is only included when dynamic address configuration is supported |
| FA Address | IP Address | N/A | IP address of candidate Foreign Agent. It is only included Mobile IPv4 is supported |
| Access Router Address | IP Address | N/A | IP address of candidate Access Router. It is only included IPv6 Stateless configuration is supported |
| IP Address Information Status | Bitmap | 0-8 | Bit 0 : IP configuration Method is not available Bit1 : DHCP Server address is not available Bit2 : FA address is not available Bit3 : Access Router Address is not available Bit4 : No information is provided due to accessibility of same entity (FA, Access Router, DHCP Server, etc) Bit5-7: Reserved |

7.6.20.4.4 Effect on receipt

On receiving the primitive the MIH user entity which originally initiated the candidate query request may decide to choose the candidate network for handover or abort it based on the information in the primitive and trigger the IP mobility protocol which can be used in the target PoS to initiate the process to get IP address to be used in the target. If the target network is chosen, it may then provide information about target link identifier from one of the candidate networks with an MIH_MN_HO_Candidate_Commit message.

[Insert in 7.6.21 MIH_N2N_HO _Query_Resouces as following, page132, line54]

7.6.21 MIH_N2N_HO_ Query_Resources

7.6.21.1 MIH_N2N_HO_ Query_Resources.request

7.6.21.1.1 Function

This primitive is used by an MIH function on the serving network to communicate with its peer MIH function on the candidate network. This is used to prepare the new link resource for impending handover , and query the available link resource and IP  address related information of the candidate network.

7.6.21.1.2 Semantics of service primitive

MIH_N2N_HO_ Query_Resources.request (

…………………..  ,

IP Configuration Method,

DHCP Server Address,

FA Address,

Access Router Address

)

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| ………………….. |  |  |  |
| IP Configuration Method | Bitmap | 0-31 | Current IP configuration method is included. Bit 0:IPv4 static configuration Bit 1:IPv4 dynamic configuration (DHCPv4) Bit 2:Mobile IPv4 with FA Bit 3: Mobile IPv4 without FA Bit 4-10:reserved for IPv4 address configuration Bit 11:IPv6 stateless address configuration Bit 12: IPv6 stateful address configuration Bit 13: IPv6 manual configuration Bit 14-31 : (Reserved) |
| DHCP Server Address | IP Address | N/A | IP address of current DHCP Server. It is only included when mobile node is using dynamic address configuration |
| FA Address | IP Address | N/A | IP address of current Foreign Agent. It is only included when mobile node is using Mobile IPv4 |
| Access Router Address | IP Address | N/A | IP address of current Access Router. It is only included when mobile node is using IPv6 |

7.6.21.2 MIH_N2N_HO_ Query_Resources.indication

7.6.21.2.1 Function

The MIH Function on the candidate network indicates that an MIH_N2N_HO_Query_Resources.request message is received from a peer MIH Function on the serving network so that the upper layer entity can identify the link resource usage, and prepare resources and provide IP address related information for the impending handover.

7.6.21.2.2 Semantics of service primitive

MIH_N2N_HO_ Query_Resources.indication (

…………………..  ,

IP Configuration Method,

DHCP Server Address,

FA Address,

Access Router Address

)

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| ………………….. |  |  |  |
| IP Configuration Method | Bitmap | 0-31 | Current IP configuration method is included. Bit 0:IPv4 static configuration Bit 1:IPv4 dynamic configuration (DHCPv4) Bit 2:Mobile IPv4 with FA Bit 3: Mobile IPv4 without FA Bit 4-10:reserved for IPv4 address configuration Bit 11:IPv6 stateless address configuration Bit 12: IPv6 stateful address configuration Bit 13: IPv6 manual configuration Bit 14-31 : (Reserved) |
| DHCP Server Address | IP Address | N/A | IP address of current DHCP Server. It is only included when mobile node is using dynamic address configuration |
| FA Address | IP Address | N/A | IP address of current Foreign Agent. It is only included when mobile node is using Mobile IPv4 |
| Access Router Address | IP Address | N/A | IP address of current Access Router. It is only included when mobile node is using IPv6 |

7.6.21.3 MIH_N2N_HO_ Query_Resources.response

7.6.21.3.1 Function

This primitive is used by an MIH function on the candidate network to communicate with its peer MIH function on the serving network with sent out an MIH_N2N_HO_Query_Resources.request. This is used to respond with the result of any resource preparation for the impending handover and to notify the MIH function on the serving network of the link resource status of the candidate network. It is also used to provide IP address related information of the candidate networks.

7.6.21.3.2 Semantics of service primitive

MIH_N2N_HO_Query_Resources.response (

…………………..  ,

IP Configuration Method,

DHCP Server Address,

FA Address,

Access Router Address,

IP Address Information Status

)

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| ………………….. |  |  |  |
| IP Configuration Method | Bitmap | 0-31 | IP configuration Method on PreferredCandidatePoA List Bit 0:IPv4 static configuration Bit 1:IPv4 dynamic configuration (DHCPv4) Bit 2:Mobile IPv4 with FA Bit 3: Mobile IPv4 without FA Bit 4-10:reserved for IPv4 address configuration Bit 11:IPv6 stateless address configuration Bit 12: IPv6 stateful address configuration Bit 13: IPv6 manual configuration Bit 14-31 : (Reserved) |
| DHCP Server Address | IP Address | N/A | IP address of candidate DHCP Server. It is only included when dynamic address configuration is supported |
| FA Address | IP Address | N/A | IP address of candidate Foreign Agent. It is only included Mobile IPv4 is supported |
| Access Router Address | IP Address | N/A | IP address of candidate Access Router. It is only included IPv6 Stateless configuration is supported |
| IP Address Information Status | Bitmap | 0-8 | Bit 0 : IP configuration Method is not available Bit1 : DHCP Server address is not available Bit2 : FA address is not available Bit3 : Access Router Address is not available Bit4 : No information is provided due to accessibility of same entity (FA, Access Router, DHCP Server, etc) Bit5-7: Reserved |

7.6.21.4 MIH_N2N_HO_ Query_Resources.confirm

7.6.21.4.1 Function

This primitive is generated by the MIH Function on the serving network to respond with the result of any resource preparation for the impending handover and to notify the link resource status of the candidate network. It also carries IP address related information on the candidate networks to MIH users on the serving network.

7.6.21.4.2 Semantics of service primitive

MIH_N2N_HO_Query_Resources.confirm (

…………………..  ,

IP Configuration Method,

DHCP Server Address,

FA Address,

Access Router Address,

IP Address Information Status

)

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| ………………….. |  |  |  |
| IP Configuration Method | Bitmap | 0-31 | IP configuration Method on PreferredCandidatePoA List Bit 0:IPv4 static configuration Bit 1:IPv4 dynamic configuration (DHCPv4) Bit 2:Mobile IPv4 with FA Bit 3: Mobile IPv4 without FA Bit 4-10:reserved for IPv4 address configuration Bit 11:IPv6 stateless address configuration Bit 12: IPv6 stateful address configuration Bit 13: IPv6 manual configuration Bit 14-31 : (Reserved) |
| DHCP Server Address | IP Address | N/A | IP address of candidate DHCP Server. It is only included when dynamic address configuration is supported |
| FA Address | IP Address | N/A | IP address of candidate Foreign Agent. It is only included Mobile IPv4 is supported |
| Access Router Address | IP Address | N/A | IP address of candidate Access Router. It is only included IPv6 Stateless configuration is supported |
| IP Address Information Status | Bitmap | 0-8 | Bit 0 : IP configuration Method is not available Bit1 : DHCP Server address is not available Bit2 : FA address is not available Bit3 : Access Router Address is not available Bit4 : No information is provided due to accessibility of same entity (FA, Access Router, DHCP Server, etc) Bit5-7: Reserved |

[Insert in 8.5.3.11 MIH_N2N_HO_Candidate_Query request as following, page214, line12]

8.5.3.11 MIH_N2N_HO_Candidate_Query request

MIH Header Fixed Fields (SID=3, Opcode=1, AID=19)

Source Identifier = sending MIHF ID

Destination Identifier = receiving MIHF ID

CurrentLinkIdentifier TLV(14)

CandidateLinkList TLV(??)

QueryResourceListTLV(??)

CandidatePoAList TLV(??)

IPConfigurationMethods TLV(17)

Serving FA Address / Serving Access Router Address / DHCP Server Address TLV(11)

8.5.3.12 MIH_MN_HO_Candidate_Query response

MIH Header Fixed Fields (SID=3, Opcode=2, AID=19)

Source Identifier = sending MIHF ID

Destination Identifier = receiving MIHF ID

CurrentLinkIdentifier TLV(14)

PreferredCandidateLinkList TLV(??)

AvailableResourceSetTLV(??)

PreferredCandidatePoAList TLV(??)

IPConfigurationMethods TLV(17)

Serving FA Address / Serving Access Router Address / DHCP Server Address TLV(11)

IP Address Information Status TLV(41)

8.5.3.13 MIH_N2N_HO_Query_Resources request

This message is used by the MIHF on the serving network to communicate with its peer MIHF on the candidate network. This is used to prepare the new link resource for impending handover and to query the available link resource of the candidate network. This message is also used to achieve IP address related information on the candidate network.

MIH Header Fixed Fields (SID=3, Opcode=1, AID=20)

Source Identifier = sending MIHF ID

Destination Identifier = receiving MIHF ID

CurrentLinkIdentifier TLV(14)

QueryResourceList TLV(34)

IPConfigurationMethods TLV(17)

Serving FA Address / Serving Access Router Address / DHCP Server Address TLV(11)

8.5.3.14 MIH_N2N_HO_Query_Resources response

This message is used by an MIHF on the candidate network to communicate with its peer MIHF on the serving network. This is used to return the result of resource preparation of the impending handover and to notify the MIHF on the serving network of the link resource status and IP address related information of the candidate network.

MIH Header Fixed Fields (SID=3, Opcode=2, AID=20)

Source Identifier = sending MIHF ID

Destination Identifier = receiving MIHF ID

CurrentLinkIdentifier TLV(14)

ResourceStatus TLV(35)

QueryResourceList TLV(34)

IPConfigurationMethods TLV(17)

Serving FA Address / Serving Access Router Address / DHCP Server Address TLV(11)

IP Address Information Status TLV(41)

[Delete 7.6.26 MIH_Network_Address_Information, page 152-158]

[Delete 8.5.3.23 MIH_Network_Address_Information request, page217,line20]

[Delete 8.5.3.24 MIH_Network_Address_Information responset, page217,line47]