IEEE 802.21
Media Independent Handover Services

Proposal to put the MIHF ID in the Fixed Header of MIH Messages

Date:	February 15, 2007

Author:	Ulises Olvera and Juan Carlos Zuniga
	InterDigital Communications

e-Mail:

Abstract

The current IEEE 802.21 document specifies that “MIHF ID is used during MIH registration and all messages that are required to identify the end points that are currently not in a session”. The current draft standard specifies the use of a TLV to send the MIHF ID when needed.

The MIHF ID is needed to uniquely identify an MIH enabled node or the source of an MIH message. This makes the identification independent of the underlying transport protocol or IP address. Thus it is seen important that an MIH capable node inserts this identifier in every MIH message.

Moreover, there is a long list of MIH messages that already include the MIHF ID of the sender of the message. The list includes the following (section 8.5, MIH protocol messages)

- MIH Capability Discover

- MIH Register

- MIH DeRegister

- MIH Event Subscribe

- MIH Event Unsubscribe

- MIH Link Up

- MIH Link Down

- MIH Link Going Down

- MIH Link Detected

- MIH Link Parameters Report

- MIH Link Event Rollback

- MIH Link Handover Imminent

- MIH Link Handover Complete

- MIH Get Status

- MIH Switch

- MIH Configure Link

- MIH Scan

- MIH Net HO Candidate Query

- MIH MN HO Candidate Query

- MIH N2N HO Query Resource

- MIH Net NO Candidate Commit

- MIH MN HO Candidate Commit

- MIH MN HO Complete

- MIH N2N HO Complete

- MIH Network Address Information

- MIH Get Information

Proposal

This submission proposes to have the MIHF ID as a constant field in the MIH Fixed Header of all MIH messages for the following reasons:

- It is needed to uniquely identify the sender of an MIH message

- Most MIH messages already have it as a parameter (see section 8.5)

- If inserted in a TLV, it takes more space than actually needed