| Project | IEEE 802.21 MIHO <http://www.ieee802.org/21/> |  |
| --- | --- | --- |
| Title | Link Event Indication Nits |  |
| Date Submitted | May 15, 2007 |  |
| Source(s) | Yoshihiro Ohba |  |
| Re: | IEEE 802.21 Session #20 in May 2007 |  |
| Abstract | This document proposes clean up description on link event indication primitives. |  |
| Purpose | LB#1d comment resolution |  |
| Notice | This document has been prepared to assist the IEEE 802.21 Working Group. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s). The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.21. |  |
| Patent Policy | The contributor is familiar with IEEE patent policy, as outlined in Section 6.3 of the IEEE-SA Standards Board Operations Manual <http://standards.ieee.org/guides/opman/sect6.html#6.3> and in Understanding Patent Issues During IEEE Standards Development <http://standards.ieee.org/board/pat/guide.html>. |  |

# Issues on “Effect on receipt” part of Link_*.indication primitives (comment

- The fact that the MIHF may also propagate the notification to a remote MIHF is not captured (it should be captured)

  - This issue is well covered by comments 5254,5255,5256,5257 and 5261 and no additional remedy is needed.

- There are many implementation-specific MIH User behavior defined (it is best to leave implementation specific behavior open to implementations.)

  - At least Section 7.3.4.4 may need to be revised.

# Suggest Remedy

Change Section 7.3.4.4 as follows:

7.3.4.4 Effect on receipt

The MIHF normally passes this link notification to the MIH User(s) which has subscribed for this notification in an MIH_Link_Up event. The MIH User(s) may take different actions on this notification. The handover policy function may start collecting routing matrices for this link and may consider this link as a potential candidate for establishing future connections. When higher layer connections are being established over this link, the Transport layer may adjust transport (TCP) related parameters based on link properties.