| Project | IEEE 802.21 Media Independent Handover <http://www.ieee802.org/21/> |  |
| --- | --- | --- |
| Title | Updates to section 7 |  |
| Date Submitted | May , 2007 |  |
| Source(s) | Vivek Gupta |  |
| Re: | IEEE 802.21 Session #20 in May 2007 |  |
| Abstract | Updates to section 7 |  |
| Purpose | LB#1d comment resolution |  |
| Notice | This document has been prepared to assist the IEEE 802.21 Working Group. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s). The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.21. |  |
| Patent Policy | The contributor is familiar with IEEE patent policy, as outlined in Section 6.3 of the IEEE-SA Standards Board Operations Manual <http://standards.ieee.org/guides/opman/sect6.html#6.3> and in Understanding Patent Issues During IEEE Standards Development <http://standards.ieee.org/board/pat/guide.html>. |  |

- Service access points (SAPs) and primitives

  - Introduction

The MIH Function uses the following media dependent and media independent SAPs for interfacing with other entities.

    - Media dependent SAPs

- MIH_LINK_SAP: Abstract media-dependent interface of MIHF with the lower layers of the media specific protocol stacks. The MIH_LINK_SAP maps to the following technology specific SAPs:

- MLME_SAP: The MIHF uses this SAP for interfacing with the link layer of IEEE 802.11 networks. The MIH_LINK_SAP defines additional primitives that map to MLME_SAP. These primitives are recommended as enhancements to IEEE 802.11 link layer SAPs. MLME_SAP is defined by IEEE 802.11 and it may include, but is not limited to primitives related to:

- System configuration

- Link state change notifications/triggers

- MIH frame transport through control or management frames

- C_SAP, M_SAP: The MIHF uses these SAPs for interfacing with the Control and Management planes of the IEEE 802.16 network. 
		
		C_SAP is defined by IEEE 802.16g and it may include, but is not limited to primitives related to:

- Handovers (e.g., notification of HO request from mobile station (MS))

- Idle mode mobility management (e.g., Mobile entering idle mode)

- Subscriber and session management (e.g., Mobile requesting session setup)

- Radio resource management

- Authentication, Authorization, and Accounting (AAA) server signaling (e.g., EAP payloads)

- Media Independent Function Services

- The primitives defined as part of C_SAP are described in Table  , “ ,”  on page  Error: Reference source not found.
		
		M_SAP is defined by IEEE 802.16g and it may include, but is not limited to primitives related to:

- System configuration

- Monitoring Statistics

- Notifications Triggers

- Multi-mode interface management

- MIH_3GLINK_SAP: This SAP defines the interface between the MIHF and the different protocol elements of the 3G system. 
		
		3GPP and 3GPP2 service primitives for GSM, UMTS, LAC, PPP and LTE system architecture evolution (SAE) may be used to access IEEE 802.21 services. This is done by establishing a relationship between the 3GPP/3GPP2 primitives and IEEE 802.21 primitives.
		
		Table   lists this mapping. Note that a 3GPP primitive group may be mapped to more than one IEEE 802.21 primitive. For example, the “SMGREG-PDP-MODIFY” primitive is mapped to the Link_Up primitive and it could also indicate the successful completion of handover and is hence also mapped to the MIH_Handover_Complete primitive.

- LSAP: defined in the IEEE 802.2 standard, provides the interface between the MIHF and the Logical Link Control sublayer in IEEE 802.3 and IEEE 802.11 networks. This SAP is used for local MIH exchanges between the MIHF and the lower layers of the IEEE 802.3 interface (as the IEEE 802.3 instantiation of the MIH_LINK_SAP) and for the L2 transport of MIH messages across IEEE 802.3 and IEEE 802.11access links.

- CS_SAP: defined in the IEEE 802.16 standard, provides the interface between the MIHF and the service-specific Convergence Sublayer in IEEE 802.16 networks. This SAP is used for the L2 transport of MIH messages across IEEE 802.16 access links.

- MIH_NET_SAP: Abstract media-dependent interface of MIHF which provides transport services over the data plane on the local node, supporting the exchange of MIH information and messages with the remote MIHF.

    - Media independent SAPs

- MIH_SAP: This SAP defines the media independent interface between the MIHF and MIH Users.

- MIH_NMS_SAP: This SAP defines the media independent interface between the MIHF and the Network Management System.

  - SAPs

This section describes the primitives of all the SAPs that are used by the MIH Function.

    - SAP primitives and parameter types

The SAPs are defined as a set of primitives. Taken together, the primitives define the services. Within the definition of each primitive there is a table of allowable parameters. Each parameter is defined using abstract data types. These types indicate the semantic value of that parameter. The parameters defined within the clause for a particular primitive are produced or consumed by that primitive. Several of the abstract data types are used in multiple primitive definitions. In each abstract data type definition, the various names applied to this type are listed.

Most of the primitives have corresponding MIH protocol messages. In those messages, there are TLV encoded parameters that implement the primitive parameter abstract data types within the protocol. The definition of the full binary encoding for each of these instantiations is in Clause .

- Primitive parameter abstract data types

| Type Name | Definition |
| --- | --- |
| BITMAP(size) | A bitmap of the specified size. Usually used to represent a list of IDs. |
| ENUMERATED | Enumeration of a list of allowable values. |
| BOOLEAN | Enumeration of two values. |
| LIST | A list of elements of same structure. |
| MAC ADDRESS | A layer 2 address of a node. |
| LINK ID | A identifier of a link as defined by IEEE 802.21. Consisting of: 1) link type, and 2) MAC address of MN, |
| CONNECTION ID | A identifier of a connection as defined by IEEE 802.21. Consisting of: 1) LINK ID, and 2) MAC address of a PoA. |
| MIHF ID | An MIHF Identifier (see definition in ). |
| IP ADDRESS | An IPv4 or IPv6 address. |
| TRANSPORT ADDRESS | A protocol layer specific transport address. |

    - Media dependent SAPs

      - MIH_LINK_SAP

The primitives defined as part of the MIH_LINK_SAP are described in Table   and their mapping to specific link technology primitives are listed in Tables  -.

- MIH_LINK_SAP primitives

| Primitives | Service category | Description |
| --- | --- | --- |
| Link_Event_Subscribe | Command | Subscribe for event notifications |
| Link_Event_Unsubscribe | Command | Unsubscribe from event notifications |
| Link_Configure_Thresholds | Command | Configure link thresholds for Link events |
| Link_Up | Event | L2 connectivity is established |
| Link_Down | Event | L2 connectivity is lost |
| Link_Going_Down | Event | L2 connectivity loss is imminent |
| Link_Event_Rollback | Event | Predicted event has not occurred and hence event indication must be rolled back. |
| Link_Detected | Event | A new link is detected |
| Link_Parameters_Report | Event | Link parameters have crossed specified thresholds |
| Link_PDU_Transmit_Status | Event | Indicate transmission status of a PDU |
| Link_Handover_Imminent | Event | L2 handover is imminent |
| Link_Handover_Complete | Event | L2 handover has been completed |
| Link_Capability_Discover | Command | Query and discover the list of supported link layer events and link layer commands. |
| Link_Get_Parameters | Command | Request parameters of air interface |
| Link_Action | Command | Request actions on a link layer connection |

- MIH_LINK_SAP/802.16 primitives mapping

| MIH_LINK_SAP Primitive | IEEE 802.16 C_SAP | IEEE 802.16 M_SAP |
| --- | --- | --- |
| Link_Event_Subscribe | N/A | N/A |
| Link_Event_Unsubscribe | N/A | N/A |
| Link_Configure_Thresholds | C-HO-REQ/RSP(HO-Scan) | N/A |
| Link_Up | C-NEM-RSP(Registration) | N/A |
| Link_Down | N/A | M-MTM-RSP(Deregistration) |
| Link_Going_Down | N/A | N/A |
| Link_Event_Rollback | N/A | N/A |
| Link_Detected | C-HO-RSP(HO-Scan) | N/A |
| Link_Parameters_Report | C-HO-IND(HO-Scan) C-HO-RSP(HO-Scan) C-RRM-RSP C-SFM-RSP | N/A |
| Link_PDU_Transmit_Status | N/A | N/A |
| Link_Handover_Imminent | C-HO-IND(HO-Start) | N/A |
| Link_Handover_Complete | C-HO-IND(HO-CMPLT) | N/A |
| Link_Capability_Discover | TBD | TBD |
| Link_Get_Parameters | C-SFM-REQ/RSP C-HO-REQ/RSP/IND (HO-Scan) C-RRM-REQ/RSP | N/A |
| Link_Action | TBD | TBD |

- MIH_LINK_SAP/802.11/802.3ah/802.1ag primitives mapping

| Primitives | IEEE 802.11 | IEEE 802.3ah | IEEE 802.1ag |
| --- | --- | --- | --- |
| Link_Event_Subscribe | N/A | N/A | N/A |
| Link_Event_Unsubscrib e | N/A | N/A | N/A |
| Link_Configure_Thresh olds | MLME-MEASURE.request MLME-MREQUEST.request | N/A | N/A |
| Link_Up | MLME-LinkUp.indication | Link Up | Dot1agCfmCCheckRestor edEvent |
| Link_Down | MLME-LinkDown.indication | Link fault | Dot1agCfgCCheckLossE vent |
| Link_Going_Down | MLME-LinkGoingDown.indi cation | Dying Gasp | N/A |
| Link_Event_Rollback | MLME-EventRollback.indica tion | N/A | N/A |
| Link_Detected | TBD | TBD | TBD |
| Link_Parameters_Report | MLME-MEASURE.confirm MLME-MREPORT.indica tion | N/A | N/A |
| Link_PDU_Transmit_St atus | MA-UNIDATA-STA TUS.indication | N/A | N/A |
| Link_Handover_Immine nt | MLME-LinkHandoverImmi nent.indication | N/A | N/A |
| Link_Handover_Comple te | MLME-LinkHandoverCom plete.indication | N/A | N/A |
| Link_Capability_Discov er | TBD | TBD | TBD |
| Link_Get_Parameters | TBD | TBD | TBD |
| Link_Action | TBD | TBD | TBD |

- MIH_LINK_SAP/3GPP/3GPP2 primitives mapping

| Primitives | 3GPP | 3GPP2 |
| --- | --- | --- |
| Link_Event_Subscribe | N/A | N/A |
| Link_Event_Unsubscribe | N/A | N/A |
| Link_Configure_Thresholds | SMREG-PDP-MODIFY | L2.Suppervision.Request |
| Link_Up | SMSM-ACTIVE RABMSM-ACTIVATE | L2.Condition.Notification LCP-Link-Open LCP-Link-Up IPCP-Link-Open |
| Link_Down | SMSM-DEACTIVEATE SMSM-STATUS RABMSM-DEACTIVATE RABMSM-STATUS RABMAS-RAB-RELEASE | LCP-Carrier-Failure LCP-Link-Quality-Failure LCP-Timeout IPCP-Link-Closed IPCP-Config-Failure IPCP-Timeout |
| Link_Going_Down | N/A | LCP-Closing |
| Link_Event_Rollback | N/A | N/A |
| Link_Detected | TBD | TBD |
| Link_Parameters_Report | SMSM-MODIFY RABMSM-MODIFY | N/A |
| Link_PDU_Transmit_Status | N/A | N/A |
| Link_Handover_Imminent | N/A | N/A |
| Link_Handover_Complete | RABMAS-RAB-ESTABLISH RABMSM-MODIFY | L2.Data.Confirm |
| Link_Capability_Discover | TBD | TBD |
| Link_Get_Parameters | TBD | TBD |
| Link_Action | TBD | TBD |

- MIH_NET_SAP

The following primitives are defined for MIH_NET_SAP.

- MIH_NET_SAP primitives

| Primitive | Service category | Description |
| --- | --- | --- |
| MIH_TP_Connect | Network Com munication | This primitive is used to set a connection oriented transport session |
| MIH_TP_Disconnect | Network Com munication | This primitive is used to disconnect a connection oriented transport session |
| MIH_TP_Reset | Network Com munication | This primitive is used to reset a connection oriented transport ses sion |
| MIH_TP_Data | Network Com munication | This primitive is used for transfer of data |

    - Media independent SAPs

      - MIH_SAP

The primitives defined as part of MIH_SAP are described in Table  .

- MIH_SAP primitives

| Primitives | Service category | Description |
| --- | --- | --- |
| MIH_Capability_Discover | Service Management | Discover list of Events and Commands supported by MIHF. |
| MIH_Register | Service Management | Register with a remote MIHF. |
| MIH_DeRegister | Service Management | Deregister with a remote MIHF. |
| MIH_Event_Subscribe | Service Management | Subscribe for MIH event notifications |
| MIH_Event_Unsubscribe | Service Management | Unsubscribe from MIH event notifications |
| MIH_Link_Up | Event | L2 connection has been established |
| MIH_Link_Down | Event | L2 connectivity is lost |
| MIH_Link_Going_Down | Event | L2 connectivity is predicted to go down |
| MIH_Link_Detected | Event | A new link is detected |
| MIH_Link_Parameters_Report | Event | Link parameters have crossed specified threshold |
| MIH_Link_Event_Rollback | Event | Predicted event has not occurred and hence event indication must be rolled back |
| MIH_Link_PDU_Transmit_Status | Event | Indicate transmission status of a PDU |
| MIH_Link_Handover_Imminent | Event | L2 handover is imminent |
| MIH_Link_Handover_Complete | Event | L2 handover has been completed |
| MIH_Get_Link_Parameters | Command | Get the status of link |
| MIH_Configure_Link | Command | Configure link parameters and parameter thresholds |
| MIH_Scan | Command | Scan the network |
| MIH_Net_HO_Candidate_Query | Command | Initiate handover |
| MIH_MN_HO_Candidate_Query | Command | Initiate MN query request for candidate network |
| MIH_N2N_HO_Query_Resources | Command | Prepare for handover and query available resources |
| MIH_Net_HO_Commit | Command | Network has committed to handover |
| MIH_MN_HO_Commit | Command | Mobile node has committed to handover |
| MIH_MN_HO_Complete | Command | Initiate MN handover complete notification |
| MIH_N2N_HO_Complete | Command | Handover has been completed |
| MIH_Get_Information | Information | Request to get information from repository |

MIH command primitives defined in MIH_SAP may indicate its destination as either the local MIHF or a remote peer MIHF. For the remote case, the local MIHF will first process the primitive to create an MIH message and then forward the message to the destination peer MIHF for execution.

        - Naming convention for MIH handover commands

Some of the MIH command primitives that are utilized during handover preparation and execution either for local or remote use are categorized as MIH handover command primitives. Generally, there are three types of MIH handover command primitives based on the functionality specified for the following scenarios: i) MN to network, ii) Network to MN, and iii) Network to Network. This classification helps to ensure the specification of right protocol functionality and the relevant parameters for specific use as determined by the origination and the destination points.

Accordingly, these commands have a convention that provides for identifying the origination point in the primitive name, as shown in Table  . This convention shall be followed by the MIHF to ensure that these commands are utilized for the intended purpose.

- Naming convention for MIH handover command primitives

| Primitive Name Prefix | Originating Point | Destination Point |
| --- | --- | --- |
| MIH_MN_HO_*** | Mobile Node | Network |
| MIH_Net_HO_*** | Network | Mobile Node |
| MIH_N2N_HO_*** | Network | Network |

      - MIH_NMS_SAP

The primitives defined as part of MIH_NMS_SAP are described in Table  .

- MIH_NMS_SAP primitives

| Primitives | Service Category | Description |
| --- | --- | --- |
| MIH_NMS_Initialize | Network Management | This primitive brings the MIHF to the initial state in the state transition. |
| MIH_NMS_Get_State | Network Management | Get state information of the MIHF indicating working status. |
| MIH_NMS_Reset | Network Management | During operation resets the MIHF parameters and brings it into the initial state. |

  - MIH_LINK_SAP primitives

    - Link_Event_Subscribe

      - Link_Event_Subscribe.request

        - Function

This primitive is used by MIHF (the subscriber) to subscribe an interest in one or more events  from a specific link layer technology particular event origination point. The response indicates which of the requested events were successfully subscribed to. Events  that were not successfully subscribed to will not be delivered to the subscriber.

        - Semantics of service primitive

The primitive parameter is as follows:

Link_Event_Subscribe.request			 	(

RequestedLinkEventList

)

| Name | Type | Valid range | Description |
| --- | --- | --- | --- |
| Requested Link Event List | BITMAP(32) | N/A | List of link layer events that the subscriber would like to receive indications for, from the Event Source The encoding uses a bitmap where the bit position represents the event ID as defined in Table . A bit set to 1 indicates that the MIH wishes to receive indication for that event. For example, if bit #0 is set to 1, the MIH requests to receive Link_Up indications. Bits corresponding to reserved event IDs must be set to 0. |

- Link event identifiers

| Event name | ID |
| --- | --- |
| Link_Up | 0 |
| Link_Down | 1 |
| Link_Going_Down | 2 |
| Link_Event_Rollback | 3 |
| Link_Detected | 4 |
| Link_Parameters_Report | 5 |
| Link_PDU_Transmit_Status | 6 |
| Link_Handover_Imminent | 7 |
| Link_Handover_Complete | 8 |
| (Reserved) | 9~31 |

        - When generated

This primitive is generated by a subscriber such as the MIHF that is seeking to receive event indications  from different link layer technologies.  a point in the protocol stacks terminating either end of the link.

        - Effect on receipt

The recipient responds immediately with Link_Event_Subscribe.confirm primitive.

      - Link_Event_Subscribe.confirm

        - Function

This primitive returns the result of the subscription request.

        - Semantics of service primitive

The primitive parameters are as follows:

Link_Event_Subscribe.confirm			 	(

ResponseLinkEventList,

Status

)

| Name | Type | Valid range | Description |
| --- | --- | --- | --- |
| Response Link Event List | BITMAP(32) | N/A | List of link layer events along with their subscription status. The encoding uses a bitmap where the bit position represents the event ID as defined in Table . A bit set to 1 indicates that the subscription for that event has been successful, otherwise either the subscription was unsuccessful or not requested. For example, if bit #0 is set to 1, the MIH requested to receive Link_Up indications and the subscription was successful. Bits corresponding to reserved event IDs must be set to 0. Mapping the bitmap used in the request and the one returned by this primitive allows quick detection of subscription failure. |
| Status | ENUMERATED | 0-4 | Status of operation: 0 Success 1 Unspecified Failure 2 Rejected 3 Authorization Failure 4 Network Error |

        - When generated

This primitive is generated in response to a Link_Event_Subscribe.request primitive.

        - Effect on receipt

The recipient may examine the returned Response_Link_Event_List and learn about the subscription status of different events. If Status does not indicate “Success”, the recipient shall ignore all other returned parameters and perform appropriate error handling.

    - Link_Event_Unsubscribe

      - Link_Event_Unsubscribe.request

        - Function

This primitive is used by the MIHF (the subscriber) to unsubscribe from a set of previously subscribed link layer events.

        - Semantics of service primitive

The primitive parameter is as follows:

Link_Event_Unsubscribe.request			 	(
				RequestedLinkEventList
				)

| Name | Type | Valid range | Description |
| --- | --- | --- | --- |
| Requested Link Event List | BITMAP(32) | N/A | List of link layer events for which indications need to be unsubscribed from the Event Source The encoding uses a bitmap where the bit position represents the event ID as defined in Table . A bit set to 1 indicates that the MIH wishes to unsubscribe from that event. For example, if bit #0 is set to 1, the MIH requests to stop receiving Link_Up indications. Bits corresponding to reserved event IDs must be set to 0. |

        - When generated

This primitive is generated by a subscriber such as the MIHF that is seeking to unsubscribe from an already subscribed set of events.

        - Effect on receipt

The recipient responds immediately with Link_Event_Unsubscribe.confirm primitive.

      - Link_Event_Unsubscribe.confirm

        - Function

This primitive returns the result of the request to unsubscribe from receiving link layer event notifications..

        - Semantics of service primitive

The primitive parameters are as follows:

Link_Event_Unsubscribe.confirm			 	(
				ResponseLinkEventList,
				Status
				)

| Name | Type | Valid range | Description |
| --- | --- | --- | --- |
| Response Link Event List | BITMAP(32) | N/A | List of link layer events along with their unsubscription status. The encoding uses a bitmap where the bit position represents the Link event ID (as defined in Table ). A bit set to 1 indicates that unsubscription succeeded. |
| Status | ENUMERATED | 0-4 | Status of operation: 0 Success 1 Unspecified Failure 2 Rejected 3 Authorization Failure 4 Network Error |

        - When generated

This primitive is generated in response to a Link_Event_Unsubscribe.request primitive.

        - Effect on receipt

The recipient may examine the Response_Link_Event_List and learn about the unsubscription status of different events. If Status does not indicate “Success”, the recipient shall ignore all other returned parameters and, perform appropriate error handling.

    - Link_Configure_Thresholds

      - Link_Configure_Thresholds.request

        - Function

This primitive is used by the MIHF to configure thresholds for the Link_Parameters_Report indication.

        - Semantics of service primitive

The primitive parameter is as follows:

Link_Configure_Thresholds.request			 	(

LinkConfigureParameterList

)

| Name | Type | Valid range | Description |
| --- | --- | --- | --- |
| Link Configure Parameter List | LIST | N/A | A list of Link Configure Parameter, as defined in Table : |

- Elements of link configure parameter

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| Parameter Type | INTEGER | 0-255 | Parameter for which thresholds may be set, as defined in Table . |
| Initiate Action Threshold | Threshold values are dependent on parameter for which they are being set. | N/A | Threshold value which may cause MIH Users to start “setup” type activ ities in response to actual parameter values crossing this threshold. |
| Rollback Action Threshold | Threshold values are dependent on parameter for which they are being set. | N/A | Threshold value which may cause MIH Users to cancel or rollback the setup type operation if the actual parameter values retreat to this threshold. |
| Execute Action Threshold | Threshold values are dependent on parameter for which they are being set. | N/A | Threshold value which may cause MIH Users to execute taking appro priate action if the actual parameter values cross this threshold. |

        - When generated

This primitive is generated by an MIHF that may need to set threshold values for different link parameters.

        - Effect on receipt

The recipient responds immediately with Link_Configure_Threshold.confirm primitive.

      - Link_Configure_Thresholds.confirm

        - Function

This primitive is sent in response to the Link_Configure_Threshold.request primitive. This primitive specifies the status of threshold configuration operation.

        - Semantics of service primitive

The primitive parameters are as follows:

Link_Configure_Thresholds.confirm (

LinkConfigureStatusList,

Status

)

| Name | Type | Valid range | Description |
| --- | --- | --- | --- |
| Link Configure Status List | LIST | N/A | A list of Link Configure Status, as defined in Table . |
| Status | ENUMERATED | 0-4 | Status of operation: 0 Success 1 Unspecified Failure 2 Rejected 3 Authorization Failure 4 Network Error |

- Elements of link configure status

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| Parameter Type | INTEGER | 0-255 | Parameter for which threshold configura tion was requested, as defined in Table . |
| Configuration Status | ENUMERATED | 0-1 | Status of configuration operation: 0 Success 1 Error |

        - When generated

This primitive is generated in response to the Link_Configure_Thresholds.request operation

        - Effect on receipt

The recipient prepares to receive Link_Parameters_Report indications on successful execution of this primitive. However, if Status does not indicate “Success”, the recipient shall ignore any other returned values and, instead, shall perform appropriate error handling.

    - Link_Up.indication

      - Function

This notification is delivered when a layer 2 connection is established on the specified link interface and when layer 3 mobility management protocol (L3MMP) and other MIH Users may send higher layer packets. All layer 2 activities in establishing the link connectivity are expected to be completed at this point of time.

      - Semantics of service primitive

The primitive parameters are as follows:

Link_Up.indication 			(
			LinkIdentifier,
			MacOldAccessRouter,
			MacNewAccessRouter,
			IPRenewalFlag,
			Mobility Management Support
			)

Parameters:

| Name | Type | Valid range | Description |
| --- | --- | --- | --- |
| Link Identifier | LINK ID | N/A | Identifier of the link associated with the event |
| MAC Old Access Router | MAC ADDRESS | N/A | (Optional) MAC Address of old Access Router |
| MAC New Access Router | MAC ADDRESS | N/A | MAC Address of new Access Router |
| IP Renewal Flag | BOOLEAN | N/A | (Optional) Indicates whether the MN shall change IP Address in the new PoA 0: Change Required 1: Change Not required |
| Mobility Management Support | BITMAP(8) | N/A | (Optional) Indicates the type of Mobility Management Protocol supported by the new PoA 0: Mobile IPv4 with FA (FA-CoA) 1: Mobile IPv4 without FA (Co-located CoA) 2: Mobile IPv6 3: Mobile IPv6 with DHCPv6 4-7: (Reserved) |

      - When generated

This notification is generated when a layer 2 connection is established for the specified link interface.

      - Effect on receipt

The MIHF normally passes this link notification to the MIH User(s) which has subscribed for this notification in an MIH_Link_Up event. The MIH User(s) may take different actions on this notification. The handover policy function may start collecting routing matrices for this link and may consider this link as a potential candidate for establishing future connections. When higher layer connections are being established over this link, the Transport layer may adjust transport (TCP) related parameters based on link properties.

    - Link_Down.indication

      - Function

This notification is delivered when a layer 2 connection is broken on the specified link and no more packets may be sent on the specified link

      - Semantics of service primitive

Link_Down.indication 			(
			LinkIdentifier,
			MacOldAccessRouter,
			ReasonCode
			)

Parameters:

| Name | Type | Valid range | Description |
| --- | --- | --- | --- |
| Link Identifier | LINK ID | N/A | Identifier of the link associated with the event |
| Mac Old Access Router | MAC ADDRESS | N/A | (Optional) MAC Address of old Access Router |
| Reason Code | ENUMERATED | 0-255 | Reason for why the link went down, as defined in Table . |

The reason code for Link_Down may be one or a combination of the following reasons.

- Link_Down reason code definition

| Reason code | Name | Description |
| --- | --- | --- |
| 0 | EXPLICIT DISCONNECT | The link is down because of explicit disconnect pro cedures initiated either by MN or network |
| 1 | PACKET TIMEOUT | The link is down because no acknowledgements were received for transmitted packets within the specified time limit. |
| 2 | NO RESOURCE | The link is down because there were no resources to maintain the connection |
| 3 | NO BROADCAST | The link is down because broadcast messages (con trol frames such as beacons) could not be received by MN |
| 4 | AUTHENTICATION FAILURE | Authentication failure |
| 5 | BILLING FAILURE | Billing failure |
| 6-127 | (Reserved) |  |
| 128-255 | Vendor specific reason codes | Vendors may specify their own specific reason codes in this range. |

      - When generated

This notification is generated when layer 2 connectivity is lost. Layer 2 connectivity may be lost explicitly in cases where the mobile node may initiate disassociate type procedures. In other cases the mobile node may infer loss of link connectivity due to successive time-outs for acknowledgements of retransmitted packets along with loss of reception of broadcast frames.

      - Effect on receipt

The MIHF normally passes this link notification to the MIH User(s) which has subscribed for this notification in an MIH_Link_Down event. The MIH User(s) may take different actions on this notification. The handover policy may eliminate this link from list of active links for routing connections and may consider handing over any potential active connections to other more suitable links.

    - Link_Going_Down.indication

      - Function

This notification is delivered when a Layer 2 connection is expected (predicted) to go down (Link_Down) within a certain time interval. Link_Going_Down event may be the indication to initiate handover procedures. Different applications may have different requirements with respect to the time interval required to anticipate in advance a Link_Going_Down and make adequate a priori preparation to handover a connection depending on Application Class. Hence MIH Users such as a L3 mobility protocol may actually configure the time interval in which a Link_Going_Down event may be delivered. This would allow the flexibility for the time interval to be in milliseconds, 100s of milliseconds or even seconds.

      - Semantics of service primitive

Link_Going_Down.indication 				(
				LinkIdentifier,
				TimeInterval,
				ConfidenceLevel,
				LinkGoingDownReason,
				UniqueEventIdentifier
				)

Parameters:

| Name | Type | Valid range | Description |
| --- | --- | --- | --- |
| Link Identifier | LINK ID | N/A | Identifier of the link associated with the event |
| Time Interval | INTEGER | 0-65535 | Time Interval (in milliseconds) in which the link is expected to go down. The link connectivity is expected to be available at least for time specified by TimeInterval |
| Confidence Level | INTEGER | 0-100 | The confidence level (%) for link to go down within the specified time interval |
| Link Going Down Reason | ENUMERATED | 0-255 | The reason for why the link is going to be down, as defined in Table . |
| Unique Event Identifier | INTEGER | 0 – 65535 | Used to uniquely identify the event. To be used in case of event rollback |

- Link_Going_Down reason code definition

| Reason code | Name | Description |
| --- | --- | --- |
| 0 | EXPLICIT DISCONNECT | The link is going to be down because explicit disconnect proce dures will be initiated either by MN or network. For example, when a BS has decided to shutdown for administrative reasons or an operator of the terminal has decided to execute a handover manually, a Link_Going_Down trigger may be sent to the MIHF. |
| 1 | LINK PARAMETER DEGRADING | The link is going to be down because broadcast messages (con trol frames such as beacons) could not be received by MN. |
| 2 | LOW POWER | The link is going to be down because the power level of the ter minal is low and the current link will not be maintained in such a low power level. Mobile terminals usually have limited battery supply, and when the battery level of the terminal is low, a termi nal may choose a link that has lower power consumption for handover according to the received Link_Going_Down triggers with the reason code RC_LOW_POWER. This will lengthen the usable time for the terminal |
| 3 | NO RESOURCE | The link is going to be down because there will be no resources to maintain the current connection. For example, a BS may have too many users and send Link_Going_Down indications to ter minals when the links with them can not be kept because of insufficient resources. An another example is that the users with higher priority may preempt the ones with lower priority when no more resources can be allocated in 3GPP, and this also may cause a Link_Going_Down indication with RC_NORESOURCE reason code. |
| 4-127 | (Reserved) | Reserved for IEEE 802.21 future use. |
| 128-255 | Vendor specific reason codes | Vendors may specify their own specific reason codes in this range. |

      - When generated

A Link_Going_Down event implies that a Link_Down is imminent within a certain time interval. If Link_Down is NOT received within specified time interval then actions due to previous Link_Going_Down may be rejected.

A 100% confidence would indicate certainty that the link is definitely going down within the specified time interval. For example a BS that may have decided to shut down for administrative reasons may send down a Link_Going_Down trigger with 100% confidence level. Predictions based on changes in link parameters such as signal strength values would have a lower confidence level. Different links would use implementation specific methods to predict future loss in link connectivity.

      - Effect on receipt

The MIHF receives this event from the link layer. The MIHF then passes this notification to the MIH User(s) which have subscribed for this notification. MIH User(s) may take different actions on this notification. MIH Users may prepare to initiate handovers based on the confidence level reported as part of the event.

    - Link_Event_Rollback.indication

      - Function

Link_Event_Rollback is used in conjunction with Link_Going_Down. In case of Link_Going_Down if the link is no longer expected to go down in the specified time interval, then a Link_Event_Rollback message is sent to the Event destination. The destination should disregard or rollback the changes associated with the event identifier in such cases.

      - Semantics of service primitive

The primitive parameters are as follows:

Link_Event_Rollback.indication 				(
				LinkIdentifier,
				UniqueEventIdentifier
				)

Parameters

| Name | Type | Valid range | Description |
| --- | --- | --- | --- |
| Link Identifier | LINK ID | N/A | Identifier of the link associated with the event |
| Unique Event Identifier | INTEGER | 0 – 65535 | Used to identify the event which needs to be rolled back |

      - When generated

This notification is generated when a predictive event expected to occur within a certain time interval is no longer expected to occur before the expiry of the indicated time interval.

      - Effect on receipt

The MIHF receives this event from the link layer. The MIHF then passes this notification to the MIH User(s) which have subscribed for this notification. The MIH User(s) may reevaluate the link under consideration by retrieving other link quality related parameters. Any handover procedures that may have already been initiated may need to be rescinded.

    - Link_Detected.indication

      - Function

Link_Detected indicates that a new type of link has been detected for use. This may imply that the mobile node is in the coverage area and may listen to a beacon, or that the mobile node may have received a response to a probe. Link_Detected does not guarantee that the mobile node would be able to establish connectivity with the detected link, but just that the mobile node may attempt to gain connectivity. MIH Users including the MIHF itself may evaluate additional properties of the link before attempting to establish a L2 connection with the link.

      - Semantics of service primitive

The primitive parameters are as follows:

Link_Detected.indication 			(
			LinkIdentifier,
			MIHCapabilityFlag
			)

Parameters

| Name | Type | Valid range | Description |
| --- | --- | --- | --- |
| Link Identifier | LINK ID | N/A | Identifier of the link associated with the event |
| MIH Capability Flag | BOOLEAN | N/A | 0: MIH Capability Not Supported 1: MIH Capability Supported |

:

      - When generated

This notification is generated when a new link type is detected. The detection may be based on a beacon or response to a probe.

      - Effect on receipt

The MIHF receives this event from the link layer. The MIHF then passes this notification to the MIH User(s) which have subscribed for this notification. The MIH User(s), including the MIHF itself, may discover additional properties of the link before selecting it for establishing connectivity.

    - Link_Parameters_Report.indication

      - Function

Link_Parameters_Report indicates changes in link parameters that have crossed specified threshold levels. This may include link layer parameters such as Speed of the link, QoS, or BER. The threshold level for each such parameter may have to be configured through a separate command to the link layer.

      - Semantics of service primitive

Link_Parameters_Report.indication	(
				LinkIdentifier,
				LinkParametersReportList
				)

Parameters

| Name | Type | Valid range | Description |
| --- | --- | --- | --- |
| Link Identifier | LINK ID | N/A | Identifier of the link associated with the event |
| Link Parameters Report List | LIST | N/A | A list of Link Parameter Report, as defined in Table . |

:

- Elements for link parameter report

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| Parameter Type | INTEGER | 0-255 | Parameter being reported, as defined in Table . |
| Current Value | Dependent on Parameter Type | N/A | The current value of the parameter. |
| Threshold Crossed | ENUMERATED | 0-2 | 0: Initiate Action Threshold crossed 1: Rollback Action Threshold crossed 2: Execute Action Threshold crossed |

The set of link layer parameters that could trigger this event is specified in .

      - When generated

This notification is generated either at a predefined regular interval determined by a user configurable timer or when a specified parameter of the currently active interface crosses a configured threshold.

      - Effect on receipt

The MIHF receives this event from the link layer. The MIHF then passes this notification to the MIH User(s) which have subscribed for this notification. The MIH User(s) may take different actions on this notification. If parameters related to link quality cross a certain threshold then that link may need to be evaluated for handing over current connections. The MIHF may collectively evaluate different parameters and give appropriate indications to higher layers regarding suitability of different links.

    - Link_PDU_Transmit_Status.indication

      - Function

Link_PDU_Transmit_Status indicates the transmission status of a higher layer PDU. A success status indicates that a higher layer packet (PDU) has been successfully delivered from the ARQ module of the link layer in the current node to the ARQ module in the link layer in the peer node. A higher layer intermediate buffer management entity could use this indication to flush higher layer packets stored in any intermediate buffers. A failure status indicates that at least one segment belonging to a higher layer packet (PDU) was not delivered successfully from the ARQ module of the link layer in the current node to the ARQ module in the link layer in the peer node.

      - Semantics of service primitive

The primitive parameters are as follows:

Link_PDU_Transmit_Status.indication 					(
					LinkIdentifier,
					PacketIdentifier,
					TransmissionStatus
					)

Parameters:

| Name | Type | Valid range | Description |
| --- | --- | --- | --- |
| Link Identifier | LINK ID | N/A | Identifier of the link associated with the event |
| Packet Identifier | INTEGER | 0-65535 | Identifier for higher layer data packet that was delivered successfully. |
| Transmission Status | ENUMERATED | 0-1 | 0: Success 1: Failure |

      - When generated

A success notification is generated when a higher layer PDU is successfully transmitted over the link. A failure notification is generated when any link layer segment corresponding to a higher layer PDU was not transmitted successfully.

      - Effect on receipt

The MIHF receives this event from the link layer. The MIHF then passes this notification to the MIH User(s) which have subscribed for this notification. The MIH User(s) may take different actions on this notification. A higher layer intermediate buffer management entity in MIH could use the success indication to flush higher layer packets stored in any intermediate buffers and a failure indication to retransmit higher layer packets stored in any intermediate buffers, especially if there are changes in the access network during handovers.

    - Link_Handover_Imminent.indication

      - Function

Link_Handover_Imminent is generated when a native link layer handover or switching decision has been made and its execution is imminent (as opposed to Link_Going_Down which only indicates that a link is losing connectivity due to a change in a certain link condition such as signal strength, but does not guarantee that an autonomous link switch-over has been decided by the link layer). It contains information about the new point of attachment of the mobile node. It also contains information about any application specific data that might be useful for the application running on the mobile node. Link_Handover_Imminent may be used by MIH Users (e.g., transport) to initiate handover specific adaptation and as an enabler for initiating effective make-before-break handovers thereby reducing packet loss due to link layer handovers. This is a link layer event that exists for intra-technology handovers defined in many media types (e.g., cellular, IEEE 802.16e).

      - Semantics of service primitive

The primitive parameters are as follows:

Link_Handover_Imminent.indication 					(
					LinkIdentifier,
					MacOldAccessRouter,
					MacNewAccessRouter
					)

Parameters:

| Name | Type | Valid range | Description |
| --- | --- | --- | --- |
| Link Identifier | LINK ID | N/A | Identifier of the link associated with the event |
| Mac Old Access Router | MAC ADDRESS | N/A | (Optional) MAC Address of old Access Router |
| Mac New Access Router | MAC ADDRESS | N/A | (Optional) MAC Address of new Access Router |

      - When generated

Depending on whether it is the Mobile node or the Network, it is generated when a native link layer handover or switching decision has been made and its execution is imminent.

      - Effect on receipt

The MIHF receives this event from the link layer. The MIHF then passes this notification to the MIH User(s) which have subscribed for this notification. The MIH User(s) may take necessary actions to minimize the effect of the pending native link layer handover or switching on user data transfer. For example, to cover the handover period (when packets cannot be transferred), the network may proactively start sending the additional data as soon as it has received indication about the event and should expect a request from the MN specifying how much data it needs to cover the handover period. This event may also be used as an indication to start buffering packets.

    - Link_Handover_Complete.indication

      - Function

Link_Handover_Complete event is generated whenever a native link layer handover/switching has just been completed (as opposed to Link_Up which only indicates that a link has been brought up for L2 connectivity, but does not indicate that a native link handover/switch-over has just been completed by the link layer). Notifying the upper layer of this event may improve transport, session and application layer responsiveness to autonomous link changes. They may better adapt their data flows by resuming flows upon receiving this indication. The upper layer may also use this event to check whether its IP configuration needs to be updated. This is a link layer event that exists for intra-technology handovers defined in many media types (e.g., cellular, IEEE 802.16e). This event is applicable for the mobile node only and is valid only for intra- technology handovers.

      - Semantics of service primitive

The primitive parameters are as follows:

Link_Handover_Complete.indication 					(
					LinkIdentifier,
					MacOldAccessRouter,
					MacNewAccessRouter
					)

Parameters:

| Name | Type | Valid range | Description |
| --- | --- | --- | --- |
| Link Identifier | LINK ID | N/A | Identifier of the link associated with the event |
| Mac Old Access Router | MAC ADDRESS | N/A | (Optional) MAC Address of old Access Router |
| Mac New Access Router | MAC ADDRESS | N/A | (Optional) MAC Address of new Access Router |

      - When generated

This is generated whenever a L2 link layer handover or switching has just been completed.

      - Effect on receipt

The MIHF receives this event from the link layer. The MIHF then passes this notification to the MIH User(s) which have subscribed for this notification. Upon reception of this event, an upper layer (e.g., transport) may stop any handover adaptation that it has engaged to cope with the just completed native link layer handover/ switching and resume normal data transfer. This event may also be used as an indication that a re-verification of the IP parameter should be considered.

    - Link_Capability_Discover

      - Link_Capability_Discover.request

        - Function

This primitive is used by the MIHF to query and discover the list of supported link layer events and link layer commands.

        - Semantics of service primitive

No primitive parameters exist for this primitive.

Link_Capability_Discover.request			 	(

)

        - When generated

This primitive is generated by the MIHF when it needs to receive link layer event notifications and learn about which link layer commands the low layer can support.

        - Effect on receipt

The recipient responds immediately with Link_Capability_Discover.confirm primitive.

      - Link_Capability_Discover.confirm

        - Function

This primitive returns the result of the query to discover link layer capability.

        - Semantics of service primitive

The primitive parameters are as follows:

Link_Capability_Discover.confirm			 	(

SupportedLinkEventList,

SupportedLinkCommandList,

Status

)

| Name | Type | Valid range | Description |
| --- | --- | --- | --- |
| Supported Link Event List | BITMAP (32) | N/A | List of link layer events supported by the link layer. The encoding uses a bitmap where the bit position represents the event ID as defined in Table . A bit set to 1 indicates that the link layer supports the event. For example, if bit #0 is set to 1, the link layer supports Link_Detected. Bits corresponding to reserved event IDs must be set to 0. |
| Supported Link Command List | BITMAP (32) | N/A | List of link layer commands supported by the link layer. The encoding uses a bitmap where the bit position represents the command ID, as defined in Table . A bit set to 1 indicates that the Link Layer supports the command. Bits corresponding to reserved command IDs must be set to 0. |
| Status | ENUMERATED | 0-4 | Status of operation: 0 Success 1 Unspecified Failure 2 Rejected 3 Authorization Failure 4 Network Error |

- Link command identifiers

| Command name | ID |
| --- | --- |
| Link_Capability_Discover | 0 |
| Link_Event_Subscribe | 1 |
| Link_Event_Unsubscribe | 2 |
| Link_Configure_Thresholds | 3 |
| Link_Get_Parameters | 4 |
| (Reserved) | 5~31 |

        - When generated

This primitive is generated in response to a Link_Capability_Discover.request primitive.

        - Effect on receipt

The recipient may examine the returned event and command list and learn about link layer capability. However, if Status does not indicate “Success”, the recipient shall ignore any other returned values and, instead, shall perform appropriate error handling.

    - Link_Get_Parameters

      - Link_Get_Parameters.request

        - Function

This primitive is used by the MIHF to obtain the current value of a set of link parameters of a specific link.

        - Semantics of service primitive

The primitive parameter is as follows:

Link_Get_Parameters.request 				(
				LinkParametersRequest
				)

| Name | Type | Valid range | Description |
| --- | --- | --- | --- |
| Link Parameters Request | BITMAP(32) | N/A | A bitmap representing the list of link parameters for which status is requested. The bit position represents the link parameter type. For example if bit 0 is set, the MIHF is requesting the maximum number of CoS supported by the link. |

        - When generated

This primitive is generated by the MIHF to obtain the current value of a set of link parameters from a link.

        - Effect on receipt

The recipient link shall respond with Link_Get_Parameters.confirm primitive.

      - Link_Get_Parameters.confirm

        - Function

This primitive is sent in response to the Link_Get_Parameters.request primitive. This primitive provides current value of the requested link parameters. (Note, how the value is measured or calculated by the link is not specified by this standard).

        - Semantics of service primitive

The primitive parameters are as follows:

Link_Get_Parameters.confirm 				(
				LinkParametersStatusList,
				Status
				)

| Name | Type | Valid range | Description |
| --- | --- | --- | --- |
| Link Parameters Status List | LIST | NA | A list of QoS Parameter as defined in Table . |
| Status | ENUMERATED | 0-4 | Status of operation: 0 Success 1 Unspecified Failure 2 Rejected 3 Authorization Failure 4 Network Error |

        - When generated

This primitive is generated in response to the Link_Get_Parameters.request operation.

        - Effect on receipt

The recipient may pass the link parameter values received to the MIH Users. However, if Status does not indicate “Success”, the recipient shall ignore any other returned values and, instead, shall perform appropriate error handling.

    - Link_Action

      - Link_Action.request

        - Function

This primitive is used by the MIHF to request actions on a link layer connection to enable optimal handling of link layer resources for the purpose of handovers. The link layer connection can be ordered, e.g. to shut down, to remain active, to perform a scan, or to come up active and remain in stand-by mode. The command execution delay time can also be specified for cases where the link layer technology under consideration supports the action..included, in case that the link layer technology supporting the current link layer connections performs the action

        - Semantics of Service Primitive

The parameters of the service primitive are as follows:

Link_Action.request 			(
			 LinkIdentifier
			 LinkAction
			 ExecutionDelayTime
			)

The following table details the possible actions that can be requested from the link layer connection.

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| Link Identifier | LINK ID | N/A | Identifier of present link |
| Link Actions | BITMAP(32) | N/A | Specifies the suggested action on link during handover. Combination of the following choices are allowed. Bit 0: LINK_DISCONNECT Bit 1: LINK_LOW_POWER Bit 2: LINK_POWER_DOWN Bit 3: LINK_NO_ACTION Bit 4: LINK_RESOURCE_RETAIN Bit 5: DATA_FORWARDING_REQUEST Bit 6: LINK_POWER_UP Bit 7-31: (Reserved) |
| Execution Delay Time | INTEGER | 0-10000 | Time (in ms) elapsed before action needs to be taken, if ExecutionDelay Time = 0 then the action is expected to be taken on receipt of this command. |

        - When generated

The MIHF generates this primitive upon request from the MIH User to perform an action on a pre-defined link layer connection.

        - Effect on receipt

Upon receipt of this primitive, the link layer technology supporting the current link layer connections performs the action specified by the Link Action IE parameter in accordance to procedures specified within the relevant standards organization and at the time specified by the parameter ExecutionDelay.

      - Link_Action.confirm

        - Function

This primitive is used by link layer technologies to provide an indication of the result of actions executed on the current link layer connection.

        - Semantics of Service Primitive

The parameters of the service primitive are as follows:

Link_Action.confirm 			(
			ScanResponseSet,
			Status
			)

The following table details the possible actions that can be requested from the link layer connection.

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| Scan Response Sets | LIST | N/A | (Optional) A list of discovered links and related information. See Table for detailed definition. |
| Status | ENUMER ATED | 0-4 | Result of the actions specified in the Link_Action.request primitive. 0 Success 1 Unspecified Failure 2 Rejected 3 Resource Unavailable 4 Resource Busy |

        - When generated

The link layer technology generates this primitive to communicate the result of the actions executed on the link layer connection

        - Effect on receipt

Upon receipt of this primitive, the MIHF determines the relevant MIH command that needs to be used to provide an indication or confirmation to the MIH User of the actions performed on the current link layer connection. If a Scan action was issued by the associated Link_Action.request, the optional ScanResponseSet field shall be included in the Link_Action.confirm response.

  - MIH_SAP primitives

The primitives defined as part of MIH_SAP are described in the following subclauses.

    - MIH_Capability_Discover

      - MIH_Capability_Discover.request

        - Function

This primitive is used by an MIH User to discover the capabilities of the local MIHF or a remote MIHF. When invoking this primitive to discover the capabilities of a remote MIHF, the MIH User can optionally piggyback the capability information of its local MIHF so that the two MIHFs can achieve mutually discover each other’s capabilities with a single invocation of this primitive.

        - Semantics of service primitive

MIH_Capability_Discover.request			 	(

DestinationIdentifier,

SupportedMihEventList,

SupportedMihCommandList,

SupportedIsQueryTypeList,

SupportedTransportList

)

| Name | Type | Valid range | Description |
| --- | --- | --- | --- |
| Destination Identifier | MIHF ID | N/A | This identifies the local MIHF or a remote MIHF which will be the destination of this request. |
| Supported MIH Event List | BITMAP(32) | N/A | (Optional) List of supported events on MIHF. The encod ing uses the bitmap defined in . A bit set to 1 indicates that event is supported. |
| Supported MIH Command List | BITMAP(32) | N/A | (Optional) List of supported commands on MIHF. The encoding uses the following bitmap where a bit set to 1 indicates that command(s) is supported. Bit 0: MIH_Get_Link_Parameters Bit 1: MIH_Configure_Link Bit 2: MIH_Scan Bit 3: MIH_Net_HO_Candidate_Query MIH_Net_HO_Commit MIH_N2N_HO_Query_Resources MIH_N2N_HO_Complete Bit 4: MIH_MN_HO_Candidate_Query MIH_MN_HO_Commit MIH_MN_HO_Complete Bit 5-31: (Reserved) |
| Supported IS Query Type List | BITMAP(8) | N/A | (Optional) List of supported MIIS query types. The encod ing uses the following bitmap where a bit set to 1 indicates that query type is supported. Bit 0: TLV Bit 1: RDF_DATA Bit 2: RDF_SCHEMA_URL Bit 3: RDF_SCHEMA Bit 4-7: (Reserved) |
| Supported Transport List | BITMAP(32) | N/A | (Optional) List of supported transport types. For each bit location a value of ‘1’ indicates that the transport option is supported. Bit 0: L2 for service management Bit 1: L3 or higher layer protocol for service management Bit 4: L2 for event service Bit 5: L3 or higher layer protocol for event service Bit 8: L2 for command service Bit 9: L3 or higher layer protocol for command service Bit 12: L2 for information service Bit 13: L3 or higher layer protocol for information service Other Bits: (Reserved) |

        - When generated

This primitive is generated by an MIH User to discover the capabilities of the local MIHF or a remote MIHF. In the case of remote discovery, this primitive may contain the SupportedMihEventList, SupportedMihCommandList, SupportedIsQueryTypeList, and SupportedTransportList parameters to enable mutual discovery of each other’s capabilities.

        - Effect on receipt

If the destination of the request is the local MIHF itself, the local MIHF shall respond with MIH_Capability_Discover.confirm. If the destination of the request is a remote MIHF, the local MIHF shall generate and send a corresponding MIH_Capability_Discover request message to the remote MIHF.

      - MIH_Capability_Discover.indication

        - Function

This primitive is used by an MIHF to notify an MIH User on the receipt of an MIH_Capability_Discover request message from a peer MIHF.

        - Semantics of Service primitive

MIH_Capability_Discover.indication (
				SourceIdentifier,
				SupportedMihEventList,
				SupportedMihCommandList,
				SupportedIsQueryTypeList,
				SupportedTransportList
				)

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| Source Identifier | MIHF ID | N/A | This identifies the invoker of this primitive which can be either the local MIHF or a remote MIHF. |
| Supported MIH Event List | BITMAP(32) | N/A | (Optional) List of supported events on MIHF. The encoding uses the bitmap defined in . A bit set to 1 indicates that event is supported. |
| Supported MIH Command List | BITMAP(32) | N/A | (Optional) List of supported commands on MIHF. The encoding uses the following bitmap where a bit set to 1 indi cates that command(s) is supported. Bit 0: MIH_Get_Link_Parameters Bit 1: MIH_Configure_Link Bit 2: MIH_Scan Bit 3: MIH_Net_HO_Candidate_Query MIH_Net_HO_Commit MIH_N2N_HO_Query_Resources MIH_N2N_HO_Complete Bit 4: MIH_MN_HO_Candidate_Query MIH_MN_HO_Commit MIH_MN_HO_Complete Bit 5-31: (Reserved) |
| Supported IS Query Type List | BITMAP(8) | N/A | (Optional) List of supported MIIS query types. The encoding uses the following bitmap where a bit set to 1 indicates that query type is supported. Bit 0: TLV Bit 1: RDF_DATA Bit 2: RDF_SCHEMA_URL Bit 3: RDF_SCHEMA Bit 4-7: (Reserved) |
| Supported Trans port List | BITMAP(32) | N/A | (Optional) List of supported transport types. For each bit location a value of ‘1’ indicates that the transport option is supported. Bit 0: L2 for service management Bit 1: L3 or higher layer protocol for service management Bit 4: L2 for event service Bit 5: L3 or higher layer protocol for event service Bit 8: L2 for command service Bit 9: L3 or higher layer protocol for command service Bit 12: L2 for information service Bit 13: L3 or higher layer protocol for information service Other Bits: (Reserved) |

        - When generated

This primitive is used by an MIHF to notify an MIH User when an MIH_Capability_Discover request message is received.

        - Effect on receipt

The MIH User may respond with an MIH_Capability_Discover.response primitive.

      - MIH_Capability_Discover.response

This primitive is used by an MIH User to convey the supported MIH capabilities to the MIH User which invoked the MIH_Capability_Discover request.

        - Semantics of Service primitive

MIH_Capability_Discover.response(
				DestinationIdentifier,

SupportedMihEventList,

SupportedMihCommandList,

SupportedIsQueryTypeList,

SupportedTransportList,

Status

)

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| Destination Iden tifier | MIHF ID | N/A | This identifies the local MIHF or a remote MIHF which will be the destination of this response. |
| Supported MIH Event List | BITMAP(32) | N/A | List of supported events on MIHF. The encoding uses the bitmap defined in . A bit set to 1 indicates that event is supported. |
| Supported MIH Command List | BITMAP(32) | N/A | List of supported commands on MIHF. The encoding uses the following bitmap where a bit set to 1 indicates that com mand(s) is supported. Bit 0: MIH_Get_Link_Parameters Bit 1: MIH_Configure_Link Bit 2: MIH_Scan Bit 3: MIH_Net_HO_Candidate_Query MIH_Net_HO_Commit MIH_N2N_HO_Query_Resources MIH_N2N_HO_Complete Bit 4: MIH_MN_HO_Candidate_Query MIH_MN_HO_Commit MIH_MN_HO_Complete Bit 5-31: (Reserved) |
| Supported IS Query Type List | BITMAP(8) | N/A | (Optional) List of supported MIIS query types. The encoding uses the following bitmap where a bit set to 1 indicates that query type is supported. Bit 0: TLV Bit 1: RDF_DATA Bit 2: RDF_SCHEMA_URL Bit 3: RDF_SCHEMA Bit 4-7: (Reserved) |
| Supported Trans port List | BITMAP(32) | N/A | (Optional) List of supported transport types. For each bit location a value of ‘1’ indicates that the transport option is supported. Bit 0: L2 for service management Bit 1: L3 or higher layer protocol for service management Bit 4: L2 for event service Bit 5: L3 or higher layer protocol for event service Bit 8: L2 for command service Bit 9: L3 or higher layer protocol for command service Bit 12: L2 for information service Bit 13: L3 or higher layer protocol for information service Other Bits: (Reserved) |
| Status | ENUMER ATED | 0-4 | Status of operation: 0 Success 1 Unspecified Failure 2 Rejected 3 Authorization Failure 4 Network Error |

        - When generated

This primitive is generated by an MIH User as a response to a received MIH_Capability_Discover.indication primitive.

        - Effect on receipt

Upon receiving this primitive, the MIHF generates and sends the corresponding MIH_Capability_Discover response message to the destination MIHF.

      - MIH_Capability_Discover.confirm

        - Function

This primitive is used by the MIHF to convey the supported MIH capabilities about Event Service, Command Service, and Information Service to the MIH User which invoked the MIH_Capability_Discover request.

        - Semantics of service primitive

MIH_Capability_Discover.confirm			 	(

SourceIdentifier,

SupportedMihEventList,

SupportedMihCommandList,

SupportedIsQueryTypeList,

SupportedTransportList,

Status

)

| Name | Type | Valid range | Description |
| --- | --- | --- | --- |
| Source Identifier | MIHF ID | N/A | This identifies the invoker of this primitive which can be either the local MIHF or a remote MIHF. |
| Supported MIH Event List | BITMAP(32) | N/A | Indicate the supported events on the MIHF. The encoding uses the following bitmap where a bit set to 1 indicates that event is supported. Bit 0: MIH_Link_Up Bit 1: MIH_Link_Down Bit 2: MIH_Link_Going_Down Bit 3: MIH_Link_Detected Bit 4: MIH_Link_Parameters_Report Bit 5: MIH_Link_Event_Rollback Bit 6: MIH_Link_PDU_Transmit_Status Bit 7: MIH_Link_Handover_Imminent Bit 8: MIH_Link_Handover_Complete Bit 9-31: (Reserved) |
| Supported MIH Command List | BITMAP(32) | N/A | List of supported commands on MIHF. The encoding uses the following bitmap where a bit set to 1 indicates that command(s) is supported. Bit 0: MIH_Get_Link_Parameters Bit 1: MIH_Configure_Link Bit 2: MIH_Scan Bit 3: MIH_Net_HO_Candidate_Query MIH_Net_HO_Commit MIH_N2N_HO_Query_Resources MIH_N2N_HO_Complete Bit 4: MIH_MN_HO_Candidate_Query MIH_MN_HO_Commit MIH_MN_HO_Complete Bit 5-31: (Reserved) |
| Supported IS Query Type List | BITMAP(8) | N/A | (Optional) List of supported MIIS query types. The encoding uses the following bitmap where a bit set to 1 indicates that query type is supported. Bit 0: TLV Bit 1: RDF_DATA Bit 2: RDF_SCHEMA_URL Bit 3: RDF_SCHEMA Bit 4-7: (Reserved) |
| Supported Transport List | BITMAP(32) | N/A | (Optional) List of supported transport types. For each bit loca tion a value of ‘1’ indicates that the transport option is sup ported. Bit 0: L2 for service management Bit 1: L3 or higher layer protocol for service management Bit 4: L2 for event service Bit 5: L3 or higher layer protocol for event service Bit 8: L2 for command service Bit 9: L3 or higher layer protocol for command service Bit 12: L2 for information service Bit 13: L3 or higher layer protocol for information service Other Bits: (Reserved) |
| Status | ENUMER ATED | 0-4 | Status of operation: 0 Success 1 Unspecified Failure 2 Rejected 3 Authorization Failure 4 Network Error |

        - When generated

This primitive is invoked by a local MIHF to convey the results of a previous MIH_Capability_Discover.request primitive from an MIH User.

        - Effect on receipt

Upon reception of this primitive the receiving entity becomes aware of the supported MIH capabilities. However, if Status does not indicate “Success”, the recipient shall ignore any other returned values and, instead, shall perform appropriate error handling.

    - MIH_Register

      - MIH_Register.request

        - Function

This primitive is used by an MIH User to register the local MIHF with peer MIHFprompt its local MIHF to communicate with a peer MIHF to perform registration.

        - Semantics of service primitive

MIH_Register.request			(
			DestinationIdentifier, 
			RequestCode
			)

Local or Remote: Remote

MIHF (Network) <> MIHF (Network)

MIHF (network) <- MIHF (MN)

MIHF(MN) <> MIHF (Network)

| Name | Type | Valid range | Description |
| --- | --- | --- | --- |
| Destination Identifier | MIHF ID | N/A | This identifies a remote MIHF which will be the destination of this request. |
| Request Code | ENUMERATED | 0-1 | 0 - Registration 1 - Re-Registration [Editor’s note: Re-registration scope to be defined.] |

        - When generated

This primitive is invoked by the MIH User when it needs to register the local MIHF with a peer MIHFperform an MIH registration to a particular destination MIHF.

        - Effect on receipt

Upon On receipt, the local MIHF shall generate and send an MIH_Register request message to the destination MIHF.

      - MIH_Register.indication

        - Function

This primitive is used by an MIHF to notify an MIH User that an MIH_Register request message has arrived been received..

        - Semantics of service primitive

MIH_Register.indication			(
			SourceIdentifier, 
			RequestCode
			)

| Name | Type | Valid range | Description |
| --- | --- | --- | --- |
| Source Identifier | MIHF ID | N/A | This identifies the invoker of this primitive which can be either the local MIHF or a remote MIHF. |
| Request Code | ENUMERATED | 0-1 | 0 - Registration 1 - Re-Registration [Editor’s note: Re-registration scope to be defined.] |

        - When generated

This primitive is generated by the peer MIHF when an MIH_Register request message is received.

        - Effect on receipt

The remote MIH User will perform necessary actions to process the registration request or re-registration and respond with an MIH_Register.response.

      - MIH_Register.response

        - Function

This primitive is used by a remote MIH User to send the indicate the result status of processing a received registration request. received from a remote MIH User.

        - Semantics of service primitive

MIH_Register.response			(
			Destination Identifier,
			ValidTimeInterval,
			Status
			)

Local or Remote: Remote

MIHF (Network) <> MIHF (Network)

MIHF (network) -> MIHF (MN)

| Name | Type | Valid range | Description |
| --- | --- | --- | --- |
| Destination Identifier | MIHF ID | N/A | This identifies a remote MIHF which will be the destination of this response. |
| ValidTimeInterval | INTEGER | 1 to 2**32 -1 | Time interval in seconds during which the registration is valid. (Should there be an option 0: Registration never expires until deregistration is explictly invoked????) |
| Status | ENUMERATED | 0-4 | Status of operation: 0 Success 1 Unspecified Failure 2 Rejected 3 Network/Operator Authorization Failure 4 Network Error |

        - When generated

This primitive is invoked by the remote MIH User to report back the result after completing the processing of a registration request from a remote MIH User.

        - Effect on receipt

Upon receipt, the local MIHF shall generate and send an MIH_Register response confirm message to the destination MIHF local MIH User.

      - MIH_Register.confirm

        - Function

This primitive is used by the local MIHF to convey the result of a registration request to an MIH User.

        - Semantics of service primitive

MIH_Register.confirm			(
			SourceIdentifier,
			ValidTimeInterval,
			Status
			)

| Name | Type | Valid range | Description |
| --- | --- | --- | --- |
| Source Identifier | MIHF ID | N/A | This identifies the invoker of this primitive which can be a remote MIHF. |
| ValidTimeInterval | INTEGER | 1 to 232-1 | Time in seconds during which the registration is valid |
| Status | ENUMERATED | 0-4 | Status of operation: 0 Success 1 Unspecified Failure 2 Rejected 3 Network/Operator Authorization Failure 4 Network Error |

        - When generated

This primitive is used by an MIHF to notify an MIH User the result of an MIH registration request.

        - Effect on receipt

Upon receipt, the MIH User can determine the result of the registration request.

    - MIH_DeRegister

      - MIH_DeRegister.request

        - Function

This primitive is used by an MIH User to deregister the local MIHF with peer MIHF.  prompt its local MIHF to communicate with a peer MIHF to perform a de-registration.

        - Semantics of service primitive

MIH_DeRegister.request			(
			DestinationIdentifier 
			)

Local or Remote: Remote

MIHF (Network) <> MIHF (Network)

MIHF(MN)	<> MIHF (Network)

MIHF (network) <- MIHF (MN)

| Name | Type | Valid range | Description |
| --- | --- | --- | --- |
| Destination Identifier | MIHF ID | N/A | This identifies a remote MIHF which will be the destination of this request. |

        - When generated

This primitive is invoked by the MIH User when it needs to terminate an existing MIH registration with a remote MIHF.

        - Effect on receipt

Upon receipt, the local MIHF shall generate and send an MIH_DeRegister request message to the destination MIHF.

      - MIH_DeRegister.indication

        - Function

This primitive is used by a remote MIHF to notify an MIH User that a MIH_DeRegister request message has arrived been received.

        - Semantics of service primitive

MIH_DeRegister.indication			(
			SourceIdentifier 
			)

| Name | Type | Valid range | Description |
| --- | --- | --- | --- |
| Source Identifier | MIHF ID | N/A | This identifies the invoker of this primitive which is a remote MIHF. |

        - When generated

This primitive is generated by a remote MIHF when an MIH_DeRegister request message is received.

        - Effect on receipt

The MIH User will perform necessary actions to process the deregistration request and respond with a MIH_DeRegister.response.

      - MIH_DeRegister.response

        - Function

This primitive is invoked by a remote MIH User tosend indicate the status of processing a received deregistration request. previously received.

        - Semantics of service primitive

MIH_DeRegister.response			(
			DestinationIdentifier,
			Status
			)

Local or Remote: Remote

MIHF (Network) <> MIHF (Network)

MIHF (MN)	<> MIHF (Network)

MIHF (network) -> MIHF (MN)

| Name | Type | Valid range | Description |
| --- | --- | --- | --- |
| Destination Identifier | MIHF ID | N/A | This identifies a remote MIHF which will be the desti nation of this response. |
| Status | ENUMERATED | 0-4 | Status of operation: 0 Success 1 Unspecified Failure 2 Rejected 3 Network/Operator Authorization Failure 4 Network Error |

        - When generated

This primitive is invoked by the remote MIH User to report back the result after completing the processing of a deregistration request. from a remote MIH User.

        - Effect on receipt

      - MIH_DeRegister.confirm

        - Function

This primitive is used by the local MIHF to convey the result of a deregistration request to the local MIH User.

        - Semantics of service primitive

MIH_DeRegister.confirm			(
			SourceIdentifier,
			Status
			)

| Name | Type | Valid range | Description |
| --- | --- | --- | --- |
| Source Identifier | MIHF ID | N/A | This identifies the invoker of this primitive which can be a remote MIHF. |
| Status | ENUMERATED | 0-4 | Status of operation: 0 Success 1 Unspecified Failure 2 Rejected 3 Network/Operator Authorization Failure 4 Network Error |

        - When generated

This primitive is used by an MIHF to notify the local MIH User the status of an MIH deregistration request.

        - Effect on receipt

Upon receipt, the MIH User can determine the status of the deregistration request.

    - MIH_Event_Subscribe

      - MIH_Event_Subscribe.request

        - Function

This primitive is used by an MIH User (the subscriber) to subscribe an interest in one or more MIH event types from the local or a remote MIHF.

        - Semantics of service primitive

The primitive parameters are as follows:

MIH_Event_Subscribe.request			 	(
				DestinationIdentifier,
				LinkIdentifier,

RequestedMihEventList
				)

| Name | Type | Valid range | Description |
| --- | --- | --- | --- |
| Destination Identifier | MIHF ID | N/A | This identifies the local MIHF or a remote MIHF which will be the destination of this request. |
| Link Identifier | LINK ID | N/A | Identifier of the link for event subscription. For local event subscription, PoA MAC Address in the LinkIdentifier needs not to be present. [Editor’s note: should this use CONNECTION ID?] |
| Requested MIH Event List | BITMAP(32) | N/A | List of MIH events that the endpoint would like to receive indications for, from the Event Source. The encoding uses the bitmap defined in where a bit set to 1 indicates that event is requested. |

        - When generated

This primitive is invoked by an MIH User when it wants to receive indications on a set of specific MIH events from the local MIHF or a remote MIHF.

        - Effect on receipt

If the destination of the request is the local MIHF itself, the local MIHF shall respond immediately with an MIH_Event_Subscribe.confirm primitive. If the destination of the request is a remote MIHF, the local MIHF shall generate and send an MIH_Event_Subscribe request message to the remote MIHF.

      - MIH_Event_Subscribe.confirm

        - Function

This primitive returns the result of an MIH event subscription request.

        - Semantics of service primitive

The primitive parameters are as follows:

MIH_Event_Subscribe.confirm			 	(
				SourceIdentifier
				LinkIdentifier,
				ResponseMihEventList,

Status
				)

| Name | Type | Valid range | Description |
| --- | --- | --- | --- |
| Source Identifier | MIHF ID | N/A | This identifies the invoker of this primitive which can be either the local MIHF or a remote MIHF. |
| Link Identifier | LINK ID | N/A | Identifier of the link for event subscription. |
| Response MIH Event List | BITMAP(32) | N/A | List of MIH events along with their sub scription status.The encoding uses the bit map defined in . A bit set to 1 indicates that subscription succeeded. |
| Status | ENUMERATED | 0-4 | Status of operation: 0 Success 1 Unspecified Failure 2 Rejected 3 Authorization Failure 4 Network Error |

        - When generated

This primitive is generated by the local MIHF at the completion of processing an MIH_Event_Subscribe.request primitive from a local MIH User or in response to the receiving of an MIH_Event_Subscribe response message from a peer MIHF.

        - Effect on receipt

The recipient MIH User may examine the returned event list and learn about the subscription status of different events. However, if Status does not indicate “Success”, the recipient shall ignore any other returned values and, instead, shall perform appropriate error handling.

    - MIH_Event_Unsubscribe

      - MIH_Event_Unsubscribe.request

        - Function

This primitive is used by an MIH User (the subscriber) to unsubscribe from a set of previous subscribed MIH events.

        - Semantics of service primitive

The primitive parameters are as follows:

MIH_Event_Unsubscribe.request			 	(
				DestinationIdentifier,
				LinkIdentifier,
				RequestedMihEventList
				)

| Name | Type | Valid range | Description |
| --- | --- | --- | --- |
| Destination Identifier | MIHF ID | N/A | This identifies the local MIHF or a remote MIHF which will be the destination of this request. |
| Link Identifier | LINK ID | N/A | Identifier of the link for event unsubscription. For local event unsubscription, PoA MAC Address in the LinkIdentifier needs not to be present. |
| Requested MIH Event List | BITMAP(32) | N/A | List of MIH events for which indications need to be deregistered from the Event Source. The encoding uses the bitmap defined in . A bit set to 1 indi cates that subscriber is seeking to unsubscribe to the event. |

        - When generated

This primitive is invoked by an MIH User (subscriber) that is seeking to unsubscribe from an already subscribed set of events from the local MIHF or a remote MIHF.

        - Effect on receipt

If the destination of the request is the local MIHF itself, the local MIHF shall respond immediately with MIH_Event_Unsubscribe.confirm primitive. If the destination of the request is a remote MIHF, the local MIHF shall generate and send an MIH_Event_Unsubscribe request message to the remote MIHF.

      - MIH_Event_Unsubscribe.confirm

        - Function

This primitive returns the result of an MIH event unsubscription request.

        - Semantics of service primitive

The primitive parameters are as follows:

MIH_Event_Unsubscribe.confirm			 	(
				SourceIdentifier,
				LinkIdentifier,
				ResponseMihEventList,
				Status
				)

| Name | Type | Valid range | Description |
| --- | --- | --- | --- |
| Source Identifier | MIHF ID | N/A | This identifies the invoker of this primi tive which can be either the local MIHF or a remote MIHF. |
| Link Identifier | LINK ID | N/A | Identifier of the link for event unsub scription. |
| Response MIH Event List | BITMAP(32) | N/A | List of MIH events along with their unsubscription status. The encoding uses the bitmap defined in . A bit set to 1 indicates that unsubscription succeeded. |
| Status | ENUMERATED | 0-4 | Status of operation: 0 Success 1 Unspecified Failure 2 Rejected 3 Authorization Failure 4 Network Error |

        - When generated

This primitive is generated by the local MIHF at the completion of processing an MIH_Event_Unsubscribe.request primitive from a local MIH User or in response to the receiving of an MIH_Event_Unsubscribe response message from a peer MIHF.

        - Effect on receipt

The recipient MIH User may examine the returned event list and learn about the unsubscription status of different events. However, if Status does not indicate “Success”, the recipient shall ignore any other returned values and, instead, shall perform appropriate error handling.

    - MIH_Link_Up

MIH_Link_Up directly corresponds to the link layer event, Link_Up. Please see the description in .

    - MIH_Link_Down

MIH_Link_Down directly corresponds to the link layer event, Link_Down. Please see the description in .

    - MIH_Link_Going_Down

MIH_Link_Going_Down directly corresponds to the link layer event, Link_Going_Down. Please see the description in .

    - MIH_Link_Detected

MIH_Link_Detected directly corresponds to the link layer event, Link_Detected. Please see the description in .

    - MIH_Link_Parameters_Report

      - MIH_Link_Parameters_Report.indication

        - Function

MIH_Link_Parameters_Report indication is sent by the local MIHF to a local MIH User to report the status of a set of parameters of a local or remote link.

The event may be local or remote.

        - Semantics of service primitive

The primitive parameters are as follows:

MIH_Link_Parameters_Report.indication 					(
					SourceIdentifier,
					LinkIdentifier,
					LinkParametersStatusList
					)

Parameters

| Name | Type | Valid range | Description |
| --- | --- | --- | --- |
| Source Identifier | MIHF ID | N/A | This identifies the invoker of this primitive which can be either the local MIHF or a remote MIHF. |
| Link Identifier | LINK ID | N/A | Identifier of the link associated with the event |
| Link Parameters Status List | LIST | N/A | A list of Link Parameter Status, as defined in Table . |

:

- Elements of link parameter status

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| Parameter Type | INTEGER | 0-255 | Parameter being reported, as defined in Table . |
| Current Value | (Varies dependent on Parameter Type) | N/A | The current value of the parameter. |
| Threshold Crossed | ENUMERATED | 0-2 | (Optional) Provided when this is a thresh old-crossing event. 0: Initiate Action Threshold crossed 1: Rollback Action Threshold crossed 2: Execute Action Threshold crossed |

        - When generated

This notification is generated by the local MIHF either:

- at a predefined regular interval determined by a user configurable timer;

- when a specified parameter of a currently active local interface crosses a configured threshold. In such a case, the local MIHF mostly likely will first receive a Link_Parameters_Report.indication from the local link layer; or

- when an MIH_Link_Parameter_Report indication message is received from a remote MIHF.

        - Effect on receipt

Upper layer entities may take different actions upon receipt of this indication. For example, a mobility management entity locally or remotely located from the event origination link may take these events into account in its decision algorithm.

    - MIH_Link_Event_Rollback

MIH_Link_Event_Rollback directly corresponds to the link layer event, Link_Event_Rollback. Please see the description in .

    - MIH_Link_PDU_Transmit_Status

MIH_Link_PDU_Transmit_Status directly corresponds to the link layer event, Link_PDU_Transmit_Status. Please see the description in .

    - MIH_Link_Handover_Imminent

      - MIH_Link_Handover_Imminent.indication

MIH_Link_Handover_Imminent is issued by the MIHF to report that a link switch is about to occur. The event notified may be either an intra-technology or inter-technology link switch. In the event of an intra- technology link switch, this indication directly corresponds to the link layer event Link_Handover_Imminent defined in .

        - Function

This primitive is issued by the MIHF to report the imminent occurrence of either an intra-technology or inter-technology link handover to MIH Users. This MIH event may be local or remote.

        - Semantics of service primitive

The primitive parameters are as follows:

MIH_Link_Handover_Imminent.indication 					(
					SourceIdentifier,
					Old Link Identifier,
					New Link Identifier,
					HO_Type,
					MacOldAccessRouter,
					MacNewAccessRouter
					)

Parameters

| Name | Type | Valid range | Description |
| --- | --- | --- | --- |
| Source Identifier | MIHF ID | N/A | This identifies the invoker of this primi tive which can be either the local MIHF or a remote MIHF. |
| Old Link Identifier | LINK ID | N/A | Identifier of the old link |
| New Link Identifier | LINK ID | N/A | Identifier of the new link |
| HO_Type | BOOLEAN | 0-1 | Indicates whether intra-technology or inter-technology handover: 0 - Intra-technology 1 - Inter-technology |
| Mac Old Access Router | MAC ADDRESS | N/A | MAC Address of old Access Router |
| Mac New Access Router | MAC ADDRESS | N/A | MAC Address of new Access Router |

:

        - When generated

This notification is generated by the MIHF when a link layer inter- or intra-technology handover is about to occur. In the case of an intra-technology handover, the event could be triggered by the reception of a Link_Handover_Imminent.indication from a link.

        - Effect on receipt

Upper layer entities may take different actions on this notification.

    - MIH_Link_Handover_Complete

      - MIH_Link_Handover_Complete.indication

MIH_Link_Handover_Complete is issued by the MIHF to report the successful completion of a link handover to MIH Users. The event notified may be either an intra-technology or inter-technology link handover. The link handover may be a result of either the execution of an MIH_MN_HO_Commit or MIH_Net_HO_Commit command previously issued by an MIH User or an autonomous L2 handover/ switching event indicated by a Link_Handover_Complete indication from the link layer.

        - Function

This primitive is issued by the MIHF to report the completion of a link handover to MIH Users. This event may notify MIH Users of a successful intra-technology or inter-technology handover. This MIH event may be local or remote.

        - Semantics of service primitive

The primitive parameters are as follows:

MIH_Link_Handover_Complete.indication 					(
					SourceIdentifier,
					OldLinkIdentifier,
					NewLinkIdentifier,
					MacOldAccessRouter,
					MacNewAccessRouter
					)

Parameters

| Name | Type | Valid range | Description |
| --- | --- | --- | --- |
| Source Identifier | MIHF ID | N/A | This identifies the invoker of this primi tive which can be either the local MIHF or a remote MIHF. |
| Old Link Identifier | LINK ID | N/A | Identifier of the old link |
| New Link Identifier | LINK ID | N/A | Identifier of the new link |
| Mac Old Access Router | MAC ADDRESS | N/A | MAC Address of old Access Router |
| Mac New Access Router | MAC ADDRESS | N/A | MAC Address of new Access Router |

:

        - When generated

This notification is generated by the MIHF when a link layer inter- or intra-technology handover is completed. In the case of an intra-technology handover, the event could be triggered by the reception of a Link_Handover_Complete.indication from a link. In the case of an inter-technology handover, the event could be triggered by the successful execution of an MIH_MN_HO_Commit or MIH_Net_HO_Commit primitive issued earlier by an MIH User.

        - Effect on receipt

Upper layer entities may take different actions on this notification. An MIH User may make use of this notification to configure other layers (IP, Mobile IP) for various upper layer handovers (e.g., L3 Handovers) that are needed. Transport layers (e.g., TCP) may also make use of this primitive to fine tune their flow control and flow congestion mechanisms.

    - MIH_Get_Link_Parameters

An MIH_Get_Link_Parameters command is issued by upper layer entities to discover and monitor the status of the currently connected and potentially available links. The destination of an MIH_Get_Link_Parameters command may be local or remote. For example, an MIH_Get_Link_Parameters issued by a local upper layer may help the policy function that resides out of the MIH to make optimal handover decisions for different applications when multiple links are available in a mobile node, while a remotely initiated MIH_Get_Link_Parameters from the network side may enable the network to collect the status information on multiple links in a mobile node through the currently connected link.

      - MIH_Get_Link_Parameters.request

        - Function

This primitive is invoked by an MIH User to discover the status of the currently connected and potentially available links.

        - Semantics of the service primitive

The parameters of the primitive are as follows:

MIH_Get_Link_Parameters.request 			(

DestinationIdentifier,
			LinkIdentifierList,
			GetStatusRequestSet
			)

| Name | Type | Valid range | Description |
| --- | --- | --- | --- |
| Destination Identifier | MIHF ID | N/A | This identifies the local MIHF or a remote MIHF which will be the destination of this request. |
| Link Identifier List | LIST | N/A | List of link identifiers for which status is requested. If the list is empty, return the status of all available links. |
| Get Status Request Set | BITMAP(32) | N/A | Indicate which link status(es) is being requested. The encoding uses a bitmap as follows: Bit #0: DEVICE_INFO Bit# 1: OPERATION_MODE Bit# 2: CHANNEL_ID Bit# 3: BATTERY_LEVEL Bit# 4: LINK_QOS_PARAMETERS_TYPE_LIST Bit# 5-255: reserved |

        - When generated

This primitive is invoked by an MIH User when it wants to request the status information of a set of local or remote links.

        - Effect of receipt

If the destination of the request is the local MIHF itself, the local MIHF shall get the requested information on the status of the specified local links and respond with an MIH_Get_Link_Parameters.confirm. If the destination of the request is a remote MIHF, the local MIHF shall generate and send an MIH_Get_Link_Parameters request message to the remote MIHF.

      - MIH_Get_Link_Parameters.confirm

        - Function

This primitive is issued by an MIHF to report the requested status of a set of specific local or remote links in response to an MIH_Get_Link_Parameters request from a local or remote MIH User.

        - Semantics of the service primitive

The parameters of the primitive are as follows:

MIH_Get_Link_Parameters.confirm 			(

SourceIdentifier,

GetStatusResponseList,

Status

)

| Name | Type | Valid range | Description |
| --- | --- | --- | --- |
| Source Identifier | MIHF ID | N/A | This identifies the invoker of this primi tive which can be either the local MIHF or a remote MIHF. |
| Get Status Response List | LIST | N/A | List of Status Responses, as defined in Table . |
| Status | ENUMERATED | 0-4 | Status of operation: 0 Success 1 Unspecified Failure 2 Rejected 3 Authorization Failure 4 Network Error |

- Elements of a status response

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| Link Identifier | LINK ID | N/A | The link identifier for which status has been requested |
| Link Status List | LIST | N/A | List of Link Status Response as defined in Table |

- Elements of link status response

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| Status Parame terType | INTEGER | 0-255 | Type of parameter: 0: DEVICE_INFO 1: OPERATION_MODE 2: CHANNEL_ID 3: BATTERY_LEVEL 4: LINK_QOS_PARAMETERS_LIST 5-255: reserved |
| Configuration Parameter Value | (Varies depending on the parameter type) | N/A | Value of the parameter |

- Link status type values

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| DEVICE_INFO | STRING | N/A | Information on manufacturer, model number, revision number of the software/ firmware and serial number in display able text are returned. |
| OPERATION_MODE | INTEGER | 0x00 Normal Mode 0x01 Power Saving Mode 0x02 Power Down | Returns the link’s current power mode. |
| CHANNEL_ID | INTEGER | N/A | The ID of the channel currently in use |
| BATTERY_LEVEL | INTEGER | 0-100; -1; unknown battery level | Battery level in percentage; |
| LINK_QOS_PARAME TERS_TYPE_LIST | LIST | N/A | List of QoS ParameterType as defined in Table . |

        - When generated

This primitive returns the results of an MIH_Get_Link_Parameters request to the requesting MIH User.

        - Effect of receipt

Upon receipt of the link status information, the MIH User makes appropriate decisions and takes suitable actions. However, if Status does not indicate “Success”, the recipient shall ignore any other returned values and, instead, shall perform appropriate error handling.

    - MIH_Configure_Link

The MIH_Configure_Link may be issued by an upper layer entity to control the behavior of lower layers, for example, to set some feature of the driver of a specific link. When the MIHF receives MIH_Configure_Link, it issues the Link_Configure_Thresholds commands to the corresponding link.

      - MIH_Configure_Link.request

        - Function

This primitive is issued by an MIH User to control the behavior of a lower layer link.

        - Semantics of the service primitive

The parameters of the primitive are as follows:

MIH_Configure_Link.request 			(

DestinationIdentifier,

LinkIdentifier,

ConfigurationRequestsList

)

| Name | Type | Valid range | Description |
| --- | --- | --- | --- |
| Destination Identifier | MIHF ID | N/A | This identifies the local MIHF or a remote MIHF which will be the destination of this request. |
| Link Identifier | LINK ID | N/A | Identifier of the link to be configured. |
| Configuration Request List | LIST | N/A | List of Configuration Requests, as defined in Table |

- Elements of configuration request

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| Configuration Parameter Type | INTEGER | 0-255 | Type of parameter to be configured: 0: Operation Mode 1: Link QoS Parameters List 2: Link Configure Parameter List (as defined in ) 3-255: (Reserved) |
| Configuration Parameter Value | (Varies depending on the parameter type) | N/A | Value of the parameter being set as defined in Table . |

.

- Link configuration parameter values

| Name | Type | Valid range | Description |
| --- | --- | --- | --- |
| Operation Mode | INTEGER | 0-2 | 0: Power UP 1: Power Saving Mode 2: Power Down |
| Link QoS Parameters List | LIST | N/A | A list of link QoS parameters and their cor responding values as specified in Table . |
| Link Configure Parameter List | LIST | N/A | A list of link parameters and their corre sponding thresholds as defined in . |

        - When generated

This primitive is invoked by an MIH User when it attempts to control the behavior of a local or remote lower layer link, for example, set some features in the driver.

        - Effect of receipt

If the destination of the request is the local MIHF itself, the local MIHF shall issue a Link_Configure_Thresholds request to the lower layer link to set the thresholds for the link according to the specified configuration parameters.

If the destination of the request is a remote MIHF, the local MIHF shall generate and send an MIH_Configure_Link request message to the remote MIHF. Upon the receipt of the message, the remote MIHF shall then issue a Link_Configure_Thresholds request to the lower layer link to set the thresholds for the link according to the specified configuration parameters.

      - MIH_Configure_Link.confirm

        - Function

This primitive is issued by an MIHF to report the result of an MIH_Configure_Link request.

        - Semantics of the service primitive

The parameters of the primitive are as follows:

MIH_Configure_Link.confirm (

SourceIdentifier,

LinkIdentifier,

ConfigurationResponseList,

Status

)

| Name | Type | Valid range | Description |
| --- | --- | --- | --- |
| Source Identifier | MIHF ID | N/A | This identifies the invoker of this primitive which can be either the local MIHF or a remote MIHF. |
| Link Identifier | LINK ID | N/A | Identifier of the link configured. |
| Configuration Response List | LIST | N/A | List of Configuration Response Elements (see Table ) |
| Status | ENUMER ATED | 0-4 | Status of operation: 0 Success 1 Unspecified Failure 2 Rejected 3 Authorization Failure 4 Network Error |

- Configuration response element

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| Configuration Parameter Type | INTEGER | 0-255 | Parameter for which configuration has been requested. |
| Configuration Status | ENUMER ATED | Success Error | Status of configuration operation |

        - When generated

This primitive returns the result of an MIH_Configure_Link request to the requesting MIH User.

        - Effect of receipt

Upon receipt of the result, the MIH User makes appropriate evaluations and takes any suitable actions. However, if Status does not indicate “Success”, the recipient shall ignore any other returned values and, instead, shall perform appropriate error handling.

    - MIH_Scan

An upper layer entity can use an MIH_Scan command to discover available PoAs in its neighborhood. An MIH_Scan command may trigger the issuance of a link scan command at the destination MIHF. The link scan command is defined in the related SDO and beyond the scope of this standard. If the current active link is to be scanned, it is most likely that the on-going user data traffic will be interrupted.

      - MIH_Scan.request

        - Function

This primitive is invoked by an MIH User to request an MIHF to survey of available PoAs in its current neighborhood.

        - Semantics of the service primitive

The parameters of the primitive are as follows:

MIH_Scan.request (

DestinationIdentifier,

ScanLinkIdentifier

)

| Name | Type | Valid range | Description |
| --- | --- | --- | --- |
| Destination Identifier | MIHF ID | N/A | This identifies the local MIHF or a remote MIHF which will be the destination of this request. |
| Scan Link Identifier | LINK ID | N/A | Identifier of the link to be scanned. |

        - When generated

This primitive is invoked by an MIH User when it wants to request an MIHF to perform a survey of available PoAs in its current neighborhood.

        - Effect of receipt

If the destination of the request is the local MIHF itself, the local MIHF shall issue the corresponding Link Scan command to the specified link. The Link Scan command is defined in the relevant SDO. If the destination of the request is a remote MIHF, the local MIHF shall generate and send an MIH_Scan request message to the remote MIHF.

      - MIH_Scan.confirm

        - Function

This primitive is issued by an MIHF to report the result of an MIH_Scan request.

        - Semantics of the service primitive

The parameters of the primitive are as follows:

MIH_Scan.confirm 			(
			SourceIdentifier,
			ScanLinkIdentifier,
			ScanResponseSets,
			Status
			)

| Name | Type | Valid range | Description |
| --- | --- | --- | --- |
| Source Identifier | MIHF ID | N/A | This identifies the invoker of this primitive which can be either the local MIHF or a remote MIHF. |
| Scan Link Identifier | LINK ID | N/A | Identifier of the link scanned. |
| Scan Response Sets | LIST | N/A | A list of discovered PoAs and related information. See Table for detailed definition. |
| Status | ENUMER ATED | 0-4 | Status of operation: 0 Success 1 Unspecified Failure 2 Rejected 3 Authorization Failure 4 Network Error |

- Definition of scan response set

| Name | Type | Valid range | Description |
| --- | --- | --- | --- |
| PoA Identifier | MAC ADDRESS | N/A | The Link-Layer Address of PoA. |
| Signal Strength | INTEGER | 0-100 | Signal strength (in percentage) associ ated to the PoA |

        - When generated

This primitive returns the result of an MIH_Scan request to the requesting MIH User.

        - Effect of receipt

The MIH User can act on the scan result accordingly.

    - MIH_Net_HO_Candidate_Query

      - MIH_Net_HO_Candidate_Query.request

        - Function

This primitive is invoked by an MIH User to communicate with a peer MIH User about its intent of handover initiation. The handover may be initiated either by the mobile node or by the network.

        - Semantics of service primitive

The parameters of the primitive are as follows:

MIH_Net_HO_Candidate_Query.request 				(

DestinationIdentifier,

SuggestedNewLinkList,

HandoverMode,

OldLinkAction,

QueryResourceList
				)

Local or Remote: Remote

MIHF (Network) -> MIHF (Mobile Node)

| Name | Type | Valid range | Description |
| --- | --- | --- | --- |
| Destination Identifier | MIHF ID | N/A | This identifies a remote MIHF which will be the destination of this request. |
| Suggested New Link List | LIST | N/A | A list of {Link Identifier, PoA List} pairs, as defined in Table , suggesting the new access networks to which handover initiation should be considered. The access networks towards the top of the list are more preferable than those towards the bottom of the list. |
| Handover Mode | ENUMERATED | 0-1 | The handover mode may influence the manner in which links are prepared for handover. 0 Make-before-Break 1 Break-before-Make |
| Old Link Actions | BITMAP (32) | N/A | Specifies the suggested action on link during handover. Combination of the following choices are allowed. Bit 0: LINK_DISCONNECT Bit 1: LINK_LOW_POWER Bit 2: LINK_POWER_DOWN Bit 3: LINK_NO_ACTION Bit 4: LINK_RESOURCE_RETAIN Bit 5: DATA_FORWARDING_REQUEST Bit 6: LINK_POWER_UP Bit 7-31: (Reserved) |
| Query Resource List | LIST | N/A | List of resources to be queried at the new can didate network. (Editor’s note: need input) |

- Element of link list

| Name | Type | Valid range | Description |
| --- | --- | --- | --- |
| Link Identifier | LINK ID | N/A | This identifies an access network. |
| PoA List | LIST | N/A | This is a list of MAC addresses representing a list of PoAs (AP/BS) on the access network. The list is sorted from most preferred first to least pre ferred last. |

The following is the description of the link actions:

- LINK_DISCONNECT: Disconnect the link connection directly.

- LINK_LOW_POWER: Cause the link to adjust its RF power level to be low power consumption.

- LINK_POWER_DOWN: Cause the link to power down so as to stop transmitting and enter idle mode.

- LINK_NO_ACTION: The link need not take any action.

- LINK_RESOURCE_RETAIN: The link will be disconnected but the resource for the link connection still remains so reestablishing the link connection later can be more efficient.

- DATA_FORWARDING_REQUEST: This indication requires the buffered data at the old serving PoS entity to be forwarded to the new target PoS entity in order to avoid data loss. This action can be taken immediately after the old serving PoS receives the MIH_Handover_Commit.response message from the new target PoS.

- LINK_POWER_UP: TBD

        - When generated

This primitive is invoked by an upper layer entity, such as a handover policy engine, or L3 mobility protocol like Mobile IP, to communicate with a peer MIH User about its intent of handover initiation. Serving PoS requests MN to provide information about resources required to initiate a handover by setting the QueryResourceFlag parameter. [Editor’s note: sentence out of date: QueryResourceFlag now?]

        - Effect on receipt

Upon receipt of this primitive by the local MIHF, it shall generate and send an MIH_Net_HO_Candidate_Query request message to the remote MIHF identified by the Destination Identifier.

      - MIH_Net_HO_Candidate_Query.indication

        - Function

This primitive is generated by an MIHF to indicate to an upper layer entity that an MIH_Net_HO_Candidate_Query request message is received from a peer MIHF.

        - Semantics of service primitive

The parameters of the primitive are as follows:

MIH_Net_HO_Candidate_Query.indication (

SourceIdentifier,

SuggestedNewLinkList,

HandoverMode,

OldLinkAction,

QueryResourceList

)

| Name | Type | Valid range | Description |
| --- | --- | --- | --- |
| Source Identifier | MIHF ID | N/A | This identifies the invoker of this primitive which can be a remote MIHF. |
| Suggested New Link List | LIST | N/A | A list of {LINK ID, PoA List} pairs, as defined in Table , suggesting the new access networks to which handover initiation should be considered. The access networks towards the top of the list are more preferable than those towards the bottom of the list. |
| Handover Mode | ENUMER ATED | 0-1 | The handover mode may influence the manner in which links are prepared for handover. 0 Make-before-Break 1 Break-before-Make |
| Old Link Actions | BITMAP (32) | N/A | Specifies the suggested action on link during handover. Combination of the following choices are allowed. Bit 0: LINK_DISCONNECT Bit 1: LINK_LOW_POWER Bit 2: LINK_POWER_DOWN Bit 3: LINK_NO_ACTION Bit 4: LINK_RESOURCE_RETAIN Bit 5: DATA_FORWARDING_REQUEST Bit 6: LINK_POWER_UP Bit 7-31: (Reserved) |
| Query Resource List | LIST | N/A | List of resources to be queried at the new can didate network. |

        - When generated

This primitive is generated by an MIHF on receiving an MIH_Net_HO_Candidate_Query request message from a peer MIHF.

        - Effect on receipt

An MIH User receiving this indication should invoke an MIH_Net_HO_Candidate_Query.response primitive towards the remote MIHF indicated by the Source Identifier in the request message. If the recipient of this indication is a network-side upper layer, it may choose to generate MIH_N2N_HO_Query_Resources request primitives to exchange MIH_N2N_HO_Query_Resources messages with MIHFs in one or more candidate networks under consideration before invoking the MIH_Net_HO_Candidate_Query response primitive.

      - MIH_Net_HO_Candidate_Query.response

        - Function

This primitive is used by the MIHF on an MN to respond to an MIH_Net_HO_Candidate_Query.request from a peer MIHF in the network.

        - Semantics of service primitive

The parameters of the primitive are as follows:

MIH_Net_HO_Candidate_Query.response 				(
				DestinationIdentifier,
				CurrentLinkIdentifier,
				HandoverAck,
				PreferredLinkList,
				RequestedResourceSet,
				ErrorCode,
				Status
				)

Local or Remote: Remote

MIHF (Mobile Node) -> MIHF (Network)

| Name | Type | Valid range | Description |
| --- | --- | --- | --- |
| Destination Identifier | MIHF ID | N/A | This identifies a remote MIHF which will be the destination of this response. |
| Current Link Identifier | LINK ID | N/A | This identifies the current access network over which the command needs to be sent. This is valid only for remote commands which need to be sent to remote MIHFs. The command is then sent either at L2 or at L3. |
| Handover Ack | BOOLEAN | TRUE/FALSE | TRUE: Initiate Handover FALSE: Abort Handover If the handover has to be aborted then a reason code is provided |
| Preferred Link List | LIST | N/A | A list of {Link Identifier, PoA List} pairs, as defined in Table , suggesting the new access networks and PoAs to which handover initiation should be considered. This may be different than the networks that were suggested in the handover request. The list is sorted from most preferred first to least preferred last. |
| Requested Resource Set | LIST | N/A | Requested resources needed by the mobile terminal. (Editor’s note: need definition) |
| Error Code | ENUMERATED | N/A | Lists the reason for aborting/declining the handover request. (Editor’s note: need input) |
| Status | ENUMERATED | 0-4 | Status of operation: 0 Success 1 Unspecified Failure 2 Rejected 3 Authorization Failure 4 Network Error |

        - When generated

The MIHF responds with this primitive in response to an MIH_Net_HO_Candidate_Query.request command from a peer MIHF entity.

        - Effect on receipt

The MIHF shall send an MIH_Net_HO_Candidate_Query response message to the peer MIHF as indicated in the Destination Identifier.

      - MIH_Net_HO_Candidate_Query.confirm

        - Function

This primitive is generated by the MIHF to confirm that an MIH_Net_HO_Candidate_Query.response message is received from a peer MIHF.

        - Semantics of service primitive

The parameters of the primitive are as follows:

MIH_Net_HO_Candidate_Query.confirm (
					SourceIdentifier,
					CurrentLinkIdentifier,
					HandoverAck,
					PreferredLinkList,
					RequestedResourceSet,
					ErrorCode,
					Status
					)

| Name | Type | Valid range | Description |
| --- | --- | --- | --- |
| Source Identifier | MIHF ID | N/A | This identifies the invoker of this primitive which can be either the local MIHF or a remote MIHF. |
| Current Link Identi fier | LINK ID | N/A | This identifies the current access network over which the command needs to be sent. This is valid only for remote commands which need to be sent to remote MIHFs. The command is then sent either at L2 or at L3. |
| Handover Ack | BOOLEAN | TRUE/ FALSE | TRUE: Initiate Handover FALSE: Abort Handover If the handover has to be aborted then a reason code is provided |
| Preferred Link List | LIST | N/A | A list of {LINK ID, PoA List} pairs, as defined in Table , suggesting the new access networks to which handover initiation should be considered. This may be different than the networks that were suggested in the handover request. The list is sorted from most preferred first to least preferred last. |
| Requested Resource Set | LIST | N/A | Requested resources needed by the mobile termi nal. |
| Error Code | ENUMERATED | N/A | Lists the reason for aborting/declining the han dover request. (Editor’s note: can we merge this into Status?) |
| Status | ENUMERATED | 0-4 | Status of operation: 0 Success 1 Unspecified Failure 2 Rejected 3 Authorization Failure 4 Network Error |

        - When generated

This primitive is generated by the MIHF on receiving an MIH_Net_HO_Candidate_Query response message from a peer MIHF or when an MIH_Net_HO_Candidate_Query.request primitive has invoked at the local MIHF.

        - Effect on receipt

On receiving the primitive the entity which originally initiated the handover request may decide to carry out the handover or abort it based on the primitive. If the recipient of the primitive is an MN upper layer and in case the handover is to be executed, it may also provide information about a selected link identifier on the selected network with an MIH_Net_HO_Commit message. However, if Status does not indicate “Success”, the recipient shall ignore any other returned values and, instead, shall perform appropriate error handling.

    - MIH_MN_HO_Candidate_Query

      - MIH_MN_HO_Candidate_Query.request

        - Function

This primitive is used by MIH Users on a mobile node to inform MIHF to perform candidates query for a possible handover initiation and query if the QoS resources and/or IP address configuration method of the ongoing data sessions can be supported in the candidate networks. This primitive also includes the current IP configuration server address (e.g., DHCP server, FA IP address, AR IP address) when the current IP configuration method is included. MIHF may forward the message remotely to the peer MIHF in the network.

        - Semantics of service primitive

The parameters of the primitive are as follows:

MIH_MN_HO_Candidate_Query.request (
				DestinationIdentifier,
				CurrentLinkIdentifier,
				CandidateLinkList,
				QueryResourceList,
				IPConfigurationMethod,
				DHCPServerAddress,
				FAAddress,
				AccessRouterAddress
				)

Local or Remote: Remote

MIHF (Mobile Node) -> MIHF (Network)

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| Destination Identifier | MIHF ID | N/A | This identifies the local MIHF or a remote MIHF which will be the destination of this request. |
| Current Link Identi fier | LINK ID | N/A | This identifies the current access network over which the request needs to be sent. This is valid only for remote commands which need to be sent to remote MIHF. The command is then sent either at L2 or at L3. |
| Candidate Link List | LIST | N/A | A list of {LINK ID, PoA List} pairs, as defined in Table , identifying candidate networks to which handover needs to be initiated. The list is sorted from most preferred first to least preferred last. |
| Query Resource List | LIST | N/A | List of resources to be queried at candidate networks. |
| IP Configuration Method | BITMAP(32) | 0-31 | Current IP configuration method is included. Bit 0: IPv4 static configuration Bit 1: IPv4 dynamic configuration (DHCPv4) Bit 2. Mobile IPv4 with FA (FA-CoA) Bit 3: Mobile IPv4 without FA (Co-located CoA) Bit 4-10: (Reserved for IPv4 address configuration) Bit 11: IPv6 stateless address configuration Bit 12: IPv6 stateful address configuration (DHCPv6) Bit 13: IPv6 manual configuration Bit 14: No configuration allowed Bit 15-31: (Reserved) |
| DHCP Server Address | IP ADDRESS | N/A | IP address of current DHCP Server. It is only included when mobile node is using dynamic address configu ration |
| FA Address | IP ADDRESS | N/A | IP address of current Foreign Agent. It is only included when mobile node is using Mobile IPv4 |
| Access Router Address | IP ADDRESS | N/A | IP address of current Access Router. It is only included when mobile node is using IPv6 |

        - When generated

This primitive is generated by an upper layer entity such as a Handover Policy engine, L3 Mobility protocol like MIP, Application or some other entity in the mobile node that may want to query other candidate networks for a possible handover. MN uses the QueryResourceList parameter to notify the Serving PoS of the resource list which it is interested in checking at the candidate networks as part of this query. MN's upper layer entity generates this primitive when it wants to query IP address related information from the candidate networks before handover.

        - Effect on receipt

Upon receipt of this primitive by the local MIHF, it shall generate and send an MIH_MN_HO_Candidate_Query request message to the remote MIHF identified by the Destination Identifier.

      - MIH_MN_HO_Candidate_Query.indication

        - Function

This primitive is used by MIHF to indicate the receipt of MIH_MN_HO_Candidate_Query request message from a mobile node to the registered MIH-Users.

        - Semantics of service primitive

The parameters of the primitive are as follows:

MIH_MN_HO_Candidate_Query.indication (
				SourceIdentifier,
				CurrentLinkIdentifier,
				CandidateLinkList,
				QueryResourceList,
				IPConfigurationMethod,
				DHCPServerAddress,
				FAAddress,
				AccessRouterAddress
				)

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| Source Identifier | MIHF ID | N/A | This identifies the invoker of this primitive which can be a remote MIHF. |
| Current Link Identi fier | LINK ID | N/A | This identifies the current access network over which the request needs to be sent. This is valid only for remote commands which need to be sent to remote MIHF. The command is then sent either at L2 or at L3. |
| Candidate Link List | LIST | N/A | A list of {LINK ID, PoA List} pairs, as defined in Table , identifying candidate networks to which handover needs to be initiated. The list is sorted from most preferred first to least preferred last. |
| Query Resource List | LIST | N/A | List of resources to be queried at candidate networks. |
| IP Configuration Method | BITMAP(32) | 0-31 | Current IP configuration method is included. Bit 0: IPv4 static configuration Bit 1: IPv4 dynamic configuration (DHCPv4) Bit 2. Mobile IPv4 with FA (FA-CoA) Bit 3: Mobile IPv4 without FA (Co-located CoA) Bit 4~10: (Reserved for IPv4 address configuration) Bit 11: IPv6 stateless address configuration Bit 12: IPv6 stateful address configuration (DHCPv6) Bit 13: IPv6 manual configuration Bit 14: No configuration allowed Bit 15~31: (Reserved) |
| DHCP Server Address | IP ADDRESS | N/A | IP address of current DHCP Server. It is only included when mobile node is using dynamic address configu ration |
| FA Address | IP ADDRESS | N/A | IP address of current Foreign Agent. It is only included when mobile node is using Mobile IPv4 |
| Access Router Address | IP ADDRESS | N/A | IP address of current Access Router. It is only included when mobile node is using IPv6 |

        - When generated

This primitive is generated by MIHF on receiving MIH_MN_HO_Candidate_Query request message from a peer MIHF in a mobile node.

        - Effect on receipt

The MIH User receiving this primitive may generate an MIH_N2N_HO_Query_Resources.request primitive to exchange MIH_N2N_HO_Query_Resources messages with MIHF in one or more candidate networks under consideration before sending a corresponding MIH_MN_HO_Candidate_Query response message.

      - MIH_MN_HO_Candidate_Query.response

        - Function

This primitive is used by MIH Users to inform MIHF of the result of the candidate query request.

        - Semantics of service primitive

The parameters of the primitive are as follows:

MIH_MN_HO_Candidate_Query.response (
				DestinationIdentifier,
				CurrentLinkIdentifier,
				PreferredCandidateLinkList,
				AvailableResourceSet,
				IPConfigurationMethod,
				DHCPServerAddress,
				FAAddress,
				AccessRouterAddress,
				IPAddressInformationStatus,
				Status
				)

Local or Remote: Remote

MIHF (Mobile Node) <- MIHF (Network)

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| Destination Identifier | MIHF ID | N/A | This identifies a remote MIHF which will be the desti nation of this response. |
| Current Link Identi fier | LINK ID. | N/A | This identifies the current access network over which the command needs to be sent. This is valid only for remote commands which need to be sent to remote MIHF. |
| Preferred Candidate Link List | LIST | N/A | A list of {LINK ID, PoA List} pairs, as defined in Table , identifying candidate networks to which handover needs to be initiated. The list is sorted from most preferred first to least preferred last. |
| Available Resource Set | LIST | N/A | Containing a set of LinkIdentifier parameters and their corresponding available resource on candidate net work(s): LinkIdentifier AvailableResourceList |
| IP Configuration Method | BITMAP(32) | 0-31 | IP configuration Method on PreferredCandidatePoA List Bit 0: IPv4 static configuration Bit 1: IPv4 dynamic configuration (DHCPv4) Bit 2. Mobile IPv4 with FA (FA-CoA) Bit 3: Mobile IPv4 without FA (Co-located CoA) Bit 4~10: (Reserved for IPv4 address configuration) Bit 11: IPv6 stateless address configuration Bit 12: IPv6 stateful address configuration (DHCPv6) Bit 13: IPv6 manual configuration Bit 14: No configuration allowed Bit 15~31: (Reserved) |
| DHCP Server Address | IP ADDRESS | N/A | IP address of candidate DHCP Server. It is only included when dynamic address configuration is sup ported |
| FA Address | IP ADDRESS | N/A | IP address of candidate Foreign Agent. It is only included Mobile IPv4 is supported |
| Access Router Address | IP ADDRESS | N/A | IP address of candidate Access Router. It is only included IPv6 Stateless configuration is supported |
| IP Address Informa tion Status | BITMAP(8) | 0-8 | Bit 0: IP configuration Method is not available Bit 1: DHCP Server address is not available Bit 2: FA address is not available Bit 3: Access Router Address is not available Bit 4: No information is provided due to accessibility of same entity (FA, Access Router, DHCP Server, etc.) Bit 5-7: Reserved |
| Status | ENUMER ATED | 0-4 | Status of operation: 0 Success 1 Unspecified Failure 2 Rejected 3 Authorization Failure 4 Network Error |

        - When generated

The MIH User responds with this primitive in response to an earlier MIH_MN_HO_Candidate_Query.request command from a peer MIHF entity in mobile node after the exchange of MIH_N2N_HO_Query_Resources messages with the MIHF in the candidate networks.

        - Effect on receipt

On receiving the response the entity which originally initiated the candidate query request decides the suitability of the candidates based on the list, available resources, and the IP address related information on each network.

      - MIH_MN_HO_Candidate_Query.confirm

        - Function

This primitive is used by MIHF to inform MIH Users of the receipt of candidate query and IP address related information response.

        - Semantics of service primitive

The parameters of the primitive are as follows:

MIH_MN_HO_Candidate_Query.confirm (
				SourceIdentifier,
				CurrentLinkIdentifier,
				PreferredCandidateLinkList,
				AvailableResourceSet,
				IPConfigurationMethod,
				DHCPServerAddress,
				FAAddress,
				AccessRouterAddress,
				IP Address Information Status,
				Status
				)

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| Source Identifier | MIHF ID | N/A | This identifies the invoker of this primitive which can be a remote MIHF. |
| Current Link Identi fier | LINK ID. | N/A | This identifies the current access network over which the command needs to be sent. This is valid only for remote commands which need to be sent to remote MIHF. |
| Preferred Candidate Link List | LIST | N/A | A list of {LINK ID, PoA List} pairs, as defined in Table , identifying candidate networks to which han dover needs to be initiated. The list is sorted from most preferred first to least preferred last. |
| Available Resource Set | LIST | N/A | Containing a set of LinkIdentifier parameters and their corresponding available resource on candidate net work(s): LinkIdentifier AvailableResourceList |
| IP Configuration Method | BITMAP(32) | 0-31 | IP configuration Method on PreferredCandidatePoA List Bit 0: IPv4 static configuration Bit 1: IPv4 dynamic configuration (DHCPv4) Bit 2. Mobile IPv4 with FA (FA-CoA) Bit 3: Mobile IPv4 without FA (Co-located CoA) Bit 4~10: (Reserved for IPv4 address configuration) Bit 11: IPv6 stateless address configuration Bit 12: IPv6 stateful address configuration (DHCPv6) Bit 13: IPv6 manual configuration Bit 14: No configuration allowed Bit 15~31: (Reserved) |
| DHCP Server Address | IP ADDRESS | N/A | IP address of candidate DHCP Server. It is only included when dynamic address configuration is supported |
| FA Address | IP ADDRESS | N/A | IP address of candidate Foreign Agent. It is only included Mobile IPv4 is supported |
| Access Router Address | IP ADDRESS | N/A | IP address of candidate Access Router. It is only included IPv6 Stateless configuration is supported |
| IP Address Informa tion Status | BITMAP(8) | 0-8 | Bit 0: IP configuration Method is not available Bit 1: DHCP Server address is not available Bit 2: FA address is not available Bit 3: Access Router Address is not available Bit 4: No information is provided due to accessibility of same entity (FA, Access Router, DHCP Server, etc.) Bit 5-7: Reserved |
| Status | ENUMER ATED | 0-4 | Status of operation: 0 Success 1 Unspecified Failure 2 Rejected 3 Authorization Failure 4 Network Error |

        - When generated

This primitive is generated by MIHF on receiving MIH_MN_HO_Candidate_Query response message from a peer MIHF in the network.

        - Effect on receipt

On receiving the primitive the MIH User entity which originally initiated the candidate query request may decide to choose the candidate network for handover or abort it based on the information in the primitive. This primitive triggers MIH User to initiate the process to get IP address to be used in the target depending on the received candidate query response.

    - MIH_N2N_HO_Query_Resources

      - MIH_N2N_HO_Query_Resources.request

        - Function

This primitive is used by an MIHF on the serving network to communicate with its peer MIHF on the candidate network. This is used to prepare the new link resource for impending handover and query the available link resource and IP address related information of the candidate network.

        - Semantics of service primitive

The parameters of the primitive are as follows:

MIH_N2N_HO_Query_Resources.request 				(

DestinationIdentifier,

QueryResourceList,

IPConfigurationMethod,

DHCPServerAddress,

FAAddress,

AccessRouterAddress

)

Local or Remote: Remote

MIHF (Network) <> MIHF (Network)

| Name | Type | Valid range | Description |
| --- | --- | --- | --- |
| Destination Identifier | MIHF ID | N/A | This identifies a remote MIHF which will be the destination of this request. |
| Query Resource List | LIST | N/A | List of resources to be queried at the new candidate network. |
| IP Configuration Method | BITMAP(32) | 0-31 | Current IP configuration method is included. Bit 0: IPv4 static configuration Bit 1: IPv4 dynamic configuration (DHCPv4) Bit 2. Mobile IPv4 with FA (FA-CoA) Bit 3: Mobile IPv4 without FA (Co-located CoA) Bit 4~10: (Reserved for IPv4 address configuration) Bit 11: IPv6 stateless address configuration Bit 12: IPv6 stateful address configuration (DHCPv6) Bit 13: IPv6 manual configuration Bit 14: No configuration allowed Bit 15~31: (Reserved) |
| DHCP Server Address | IP ADDRESS | N/A | IP address of current DHCP Server. It is only included when mobile node is using dynamic address configu ration |
| FA Address | IP ADDRESS | N/A | IP address of current Foreign Agent. It is only included when mobile node is using Mobile IPv4 |
| Access Router Address | IP ADDRESS | N/A | IP address of current Access Router. It is only included when mobile node is using IPv6 |

        - When generated

This primitive is generated after receiving the MIH_Net_HO_Candidate_Query request message from the MIHF on the mobile node in the case of mobile-initiated handover. In the case of network-initiated handover, this primitive is generated after receiving the MIH_Net_HO_Candidate_Query.response message from the mobile node.

        - Effect on receipt

This peer MIHF on the candidate network identifies the link resource usage and prepares resources for the impending handover. A corresponding indication (MIH_N2N_HO_Query_Resources.indication) may be triggered to send to all subscribed MIH User entities in the local stack. The associated parameters for the generated indication are the same as those used in the command request.

      - MIH_N2N_HO_Query_Resources.indication

        - Function

The MIHF on the candidate network indicates that an MIH_N2N_HO_Query_Resources.request message is received from a peer MIHF on the serving network so that the upper layer entity can identify the link resource usage, prepare resources, and provide IP address related information for the impending handover.

        - Semantics of service primitive

The parameters of the primitive are as follows:

MIH_N2N_HO_Query_Resources.indication (
					SourceIdentifier,
					QueryResourceList,
					IPConfigurationMethod,
					DHCPServerAddress,
					FAAddress,
					AccessRouterAddress
					)

Local or Remote: Both

MIHF (Network) <> MIHF (Network)

| Name | Type | Valid range | Description |
| --- | --- | --- | --- |
| Source Identifier | MIHF ID | N/A | This identifies the invoker of this primitive which can be either the local MIHF or a remote MIHF. |
| Query Resource List | LIST | N/A | List of resources to be queried at the new candidate network. |
| IP Configuration Method | BITMAP(32) | 0-31 | Current IP configuration method is included. Bit 0: IPv4 static configuration Bit 1: IPv4 dynamic configuration (DHCPv4) Bit 2. Mobile IPv4 with FA (FA-CoA) Bit 3: Mobile IPv4 without FA (Co-located CoA) Bit 4~10: (Reserved for IPv4 address configuration) Bit 11: IPv6 stateless address configuration Bit 12: IPv6 stateful address configuration (DHCPv6) Bit 13: IPv6 manual configuration Bit 14: No configuration allowed Bit 15~31: (Reserved) |
| DHCP Server Address | IP ADDRESS | N/A | IP address of current DHCP Server. It is only included when mobile node is using dynamic address configu ration |
| FA Address | IP ADDRESS | N/A | IP address of current Foreign Agent. It is only included when mobile node is using Mobile IPv4 |
| Access Router Address | IP ADDRESS | N/A | IP address of current Access Router. It is only included when mobile node is using IPv6 |

        - When generated

This primitive is generated by MIHF when the MIHF on the candidate network receives MIH_N2N_HO_Query_Resources request message from a peer MIHF on the serving network.

        - Effect on receipt

The MIH User on the candidate network identifies the link resource usage and prepares resources for the impending handover. It also replies with MIH_N2N_HO_Query_Resources.response primitive.

      - MIH_N2N_HO_Query_Resources.response

        - Function

This primitive is used by an MIHF on the candidate network to communicate with its peer MIHF on the serving network which sent out an MIH_N2N_HO_Query_Resources request. This is used to respond with the result of any resource preparation for the impending handover and to notify the MIHF on the serving network of the link resource status of the candidate network. It is also used to provide IP address related information of the candidate networks.

        - Semantics of service primitive

The parameters of the primitive are as follows:

MIH_N2N_HO_Query_Resources.response 				(
				DestinationIdentifier,
				ResourceStatus,
				AvailableResourceSet,
				IPConfigurationMethod,
				DHCPServerAddress,
				FAAddress,
				AccessRouterAddress,
				IPAddressInformationStatus,
				Status
				)

Local or Remote: Both

MIHF (Network) <> MIHF (Network)

| Name | Type | Valid range | Description |
| --- | --- | --- | --- |
| Destination Identifier | MIHF ID | N/A | This identifies the local MIHF or a remote MIHF which will be the destination of this response. |
| Resource Status | BOOLEAN | N/A | Specifies whether requested resources are available or not at the new PoA. 0: Not Available 1: Available |
| Available Resource Set | LIST | N/A | Containing a set of LinkIdentifier parameters and their corresponding available resource on candidate network(s): LinkIdentifier AvailableResourceList [Editor’s note: need better def] |
| IP Configuration Method | BITMAP (32) | 0-31 | IP configuration Method on PreferredCandidatePoA List Bit 0: IPv4 static configuration Bit 1: IPv4 dynamic configuration (DHCPv4) Bit 2. Mobile IPv4 with FA (FA-CoA) Bit 3: Mobile IPv4 without FA (Co-located CoA) Bit 4~10: (Reserved for IPv4 address configuration) Bit 11: IPv6 stateless address configuration Bit 12: IPv6 stateful address configuration (DHCPv6) Bit 13: IPv6 manual configuration Bit 14: No configuration allowed Bit 15~31: (Reserved) |
| DHCP Server Address | IP ADDRESS | N/A | IP address of candidate DHCP Server. It is only included when dynamic address configuration is sup ported |
| FA Address | IP ADDRESS | N/A | IP address of candidate Foreign Agent. It is only included Mobile IPv4 is supported |
| Access Router Address | IP ADDRESS | N/A | IP address of candidate Access Router. It is only included IPv6 Stateless configuration is supported |
| IP Address Informa tion Status | BITMAP (8) | 0-8 | Bit 0: IP configuration Method is not available Bit 1: DHCP Server address is not available Bit 2: FA address is not available Bit 3: Access Router Address is not available Bit 4: No information is provided due to accessibility of same entity (FA, Access Router, DHCP Server, etc.) Bit 5-7: Reserved |
| Status | ENUMER ATED | 0-4 | Status of operation: 0 Success 1 Unspecified Failure 2 Rejected 3 Authorization Failure 4 Network Error |

        - When generated

The MIHF on the candidate network responds with this primitive in response to an MIH_N2N_HO_Query_Resources request from a peer MIHF entity on the serving network.

        - Effect on receipt

After receiving the response, the MIHF on the serving network may send MIH_Net_HO_Candidate_Query response to the mobile node.

      - MIH_N2N_HO_Query_Resources.confirm

        - Function

This primitive is generated by the MIHF on the serving network to respond with the result of any resource preparation for the impending handover and to notify the link resource status of the candidate network. It also carries IP address related information on the candidate networks to MIH Users on the serving network.

        - Semantics of service primitive

The parameters of the primitive are as follows:

MIH_N2N_HO_Query_Resources.confirm (
				SourceIdentifier,
				ResourceStatus,
				AvailableResourceSet,
				IPConfigurationMethod,
				DHCPServerAddress,
				FAAddress,
				AccessRouterAddress,
				IPAddressInformationStatus,
				Status
				)

Local or Remote: Both

MIHF (Network) <> MIHF (Network)

| Name | Type | Valid range | Description |
| --- | --- | --- | --- |
| Source Identifier | MIHF ID | N/A | This identifies the invoker of this primitive which can be either the local MIHF or a remote MIHF. |
| ResourceStatus | BOOLEAN | N/A | Specifies whether requested resources are available or not at the new PoA. 0: Not Available 1: Available |
| Available Resource Set | LIST | N/A | Containing a set of LinkIdentifier parameters and their corresponding available resource on candidate network(s): LinkIdentifier AvailableResourceList [Editor’s note: need better def] |
| IP Configuration Method | BITMAP (32) | N/A | IP configuration Method on PreferredCandidatePoA List Bit 0: IPv4 static configuration Bit 1: IPv4 dynamic configuration (DHCPv4) Bit 2. Mobile IPv4 with FA (FA-CoA) Bit 3: Mobile IPv4 without FA (Co-located CoA) Bit 4~10: (Reserved for IPv4 address configuration) Bit 11: IPv6 stateless address configuration Bit 12: IPv6 stateful address configuration (DHCPv6) Bit 13: IPv6 manual configuration Bit 14: No configuration allowed Bit 15~31: (Reserved) |
| DHCP Server Address | IP ADDRESS | N/A | IP address of candidate DHCP Server. It is only included when dynamic address configuration is supported |
| FA Address | IP ADDRESS | N/A | IP address of candidate Foreign Agent. It is only included Mobile IPv4 is supported |
| Access Router Address | IP ADDRESS | N/A | IP address of candidate Access Router. It is only included IPv6 Stateless configuration is supported |
| IP Address Infor mation Status | BITMAP(8) | N/A | Bit 0: IP configuration Method is not available Bit 1: DHCP Server address is not available Bit 2: FA address is not available Bit 3: Access Router Address is not available Bit 4: No information is provided due to accessibility of same entity (FA, Access Router, DHCP Server, etc.) Bit 5-7: Reserved |
| Status | ENUMER ATED | N/A | Status of operation: 0 Success 1 Unspecified Failure 2 Rejected 3 Authorization Failure 4 Network Error |

        - When generated

This primitive is generated by the MIHF when the MIHF on the serving network receives an MIH_N2N_HO_Query_Resources.response message from a peer MIHF on the candidate network.

        - Effect on receipt

After receiving this primitive, the upper layer entity on the serving network may send an MIH_Net_HO_Candidate_Query.response primitive to the MIHF in case it was indicated with MIH_Net_HO_Candidate_Query.indication primitive before. However, if Status does not indicate “Success”, the recipient shall ignore any other returned values and, instead, shall perform appropriate error handling.

    - MIH_Net_HO_Commit

      - MIH_Net_HO_Commit.request

        - Function

This primitive is used by a peer MIHF to communicate with the remote MIHF on the network. The primitive is used to request the peer MIHF the commitment to perform link handover actions based on selected choices for a candidate network and PoA.

        - Semantics of service primitive

The parameters of the primitive are as follows:

MIH_Net_HO_Commit.request 				(

DestinationIdentifier,
				LinkIdentifier(1),

LinkAction(1),

ExecutionTime(1),

...

LinkIdentifier(n),

LinkAction(n),

ExecutionTime(n)

)

Local or Remote: Both

MIHF (Network) ->MIHF (Mobile Node)

| Name | Type | Valid range | Description |
| --- | --- | --- | --- |
| Destination Identifier | MIHF ID | N/A | This identifies the local MIHF or a remote MIHF which will be the destination of this request. |
| Link Identifier (n) | LINK ID | N/A | Identifier of a present link |
| Link Actions (n) | BITMAP (32) | N/A | Specifies the suggested action on link during handover. Combination of the following choices are allowed. Bit 0: LINK_DISCONNECT Bit 1: LINK_LOW_POWER Bit 2: LINK_POWER_DOWN Bit 3: LINK_NO_ACTION Bit 4: LINK_RESOURCE_RETAIN Bit 5: DATA_FORWARDING_REQUEST Bit 6: LINK_POWER_UP Bit 7-31: (Reserved) |
| Execution Time (n) | INTEGER | 0-10000 | Time (in ms) elapsed before action needs to be taken, if Execution Time = 0 then the action is expected to be taken on receipt of this command. |

        - When generated

The MIH User generates this primitive to order specific handover actions on one or more links.

        - Effect on receipt

Upon receipt of the MIH_Net_HO_Commit.request, the MIHF issues one or more specific link action commands to instruct the relevant link layer technologies to perform the specified actions on the link or links, identified by the Link Identifier(s). The actions that need to be performed are signalled through the Link Action parameters. It is up to the MIHF to either wait for the Execution Time period before issuing the Link Action for immediate execution (i.e., with an Execution Time of zero value), or issue the Link Action immediately with the same Execution Time parameter and leave the future execution responsibility to the specific link layer technology.

A corresponding indication (MIH_Net_HO_Commit.indication) may be triggered to send to all subscribed MIH User entities in the local stack. The associated parameters for the generated indication are the same as those used in the command request.

      - MIH_Net_HO_Commit.indication

        - Function

This primitive is generated by a MIHF to indicate that an MIH_Net_HO_Commit request message has been received from a peer MIHF.

        - Semantics of service primitive

The parameters of the primitive are as follows:

MIH_Net_HO_Commit.indication (

SourceIdentifier,
				LinkIdentifier(1),

LinkAction(1),

ExecutionTime(1),

...

LinkIdentifier(n),

LinkAction(n),

ExecutionTime(n)

)

Local or Remote: Both

MIHF (Network) -> MIHF (Mobile Node)

| Name | Type | Valid range | Description |
| --- | --- | --- | --- |
| Source Identifier | MIHF ID | N/A | This identifies the invoker of this primitive which can be either the local MIHF or a remote MIHF. |
| Link Identifier (n) | LINK ID | N/A | Identifier of a present link |
| Link Actions (n) | BITMAP (32) | N/A | Specifies the suggested action on link during han dover. Combination of the following choices are allowed. Bit 0: LINK_DISCONNECT Bit 1: LINK_LOW_POWER Bit 2: LINK_POWER_DOWN Bit 3: LINK_NO_ACTION Bit 4: LINK_RESOURCE_RETAIN Bit 5: DATA_FORWARDING_REQUEST Bit 6: LINK_POWER_UP Bit 7-31: (Reserved) |
| Execution Time (n) | INTEGER | 0-10000 | Time (in ms) elapsed before action needs to be taken, if Execution Time = 0 then the action is expected to be taken on receipt of this command. |

        - When generated

This primitive is generated by an MIHF on receiving an MIH_Net_HO_Commit request message from a peer MIHF.

        - Effect on receipt

The MIH User receiving this primitive replies with an MIH_Net_HO_Commit.response primitive.

      - MIH_Net_HO_Commit.response

        - Function

This primitive is used by an MIHF to communicate with a peer MIHF from which an MIH_Net_HO_Commit request message is received. The primitive is used to communicate the response of a handover commit request.

        - Semantics of service primitive

The parameters of the primitive are as follows:

MIH_Net_HO_Commit.response 	(

DestinationIdentifier,

LinkIdentifier(1),

LinkActionResultCode(1),

...

LinkIdentifier(n),

LinkActionResultCode(n),

Status

)

Local or Remote: Both

MIHF (Mobile Node) <> MIHF (Network)

| Name | Type | Valid range | Description |
| --- | --- | --- | --- |
| Destination Identifier | MIHF ID | N/A | This identifies the local MIHF or a remote MIHF which will be the destination of this response. |
| Link Identifier (n) | LINK ID | N/A | Identifier of a present link |
| Link Actions Result Code (n) | ENUMERATED | 0-4 | Result of the actions specified in the Link_Action.request primitive. Result Codes are given as follows: 0 Resource Unavailable 1 Resource Busy 2 Resource Available 3 Abort Handover 4 Perform Handover |
| Status | ENUMERATED | 0-4 | Status of operation: 0 Success 1 Unspecified Failure 2 Rejected 3 Authorization Failure 4 Network Error |

        - When generated

This primitive is generated in response to an MIH_Net_HO_Commit.request primitive.

        - Effect on receipt

Upon receipt, the old Serving PoS is informed about the status of the previously issued command request so that it can react accordingly. For instance, it may start forwarding any pending or remaining packets to the new Target PoS. The MN recipient may initiate a handover process and begin the setting up of the new layer 2 connection. Similarly, the network recipient may determine that the handover procedure is in progress to the intended network.

      - MIH_Net_HO_Commit.confirm

        - Function

This primitive is generated by the MIHF to confirm that an MIH_Net_HO_Commit response message is received from a peer MIHF.

        - Semantics of service primitive

The parameters of the primitive are as follows:

MIH_Net_HO_Commit.confirm (
				SourceIdentifier,
				LinkIdentifier(1),
				LinkActionResultCode(1),
					...
				LinkIdentifier(n),
				LinkActionResultCode(n),
				Status
				)

Local or Remote: Both

MIHF (Mobile Node) <> MIHF (Network)

MIHF (Network) <> MIHF (Network)

| Name | Type | Valid range | Description |
| --- | --- | --- | --- |
| Source Identifier | MIHF ID | N/A | This identifies the invoker of this primitive which can be either the local MIHF or a remote MIHF. |
| Link Identifier (n) | LINK ID | N/A | Identifier of a present link |
| Link Actions Result Code (n) | ENUMERATED | 0-4 | Result of the actions specified in the Link_Action.request primitive. Result Codes are given as follows: 0 Resource Unavailable 1 Resource Busy 2 Resource Available 3 Abort Handover 4 Perform Handover |
| Status | ENUMERATED | 0-4 | Status of operation: 0 Success 1 Unspecified Failure 2 Rejected 3 Authorization Failure 4 Network Error |

        - When generated

This primitive is generated by the MIHF on receiving an MIH_Net_HO_Commit response message from a peer MIHF.

        - Effect on receipt

Upon receipt, the old serving PoS is informed about the status of the previously issued command request so that it can react accordingly. For instance, it may start forwarding any pending or remaining packets to the new target PoS. The MN recipient may initiate a handover process and begin the setting up of the new layer 2 connection. The network recipient may determine that the handover procedure is in progress to the intended network. However, if Status does not indicate “Success”, the recipient shall ignore any other returned values and, instead, shall perform appropriate error handling.

    - MIH_MN_HO_Commit

      - MIH_MN_HO_Commit.request

        - Function

This primitive is used by MIH Users on a mobile node to inform MIHF to commit the handover to a specific target network. MIHF may forward the message remotely to the peer MIHF in the network. The MN may, instead, continue the handover procedure utilizing IP mobility protocols after activating target L2 links natively.

        - Semantics of service primitive

The parameters of the primitive are as follows:

MIH_MN_HO_Commit.request (
				DestinationIdentifier,
				CurrentLinkIdentifier,
				TargetLinkIdentifier,
				TargetPoA,
				OldLinkActions
				)

Local or Remote: Remote

MIHF (Mobile Node) -> MIHF (Network)

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| Destination Identifier | MIHF ID | N/A | This identifies a remote MIHF which will be the desti nation of this request. |
| Current Link Identi fier | LINK ID | N/A | This identifies the current access network over which the command needs to be sent. This is valid only for remote commands which need to be sent to remote MIHF. |
| Target Link Identifier | LINK ID | N/A | This is the identifier target network to which handover needs to be initiated |
| Target PoA | MAC ADDRESS | N/A | This is the Target Point of Attachment List (AP/BS). [Editor’s note: need better def] |
| Old Link Actions | BITMAP(32) | N/A | Specifies the suggested action on link during handover. Combination of the following choices are allowed. Bit 0: LINK_DISCONNECT Bit 1: LINK_LOW_POWER Bit 2: LINK_POWER_DOWN Bit 3: LINK_NO_ACTION Bit 4: LINK_RESOURCE_RETAIN Bit 5: DATA_FORWARDING_REQUEST Bit 6: LINK_POWER_UP Bit 7-31: (Reserved) |

        - When generated

This primitive may be generated when a mobile node decides to perform a handover based on suggested choices for a candidate network and PoA.

        - Effect on receipt

Upon receipt of this primitive by the local MIHF, it shall generate and send an MIH_MN_HO_Commit request message to the remote MIHF identified by the Destination Identifier.

      - MIH_MN_HO_Commit.indication

        - Function

This primitive is used by MIHF to inform MIH Users locally that MIH_MN_HO_Commit request is received.

        - Semantics of service primitive

The parameters of the primitive are as follows:

MIH_MN_HO_Commit.indication (
				SourceIdentifier,
				CurrentLinkIdentifier,
				TargetLinkIdentifier,
				TargetPoA,
				OldLinkActions
				)

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| Source Identifier | MIHF ID | N/A | This identifies the invoker of this primitive which can be a remote MIHF. |
| Current Link Identi fier | LINK ID | N/A | This identifies the current access network over which the command needs to be sent. This is valid only for remote commands which need to be sent to remote MIHF. |
| Target Link Identifier | LINK ID | N/A | This is the identifier target network to which handover needs to be initiated |
| Target PoA | MAC ADDRESS | N/A | This is the Target Point of Attachment List (AP/BS). [Editor’s note: need better def] |
| Old Link Actions | BITMAP (32) | N/A | Specifies the suggested action on link during handover. Combination of the following choices are allowed. Bit 0: LINK_DISCONNECT Bit 1: LINK_LOW_POWER Bit 2: LINK_POWER_DOWN Bit 3: LINK_NO_ACTION Bit 4: LINK_RESOURCE_RETAIN Bit 5: DATA_FORWARDING_REQUEST Bit 6: LINK_POWER_UP Bit 7-31: (Reserved) |

        - When generated

This primitive may be generated when MIH_MN_HO_Commit request is received.

        - Effect on receipt

The recipient may identify that the mobile node may have started handover to the selected target network. A corresponding response (MIH_Net_HO_Commit.response) maybe triggered to acknowledge the receipt.

      - MIH_MN_HO_Commit.response

        - Function

This primitive is used by MIH Users to send a response to MIH_MN_HO_Commit request.

        - Semantics of service primitive

The parameters of the primitive are as follows:

MIH_MN_HO_Commit.response (
				DestinationIdentifier,
				CurrentLinkIdentifier,
				Status
				)

Local or Remote: Remote

MIHF (Network) -> MIHF (MN)

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| Destination Identifier | MIHF ID | N/A | This identifies a remote MIHF which will be the desti nation of this response. |
| Current Link Identi fier | LINK ID | N/A | This identifies the current access network over which the command needs to be sent. This is valid only for remote commands which need to be sent to remote MIHF. |
| Status | ENUMER ATED | 0-4 | Status of operation: 0 Success 1 Unspecified Failure 2 Rejected 3 Authorization Failure 4 Network Error |

        - When generated

This primitive may be generated when MIH_MN_HO_Commit request is received.

        - Effect on receipt

MN may now proceed with the next step in handover after taking any local link actions. A corresponding confirm is generated to the MIH Users.

      - MIH_MN_HO_Commit.confirm

        - Function

This primitive is used by MIHF to inform MIH Users locally that MIH_MN_HO_Commit response is received.

        - Semantics of service primitive

The parameters of the primitive are as follows:

MIH_MN_HO_Commit.confirm (
				SourceIdentifier,
				CurrentLinkIdentifier,
				Status
				)

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| Source Identifier | MIHF ID | N/A | This identifies the invoker of this primitive which can be either the local MIHF or a remote MIHF. |
| Current Link Identi fier | LINK ID. | N/A | This identifies the current access network over which the command needs to be sent. This is valid only for remote commands which need to be sent to remote MIHF. |
| Status | ENUMER ATED | 0-4 | Status of operation: 0 Success 1 Unspecified Failure 2 Rejected 3 Authorization Failure 4 Network Error |

        - When generated

This primitive may be generated when MIH_MN_HO_Commit response is received.

        - Effect on receipt

MN may now proceed with the next step in handover.

    - MIH_MN_HO_Complete

      - MIH_MN_HO_Complete.request

        - Function

This primitive may be optionally used by MIH Users to indicate the completion of MIH level handover aiding procedure.

        - Semantics of service primitive

The parameters of the primitive are as follows:

MIH_MN_HO_Complete.request (
				DestinationIdentifier,
				LinkIdentifier,
				HandoverResult
				)

Local or Remote: Remote

MIHF (MN) -> MIHF (Network)

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| Destination Identifier | MIHF ID | N/A | This identifies a remote MIHF which will be the destination of this request. |
| Link Identifier | LINK ID | N/A | This identifies the current access network over which the command needs to be sent. This is valid only for remote commands which need to be sent to remote MIHF. |
| Handover Result | ENUMERATED | 0-2 | This identifies the status of handover. 0: Success 1: Failure 2: Rejected |

        - When generated

This primitive may be generated when MIH level handover procedure is complete.

        - Effect on receipt

Upon receipt of this primitive by the local MIHF, it shall generate and send an MIH_MN_HO_Complete request message to the remote MIHF identified by the Destination Identifier.

      - MIH_MN_HO_Complete.indication

        - Function

This primitive is used by MIHF to inform MIH Users locally that MIH_MN_HO_Complete request is received.

        - Semantics of service primitive

The parameters of the primitive are as follows:

MIH_MN_HO_Complete.indication (
				SourceIdentifier,
				LinkIdentifier,
				HandoverResult
				)

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| Source Identifier | MIHF ID | N/A | This identifies the invoker of this primitive which can be either the local MIHF or a remote MIHF. |
| Link Identifier | LINK ID | N/A | This identifies the current access network over which the command needs to be sent. This is valid only for remote commands which need to be sent to remote MIHF. |
| Handover Result | ENUMERATED | 0-2 | This identifies the status of handover. 0: Success 1: Failure 2: Rejected |

        - When generated

This primitive may be generated when an MIH_MN_HO_Commit request message is received.

        - Effect on receipt

This indicates the completion of the MIH level mobile initiated handover. A corresponding response is generated.

      - MIH_MN_HO_Complete.response

        - Function

This primitive is used by MIH Users to send a response to MIH_MN_HO_Complete request.

        - Semantics of service primitive

The parameters of the primitive are as follows:

MIH_MN_HO_Complete.response (
				DestinationIdentifier,
				CurrentLinkIdentifier,
				Status
				)

Local or Remote: Both

MIHF (Network) -> MIHF (MN)

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| Destination Identifier | MIHF ID | N/A | This identifies the local MIHF or a remote MIHF which will be the destination of this response. |
| Current Link Identi fier | LINK ID | N/A | This identifies the current access network over which the command needs to be sent. This is valid only for remote commands which need to be sent to remote MIHF. |
| Status | ENUMERATED | 0-4 | Status of operation: 0 Success 1 Unspecified Failure 2 Rejected 3 Authorization Failure 4 Network Error |

        - When generated

This primitive may be generated when MIH_MN_HO_Complete request is received.

        - Effect on receipt

This indicates the completion of the MIH level handover aiding procedure.

      - MIH_MN_HO_Complete.confirm

        - Function

This primitive is used by MIHF to inform MIH Users locally that an MIH_MN_HO_Complete response message is received.

        - Semantics of service primitive

The parameters of the primitive are as follows:

MIH_MN_HO_Complete.confirm (
				SourceIdentifier,
				CurrentLinkIdentifier,
				Status
				)

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| Source Identifier | MIHF ID | N/A | This identifies the invoker of this primitive which can be either the local MIHF or a remote MIHF. |
| Current Link Identi fier | LINK ID | N/A | This identifies the current access network over which the command needs to be sent. This is valid only for remote commands which need to be sent to remote MIHF. |
| Status | ENUMERATED | 0-4 | Status of operation: 0 Success 1 Unspecified Failure 2 Rejected 3 Authorization Failure 4 Network Error |

        - When generated

This primitive may be generated when MIH_MN_HO_Commit response message is received.

        - Effect on receipt

This indicates the completion of the MIH level handover aiding procedure.

    - MIH_N2N_HO_Complete

      - MIH_N2N_HO_Complete.request

        - Function

This primitive is used by an MIH User in the network to communicate with a peer network MIH entity about the completion of handover operation.

        - Semantics of service primitive

The parameters of the primitive are as follows:

MIH_N2N_HO_Complete.request 	(
				DestinationIdentifier,
				CurrentLinkIdentifier,
				HandoverResult
				)

Local or Remote: Both

MIHF (Network) <> MIHF (Network)

| Name | Type | Valid range | Description |
| --- | --- | --- | --- |
| Destination Identifier | MIHF ID | N/A | This identifies the local MIHF or a remote MIHF which will be the destination of this request. |
| Current Link Identifier | LINK ID | N/A | This identifies the current access network over which the command needs to be sent. This parameter is included only for the case where the accompanying MIH_N2N_HO_Complete request message is transferred between the mobile node and the Serving PoS. |
| Handover Result | ENUMERATED | 0-2 | This identifies the status of handover. 0: Success 1: Failure 2: Rejected |

        - When generated

The MIH User invokes this primitive when handover operations to the new network PoA have been completed.

        - Effect on receipt

Upon receipt of this primitive by the local MIHF, it shall generate and send an MIH_N2N_HO_Complete request message to the remote MIHF identified by the Destination Identifier.

      - MIH_N2N_HO_Complete.indication

        - Function

This primitive is generated by the MIHF to indicate the status of the handover operation.

        - Semantics of service primitive

The parameters of the primitive are as follows:

MIH_N2N_HO_Complete.indication(

SourceIdentifier,

CurrentLinkIdentifier,

HandoverResult

)

| Name | Type | Valid range | Description |
| --- | --- | --- | --- |
| Source Identifier | MIHF ID | N/A | This identifies the invoker of this primitive which can be either the local MIHF or a remote MIHF. |
| Current Link Identifier | LINK ID | N/A | This identifies the current access network over which the command needs to be sent. This parameter is included only for the case where the accompanying MIH_N2N_HO_Complete request message is transferred between the mobile node and the Serving PoS. |
| Handover Result | ENUMER ATED | 0-2 | This identifies the status of handover. 0: Success 1: Failure 2: Rejected |

        - When generated

This primitive is generated by the MIHF on receiving an MIH_N2N_HO_Complete request message from a peer MIHF.

        - Effect on receipt

The MIH User receiving this primitive replies with an MIH_N2N_HO_Complete.response primitive.

      - MIH_N2N_HO_Complete.response

        - Function

This primitive is used to send a response to a handover complete request.

        - Semantics of service primitive

The parameters of the primitive are as follows:

MIH_N2N_HO_Complete.response (
				DestinationIdentifier,
				CurrentLinkIdentifier,
				ResourceStatus,
				Status
				)

Local or Remote: Both

MIHF (Network) <> MIHF (Network)

| Name | Type | Valid range | Description |
| --- | --- | --- | --- |
| Destination Identifier | MIHF ID | N/A | This identifies the local MIHF or a remote MIHF which will be the destination of this response. |
| Current Link Identifier | LINK ID | N/A | This identifies the current access network over which the command needs to be sent. This parameter is included only for the case where the accompanying MIH_N2N_HO_Complete response message is transferred between the mobile node and the Serving PoS. |
| Resource Status | BOOLEAN | TRUE/FALSE | TRUE: Retain resources FALSE: Release resources |
| Status | ENUMERATED | 0-4 | Status of operation: 0 Success 1 Unspecified Failure 2 Rejected 3 Authorization Failure 4 Network Error |

        - When generated

The MIHF responds with this primitive after processing the handover complete request.

        - Effect on receipt

Upon receipt, the MN MIHF may determine that the handover complete request was processed successfully. Upon receipt, the MIHF in the new network may forward this message to the MN.

      - MIH_N2N_HO_Complete.confirm

        - Function

This primitive is generated by the MIHF to confirm that an MIH_N2N_HO_Complete response message is received from a peer MIHF.

        - Semantics of service primitive

The parameters of the primitive are as follows:

MIH_N2N_HO_Complete.confirm 	(
				SourceIdentifier,
				CurrentLinkIdentifier,
				ResourceStatus,
				Status
				)

| Name | Type | Valid range | Description |
| --- | --- | --- | --- |
| Source Identi fier | MIHF ID | N/A | This identifies the invoker of this primitive which can be either the local MIHF or a remote MIHF. |
| Current Link Identifier | LINK ID | N/A | This identifies the current access network over which the command needs to be sent. This parameter is included only for the case where the accompanying MIH_N2N_HO_Complete response message is trans ferred between the mobile node and the Serving PoS. |
| Resource Status | BOOLEAN | TRUE/FALSE | TRUE: Retain resources FALSE: Release resources |
| Status | ENUMER ATED | 0-4 | Status of operation: 0 Success 1 Unspecified Failure 2 Rejected 3 Authorization Failure 4 Network Error |

        - When generated

This primitive is generated by MIHF on receiving MIH_N2N_HO_Complete response message from a peer MIHF.

        - Effect on receipt

Upon receipt, the MN upper layer entity may determine that the handover complete request was processed successfully. Upon receipt, the MIHF in the new network may forward this message to the MN. However, if Status does not indicate “Success”, the recipient shall ignore any other returned values and, instead, shall perform appropriate error handling.

    - MIH_Get_Information

      - MIH_Get_Information.request

        - Function

This primitive is used by an MIH User to request information from an MIH information server. The information query may be related to a specific interface, attributes to the network interface, as well as the entire network capability. The service primitive has the flexibility to query either a specific data within a network interface or extended schema of a given network.

        - Semantics of service primitive

The parameters of the primitive are as follows:

MIH_Get_Information.request 				(

InfoQueryType,

InfoQueryParameters,

MaxResponseSize

)

| Name | Type | Valid range | Description |
| --- | --- | --- | --- |
| Info Query Type | ENUMERATED | 1-4 | The type of query that is specified. It is expected that this field contains one of the query types that are known to be supported by the communicating peers via MIH capability discovery. Allowed types are: 1: TLV 2: RDF_DATA 3: RDF_SCHEMA_URL 4: RDF_SCHEMA |
| Info Query Parameters | (Varies depending on Query type specific parameters) | N/A | Query type specific parameters which indicate the type of information the MN may be interested in. |
| Max Response Size (optional) | INTEGER | 1-65,535 | This field specifies the maximum size of InfoQueryParameters in MIH_Get_Information response primitive in octets. If this field is not specified, the maximum size is set to 65,535. The actual maximum size forced by the IS server may be smaller than that specified by the IS client. |

InfoQueryType:

- TLV: When this InfoQueryType is specified, the InfoQueryParameters shall be a binary string which encodes Information Element TLVs that carry requests as defined in clause .

- RDF_DATA: When this InfoQueryType is specified, InfoQueryParameters is a string which contains a SPARQL (Protocol and RDF Query language) query [2] where the SPARQL query is supposed to contain an appropriate query for obtaining expected RDF/XML data.

- RDF_SCHEMA_URL: When this InfoQueryType is specified, InfoQueryParameters is a null string. This InfoQueryType is used for obtaining the URL of the extended schema.

- RDF_SCHEMA: When this InfoQueryType is specified, InfoQueryParameters carries either the URL of the extended schema the query originator wants to obtain or a null string when the URL of the extended schema is unknown.

        - When generated

This primitive is generated by an MIH User that is seeking to retrieve information.

        - Effect on receipt

The recipient will forward the query in an MIH message to the designated MIIS server. The MIIS server tries to interpret the query request and retrieve the specified information.

      - MIH_Get_Information.indication

        - Function

This primitive is generated by the MIHF to indicate that an MIH_Get_Information Request message is received from a peer MIHF.

        - Semantics of service primitive

The parameters of the primitive are as follows:

MIH_Get_Information.indication (

InfoQueryType,

InfoQueryParameters,

MaxResponseSize

)

| Name | Type | Valid range | Description |
| --- | --- | --- | --- |
| Info Query Type | ENUMERATED | 1-4 | The type of query that is specified. The following types are allowed: 1: TLV 2: RDF_DATA 3: RDF_SCHEMA_URL 4: RDF_SCHEMA |
| Info Query Parameters | (Varies depending on Query type specific parameters) | N/A | Query type specific parameters which indicate the type of information the client may be interested in. |
| Max Response Size (optional) | INTEGER | 1-65,535 | This field specifies the maximum size of InfoQuery Parameters in MIH_Get_Information response primi tive in octets. If this field is not specified, the maximum size is set to 65,535. The actual maximum size forced by the IS server may be smaller than that specified by the IS client. |

InfoQueryType:

- TLV: When this InfoQueryType is specified, the InfoQueryParameters shall be a binary string which encodes Information Element TLVs that carry requests as defined in clause .

- RDF_DATA: When this InfoQueryType is specified, InfoQueryParameters is a string which contains a SPARQL (Protocol and RDF Query language) query [2] where the SPARQL query is supposed to contain an appropriate query for obtaining expected RDF/XML data.

- RDF_SCHEMA_URL: When this InfoQueryType is specified, InfoQueryParameters is a null string. This InfoQueryType is used for obtaining the URL of the extended schema.

- RDF_SCHEMA: When this InfoQueryType is specified, InfoQueryParameters carries either the URL of the extended schema the query originator wants to obtain or a null string when the URL of the extended schema is unknown.

        - When generated

This primitive is generated by the MIHF on receiving an MIH_Get_Information request message from a peer MIHF.

        - Effect on receipt

The recipient tries to interpret the query request and retrieve the specified information. Once the information is retrieved, the recipient replies with the MIH_Get_Information.response primitive.

      - MIH_Get_Information.response

        - Function

This primitive is used by an MIH User to respond to an MIH_GET_Information.indication primitive.

        - Semantics of service primitive

The parameters of the primitive are as follows:

MIH_Get_Information.response (

InfoQueryType,

InfoQueryResponse,

InfoQueryStatus

)

| Name | Type | Valid range | Description |
| --- | --- | --- | --- |
| Info Query Type | ENUMERATED | 1-4 | The type of query that is specified. It is expected that this field contains one of the query types that are known to be supported by the communicating peers via MIH capability discovery. Allowed types are: 1: TLV 2: RDF_DATA 3: RDF_SCHEMA_URL 4: RDF_SCHEMA |
| Info Query Response | STRING | N/A | Report consisting of information requested by the MIH User |
| Info Query Status | ENUMERATED | 0-4 | Status of operation: 0 Success 1 Unspecified Error 2 Mismatch in supported query type 3 Authorization Failure 4 Network Error InfoQueryResponse contains meaningful data only when Status is ‘0’. |

InfoQueryType:

- TLV: When this InfoQueryType is specified, the InfoQueryParameters shall be a binary string which encodes Information Element TLVs that carry responses as defined in clause .

- RDF_DATA: When this InfoQueryType is specified, InfoQueryResponse is a string which contains a SPARQL (Protocol and RDF Query language) query response [2].

- RDF_SCHEMA_URL: When this InfoQueryType is specified, InfoQueryResponse is an URL of extended schema.

- RDF_SCHEMA: When this InfoQueryType is specified, InfoQueryResponse carries an extended schema.

        - When generated

This primitive is generated by an MIH User in response to an received MIH_Get_Information request.

        - Effect on receipt

The MIH User that requested the information tries to interpret the InfoQueryResponse and take suitable action. However, if Status does not indicate “Success”, the recipient shall ignore any other returned values and, instead, shall perform appropriate error handling.

      - MIH_Get_Information.confirm

        - Function

This primitive is generated by the MIH to respond to an MIH_GET_Information.request primitive.

        - Semantics of service primitive

The parameters of the primitive are as follows:

MIH_Get_Information.confirm 				(
				InfoQueryType,
				InfoQueryResponse,
				Status
				)

| Name | Type | Valid range | Description |
| --- | --- | --- | --- |
| Info Query Type | ENUMERATED | 1-4 | The type of query that is specified. It is expected that this field contains one of the query types that are known to be supported by the communicating peers via MIH capability discovery. Allowed types are: 1: TLV 2: RDF_DATA 3: RDF_SCHEMA_URL 4: RDF_SCHEMA |
| Info Query Response | STRING | N/A | Report consisting of information requested by the MIH User |
| Status | ENUMERATED | 0-4 | Status of operation: 0 Success 1 Unspecified Error 2 Mismatch in supported query type 3 Authorization Failure 4 Network Error InfoQueryResponse contains meaningful data only when Status is ‘0’. |

InfoQueryType:

- TLV: When this InfoQueryType is specified, the InfoQueryParameters shall be a binary string which encodes Information Element TLVs that carry responses as defined in clause .

- RDF_DATA: When this InfoQueryType is specified, InfoQueryResponse is a string which contains a SPARQL (Protocol and RDF Query language) query response [2].

- RDF_SCHEMA_URL: When this InfoQueryType is specified, InfoQueryResponse is an URL of extended schema.

- RDF_SCHEMA: When this InfoQueryType is specified, InfoQueryResponse carries an extended schema.

        - When generated

This primitive is generated by the MIHF on receiving an MIH_Get_Information Response message from a peer MIHF.

        - Effect on receipt

The MIH User that requested the information tries to interpret the InfoQueryResponse and takes suitable action. However, if Status does not indicate “Success”, the recipient shall ignore any other returned values and, instead, shall perform appropriate error handling.

  - MIH-3GLINK-SAP primitives

Primitives are a conceptual representation of functions that are invoked across protocol layers. MIH- 3GLINK-SAP primitives are part of a larger set of primitives that run from the MIHF to the relevant link layer technology across a conceptual SAP referred to as MIH-LINK-SAP. This is depicted in Table  .

  - MIH_NET_SAP primitives

    - MIH_TP_Connect

The primitives associated with the opening of a connection-oriented data transfer session are as follows:

- MIH_TP_Connect.request

- MIH_TP_Connect.indication

- MIH_TP_Connect.response

- MIH_TP_Connect.confirm

The MIHF uses the MIH_TP_Connect.request primitive to request that a connection-oriented data transfer session be opened with a remote MIHF instance. The transport service provider uses the MIH_TP_Connect.indication primitive to indicate the arrival of a request to open a connection-oriented data transfer session. The remote MIHF uses the MIH_TP_Connect.response primitive to signal acceptance (or rejection) of the connection-oriented data transfer session. The transport service provider uses the MIH_TP_Connect.confirm primitive to communicate the response received from the remote MIHF.

      - MIH_TP_Connect.request

        - Function

This primitive is the request for the establishment of a connection-oriented transport service.

        - Semantics

The primitive shall provide parameters as follows:

MIH_TP_Connect.request 				(

TransportType,

SourceAddress,

DestinationAddress

)

| Name | Type | Valid range | Description |
| --- | --- | --- | --- |
| Transport Type | ENUMERATED | 0-1 | Identifies the protocol layer specific transport option. 0: L2 1: L3 or higher layer protocol |
| Source Address | TRANSPORT ADDRESS | N/A | Protocol layer specific Transport Address of entity which has the Source MIHF |
| Destination Address | TRANSPORT ADDRESS | N/A | Protocol layer specific Transport Address of entity which has the Destination MIHF |

        - When generated

This primitive is used to request that a connection-oriented transport session be established with a remote MIHF instance.

        - Effect on receipt

The receipt of this primitive causes the selected transport service provider to attempt to establish a connection-oriented transport session with the remote MIHF identified in the request.

      - MIH_TP_Connect.indication

        - Function

This primitive is the indication for establishment of a connection-oriented transport session.

        - Semantics

The primitive shall provide parameters as follows:

MIH_TP_Connect.indication 				(

TransportType,

SourceAddress,

DestinationAddress

)

| Name | Type | Valid range | Description |
| --- | --- | --- | --- |
| Transport Type | ENUMERATED | 0-1 | Identifies the protocol layer specific transport option. 0: L2 1: L3 or higher layer protocol |
| Source Address | TRANSPORT ADDRESS | N/A | Protocol layer specific Transport Address of entity which has the Source MIHF |
| Destination Address | TRANSPORT ADDRESS | N/A | Protocol layer specific Transport Address of entity which has the Destination MIHF |

        - When generated

This primitive is passed from the transport service provider to the MIHF to indicate that a request to open a connection-oriented transport session has been received from a remote MIHF.

        - Effect on receipt

The receiving MIHF instance shall issue either an MIH_TP_Connect.response primitive to accept the connection, or an MIH_TP_Disconnect.request primitive to refuse the connection.

      - MIH_TP_Connect.response

        - Function

This primitive is used for the establishment of a connection-oriented transport service.

        - Semantics

The primitive shall provide parameters as follows:

MIH_TP_Connect.response 				(

TransportType,

SourceAddress,

DestinationAddress,
	Status

)

| Name | Type | Valid range | Description |
| --- | --- | --- | --- |
| Transport Type | ENUMERATED | 0-1 | Identifies the protocol layer specific transport option. 0: L2 1: L3 or higher layer protocol |
| Source Address | TRANSPORT ADDRESS | N/A | Protocol layer specific Transport Address of entity which has the Source MIHF |
| Destination Address | TRANSPORT ADDRESS | N/A | Protocol layer specific Transport Address of entity which has the Destination MIHF |
| Status | ENUMERATED | 0-4 | Status of operation: 0 Success 1 Unspecified Failure 2 Rejected 3 Authorization Failure 4 Network Error |

        - When generated

The remote MIHF uses this service primitive when a request to open a connection-oriented transport session is received.

        - Effect on receipt

The receipt of this primitive causes the selected transport service provider to issue an MIH_TP_Connect.confirm to a local MIHF.

      - MIH_TP_Connect.confirm

        - Function

This is primitive is used to confirm the establishment of a connection-oriented transport service.

        - Semantics

The primitive shall provide parameters as follows:

MIH_TP_Connect.confirm 				(

TransportType,

SourceAddress,

DestinationAddress,

Status

)

| Name | Type | Valid range | Description |
| --- | --- | --- | --- |
| Transport Type | ENUMERATED | 0-1 | Identifies the protocol layer specific transport option. 0: L2 1: L3 or higher layer protocol |
| Source Address | TRANSPORT ADDRESS | N/A | Protocol layer specific Transport Address of entity which has the Source MIHF |
| Destination Address | TRANSPORT ADDRESS | N/A | Protocol layer specific Transport Address of entity which has the Destination MIHF |
| Status | ENUMERATED | 0-4 | Status of operation: 0 Success 1 Unspecified Failure 2 Rejected 3 Authorization Failure 4 Network Error |

        - When generated

The transport service provider responds with this service primitive in response to an MIH_TP_Connect.request from a local MIHF. It is passed from the transport service provider to the MIHF to confirm a data transport over an acknowledged connection-oriented transport session.

        - Effect on receipt

The receiving MIHF instance may stop its retry timer for connecting to a remote MIHF.

    - MIH_TP_Disconnect

The primitives associated with closing of a connection-oriented data transfer session are as follows:

- MIH_TP_Disconnect.request

- MIH_TP_Disconnect.indication

The MIHF uses the MIH_TP_Disconnect.request primitive to request that a connection-oriented data transfer session be closed with a remote MIHF instance. The transport service provider uses the MIH_TP_Disconnect.indication primitive to indicate the arrival of a request to close a connection-oriented data transfer session.

      - MIH_TP_Disconnect.request

        - Function

This primitive is the request for initiating the closing of a connection-oriented transport session.

        - Semantics

The primitive shall provide parameters as follows:

MIH_TP_Disconnect.request 				(

TransportType,

SourceAddress,

DestinationAddress

)

| Name | Type | Valid range | Description |
| --- | --- | --- | --- |
| Transport Type | ENUMERATED | 0-1 | Identifies the protocol layer specific transport option. 0: L2 1: L3 or higher layer protocol |
| Source Address | TRANSPORT ADDRESS | N/A | Protocol layer specific Transport Address of entity which has the Source MIHF |
| Destination Address | TRANSPORT ADDRESS | N/A | Protocol layer specific Transport Address of entity which has the Destination MIHF |

        - When generated

This primitive is passed from the MIHF to the transport service provider to request that a connection-oriented transport session be closed with the remote MIHF.

        - Effect on receipt

The receipt of this primitive causes the selected transport service provider to attempt to close a connection- oriented transport session to the remote MIHF identified in the request.

      - MIH_TP_Disconnect.indication

        - Function

This primitive is the indication for the closing a connection-oriented transport session.

        - Semantics

The primitive shall provide parameters as follows:

MIH_TP_Disconnect.indication 				(

TransportType,

SourceAddress,

DestinationAddress

)

| Name | Type | Valid range | Description |
| --- | --- | --- | --- |
| Transport Type | ENUMERATED | 0-1 | Identifies the protocol layer specific transport option. 0: L2 1: L3 or higher layer protocol |
| Source Address | TRANSPORT ADDRESS | N/A | Protocol layer specific Transport Address of entity which has the Source MIHF |
| Destination Address | TRANSPORT ADDRESS | N/A | Protocol layer specific Transport Address of entity which has the Destination MIHF |

        - When generated

This primitive is passed from the transport service provider to the MIHF to indicate that a request to close a connection-oriented transport session has been received from a remote MIHF.

        - Effect on receipt

The receiving MIHF shall disconnect the connection.

    - MIH_TP_Reset

The primitives associated with the resetting of a connection-oriented data transfer session are as follows:

- MIH_TP_Reset.request

- MIH_TP_Reset.indication

- MIH_TP_Reset.response

- MIH_TP_Reset.confirm

The MIHF uses the MIH_TP_Reset.request primitive to request that a connection-oriented data transfer session be reset with a remote MIHF. The transport service provider uses the MIH_TP_Reset.indication primitive to indicate the arrival of a request to reset a connection-oriented data transfer session. The remote MIHF uses the MIH_TP_Reset.response primitive to signal the acceptance of resetting the connection-oriented data transfer session. The transport service provider uses the MIH_TP_Reset.confirm primitive to communicate the response received from the remote MIHF.

      - MIH_TP_Reset.request

        - Function

This primitive is the request to reset a connection-oriented transport session.

        - Semantics

The primitive shall provide parameters as follows:

MIH_TP_Reset.request 				(

TransportType,

SourceAddress,

DestinationAddress

)

| Name | Type | Valid range | Description |
| --- | --- | --- | --- |
| Transport Type | ENUMERATED | 0-1 | Identifies the protocol layer specific transport option. 0: L2 1: L3 or higher layer protocol |
| Source Address | TRANSPORT ADDRESS | N/A | Protocol layer specific Transport Address of entity which has the Source MIHF |
| Destination Address | TRANSPORT ADDRESS | N/A | Protocol layer specific Transport Address of entity which has the Destination MIHF |

        - When generated

This service primitive is used to request that a connection-oriented transport session be reset.

        - Effect on receipt

The receipt of this primitive causes the selected transport service provider to attempt to reset a connection- oriented transport session.

      - MIH_TP_Reset.indication

        - Function

This primitive is the indication to reset a connection-oriented transport service

        - Semantics

The primitive shall provide parameters as follows:

MIH_TP_Reset.indication 				(

TransportType,

SourceAddress,

DestinationAddress

)

| Name | Type | Valid range | Description |
| --- | --- | --- | --- |
| Transport Type | ENUMERATED | 0-1 | Identifies the protocol layer specific transport option. 0: L2 1: L3 or higher layer protocol |
| Source Address | TRANSPORT ADDRESS | N/A | Protocol layer specific Transport Address of entity which has the Source MIHF |
| Destination Address | TRANSPORT ADDRESS | N/A | Protocol layer specific Transport Address of entity which has the Destination MIHF |

        - When generated

This primitive is used to indicate that a request to reset a connection-oriented transport session has been received from a remote MIHF.

        - Effect on receipt

The receiving MIHF shall issue an MIH_TP_Reset.response primitive to reset the connection.

      - MIH_TP_Reset.response

        - Function

This is the response for the request to reset a connection-oriented transport session.

        - Semantics

The primitive shall provide parameters as follows:

MIH_TP_Reset.response 				(

TransportType,

SourceAddress,

DestinationAddress,
		Status

)

| Name | Type | Valid range | Description |
| --- | --- | --- | --- |
| Transport Type | ENUMERATED | 0-1 | Identifies the protocol layer specific transport option. 0: L2 1: L3 or higher layer protocol |
| Source Address | TRANSPORT ADDRESS | N/A | Protocol layer specific Transport Address of entity which has the Source MIHF |
| Destination Address | TRANSPORT ADDRESS | N/A | Protocol layer specific Transport Address of entity which has the Destination MIHF |
| Status | ENUMERATED | 0-4 | Status of operation: 0 Success 1 Unspecified Failure 2 Rejected 3 Authorization Failure 4 Network Error |

        - When generated

The MIHF responds with this service primitive to indicate that a request to reset a connection-oriented transport session has been received from a remote MIHF.

        - Effect on receipt

The receiving MIHF issues an MIH_TP_Reset.response primitive to accept resetting the connection.

      - MIH_TP_Reset.confirm

        - Function

This is the confirm primitive to reset a connection-oriented transport session.

        - Semantics

The primitive shall provide parameters as follows:

MIH_TP_Reset.confirm 				(

TransportType,

SourceAddress,

DestinationAddress,

Status

)

| Name | Type | Valid range | Description |
| --- | --- | --- | --- |
| Transport Type | ENUMERATED | 0-1 | Identifies the protocol layer specific transport option. 0: L2 1: L3 or higher layer protocol |
| Source Address | TRANSPORT ADDRESS | N/A | Protocol layer specific Transport Address of entity which has the Source MIHF |
| Destination Address | TRANSPORT ADDRESS | N/A | Protocol layer specific Transport Address of entity which has the Destination MIHF |
| Status | ENUMERATED | 0-4 | Status of operation: 0 Success 1 Unspecified Failure 2 Rejected 3 Authorization Failure 4 Network Error |

        - When generated

The MIHF responds with this service primitive in response to a reset request that has been received. It is passed from the transport service provider to the MIHF to reset a connection-oriented transport session.

        - Effect on receipt

The receiving MIHF issues an MIH_TP_Reset.response primitive to reset the connection.

    - MIH_TP_Data

The primitives associated with data transfers are as follows:

- MIH_TP_Data.request

- MIH_TP_Data.indication

- MIH_TP_Data.response

- MIH_TP_Data.confirm

The MIHF uses the MIH_TP_Data.request primitive to request that an MIH PDU be transported. The transport service provider uses the MIH_TP_Data.indication primitive to indicate the arrival of an MIH PDU. MIH_TP_Data.response and MIH_TP_Data.confirm primitives are used to acknowledge the receipt of the MIH PDU only when the sending MIHF requested the MIH PDU be sent reliably.

      - MIH_TP_Data.request

        - Function

This primitive is the request for transfer of an MIH PDU.

        - Semantics

The primitive shall provide parameters as follows:

The parameters of the primitive are as follows:

MIH_TP_Data.request 				(

TransportType,

SourceAddress,

DestinationAddress,

ReliableDeliveryFlag,

MIHProtocolPDU

)

| Name | Type | Valid range | Description |
| --- | --- | --- | --- |
| Transport Type | ENUMERATED | 0-1 | Identifies the protocol layer specific transport option. 0: L2 1: L3 or higher layer protocol |
| Source Address | TRANSPORT ADDRESS | N/A | Protocol layer specific Transport Address of entity which has the Source MIHF |
| Destination Address | TRANSPORT ADDRESS | N/A | Protocol layer specific Transport Address of entity which has the Destination MIHF |
| Reliable Delivery Flag | BOOLEAN | TURE FALSE | Indicate whether an acknowledgment is required. |
| MIH Protocol PDU | STRING | N/A | MIH Protocol PDU to be transferred |

        - When generated

This primitive is used to request that an MIH PDU be transported to a remote MIHF.

        - Effect on receipt

The receipt of this primitive causes the selected transport service provider to attempt to transport the MIH PDU.

      - MIH_TP_Data.indication

        - Function

This primitive is the indication for transfer of an MIH PDU.

        - Semantics

The primitive shall provide parameters as follows:

The parameters of the primitive are as follows:

MIH_TP_Data.indication 				(

TransportType,

SourceAddress,

DestinationAddress,

ReliableDeliveryFlag,

MIHProtocolPDU

)

| Name | Type | Valid range | Description |
| --- | --- | --- | --- |
| Transport Type | ENUMERATED | 0-1 | Identifies the protocol layer specific transport option. 0: L2 1: L3 or higher layer protocol |
| Source Address | TRANSPORT ADDRESS | N/A | Protocol layer specific Transport Address of entity which has the Source MIHF |
| Destination Address | TRANSPORT ADDRESS | N/A | Protocol layer specific Transport Address of entity which has the Destination MIHF |
| Reliable Delivery Flag | BOOLEAN | TURE FALSE | Indicate whether an acknowledgment is required. |
| MIH Protocol PDU | STRING | N/A | MIH Protocol PDU received. |

        - When generated

This primitive is used by the transport service provider to indicate that an MIH PDU has been received from a remote MIHF.

        - Effect on receipt

The receipt of this primitive causes the MIHF to receive the MIH PDU that was transported. If the Reliable Delivery Flag is set to TRUE, the MIHF shall respond with an MIH_TP_Data.response primitive.

      - MIH_TP_Data.response

        - Function

This primitive is used to notify the transport service provider of the receipt of an MIH PDU.

        - Semantics

The primitive shall provide parameters as follows:

MIH_TP_Data.response 			(
			TransportType,
			SourceAddress,
			DestinationAddress,
			Status
			)

| Name | Type | Valid range | Description |
| --- | --- | --- | --- |
| Transport Type | ENUMERATED | 0-1 | Identifies the protocol layer specific transport option. 0: L2 1: L3 or higher layer protocol |
| Source Address | TRANSPORT ADDRESS | N/A | Protocol layer specific Transport Address of entity which has the Source MIHF |
| Destination Address | TRANSPORT ADDRESS | N/A | Protocol layer specific Transport Address of entity which has the Destination MIHF |
| Status | ENUMERATED | 0-4 | Status of operation: 0 Success 1 Unspecified Failure 2 Rejected 3 Authorization Failure 4 Network Error |

        - When generated

The MIHF responds with this service primitive when a request to transfer an MIH PDU has been received from a remote MIHF.

        - Effect on receipt

The receipt of this primitive causes the selected transport service provider to issue an MIH_TP_Data.confirm to a local MIHF.

      - MIH_TP_Data.confirm

        - Function

This primitive is used to confirm an acknowledged transfer.

        - Semantics

The primitive shall provide parameters as follows:

MIH_TP_Data.confirm 				(
				TransportType,
				SourceAddress,
				DestinationAddress,
				Status
			 	)

| Name | Type | Valid range | Description |
| --- | --- | --- | --- |
| Transport Type | ENUMER ATED | 0-1 | Identifies the protocol layer specific transport option. 0: L2 1: L3 or higher layer protocol |
| Source Address | TRANSPORT ADDRESS | N/A | Protocol layer specific Transport Address of entity which has the Source MIHF |
| Destination Address | TRANSPORT ADDRESS | N/A | Protocol layer specific Transport Address of entity which has the Destination MIHF |
| Status | ENUMER ATED | 0-4 | Status of operation: 0 Success 1 Unspecified Failure 2 Rejected 3 Authorization Failure 4 Network Error |

        - When generated

This primitive is passed from the transport service provider to the MIHF to confirm that a request to transfer an MIH PDU succeeded.

        - Effect on receipt

Upon receipt of this primitive, the receiving MIHF may stop its retransmission timer for the corresponding request. When the MIHF does not receive this primitive for a pre-defined time after transmitting an MIH_TP_Data.request with ReliableDeliveryFlag set to TRUE, the MIHF may try to retransmit the MIH_TP_Data.request.

  - MIH_NMS_SAP primitives

MIH_NMS_SAP provides primitives for NMS to monitor and configure the MIHF.

    - MIH_NMS_Initialize

The following primitives are used to initialize the MIHF:

- MIH_NMS_Initialize.request

- MIH_NMS_Initialize.confirm

NMS sends the MIH_NMS_Initialize.request to set the MIHF to the initial working state. The MIHF responds with the MIH_NMS_Initialize.confirm primitive.

      - MIH_NMS_Initialize.request

        - Function

This primitive is used by NMS to initialize MIHF.

        - Semantics

This primitive contains no parameters.

MIH_NMS_Initialize.request 				(
				)

        - When generated

This primitive is generated by the NMS when it needs to initialize the MIHF.

        - Effect on receipt

MIHF executes initialization.

      - MIH_NMS_Initialize.confirm

        - Function

This primitive returns the result of the MIHF initialization.

        - Semantics

The parameter of the primitive is as follows:

MIH_NMS_Initialize.confirm   				(
				Status
				)

| Name | Type | Valid range | Description |
| --- | --- | --- | --- |
| Status | ENUMERATED | 0-4 | Result of MIHF initialization: 0 Success 1 Unspecified Failure 2 Rejected 3 Authorization Failure 4 Network Error |

        - When generated

This primitive is generated by the MIHF when the MIHF has processed an MIH_NMS_Initialize.request from the NMS.

        - Effect on receipt

NMS may confirm whether the MIHF works or not.

    - MIH_NMS_Get_State

The primitives associated with retrieval of MIHF state information are as follows:

- MIH_NMS_Get_State.request

- MIH_NMS_Get_State.confirm

NMS uses the MIH_NMS_Get_State.request primitive to get the state information from the MIHF. The MIHF uses the MIH_NMS_Get_State.confirm primitive to send the state information to the NMS.

The state information may include information as follows:

- Link level state information

- Subscribed link layer events

- Link Configuration Thresholds (Link Parameter Type, Initiate Action Threshold, Rollback Action Threshold, Execute Action Threshold)

- Link status (Up/Going Down/Down)

- Link Handover status (Imminent/Complete)

- Link Capability (Supported MIES/MICS List)

- Link Parameters (RSSI, NDR, C/I, BER, PER, Link Data Rate)

- Upper layer specific information

- MIH Capability (Supported MIES/MICS/MIIS List)

- MIH Status (Device Info, Operation Mode, Link ID, Channel ID, Battery Level, Link QoS Parameter List)

- MIH Handover status

- Transport specific information

- Connectivity of data transfer session (Connect/Disconnect)

      - MIH_NMS_Get_State.request

        - Function

This primitive is used by the NMS to get the state information.

        - Semantics

The parameter of the primitive is as follows:

MIH_NMS_Get_State.request   				(
				StateInformationRequestList
				)

| Name | Type | Valid range | Description |
| --- | --- | --- | --- |
| State Information Request List | LIST | N/A | List of the state information to be queried |

        - When generated

This primitive is generated by the NMS when it needs to get the state information from the MIHF.

        - Effect on receipt

The MIHF collects the requested information.

      - MIH_NMS_Get_State.confirm

        - Function

This primitive returns the state information to the NMS.

        - Semantics

The parameter of the primitive is as follows:

MIH_NMS_Get_State.confirm 				(
				StateInformationResponseList,
				Status
				)

| Name | Type | Valid range | Description |
| --- | --- | --- | --- |
| State Information Response List | LIST | N/A | A list of a pair of {state information; state information value}. |
| Status | ENUMER ATED | 0-4 | Status of operation: 0 Success 1 Unspecified Failure 2 Rejected 3 Authorization Failure 4 Network Error |

        - When generated

This primitive is generated by the MIHF when the MIHF receives an MIH_NMS_Get_State.request from the NMS.

        - Effect on receipt

The NMS may manage the MIHF based on the state information.

    - MIH_NMS_Reset

The following primitives are used to reset the MIHF:

- MIH_NMS_Reset.request

- MIH_NMS_Reset.confirm

The NMS sends the MIH_NMS_Reset.request to set the MIHF to the initial working state. The MIHF responds with the MIH_NMS_Reset.confirm primitive.

      - MIH_NMS_Reset.request

        - Function

This primitive is used by the NMS to reset the MIHF.

        - Semantics

This primitive has no parameters.

MIH_NMS_Reset.request ()

        - When generated

This primitive is generated by the NMS when it needs to reset the MIHF.

        - Effect on receipt

The MIHF resets and enters initial state.

      - MIH_NMS_Reset.confirm

        - Function

This primitive returns the result of resetting the MIHF.

        - Semantics

The parameter of the primitive is as follows:

MIH_NMS_Reset.confirm (
			 Status
			)

| Name | Type | Valid range | Description |
| --- | --- | --- | --- |
| Status | ENUMERATED | 0-4 | Result of the MIHF initialization: 0 Success 1 Unspecified Failure 2 Rejected 3 Authorization Failure 4 Network Error |

        - When Generated

This primitive is generated by the MIHF when the MIHF has processed an MIH_NMS_Reset.request from the NMS.

        - Effect on receipt

The NMS may confirm whether the MIHF has been reset or not and whether it is operational.

It is used to configure threshold values for Link_Parameters_Report. Thresholds are used for triggering reports. IEEE 802.11k primitives, MLME-MEASURE.request(local) and MLME-MREQUEST.request(remote), can be used that purpose. Only Beacon Request IE and QoS Metric IE can be used for setting thresholds and triggering reports. MLME-MEASURE primitive does not support confirmation to confirm the threshold setting results. It means that MLME-MEASURE primitive does not have the corresponding primitive to Link_Configure_Threshold.confirm. MLME-MEASURE.confirm is used to deliver the measurement results not to confirm the threshold setting.

IEEE 802.11k MLME-MEASURE.confirm and MLME-MREPORT.indication can be used. If MLME-MEA SURE.request or MLME-MREQUEST.request includes Beacon Request IE or QoS Metric IE, then MLME- MEASURE.confirm or MLME-MREPORT.indication is delivered to the MIHF when one of the reporting con ditions (thresholds) is satisfied. Link_Parameter_Report.indication can be also generated at a predefined regular interval determined by a user configurable time. This is also performed by MLME-MEASURE.request and MLME-MEASURE.confirm (local) or MLME-MREQUEST.request and MLME-MREPORT.indication (re mote) with measurement duration setting.