| Project | IEEE 802.21 MIHS <http://www.ieee802.org/21/> |  |
| --- | --- | --- |
| Title | Combined MIHF Discovery and MIH Capability Discovery |  |
| DCN | 21-07-0176-00-0000 |  |
| Date Submitted | 2007-05-08 |  |
| Source(s) | Jin Lee LG Eletronics, Inc. | Email : jins978@lge.com |
| Re: | IEEE 802.21 Session #20 in Montreal |  |
| Abstract | This contribution proposes broadcast MIHF ID for MIHF Capability message |  |
| Purpose | Discuss and adopt it |  |
| Notice | This document has been prepared to assist the IEEE 802.21 Working Group. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s). The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that IEEE 802.2 may make this contribution public. |  |
| Patent Policy | The contributor is familiar with IEEE patent policy, as outlined in Section 6.3 of the IEEE-SA Standards Board Operations Manual <http://standards.ieee.org/guides/opman/sect6.html#6.3> and in Understanding Patent Issues During IEEE Standards Development <http://standards.ieee.org/board/pat/guide.html>. |  |

1. Introduction

MIHF capability discovery is discovering the peer MIHF transport address, MIHF ID and MIHF capabilities at the same time. The reason why MIHF discovery uses MIHF capability discovery is after discovering MIHF it will send another message to discover MIHF capabilities. Since MIHF sends protocol message which consumes network bandwidth already in the process of MIHF discovery, it was decided by 802.21 working group to perform MIHF discovery and capability discovery with one message. This mechanism was described as combined MIH function discovery and capability discovery in Section 8.2.3.7.

And, in the combined MIHF and capability discovery mechanism, broadcast mechanism is used in both unsolicited MIH capability discovery and solicited MIH capability discovery since destination MIHF transport address and MIHF ID have not been discovered yet.

However, since we haven’t defined any MIHF ID value for broadcast problem related to this is identified. Therefore, I propose to define a dedicated MIHF ID value for broadcast so the transport of MIHF message becomes more comprehensible.

2. Suggested Changes

Remedy 1:

7.4.1.1 MIH_Capability_Discover.request

7.4.1.1.2 Semantics of service primitive

| Name | Type | Valid range | Description |
| --- | --- | --- | --- |
| Destination Identifier | MIHF ID | N/A | This identifies the local MIHF or a remote MIHF which will be the destination of this request. This is filled with broadcast MIHF ID except destination MIHF ID is pre-configured. |
| ….. | …. | … | … |

7.4.1.1.4 Effect on receipt

If the destination of the request is the local MIHF itself, the local MIHF shall respond  with MIH_Capability_Discover.confirm. if the destination of the request is a remote MIHF, the local MIHF shall generate and send a corresponding MIH_Capability_Discover request message to the remote MIHF. When MIHF ID is set to broadcast MIHF, the local MIHF shall generate and broadcast a corresponding MIH_Capability_Discover request message.

Remedy 2:

8.3.1 MIHF ID

| Type | Length | Value |
| --- | --- | --- |
| TYPE_HDR_TLV_MIHF | Variable | MIHF ID of the sender/receiver from/to where the MIHF frame originated/transported |

MIHF ID Value

-MIHF-ID shall be invariant even if the device obtains different interfaces or other components in its registration lifetime

-For example, MIHF-ID may be an FQCN or NAI of the sender of the registration request

-Broadcast MIHF-ID shall be used for MIHF and capability discovery when destination MIHF ID is unknown.

Remedy 3:

8.2.3.7 Combined MIH function discovery and capability discover at layer 2

As stated in 6.1.2, MIHF Discovery can be implicitly performed using the MIH Capability Discovery with broadcast MIHF ID at layer 2 when both MIHFs are residing in the same broadcast domain. The following sub clauses refer to the MIH Capability Discovery both as a means to discover the MIHF and its capabilities.