| Project | IEEE 802.21 MIHS <http://www.ieee802.org/21/> |  |
| --- | --- | --- |
| Title | Proposal about handover commit primitives |  |
| DCN | 21-07-0xxx-00-0000 |  |
| Date Submitted | April 30, 2007 |  |
| Source(s) | Junxiang Guo, Yan Peng |  |
| Re: | IEEE 802.21 Session #20 in Montreal |  |
| Abstract | This contribution proposes to add MIH_N2N_HO_Commit primitives/messages in current draft D5.0. |  |
| Purpose |  |  |
| Notice | This document has been prepared to assist the IEEE 802.21 Working Group. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s). The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that IEEE 802.2 may make this contribution public. |  |
| Patent Policy | The contributor is familiar with IEEE patent policy, as outlined in Section 6.3 of the IEEE-SA Standards Board Operations Manual <http://standards.ieee.org/guides/opman/sect6.html#6.3> and in Understanding Patent Issues During IEEE Standards Development <http://standards.ieee.org/board/pat/guide.html>. |  |

1. Background

After splitting of MIH handover messages, handover commit  messages between Serving PoS and Target PoS disappear, we think that it is needed to keep these two messages for at least the following reasons:

1. They can be used to inform the Target PoS about the movement of active MN from serving PoS to Target PoS, the benefit is the Target PoS can make some handover preparations in advance.

2. They can be used to carry relative context information if needed for pre-authentication and authorization, for example carrying user identity or security context etc., even the sequence numbers of in-sequence delivery packets for lossless handover.

Overall, there is no drawback of keeping these two messages exchange, but only benefits.

2. Proposal

Suggest to add MIH_N2N_HO_Commit primitives/messages as following text:

7.4.2X MIH_N2N_HO_Commit

7.4.2X.1 MIH_N2N_HO_Commit.request

7.4.2X.1.1 Function

This primitive is used by MIH Users on the serving network to inform MIHF to commit the handover to a specific target network, MIH function on the serving network may forward the message remotely to its peer MIH function on the target network in order to inform about the handover decision. The primitive is used to communicate commitment to perform handover based on selected network.

7.4.2X.1.2 Semantics of service primitive

The parameters of the primitive are as follows:

MIH_N2N_HO_Commit.request 				(

DestinationIdentifier,
				CurrentLinkIdentifier,

TargetLinkIdentifier,

OldLinkActions

)

Local or Remote: Remote

MIHF (Network) <> MIHF (Network)

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| Destination Identifier | MIHF ID | N/A | This identifies a remote MIHF which will be the destination of this request. |
| CurrentLinkIdentifier | Link ID | N/A | This identifies the current access network over which the command needs to be sent. This is valid only for remote commands which need to be sent to remote MIHF |
| TargetLinkIdentifier | Link ID | N/A | This is the identifier of target network to which handover needs to be initiated . This value includes the MAC address of target PoA. |
| OldLinkActions | BITMAP(32) | N/A | Specifies the suggested action on link dur ing handover. Combination of the below choices are allowed. Bit #0: LINK_DISCONNECT Bit #1: LINK_LOW_POWER Bit #2: LINK_POWER_DOWN Bit #3: LINK_NO_ACTION Bit #4: LINK_RESOURCE_RETAIN Bit #5: DATA_FORWARDING_REQUEST Bit #6: BI_CASTING_REQUEST Bit #7: HANDOVER_CANCEL Bit #8: LINK_POWER_UP Bit #9-31: Reserved |

7.4.2X.1.3 When generated

For the MN initiating handover case, this primitive is generated when successful MIH_MN_HO_Commit request/indication is received by MIH Users on serving PoS from MN to request to perform actual handover based on suggested choices for candidate network and PoA. For network initiating handover case, this primitive is generated when successful MIH_N2N_HO_Query_Resources response is received from candidate networks and at least one candidate network indicate in the response message that the resource status is available.

7.4.2X.1.4 Effect on receipt

Upon receipt of this primitive by the local MIHF, it shall generate and send an MIH_N2N_HO_Commit request message to the remote MIHF identified by the Destination Identifier.

7.4.2X.2 MIH_N2N_HO_Commit.indication

7.4.2X.2.1 Function

This primitive is used by MIH Function to inform MIH Users locally that MIH_N2N_HO_Commit request message is received.

7.4.2X.2.2 Semantics of service primitive

The parameters of the primitive are as follows:

MIH_N2N_HO_Commit.indication (

SourceIdentifier,
	CurrentLinkIdentifier,
	TargetLinkIdentifier,
	OldLinkActions
	)

Local or Remote: Local

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| Source Identifier | MIHF ID | N/A | This identifies the invoker of this primitive which can be a remote MIHF. |
| CurrentLinkIdentifier | Link ID | N/A | This identifies the current access network over which the command needs to be sent. This is valid only for remote commands which need to be sent to remote MIHF |
| TargetLinkIdentifier | Link ID | N/A | This is the identifier of target network to which handover needs to be initiated.This value includes the MAC address of target PoA.. |
| TargetPoA | MAC_ADDRESS | N/A | This is the target Point of Attachment (AP/BS). |
| OldLinkActions | BITMAP(32) | N/A | Specifies the suggested action on link dur ing handover. Combination of the below choices are allowed. Bit #0: LINK_DISCONNECT Bit #1: LINK_LOW_POWER Bit #2: LINK_POWER_DOWN Bit #3: LINK_NO_ACTION Bit #4: LINK_RESOURCE_RETAIN Bit #5: DATA_FORWARDING_REQUEST Bit #6: BI_CASTING_REQUEST Bit #7: HANDOVER_CANCEL Bit #8: LINK_POWER_UP Bit #9-31: Reserved |

7.4.2X.3 When generated

This primitive is generated by MIH Function on receiving MIH_N2N_HO_Commit request message from a peer MIH Function.

7.4.2X.4 Effect on receipt

The recipient may identify that the Serving PoS has decided to commit handover to the selected target network. A corresponding response (MIH_N2N_HO_Commit.response) may be triggered to acknowledge the receipt.

7.4.2X.3 MIH_N2N_HO_Commit.response

7.4.2X.3.1 Function

This primitive is used by MIH Users to send a response to MIH_N2N_HO_Commit request.

7.4.2X.3.2 Semantics of service primitive

The parameters of the primitive are as follows:

MIH_N2N_HO_Commit.response 	(

DestinationIdentifier,

CurrentLinkIdentifier,

TargetLinkIdentifier,

OldLinkActions,

HandoverStatus,

Status

)

Local or Remote: Both

MIHF (Network) <> MIHF (Network)

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| Destination Identifier | MIHF ID | N/A | This identifies a remote MIHF which will be the destination of this request. |
| CurrentLinkIdentifier | Link ID | N/A | This identifies the current access network over which the command needs to be sent. This is valid only for remote commands which need to be sent to remote MIHF |
| TargetLinkIdentifier | Link ID | N/A | This is the identifier of target network to which handover needs to be initiated.This value includes the MAC address of target PoA.. |
| OldLinkActions | BITMAP(32) | N/A | Specifies the suggested action on link during handover. Combination of the following choices are allowed. Bit #0: LINK_DISCONNECT Bit #1: LINK_LOW_POWER Bit #2: LINK_POWER_DOWN Bit #3: LINK_NO_ACTION Bit #4: LINK_RESOURCE_RETAIN Bit #5: DATA_FORWARDING_REQUEST Bit #6: BI_CASTING_REQUEST Bit #7: HANDOVER_CANCEL Bit #8: LINK_POWER_UP Bit #9-31: Reserved |
| HandoverStatus | Boolean | N/A | 0: Abort handover 1: Perform handover |
| Status | Enumerated | 0-4 | Status of operation: 0:Success 1:Unspecified Failure 2:Rejected 3:Authorization Failure 4:Network Error |

7.4.2X.3.3 When generated

This primitive maybe generated when MIH_N2N_HO_Candidate_Commit request is received.

7.4.2X.3.4 Effect on receipt

Upon receipt, the old Serving PoS may start forwarding any pending or remaining packets to the new Target PoS. A corresponding confirm is generated by local MIHF to be sent to the MIH Users.

7.4.2X.4 MIH_N2N_HO_Commit.confirm

7.4.2X.4.1 Function

This primitive is used by MIH Function to inform MIH Users that MIH_N2N_HO_Commit Response message is received from a peer MIH Function.

7.4.2X.4.2 Semantics of service primitive

The parameters of the primitive are as follows:

MIH_N2N_HO_Commit.confirm(
				SourceIdentifier,
				CurrentLinkIdentifier,
				TargetLinkIdentifier,
				OldLinkActions,
				HandoverStatus,
				Status
				)

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| Source Identifier | MIHF ID | N/A | This identifies the invoker of this primitive which can be either the local MIHF or remote MIHF. |
| CurrentLinkIdenti fier | Link ID | N/A | This identifies the current access network over which the command needs to be sent. This is valid only for remote commands which need to sent to remote MIHF |
| TargetLinkIdentifier | Link ID | N/A | This is the identifier of target network to which handover needs to be initiated . |
| OldLinkActions | Link Action (13) | Any valid value of Link Action (13) | Specifies the suggested action on link during han dover. Combination of the below choices are allowed. Bit #0: LINK_DISCONNECT Bit #1: LINK_LOW_POWER Bit #2: LINK_POWER_DOWN Bit #3: LINK_NO_ACTION Bit #4: LINK_RESOURCE_RETAIN Bit #5: DATA_FORWARDING_REQUEST Bit #6: BI_CASTING_REQUEST Bit #7: HANDOVER_CANCEL Bit #8-31: Reserved |
| HandoverStatus | Boolean | N/A | 0: Abort handover 1: Perform handover |
| Status | Enumerated | 0-4 | Status of operation: 0:Success 1:Unspecified Failure 2:Rejected 3:Authorization Failure 4:Network Error |

7.4.2X.4.3 When generated

This primitive may be generated when MIH_N2N_HO_Candidate_Commit response message is received from a peer MIH Function.

7.4.2X.4.4 Effect on receipt

Upon receipt, the old serving PoS may start forwarding any pending or remaining packets to the new target PoS. For MN initiating handover case, the MIH Users on the Serving PoS may trigger the local MIHF to send a MIH_MN_HO_Commit response message to the remote peer MIHF on MN. For network initiating handover case, the MIH Users on the Serving PoS may trigger the local MIHF to send a MIH_NET_HO_Commit request message to the remote MIHF on MN.. However, if Status indicates an error, the recipient shall ignore any other returned values and, instead, shall perform appropriate error handling.

8.6.3.X MIH_N2N_HO_Commit request

This message is used by MIHF on the Serving PoS to communicate with remote MIHF on the Target PoS the intent to commit a handover to a specific network.

MIH Header Fixed Fields (SID=3, Opcode=1, AID=xx)

Source Identifier = Sending MIHF ID (TLV type=0, format see 8.3.1)

Destination Identifier = Receiving MIHF ID (TLV type=1, format see 8.3.1)

CurrentLinkIdentifier (TLV type=2; format see 8.5.9)

TargetLinkIdentifier (TLV type=3; format see 8.5.9)

OldLinkActions (TLV type=4; format see 8.5.27)

8.6.3.Y MIH_N2N_HO_Commit response

This message is used by MIHF on the Target PoS to communicate with remote MIHF on the Serving PoS the response of request to commit a handover to a specific network.

MIH Header Fixed Fields (SID=3, Opcode=1, AID=xx)

Source Identifier = Sending MIHF ID (TLV type=0, format see 8.3.1)

Destination Identifier = Receiving MIHF ID (TLV type=1, format see 8.3.1)

CurrentLinkIdentifier (TLV type=2; format see 8.5.9)

TargetLinkIdentifier (TLV type=3; format see 8.5.9)

OldLinkActions (TLV type=4; format see 8.5.27)

HandoverStatus (TLV type=5; format see 8.5.35)

Status (TLV type=6; format see 8.5.7)

8.4.4 Command service messages

Table 51 – Command service messages

| MIH action | Service identifier (SID) | Opcode | Action identifier (AID) |
| --- | --- | --- | --- |
| MIH_Get_Link_Parameters | Command Service | Request, Response | 1 |
| MIH_Configure_Link | Command Service | Request, Response | 2 |
| MIH_Scan | Command Service | Request, Response | 3 |
| MIH_Net_HO_Candidate_Query | Command Service | Request, Response | 4 |
| MIH_MN_HO_Candidate_Query | Command Service | Request, Response | 5 |
| MIH_N2N_HO_Candidate_Query | Command Service | Request, Response | 6 |
| MIH_Net_HO_Commit | Command Service | Request, Response | 7 |
| MIH_MN_HO_Commit | Command Service | Request, Response | 8 |
| MIH_N2N_HO_Commit | Command Service | Request, Response | 9 |
| MIH_MN_HO_Complete | Command Service | Request, Response | 10 |
| MIH_N2N_HO_Complete | Command Service | Request, Response | 11 |