| Project | IEEE 802.21 MIHS <http://www.ieee802.org/21/> |  |
| --- | --- | --- |
| Title | Notify Mobility Management Protocol supported before handover |  |
| DCN | 21-07-0xxx-00-0000 |  |
| Date Submitted | July 9, 2007 |  |
| Source(s) | Yan PENG, Jing LIU, Junxiang GUO |  |
| Re: | IEEE 802.21 Session # 21 in San Francisco |  |
| Abstract | This contribution suggests notifying the MN or network about the Mobility Management Protocol supported, which would help handover decision, during the handover preparation phase instead of Link_Up.indication primitive. |  |
| Purpose |  |  |
| Notice | This document has been prepared to assist the IEEE 802.21 Working Group. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s). The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that IEEE 802.2 may make this contribution public. |  |
| Patent Policy | The contributor is familiar with IEEE patent policy, as outlined in Section 6.3 of the IEEE-SA Standards Board Operations Manual <http://standards.ieee.org/guides/opman/sect6.html#6.3> and in Understanding Patent Issues During IEEE Standards Development <http://standards.ieee.org/board/pat/guide.html>. |  |

# Introduction

In Link_Up.indication, the parameter Mobility Management Support is carried indicating the type of Mobility Management Protocol (MMP) supported by the new PoA.

Currently only four values are defined in this parameter: Mobile IPv4 with FA, Mobile IPv4 without FA, Mobile IPv6 and Mobile IPv6 with DHCPv6.

# Problem Statement

When receiving this Link_Up.indication primitive, the MN has established the L2 connection on the specified link interface. To achieve total connectivity of both L2 and L3, Mobility Management Protocol should be used to perform L3 handover, thus achieve L3 connectivity. However, if the MN and candidate network can’t support same L3 Mobility Management Protocol, the MN has to choose another candidate network to attach or rollback to the previous network, which results in increased handover failure ratio, and handover latency.

Supposing the following scenario:

There are three network operators: WiMAX, WLAN and UMTS. The WiMAX operator gets contract with the other two. Between WiMAX and WLAN, any MN which supports Simple IP or PMIP or MIP (v4 or v6) could enjoy seamless mobility service. However, only the MN which supports MIPv4 could enjoy seamless mobility service between WiMAX and UMTS. Now a MN is served by WiMAX operator. Because it's relative old, this MN can only support Simple IP and PMIP. Although the WiMAX operator have contract with the two networks, while handover decision making, this MN has to choose WLAN to achieve seamless handover.

In addition, there are some other mobility management protocols defined by IETF and widely used in different networks. Four values are not enough for MMP parameter.

# Suggested Remendy

In order to reduce the risk of the handover failure due to the mismatch of MMPs supported by the MN and target network, we propose to notify the MN about the supported MMPs before the MN attaches to the target network, (i.e. before Link_Up.indication). This info of candidate networks would help the MN to choose the best target network.

Using MIIS is a good method. We suggest adding MMP TLV to MIIS query type. Then the MN can choose the best target network according to the returned MMP(s) supported by each candidate networks.

In addition, MN could support several mobility management protocols (e.g. MN in WiMAX system). In the case of network-initiated handover, the serving network want to know the MMP attribute of MN to make select candidate networks. It’s not possible for the serving PoS to know that MMPs in MN. In this case carrying MMP attribute in MIH handover message is another useful method.

[Modify the text as suggested below: Page 105, line 58]

7.4.18.3 MIH_Net_HO_Candidate_Query.response

7.4.18.3.1 Function

7.4.18.3.2 Semantics of service primitive

MIH_Net_HO_Candidate_Query.response(

DestinationIdentifier,

CurrentLinkIdentifier,

PreferredLinkList,

RequestedResourceSet,

HandoverStatus,

MobilityManagementSupported,

Status

)

Parameters:

| Name | Type | Description |
| --- | --- | --- |
| Destination Identifier | MIHF_ID | This identifies a remote MIHF which will be the destination of this request. |
| Current Link Identifier | LINK_TUPLE_ ID | This identifies the current link. |
| Preferred Link List | LIST(LINK_POA_LIST) | A list of PoAs for each link, suggesting the new access networks and PoAs to which handover initiation should be considered. This may be different than the networks that were suggested in the handover request. The list is sorted from most preferred first to least preferred last. |
| Requested Resource Set | LIST(QOS_PARAM) | Requested resources needed by the mobile terminal. |
| Handover Status | HANDOVER_STATUS | Lists the reason for aborting/declining the handover request. |
| Mobility Management Supported | IP_MOBILITY_MGMT | (Optional)This indicates the type of Mobility Management Protocol supported by mobile terminal. |
| Status | STATUS | Status of operation. |

[Modify the text as suggested below: Page 106, line 52]

7.4.18.4 MIH_Net_HO_Candidate_Query.confirm

7.4.18.4.1 Function

7.4.18.4.2 Semantics of service primitiveMIH_Net_HO_Candidate_Query.confirm (

SourceIdentifier,

CurrentLinkIdentifier,

PreferredLinkList,

RequestedResourceSet,

HandoverStatus,

MobilityManagementSupported,

Status

)

Paramters:

| Name | Type | Description |
| --- | --- | --- |
| Source Identifier | MIHF_ID | Shall contain the MIHF ID of the mobile node that sent the MIH_Net_HO_Candidate_Query response message. |
| Current Link Identifier | LINK_TUPLE_ ID | This identifies the current link. |
| Preferred Link List | LIST(LINK_POA_LIST) | A list of PoAs, suggesting the new access networks to which handover initiation should be considered. This may be different than the networks that were suggested in the handover request. The list is sorted from most preferred first to least preferred last. |
| Requested Resource Set | LIST(QOS_PARAM) | Requested resources needed by the mobile terminal. |
| Handover Status | HANDOVER_STATUS | Lists the reason for aborting/declining the handover request. |
| Mobility Management Supported | IP_MOBILITY_MGMT | (Optional)This indicates the type of Mobility Management Protocol supported by mobile terminal. |
| Status | STATUS | Status of operation. |

[Modify the text as suggested below: Page 162, line 46]

8.6.3.8 MIH_Net_HO_Candidate_Query response

MIH Header Fields（SID=3, Opcode=2, AID=4）

SourceIdentifier = sending MIHF ID (Source MIHF ID TLV)

DestinationIdentifier =receiving MIHF ID (Destination MIHF ID TLV)

CurrentLinkIdentifier (Link Identifier TLV)

PreferredLinkList (List of Link PoA List TLV)

RequestedResourceSet (Handover Resource Response Set TLV)

HandoverStatus (Handover Status TLV)

MobilityManagementSupported (Optional) (Mobility Management Protocol TLV)

Status (Status TLV)

[Modify the text as suggested below: Page 202, line 4]

Annex C Type values for TLV encoding

(normative)

| TLV Type Name | TLV Type Value | Data Type |
| --- | --- | --- |
| ........ | ........ | ........ |
| Info Response RDF Schema URL List | 61 | LIST(INFO_RSP_SCHEMA_URL) |
| Info Response RDF Schema List | 62 | LIST(INFO_RSP_RDF_SCHEMA) |
| Mobility Management Protocol | 63 | IP_MOBILITY_MGMT |

[Modify  the text as suggested below: Page 49 Line 9]

| Access Network Specific Information Elements |  |  |
| --- | --- | --- |
| TYPE_IE_ACCESS_NETWORK_IDENTIFIER | Identifier for the access network. | ACCESS_NETWORK_ ID |
| TYPE_IE_NETWORK_AUX_ID | An auxiliary access network identifier. For an IEEE 802.11 network, this is the SSID of the network | NETWORK_SYSTEM_ ID |
| TYPE_IE_ROAMING_PARTNERS | Roaming Partners. Network Operators with which the current network operator has direct roaming agreements | ROAMING_PARTNERAS |
| TYPE_IE_COST | Cost. Indication of cost for service or network usage | COST |
| TYPE_IE_NETWORK_SECURITY | Security characteristics of the link layer | NETWORK_SECURITY |
| TYPE_IE_NETWORK_QOS | QoS characteristics of the link layer | NETWORK_QOS |
| TYPE_IE_NETWORK_DATA_RATE | Data Rate. The minimum and maximum value of data rate supported by the link layer of the access network | DATA_RATE |
| TYPE_IE_NETWORK_IP_CONFIG_METHODS | IP Configuration Methods supported by the access network | IP_CONFIG_METHODS |
| TYPE_IE_NETWORK_CAPABILITIES | Bitmap of access network capabilities | NETWORK_CAPABILITIES |
| TYPE_IE_LIST_SUPPORTED_LCP | List of location configuration protocols supported by the access network | SUPPORTED_LCP |
| TYPE_IE_MOBILITY_MANAGEMENT_PROTOCOL | The type of mobility management protocol supported by MN or access network | IP_MOBILITY_MGMT |

[Modify the text as suggested below: Page 187, line 26]

B.2.6 Data types for IP configuration

| Type Name | Derived From | Definition | Valid Range |
| --- | --- | --- | --- |
| IP_CONFIG_METHODS | BITMAP(32) | A set of IP configuration methods | Bit 0: IPv4 static configuration Bit 1: IPv4 dynamic configuration(DHCPv4) Bit 2: Mobile IPv4 with foreign agent(FA) care of address(CoA) (FA-CoA) Bit 3: Mobile IPv4 without FA(Co-located CoA) Bit 4-10: reserved for IPv4 address configurations Bit 11: IPv6 stateless address configuration Bit 12: IPv6 statefull address configuration(DHCPv6) Bit 13: IPv6 manual configuration Bit 14-31: Reserved |
| IP_MOBILITY_MGMT | BITMAP(16) | Indicates the supported mobility management protocols | Bit 0: Mobile IPv4 Bit 1: Mobile IPv4 Regional Registration Bit 2: Proxy Mobile IPv4 Bit 3: Mobile IPv6 Bit 4: Hierarchical Mobile IPv6 Bit 5: Proxy Mobile IPv6 Bit 6: Fast Mobile IPv6 Bit 7-15: (Reserved) |
| IP_CONFIG_STATUS | BITMAP(8) | Status of the IP configuration methods | Bit 0: IP configuration method is not available Bit 1: DHCP Server address is not available Bit 2: FA address is not available Bit 3: Access Router address is not available Bit 4: No information is provided due to accessibility of same entity(FA, Access Router, DHCP Server, etc) Bit 5-7: Reserved |
| IP_PREFIX_LEN | UNSIGNED_INT(1) | The length of an IP subnet prefix | 0-32 for IPv4 subnet 0-128 for IPv6 subnet |
| IP_SUBNET_INFO | SEQUENCE(IP_PREFIX_LEN, IP_ADDRESS) | Represent an IP subnet. The IP_PREFIX_LEN contains the bit length of the prefix of the subnet to which the IP_ADDRESS belongs | N/A |
| IP_RENEWAL_FLAG | BOOLEAN | Indicates whether MN’s IP address needs to changed or not | True: Change required False: Change not required |