| Project | IEEE 802.21 MIHS <http://www.ieee802.org/21/> |  |
| --- | --- | --- |
| Title | Clarify the PoS and PoA relationship |  |
| DCN | 21-07-0xxx-00-0000 |  |
| Date Submitted | July 12, 2007 |  |
| Source(s) | Junxiang Guo |  |
| Re: | IEEE 802.21 Session #21 in San Francisco |  |
| Abstract | This contribution proposes to add MIH PoS information in Access Network Container IE and carry the PoS and PoA relationship in MIH_MN_HO_Candidate.request/Request to make it possible for the serving PoS to identify the message destination. |  |
| Purpose |  |  |
| Notice | This document has been prepared to assist the IEEE 802.21 Working Group. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s). The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that IEEE 802.2 may make this contribution public. |  |
| Patent Policy | The contributor is familiar with IEEE patent policy, as outlined in Section 6.3 of the IEEE-SA Standards Board Operations Manual <http://standards.ieee.org/guides/opman/sect6.html#6.3> and in Understanding Patent Issues During IEEE Standards Development <http://standards.ieee.org/board/pat/guide.html>. |  |

1. Background

Currently, for MN initiated handover, MN will send a MIH_MN_HO_Candidate_Query Request message to its Serving PoS to initiate the handover, but maybe this message cannot get to the correct candidate PoS.  Let’s see the definition of this primitive first:

MIH_MN_HO_Candidate_Query.request (
				DestinationIdentifier,
				CurrentLinkIdentifier,
				CandidateLinkList,
				QueryResourceList,
				IPConfigurationMethod,
				DHCPServerAddress,
				FAAddress,
				AccessRouterAddress
				)

| Name | Type | Description |
| --- | --- | --- |
| Destination Identifier | MIHF_ID | This identifies a remote MIHF which will be the destination of this request. |
| Candidate Link List | LIST(LINK_POA_LIST) | A list of PoAs, identifying candidate networks to which handover needs to be initiated. The list is sorted from most preferred first to least preferred last. |
| …… | …… | …… |

What is the meaning of DestinationIdentifier? Is it the MIHF_ID of the Serving PoS or the Candidate PoSs?  If it is the Serving PoS, then when Serving PoS receives this message, how does it know the Candidate PoS(s) with which the PoAs contained in this message is associated. If it is the Candidate PoS(s), but as the Candidate PoS(s) may be several, one DestinationIdentifier is not enough to represent multiple Candidate PoSs.

Let’s explain this problem more clearly. As the below figure show, AP1 and AP2 have connectivity with Candidate PoS 1 (WLAN AR), BS1 and BS2 have connectivity with Candidate PoS 2 (WiMAX ASN-GW). MN is now connected to AP0 served by Serving PoS, MN decides to handover and collects information about neighboring PoAs of AP1, AP2, BS1, BS2, then it will send a MIH_MN_HO_Candidate_Query Request message to its Serving PoS to initiate the handover, this message carry a DestinationIdentifier and a list of PoAs comprises AP1, AP2, BS1, BS2. When Serving PoS receive this message, if the DestinationIdentifier is the MIHF_ID of itself, as there is no information in this message about the relationship between Candidate PoS(s) and APs/BSs, the Serving PoS does not know to which Candidate PoS (denoted by DestinationIdentifier)  the AP1,AP2,BS1,BS2 belongs respectively, so it cannot identify the destination. However, if the DestinationIdentifier is the MIHF_ID of Candidate PoS, as there is more than one Candidate PoS involved(Candidate PoS1, Candidate PoS 2),  the Serving PoS also does not know to which Candidate PoS the message should be destined.

2. Proposal

As the above said, the problem is caused by lack of information about the relation between Candidate PoSs and PoAs, especially in the case of Candidate PoS and PoA are separately located, as well as one candidate PoS has connection with several PoAs.  Due to this reason MN cannot get this information from IS server and it is impossible for MN to carry this information to the Serving PoS.

2.1 Propose to modify Access Network Container IE

So it is needed to add the information of the relationship between Candidate PoSs and PoAs in IE containers, suggest to modify Access Network Container in section 6.4.5.1 like below:

- IE containers

- Access Network Container - contains all the information depicting an access network. 

An Access Network Container shall contain at least one PoA Container and may contain one or more Vendor Specific Network IEs:

| Type=TYPE_IE_CONTAINER_NETWORK | Length= Variable |
| --- | --- |
| MIH PoS Identifier IE |  |
| Network Type IE |  |
| Operator Identifier IE |  |
| Service Provider Identifier IE (optional) |  |
| Access Network Identifier IE (optional) |  |
| Network System ID IE (optional) |  |
| Roaming Partners IE (optional) |  |
| Cost IE (optional) |  |
| Network Security IE (optional) |  |
| Network QoS IE (optional) |  |
| Network Data Rate IE (optional) |  |
| Network IP Configuration Methods IE (optional) |  |
| Network Capabilities IE (optional) |  |
| List of Supported LCP IE (optional) |  |
| PoA Container #1 |  |
| PoA Container #2 (optional) |  |
| … |  |
| PoA Container #k (optional) |  |
| Vendor Specific Network IE (optional) |  |

2.2 Propose to modify LINK_POA_LIST

When MN initiates handover, it can first get the information about the PoAs and the MIH PoSs to which these PoAs are pertaining from the IS server. After this step, MN can send the MIH_MN_HO_Candidate_Query Request message to the serving PoS which contains the list of PoAs and the corresponding MIH PoSs via the following modification.

- Data types for link identification and manipulation

The data types defined in this subclause are used for representing attributes for identification and manipulation of links.

| Type Name | Derived From | Definition | Valid Range |
| --- | --- | --- | --- |
| LINK_TYPE | UNSIGNED_INT(4) | Represents the link type. | 0: Reserved 1: Wireless - GSM 2: Wireless - GPRS 3: Wireless - EDGE 15: Ethernet 18: Wireless - Other 19: Wireless - IEEE 802.11 22: Wireless - CDMA2000 23: Wireless - UMTS 24: Wireless - cdma2000-HRPD 27: Wireless - IEEE 802.16 28: Wireless - IEEE 802.20 29: Wireless - IEEE 802.22 |
| LINK_ID | SEQUENCE( LINK_TYPE MAC_ADDRESS) | The identifier of a link that is not associated with any PoA. The MAC_ADDRESS contains a MAC address of MN. This may be used for the current link being used by MN or the link that was used by MN before handover. | N/A |
| LINK_TUPLE_ID | SEQUENCE( LINK_ID, CHOICE( MAC_ADDRESS, NULL)) | The identifier of a link that may be associated with a PoA. The optional MAC_ADDRESS contains a MAC address of PoA. | N/A |
| LINK_POA_LIST | SEQUENCE( LINK_ID, LIST( PoS_MAC_ADDRESS, PoA_MAC_ADDRESS) ) | A list of PoAs and MIH PoSs to which these PoAs are pertaining for a particular link. The LIST( PoS_MAC_ADDRESS, PoA_MAC_ADDRESS) is a list of PoA MAC address and its corresponding PoS MAC address and is sorted from most preferred first to least preferred last. | N/A |
| … | … | … | … |