| Project | IEEE 802.21 MIHO <http://www.ieee802.org/21/> |  |
| --- | --- | --- |
| Title | MIH Discovery Simplification |  |
| Date Submitted | June 10, 2007 |  |
| Source(s) | Yoshihiro Ohba |  |
| Re: | IEEE 802.21 Session #20.5 in June 2007 |  |
| Abstract | This document addresses LB#1d Issue 1 (MIH capability Discover request primitive removing the parameters) which is related to comment #5238 and #5237. |  |
| Purpose | LB#1d comment resolution |  |
| Notice | This document has been prepared to assist the IEEE 802.21 Working Group. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s). The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.21. |  |
| Patent Policy | The contributor is familiar with IEEE patent policy, as outlined in Section 6.3 of the IEEE-SA Standards Board Operations Manual <http://standards.ieee.org/guides/opman/sect6.html#6.3> and in Understanding Patent Issues During IEEE Standards Development <http://standards.ieee.org/board/pat/guide.html>. |  |

[NOTE: This contribution needs to cooperate with Jin Lee on issues 11, comment 5069.]

# Problem

## Unnecessary primitive (LB#1d Issue 2, Cmt 5242, 5244, 5245)

It is questionable as to why MIH_Capability_Discovery.indication is needed. The local MIHF that receives an MIH_Capability_Discovery request message should be able to generate an MIH_Capability_Discovery response response message without generating MIH_Capability_Discovery.indication primitive to the local MIH User.

# Proposed Remedy

## Remedy

Change Section 7.4.1.2.3 as follows:

7.4.1.2.3 When generated

This primitive is used by an MIHF to notify an MIH User when an MIH_Capability_Discover request message is received. The MIHF may also immediately return an MIH_Capability_Discover response message without generating this primitive to the MIH User.