| Project | IEEE 802.21 MIHO <http://www.ieee802.org/21/> |  |
| --- | --- | --- |
| Title | Comment Resolution Input for IEEE 802.21 D.71 Comment 643 |  |
| Date Submitted | Nov 9, 2007 |  |
| Source(s) | G. Scott Henderson |  |
| Re: | Table B-13 NETWORK_TYPE_INCLUSION |  |
| Abstract | Comment Resolution Input for IEEE 802.21 D.71 Comment 643 |  |
| Purpose | Reasoning and proposed solution for resolution of comment 643. |  |
| Notice | This document has been prepared to assist the IEEE 802.21 Working Group. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s). The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.21. |  |
| Patent Policy | The contributor is familiar with IEEE patent policy, as outlined in Section 6.3 of the IEEE-SA Standards Board Operations Manual <http://standards.ieee.org/guides/opman/sect6.html#6.3> and in Understanding Patent Issues During IEEE Standards Development <http://standards.ieee.org/board/pat/guide.html>. |  |

Date: Nov 8, 2007

Comment Resolution Input for IEEE 802.21 D.71 Comment 643

# Discussion on Comment 643

The current format for Network Type, as related in Table B-13, is not sufficiently granular to support real world implementations.  For example, 3GPP version 4 networks can contain selective features from version 6.  These features may be deployed by different carriers at different times and may be sufficient to differentiate which network a user will want to select.  As another example, a carrier may wish to deploy promotional features for a short time or only in one specific geographic area.  Permanently assigning a value for every possible permutation of feature sets and or short term deployments is likely to cause considerable administrative burden, restrict flexibility for the carriers, and potentially result in unreliable or incorrect data values.

# Proposed Solution

Assign a single value per Network Type as a tag field for identifying a generic extension container message.  When this value is set, the user would look for and interpret a technology specific and/or carrier specific container for additional information about services/features available from that carrier and location.

In this suggested solution, the user MIHF would first look for the general network type (e.g. Link Type) and then Revision.  If the revision field indicated the value for an extension field, the user MIHF would then look at the generic extension container.

The generic extension container would be of unfixed length and format, with the content and format provided by the supporting carrier.