| Project | IEEE 802.21 MIHS <http://www.ieee802.org/21/> |  |
| --- | --- | --- |
| Title | List of IEs from TGu |  |
| DCN | 21-07-0445-00-0000 |  |
| Date Submitted | November 2007 |  |
| Source(s) | Stephen McCann |  |
| Re: | IEEE 802.21 Session #23 in Atlanta, GA |  |
| Abstract | This contribution proposes resolution to SB comments related to the List of IEs |  |
| Purpose | Provide list of IEs. |  |
| Notice | This document has been prepared to assist the IEEE 802.21 Working Group. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s). The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that IEEE 802.21 may make this contribution public. |  |
| Patent Policy | The contributor is familiar with IEEE patent policy, as stated in Section 6 of the IEEE-SA Standards Board bylaws <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and in Understanding Patent Issues During IEEE Standards Development http://standards.ieee.org/board/pat/faq.pdf |  |

- HESSID Information element

The Homogenous Extended Service Set ID (HESSID) Information element contains description of HESSID. The format of the HESSID element is shown in Figure u1.

|  | Element ID | Length | HESSID | Network Type |
| --- | --- | --- | --- | --- |
| Octets: | 1 | 1 | 6 | 1 |

Figure u1—HESSID Information element format

The Element ID field is equal to the HESSID value in Table 26.

The value of the Length field is 7.

The HESSID field specifies the definition of HESSID.

In the infrastructure mode, the HESSID definition includes the BSSID value of one of the APs in the ESS. The HESSID and the SSID together to provide is a unique value that is advertised in Beacons and Probe Responses and by GAS Native Query/Response exchange so that the non-AP STA is aware of continued applicability of previously discovered interworking and advertising services when moving from one AP to another within the scope of the HESSID.

The Network Type field is defined in Figure u13a.  This field is used to advertise the type of network for every SSID included in the BSSID set.

|  | b0 | b2 | b3 | b4 | b5 | b6 | b7 |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Network Type Code |  | NSR | Intranet/ Internet | Advertisement Policy |  | Reserved |
| Bits: | 3 |  | 1 | 1 | 2 |  | 1 |

Figure u2—HESSID Network Type Field

Bits 0-2 are the Network Type Code bits.  These bits are set with the value from Table u1a.

Table u1—Network Type Code bit assignments

| Network Type Code Bits (b0-b1-b2) | Meaning | Description |
| --- | --- | --- |
| 000 | Private network | Network requires user accounts for access |
| 001 | Private network with guest access | Network requires user accounts for access, but guest accounts area available |
| 010 | Chargeable | Access to the network requires payment. Further information on types of charges may be available through other methods (802.21, UAM, etc). |
| 011 | Free | No charges apply for network use. Bits 4-6 are also used in this case. |
| 100 | Emergency Services Only (ESO) network | The network supports emergency services. Further information is available from a GAS native query (see 7.3.2.53) |
| 101-111 | Reserved | Not used. |

- The ESO (Emergency Services Only on this SSID) network type indicates higher layer emergency services are reachable via the default SSID.  This network type shall only be advertised by an AP if all the following conditions are true (cf. Annex P.1 for further information):

- The default SSID is configured for open authentication and used exclusively for providing emergency services

- dot11QosOptionImplemented is true

- Expedited Bandwidth Request Service capability bit is set to 1

- Location information capability is enabled on AP if AP is located in a regulatory domain that requires this.

- Network supports end-to-end QoS from STA to all APs in ESS to PSAP. It is outside the scope of Interworking Services how this information is known to the AP.

Bit 3 is the Next Step Required (NSR) bit.  This bit is set to one to indicate that the network requires a further step, as defined in 7.3.2.53.

Bit 4 is the Internet/Intranet bit.  This bit is only used when the Network Type Code in Table u3 is set to 011.  It is set to 1 to indicate that the network provides Internet access, and 0 to indicate the network provides access only to local networks.  If the Network Type Code is not 011, these bits is set to 0.

Bits 5 and 6 are the Advertisement Policy field, which has values set according to Table u3.  These bits are only used when the Network Type Code in Table u2 is set to 011  If the Network Type Code is not 011, these bits is set to 0.

Table u2—Advertisement Policy bit assignments

| Advertisement Policy bits (b5-b6) | Meaning | Description |
| --- | --- | --- |
| 00 | No Advertisements | The network does not require the end user to view commercial advertisements |
| 01 | Advertisements present | The network requires end users to view advertisements to support the service |
| 10 | Mixed advertisement policy | The network may require end users to view advertisements, but only for certain services. |
| 11 | Reserved | Not used. |

Bit 7 is reserved and is set to zero.

      - Emergency Services Public Credential Information element

The Emergency Service Public Credential information element defined in Figure u3 provides a STA with information which public identifier to use to gain network access for emergency services along with the specific authentication type supported for that public identifier.

|  | Element ID | Length | Emergency Authentication Type Control | Emergency Authentication EAP Type | Emergency Authentication Tunnelled Type (optional) | Emergency Public Identifier |
| --- | --- | --- | --- | --- | --- | --- |
| Octets: | 1 | 1 | 1 | 1 | 1 | variable |

Figure u3—Emergency Service Public Credential element description

The Element ID field is equal to the Emergency Services NAI value in Table 26.

The Length field shall be set to the length in octets of the Emergency Public Identifier field plus 3, if the Type control includes a tunnelled method or the length in octets of the Emergency Public Identifier field plus 2, if not.

Emergency Authentication EAP type is one of the EAP methods as specified in EAP IANA registry. If the Type control indicates the presence of a tunnelled type, then this field represents the outer EAP type.

The Emergency Authentication Type Control values are specified in Table u4.

Table u3—Emergency Authentication Tunnelled Control Field Values

| Name | Value |
| --- | --- |
| Tunnelled Method not used | 0 |
| EAP based tunneling method | 1 |
| PPP based tunneling method | 2 |
| Reserved | 3 – 255 |

If the Tunnelled Control field value is 0, the Emergency Authentication Tunnelled type field is not present. If the Tunnelled Control field value is 1, the tunnelled type value is set to one of EAP types and lastly, if the Tunnelled Control field value is 2, the tunnelled type value is set to one of PPP authentication types as pecified in PPP IANA registry along with relevant outer EAP type value (e.g. EAP-TTLS)

The Emergency Public Identifier field is a variable length field of Network Access Identifier (NAI) format as defined in RFC 4282.

      - Expedited Bandwidth Request element

The Expedited Bandwidth Request element is transmitted from a non-AP STA to an AP in an ADDTS action frame containing a TSPEC request and provides usage information on the bandwidth request.  The expedited bandwidth request element format is shown in Figure u4.

|  | Element ID | Length | Bandwidth Use |
| --- | --- | --- | --- |
| Octets: | 1 | 1 | 1 |

Figure u4—Expedited Bandwidth Request element

The Element ID field is equal to the Expedited bandwidth request value in Table 26.

The Length field shall be set to one

The Bandwidth Use field is provided in Figure u5.

| Bandwidth Use value | Description |
| --- | --- |
| 0 | Emergency call |
| 1 | First responder (public entity), Optional |
| 2 | First responder (private entity), Optional |
| 3 | MLPP Level A, Optional |
| 4 | MLPP Level B, Optional |
| 5 | MLPP Level 0, Optional |
| 6 | MLPP Level 1, Optional |
| 7 | MLPP Level 2, Optional |
| 8 | MLPP Level 3, Optional |
| 9 | MLPP Level 4, Optional |
| 10-255 | Reserved |

Figure u5—Bandwidth Use field description

The lower the value of bandwidth use, the higher the bandwidth usage priority.

      - QoS Map Set element

The QoS Map Set is transmitted from an AP to a non-AP STA and provides the mapping from DSCP contained in a packets IP header to the user priority which shall be used for transmission over the air.   The QoS Map Set element is shown in Figure u6.

| Order | Size (octets) | Description |
| --- | --- | --- |
| 1 | 1 | Element ID |
| 2 | 1 | Length |
| 3 | 2 | DSCP exception element #1 (optional) |
| 4 | 2 | DSCP exception element #2 (optional) |
| 4+n-2 | 2 | DSCP exception element #n (optional) |
| 5+n-2 | 2 | UP 0 DSCP Range element |
| 6+n-2 | 2 | UP 1 DSCP Range element |
| 7+n-2 | 2 | UP 2 DSCP Range element |
| 8+n-2 | 2 | UP 3 DSCP Range element |
| 9+n-2 | 2 | UP 4 DSCP Range element |
| 10+n-2 | 2 | UP 5 DSCP Range element |
| 11+n-2 | 2 | UP 6 DSCP Range element |
| 12+n-2 | 2 | UP 7 DSCP Range element |

Figure u6—QoS Map Set element description

The Element ID field is equal to the QoS Map set value in Table 26.

The Length field shall be set to 16 plus 2n, where n is the number of Exception elements in the QoS Map set.

DSCP exception elements are optionally included in the QoS Map Set.  If included, the QoS Map Set shall have a maximum of 8 exception elements.  The format of the exception element is shown in Figure u7.  The DSCP Value shall be between 0 and 63 inclusive; the User Priority value shall be between 0 and 7 inclusive.

- Any packet to be transmitted by a non-AP STA whose DSCP matches any exception value in the QoS Map Set shall use the corresponding UP for transmission; that is, it overrides the UP values provided in the UP n DSCP Range portion of the IE.

- Each Exception Element shall have a different DSCP value.

The QoS Map Set has a DSCP Range element corresponding to each of the 8 user priorities.  The format of the range element is shown in Figure u8.  All DSCP values shall be between 0 and 63 inclusive.  For any packet to be transmitted by a non-AP STA whose DSCP is contained within the range (inclusive of the high and low values) for a given UP, that the packet shall be transmitted with the corresponding UP.

- The DSCP range for each user priority shall be non-overlapping.

- A DSCP Range with low value equal to high value is permitted.

- If the DSCP Range high value and low value are both equal to 255, then the corresponding UP shall not be used.

For any packet to be transmitted by a non-AP STA whose DSCP does not match any of the DSCP Ranges or DSCP Exception elements, that packet shall be transmitted at UP=0.

|  | DSCP Value | User Priority |
| --- | --- | --- |
| Octets: | 1 | 1 |

Figure u7—DSCP Exception element description

|  | DSCP Low Value | DSCP High Value |
| --- | --- | --- |
| Octets: | 1 | 1 |

Figure u8—DSCP Range element description

The DSCP Low Value shall be between 0 and 63 inclusive; the DSCP High Value shall be between 0 and 63 inclusive and shall be equal to or greater than the DSCP Low Value.