| Received: TECHNOLOGY | INTERNATIONAL TELECOMMUNICATION UNION | *** DRAFT *** |
| --- | --- | --- |
|  | RADIOCOMMUNICATION STUDY GROUPS | Document 8F/IEEE-3-E |
|  |  | 15 March 2007 |
|  |  | English only |

Subject:		Question ITU-R 229-1/8

*** DRAFT ***
Institute of Electrical and Electronics Engineers (IEEE)

[Insert appropriate IEEE 802 intro/boiler plate]

TABLE OF CONTENTS

Page

1	Introduction

2	Scope and Purpose

3	Related Documents

4	General Requirements

5	Technical Requirements

5.1	Technological items required to describe candidate air interface

5.1.1	Radio transmission technologies functional blocks

5.1.2	Other functional blocks

5.2	Required technology items for evaluation

5.2.1	Spectrum efficiency/ Coverage efficiency

5.2.2	Technology complexity

5.2.3	Quality

5.2.4	Flexibility of radio interface

5.2.5	Implication on network interface

5.2.6	Cell Coverage

5.2.7	Power efficiency

5.2.8	Spectrum compatibility

5.2.x	xxxxxxxxxxxxxxxxxxxxxx

6	Conclusions

7	Terminology, abbreviations

Appendices

1	Spectrum and deployment

2	Radio Access Interface and Network

2.1	Network topology

2.2	Duplexing

2.3	Multiple-Access technologies

2.4	Multiple-Antenna technologies

2.5	Channel Coding

2.6	Mobility management and RRM

3	Mobile user interface

3.1	Mobile user terminal design

3.2	New innovative network to humane interfaces

3.3	Human-free interface

3.4	RF micro-electro-mechanical systems (MEMS)

1	The multi-antenna system application scenario

2	MIMO’s impact on mobility

# 1	Introduction

[Editor’s note:

Text will be imported from the common text which is discussed in WG-SERV.]

# 2	Scope and Purpose

IMT.TECH describes requirements related to technical system performance for IMT-Advanced candidate radio interfaces.  These requirements are used in the development IMT.EVAL, and will be attached as Annex 4 to the Circular Letter to be sent announcing the process for IMT-Advanced candidacy.

IMT.TECH also provides the necessary background information about the individual requirements (technology enablers) and the justification for the items and values chosen. Provision of such background information is needed for wider reference and understanding.

IMT.TECH is based on the ongoing development activities from external research and technology organizations.  The information in IMT.TECH will also feed in to the IMT.SERV document.  IMT.TECH provides the radio interface requirements which will be used in the development of IMT.RADIO

# 3	Related Documents

Recommendation ITU-R M.[IMT.SERV]

Recommendation ITU-R M.1645

Recommendation ITU-R M.1768

Report ITU-R M.2038

Report ITU-R M.2072

Report ITU-R M.2074

Report ITU-R M.2078

Report ITU-R M.2079

Recommendation ITU-R M.1224

Recommendation ITU-R M.1225

[Recommendation ITU-T Q.1751

Recommendation ITU-T Q.1761

Recommendation ITU-T Q.1711

Recommendation ITU-T Q.1721

Recommendation ITU-T Q.1731

Recommendation ITU-T Q.1703]

[Editor’s note: Document to be added]

# 4	General Requirements

[Editor’s note: This section is for describing general requirements for cellular systems including IMT which are requested by market not only developed but also developing countries]

The following are the general system requirements and features that IMT-Advanced system shall support:

- Higher spectral efficiencies and peak data rates.

- Lower latencies (air-link access latency, [Inter-FA HO, Intra-FA HO, inter-RAN HO] latencies) to enable new delay-sensitive applications.

- Mobility Support: Cellular systems including IMT-Advanced are required to support the environments described in following:

  - Stationary (Fixed applications) (i.e. can be used as a FWA systems)

  - Pedestrian (Pedestrian speeds up to 10 km/h)

  - Typical Vehicular (Vehicular speeds up to 120  km/h)

  - High Speed Vehicular	(Vehicular speeds up to 500 km/h)

  - Optimized system performance for low mobility environments

  - Seamless application connectivity to other mobile networks and other IP networks (global roaming capabilities).

- Support for larger cell sizes and improved cell-edge performance.

- Low-cost and low-complexity terminals for worldwide use.

- Mobile user interface.

- Ubiquitous Access.

- Improved unicast and multicast broadcast services.

- Provision for PAN/LAN/WAN Co-location / Coexistence.

The IMT Advanced systems shall be designed to provide best-in-class performance attributes such as peak and sustained data rates and corresponding spectral efficiencies, capacity, latency, overall network complexity and quality-of-service management.

The IMT Advanced system shall support applications that conform to open standards and protocols. This allows applications including, but not limited to, video, full graphical web browsing, e-mail, file uploading and downloading without size limitations (e.g., FTP), streaming video and streaming audio, IP Multicast, Location based services, VPN connections, VoIP, instant messaging and on- line multiplayer gaming.

The IMT Advanced systems shall provide the mobile user with an "always-on" experience while also taking into account and providing features needed to preserve battery life. The connectivity from the mobile terminal to the base station (BS) shall be automatic and transparent to the user as it moves between mobile networks.

The IMT-Advanced systems shall work in dense urban, urban, suburban, rural, outdoor-indoor, pedestrian, and vehicular environments and the relevant channel models shall be applicable. Systems are intended to provide ubiquitous mobile broadband wireless access in a cellular architecture (e.g. macro/micro/pico cells). The system shall support non-line of sight outdoor to indoor scenarios and indoor coverage.

# 5	Technical Requirements

[Editor note: This chapter specifies the technical independent requirements that determine the performance of the IMT-Advanced systems.]

## 5.1	Technological items required to describe candidate air interface

[Editor’s note: This section is for listing up technology enablers which need to be described in the candidate air interface proposal for IMT-Advanced and also the general explanation why those each technology enablers are important to be described.]

### 5.1.1	Radio transmission technologies functional blocks

#### 5.1.1.1	Multiple access methods

[The choice of the multiple access technology has major impact on the design of the radio interface. For instance, OFDMA, CDMA and also Single-carrier/Multi-carrier operation]

#### 5.1.1.2	Modulation scheme

[The choice of the modulation technology depends mainly on radio environment and the spectrum efficiency requirements.]

#### 5.1.1.3	Duplex methods

[The choice of the duplexing technology mainly affects the choices of the RF-channel bandwidth and the frame length. Duplexing technology may be independent of the access technology since for example either frequency division duplex (FDD) , time division duplex (TDD) or half-duplex FDD may be used. It also affects band allocations, sharing studies, and cell size.]

IMT-Advanced systems shall support both TDD and FDD operational modes.  The FDD mode shall support both full duplex and half duplex mobile station operation.  Specifically, a half-duplex FDD mobile station is defined as a mobile station that is not required to transmit and receive simultaneously.

IMT-Advanced systems shall support both unpaired and paired frequency allocations, with fixed duplexing frequency separations when operating in full duplex FDD mode.

System performance in the desired bandwidths specified in Section 5.1.1.3 should be optimized for both TDD and FDD independently while retaining as much commonality as possible.

The UL/DL ratio should be configurable. In TDD mode, the DL/UL ratio should be adjustable. In FDD mode, the UL and DL channel bandwidths may be different and should be configurable (e.g. 10MHz downlink, 5MHz uplink).  In the extreme, the IMT-Advanced system should be capable of supporting downlink-only configurations on a given carrier.

Asymmetrical operation should be supported in addition to symmetrical operation.

5.1.1.3.1	System Bandwidth

The IMT Advanced systems shall support required bandwidths from 5 to 20 MHz, and optional bandwidths up to 40 MHz, in both cases supporting flexible and scalable AI parameters.

The IMT-Advanced systems air interface shall support system implementation in TDD or FDD licensed spectrum below [TBD] GHz and allocated to the mobile service. The system’s frequency plan shall include both paired and unpaired channel plans with multiple bandwidths to allow co-deployment with existing cellular systems. It is desirable that channel bandwidths are consistent with frequency plans and frequency allocations for other wide-area systems. The IMT-Advanced air interface should be readily extensible to wider channels as they become available in the future.

#### 5.1.1.4	Error control coding scheme

[The choice of the error control coding affects qualities of air link, throughput, terminal complexity and also delay performance of communications.]

#### 5.1.1.5	Physical channel structure and multiplexing

[The physical channel is a specified portion of one or more radio frequency channels as defined in frequency, time spatial and code domain.]

#### 5.1.1.6	Frame Structure

[The frame structure depends mainly on the multiple access technology (e.g. OFDMA, TDMA, CDMA) and the duplexing technology (e.g. FDD, TDD). Commonality should be maximised by maintaining the same frame structure whenever possible. That is, data fields identifying physical and logical channels, as well as the frame length should be maintained when possible.]

#### 5.1.1.7	[FFT size, Chip rate etc.]

#### 5.1.1.8      Support of Advanced Antenna Techniques

IMT-Advanced systems shall support MIMO and beamforming including features to support multi-antenna capabilities at both the base station and at the mobile terminal, including MIMO operation for both UL and DL, both UL and DL beamforming, SDMA, and precoding. .

Minimum antenna configuration requirements shall be:

- For the base station, a minimum of two transmit and two receive antennas shall be supported.

- For the MS, a minimum of one transmit and two received antennas shall be supported.  This minimum is consistent with a 2x2 downlink configuration and a 1x2 uplink configuration.

#### 5.1.1.9	Use of Coverage Enhancing Technologies

The system shall support the use of coverage enhancing technologies.

#### 5.1.1.10	Link Adaptation and Power Control

IMT-Advanced systems shall support automatic selection of optimized user data rates that are consistent with the RF environment constraints and application requirements. The IMT-Advanced shall provide for graceful reduction or increase of user data rates, on the downlink and uplink, as a mechanism to maintain an appropriate frame error rate performance.

Link adaptation (e.g, adaptive modulation and coding) shall be used by the IMT-Advanced systems for increasing spectral efficiency, data rate, and cell coverage reliability.

Both base station and mobile terminal should employ transmit power control mechanisms and exchange control and monitoring information required to achieve optimal performance while keeping the environmental noise floor as low as possible and helping the MS preserve its battery power. The number of transmit Power levels as well as the associated control messaging should be optimized for cost effectiveness and performance.

#### 5.1.1.x	xxxxxxxxxxxxxxxxxxxxxx

### 5.1.2	Other functional blocks

#### 5.1.2.1	Source coder

[The choice of the source coder may generally be made independently of the access method.]

#### 5.1.2.2	Interworking

[The interworking function (IWF) converts standard data services to the rates used internally by the radio transmission subsystem. The IWF feeds into the channel coder on the transmit side and is fed from the channel decoder on the receiver side. It also take some functionalities to deal with the applications such as voice, images, etc.]

#### 5.1.2.3	Latency

[The latency is important factor especially if delay sensitive communication required.]

Latency should be further reduced as compared to IMT-2000 systems for all aspects of the system including the air link, state transition delay, access delay, and handover.

The following latency requirements shall be met by the system, under unloaded conditions.

5.1.2.3.1   Data Latency

Requirements for air link data latency are specified in terms of the time for delivery of a MAC PDU, transmissible as a Layer 1 codeword (i.e. without fragmentation), from the MAC interface of a base station or mobile station entity to the MAC interface of the corresponding mobile station or base station entity, excluding any scheduling delay at the base station. A single Layer 1 re-transmission of the codeword is included in the definition. The latency does not include bandwidth requests. The corresponding maximum latency for delivery of the MAC PDU appears in Table 1.

Table 1. Maximum Data Latency

| Link Direction | Max. Latency (ms) |
| --- | --- |
| Downlink (BS->MS) | 10 |
| Uplink (MS->BS) | 10 |

5.1.2.3.2   State Transition Latency

Performance requirements for state transition delay define the transition from IDLE mode to ACTIVE mode.

IDLE to ACTIVE_STATE is defined as the time it takes for a device to go from an idle state (fully authenticated/registered and monitoring the control channel) to when it begins exchanging data with the network on a traffic channel or timeslot measured from the paging indication (i.e. not including the paging period).

Table 2. State Transition Latency

| Metric | Max. Latency (ms) |
| --- | --- |
| IDLE_STATE to ACTIVE_STATE | 100 ms |

5.1.2.3.3   Handover Interruption Time

Handover performance requirements, and specifically the interruption times applicable to handovers for compatible IMT-2000 and IMT-Advanced systems, and intra- and inter-frequency handover should be defined.

The maximum MAC-service interruption times during handover are specified in Table 3.

Table 3. Maximum Handover Interruption.

| Handover Type | Max. Interruption Time (ms) |
| --- | --- |
| Intra-Frequency | 50 |
| Inter-Frequency | 150 |

#### 5.1.2.3.4	Latency and Packet Error Rates

IMT-Advanced systems shall support the configuration (e.g., by the system operator) of a flexible set variety of traffic classes with different latency and packet error rates performance, in order to meet the end-user QoS requirements for the various applications,

Specifically, it is important for IMT-Advanced systems to

- Have the ability to negotiate the traffic class associated with each packet flow.

- Permit the set of traffic classes to be defined by the system operator in terms of QoS attributes (along with the range of allowed values) that include the following:

- Data rate (ranging from the lowest supported data rate to maximum data rate supported by the MAC/PHY),

- Latency (delivery delay) (ranging from 10 ms to 10 seconds),

- Packet error rate (after all corrections provided by the MAC/PHY layers) (ranging from 10E-8 to 10E-1), and

- Delay variation (jitter) (ranging from 0 to 10 seconds).

- Support (but not require) PHY/MAC implementations that satisfy the QoS characteristics that are specified by the following traffic classes:

[ADD TRAFFIC LIST HERE]

As is the case for all wireless networks, the specified QoS characteristics for certain traffic classes or services need only be satisfied in deployments and RF link conditions that are appropriate to permit the desired characteristics to be feasible.  However, the MAC/PHY structure IMT-Advanced systems should support the capabilities to negotiate and deliver all of the QoS characteristics specified for the indicated traffic classes.

#### 5.1.2.4	QoS Management scheme

[The QoS is important factor especially the applications which are originally supported by circuit switched network in delay/jitter.]

IMT-Advanced systems shall support QoS classes, enabling an optimal matching of service, application and protocol requirements (including higher layer signaling) to RAN resources and radio characteristics. This includes enabling applications such as interactive gaming.

When feasible, support shall be provided for preserving QoS when switching between networks associated with other radio access technologies (RAT’s).

QoS implementation should include link level support between base stations and mobile terminals with a structure to provide sufficient capabilities to conform to an end-to-end QoS architecture e.g., as negotiated by upper layer protocols such as RSVP.

It is important that IMT Advanced systems be able to support the ability to enforce QoS authorizations for each user in addition to supporting various policies determined by the system operator to resolve air interface contention issues between users based on the individual users’ QoS authorization and QoS requests.

The IMT Advanced systems should define a common set of parameters to address all classes of service and QoS parameters for all services. A QoS based IP network may employ the Resource Reservation Protocol (RSVP) to signal the allocation of resources along a routed IP path.

Other QoS factors include:

- Supporting IPv4 and IPv6 enabled QoS resolutions. with efficient radio resource management (allocation, maintenance, and release) to satisfy user QoS and policy requirements.

- Providing the MAC and PHY layer capabilities to satisfy link-level QoS requirements by resolving system resource demand conflicts between all mobile terminals while still satisfying the negotiated QoS commitments for each individual terminal. A given user may be using several applications with differing QoS requirements at the same time (e.g., web browsing while also participating in a video conferencing activity with separate audio and video streams of information).

- Providing MAC and PHY layer capabilities to distinguish between various packet flows from the same mobile terminal or user and provide differentiated QoS delivery to satisfy the QoS requirement for each packet flow.

- Providing the ability to negotiate the traffic flow templates that define the various packet flows within a user's IP traffic and to associate those packet flows with the QoS requirements for each flow (i.e., QoS parameters such as delay, bit rate, error rate, and jitter).

#### 5.1.2.5	Security Aspects

[The secure communication should be achieved at least the same level as the IMT-2000.]

Network security in IMT Advanced systems are needed to protect the service provider from theft of service, the user’s privacy and mitigate against denial of service attacks. IMT Advanced systems will need provisions for authentication of both base station and mobile terminal, for privacy, and for data integrity. The IMT Advanced link layer security shall be part of an end-to-end security mechanism that includes higher layers such as TLS, SSL, IPSec, etc. Encryption across the air interface to protect user data traffic and signaling messages, from unauthorized disclosure shall be supported. The IMT Advanced systems shall provide protection from unauthorized disclosure of the device permanent identity to passive attackers.

Security aspects include:

- Both the network and mobile terminal having to perform mutual entity authentication and session key agreement protocol. After authentication of the mobile terminal the network may perform authorization before providing service.

- Providing a method that will enable message integrity across the air interface to protect user data traffic and signaling messages from unauthorized modification.

- Making it possible to operate the MAC and PHY with any of the following combinations of privacy and integrity:

  - Encryption and message integrity.

  - Encryption and no message integrity.

  - Message integrity and no encryption.

  - No message integrity and no encryption.

#### 5.1.2.5.1	Privacy and Authentication Aspects

IMT-Advanced systems shall include a privacy and authentication functions which provides the necessary means to achieve:

- Protection of the integrity of the system (e.g. system access, stability and availability).

- Protection and confidentiality of user-generated traffic and user-related data (e.g. location privacy, user identity).

- Secure access to, secure provisioning and availability of services provided by the system.

Example procedures that can be used to achieve the above-stated goals include user/device authentication, integrity protection of control and management messages, enhanced key management, and encryption of user generated and user-related data. The impact of these procedures on the performance of other system procedures, such as handover procedures, shall be minimized.

#### 5.1.2.6	Capacity considerations/ Supported user density

[Requirements that specify how many users could be supported in different scenarios, e.g rural, urban and hotspot.]

#### 5.1.2.7	Network Topology

[Proposed radio interface technology need to be considered for applying to Single-hop mode, Multi-hop mode, Mesh mode and Peer to peer mode.]

5.1.2.8	    Mobility management and RRM

[Centrarized/Distributed RRM, Inter-RAT spectrum sharing/mobility management need to be considered.]

5.1.2.8.1   Reporting

IMT-Advanced systems shall enable advanced radio resource management by enabling the collection of reliable statistics over different timescales, including system (e.g. dropped call statistics), user (e.g. terminal capabilities, mobility statistics, battery life), flow, packet, etc.

5.1.2.8.2   Interference Management

IMT-Advanced systems shall support advanced interference mitigation schemes and enhanced flexible frequency re-use schemes.

5.1.2.8.3   Inter-RAT Mobility

IMT-Advanced systems shall support inter-RAT operations.5.1.2.x.

#### Reporting , Measurements, and Provisioning  Support

The IMT-Advanced systems shall provide a mechanism to enable the provisioning and collection of metrics, so that the network operator can effectively control, monitor, and tune the performance of the air-interface.

For example, the air interface shall support measurements in the physical layer of both the base station and the mobile terminal. These physical layer measurements should include: signal strength, signal quality (C/I), error rates, access delays, session interruption, effective throughput, neighboring cells’ signals and provide any other measurement needed for handoff support, maintenance and quality of service monitoring. Some of these measurements should be reported to the opposite side of the air link on a periodic basis, and/or upon request.

#### 5.1.2.8.5	Handoff Support

IMT-Advanced systems shall provide handoff methods to facilitate providing continuous service for a population of moving mobile terminals. The handoff methods shall enable mobile terminals to maintain connectivity when moving between cells, between systems, between frequencies, and at the higher layer between IP Subnets.

#### 5.1.2.8.6   IP-Level Handoff

In supporting high speed mobility in an all IP network, the IMT-Advanced air interface standard shall allow the use of MobileIPv4, MobileIPv6 or of SimpleIP.

#### 5.1.2.9	User State Transitions

The IMT-Advanced systems’ air interface shall support multiple protocol states with fast and dynamic transitions among them. It will provide efficient signaling schemes for allocating and de-allocating resources, which may include logical in-band and/or out-of-band signaling, with respect to resources allocated for end-user data. The air interface shall provide power conservation features to improve battery life for idle mobile terminals.

#### 5.1.2.x	xxxxxxxxxxxxxxxxxxxxxx

## 5.2	Required technology items for evaluation

### 5.2.1	Spectrum efficiency/ Coverage efficiency

[The supported information transmission rate under some constrains, e.g, bandwidth, area, time and system load.]

### 5.2.1.1  Spectral Efficiency

“System Spectral Efficiency” is defined in the context of a full block assignment deployment and is calculated as the average aggregate throughput per sector (in bps/Hz/sector), divided by the spectrum block assignment size (in Hz) (excluding all PHY/MAC layer overhead).

For proposal evaluation purposes, the Spectral Efficiency of the IMT-Advanced systems’ air interface shall be quoted for the case of a three sector baseline configuration for a given block assignment size. It shall be computed in a loaded multi-cellular network setting [NEED REFERENCE TO EVAL CRITERIA]. It shall consider, among other factors, a minimum expected data rate/user and/or other fairness criteria, QoS, and percentage of throughput due to duplicated information flow.

The system spectral efficiency of the IMT-Advanced systems’ air interface shall be greater than the values indicated in Table 4. The spectral efficiency at higher speeds than those shown will degrade gracefully.

Table 4 Spectral Efficiency Targets

| Parameter | Spectral Efficiency Requirements |  |  |  |
| --- | --- | --- | --- | --- |
|  | Downlink |  | Uplink |  |
|  | Pedestrain 3 km/hr | Vehicular 10-120 km/hr | Pedestrain 3 km/hr | Vehicular 10-120 km/hr |
| Spectral Efficiency (b/s/Hz/sector) | TBD | TBD | TBD | TBD |

### 5.2.1.2     Peak Data Rates per User

Peak data rates per user (or peak user data rate) – The peak data rate per user is the highest theoretical data rate available to applications running over an IMT-Advanced air interface and assignable to a single mobile terminal.  The peak data rate per user can be determined from the combination of modulation constellation, coding rate and symbol rate that yields the maximum data rate.

The IMT-Advanced systems’ air interface shall support peak data rate/user/MHz in excess of the values shown in Table . These peak data rate targets are independent of channel conditions, traffic loading, and system architecture.

Table 5 Peak Data Rate

|  | Downlink | Uplink |
| --- | --- | --- |
| Peak Date Rate bps/MHz/user | TBD | TBD |

### 5.2.1.3    Aggregate Data Rates

Aggregate throughput is defined as the total throughput to all users in the system (user payload only).  The IMT-Advance systems’ air interface shall exceed the values shown in Table . Note that these aggregate data rate values for downlink and uplink shall be consistent with the spectral efficiency values in 5.2.1.1 above.

Table 6 Aggregate Data Rate

|  | Downlink | Uplink |
| --- | --- | --- |
| Aggregate Data bps | TBD | TBD |

### 5.2.1.4	Throughput and Capacity

[The supported information transmission rate under some constrains, e.g, bandwidth, area, time and system load.]

## 5.2.1.4.1 User throughput

The targets for average user-throughput and cell-edge user throughput of downlink/uplink for data only system for minimum antenna configuration are shown in Table .  Both targets should be achieved as per minimum antenna configuration defined in section 5.1.1.8.

Table 7. Data only system

| Metric | Throughput |  |
| --- | --- | --- |
|  | DL Data | UL Data |
| Average User Throughput | TBD | TBD |
| Cell Edge User Throughput | TBD | TBD |

## 5.2.1.5 Sector Capacity

Sector Throughput is defined as the total unidirectional sustained throughput (downlink/uplink), excluding MAC & PHY layer overheads, across all users scheduled on the same RF channel. Sector throughput requirements must be supported for realistic distributions of users of a fully loaded cell surrounded by other fully loaded cells using the same RF channel (i.e. an interference limited environment with full frequency reuse).

Table 8. Sector Throughput (bps/Hz/sector)

| Speed (km/h) | DL | UL |
| --- | --- | --- |
| TBD | TBD | TBD |

Table 9. Voice-over-IP Capacity

Capacity (Active Users/MHz/sector)

> 60 (FDD)

## 5.2.1.6 Mobility

IMT-Advanced shall be optimized for low speeds such as mobility classes from stationary to pedestrian and provide high performance for higher mobility classes. The performance shall be degraded gracefully at the highest mobility. In addition, IMT-Advanced shall be able to maintain the connection up to highest supported speed and to support the required spectral efficiency.

Table  summarizes the mobility performance.

Table 10. IMT-Advanced mobility support

| Mobility | Performance |
| --- | --- |
| Low (0 –15 km/h) | Optimized |
| High (15– 120 km/h) | Marginal degradation |
| Highest (120 km/h to 350 km/h) | System should be able to maintain connection |

### 5.2.1.7	   Number of Simultaneous Active Users

The number of active users controlled by the MAC layer of an IMT-Advanced systems should be greater than [TBD] simultaneous active sessions per sector for a given bandwidth assignment of 2 x X MHz in FDD and 2X MHz in TDD. An active session is a time duration during which a user can receive and/or transmit data with potentially a short delay (i.e. in the absence of service level constraints such as delays caused by the needs to satisfy QoS commitments to other users). In this state the user should have a radio bearer channel available with a delay of less than [TBD] ms with probability of at least [TBD]. This requirement shall be met regardless of whether the sessions are all on one or multiple terminals.

Note that certain applications will have to be given preferential treatment with respect to delay in order to satisfy QoS requirements, e.g. VoIP. This parameter should scale linearly with system bandwidth if the same application mixes are assumed.

### 5.2.2   Technology complexity

The IMT-Advanced systems PHY/MAC should enable a variety of hardware platforms with differing performance and complexity requirements.

IMT-Advanced shall minimize complexity of the architecture and protocols and avoid excessive system complexity.

### 5.2.3	Quality

### 5.2.4	Flexibility of radio interface

### 5.2.5	Implication on network interface

### 5.2.6	Cell Coverage

[Requirements that specify the area could be covered by a cell of the IMT-Advanced system.]

A cell radius over 50km should be supported by proper configuration of the system parameters

Support for larger cell sizes should not compromise the performance of smaller cells. Specifically, IMT-Advanced systems shall support the following deployment scenarios in terms of maximum cell range:

Table 11. IMT-Advanced Deployment Scenarios

| Cell Range | Performance target |
| --- | --- |
| Up to 5 km | Performance targets defined in section 5.2.1 should be met |
| 5-30 km | Graceful degradation in system/edge spectral efficiency |
| 30-100 km | System should be functional (thermal noise limited scenario) |

### 5.2.7	Power efficiency

[The maximum transmission power allowed for achieving the performance requirements]

### 5.2.8	Spectrum compatibility

[Requirements that specify how the technology utilize spectrum and minimize interference to the adjacent spectrum. MiMo or Beam-Forming is a candidate technology for this requirement.]

#### 5.2.x	xxxxxxxxxxxxxxxxxxxxxx

5.2.9 Voice-over-IP

Table 12. VoIP Capacity

Capacity (Active Users/MHz/sector)

> 60 (FDD)

VoIP capacity assumes a 12.2 kbps codec with a 40% activity factor such that the percentage of users in outage is less than 3% where outage is defined such 97% of the VoIP packets are delivered successfully to the users within the delay bound of 80 msec.

5.2.10 Enhanced Location Based Services (LBS)

IMT-Advanced systems shall provide support for high resolution location determination.

5.2.11 Enhanced Multicast Broadcast Service (E-MBS)

IMT-Advanced systems shall provide support for an Enhanced Multicast Broadcast Service (E-MBS), providing enhanced multicast and broadcast spectral efficiency (Section 5.2.11.2). E-MBS delivery shall be supported via a dedicated carrier.

IMT-Advanced systems shall support optimized switching between broadcast and unicast services, including the case when broadcast and unicast services are deployed on different frequencies.

### 5.2.11.1 MBS Channel Reselection Delay and Interruption Times

E-MBS functionality defined as part of IMT-Advanced systems shall support the following requirements for maximum MBS channel change interruption times when applied to broadcast streaming media.

Table 4. MBS channel reselection maximum interruption times.

| MBS Channel Reselection Mode | Max. Interruption Time (s) |
| --- | --- |
| Intra-frequency | 1.0 |
| Inter-frequency | 1.5 |

Note that requirements of Table 4 apply to the interruption time between terminating delivery of MAC PDU’s from a first MBS service to the MAC layer of the mobile station, and the time of commencement of delivery of MAC PDU’s from a second MBS service to the mobile station MAC layer.

5.2.11.2 Minimum performance requirements for E-MBS

Minimum performance requirements for E-MBS, expressed in terms of spectral efficiency over the coverage area of the service, appear in Table 5.

Table 5. MBS minimum spectral efficiency vs. inter-site distance

| Inter-Site Distance (km) | Min. Spectral Efficiency (bps/Hz) |
| --- | --- |
| 0.5 | 4 |
| 1.5 | 2 |

The following notes apply to Table 5:

- The performance requirements apply to a wide-area multi-cell multicast broadcast single frequency network (MBSFN).

- The specified spectral efficiencies neglect overhead due to ancillary functions (such as synchronization and common control channel) and apply to both mixed unicast-broadcast and dedicated MBS carriers, where the performance is scalable with carrier frequency bandwidth.

# 6	Conclusions

This Report provides useful information on technology issue which is required for evaluate the air interface(s) for IMT-Advanced.

# 7	Terminology, abbreviations

- Active users - An active user is a terminal that is registered with a cell and is using or seeking to use air link resources to receive and/or transmit data within a short time interval (e.g., within 100 ms).

- Aggregate Throughput - Aggregate throughput is defined as the total throughput to all users in the system (user payload only).

- Airlink MAC Frame RTT - The round-trip time (RTT) over the airlink for a MAC data frame is defined here to be the duration from when a data frame is received by the physical layer of the transmitter to the time when an acknowledgment for that frame is received by the transmitting station.

- Air Interface (“AI”) –

  - The air interface is the radio-frequency portion of the transmission path between the wireless terminal (usually portable or mobile) and the active base station or access point.

  - The air interface is the shared boundary between a wireless terminal and the base station or access point.

- [Bandwidth or] Channel bandwidth - the spectrum required by one channel and contains the occupied bandwidth plus buffer spectrum [which may be] necessary to meet the radio performance specifications in same-technology, adjacent channels deployment. The concept is depicted in the following figure.

Note: In this document, the extra buffer spectrum included in a radio channel bandwidth is referred to as “in-channel guard-bands”.

- Block Assignment – A block assignment which may consist of paired or unpaired spectrum is the block of licensed spectrum assigned to an individual operator. It is assumed here that the spectrum adjacent to the block assignment is assigned to a different network operator. At the edges of the block assignment the applicable of band emission limits shall apply (for example, the limits defined in 47 CFR 24.238 for PCS). A block is typically occupied by one or more channels.

- Broadcast Service - the ability to transmit a packet of information (e.g., an IP broadcast datagram) to all mobile terminals within a geographical area.  Note that a particular mobile terminal may choose to receive or ignore individual information packets that are delivered via the broadcast service.

Note: This term should not be confused with term “broadcasting service” as defined in the ITU Radio Regulations.

- Cell - The term “cell” refers to one single-sector base station or to one sector of a base station deployed with multiple sectors.

- Cell sizes – The maximum distance from the base station to the mobile terminal over which an acceptable communication can maintained or before which a handoff would be triggered determines the size of a cell.

- Coverage Enhancing Technologies - In the context of wireless communications - technologies that augment the radio signal, in areas within the boundary of a cell, where the BS/MS transmit signal is obstructed and significantly attenuated by terrain or man-made structures.

- Frequency Arrangements – The frequency arrangement of the spectrum refers to its allocation for paired or unpaired spectrum bands to provide for the use of Frequency-Division Duplexing (FDD) or Time-Division Duplexing (TDD), respectively.  The PAR states that the 802.20 standard should support both these frequency arrangements.

- Frequency reuse - (N) is defined as the total number of sectors in a given configuration divided by the number of times that the same frequency is reused.

- Handoff - The act of switching the communications of a mobile terminal from one cell (or sector) to another cell (or sector), or between radio channels in the same cell (or sector).

- Interoperable – Systems that conform to the 802.20 specifications should interoperate with each other, e.g., regardless of manufacturer. (Note that this statement is limited to systems that operate in accordance with the same frequency plan. It does not suggest that an 802.20 TDD system would be interoperable with an 802.20 FDD system.)

- Intra-Technology Handoff -A handoff between two cells employing the same air interface technology.

- Inter-Technology Handoff - A handoff between two cells employing different air interface technologies (e.g. between 802.11 and 802.20 cells).

- Licensed bands below 3.5 GHz – This refers to bands that are allocated to the mobile service and licensed for use by mobile cellular wireless systems operating below 3.5 GHz.

- MAN – Metropolitan Area Network.

- Mobile Broadband Wireless Access systems – This may be abbreviated as MBWA and is used specifically to mean “802.20 systems” or systems compliant with an 802.20 standard.

- Multicast Service - the ability to transmit a packet of information (e.g., an IP multicast datagram) to a subset of all mobile terminals within a geographical area.  The multicast target for a multicast information packet is identified by a multicast address.  Each mobile terminal can choose to receive multicast information packets based on the desired multicast address(es).

- Network Wide Bandwidth - The network wide bandwidth is the total spectrum in use by the unique carriers deployed in the network.

- Occupied bandwidth - The width of a frequency band such that, below the lower and above the upper frequency limits, the mean powers emitted are each equal to a specified percentage B /2 of the total mean power of a given emission. Unless otherwise specified in an ITU-R Recommendation for the appropriate class of emission, the value of B /2 should be taken as 0.5%.

Note 1: The percentage of the total power outside the occupied bandwidth is represented by B.

Note 2: In some cases, e.g., multichannel frequency-division multiplexing systems, use of the 0.5% limits may lead to certain difficulties in the practical application of the definition of occupied and necessary bandwidth; in such cases, a different percentage may prove useful.”

- Necessary bandwidth - For a given class of emission, the width of the frequency band which is just sufficient to ensure the transmission of information at the rate and with the quality required under specified conditions.

- Optimized for IP Data Transport – Such an air interface is designed specifically for carrying Internet Protocol (IP) data traffic efficiently. This optimization could involve (but is not limited to) increasing the throughput, reducing the system resources needed, decreasing the transmission latencies, etc.

- Peak aggregate data rate per cell – The peak aggregate data rate per cell is the total data rate transmitted from (in the case of DL) or received by (in the case of UL) a base station in a cell (or in a sector, in the case of a sectorized configuration), summed over all mobile terminals that are simultaneously communicating with that base station.

- Peak data rates per user (or peak user data rate) – The peak data rate per user is the highest theoretical data rate available to applications running over an 802.20 air interface and assignable to a single mobile terminal.  The peak data rate per user can be determined from the combination of modulation constellation, coding rate and symbol rate that yields the maximum data rate.

- Roaming - The use of a communications device outside a specified administrative domain (home domain) defined by the service provider. A home domain may be defined as a geographic area (home area) or a network (home network). Agreements between service providers and/or third parties are required to support roaming.

- SimpleIP - A service in which the mobile terminal is assigned a dynamic IP address from the local IP sub-network and is provided IP routing service by a service provider network. The mobile terminal retains its IP address as long as it is served by a radio network which has connectivity to the address assigning IP sub-network.

- Sustained user data rates – Sustained user data rates refer to the typical data rates that could be maintained by a user, over a period of time in a loaded system.  The evaluation of the sustained user data rate is generally a complicated calculation to be determined that will involve consideration of typical channel models, environmental and geographic scenarios, data traffic models and user distributions.

- System gain - is defined as the difference, in dB, between transmitter power output at the base station and the receiver threshold (sensitivity) at the mobile terminal.

- System spectral efficiency – System spectral efficiency is defined as the ratio of the aggregate throughput (bits/sec) to all users in the system divided by the network wide bandwidth (Hz) and divided by the number of sectors in the system.

- Targets for 1.25 MHz channel bandwidth – This is a reference bandwidth of 2 x 1.25 MHz for paired channels for FDD systems or a single 2.5 MHz channel for TDD systems. This is established to provide a common basis for measuring the bandwidth-dependent characteristics. The targets in the table indicated by the asterisk (*) are those dependent on the channel bandwidth. Note that for larger bandwidths the targets may scale proportionally with the bandwidth.

- Various vehicular mobility classes – Recommendation ITU-R M.1034-1 establishes the following mobility classes or broad categories for the relative speed between a mobile and base station:

  - Stationary (0 km/h),

  - Pedestrian (up to 10 km/h)

  - Typical vehicular (up to 100 km/h)

  - High speed vehicular (up to 500 km /h)

  - Aeronautical (up to 1 500 km/h)

  - Satellite (up to 27 000 km/h).}*

# Appendices

The following 2 appendices illustrate technology enablers which can be used for IMT-Advanced Radio Interface(s)

Appendix 1 

Overview of major new technologies

## 1	Spectrum and deployment

[Editor note: Technologies that can improving spectrum efficiency, flexibility and sharing possibility could be included in this section.]

## 2	Radio Access Interface and Network

[Editor note: New radio access technologies, such as soft-defined radio, short range radio and new multiple access method etc, could be include in this section. The innovations of network deployment, e.g. wireless relay enhanced cellular, can also be included in this section]

### 2.1	Network topology

#### 2.1.1	Single-hop mode

The information is transmitted between radio access point (e.g. base-station) and mobile stations (e.g. user terminals) directly in a single hop. An example of network topology in this case is shown in Figure 2.1.1.1).

Figure 2.1.1.1

Working mode of radio access network – Single Hop Mode

#### 2.1.2	Multi-hop mode

The direct communications between BSs and the data transportation through multihop across BSs should be considered.

The information is transmitted between radio access point to mobile stations in more than one hop. The intermediate points between access point and destination are relay nodes that regenerate and re-transmit radio signals. The topology of multi-hop mode is shown in Figure 2.1.2.1.

Figure 2.1.2.1

Working mode of radio access network – Multi Hop Mode

#### 2.1.3	Mesh mode

This mode is similar to multi-hop mode. However, in this mode, relay nodes are supposed to have connections between each of them, if physically possible. Routing algorithms between relay nodes are necessary in this mode. An example of network topology in this case is shown in Figure 2.1.3.1.

Figure 2.1.3.1

Working mode of radio access network – Mesh Mode

#### 2.1.4	Peer-to-peer mode

In this mode, mobile stations are connected directly or through relay nodes, but no radio access point are explicit in their connections. An example of network topology in this case is shown in Figure 2.1.4.1.

Figure 2.1.4.1

Working mode of radio access network – Peer-to-Peer Mode

### 2.2	Duplexing

#### 2.2.1	FDD

Conventional frequency division duplex (FDD) operation allocates equal-size paired spectrum for uplink and downlink. It is expected that the future IMT-Advanced systems would require higher data rate and throughput mainly in downlink to support ultra high-speed asymmetric services, e.g. large-size file downloading (similar to broadband internet access) and high-quality video broadcasting (similar to digital TV). These asymmetric services encourage an asymmetric spectrum allocation for IMT-Advanced deployment.

#### 2.2.2	TDD

Conventional time division duplex (TDD) operation can support asymmetric transmission very well. Flexibility is available with respect to the degree of traffic asymmetry, depending on the co-channel and adjacent channel interference conditions. The spectrum efficiency of the arrangement is less dependent on the actual network traffic asymmetry since TDD can vary the degree of asymmetry within a specified range.

#### 2.2.3	Half duplex FDD

TBD

### 2.3	Multiple-Access technologies

#### 2.3.1	Single-carrier transmission

TBD

#### 2.3.1	Multi-carrier transmission

##### 2.3.1.1	OFDMA

##### 2.3.1.2	Multi-carrier CDMA

### 2.4	Multiple-Antenna technologies

#### 2.4.1	MIMO (MTMR)

##### 2.4.1.1	Single-User MIMO

##### 2.4.1.2	Multi-User MIMO

#### 2.4.2	Beam forming (Smart Antenna)

### 2.5	Channel Coding

#### 2.5.1	Turbo codes

Double binary tail-biting turbo codes can be regarded as one choice of improved turbo codes.

For the component encoder of the improved turbo codes, the Double Binary Circular Recursive Systematic Convolutional codes shall substitute the original Binary Recursive Systematic Convolutional Codes, which leads to the improvement of the link performance. Compared to the original binary turbo codes, the double binary turbo codes can eliminate the error floor, decrease the performance gap between the optimal algorithm and the approximate algorithm, and enhance the performance of high code rate.

Since the tail bits of UTRA Turbo coding reduce the throughput, tail-biting trellis termination can be considered to improve the transmission efficiency, and then the tail bits can be removed.

To obtain variable code rate and extend the application fields, the combination of rate matching and the improved turbo codes should be considered as a complement of turbo coding.

The improved turbo codes should have the capability of supporting iterative redundancy HARQ (IR_HARQ).

#### 2.5.2	Low density parity check codes (LDPC)

LDPC coding can be considered an alternative channel coding scheme in that it has such benefits as low complexity, large decoder throughput, low latency, and high coding performance.

A special type of LDPC codes, namely structured-LDPC codes, can achieve very efficient hardware architecture and routing. The code rate of LDPC codes is flexible by using different base matrices or by shortening or puncturing base matrices. The code size can be flexible by modifying one base matrix. As a typical choice, with single uniform base matrix and single uniform hardware structure, any code rate and any code size can be supported.

The LDPC codes should have the capability of supporting IR_HARQ.

For irregular LDPC codes, the protection abilities vary differently from the nodes’ degrees, and the differential protection ability of different degrees should be considered (e. g. HARQ).

The LDPC coded modulation possibly shall be exploited to improve the link performance.

### 2.6	Mobility management and RRM

#### 2.6.1	Centralized RRM

#### 2.6.2	Distributed RRM

#### 2.6.3	Inter-RAT spectrum sharing

#### 2.6.4	Inter-RAT mobility management

## 3	Mobile user interface

[Editor note: This section include new technologies that can improve user experience when using mobile communication service.]

### 3.1	Mobile user terminal design

### 3.2	New innovative network to humane interfaces

### 3.3	Human-free interface

### 3.4	RF micro-electro-mechanical systems (MEMS)

3.5	Reconfigurable networks

Appendix 2 

The application of multi-input-multi-output technology in 
IMT-Advanced system

In the IMT-Advanced system, MIMO technology mainly is introduced in the region the capacity already has approached the limit, or hot spot area.

# 1	The multi-antenna system application scenario

Better performance can be achieved in the following scenarios by using MIMO technology.

Scenario A (suburban macro): The wireless downlink channel, the base station position is high, the wireless signal scattering spots around the mobile terminations are rich. Then, looking from the terminal antenna, the wireless channel relevance of the base station with many transmit antenna is high, but looking from the base station antenna, the wireless channel relevance of the terminal with many receiving antenna is weak, namely low transmit diversity, high receive diversity scenario.

Scenario B (urban macro): The uplink wireless channel of scenario A, high transmit diversity, low receive diversity scenario.

Scenario C (urban micro): The wireless channel relevance of transmit, receiving antenna in uplink, downlink channel is medium, namely the medium transmit diversity, the medium receive diversity scenario.

Scenario D (line of sight-LOS): Because of the existence of the LOS component signal, the relevance between transmit and receive antennas is very strong, namely the low transmit diversity, the low receive diversity scenario.

Performance lost may be suffered in the following scenario: low SNR area and high mobile scenario.

Because MIMO technical may need channel information feedback between receiving and transmitting, based on present feedback mechanism, when UE makes the high speed migration (e.g. velocity >50km/h), The feedback speed is unable to support the variation rate of measure information; These measure information including the scope and phase information in closed loop diversity pattern, as well as feedback link quality information.

In addition, the micro honeycomb environment with rich multi-diameter condition can maximize the MIMO antenna gain, therefore the multi-antenna technology more suits for the micro honeycomb scenario such as the crowded city, the city, the room and so on. One kind of intelligent MIMO system based on the using boundary and user demand is shown in Figure 1.

Figure 1

The application of smart MIMO in different scenarios

# 2	MIMO’s impact on mobility

After introducing MIMO, the wireless environment of cell has improved, and the carry frequency quality of UE has obtained quite large gain, and the number of hand-over in mobility management has decreased. Because every pair of antennas have been configured a dedicated pilot channel, not a common pilot channel as in SISO. The condition of hand-over synthetically considers multi-pilot channel quality according to some algorithm.

Considering the following network configuration, there are MIMO cells and non-MIMO cells in the neighbour NodeB and in different frequency within a NodeB. Because of the mobility of UE and payload, that may lead to the following scenario.

figure 2

- UEs work at the F1 frequency in NodeB3, and move towards NodeB1 (Figure 2 A)

  - If the current UE is MIMO UE, when UE moves from NodeB3 towards NodeB1, system should touch off the soft hand-over. For service channel, network can select a best cell according to channel quality, make it as service cell.

  - If the current UE is MIMO UE, but works at the frequency F2 in NodeB3, when moving towards Node B, there are two different strategies: one is to make soft hand-over in same frequency, and the other is to make hard hand-over in different frequency, that makes the UE hand off the frequency which supports MIMO. The former can make use of the benefit which is leaded by soft hand-over, and the disadvantage is the UE still works on the non-MIMO cell. The latter avoids the disadvantage, but that leads the complexity of hand-over increases.

  - If the current UE is MIMO UE, whether working at F1 or F2, soft hand-over should be the optimum choice.

- When the above example occurs in one NodeB, the strategy should be the same as the different NodeB. The only difference is the hand-over is the softer hand-over.

- If MIMO UE moves into a non-MIMO cell(C), the network side can balance between to hold the MIMO service and to ensure UE interference to system at the same frequency is minimum. That is to say, network can configure higher threshold which is used to touch off moving towards non-MIMO, that ensures the largest delay of MIMO service. We can also use the same threshold as the normal hand-over, to ensure MIMO UEs can not produce too large payload to network.

- At different frequency in one NodeB, we also solve the payload balance through blind hand-over in one NodeB (D). The blind hand-over in one NodeB can be touched by the change of channel type. This can place the MIMO UEs and non-MIMO UEs in MIMO cells and non-MIMO cells as possible to ensure the performance of MIMO UE.

Appendix 3 

List of Acronyms and Abbreviations

| Terms | Descriptions |
| --- | --- |
| FDD | Frequency Division Duplex |
| TDD | Time Division Duplex |
| DL | Downlink |
| UL | Uplink |
| MAC | Media Access Control |
| PDU | Protocol Data Unit |
| RAT | Radio Access Technology |

There can be multiple packet flows associated with a single user, and multiple users associated with a single mobile terminal, e.g., in the case where a mobile terminal is a device providing service for multiple end devices.

No specific granularity for these parameters is implied by this requirement.