| Project | IEEE 802.21 Media Independent Handover Services <http://www.ieee802.org/21/> |  |  |
| --- | --- | --- | --- |
| Title | IMT-Advanced Requirements |  |  |
| Date Submitted | April 2007 |  |  |
| Source(s) | 802.21 WG Editor: Vivek Gupta Intel Corporation 2111 NE 25th Avenue, Hillsboro, OR 97124 | Voice:+1 503 712 1754 Fax: +1 503 264 4230 Mailto: vivekggupta@ieee.org |  |
| Re: | IMT-Advanced Requirements for inter-technology Handovers |  |  |
| Abstract | These are the requirements for inter-technology handover in IMT-Advanced systems. This input will be submitted to ITU-R through the process outlined by 802.18 WG. Eventually this input will be incorporated into the appropriate revision of the IMT.TECH document. |  |  |
| Purpose | Adoption |  |  |
| Notice | This document has been prepared to assist the IEEE 802.21 Working Group. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s). The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein. |  |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.21. |  |  |
| Patent Policy | The contributor is familiar with IEEE patent policy, as outlined in Section 6.3 of the IEEE-SA Standards Board Operations Manual <http://standards.ieee.org/guides/opman/sect6.html#6.3> and in Understanding Patent Issues During IEEE Standards Development <http://standards.ieee.org/board/pat/guide.html>. |  |  |

- Requirements for Inter-technology Handovers in

- IMT-Advanced systems

- 1 Overview

## 1.1 Purpose

The purpose of this document is to list requirements for inter-technology handovers in IMT-Advanced systems. These requirements will be submitted to ITU-R as per the process specified by the 802.18 WG. These requirements are expected to be incorporated in the appropriate revision of IMT.TECH document under development in 802.18 WG.

## 1.2 Radio Access Technologies (RATs) supported

The Mobile node (MN) is assumed to be multi-mode node with support for more than one interface type between which a handover can take place. The interfaces can be of the following types:

- IEEE 802.xx based interfaces such as 802.11, 802.15, 802.16, 802.20, 802.22

  - 3GPP based interfaces such as GSM/EDGE, UMTS, WCDMA and LTE networks

- 3GPP2 based interfaces such as CDMA2000 networks

- Support for IMT-2000 standard is highly desirable.

- 2 Definitions

## 2.1 Network detection

The process by which a mobile node collects information on networks in its locality, identifies its current point of attachment, and ascertains the validity of its link-layer configuration.

## 2.2 Network selection

The process by which a mobile node or a network entity collects policy information on networks in its locality and selects a network (possibly out of many available) to establish network-layer connectivity.

## 2.3 Seamless handover

Handover associated with a link switch between points of attachment, where the mobile node either experiences no degradation in service quality, security, and capabilities, or experiences some degradation in service parameters that is mutually acceptable to the mobile subscriber and to the target network that serves the newly connected interface.

## 2.4 Service continuity

Transparent maintenance of an active service while the mobile node transitions across coverage area of different networks.

- Functional Requirements

The following are the functional requirements for inter-technology handovers in IMT-Advanced systems.

## 3.1 Service Continuity

During handovers service continuity shall be maintained within a network interface and between different network interfaces. IMT-Advanced systems may support seamless handovers independent of the type of coupling between different access networks. The system should also be able to support broadcast based services.

## 3.2 Application Class

IMT-Advanced system shall support handovers for the following classes of applications and meet the performance requirements associated with them.

- Real time

- Delay sensitive

- Delay insensitive

- Best effort

## 3.3 Quality of Service (QoS)

The IMT-Advanced system shall provide a means for obtaining QoS information of each network involved in the handover process. It should be possible to obtain QoS information of target network while the MN is connected to the serving network.

During handovers, if the target network is unable to support the QoS levels of the serving network, the handover policy may not preclude the degradation of the QoS level of the session in the new network for the purpose of maintaining session continuity.

Other QoS parameters under consideration:

- Packet Transfer Delay

- Packet Loss

- Latency

- Throughput

- Packet Loss Rate

- Packet Error Rate

- Packet Transfer Delay Jitter

## 3.4 Measurement Reports

## IMT-Advanced system shall specify a means of reporting ACTIVE mode measurements of other target networks

## to facilitate in appropriate handover decision making and maintaining service continuity.

## 3.5 Network Discovery

IMT-Advanced systems shall provide services to the higher layers to help with network detection. There may be mechanisms that allow a mobile terminal to optimize detection of a useable attachment to a network through appropriate MAC and PHY indications (link layer events). These events may also trigger the handover process based on the results of network discovery. Other methods for optimized scanning and system discovery may also be considered.

## 3.6 Network Selection

IMT-Advanced system shall provide services to achieve optimum selection of networks prior to the handover process initiation. These services shall include mechanisms to obtain detailed information about different network elements such as link access and utilization, link quality, cost, security mechanisms, provider information and other such information elements which can aid in the handover decision making process. The system shall enable this information exchange between the mobile terminal and the network attachment point in a standard manner across different access networks. These services shall be available at initial device attachment or at other times (for background scanning of different networks) as well.

## 3.7 Security

IMT-Advanced systems shall provide a means to map security requirements according to individual access technologies. Security schemes in individual access technology shall be reused.

Mobile terminals may transition from one network interface to another network interface (different) as a result of handovers. Under such circumstances the handover shall not compromise the security of:

- Serving network interface

- Target network interface

- Mobile terminal

## 3.8 Handover Initiation

IMT-Advanced systems shall support both mobile initiated and network initiated handovers. The system shall also support both mobile controlled and network controlled handovers.

## 3.9 Power Management

IMT-Advanced systems shall support effective device power management by employing battery efficient network scanning procedures.  Wherever applicable the system shall enable network scanning opportunities to coincide with scheduled wake-up times to minimize the expended power by switching between the active and power save modes of operation for the wireless media employed.

## 3.10 Handovers due to Mobile Terminal Movement

The mobile terminal can move between networks at different speeds. IMT-Advanced systems shall support handovers with no measurable impact on the quality of any service when handover due to mobile terminal movement between any two networks occur at speeds below the limits specified by the involved networks (up to 120km/h for IMT-Advanced systems).

- References

- IEEE P802.21 Media Independent Handover Service Draft Technical Requirements, (doc 21-04-0087-12-0000), Sept 2004.