| Project | IEEE 802.21 Media Independent Handover Services <http://www.ieee802.org/21/> |  |
| --- | --- | --- |
| Title | Requirements and Suggested Amendments for IEEE 802.16 |  |
| Date Submitted | August, 2005 |  |
| Source(s) |  |  |
| Re: | 21-05-xxxx-00-0000-Requirements_Amendments_802_16 |  |
| Abstract | This contribution has initial set of requirements and suggested amendments for 802.16 access technology |  |
| Purpose | Requirements and Suggested Amendments for 802.16 |  |
| Notice | This document has been prepared to assist the IEEE 802.21 Working Group. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s). The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.21. |  |
| Patent Policy | The contributor is familiar with IEEE patent policy, as outlined in Section 6.3 of the IEEE-SA Standards Board Operations Manual <http://standards.ieee.org/guides/opman/sect6.html#6.3> and in Understanding Patent Issues During IEEE Standards Development <http://standards.ieee.org/board/pat/guide.html>. |  |

# 1   Introduction

# The purpose of this document is to identify requirements to be satisfied by IEEE 802.16 specification for supporting MIH Services as specified by the IEEE 802.21 specification. This document also includes suggestions for possible amendments to the IEEE 802.16g specification for satisfying the identified requirements.

# Scope of IEEE 802.16g Specification

## The IEEE 802.16g specification (current revision IEEE 802.16g-04/03r3, May 2005 http://ieee802.org/16/netman/docs/80216g-04_03r3.pdf  ) specifies amendments to 802.16 Air Interface for Fixed and Mobile Broadband Wireless Access Systems. These amendments are for Management Plane Procedures and Services.

The 802.16g specification includes primitives which are exposed to upper layers in a consistent manner for use by control and management plane protocols. The network that manages and controls the 802.16 device is abstracted as a Network Control and Management System (NCMS) that interfaces with the base stations. The NCMS is made up of several different functional entities that may be centrally located or distributed across the network. The exact functionality of these entities is outside the scope of the 802.16g specification. Any necessary inter-BS coordination in is handled through the NCMS. NCMS protocols are not included in the 802.16g specification.

The 802.16g specification only describes the management and control interactions between MAC/PHY/CS layers of the 802.16 devices and the NCMS. The specification also includes the definition of various Information Elements (IEs) and other protocol primitives that are exposed using Service Access Points (SAPs). This includes CS, MAC and PHY layer context information used by NCMS protocols to manage and control the air interface.

# 2   Requirements

## This section identifies specific 802.21 requirements that need to be met by the 802.16g(?) specification.

## The NCMS shall support MIH Function services for MS and BS. The NCMS shall deliver MIH Function services primitives which are transmitted through C_SAP or M_SAP to MIH Function via MIH_SME_SAP.

## The 802.16 Reference Model shall support MIH Function services.

## The 802.16 specification shall support link layer events as specified in the 802.21 specification. For each link layer event this may result in definition of a new primitive, or change in semantics of an existing primitive or just identification of an existing primitive with appropriate semantics in the 802.16 specification. The link layer events are identified in Table-1 in IEEE 802.21 specification.

## The 802.16 specification shall support link layer commands as specified in the 802.21 specification. For each link layer command this may result in definition of a new primitive, or change in semantics of an existing primitive or just identification of an existing primitive with appropriate semantics in the 802.16 specification.

## The 802.16 specification shall support a L2 transport for transferring remote events and remote command messages over the air interface between MIH Function on SS/MS and MIH Function on PoA (BS). The Remote event and remote command messages shall be transferable over either a basic connection or a primary connection.

## The 802.16 specification shall support a new ethertype for MIH. Convergence Sublayer in the 802.16 shall distinguish MIH ethertype and CS in MS or BS shall setup service flow with appropriate QoS with BS or MS respectively.

## The 802.16 specification shall support interactions between MIH Function on BS and MIH Function on some other PoA (such as AP in 802.11 network). These shall be supported as interactions between 802.16 BS and NCMS.

## The 802.16 specification shall support Information Elements identified by the 802.21 specification.

## The 802.16 specification shall support a L2 transport for querying/setting information elements (IEs) over the air interface between MIH Function on SS/MS and MIH Function on PoA (BS). The IEs shall be transferable over either a basic connection  or a primary connection .

## The 802.16 specification shall support MIH capable BS’s MIH Capability Delivery through broadcast message. This shall be accomplished by including capability information recommended by 802.21 specification in both DL-MAP message and Neighbor Advertisement (MOB_NBR-ADV) MAC management message.

## The 802.16 specification shall support advertisement of heterogeneous network information specified by 802.21 specification. The advertisement shall be achieved by including heterogeneous network information in the Neighbor Advertisement (MOB_NBR-ADV) MAC management message.

# Suggested Amendments

## The NCMS shall include support for MIH Function Services as shown in Fig-1 below. The MIH Function Services shall provide support for MIH Event service, MIH Command service and MIH Information service.

- Fig 1: Updates to NCMS Entities

## The 802.16 Reference Diagram shall be updated as per Figure 2. The C_SAP and M_SAP SAPs shall be amended to add support for 802.21 primitives.

- Fig 2: MIH Reference Diagram for 802.16

## The following set of events supported in the 802.21 specification shall be supported. (Table-1)

| Event Identifier | Event Type | Event Name | Description | 802.16 Primitive |
| --- | --- | --- | --- | --- |
| 1 | State Change | Link Up | L2 connection has been established |  |
| 2 | State Change | Link Down | L2 connection has been broken |  |
| 3 | Predictive | Link Going Down | L2 connection loss is imminent |  |
| 4 | State Change | Link Detected | A new link has been detected |  |
| 5 | State Change | Link Parameters Change | Link parameters has crossed specified threshold |  |
| 6 | Administrative | Link Event Rollback | Link event needs to be rolled back since the predictive event did not occur as predicted |  |
| 7 | Link Transmission | Link SDU Transmit Success | Success in transmitting SDU |  |
| 8 | Link Transmission | Link SDU Transmit Failure | Failure in SDU transmission |  |
| 9 | Link Synchronous | Link Handoff Imminent |  |  |
| 10 | Link Synchronous | Link Handoff Proceeding |  |  |
| 11 | Link Synchronous | Link Handoff Complete |  |  |
| ….. | …… | …… |  |  |

## The following set of IEs shall be specified in the 802.16 specification:

<TBD>

## A new MIH ethertype shall be defined. The MIH payload shall be encapsulated in 802.3 frames and sent using CS_SAP over the air interface (data plane).

## The following handover primitives defined in 802.16g specification in section 14.5.9.7.1, interactions between BS and NCMS, shall be amended to support inter-technology handovers as per 802.21 specification.

HO Request

HO Indication

HO Response

HO Confirmation

HO Start

HO Cancel

HO Directive