| Project | IEEE 802.21 Media Independent Handover Services <http://www.ieee802.org/21/> |  |
| --- | --- | --- |
| Title | Information Elements |  |
| Date Submitted | November, 2005 |  |
| Source(s) |  |  |
| Re: | 21-05-xxxx-00-0000-Information_Elements |  |
| Abstract | List of Information Elements |  |
| Purpose | List the common Information Elements |  |
| Notice | This document has been prepared to assist the IEEE 802.21 Working Group. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s). The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.21. |  |
| Patent Policy | The contributor is familiar with IEEE patent policy, as outlined in Section 6.3 of the IEEE-SA Standards Board Operations Manual <http://standards.ieee.org/guides/opman/sect6.html#6.3> and in Understanding Patent Issues During IEEE Standards Development <http://standards.ieee.org/board/pat/guide.html>. |  |

802.21 Information Service

The main goal behind the 802.21 Information service is to allow mobile terminals and network entities to discover information that can help/influence them in selection of appropriate networks during handovers. This information is intended to be primarily used by a policy engine type entity that can make effective handover decisions based on this information. This information service is expected to provide mostly static type of information. Other dynamic information about different access networks such as current available resource levels, state parameters, dynamic statistics etc. should be obtained directly from the respective access networks.

Some of the key motivations behind the 802.21 Information Service are as follows:

1] Provide information about different available access networks in a geographical area. Further this information could be retrieved using any wireless network. Thus in essence information about a nearby WiFi hotspot could be obtained by using a GSM or other cellular network

2] Provide static link layer information parametes that could help the mobile devices in selecting the appropriate access network. For example knowledge of whether security and QoS are supported on a particular access network may influence the decision to select such an access network during handovers.

3] The link layer information comprising of neighbor reports and information about capabilities of different PoAs could also aid in configuring the radios optimally (to the extent possible) for connecting to available/selected access networks. For example knowing about supported channels by different PoAs may help in configuring the channels optimally as opposed to scanning, beaconing etc and then finding out this information. However for most part dynamic link layer parameters have to be obtained or selected based on direct interaction with the access networks and the Information Service may not be able to help much in that regard.

4] Provide an indication of higher layer services supported by different access networks and other relevant information that may aid in making handover decisions. Such information may not be available (or could not be made available) directly from MAC/PHY layers of specific access networks, but could be provided as part of 802.21 Information service. For example in certain cases classification of different networks into categories such as public, enterprise, home, others etc. may influence the decision to handover. Other information here may be more vendor/network specific in nature and could be specified in that form.

# Suggested set of Information Elements

## The below table lists the set of suggested IEs. The IEs are organized as follows in the below table.

1] List of Available networks in a particular area

2] Information about individual access networks

3] Information pertaining to individual PoA for different access networks

4] Other information elements

The table also includes individual columns for 802.11 and 802.16 Applicability, indicating how relevant the IEs are to individual access networks and what specific amendments (if any) may be needed from different access networks.

| No | Name of Information Element | Description |  | Comments | 802.11 Applicability | 802.16 Applicability |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1.1 | List of Neighboring Access Networks | Link types of the networks that are available in a given geographical area. |  | : Ethernet : Wireless - Other : Wireless - IEEE 802.11 : Wireless – IEEE 802.16 : Wireless - CDMA2000 : Wireless - UMTS : Wireless - 1X-EV Etc. |  |  |  |  |  |  |
| For each Access Network the following IEs are defined |  |  |  |  |  |  |  |  |  |  |
| 2.1 | Number of Point of Attachments (PoA) for a specific Access Network in the Neighborhood | Number of APs, BSs etc. in the vicinity of client device |  |  | From Neighbor Reports | From Neighbor Reports |  |  |  |  |
| 2.2 | Network Operator | The operator of a network. |  |  | To be defined | First 24 bits of BS ID |  |  |  |  |
| 2.3 | Roaming Partners | Operators with which the current network operator has direct roaming agreements. |  | Each operator has the same structure as Network Operator information element. | To be defined | NSI_List |  |  |  |  |
| 2.4 | Cost | Indication of cost for service or network usage. |  | Cost is represented as a binary value, i.e., free or charged. Is it feasible to define this??? | To be defined | To be defined |  |  |  |  |
| 2.5 | Link Layer Security Capabilities | Security characteristics of the link layer |  | For example, authentication methods and cipher suites can be part of the security characteristics. | Already available | Already available |  |  |  |  |
| 2.6 | Link Layer QoS capabilities | QoS (Quality of Service) characteristics of the link layer |  | QoS classes, traffic priorities | Already available | Already available |  |  |  |  |
| For each PoA within an access network the following IEs can be defined |  |  |  |  |  |  |  |  |  |  |
| 3.1 | Address Information | MAC Address of PoA |  |  | Already available BSSID | Already available (BS ID) |  |  |  |  |
| 3.2 | Location of PoA | Geographical location of a given PoA. Multiple location types are supported including coordinate-based location information and civic address. | The coordinate-based location information is defined in RFC 3825 and consists of: - Latitude - Longitude - Altitude The civic address location information is TBD. |  |  |  |  |  |  |  |
| 3.3 | Data Rate | The minimum and maximum value of data rate supported by the link layer of a given PoA. | A data rate is represented as a 32-bit unsigned integer in unit of Kbps. |  | Already available | Already available (Traffic Rate/Service Flows) |  |  |  |  |
| 3.4 | PHY Type | The media PHY type. | The PHY type can be defined by media-specific MIB. |  | Already available | Already available |  |  |  |  |
| 3.5 | MAC Type | The media MAC type. | The MAC type can be defined by media-specific MIB. |  | Already available | Already available |  |  |  |  |
| 3.6 | Channel Range/Parameters | Spectrum range supported by the Channel for that PoA | This could be range in MHz, GHz etc. |  | Already available | Already available |  |  |  |  |
| 3.7 | Subnet Information | Information about subnets supported by a typical PoA | This could help in inter-subnet handovers. Is this feasible? |  |  |  |  |  |  |  |
| 3.8 | Individual PoA Capabilities Bitmap | Bitmap of PoA capabilities | Security Available Y/N QoS Available Y/N Internet access available Y/N ?? IP Version 4 supported Y/N ?? IP Version 6 supported Y/N ?? Others : |  |  |  |  |  |  |  |
| Other Information Elements |  |  |  |  |  |  |  |  |  |  |
| 4.1 | Operator/User policies etc. | Variable length datatype with a maximum bound |  |  |  |  |  |  |  |  |
| 4.2 | Vendor specific IEs |  |  |  |  |  |  |  |  |  |