| Project | IEEE 802.21 Media Independent Handover Services <http://www.ieee802.org/21/> |  |
| --- | --- | --- |
| Title | Requirements and Suggested Amendments for IEEE 802.11 |  |
| Date Submitted | November, 2005 |  |
| Source(s) | Vivek Gupta (Editor) |  |
| Re: | 21-05-0350-04-0000-Requirements_Amendments_802_11 |  |
| Abstract | This contribution has initial set of requirements and suggested amendments for 802.11 access technology |  |
| Purpose | Requirements and Suggested Amendments for 802.11 |  |
| Notice | This document has been prepared to assist the IEEE 802.21 Working Group. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s). The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.21. |  |
| Patent Policy | The contributor is familiar with IEEE patent policy, as outlined in Section 6.3 of the IEEE-SA Standards Board Operations Manual <http://standards.ieee.org/guides/opman/sect6.html#6.3> and in Understanding Patent Issues During IEEE Standards Development <http://standards.ieee.org/board/pat/guide.html>. |  |

Key Contributors

| Name | Company | Address | Phone | Fax | Email |
| --- | --- | --- | --- | --- | --- |
| Eleanor Hepworth | Siemens Roke Manor | Roke Manor, Romsey, Hampshire, SO51 0ZN, UK | +44 1794-833341 | +44 1794 833434 | eleanor.hepworth@roke.co.uk |

# 1.  Introduction

# The purpose of this document is to identify requirements to be satisfied by IEEE 802.11 specification for supporting MIH Services as specified by the IEEE 802.21 specification. This document also includes suggestions for possible amendments to the IEEE 802.11 specification for satisfying the identified requirements.

The IEEE 802.21 Information Elements can be categorized in terms of the phase of the network join process at which they are exchanged. Three main phases can be identified that apply generally, both to IEEE 802.11 and other networks:

- Pre-attachment – before any resources or state for the client are established in the network. In IEEE 802.11 this is equivalent to pre-association.

- Pre-authentication – when data can be exchanged over the IEEE 802.1X uncontrolled port. At this point the protocol frames may be at a slightly higher layer, but the information that may be exchanged is still limited.

- Post-authentication – when data can be exchanged over the IEEE 802.1X controlled port. At this point any information can be exchanged.

# 2.  Requirements

This section identifies specific 802.21 requirements that need to be met by the 802.11 specification.

## 2.1  General Requirements

- The 802.11 Reference Model shall support Media Independent Handover (MIH) Function services. The 802.11 specification shall define appropriate SAPs, primitives and information elements to support IEEE 802.21 MIH functionalities.

- The IEEE 802.11 specification shall provide a means for APs and STAs to indicate support for the 802.21 specification.  This shall be in the form of a Media Independent Handover capability indication that should be made available before attaching to the network.

## 2.2.  SAP Requirements

- The IEEE 802.11 specification shall support link layer events as specified in the 802.21 specification. For each link layer event, this may result in definition of a new primitive, or change in semantics of an existing primitive or just identification of an existing primitive with appropriate semantics in the 802.11 specification.  The link layer events are identified in Table-1 in the IEEE 802.21 specification (and also repeated in section 3 of this document).

- The 802.11 specification shall support link layer commands as specified in the 802.21 specification. For each link layer command this may result in definition of a new primitive, or change in semantics of an existing primitive or just identification of an existing primitive with appropriate semantics in the 802.11 specification.

<Currently no link layer commands are specified by 802.21>

- The 802.11 specification shall support the handover commands as specified in the 802.21 specification. These handover commands provide support for network initiated handoffs across different networks. These commands are listed in Table-3 of the 802.21 specification.

## 2.3  Information Elements

The following Information Elements (IEs) must be supported by the 802.11 specification. The IEs help the handover policy engine in making appropriate handover decisions.

The table also includes individual columns for 802.11 Applicability and Availability, indicating how relevant the IEs are to individual 802.11 access networks and what specific amendments (if any) may be needed from 802.11 access network.

| No | Name of Information Element |  | Description |  | Comments | 802.11 Applicability |  | Availability |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1.1 | List of Neighboring Access Networks |  | Link types of the networks that are available in a given geographical area. |  | : Ethernet : Wireless - Other : Wireless - IEEE 802.11 : Wireless – IEEE 802.16 : Wireless - CDMA2000 : Wireless - UMTS : Wireless - 1X-EV Etc. |  |  | Pre-authentication |  |  |  |  |
| For each Access Network the following IEs are defined |  |  |  |  |  |  |  |  |  |  |  |  |
| 2.1 | Number of Point of Attachments (PoA) for a specific Access Network in the Neighborhood |  | Number of APs, BSs etc. in the vicinity of client device |  |  | From 802.11 site Reports |  | Pre-authentication |  |  |  |  |
| 2.2 | Network Operator |  | The operator of a network. |  | Is this access network operator or core network operator | To be defined |  | Pre-attachment |  |  |  |  |
| 2.3 | Roaming Partners |  | Operators with which the current network operator has direct roaming agreements. |  | Each operator has the same structure as Network Operator information element. | To be defined |  | Pre-authentication |  |  |  |  |
| 2.4 | Cost |  | Indication of cost for service or network usage. |  | Cost is represented as a binary value, i.e., free or charged. We may need more details of cost Is it feasible to define this in 802.11 as free/not free??? | To be defined |  | Pre-authentication |  |  |  |  |
| 2.5 | Link Layer Security Capabilities |  | Security characteristics of the link layer |  | For example, authentication methods and cipher suites can be part of the security characteristics. | Already available |  | Pre-authentication |  |  |  |  |
| 2.6 | Link Layer QoS capabilities |  | QoS (Quality of Service) characteristics of the link layer |  | QoS classes, traffic priorities | Already available |  | Pre-authentication |  |  |  |  |
| For each PoA within an access network the following IEs can be defined |  |  |  |  |  |  |  |  |  |  |  |  |
| 3.1 | Address Information | MAC Address of PoA |  |  |  |  | Already available BSSID | Pre-authentication |  |  |  |  |
| 3.2 | Location of PoA | Geographical location of a given PoA. Multiple location types are supported including coordinate-based location information and civic address. |  | The coordinate-based location information is defined in RFC 3825 and consists of: - Latitude - Longitude - Altitude The civic address location information is TBD. |  |  | ???? |  |  |  |  |  |
| 3.3 | Data Rate | The minimum and maximum value of data rate supported by the link layer of a given PoA. |  | A data rate is represented as a 32-bit unsigned integer in unit of Kbps. |  |  | Already available | Pre-attachment |  |  |  |  |
| 3.4 | PHY Type | The media PHY type. |  | The PHY type can be defined by media-specific MIB. |  |  | Already available | Pre-authentication |  |  |  |  |
| 3.5 | MAC Type | The media MAC type. |  | The MAC type can be defined by media-specific MIB. |  |  | Already available | Pre-authentication |  |  |  |  |
| 3.6 | Channel Range/Parameters | Spectrum range supported by the Channel for that PoA |  | This could be range in MHz, GHz etc. |  |  | Already available | Pre-authentication |  |  |  |  |
| 3.7 | Subnet Information | Information about subnets supported by a typical PoA |  | This could help in inter-subnet handovers. Is this feasible? |  |  | Not applicable | Pre-authentication |  |  |  |  |
| 3.8 | Individual PoA Capabilities Bitmap | Bitmap of PoA capabilities |  | Security Available Y/N QoS Available Y/N Internet access available Y/N ? IP Version 4 supported Y/N ? IP Version 6 supported Y/N ? Emergency services supported Y/N Others : Does it make sense to classify 802.11 networks as Public, Home, Enterprise, etc. |  |  | 802.11 specific IEs to be identified and defined | Pre-authentication |  |  |  |  |
| Other Information Elements |  |  |  |  |  |  |  |  |  |  |  |  |
| 4.1 | Operator/User policies etc. |  | Variable length datatype with a maximum bound |  |  | Not Applicable |  | Post-authentication |  |  |  |  |
| 4.2 | Vendor specific IEs |  |  |  |  | Not Applicable |  | Post-authentication |  |  |  |  |

## 2.4   Transport Requirements

- The 802.11 specification shall support a L2 transport for transferring remote events and remote command messages over the air interface between the MIH function on the STA and the MIH Function on the PoA (AP).

- The 802.11 specification shall support a L2 transport that allows the STA to query and set the values of different Information Elements as identified by the 802.21 specification.

- The STA shall be able to use the information service and query specific IEs even when not attached, or attached but not authenticated with the network. The IEs that need to be supported even in the pre-attachment and pre-authentication states shall be specified by the 802.21 specification.

- The 802.11 specification shall support a new ether type for supporting MIH functionality over the data plane.

# 3.  Suggested Amendments

## 3.1 The 802.11 Reference Diagram shall be updated as per Figure 1.

- Fig 1: MIH Reference Diagram for 802.11

- 3.2 The remote event and remote command messages shall also be transferable using a management frame, as this allows commands and event messages to have a higher priority than normal data plane traffic.

| Event Id | Event Type | Event Name | Description | Comments | 802.11 Primitive |
| --- | --- | --- | --- | --- | --- |
| 1 | State Change | Link Up | L2 connection has been established | Define all of these events in Infrastructure mode only | Association and 4 way handshake successfully completed |
| 2 | State Change | Link Down | L2 connection has been broken | There may be several reasons for Link Down | Disassociation completed or last 3 consecutive L2 transmissions have timed out and STA cannot listen to beacons. |
| 3 | Predictive | Link Going Down | L2 connection loss is imminent | Based on signal hysteresis and other analysis. Specific analysis is outside of spec, just specify the semantics |  |
| 4 | State Change | Link Detected | A new link has been detected |  | Can listen to the beacon, or got response to a probe message |
| 5 | State Change | Link Parameters Change | Link parameters has crossed specified threshold | Parameters like signal strength, speed, QoS etc. have crossed pre-specified thresholds. |  |
| 6 | Administrative | Link Event Rollback | Link event needs to be rolled back since the predictive event did not occur as expected |  |  |
| 7 | Link Transmission | Link SDU Transmit Success | Success in transmitting SDU | These may be useful during subnet changes (intra-ESS handovers) |  |
| 8 | Link Transmission | Link SDU Transmit Failure | Failure in SDU transmission | These may be useful during subnet changes (intra-ESS handovers). |  |
| 9 | Link Synchronous | Link Handoff Imminent |  | These may be useful during subnet changes (intra-ESS handovers) |  |
| 10 | Link Synchronous | Link Handoff Proceeding |  | These may be deleted from 802.21 draft |  |
| 11 | Link Synchronous | Link Handoff Complete |  | These may be deleted from 802.21 draft |  |
| ….. | …… | …… |  |  |  |

3.3 The following table provides an overview of the commands/messages that need to be supported to enable selection of new network between MIH on network and MIH on STA. These may be primarily supported at L3.

Do we need to support these at L2?

If yes, then do we need more than just a L2 transport from 802.11?

| Id | Command Name | MIHF <> MIHF | Description | Comments |
| --- | --- | --- | --- | --- |
| 1 | MIH Handover Initiate | Client <> Network PoA | Initiates handovers and sends a list of suggested networks and suggested PoA. |  |
| 2 | MIH Handover Prepare | Network (old PoA) <> Network (new PoA) | This command is sent by MIHF on old PoA to MIHF on suggested new network at new PoA. This allows the client to query for resources on new PoA and also allows to prepare the new PoA for handover |  |
| 3 | MIH Handover Commit | Client <> Network | In this case the client commits to do the handover based on selected choices for network and PoA. |  |
| 4 | MIH Handover Complete | Network (new PoA) <> Network (old PoA) | This is a notification from new PoA to old PoA that handover has been completed, new PoA has been established and any pending packets may now be forwarded to the new new PoA. |  |
| 5 | MIH Network Address Information | Network (new PoA) <> Network(old PoA) <> Network (Access Router/ Foreign Agent) | This command is sent by MIHF on old PoA to MIHF on suggested new network at new PoA. New PoA may relay this command to the AR with MIHF. This allows the client to have network address related information prior to the handover to the new PoA. |  |

### 3.4 MIH Capability Information field

As the capability information field is nearly exhausted, the last bit in the Capability Information Field as defined in the IEEE 802.11 1999 (Reaff 2003) standard specification is set to 1 in the existence of the Extended Capability Information field.

| B0 | B1 | B2 | B3 | B4 | B5 | B6 | B7 | Reserved | B15 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ESS | IBSS | CF Pollable | CF-Poll Request | Privacy | Short Preamble | PBCC | Channel Agility | B8:B14 | Extended Capability Field |
| ← 2 octets → |  |  |  |  |  |  |  |  |  |

Figure 2 Capability Information Fixed Field

A new information element is defined to enable further extensions to be advertised and negotiated in IEEE 802.11.  The Extended Capability information element field is present under the same conditions for the Capability information variable length field.  The format for this information element is defined in Figure :

| Order | Size (octets) | Description |
| --- | --- | --- |
| 1 | 1 | TBD Element ID (to be assigned by the IEEE Assigned Numbers Authority): defines the Extended Capability IE |
| 2 | 1 | Length |
| 3 | N | Extended capabilities (Variable Size) |

Figure 3 Extended Capability Information Element

Where:

Length indicates a value representing the variable size of the Extended Capabilities field.

The currently defined capabilities in this field are defined as follows:

| Byte 0 | Bit 0 | MIH Capability | Indicates that 802.21 MIH Capability is supported |
| --- | --- | --- | --- |
|  | Bit 1-7 | Reserved | Set to 0, if not used |

Figure 4 Extended Capabilities Field

Where:

Byte 0, Bit 0: a STA or AP advertises its ability to support MIH Capability.  When this bit is set to 1 within the beacons, probes, association and re-association request or response messages, then it indicates support for the 802.21 Specification. If this bit is set to 0 then the device does not support 802.21 Specification