Use case example for inclusion in Annex (alternative)

We propose to include the following use case for information purposes in an Annex.

This use case considers one terminal and one Mobility Management Entity (MME) in the network, although the decision engine can be local to the terminal.

A - First step: setting the QoS requirements

QoS requirements passed along side app session setup signalling (e.g., SIP/SDP)

In this step, the application communicates its QoS requirements to the Decision Engine.

The MIH is the interface between the application and the decision engine.

B - Second Step: Configuring thresholds

Over app control signalling

(SIP, etc.):

Config_threshold_request

MIH_Configure_Thresholds.request

MIH_Configure_Thresholds.response

Application

Config_threshold_response

Based on the QoS requirements set in step A, the Decision Engine can (re)configure the terminal's thresholds for different QoS parameters.

C- Third step: reporting QoS measurements

In this step, if a threshold is crossed for a particular QOS parameter, the Link_Parameter_Report is sent to the MIH which in turns generates a MIH_Link_Parameter_Report and sends it to the Decision Engine.

D - Fourth Step: Handover

In this step, if the Decision Engine decides an HO is needed for terminal A (based on the QoS Parameter reports it got at step C), it sends a MIH_Switch.request to the MIH which trigger the actual HO. Upon success, the Decision Engine (and possibly the application) is notified. The application can keep on operating at an optimal QoS level.