| Project | IEEE 802.21 Media Independent Handover Services <http://www.ieee802.org/21/> |
| --- | --- |
| Title | MEDIA INDEPENDENT HANDOVER |
| Date Submitted | November, 2005 |
| Source(s) | Qiaobing Xie, Peretz Feder, Andrea Francini, Subir Das, Vivek Gupta |
| Re: | 21-05-0xxx-00-0000-communication_model_text |
| Abstract | Draft Text for MEDIA INDEPENDENT HANDOVER Specification |
| Purpose | Proposal for 802.21 |
| Notice | This document has been prepared to assist the IEEE 802.21 Working Group. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s). The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein. |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.21. |
| Patent Policy | The contributor is familiar with IEEE patent policy, as outlined in Section 6.3 of the IEEE-SA Standards Board Operations Manual <http://standards.ieee.org/guides/opman/sect6.html#6.3> and in Understanding Patent Issues During IEEE Standards Development <http://standards.ieee.org/board/pat/guide.html>. |

List of Contributors

| Name | Company | Address | Phone | Fax | Email |
| --- | --- | --- | --- | --- | --- |
| Peretz Feder, Andrea Francini | Lucent Technologies | 67 Whippany Rd , Whippany, NJ 07981, USA | 919 932 4369 |  | pfeder@lucent.com |
| Qiaobing Xie | Motorola | 1501 W. Shure Drive, Arlington Heights, Il 60004 | 847-632-3028 |  | qxie1@motorola.com |
| Subir Das | Telcordia Technologies Inc | One Telcordia Drive, RRC-1B229, Piscataway, NJ 08854-4157 | 732 699 2483 |  | subir@research.telcordia.com |
| Vivek Gupta | Intel Corporation | 2111 NE 25th Avenue, Hillsboro, OR 97124 | 503 712 1754 |  | vivek.g.gupta@intel.com |

### 5.3.0 Communication Model

MIH functions may communicate with each other for various purposes, such as in the case of remote MIIS, MIES, and MICS. MIH communication happens when MIH related information is passed from one MIHF to another.

The following definitions are used to describe the MIH communication model:

• Link: Communication medium for the exchange of L2 messages between adjacent L2 entities.

• Point of Attachment (PoA): Network-side endpoint of link with UE as the other endpoint.

• MIH Point of Service (MIH PoS): Network-side counterpart of an MIH-enabled UE for the exchange of MIH information.

A UE attaches to a network through a link that terminates at a PoA.

The UE exchanges MIH information with MIH PoS's. Generally, the MIH PoS resides at an interface on a network node with which the MIH function of the node is registered for MIH services. An MIH PoS may or may not be co-located with a PoA on the same (L2) interface.

It can happen that an interface on an MIH-capable node (i.e., a node that contains an MIH function instance) does not host an MIH PoS because the local MIH function is not registered with that interface for MIH support. However, an MIH PoS may still be found in the same network node at a different interface.

The UE may use L2 transport for exchanging MIH information with an MIH PoS that is co-located with a PoA.

The UE must use L3 transport for exchanging MIH information with an MIH PoS that is not co-located with a PoA.

Figure X shows the MIH communication model. The model intends to show MIH functions in different distinctive roles and the communications relationships amongst them.

It is important to note that each of the communication relationships shown in the diagram below does not imply a particular interface or transport mechanism. Rather, a communication relationship only intends to show that MIH related information passing is possible between the two distinctive MIH functions. Moreover, each communication relationship shown in the diagram may encompass different type of 802.21 interfaces, different transport mechanisms used (e.g., L2, L3), and/or different MIH related content being passed (e.g., IS, CS, ES, etc.).

Figure X: Communication Model

Depending on its position in the system, five distinctive roles of MIH function are identified in the communication model:

- MIH on a UE,

- MIH PoS on a serving PoA (which is the PoA that  the UE is currently attached to),

- MIH PoS on a candidate PoA (which is a PoA that the UE is aware of but not currently attached to, and can potential become the target PoA when a handover occurs),

- MIH PoS on a non-PoA network entity, and

- MIH (not a PoS) on a non-PoA network entity.

The following five communication reference points are also identified in the communication model:

- Communication reference point R1

Reference Point R1 refers to MIH procedures between the MIH on a UE and an MIH PoS on the serving PoA. R1 may encompass communications interfaces over both L2 and L3 and above, and content passed over R1 can be related to MIIS, MIES, or MICS.

- Communication reference point R2

Reference Point R2 refers to MIH procedures between the MIH on a UE and an MIH PoS on a candidate PoA. R2 may encompass communications interfaces over L2, and content passed over R2 can be related to MIIS, MIES, or MICS.

- Communication reference point R3

Reference Point R3 shows MIH procedures between the MIH on a UE and an MIH PoS on a network entity that is not a PoA. R3 may encompass communications interfaces over L3 and above and MIH content passed over R3 may be related to MIIS, MIES, or MICS.

- Communication reference point R4

Reference Point R4 refers to MIH procedures between an MIH PoS and a non-PoS MIH. R4 may encompass communications interfaces over L3 and above, and MIH content passed over R4 may be related to MIIS, MIES, or MICS.

- Communication reference point R5

Reference Point R5 refers to MIH procedures between two MIH PoSs. R5 may encompass communications interfaces over L3 and above, and MIH content passed over R5 may be related to MIIS, MIES, or MICS.