IEEE P802.22
Wireless RANs

| Pros and Cons of TDD and FDD |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2005-03-14 |  |  |  |  |
| Author(s): Ashish Pandharipande, Gerald Chouinard |  |  |  |  |
| Name | Company | Address | Phone | email |
| Gerald Chouinard | CRC | 3701 Carling Ave. Ottawa, Ontario, Canada | 613-998-2500 | gerald.chouinard@crc.ca |

Abstract

This document tries to summarize the pros and cons of TDD and FDD duplexing techniques
in the context of Wireless RAN’s.

Notice: This document has been prepared to assist IEEE 802.22. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s).  The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein.

Release: The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication.  The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.22.

Patent Policy and Procedures: The contributor is familiar with the IEEE 802 Patent Policy and Procedures

<http://standards.ieee.org/guides/bylaws/sb-bylaws.pdf>, including the statement "IEEE standards may include the known use of patent(s), including patent applications, provided the IEEE receives assurance from the patent holder or applicant with respect to patents essential for compliance with both mandatory and optional portions of the standard."  Early disclosure to the Working Group of patent information that might be relevant to the standard is essential to reduce the possibility for delays in the development process and increase the likelihood that the draft publication will be approved for publication.  Please notify the Chair <Carl R. Stevenson> as early as possible, in written or electronic form, if patented technology (or technology under patent application) might be incorporated into a draft standard being developed within the IEEE 802.22 Working Group. If you have questions, contact the IEEE Patent Committee Administrator at <patcom@ieee.org>.

Pros and cons of FDD and TDD for Wireless LAN’s

TDD pros:

- flexible split between downstream (forward) and upstream (return) peak capacity (sustained capacity can theoretically be increased to the full system capacity for a single subscriber)

- this split can be changed on the fly at the end of a frame

- needs only one RF channel

- Rx channel characteristics can be used for Tx (e.g., Transmit Power Control, adaptive antenna arrays)

- same antenna is used for both Tx and Rx

- no need for RF duplex filtering, a simple switch to the antenna in needed

- simpler quiet period and RF interference sensing on a single channel.

TDD cons:

- does not allow for full-duplex operation

- needs to include quiescent period to allow for signal round-trip delay, resulting in loss of capacity (e.g., 79% to 97.4% for 500 μs packet at 40 km)

- needs more complex synchronization and polling schemes

- needs for more complex broadcast frames for terminal association

- needs synchronization among wireless networks in the same area to avoid base station to base station interference, either co-channel or adjacent channel (such synchronization would remove the flexibility of changing the downstream/upstream peak capacity split on a network basis)

- where base station to base station synchronization is not possible in a multicell environment, frequency reuse from cell to cell decreases significantly (high elevation omni-directional base station antennas create interference over large distance)

- more stringent noise isolation requirements between the Tx and Rx chain given that it is the same channel for both

- difficult to allow time stretching of the Tx burst from CPE’s to reduce the peak power (e.g., OFDMA).

FDD pros:

- use of two separate RF channels for downstream (the forward direction from the base station) and for upstream (the return channel to the base station).  Each channel can be selected to better suit the requirement, i.e., using a lower frequency channel for return to take advantage of better propagation to maximize range for a given maximum subscriber station EIRP, and choosing channel to avoid specific interference in a specific direction.

- allows choice of largely separated channels to ease the filtering needed to duplex the Tx and Rx signals

- downstream/upstream channel sizes can be set to correspond to the peak traffic asymmetry profile, i.e., a 4 to 1 profile can result in a downstream channel four times greater than the upstream channel (upstream TV channel could be shared by more than one network in an area).  (Sustained capacity can theoretically be increased to the peak system capacity in each direction for a single subscriber)

- allows full duplex operation through proper duplexing (RF channel filtering) or circulator (if orthogonal polarizations are used for Tx and Rx) at the base and subscriber stations

- allows use of full duplex operation at the base station to maintain full capacity but half-duplex operation at the CPE to eliminate the need for complex duplexing filters or orthomode transducers, a simple switch can then be used with a single antenna (no switch needed for separate antennas) to keep CPE cost down

- easier monitoring of the RF signal for capturing broadcast frames for terminal association

- easier timing, synchronization and polling

- interference to nearby cells is limited by CPE antenna discrimination and terrain blockage if proper channels are used (avoid using a downstream channel for upstream in the same area, same thing for adjacent channels)

- accomodates time stretching of the Tx burst from CPE’s to reduce the peak power (e.g., OFDMA).

FDD cons:

- needs paired RF channels

- needs two RF chains at two different frequencies (tunable for frequency agility)

- needs duplexing filters for channel separation between for Tx and Rx for co-polarized operation or orthomode transducers for Tx-Rx cross-polarized solutions

- more complex CPE combined antenna (esthetics, form factor and cost are critical for success)

- does not allow change in split of peak capacity between forward and return links

- more difficult transmit power control prior to full association (propagation on Tx and Rx channels can be different for large frequency separation)

- more complex interference sensing on two channels (need quiet period on each link) and difficulty in sensing a remote base station.

_____________________