IEEE P802.22
Wireless RANs

| Proposed improvements to the DS-MAP and US-MAP |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2007-07-05 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Gerald Chouinard | CRC | 3701 Carling Avenue, Ottawa, Ontario, Canada K2H 8S2 | 1-613-998-2500 | gerald.chouinard@crc.ca |

Abstract

This contribution proposes changes to the DS-MAP and US-MAP sections of the 802.22 Working Document version 0.3.6.  Most changes are explained in the accompanying comments.

Notice: This document has been prepared to assist IEEE 802.22. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s).  The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein.

Release: The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication.  The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.22.

Patent Policy and Procedures: The contributor is familiar with the IEEE 802 Patent Policy and Procedures

<http://standards.ieee.org/guides/bylaws/sb-bylaws.pdf>, including the statement "IEEE standards may include the known use of patent(s), including patent applications, provided the IEEE receives assurance from the patent holder or applicant with respect to patents essential for compliance with both mandatory and optional portions of the standard."  Early disclosure to the Working Group of patent information that might be relevant to the standard is essential to reduce the possibility for delays in the development process and increase the likelihood that the draft publication will be approved for publication.  Please notify the Chair <Carl R. Stevenson> as early as possible, in written or electronic form, if patented technology (or technology under patent application) might be incorporated into a draft standard being developed within the IEEE 802.22 Working Group. If you have questions, contact the IEEE Patent Committee Administrator at <patcom@ieee.org>.

Proposed improvements to the DS-MAP and US-MAP section

    - Downstream Map (DS-MAP)

The format of a DS-MAP message is shown in Table 1. The DS-MAP message defines the access to the downstream information. If the length of the DS-MAP message is a non-integral number of bytes, the length field in the MAC header is rounded up to the next integral number of bytes. The message shall be padded to match this length, but the CPE shall disregard the four pad bits.

- —DS-MAP message format

| Syntax | Size | Notes |
| --- | --- | --- |
| DS-MAP_Message_Format() { |  |  |
| Management Message Type = 1 | 8 bits |  |
| Synchronization Field | 16 bits | Table 2 |
| DCD Count | 8 bits | Matches the value of the configuration change count of the DCD, which describes the downstream burst profiles that apply to this map. |
| BS ID | 48 bits | See Table 43 |
| AMC Subchannel | 1 bit | Indicates whether the adjacent subcarrier permutation is used in the downstream or not. |
| Begin PHY Specific Section { |  |  |
| for (i = 1; i  n; i++) { |  |  |
| DS-MAP_IE() | Variable | PHY specific () |
| } |  |  |
| } |  |  |
| Padding Nibble | 4 bits |  |
| } |  |  |

- —Synchronization field

| Syntax | Size | Notes |
| --- | --- | --- |
| Synchronization_field() { |  |  |
| Frame Duration Code | 8 bits | See Table 43 |
| Frame Number | 8 bits | See Table 43 |
| } |  |  |

      - DS-MAP IE

- —DS-MAP information elements

| Syntax | Size | Notes |
| --- | --- | --- |
| DS-MAP_IE() { |  |  |
| DIUC | 5 bits | 6.8.2.1.1 |
| If (DIUC == 30) |  |  |
| Extended DIUC Dependent IE | Variable | 6.8.2.1.2 |
| else { |  |  |
| If (INCLUDE_CID) { |  | The DS-MAP starts with INCLUDE_CID =0. INCLUDE_CID is toggled between 0 and 1 by the CID-SWITCH_IE() – see 6.8.2.1.2.2 |
| N_CID | 4 bits | Number of CIDs assigned for this IE |
| for (i=0; i<N_CID; i++) |  |  |
| CID | 12 bits | Unicast/Multicast/Broadcast CID. In case of a dummy allocation, the BS shall use the Padding CID – see Table 276 |
| } |  |  |
| Duration | 11 bits | Number of MAC slots (29*60= 1740) |
| Boosting | 3 bits | 000: normal (not boosted) 001: +6dB 010: -6dB 011: +9dB 100: +3dB 101: -3dB 110: -9dB 111: -12dB |
| } |  |  |
| If (!byte_boundary) |  |  |
| Padding Nibble | 4 bits |  |
| } |  |  |

        - DIUC Allocations

Table 4  illustrates the various DIUC values used in the MAC. In particular, it is important to highlight the use of the self-coexistence DIUC interval in Passive Mode, which can be seen as an in-frame quiet period, where the intention is to sense for the presence of other co-located 802.22 cells in the area. More specifically, this interval is used by 802.22 stations to look for both CBP and BS beacons. This allows flexibility for the 802.22 system to efficiently coexist with other nearby 802.22 systems.

The DIUC = 12 may be used for allocation of channels for PAPR reduction schemes.

- —DIUC values

| DIUC | Usage |
| --- | --- |
| 0 | Self-Coexistence (Passive Mode) |
| 1-28 | Burst Profiles (Fixed: 4 rates*3 modulations + 1/4QPSK= 13) [Note: type of FEC codec indicated in DCD, one type per BS] |
| 29 | PAPR Reduction |
| 30 | Extended DIUC |
| 31 | End of MAP |

        - DS-MAP Extended DIUC IE

- —DS-MAP extended IE general format

| Syntax | Size | Notes |
| --- | --- | --- |
| DS_Extended_IE() { |  |  |
| Extended DIUC | 4 bits |  |
| Length | 4 bits |  |
| Unspecified Data | Variable | All bits set to zero. |
| } |  |  |

    - Upstream Map (US-MAP)

The format of a US-MAP message is shown in Table 6. The US-MAP message defines the access to the upstream channel.

The CID field carried in the MAC header of the PDU where this message is transmitted represents the assignment of the IE to a unicast address, or the padding CID. When specifically addressed to allocate a bandwidth grant, the CID shall be the Basic CID of the CPE. A UIUC shall be used to define the type of upstream access and the upstream burst profile associated with that access. An Upstream_Burst_Profile shall be included in the UCD for each UIUC to be used in the US-MAP.

- —US-MAP message format

| Syntax | Size | Notes |
| --- | --- | --- |
| US-MAP_Message_Format() { |  |  |
| Management Message Type = 3 | 8 bits |  |
| UCD Count | 8 bits | Matches the value of the Configuration Change Count of the UCD, which describes the upstream burst profiles that apply to this map. |
| Allocation Start Time | 5 bits | Effective start time (in symbols) of the upstream allocation defined by the US-MAP |
| If (AMC Subchannel){ |  |  |
| US_BAND_MAP | Variable | See Table 7 |
| } |  |  |
| Begin PHY Specific Section { |  |  |
| for (i = 1; i  n; i++) { |  |  |
| US-MAP_IE() | Variable | PHY specific (6.8.4.1) Define upstream bandwidth allocations. Each US-MAP message shall contain at least one IE that marks the end of the last allocated burst. (UIUC=31 as defined in Table 9). |
| } |  |  |
| } |  |  |
| If (!byte_boundary) |  |  |
| Padding Nibble | 0-7 bits |  |
| } |  |  |

- — US_BAND_MAP

| Syntax | Size | Notes |
| --- | --- | --- |
| US_BAND _MAP() { |  |  |
| New format Indicator | 1bit | 0 = Use the latest US_BAND_MAP 1 = New US_BAND_MAP It can be 1 or 0 on the first frame, but only 0 on the other frames in the superframe. |
| if(New format Indicator) |  |  |
| { |  |  |
| Nband | 6bits | The total number of AMC bands selected for AMC subchannel |
| if(Nband < 10 or Nband > 50){ |  |  |
| for(i=0 ; i<Nband ; i++) |  |  |
| Band Index | 6bits | If Nband is less than 10, each BandIndex indicates the index of the selected AMC band for AMC subchannels. If Nband is larger than 50, it indicates the index of the diversity band not using for AMC subchannels. |
| }else{ |  |  |
| BandBITMAP | 60bits | The n-th LSB represents the n-th band. If n-th band is AMC band for AMC subchannel, it is 1. |
| } |  |  |
| } |  |  |
| } |  |  |

      - US-MAP IE

The US-MAP IE is shown in Table 9, and is used to define the upstream bandwidth allocations. The first US-MAP IE shall start at the lowest numbered sub-channel on the first symbol defined by the allocation start time field of the US-MAP message. These IEs shall represent the number of sub-channels provided for the allocation. Each allocation IE shall start immediately following the previous allocation and shall advance in the frequency domain within the number of sub-channeis allocated to this IE and then jump to the next symbol in the first sub-channel of the allocation and so on until the end of the US frame. In the case of the CDMA bursts, they can be concatenated in the time direction A Burst Descriptor shall be specified in the UCD for each UIUC to be used in the US-MAP.

- —US-MAP information elements

| Syntax | Size | Notes |
| --- | --- | --- |
| US-MAP_IE() { |  |  |
| CID | 12 bits | Table 3 |
| UIUC | 5 bits | 6.8.4.1.1 |
| If ((UIUC  0) && (UIUC  1)) { |  |  |
| Number of symbols | 5 bits | Number of symbols for the coexistence window at the end of the frame including the one symbol buffer before the CBP burst and the one symbol buffer following the CBP burst to absorb the propagation time. |
| } |  |  |
| } else if (UIUC 2) && (UIUC  3) ) { |  |  |
| Number of Subchannels | 8 bits | Align Figure 11 for allocation spanning the total width of the upstream sub-frame. |
| } |  |  |
| } else if (UIUC 4) && (UIUC  6) ) { |  |  |
| US-MAP CBP Channel IE | 16 bits | 6.8.4.1.2.3 |
| Number of Subchannels | 8 bits |  |
| Number of symbols | 7 bits |  |
| } |  |  |
| } else if (UIUC == 15) { |  |  |
| US_Extended_IE() | Variable | 6.8.4.1.2 |
| } else if (UIUC == 14) { |  |  |
| CDMA_Allocation_IE() | 32 bits | 6.8.4.1.2.2 |
| } else { |  |  |
| AMC_Subchannel | 1 bit | Indicates whether AMC subchannel is used for the burst of this CID 0= AMC subchannel is not used 1= AMC subchannel is used |
| If(AMC_Subchannel){ |  |  |
| If(frame==0){ |  | The same band should be used during the superframe. |
| SelectedBandBITMAP | variable | It represents the index of bands to be allocated for this CID among the all AMC bands selected for AMC subchannel. It consists of the bits with same value of Nband. The n-th bit is 1 if n-th band is allocated. |
| }else{ |  |  |
| Number of Subchannels | 8 bits | Align Figure 11 for allocations spanning the total width of the upstream sub-frame. |
| MDP | 1 bit | Measurement Data Preferred Used by the BS to indicate to the CPE that this upstream allocation is to be preferably used by the CPE for the specific purpose of reporting back any measurement data. The measurement data to be reported is in connection to the specified Transaction ID. In case the CPE does not have anything to report, it can use this allocation for any other data. This is useful, for example, after a quiet period. 0 = Measurement data not required (default) 1 = Measurement data preferred |
| MRT | 1 bit | Measurement Report Type In case MDP == 1, this field indicates which type of report the BS wants the CPE to send back. 0 = Detailed (see 6.8.22.3.1.1 thru 6.8.22.3.1.5) 1 = Consolidated (see 6.8.22.3.1.6) |
| CMRP | 1 bit | Channel Management Response Preferred Used by the BS to indicate to the CPE that this upstream allocation is to be used for confirming or not the receipt of the channel management command with the Transaction ID specified. 0 = Channel management response not required (default) 1 = Channel management response required |
| Transaction ID | 16 bits | The transaction ID of the corresponding channel management or measurement request for which a response/report is required by the BS. |
| } |  |  |
| If (!byte_boundary) |  |  |
| Padding Nibble | 0-7 bits |  |
| } |  |  |

        - UIUC Allocations

Table 8 specifies the UIUC incorporated into the MAC. In particular, the self-coexistence UIUCs (in both modes) have the same applicability to their DIUC counterpart (see 6.8.2.1.1).

- —UIUC values

| UIUC | Usage |
| --- | --- |
| 0 | Self-Coexistence (Active Mode) |
| 1 | Self-Coexistence (Passive Mode) |
| 2 | UCS Notification |
| 3 | BW Request |
| 4 | CDMA UCS Notification |
| 5 | CDMA BW Request |
| 6 | CDMA Ranging |
| 7-28 | Burst Profiles (Fixed: 4 rates*3 modulations + 1/4QPSK= 13) [Note: type of FEC codec indicated in DCD, one type per BS] |
| 29 | CDMA Allocation IE |
| 30 | Extended UIUC |
| 31 | End of Map |

        - US-MAP Extended UIUC IE

- —US-MAP extended IE general format

| Syntax | Size | Notes |
| --- | --- | --- |
| US_Extended_IE() { |  |  |
| Extended UIUC | 5 bits | Values specific to the Extended IE: 0-15. |
| Length | 4 bits | Length is the number of octets following this field – range = 0-15 octets |
| Unspecified Data | Variable |  |
| } |  |  |

          - US-MAP Power Control IE

- —US-MAP power control IE format

| Syntax | Size | Notes |
| --- | --- | --- |
| Power_Control_IE() { |  |  |
| Extended UIUC | 5 bits | Fast power control = 0x00 |
| Length | 4 bits | Length is the number of octets following this field |
| Power Control | 7 bits | Signed integer which expresses the change in power level (in 0.25 dB units) that the CPE should apply to correct its current transmission power |
| } |  |  |

- 6.8.4.1.2.2 CPE Maximum Transmit EIRP IE

The upper bound in maximum transmit EIRP for the CPE defined according to the algorithm described in 6.13.5 shall be indicated throught the IE described in Table 62a.

- Table 62a—US-MAP maximum EIRP IE format

| Syntax | Size | Notes |
| --- | --- | --- |
| Maximum_EIRP_Control_IE() { |  |  |
| Extended UIUC | 5 bits | Maximum EIRP control = 0x01 |
| Length | 4 bits | Length is the number of octets following this field |
| Maximum CPE Transmit EIRP | 7 bits | Integer which expresses the maximum EIRP level (in 0.5 dB units) from 0 dBm to +63.5 dBm that the CPE should apply to its current transmission power |
| } |  |  |

- The default value of maximum EIRP is 36dBm if not indicated.

- 6.8.4.1.2.3 US-MAP Dummy IE

- —US-MAP dummy IE format

| Syntax | Size | Notes |
| --- | --- | --- |
| Dummy_IE() { |  |  |
| Extended UIUC | 5 bits | 0x03 |
| Length | 4 bits | Length is the number of octets following this field – range = 0-15 octets |
| Unspecified Data | Variable |  |
| } |  |  |

          - 4S-MAP CBP Channel IE

- —US-MAP CBP Channel IE format

| Syntax | Size | Notes |
| --- | --- | --- |
| Dummy_IE() { |  |  |
| Extended UIUC | 5 bits | 0x04 |
| Length | 4 bits |  |
| Channel Number | 7 bits | Channel number in which the CPE shall listen to the medium for a coexistence beacon. In case the channel number is different from the operating channel, the Number of Slots filed in the US-MAP shall indicate the amount of time the CPE can stay in the foreign channel before returning to its operating channel. |
| } |  |  |

        - CDMA Allocation IE

- —CDMA allocation IE format

| Syntax | Size | Notes |
| --- | --- | --- |
| CDMA_Allocation_IE() { |  |  |
| Repetition Coding Indication | 2 bits | Indicates the repetition code used inside the allocated burst. 0b00 – No repetition coding 0b01 – Repetition coding of 2 used 0b10 – Repetition coding of 4 used 0b11 – Repetition coding of 6 used |
| Code | 1 bit | Indicates the Code sent by the CPE. |
| Symbol | 8 bits | Indicates the PHY symbol used by the CPE. |
| Channel | 7 bits | Identifies the channel used by the CPE to send the Code. |
| Usage | 1 bit | This field can mean two different things depending upon the type of Code used. If (Code = Incumbent) This field indicates whether the CPE shall transmit only the MAC header with the notification. 1 = yes; 0 = no |
| } |  |  |

_______________________________________