IEEE P802.22
Wireless RANs

| The Spectrum Sensing Function |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2007-03-27 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | Email |
| Steve Shellhammer | Qualcomm | 5775 Morehouse Drive San Diego, CA 92121 | (858) 658-1874 | Shellhammer@ieee.org |

Abstract

This text is to be included as a clause in the draft specifying the spectrum sensing function.  This specification gives the inputs and outputs of the function and specifies the behaviour of the spectrum sensing function, without specifying a specific implementation.

Notice: This document has been prepared to assist IEEE 802.22. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s).  The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein.

Release: The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication.  The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.22.

Patent Policy and Procedures: The contributor is familiar with the IEEE 802 Patent Policy and Procedures

<http://standards.ieee.org/guides/bylaws/sb-bylaws.pdf>, including the statement "IEEE standards may include the known use of patent(s), including patent applications, provided the IEEE receives assurance from the patent holder or applicant with respect to patents essential for compliance with both mandatory and optional portions of the standard."  Early disclosure to the Working Group of patent information that might be relevant to the standard is essential to reduce the possibility for delays in the development process and increase the likelihood that the draft publication will be approved for publication.  Please notify the Chair <Carl R. Stevenson> as early as possible, in written or electronic form, if patented technology (or technology under patent application) might be incorporated into a draft standard being developed within the IEEE 802.22 Working Group. If you have questions, contact the IEEE Patent Committee Administrator at <patcom@ieee.org>.

AUTHOR NOTE: This document is written as instructions to the editor to modify the current 802.22 draft (P802.22/D0.2).  It is the intention of the author to have this document reviewed by the spectrum sensing team and the working group. And once the document has been reviewed the WG technical editor is to modify the draft according to the instructions in this document.  The details of the procedure are to be set by the working group chair.

Instructions to the editor are written in bold italics.

3. Definitions

Insert the following new definitions in alphabetical order and renumber accordingly:

3.xx Available channel list (ACL): A list of which television channels are available for use by the WRAN.  The list is sorted in order of preference. The first channel in the list is the most preferred channel for use by the WRAN.  The last channel in the list is the least preferred channel.  Any channel not on the list shall not be used by the WRAN.

3.xx Channel selection function (CSF): The function that combines the outputs of the spectrum sensing function from the base station and the CPEs and generates the available channel list.

3.xx Confidence metric (CM): A metric indicating the confidence in a signal present decision.

3.xx Confidence vector (CV): A vector (one dimensional array) of confidence metrics.

3.xx Error standard deviation (ESD): The standard deviation of the error of a field strength estimate.

3.xx Error standard deviation vector (ESDV): A vector (one dimensional array) of error standard deviations.

3.xx Field strength estimate (FSE): An estimate of the field strength of a specific signal type at a WRAN station.

3.xx Field strength estimate vector (FSEV): A vector (one dimensional array) of field strength estimates.

3.xx Sensing mode (SM): A mode of operation the spectrum sensing function that specifies the outputs of the spectrum sensing function

3.xx Signal type vector (STV): A vector (one dimensional array) indicating which signal types the spectrum sensing function is to sense.

3.xx Spectrum sensing function (SSF):  The process of observing the RF spectrum of a television channel for a set of signal types and reporting the results of that observation.  The format of that report is specified by the sensing mode.

3.xx Sensing window specification (SWS): A specification of the time window over which the spectrum sensing function is to observe the television channel for a specific signal type.

3.xx Sensing window specification vector (SWSV): A vector (one dimensional array) of sensing window specifications for a specific signal vector.

3.xx Unavailable channel list (UCL): A list of which channels are unavailable for use by the WRAN

4. Abbreviations and Acronyms

Insert the following new abbreviations and acronyms in alphabetical order:

| ACL | Available Channel List |
| --- | --- |
| CM | Confidence Metric |
| CSF | Channel Selection Function |
| CV | Confidence Vector |
| ESD | Error Standard Deviation |
| ESDV | Error Standard Deviation Vector |
| FSE | Field Strength Estimate |
| FSEV | Field Strength Estimate Vector |
| MPFA | Maximum Probability of False Alarm |
| SM | Sensing Mode |
| SPD | Signal Present Decision |
| SPV | Signal Present Vector |
| SSF | Spectrum Sensing Function |
| STV | Signal Type Vector |
| SWS | Sensing Window Specification |
| SWSV | Sensing Window Specification Vector |
| UCL | Unavailable Channel List |

Insert this new clause between the current Clause 8 and Clause 9 and renumber subsequent clauses:

9. Spectrum Sensing

Spectrum sensing is the process of observing the RF spectrum of a television channel to determine if that channel is available for use by the WRAN. The process is divided into two parts: the spectrum sensing function and the channel selection function.

The base station and all CPEs shall implement the spectrum sensing function. The base station shall implement the channel selection function.

The spectrum sensing function observes the RF spectrum of a television channel and reports the results of that observation.  The spectrum sensing function is described in sub-clause 9.1.

The channel selection function utilizes reports from the spectrum sensing function within the base station and each of the CPEs to generate the available channel list.  The channel selection function is described in sub-clause 9.2.

The spectrum manager in the MAC utilizes the available channel list, as well as other information obtained from other sources (e.g. geolocation and database information), to select the WRAN channel of operation.

9.1 The Spectrum Sensing Function

The spectrum sensing function observes the RF spectrum of a television channel for a set of signal types and reports the results of this observation.  The spectrum sensing function is implemented in both the base station and the CPEs.  There are MAC management frames that allow the base station to control the operation of the spectrum sensing function within each of the CPEs.  Figure 1 illustrates the inputs and outputs of the spectrum sensing function.

The inputs to the spectrum sensing function come from the spectrum manager (SM) within the MAC. The inputs to the spectrum sensing function are described in Sub-clause 9.1.1. The outputs from the spectrum sensing function are sent to the channel selection function. The outputs of the spectrum sensing function are described in Sub-clause 9.1.2.  The behaviour of the spectrum sensing function is described in Sub-clause 9.1.3.

Possible sensing techniques that can be used to realize the spectrum sensing function are described in Annex XXX.  The use of any specific sensing technique is optional, as long as the inputs, outputs and behaviour meet the description in this sub-clause.

Figure 1: The Spectrum Sensing Function

9.1.1 SSF Inputs

A summary of the spectrum sensing inputs is given in Table 1.

| Signal Name | Signal Description |
| --- | --- |
| RF | Radio Frequency |
| Channel Frequency | The center frequency of the television channel which is to be sensed by the SSF |
| Channel Bandwidth | The bandwidth of the channel to be sensed by the SSF |
| Signal Type Vector | A vector indicating which signal types the SSF is to sense |
| Sensing Window Specification Vector | A vector of sensing window specifications. Each SFS specifies the details of the sensing window for a given signal type being sensed |
| Sensing Mode | The sensing mode specifies which SSF outputs are valid and in some cases it specifies the behaviour of the SSF. |
| Maximum Probability of False Alarm | In sensing modes 0 and 1 this value specifies the maximum false alarm rate for each sensing mode decision in the sensing present vector. |

Table 1: The Spectrum Sensing Function Input Signals

The RF input shall be connected to the output of a series of RF amplifiers which are themselves connected to the WRAN sensing antenna.

The channel frequency is the center frequency of the television channel that the SSF is to sense.

The channel bandwidth is the bandwidth of the television channel that the SSF is to sense.

The signal type vector (STV) indicates to which signal types are to be sensed for by the SSF.  The vector is a one-dimensional array of length STVLength, indexed from 0 to STVLength -1.  The STV is a binary vector whose elements are either zero or 1.  The i-th element in the vector specifies whether the SSF is to sense for i-th signal type.  The mapping of STV index to signal type is given in Table 2.

The value of STVLength is 8.

Author Note: I have rewritten Table 70 from the draft and placed it here. I believe it belongs here and not in the MAC section.  The MAC should reference this section.

| STV Index | Signal Type |
| --- | --- |
| 0 | Any Signal Type |
| 1 | IEEE 802.22 WRAN |
| 2 | IEEE 802.22.1 Sync Burst |
| 3 | IEEE 802.22.1 PPDU |
| 4 | ATSC |
| 5 | NTSC |
| 6 | Wireless Microphone |
| 7 | DVB-T |
| 8-255 | Reserved |

Table 2: Signal Type Vector Indices

A one in index zero of the STV indicates sensing for any signal type, with no distinction between signal types.  A one in index one of the STV indicates the SSF should sense for an 802.22 WRAN.

As an example, if the STV is given as follows,

Then the SSF shall sense for an 802.22.1 Sync Burst, an ATSC signal, and NTSC signal and a wireless microphone.

The sensing window specification vector (SWSV) is a one-dimensional array of length STVLength.  Each element in the SWSV is a sensing window specification (SWS).  If the i-th element of the STV is one (1) then the i-th element of the SWSV shall be set to a valid sensing window specification.  If the i-th element of the STV is set to zero (0) then the i-th element of the SWSV does not need to be set to a valid SWS since it will be ignored by the SSF.

A sensing window specification (SWS) consists of four parameters.  These four parameters specify the window of time over which the SSF shall sense for specified signal type.  Figure 2 illustrates a sensing window.

A sensing window begins at time SenseWindowStartTime. The sense window start time is measure from the end of the frame that contains the message with the SSF inputs.  The start window time is measured in units of frames. So if the sensing window is to start at the beginning of the frame following the frame carrying the SSF inputs, then SenseWindowStartTime is set to zero.  If SenseWindowStartTime is set to N then the sensing window starts at the beginning of the (N+1) frame following the frame that contains the SSF inputs.

A sensing window consists of NumIntervals number of intervals. The minimum number of sense intervals is one and the maximum number of intervals is TBD.

Each interval is of duration SenseIntervalDuration ms.

The time between sensing intervals is SenseSeparation ms.

The parameters in a sensing window specification are given in Table 3.

Figure 2: Sensing Window

Author Note: We need to decide value of the TBDs.

| Parameter Name | Units | Value |
| --- | --- | --- |
| SenseWindowStartTime | Frames | 0-TBD |
| NumIntervals | Integer | 1-TBD |
| SenseIntervalDuration | milliseconds | 1-TBD |
| SenseSeparation | milliseconds | 1 - TBD |

Table 3: Parameters of a Sensing Window Specification

The sensing mode specifies which outputs of the SSF are valid and in some cases the behaviour of the SSF.  Table 4 summarizes the valid SSF outputs for each of the sensing modes.  The table also includes a description of each sensing mode.

| Sensing Mode | Valid SSF Outputs | Description |
| --- | --- | --- |
| 0 | Signal Present Vector | For each signal type the SSF generates a binary decision as to whether the signal is present in the television channel |
| 1 | Signal Present Vector Confidence Vector | Same as sensing mode 0 with the addition of a confidence metric for binary decision |
| 2 | Field Strength Estimate Vector Error Standard Deviation Vector | For each signal type the SSF generates an estimate of the field strength of that signal and the standard deviation of that estimate |

Table 4: Summary of the Sensing Modes

Mode 0 is mandatory at the BS and the CPE.

Authors Note: The WG needs to decide if Mode 1 is mandatory or optional.

Mode 2 is optional at the BS and the CPE.

The maximum probability of false alarm input specifies the behaviour of the elements of the signal present vector when no signal is present on the RF input.  The details of the behaviour of the SSF, and its dependence on this input parameter, are given in Clause 9.1.3.

9.1.2 SSF Outputs

The sensing mode and the signal type vector are passed through the SSF.  These parameters indicate which of the other SSF outputs are valid and hence are useful for subsequent processing.

The signal present vector is a one dimensional array of length STVLength.  Each element in the vector is a signal present decision.  Each decision can take on three possible values, which are given in Table 5.  The i-th signal present decision corresponds to the i-th signal type as listed in Table 2.

| Value of Signal Present Decision | Description |
| --- | --- |
| TRUE | SSF decides that signal is present in TV channel |
| FALSE | SSF decides that the signal is not present in the TV channel |
| NODECISION | The SSF makes no decision regarding the presence of the signal in the TV channel. This is the output when the SSF was not instructed to sense for the signal type. |

Table 5: Values of the Signal Present Decision

The confidence vector is a one dimensional array of confidence metrics.  The i-th confidence metric indicated the confidence in the i-th signal present decision.  A confidence metric varies between a minimum of zero indicating no confidence in the signal present decision and one indicating total confidence in the signal present decision.  In practice since it is unlikely the SSF will have no confidence in the SPD or have total confidence in the SPD, the confidence metric is typically larger than zero and smaller than one.  The range of a confidence metric is given in Table 6.

| Limit | Value | Description |
| --- | --- | --- |
| Minimum confidence metric | 0 | No confidence in SPD |
| Maximum confidence metric | 1 | Total confidence in SPD |

Table 6: Range of a Confidence Metric

The field strength estimate vector is a one dimensional array of field strength estimates.  The i-th field strength estimate is an estimate of the i-th signal type measured at the location of the station (base station or CPE) in which the SSF resides.  The range of a field strength estimate is given in Table 7.

Author Note: We need to decide value of the TBDs.

| Limit | Value | Units | Description |
| --- | --- | --- | --- |
| FSMin | TBD | dBV/m | Minimum Field Strength Estimate |
| FSMax | TBD | dBV/m | Maximum Field Strength Estimate |

Table 7: Range of a Field Strength Estimate

The error standard deviation vector is a one dimensional array with each element in the vector is the standard deviation of the corresponding estimate in the field strength estimate vector.  The range of an error standard deviation of a field strength estimate is given in Table 8.

Author Note: We need to decide value of the TBD.

| Limit | Value | Units | Description |
| --- | --- | --- | --- |
| ESDMin | 0 | dB | Minimum Standard Deviation of the Field Strength Estimate Error |
| ESDMax | TBD | dB | Maximum Standard Deviation of the Field Strength Estimate Error |

Table 8: Range of the Standard Deviation of a Field Strength Estimate Error

9.1.3 SSF Behavior

In this clause all references to signal power refer to the signal power measured as the input to the sensing receiver.

9.1.3.1 Sensing Mode 0

When the sensing mode is set to zero the only valid SSF outputs are listed in Table 9.

Sensing Mode

Signal Type Vector

Signal Present Vector

Table 9: Valid outputs in Sensing Mode 0

The values of Sensing Mode and Signal Type Vector are the same as their input values.

In sensing mode zero, if the i-th element of the signal type vector is zero then the i-th element of the SPV shall be set to NODECISION.

| Sensing Mode | STV Index i | Signal Type | STV(i) | SPV(i) |
| --- | --- | --- | --- | --- |
| 0 | 0 | Any Signal Type | 0 | NODECISION |
| 0 | 1 | IEEE 802.22 WRAN | 0 | NODECISION |
| 0 | 2 | IEEE 802.22.1 Sync Frame | 0 | NODECISION |
| 0 | 3 | IEEE 802.22.1 PPDU | 0 | NODECISION |
| 0 | 4 | ATSC | 0 | NODECISION |
| 0 | 5 | NTSC | 0 | NODECISION |
| 0 | 6 | Wireless Microphone | 0 | NODECISION |
| 0 | 7 | DVB | 0 | NODECISION |

Table 10: Summary of SPV for Sensing Mode Zero with STV(i) set to 0

In sensing mode zero if the i-th element of the signal type vector is set to one and there is no signal present at the sensing antenna then the i-th element of the SPV shall be FALSE with probability greater than or equal to , where MPFA is the maximum probability of false alarm [KAY1] input to the SSF. Under these conditions the i-th element of the SPV is set to TRUE with probability less than or equal to MPFA.  The behavior of the SSF in sensing mode 0 with the i-th element of STV set to one and with no signal present at the sensing antenna is summarized in Table 11.

| Sensing Mode | STV Index i | Signal Type | STV(i) | SPV(i) | Probability |
| --- | --- | --- | --- | --- | --- |
| 0 | 0 | Any Signal Type | 1 | FALSE |  |
| 0 | 1 | IEEE 802.22 WRAN | 1 | FALSE |  |
| 0 | 2 | IEEE 802.22.1 Sync Frame | 1 | FALSE |  |
| 0 | 3 | IEEE 802.22.1 PPDU | 1 | FALSE |  |
| 0 | 4 | ATSC | 1 | FALSE |  |
| 0 | 5 | NTSC | 1 | FALSE |  |
| 0 | 6 | Wireless Microphone | 1 | FALSE |  |
| 0 | 7 | DVB | 1 | FALSE |  |

Table 11: Summary of SPV for Sensing Mode Zero with STV(i) set to 1 with no Signal Present

In sensing mode zero with the i-th element of the STV set to one and with the signal present in the channel at the specified signal power level given in Table 12, the i-th output of the SPV shall be TRUE with probability greater or equal to 0.9.

The behaviour of the SSF output SPV is specified in Table 12.

| Sensing Mode | STV Index i | Signal Type | STV(i) | Signal Power | SPV(i) | Probability |
| --- | --- | --- | --- | --- | --- | --- |
| 0 | 0 | Any Signal Type | 1 | TBD | TRUE |  |
| 0 | 1 | IEEE 802.22 WRAN | 1 | TBD | TRUE |  |
|  | 2 | IEEE 802.22.1 Sync Burst | 1 | TBD | TRUE |  |
| 0 | 3 | IEEE 802.22.1 PPDU | 1 | TBD | TRUE |  |
| 0 | 4 | ATSC | 1 | -116 dBm | TRUE |  |
| 0 | 5 | NTSC | 1 | -94 dBm | TRUE |  |
| 0 | 6 | Wireless Microphone | 1 | -107 dBm | TRUE |  |
| 0 | 7 | DVB-T | 1 | TBD | TRUE |  |

Table 12: Summary of SSF Outputs for Sensing Mode Zero

9.1.3.1 Sensing Mode 1

When the sensing mode is set to one the only valid SSF outputs are listed in Table 13.

Sensing Mode

Signal Type Vector

Signal Present Vector

Confidence Vector

Table 13: Valid outputs in Sensing Mode 1

The values of sensing mode, signal type vector and signal present vector are the same as in sensing mode 0.

If the i-th element of the signal type vector is set to zero then the i-th element of the confidence vector is set to 0.

If the i-th element of the signal type vector is set to one then the i-th element of the confidence vector is a confidence metric indicating the confidence the SSF has in the i-th element of the signal present vector.  A confidence metric is a measure of the confidence of a decision.  The range of a confidence metric is given in Table 6.

If the i-th element of the SPV is TRUE and the signal power of the i-th signal type is increased then the average value of the confidence metric should increase.

If the i-th element of the SPV is FALSE and the signal power of the i-th signal type is decreased then the average value of the confidence metric should increase.

9.1.3.1 Sensing Mode 2

When the sensing mode is set to two the only valid SSF outputs are listed in Table 14.

Sensing Mode

Signal Type Vector

Field Strength Estimate Vector

Error Standard Deviation Vector

Table 14: Valid outputs in Sensing Mode 2

The values of sensing mode and signal type vector are the same as their input values.

If the i-th element of the STV is set to zero then the i-th element of the FSEV shall be invalid since the i-th signal type is not sensed.

If the i-th element of the STV is set to one then the i-th element of the FSEV shall be an unbiased estimate [KAY2] of the field strength of the i-th signal type at the location of the antenna connected to the station in which the SSF resides.

If the i-th element of the STV is set to zero then the i-th element of the ESDV shall be invalid since the i-th signal type is not sensed.

If the i-th element of the STV is set to one then the i-th element of the ESDV shall be the average standard deviation of the i-th FSEV.  This average standard deviation is the square root of the average variance of the field strength estimate [KAY2].

9.2 The Channel Selection Function

The channel selection function shall reside in the WRAN base station.

9.2.1 CSF Inputs

The channel selection function receives the outputs of the SSF within the base station and the each of the SSFs within each of the CPEs.  The inputs to the CSF are the outputs of the SSF listed in Table 1.

9.2.1 CSF Outputs

The channel selection function generates two lists of channels: the available channel list (ACL) and the unavailable channel list (ACL).  These two lists are sent to the spectrum manager.

The relationship between the spectrum manager, the SSF, the CSF and the spectrum manager are show in Figure 3.

The available channel list is a shorted list of channels that are available for use by the WRAN.  The first channel in the list is most available, based on the SSF results.  The last channel in the list is the least available of the known available channels.

The unavailable channel list of the channels that are not available for use by the WRAN based on SSF results.

The availability of any channel not in either the ACL or the UCL is unknown to the CSF, based on SSF results.

9.2.1 CSF Behavior

Author Note:  The WG needs to decide what behavior to specify for the CSF?

The spectrum manager shall combine the information from the ACL, UCL and any information about the availability of the channels from other sources other then the SSF (e.g. geolocation information) and shall select the channel in which the WRAN shall operate.

Figure 3: The Channel Selection Function

Annex E. Bibliography

Insert the following new bibliographic references and match references in the text:

[KAY1] Steven M. Kay, Fundamentals of Statistical Signal Processing, Volume I: Estimation Theory, Prentice Hall, 1993

[KAY2] Steven M. Kay, Fundamentals of Statistical Signal Processing, Volume 2: Detection Theory, Prentice Hall, 1998