IEEE P802.22
Wireless RANs

| Initialization of the PPD |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2007-10-15 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Jianwei Zhang | Huawei Technologies | 450 Jinyu Road, Pudong, Shanghai, P. R. China, 201206 | 86-21-50993036 | zhangjianwei@huawei.com |
| Xuesheng Zhu | Huawei Technologies | 450 Jinyu Road, Pudong, Shanghai, P. R. China, 201206 | 86-21-50993036 | zhuxuesheng@huawei.com |
| Ke Wang | Huawei Technologies | 450 Jinyu Road, Pudong, Shanghai, P. R. China, 201206 | 86-21-50993034 | wincle@huawei.com |

Abstract

In the current IEEE 802.22.1/D2, the existence of the parameters of Start and Periodic in the primitive of MLME-START-BEACON.request is redundant. So they should be removed from the primitive parameter set.

Notice: This document has been prepared to assist IEEE 802.22. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s).  The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein.

Release: The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication.  The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.22.

Patent Policy and Procedures: The contributor is familiar with the IEEE 802 Patent Policy and Procedures

<http://standards.ieee.org/guides/bylaws/sb-bylaws.pdf>, including the statement "IEEE standards may include the known use of patent(s), including patent applications, provided the IEEE receives assurance from the patent holder or applicant with respect to patents essential for compliance with both mandatory and optional portions of the standard."  Early disclosure to the Working Group of patent information that might be relevant to the standard is essential to reduce the possibility for delays in the development process and increase the likelihood that the draft publication will be approved for publication.  Please notify the Chair <Carl R. Stevenson> as early as possible, in written or electronic form, if patented technology (or technology under patent application) might be incorporated into a draft standard being developed within the IEEE 802.22 Working Group. If you have questions, contact the IEEE Patent Committee Administrator at <patcom@ieee.org>.

- Problem Description:

According to current IEEE 802.22.1/D2 [1], the primitive of MLME-START-BEACON.request issued by the NHL to the MAC has two parameters of Start and Periodic.

With the development of the standard draft, these two parameters lose the necessity of existence.

- Suggested Remedy:

Remove the parameter of Periodic:

In Table 58 [1], there are such definitions:

- Periodic (PPD) 	The NHL should always set the value to TRUE for a PPD.

- Periodic (SPD) 	The NHL should always set the value to FALSE for an SPD.

That means the PPD and SPDs have the fixed transmission setting. So it is unnecessary to keep such a parameter.

Remove the parameter of Start:

The reason to remove Start is very simple. This parameter is redundant.

With the current understanding,

- The PPD always sends its beacon frame periodically. The PPD MAC just keeps the copy of the PPD beacon frame and resends it again and again. Sometimes an SPD may interrupt this transmission. So the periodical transmission is not strict periodically. When the PPD NHL wants send the updated beacon frame, it just issues one new MLME-START-BEACON.request to replace the beacon copy in the MAC.

- The SPD always sends its beacon frame only once.

Table 41—MLME-START-BEACON.request parameters

| Name | Type | Valid range | Description |
| --- | --- | --- | --- |
| … |  |  |  |
| Start | Boolean | TRUE or FALSE | If this parameter is TRUE, the device is to begin beacon frame transmission. Otherwise, the device is to stop beacon frame transmissions. |
| Periodic | Boolean | TRUE or FALSE | If this parameter is TRUE, the device transmits periodic beacon frames. Otherwise, the device transmits one beacon frame. This parameter is ignored if the Start parameter is FALSE. |

References:

- Part 22.1: Enhanced Protection for Low-Power, Licensed Devices Operating in Television Broadcast Bonds, P802.22.1/D2, October 2007.