# LDPC Coded Modulation Schemes Comparison

v1.1

Amir Mezer, Intel

July 2004

## Revision History

| Version | Date | Comments/Changes |
| --- | --- | --- |
| 1.0 (Assigned to initial draft) | 7/20/2004 | Preliminary draft |
| 1.1 | 7/21/2004 | Added item 4b. Dropped min. distance from table Added Es/No entry (6b., 11) Placed #iterations as a separate entry Added encoding complexity Added code construction details entry (13) |

## Contents

LDPC Coded Modulation Schemes Comparison	1

Revision History	1

Contents	1

Background	1

LDPC Coded Modulation Schemes- Comparison Table	2

Explanation Of The Parameters/Criteria	3

## Background

As we discussed during the last meeting in Portland, in order to choose the best LDPC coded modulation scheme, it is necessary to be able to perform a uniform comparison between the proposed schemes.

This document contains a suggestion for the criteria to be used for the comparison, and will be updated according to the feedback received.

First, a comparison table for comparing between the schemes is presented.

Then, explanations of the different parameters are given.

Finally, addition parameters for comparison are suggested, probably a bit later on in the future, after the first round of comparison is over.

## LDPC Coded Modulation Schemes- Comparison Table

| Parameter | Scheme #1 | Scheme #2… |
| --- | --- | --- |
| 1. PAM Dimension |  |  |
| 2. Block length, symbols |  |  |
| 3. # Information bits |  |  |
| 4a. Code rate (information bits/PAM symbol) |  |  |
| 4b. (# information bits)/(total # bits) |  |  |
| 5. Intrinsic Latency Tx+Rx [µs] |  |  |
| 6a. Eb/N0 [dB] required for BER=1e-12 over AWGN channel |  |  |
| 6b. Es/N0 [dB] required for BER=1e-12 over AWGN channel |  |  |
| 7. Number of iterations required |  |  |
| 8. Lowest BER Simulated, Confidence level (Number of errors) |  |  |
| 9. Expected Error Floor BER |  |  |
| 10a. Encoding complexity [operations/information bit] |  |  |
| 10b. Decoding complexity [operations/information bit] |  |  |
| 11. BER vs. Eb/N0 data available & BER vs. Es/N0 data available |  |  |
| 12. Further comparison |  |  |
| 13. Code construction details as an attachment |  |  |
| 14. Advantages/comments |  |  |

## Explanation Of The Parameters/Criteria

1. PAM Dimension- straightforward

2. Block length in terms of number of PAM symbols

3. Total # of information bits in each block of <Block_length> PAM symbols

4a. Code rate- #3 above divided by #2

4b. #3 divided by the total number of bits

5. The intrinsic latency of the coding scheme, including Tx and Rx

6a. The Eb/N0 required for BER=1e-12 over an AWGN channel. This will enable comparison of how strong the coding scheme is at the required operating point.

6b. The Es/N0 required for BER=1e-12 over an AWGN channel. After the decision on PAM12/PAM8 and the requires code rate, it would make sense to drop 6a. and remain with this entry only.

7. The number of iterations required to achieve #6, will be used to estimate decoding complexity (see #10).

8. Lowest BER simulated and number of errors will give an idea of how high the confidence level of the performance data is.

9. BER of expected error floor.

10a. Encoding complexity estimation- will be added.

10b. Decoding complexity will be given in operations per information bit.

Refer to:

“Reduced complexity iterative decoding of low-density parity check codes based on belief propagation”
Fossorier, M.P.C.; Mihaljevic, M.; Imai, H.;
Communications, IEEE Transactions on , Volume: 47 , Issue: 5 , May 1999 
Pages:673 – 680

Assuming that no-one is going to use standard belief propagation decoding, but rather something more similar to UMP-BP decoding, an estimation of the decoding complexity is presented on pages 678 and 679.

To summarize, the estimation of decoding complexity is:

| XORs (required for UMP implementation, derived internally) | Forward and Backward Recursions | Min. values computation and auxiliary comparisons | Iterations | Operations/ Information Bit |
| --- | --- | --- | --- | --- |
| X= 3*I*R*N+(J+1)*N | Y= 3*N*(J-1) additions | Z= R*N*(I+log2(I)-2)+2*J*N | T | (X+Y+Z)*T/ (# information bits per block) |

Where

N is the LDPC codeword length

R is the code rate

I is the number of ones in each row

J is the number of ones in each column

11. BER vs. Eb/N0 data available- to be able to draw graphs for comparing between the schemes. In addition BER vs. Es/N0 (see comments in 6b)

12. Suggested further comparison criteria (as a future step, after the first round of comparison is over)

-Performance over Cable models 1,2,3 with SNR vs. BER, taking into account the code rate

-Performance with ARQ using higher layers. We are working on a proposal which we plan to send out soon

-Any other ideas are welcome

13. A detailed description of the construction of the proposed scheme. This will be placed as an attachment in the table.

The description should hold enough detail to enable implementation of the scheme.

14. Any advantages/comments of the proposed scheme