Add to the end of table 45.5.3.3 PMA/PMD management functions:

| MM119 | Writes to this register have no effect | 45.2.1.11 |  | PMA:M | Yes [ ] No [ ] N/A [ ] |
| --- | --- | --- | --- | --- | --- |

Add to the end of table 45.5.3.7 PCS management functions

| RM79 | FEC corrected blocks counter is reset when read or upon PHY reset. | 45.2.3.32 |  | FEC-PR:M | Yes [ ] No [ ] N/A [ ] |
| --- | --- | --- | --- | --- | --- |
| RM80 | FEC corrected blocks counter is held at all ones in the case of overflow | 45.2.3.32 |  | FEC-PR:M | Yes [ ] No [ ] N/A [ ] |
| RM81 | FEC uncorrected blocks counter is reset when read or upon PHY reset. | 45.2.3.33 |  | FEC-PR:M | Yes [ ] No [ ] N/A [ ] |
| RM82 | FEC uncorrected blocks counter is held at all ones in the case of overflow | 45.2.3.33 |  | FEC-PR:M | Yes [ ] No [ ] N/A [ ] |