Changes to the OLT synchronizer to resolve problems with missing gearbox and the fact that bit errors could impair the append_inbuffer action.

Rename “BlockFromGearbox” function to “BlockFromPMA” at every occurrence.

Redefine BlockFromGearbox as follows:

BlockFromPMA

Function that accepts the received data from the PMA.  Conceptually, this function serializes the 16 bit rx_data_group<0..15> to a bit stream at 10.3125Gb/s, and then deserializes the resulting bit stream into a 66 bit wide rx_coded<0..65> block of data.  It does not return until 66 bits have been transferred.  Note that the internal design by which this function is accomplished is an implementation choice; however, the design must operate such that a new 66 bit block is made available at the regular interval of 6.4ns, and the transfers are made synchronous to the XGMII clock.

Redefine SLIP_One_Bit as follows:

SLIP_One_Bit

Causes the next candidate block sync position to be tested. The next candidate must be exactly one bit later than the previous candidate – no burst alignments may be skipped.  Following the conceptual model mentioned in “BlockFromPMA,” this function transfers one more bit from the 16 bit serializer to the 66 bit deserializer.

Modify the Append_inbuffer as follows:

Append_inbuffer()

{

BlockFromPMA()

if(sh_cnt<27){

inbuffer[input_buffer_location]=rx_coded<1>

input_buffer_location++

}

for(i=2, i<66, i++) {

inbuffer[input_buffer_location]=rx_coded<i>

input_buffer_location++

}

}

Augment the description of BlockToDescrambler to read:

BlockToDescrambler

Function that sends the next rx_coded_corrected<0..65> block to the descrambler. It does not return until the transfer is completed, and each transfer shall take 6.4ns and be synchronized to the XGMII clock.