Minutes of Task 4 group 9/17/2004

Chaired by Yu Sun   Optium

Attendees

Brent Whitlock       		Rsoft

Gary Shaulov          		Rsoft

John George            		OFS

John Ewen             		 JDS

David Cunningham             	Agilent

Ali Ghiasi 		            Broadcom

Jim Morris			DOC

Actions

- Go over the minutes of last call

The minutes of last call was accepted.

- Presentation 1: Comparison of some Cambridge model results with simulation results of RSoft and Optium in-house simulators, presented by Gary Shaulov

Outline of the talk:

- The purpose of these preliminary studies was to check the results of Cambridge model for scaled/non-scaled index profiles against results by other mode solvers (both commercial and proprietary);

- Just two fiber models were studies – fibers #23 and #29 – before and after scaling;

- Optium in-house simulator results were compared with RSoft commercial multimode simulator (ModeSYS™ ) results

- Optium and RSoft simulation results were compared with results from Cambridge model for output pulse shapes, DMD evolution, scaling factors, and OFL bandwidth.

Preliminary conclusions:

- The agreement between RSoft and Optium in-house simulators is excellent.

- Both RSoft and Optium results are in good agreement with Cambridge results for fiber #29, however for fiber #23 there is discrepancies between Cambridge and RSoft/Optium

- The agreement between RSoft and Cambridge models results in DMD evolution behavior is good except points at very high values of offsets

- More studies needed to identify if the difference comes from difference in approaches to calculate mean DMD

- Cambridge model prescribes to ignore degenerate mode groups with g>20 (or g>18?). It may be not always the case for RSoft and Optium mode solvers – can that be the cause of discrepancies, and should we worry about higher order modes?

Discussions:

  - David Cunningham suggested since Cambridge using 18 modal groups, a fair comparison should be done under the same situation. He suggested to compare

    - modal group delay

    - mode part distribution

    - what mode has been included in the calculation

David also suggested whether 18 modal groups are enough should be confirmed from fiber manufactures.

  - Ali Ghiasi expressed concerns of power deduction due to eliminating higher order modes.

  - John George offered to take action on the confirmation of using 18 modal groups

- Presentation 2: Preliminary results of PIE comparison: pulse width and distance, presented by Yu Sun

Outline

      - PIE metrics of different input pulse are compared.

      - PIE metrics of different distance are compared.

Preliminary conclusions

- PIE metrics of offset launch degrade as the distance increase.

- PIE metrics for center launch is not sensitive to the change of fiber length.

- PIE_L and PIE_D metrics metrics for center launch case meet the 4.5 dB specified penalty at 220 m and 300m

- The “umbrella” slide for Ad Hoc group

The draft was accepted.

Next Meeting

4th Oct, 2004 (Monday), 11am east coast, 8 am west coast and 4 pm UK