# Table 45-3 Control register 1

| Bit name | Applicable to PMD | Applicable to PMA |
| --- | --- | --- |
| Reset | X | X |
| Loopback * | X | X |
| Speed select (LSB) |  | X |
| Power down | X | X |
| Speed select (MSB) |  | X |

*  Figure 45-2 shops that loopback needs to be enabled either for the PMA

or the PMD, but not for both at the same time.

# Table 45-4 Status register 1

| Bit name | Applicable to PMD | Applicable to PMA |
| --- | --- | --- |
| Local fault | X | X |
| Received link status |  | X |

Table 45-5 Control register 2

The values 001 – 111 are all applicable to the PMA sublayer.

Only E, L, S, and LX-4 are needed for the PMD sublayer

Table 45-5 Status register 2

| Bit name | Applicable to PMD | Applicable to PMA | Notes |
| --- | --- | --- | --- |
| Device present | X | X | Only one bit is needed |
| Xmit local fault ability | X | X |  |
| Rcv local fault ability | X | X |  |
| Xmit local fault | X | X |  |
| Rcv local fault | X | X |  |
| Loopback ability | X | X |  |
| PMD xmit disable ability * | X |  |  |
| 10GBASE-SR ability | X | X | Only S is needed for the PMD |
| 10GBASE-LR ability | X | X | Only L is needed for the PMD |
| 10GBASE-ER ability | X | X | Only E is needed for the PMD |
| 10GBASE-LX-4 ability | X | X |  |
| 10GBASE-SW ability |  | X | See SR ability note |
| 10GBASE-LW ability |  | X | See LR ability note |
| 10GBASE-EW ability |  | X | See ER ability note |

*  PMD transmit disable ability should be two variables – All_lanes_transmit_disable_ability and

Lane_by_lane_transmit_disable_ability”

Table 45-7 10G PMD transmit disable

This is a PMD-only table. “PMD transmit disable 0” has different meanings for serial and

WWDM PMDs. Since transmit disable for the WWDM PMD could be either all lanes or

Lane-by-lane, I suggest that we add an “All lanes transmit disable” control variable.

Table 45-8 10G PMD receive signal detect

This is also a PMD-only table. It is OK as is.