Background:

The objective and the process below were developed by the

HSSG Cabling Survey Ad Hoc.

Objective:

To define requirements for information gathering about installed cabling and cabling trends in order to inform potential future HSSG decisions regards media and link distances.

Process:

Develop a survey instrument targeting network managers and BICSI members that possess detailed measured data about installed cabling, specifically: sites with OTDR certification data bases.

Target with discrimination the sample audience with a very short questions to obtain granular data.

Survey:

# Purpose

- The purpose of the survey is to characterize the lengths and type of installed optical fiber used in commercial building telecommunication designs.

- Survey definitions are based on TIA/ EIA- 568- A Commercial

Building Telecommunications Cable Standard and TIA/EIA-569-Commercial Building Standard for Telecommunications Pathways

and Spaces.

- The survey results will allow accurate characterization of the installed intrabuilding fiber backbone. In order to ensure the accuracy of the lengths reported,  the installed cabling survey data shall be taken from as-built documents where the lengths are recorded from Optical Time Domain Reflectometry (OTDR) measurements.

Survey:

## Installed Optical Fiber Cabling Distances

## Scope:

## Survey: Installed Optical Fiber Cabling Distances

# Characterization of the lengths and types of installed optical fiber

cable used in commercial building telecommunication designs is of           critical importance to the telecommunication industry. Your survey response will be applied to the development of Optical Fiber Standards. Please take the time to fill in the following questionnaire. Your participation is appreciated.

## Preface

Respondent Information - Optical Fiber Cable Distance Survey

-----------------------------------------------------------------------------------------

Table 1:  Company and Personal Profile - Please Print

-----------------------------------------------------------------------------------------

Your Name                        _____________________________

Your Company Name                _____________________________

City/State/Zip                   _____________________________

E-MAIL                           _____________________________

Your Job Function ______________________________________

### 1  Purpose

The purpose of the survey is to characterize the lengths and type of

installed optical fiber cable used in commercial building telecommu-

nication designs. Cable backbone distances have traditionally been

associated with application specific distance limitations e.g.,

10Base5 (500m).

2. Survey Data Reporting

The Installed cabling survey data shall be taken from the as-built test documents where the cable lengths have been recorded from

the field tests measurements (OTDR for fiber) or where the lengths had been recorded from the cable length marks.

3. Survey Data Format

The survey data should be submitted in a spread sheet compatible format.

# 4. Survey Questionnaire –

4.1. Campus Environment:

Number of Buildings

Number of Floors:

4.2. Building Environment:

4.2.1. Interbuilding Backbone cable

For each building recorded please respond by providing the information requested  in Table 1. for each interbuilding fiber installed.

Interbuilding Backbone cable - is the cable distance between building equipment rooms as illustrated in Figure 1. Interbuilding Backbone cable does not include patch cord, jumper or equipment cable.

For Hybrid Interbuilding Backbone Cable- a backbone cable with different fiber types covered by one overall sheath- report number of strands for  each fiber type.

Table 1.  Interbuilding Backbone cable

| Building –to-Building Identifier: | Backbone Cable Identifier: | Fiber Type #### Number of Strands Multimode Single 62/125 50/125 Mode |  |  | Measured Length | Number of Terminated Strands |
| --- | --- | --- | --- | --- | --- | --- |

4.2.2. Intrabuilding Backbone cable

For each building recorded please respond by providing the information requested  in Table 2. for each intrabuilding fiber installed.

Intrabuilding Backbone cable - Backbone cable within buildings.

Intrabuilding Backbone cable distance is the cable distance

between telecommunication closets and the equipment room as

illustrated in Figure 1. Intrabuilding Backbone cable does not

include patch cord, jumper, or equipment cable.

For Hybrid Intrabuilding Backbone Cable- a backbone cable with different fiber types covered by one overall sheath- report number of strands for  each fiber type.

Table 2.  Intrabuilding Backbone cable

| Building Identifier: | Floor Identifier: | Backbone Cable Identifier: | Fiber Type #### Number of Strands Multimode Single 62/125 50/125 Mode |  |  | Measured Length | Number of Terminated Strands |
| --- | --- | --- | --- | --- | --- | --- | --- |

4.2.3. Horizontal cable

For each building floor recorded please respond with the information requested in Table 3. for each horizontal fiber installed.

Horizontal cable - the cable between the telecommunication outlet and the telecommunications closet. Horizontal cable does not

include patch cord, jumper, or equipment cable.

Table 3.  Horizontal Fiber cable

| Building Identifier: | Floor Identifier: | HorizontalCable Identifier: | Fiber Type Number Of Strands Multimode Single 62/125 50/125 Mode |  |  | Measured Length | Number of Terminated strands |
| --- | --- | --- | --- | --- | --- | --- | --- |

For each building floor recorded please respond by providing the information requested in Table 4. for each horizontal copper cable installed.

Table 4.  Horizontal copper cable

| Building Identifier: | Floor Identifier: | HorizontalCable Identifier: | Measured Length |
| --- | --- | --- | --- |

4.2.5. Centralized Optical Fiber Cabling (TIA/EIA-TSB-72)  - Fiber optic cabling between the Equipment Room and the telecommunication outlet with the use of either pull through cables , an interconnect, or a splice in the telecommunications closet.

For each building floor recorded please respond by providing the information requested in Table 5. for each fiber optic cable between the Equipment Room and the telecommunication outlet. This cabling does not include a crossconnect, patch cord, jumper, or equipment cable.

Table 5.  Fiber Optic Cable from Equipment Room to Telecommunication outlet.

| Building Identifier: | Floor Identifier: | Backbone Cable Identifier: | Fiber Type #### Number of Strands Multimode Single 62/125 50/125 Mode |  |  | Measured Length | Number of Terminated Strands |
| --- | --- | --- | --- | --- | --- | --- | --- |

Figure 1.

TP

TO

TO

Horizontal Cable

TC

HC

TO

TC

HC

TC

ER

TC

Interbuilding Backbone

HC

ER

Intrabuilding Backbone

Cable Reference Model key

ER   = Equipment Room

TC   = Telecommunication Closet

TO    = Telecommunication Outlet

- New Cabling

5.1. Can you rank the difficulty in adding additional cable (1 -3 )      where 1 is the most difficult ?

[] Interbuilding building backbone

[] Intrabuiding building backbone

[] horizontal cable

  - Would you install higher performance multimode fiber optic cable for 1 Gigabit Ethernet if it supported your cable distances (y/n)?

[] Interbuilding building backbone

[] Intrabuiding building backbone

[] horizontal cable

  - Would you install higher performance multimode fiber optic cable for 10 Gigabit Ethernet if it supported your cable distances (y/n)?

[] Interbuilding building backbone

[] Intrabuiding building backbone

[] horizontal cable