Category 6 Draft 5 Cabling Specification Summary:

Many Ad Hoc members requested Category 6 specifications.

The current available draft is difficult to read. For ease of use I’ve extracted the following from the current Draft (5).

#### 1. Cable Attenuation

Attenuation(f) <= 1.82*SQRT(f) + 0.017*f + 0.2/SQRT(f)	(1)

Alternate proposal “A” :

Attenuation(f) <= 1.63*SQRT(f) +   0.017*f  +  0.2/SQRT(f)

For all frequencies from 1 MHz to 250 MHz

#### 2. Connecting hardware attenuation

Attenuation <= 0.02 * SQRT(f)

(2)

For all frequencies from 1 MHz to 250 MHz

### 3. Cabling attenuation

(3)

where:

(4)

For all frequencies from 1 MHz to 250 MHz

Note:  A 10% increase in attenuation is allowed over Cat 6 horizontal cable attenuation for work area and patch cords as shown in equation 4.

My Note: The patch attenuation should be 20% based on committee agreement.

Note: The alternate proposal “A” would keep the same attenuation value (i.e., 22.2 dB @ 100 MHz) for flexible cable as the original proposal.

#### 4. Cable pair-to-pair NEXT loss

NEXT(f) >= 76 - 15log(f/0.772)	(7)

For all frequencies from 0.772 MHz to 250 MHz

Note: Alternate “A” is 6 dB tighter pair-to-pair and power sum NEXT values than the original formulas.

#### 5. Connecting hardware pair-to-pair NEXT loss

NEXT(f) >= 54 - 20log(f/100)	(8)

For all frequencies from 1 MHz to 250 MHz

Note: Alternate “A” is 6 dB tighter pair-to-pair and power sum NEXT values than the original formulas.

### 6. Cabling pair-to-pair NEXT loss

(9)

For all frequencies from 1 MHz to 250 MHz

#### 7. Cable power sum NEXT loss

For all frequencies from 0.772 MHz to 250 MHz, cable power sum NEXT shall meet the values determined using equation (12). The values in Table 9 are provided for information only.

PSNEXT(f) >= 74 - 15log(f/0.772) 							(12)

#### For all frequencies from 0.772 MHz to 250 MHz

#### 8. Connecting hardware power sum NEXT loss

PSNEXT(f) >= 50 - 20log(f/100)	(13)

For all frequencies from 1 MHz to 250 MHz

Editor:  Expecting comments / contributions from connector task group.

#### 9. Cabling power sum NEXT loss (worst pair)

(14)

For all frequencies from 1 MHz to 250 MHz

#### 10. Pair-to-pair cable ELFEXT

ELFEXT(f) >= 70 - 20log(f/0.772)	(16)

For all frequencies from 1 MHz to 250 MHz

Alternate “A”  6 dB improvement

### 11. Pair-to-pair connecting hardware FEXT loss

FEXT(f) >= 43.1 - 20*log(f/100)					      (17)

For all frequencies from 1 MHz to 250 MHz

#### 12. Pair-to-pair cabling ELFEXT

ELFEXTchannel,pairtopair≥−20log(10−ELFEXTcable,pairtopair20+4∗10−FEXTconnector,pairtopair20)    (18)

For all frequencies from 1 MHz to 250 MHz

#### 13. Cable power sum ELFEXT

PSELFEXT cable,(f) = 67 - 20LOG(f/0.772) dB/100m (dB/328 ft)				     (20)

For all frequencies between 1 MHz and 250 MHz.

### 14. Connecting hardware power sum FEXT Loss

PSFEXT(f) >= 34.1 - 20log(f/200)	      (21)

For all frequencies between 1 MHz and 250 MHz.

#### 15. Cabling power sum ELFEXT

(22)

For all frequencies from 1 MHz to 250 MHz

### 16. Horizontal Cable return loss

Table 1 - Category 6 horizontal cable return loss @ 20C  3C (68 F  5.5 F)

For a length of 100 meters

| Frequency (MHz) | Return Loss (dB) |  |
| --- | --- | --- |
| 1  f < 10 10  f < 20 20  f  250 | 20 + 5*log(f) (TBD) 25 (TBD) 25 – 7*log(f/20) (TBD) | (24) |

Table 2 - Category 6 stranded cable return loss @ 20 °C +/-3 °C (68 F  5.5 F)

For a length of 100 m (328 ft)

| Frequency (MHz) | Return Loss (dB) |  |
| --- | --- | --- |
| 1  f < 10 10  f < 20 20  f  100 | 20+5log(f) (TBD) 25 (TBD) 25 – 8.6log(f /20) (TBD) | (25) |

17. Connecting Hardware  return loss

Table 3 - Category 6 connecting hardware return loss

| Frequency (MHz) | Return Loss (dB) |  |
| --- | --- | --- |
| 1  f < 28 28  f  250 | 35 dB 24 – 20log(f/100) dB | (26) |

###### 18. Modular Patch Cord return loss

Table 4 - Category 6 modular patch cord return loss

| Frequency (MHz) | Return Loss (dB) |  |
| --- | --- | --- |
| 1  f < 20 20  f  100 | 25 (TBD) 25 – 10log(f/20) (TBD) | (27) |

### 19. Cabling Return Loss

Table 5 – Category 6 channel return loss

Length of horizontal cabling: 90 meters

Length of equipment cords, patch cords and jumpers: 10 meters

| Frequency (MHz) | Return Loss (dB) |  |
| --- | --- | --- |
| 1  f < 20 20  f  250 | 19 19-10log(f/20) | (28) |

## 20.	Propagation delay/delay skew

The propagation delay and delay skew assume that the cable is the predominant contributor to the delay and delay skew of a channel. See TIA/EIA-568-A-1 for additional information.

# 21.	Propagation delay

# The propagation delay of any pair measured in accordance with TIA/EIA-568-A-1 and ASTM D4566 shall meet equation (30) from 1 to 250 MHz.

# delay(ns100m)≤534+36fMHz									(30)

Editor’s Note:  re-format section to include propagation delay of connecting hardware.

22.	Propagation delay skew

- The difference in propagation delay between the fastest and slowest pair in a cable shall not exceed 45 ns/100 m between 1 MHz and 250 MHz and the skew between all pair combinations shall not vary more than  10 ns when measured in accordance with TIA/EIA-568-A-1 and ASTM D 4566.