DTE Power via MDI task Force.

## AC disconnect detection ad hoc

Yair Darshan

AC disconnect detection group members: stds-802-3-pwrviamdi-detection@majordomo.ieee.org

Yair Darshan: 	yaird@powerdsine.com

Chris Cullin: 	ccullin@cisco.com

Roger Karam: 	rkaram@cisco.com

Dave Dwelley: 	ddwelley@linear.com

Steve Hema: 	steven_hemmah@ti.com

Dan Dove	: 	dan_dove@hp.com

Thong Huynd	:	Thong_Huynh@maximhq.com

Scott Burton:	scott_burton@mitel.com

Chris Di Minico:	cd@mohawk-cdt.com

John Jetzt	:	jetzt@avaya.com

Rich	graham:     'grahamri@enterasys.com'

Fred Schindler:	frs@cisco.com

# 1.	Scope of work

To demonstrate technical and economical feasibility of PSE based ac disconnect detection and determined if there is impact on previous functions in the draft.

# 2.	Tests List/ A.I

| Item | Test Description | Owner | Status | Ends by |
| --- | --- | --- | --- | --- |
| 1 | Test Setup Circuits and general requirements |  |  |  |
| 1.1 | Send over the reflector to ad-hoc members the standard circuit for all testing | Yair, Dave, Thong | Done |  |
| 1.2 | To prepare a specifications for ac disconnect to be used later in the spec or in test setup definitions | Yair | Initial version | April 30, 2002 |
| 1.3 | Tests will done in order to confirm Table 12 by checking min/max parameters | All |  | April 30, 2002 |
| 2 | Sensitivity Analysis, Setting Parameters Range |  |  | April 30, 2002 |
| 2.1 | To define the pulse spectrum of the ac signal (Resources: Simulations, single port) | Yair |  | April 30, 2002 |
| 2.2 | Effect on feedback loop of SMPS (Resources: 1 port , Simulations) | Yair |  | April 30, 2002 |
| 2.3 | To analyze the PD input ac impedance without DC current (Resources: Simulations, confirm with 2-3 phones, single port) Zac max over a freq band and the dc bias it is tested at, we need this with 37v-57v spec This should take into account the min/max L, C per table 12. | Yair, Dave,Fred |  | April 30, 2002 |
| 2.4 | To set the optimum probing frequency range (narrow the range) To perform sensitivity Analysis for the probing frequency and define the tolerance required (Resources: Simulations, confirm with 2-3 phones, single port ) | Yair, Dave,Fred |  | April 30, 2002 |
| 3 | Audio Interference tests |  |  |  |
| 3.1 | Audio interference with existing phones, LCD | Roger, Scott |  | April 30, 2002 |
| 3.2 | –Immunity tests - check specifically at line frequency (Resources: Phone and/or high power PD, Single port PSE) | Yair, Roger |  | April 30, 2002 |
| 3.3 | Sensitivity to line frequency and ringing interference with CAT 3 cable (Use florescent turn on/turn off to test sensitivity) (Resources: Phones, Ring generators, 1 ports PSE min) | Yair |  | April 30, 2002 |
| 3.4 | How port-to-port cross regulation, cross talk affected by the ac circuit. (Resources: Lab tests, 3 port min look at the centered one) | Yair |  | April 30, 2002 |
| 4 | ### RF interference |  |  |  |
| 4.1 | RF interference (Resources: Phone, Single port PSE) | Roger, Scott |  | April 30, 2002 |
| 4.2 | –EFT test (Resources: 2 port tests) | Roger |  | April 30, 2002 |
| 4.3 | EN55024 Need to specify the relevant tests (Resources: 2 port tests) | Scott |  | April 30, 2002 |
| 4.4 | EN55022/CISPR22/FCC NB30/MPT 1570 Need to specify the relevant tests (Resources: 4 port tests) | Roger, Yair |  | April 30, 2002 |
| 5 | Effects on other 802.3af functions |  |  |  |
| 5.1 | Test how over-current affected by the ac disconnect pulses (Resources: Simulations, Lab tests, single port) | Yair, Dave |  | April 30, 2002 |
| 5.2 | Effects of PD load transient. (Resources: Lab test, single port) | Yair |  | April 30, 2002 |
| 5.3 | Test for PSE output voltage step Resources: Simulations, single port ) | Dave, Yair, Thong |  | April 30, 2002 |
| 5.4 | To supply PD DC/DC Vout/Vin Ripple rejection ration as function of frequency | Yair |  | April 30, 2002 |
| 6 | Other tests |  |  |  |
| 6.1 | Evaluate the impact of two Midspan PSE’s with ac disconnect scheme. | Dan Dove |  | April 30, 2002 |
| 6.2 | Roger tasks for 1000BT (Roger to pass to Broadcom?) | Roger |  | April 30, 2002 |

# Item 1.1- Test Setup Circuits and general requirements

Circuit # 1.

Description: The hardware to be used to inject the ac disconnect probing signal across PSE port. The circuit does not include the sensing hardware, which is implementation specific.

Circuit # 2.

Description: The hardware to be used to emulate the PD during normal powering mode. (Normal powering mode is the mode, which the PD is receiving power after successful detection and classification phase.

The circuit contains a switch at its input to be used for PD disconnection emulations.

# Item 1.2 - AC Disconnect black-box specifications

| PSE side |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Item | Parameter | ##### Sym | Unit | Min | Max | Notes |
|  | Port disconnect probing ac voltage when PD is disconnected | ##### V_open | Vpp | 2.5 | 4.4 | 1 |
|  | Port dc voltage when PD is disconnected | ##### V_open | Vdc | 44 | 60 | 1 |
|  | Port ac voltage when PD is Connected |  |  |  | 0.5 | 2 |
|  | AC probing signal output impedance From 50Hz to 100Khz | ##### Zo_ps | K | 5 |  | 3 |
|  | AC probing signal output impedance for DC signal | ##### Zo_ps1 | K | 70 |  | 4 |
|  | AC probing signal frequency | ##### Fp | Hz | 50 | 150 | 5 |
|  | AC probing signal duty cycle | ##### Duty | % | 40 | 60 | 5 |
|  | AC probing signal rise / fall time | ##### tr | uS | 10 |  | 7 |
|  | Disconnect detection threshold min. | ##### Vth | V | 0.7V_open | 0.8V_open | 5 |
|  | Disconnect detection time | ##### Tpmdo1 | ms | 300 | 400 | 6 |
|  | Min number of ac pulses above Vth before disconnect | ##### N1 |  | 2 |  | 5 |
|  | Spec pulse width, rise time, fall time, spectrum and tolerances |  |  |  |  |  |
|  | ###### Functional requirements |  |  |  |  |  |
|  | AC probing signal should be off during detection and classification. |  |  |  |  |  |
|  | AC probing signal should be off after disconnect detection is done. |  |  |  |  |  |
| PD side |  |  |  |  |  |  |
|  | AC impedance for “must remain powered” | ##### Z_pd1 | K |  | 5 | 5,8 |
|  | AC impedance for “may or may not disconnect” | ##### Z_pd2 | K | 5 | TBD | 5,8 |
|  | AC impedance for “must disconnect” | ##### Z_pd3 | K | TBD |  | 5,8 |
|  | PD power supply output to input ripple rejection ration from 10Hz to 500Hz | ##### PSRRR | dB | TBD |  | 5 |

| #### Notes |  |
| --- | --- |
| 1 | Safety standards defined a DC voltage as a voltage that has 10% max ripple content. To meet 60V max for SELV requirements the ac contents must not exceed 10% therefore 10% of 44Vdc is 4.4Vpp max. The 3V min was derived from the following reasons: - If PSE and PD generate low frequency noise OF 0.5Vpp each, in phase that the total could be 1Vpp. - The PD specified with 2V offset (2 diodes in series at low temperature). In order to forward bias them without the PSE DC bias, we need 2V min. The worst case from the above is 2V min. Having 0.5Vpp min margin for the detection action, adds up to 2.5Vpp |
| 2 | 0.5Vpp as specified in table 5, draft 3. |
| 3 | To set a minimum impedance in order to limit the ac current at short circuit condition |
| 4 | To be equivalent to Resistor Detection output source impedance, in order to limit DC current to ground from DC source to ground and protect the ac signal source. Practically it means the ac source is buffered by a capacitor. |
| 5 | Initial values. To be optimized |
| 6 | As specified by TPMDO at table 5, draft 3. |
| 7 | As specified for detection signal |
| 8 | At PD input voltage from 37V to 57V |

# Item 2 - Sensitivity Analysis, Setting Parameters Range

# Item 3 - Audio Interference tests

# Item 4- RF interference

# Item 5-Effects on other 802.3af functions

# Item 6- Other Tests