10GBase-T task force members:

I have collected a full set of test data for my latest version of a magnetic module with a bandwidth wide enough to work for 10GBase-T systems.

Data files are for 4 channels (labeled as A, B, C, and D) of a sample.

File names for differential test data start with D- ; file names for single-ended test data start with S-.  Example D-A contains differential data for channel A.

For differential test data file, the column heading mnemonics SXXYY(Z) are formed with the following abbreviations:

S = Scattering parameter (all file names start with S)

X = C for common mode; X = D for differential mode

Y = 1 for balanced port 1; Y = 2 for balanced port 2.

Z = P, if present, specifies phase angle, in degrees

In addition, the first character in each group (XX and YY) specifies the response port, and the second character specifies the stimulus port.

For example, SCD21 is the ratio in dB of the common-mode signal received at port 2 over the differential-mode signal sent by port 1.  Please note that in differential mode, port 1 in the test data is the combination of ports 1-2 on the network analyzer, and port 2 in the test data is ports 3-4 on the analyzer.

For single-ended test data, the same convention applies, except there is no modes (common or differential), and the port numbers are exactly the same as those on the network analyzer.

Please let me know your feedback on these data.

Thuyen Dinh

Pulse

tdinh@pulseeng.com (858) 385-8053 (in USA)

Thank you.