# Nelco backplane

- More detailed information on the nomenclature, backplane construction, and measurement method is available in file “IEEE Proto s_param release_1_1_0 Rev 1.ppt”

- The backplane is fabricated from Nelco N4000-13SI

- The backplane contains two sets of four channels

  - Outbound channels

    - These channels are in layer 3, which is near the top of the stack.

    - Traffic moves from left to right.

    - The stub length is 191 um, so a significant notch is observed in the SDD21 plot.

  - Inbound channels

    - These channels are in layer 18, which is near the bottom of the stack.

    - Traffic moves from right to left.

    - The stub length is 34 um; no significant stub effects are observed.

- Each channel consists of backplane plus two connectors plus two daughter cards

  - Total channel length is 1 m

  - 2.5 inches of total channel length is contained in each daughter card

  - GbX™ connectors are used

  - The channel pitch is typically 1.85 mm

- Nomenclature

  - As shown below, channels are named according to the connector pins on which they terminate:

  - For example, the outbound channel that begins on pins J2 and K2, and ends on pins G2 and H2 is named J2K2G2H2

  - The complete list of outbound channels is

    - J2K2G2H2

    - J3K3G3H3

    - J4K4G4H4

    - J5K5G5H5

  - The same naming convention applies to inbound channels

  - The complete list of inbound channels is

    - J2K2G2H2

    - J3K3G3H3

    - J4K4G4H4

    - J5K5G5H5

- Through channel S-parameter data

  - Data are provided for each of the four outbound channels and the four inbound channels

  - File names are consistent with the channel names.  For example, data obtained by driving J2K2 and detecting signal on G2H2 is in a file named “sj2k2g2h2.s4p”.

- Port numbering scheme

  - Ports are numbered as follows:

- NEXT data

  - Single-ended NEXT data not available yet

- FEXT data

  - Single-ended FEXT data not available yet