# 438 Cl 69A SC 69A. P 63 L? Kim, Yong

As i understand it, an Annex is informative, the normative information

is in the PMD clause.  In any event, whether 69A is normative or informative

all relevant information should be included.  In my opinion, the information

asked for is not relevant.

_

__________________________________________________________

# 225 Cl 69A SC 69A. P 63 L 41 Grow, Robert

# 517 Cl 69A SC 69A.2 P 64 L 19 Dawe, Piers

I suggest we include the following table:

acronym     meaning                        defined in

NEXT         Near End Cross Talk

FEXT         Far End Cross Talk

SI           Self Interference                    69A.1b

ISI          InterSymbol Interference

IL(f)        Channel Insertion loss               69.3.3.3

Amax(f)      Worst case channel attenuation       69.3.3.2 eq 69-6

A(f)         Smoothed fit to IL(f)                69

f1           a lower frequency bound              69.3.3.1 table 69-2

f2           a upper frequency bound              69.3.3.1 table 69-2

ISIloss      IL(f2)-IL(f1)                        69A.2                  *

minISIloss   Spec limit to ISIloss                table 70-8, 71-8, 72-10 *

BER          Bit Error Ratio

eBER         extrapolated BER

mBER         measured BER                         69A.5 Paragraph 4

EO           correction to measured IT            69A.5 Paragraph 7

EIT          Extrapolated Interference Tolerance  69A.5 Paragraph 8

EIT baseline Spec limit for EIT vs frequency      69A.5 eq 69 ??

EIT base     Parameter used in eq 69 ??           table 70-8, 71-8, 72-10 *

DUT          Device Under Test

___________________________________________________________

# 131 Cl 69A SC 69A. 1 P 63 L John, D'Ambrosia

Recommend:  accept new text reads:

b) Self Interference (SI), caused by reflection, due to impedance

discontinuities, stubs, etc. This is really just a form of

inter-symbol interference (ISI) beyond what a reasonable equalizer i

can handle

(i have changed which into what)

___________________________________________________________

# 627 Cl 69A SC 69A. 1 P 63 L Kundu, Aniruddha

Recommend:  reject

Either I do not understand this or I dissagree.  I beleive that connection

involves full duplex with two differential signal pairs one for data

traveling in each direction.  That is what try to show in figure 69A-1

___________________________________________________________

# 615 Cl 69A SC 69A. 1 P 63 L 16  Beaudoin, Denis

I feel that this is not a test issue and should be handled elsewhere

___________________________________________________________

# 15 Cl 69A SC 69A. 1 P 63 L 18  King, Iain

# 603 Cl 69A SC 69A. 1 P 63 L 18 Booth, Brad

# 412 Cl 69A SC 69A. 1 P 63 L 19 Barrass, Hugh

# 132 Cl 69A SC 69A. 1 P 63 L 21 John, D'Ambrosia

combined:

Recommend:  accept new text reads:

Alien Interference, cross talk from unrelated sources such as clocks,

other kinds of data, power supply noise etc.

___________________________________________________________

# 661 Cl 69A SC 69A. 1 P 63 L 35 David V James

# 660 Cl 69A SC 69A. 1 P 63 L 36 David V James

recommend whatever Shelto thinks.

___________________________________________________________

# 256 Cl 69A SC 69A. 1 P 63 L 39 Healey, Adam

# 84 Cl 69A  SC 69A.2 P 64  L 10 Weiner, Nick

recommend accept, especially if Adam will provide figure

___________________________________________________________

# 262 Cl 69A SC 69A. 1 P 63 L 4 Brink, Robert

recommend reject.

If Robert means to do the test at 10.31353Gb/s and 10.31147Gb/s I do

not see what value will be gained by doubling the test time.  If he

wants to swing back and forth between the two:

1.  He needs to specify the rate.

2.  Complexity get much greater

3.  Utility is unclear

___________________________________________________________

# 578 Cl 69A SC 69A. 1 P 63 L 40 Ghiasi, Ali

recommend reject

Part will be used in a compact enviornment where there is no reason

to expect large swings in Tx phase, and Rx is likely to have exactly

the same frequency reference as the Tx.

Also this would make a major increase in complexity and time for test.

___________________________________________________________

# 581 Cl 69A SC 69A. 1 P 63 L 41 Ghiasi, Ali

recommend reject

changes to be made are insufficiently clear.  At minimum i would like:

1.  What > 10Gs/s FIR would be used

2.  FIR tap weightings to be used

Also, current channel specs are in frequency domain, to connect Rx

specs and channel specs it is usefull to use similar means of

expression.

___________________________________________________________

# 664 Cl 69A SC 69A. 1 P 63 L 42 David V James

# 663 Cl 69A SC 69A. 1 P 63 L 43 David V James

# 662 Cl 69A SC 69A. 1 P 63 L 43 David V James

# 665 Cl 69A SC 69A. 1 P 63 L 52 David V James

recommend whatever Shelto thinks.

_

__________________________________________________________

# 221 Cl 69A SC 69A. 1 P 63 L 6 Grow, Robert

recommend accept, new text reads:

A major problem in communicating across crowded backplanes is interference.

The interfering signal can come from a variety of sources including:

___________________________________________________________

# 518 Cl 69A SC 69A. 1 P 64 L 3 Dawe, Piers

recommend ???

_

__________________________________________________________

# 299 Cl 69A SC 69A. 1 P 64 L 5 Abler, Joe

recommend accept in principle

Annex 69A assumes that the test is being done by someone who wants the

Rx to pass.  The intent is to limit how easy the test can be made and

let the natural inclination of the tester drive the test to as near

to the limit as possible.  Perhaps this should be speciffically stated

in the introduction.  Add text in 69A.1 P6 line 27:

"The test is prescribed with the assumption that the person doing the

test wants the reciever to pass.  Limits on the test and setup are

prescribed to limit how easy the test can be made but the description

is simplified by not prescribing how hard it can be made.  The tester

is expected to make the test as easy to pass as possible given the

limits imposed by the test and his technical resources."

and add 69A.1 P64 end of line 6:

" or a piece of test equipment whose output meets the specification

of the appropriate transmitter, except that if test equipment is used

its output level must be in the lower half of the output level range

allowed to the appropriate transmitter."

___________________________________________________________

#259  69A SC 69A.1  P 64 L 5 Healey, Adam

recommend reject:

The intent of Interference Tolerance test is to have margin in EIT base

large enough to cover the fact that not everything is worst case.  The

difficulty of specifying and building reliably a Tx with worst case

equivalent jitter and equalizer resolution is likely to be excessive.

___________________________________________________________

#71  Cl 69A SC 69A.2 P 64 L 10 Alping, Arne

recommend accept in principle:

Add to figure change recommended for comment 256 & comment 84,

delete first sentence of 69A.2 and replace it with:

"Compliance interconnect is a 100Ohm differential system specified with

respect to insertion loss.  It consists of a frequency dependent attenuator,

a interference injection block and interconnect necessary to conect them

to each other and to TP1 and TP4.

___________________________________________________________

#322 Cl 69A SC 69A.2 P 64 L 11 Baumer, Howard

recommend reject:

The intend to 69A in to provide the minimum number of specifications possible

and assume that the tester will optimize the system to give good Rx performance.

See reply to comment 299.  It is expected that tester will minimize return loss.

___________________________________________________________

#162  Cl 69A SC 69A.2 P 64 L 13  Spagna, Fulvio

recommend accept.

___________________________________________________________

# 222 Cl 69A SC 69A.2 P 64i L 16 Grow, Robert

recommend reject:

This is not a typo:  SI was intended.  Table given in response to comment 225

will help make this clear.

___________________________________________________________

# 163 Cl 69A SC 69A.2 P 64 L 17 Spagna, Fulvio

recomment accept.

___________________________________________________________

# 133 Cl 69A SC 69A.2 P 64 L 17 John, D'Ambrosia

recommend accept in principle but see also

reply to comment 103

___________________________________________________________

# 136 Cl 69A  SC 69A.2 P 64 L 18 John, D'Ambrosia

recomment accept:

but if i change it in tables 70-7, 71-7 and 72-7 really I should

change it everywhere

___________________________________________________________

# 103 Cl 69A SC 69A. 2 P 64 L 21 Moore, Charles

recommend look at acompanying presentation and accept that

verbage.

___________________________________________________________

# 311 Cl 69A SC 69A. 2 P 64 L 22 Seemann, Brian

recommend reject:

The test is intended to be run at near worst-case conditions.  A clean

channel, just worse than Amax(f) is likely to be no worse than a channel

with significant ripple in the attnuation and just above Amax.

___________________________________________________________

# 164 Cl 69A SC 69A. 2 P 64 L 25 Spagna, Fulvio

I think that if we follow my recommendation on comment 103 that this

will be taken care of.

___________________________________________________________

# 516 Cl 69A SC 69A. 2 P 64 L 25 Dawe, Piers

# 116 Cl 69A SC 69A. 2 P 64 L 37 Andre, Szczepanek

# 323 Cl 69A SC 69A. 2 P 64 L 37 Baumer, Howard

# 87 Cl 69A SC 69A. 2 P 64 L 37 Weiner, Nick

recommend accept comment 516.  This should fix 116, 323, and 87

This will require searching out all Amin and Amax references and

fixing them

___________________________________________________________

Cl 69A SC 69A. 2 P 64 L 25 Weiner, Nick

recommend ??

Well i am not going to spec phase:  the chances that any realizable channel

with the correct amplitude characteristics will have workable phase

characteristics as well is much greater than the chance we can write a

useful and usable spec.  The question is should we include a note saying

that specifying phase response is unnecessary.  I am inclined to say no

but others may over ride me.

___________________________________________________________

# 165 Cl 69A SC 69A. 2 P 64 L 27 Spagna, Fulvio

# 519 Cl 69A SC 69A. 2 P 64 L 31 Dawe, Piers

recommend accept

___________________________________________________________

# 134 Cl 69A SC 69A. 2 P 64 L 36 John, D'Ambrosia

recommend reject

This is the old sign change, loss vs gain thing.  Above f2 the gain or response

should be less than the value at f2, but the loss should be greater.

___________________________________________________________

# 257 Cl 69A SC 69A. 2 P 64 L 39 Healey, Adam

# 166 Cl 69A SC 69A. 2 P 65 L 1 Spagna, Fulvio

If my solution on comment 103 is accepted, minISIloss goes away and both

comments are taken care of.

___________________________________________________________

# 666 Cl 69A SC 69A. 2 P 65 L 13 David V James

recommend whatever Shelto thinks is right

___________________________________________________________

# 521 Cl 69A SC 69A. 2 P 65 L 20 Dawe, Piers

recommend accept or accept in principle

Either we should add fbaud to the table of definitions or remove and replace

it everywhere.  I have started replacing it, at least in my presentations

but defining it may be better.  What do you think?

___________________________________________________________

# 520 Cl 69A SC 69A. 2 P 65 L 22 Dawe, Piers

recommend accept

___________________________________________________________

# 118 Cl 69A SC 69A. 2 P 69 L 22 Andre, Szczepanek

I want help here from someone who knows his way around the IEEE802

specifying system.  If this comment is correct, we or i will have

to do a lot of re-writing and if it is me i will need some advice.

___________________________________________________________

# 167 Cl 69A SC 69A. 3 P 65 L 27 Spagna, Fulvio

recommend accept

with mixed feelings.  Fulvio is correct but i think that the hint included

in the existing text is useful

___________________________________________________________

# 324 Cl 69A SC 69A. 3 P 65 L 27  Baumer, Howard

recommend accept

___________________________________________________________

# 169 Cl 69A SC 69A. 3 P 66 L 21 Spagna, Fulvio

# 106 Cl 69A SC 69A. 5 P 66 L 21 Moore, Charles

# 332 Cl 69A SC 69A. 5 P 66 L 21 Baumer, Howard

# 81 Cl 69A SC 69A. 5 P 66 L 21 Altmann, Michael

# 335 Cl 69A SC 69A. 5 P 66 L 23 Baumer, Howard

# 333 Cl 69A SC 69A. 5 P 66 L 23 Baumer, Howard

recommend accept in principle

use wording from comment 106 is one possibility, or see my separate

presentation on these comments

___________________________________________________________

# 168 Cl 69A SC 69A. 3 P 66 L 6 Spagna, Fulvio

recommend accept

___________________________________________________________

# 227 Cl 69A SC 69A. 3 P 67 L 21 Grow, Robert

recommend accept

Most likely we will need to change these figures anyway.

___________________________________________________________

# 105 Cl 69A SC 69A. 3. 3. 5 P 59 L 11 Moore, Charles

move this into clause 69.3.3.5 and accept of course

___________________________________________________________

# 100 Cl 69A SC 69A. 4 P 65 L 34 Gao, Xiao Ming

recommend reject

comment has great merit but it lacks sufficiently detailed method of

resolution.  When we have a specific test described with experimental

results showing that it works i will revise my opinion.

___________________________________________________________

# 302 Cl 69A SC 69A. 4 P 65 L 35 Abler, Joe

recommend accept in principle

Replace first sentence in paragraph beginning Page 66 Line 27 with

The frequency of the inteference generator is then stepped from f1 to

greater than {fbaud or SignalFrequency}.  The step size shall be

selected so that no samples fall at integer submultiples of {fbaud

or SignalFrequency}

___________________________________________________________

# 325 Cl 69A SC 69A. 4 P 65 L 36 Baumer, Howard#

recommend accept in principle :

change second sentence of 69A.4 to read:

The path of the interfering signal to the DUT should be calibrated so

the amplitude of interference at the DUT will be known to an accurace

of 0.5dB or better.

___________________________________________________________

# 326 Cl 69A SC 69A. 4 P 65 L 36 Baumer, Howard

recommend reject:

69A.3 covers the method of combining interference and data

___________________________________________________________

# 628 Cl 69A SC 69A. 4 P 65 L 36 Kundu, Aniruddha

recommend reject:

phase is only meaningfull with respect to another signal of the same

frequency.  There is no other signal at the same frequency as the

interfering signal.

___________________________________________________________

# 104 Cl 69A SC 69A. 5 P 64 L 21 Moore, Charles

# 301 Cl 69A SC 69A. 5 P 66 L 27 Abler, Joe

Recommend accept 301 with added text in 69A.4 after the text given

under comment 302:

A minimum of 20 samples shall be take from f1 to {fbaud or SignalFrequency}.

Less sure about104 but it will reduce the penalty for choosing a large number

of samples

___________________________________________________________

# 327 Cl 69A SC 69A. 5 P 65 L 42 Baumer, Howard

recommend reject:

error rate is how many errors there are in an amount of time, BER

is Bit Error Ratio, the ratio of error rate to {fbaud or SignalFrequency}.

The existing text uses the terms correctly.

___________________________________________________________

# 328 Cl 69A SC 69A. 5 P 66 L 1 Baumer, Howard

# 330 Cl 69A SC 69A. 5 P 66 L 4 Baumer, Howard

recommend accept

___________________________________________________________

# 329 Cl 69A SC 69A. 5 P 66 L 4 Baumer, Howard

recommend accept

I guess "or" should sit on its own line

___________________________________________________________

# 226 Cl 69A SC 69A. 5 P 66 L 16 Grow, Robert

recommend accept

___________________________________________________________

# 228 Cl 69A SC 69A. 5 P 66 L 23 Grow, Robert

# 334 Cl 69A SC 69A. 5 P 66 L 23 Baumer, Howard

# 232 Cl 69A SC 69A. 5 P 66 L 23 Dudek, Mike

recommend accept

___________________________________________________________

# 336 Cl 69A SC 69A. 5 P 66 L 28 Baumer, Howard

# 337 Cl 69A SC 69A. 5 P 66 L 28 Baumer, Howard

recommend accept

___________________________________________________________

# 338 Cl 69A SC 69A. 5 P 66 L 29 Baumer, Howard

recommend accept

___________________________________________________________

# 340 Cl 69A SC 69A. 5 P 66 L 34 Baumer, Howard

# 229 Cl 69A SC 69A. 5 P 66 L 34 Grow, Robert

recommend accept

I will not suggest equations numbers since previous changes may affect

the number

___________________________________________________________

# 303 Cl 69A SC 69A. 5 P 66 L 34 Abler, Joe

# 339 Cl 69A SC 69A. 5 P 66 L 34 Baumer, Howard

# 108 Cl 69A SC 69A. 5 P 66 L 34 Liu, Cathy

# 231 Cl 69A SC 69A. 5 P 66 L 34 Dudek, Mike

# 88 Cl 69A SC 69A. 5 P 66 L 34  Weiner, Nick

recommend accept

except fbaud may change to SignalFrequency

___________________________________________________________

# 230 Cl 69A SC 69A. 5 P 66 L 36 Grow, Robert

recommend accept

Most likely we should use EIT baseline

___________________________________________________________

# 85 Cl 69A SC 69A. 5 P 66 L 40 Weiner, Nick

recommend accept in principle

The confusion comes from using BREIT which is hard to understand and

not all that useful.  Change:

"The difference between the EIT baseline and EIT for lowest

EIT relative to the EIT baseline is the baseline relative EIT

(BREIT). BREIT is reported as the result for the interference

tolerance test."

to

"At each sample EIT shall be greater than EIT baseline."

also eliminate BREIT from table 70-8, 71-8, 72-10.  Note it is not included in

my list of definitions.

___________________________________________________________

# 331 Cl 69A SC 69A. 5 P 66 L 8 Baumer, Howard

recommend accept in principle

the paragraph beginning at line 7 is not necessary to the test, it is

just a sanity check before the test begins and most DUTs will perform

their own sanity check.  Delet this paragraph

___________________________________________________________

# 668 Cl 69A SC 69A. 5 P 67 L 43 David V James

# 669 Cl 69A SC 69A. 5 P 67 L 51 David V James

# 667 Cl 69A SC 69A. 5 P 67 L 8  David V James

# 670 Cl 69A SC 69A. 5 P 69 L 2  David V James

recommend, ask Shelto.

___________________________________________________________

# 223 Cl 69A SC Figure 69A- 2 P 65 L 15 Grow, Robert

if my recommendation for comment 103 is accepted, delete confusing figure,

otherwise:

shade area above Amax(f)..Amax(f2) line and mark it as reject region or

similar name

___________________________________________________________

# 224 Cl 69A SC Figure 69A- 2 P 65 L 22 Grow, Robert

recommend accept unless figures are made unnecessary by other changes.