GDMO specification for IEEE 802.3 managed object classes

This annex formally defines the protocol encodings for CMIP and ISO/IEC 15802-2:1995 [ANSI/IEEE Std 802.1B and 802.1k, 1995 Edition] for the IEEE 802.3 Managed Objects using the templates specified in ISO/IEC 10165-4:1992. The application of a GDMO template compiler against DTE MAC entity managed object class to PME managed object class will produce the proper protocol encodings.

NOTE 1—The arcs (that is, object identifier values) defined in Annex 30A deprecate the arcs previously defined in Annexes H.1 (Layer Management), H.2 (Repeater Management), and H.3 (MAU Management). See IEEE Std 802.1F-1993, Annex C.4.

NOTE 2—During the update for IEEE Std 802.3-2005 differences between objects in the root of all registration arcs were harmonized and aligned with the recommendation of IEEE Std 802b-2004. All instances were changed to {iso(1) std(0) iso8802(8802) csma(3)... For maximum compatibility with previous implementations it is recommended that all implementations respond equally to requests for communication based on registration arc {iso(1) member-body(2) us(840) ieee802dot3(10006)...

NOTE 3— The Link Aggregation related managed object classes in Aggregator managed object class to Aggregation Port Debug Information managed object class are deprecated by IEEE Std 802.1AX-200X.

Each attribute definition in this clause references directly by means of the WITH ATTRIBUTE SYNTAX construct or indirectly by means of the DERIVED FROM construct an ASN.1 type or subtype that defines the attribute’s type and range. Those ASN.1 types and subtypes defined exclusively for CSMA/CD Management appear in a single ASN.1 module in Common attributes template.

Counters for these protocol encodings are specified as either 32 or 64 bits wide. Thirty-two bit counters are used for the protocol encoding of counter attributes, providing the minimum rollover time is 58 min or more. Sixty-four bit counters are used for the protocol encoding of counter attributes that could roll over in less than 58 min with a 32-bit counter. Approximate counter rollover times are provided as notes below each counter BEHAVIOUR definition. 10 Gb/s counters are 10 times faster than 1000 Mb/s counters, 1000 Mb/s counters are 10 times faster than 100 Mb/s counters and 100 Mb/s counters are 10 times faster than      10 Mb/s counters. For formal definition of the counter, refer to the BEHAVIOUR bCMCounter in Common attributes template.

- DTE MAC entity managed object class

- DTE MAC entity formal definition

oMACEntity	MANAGED OBJECT CLASS

DERIVED FROM			'CCITT Rec. X.721 (1992) | ISO/IEC 10165-2:1992':top;

CHARACTERIZED BY

pBasic		PACKAGE

ATTRIBUTES		aMACID					GET;

ACTIONS		acInitializeMAC;

;

;

CONDITIONAL PACKAGES

pMandatory		PACKAGE

ATTRIBUTES		aFramesTransmittedOK					GET,

aSingleCollisionFrames					GET,

aMultipleCollisionFrames					GET,

aFramesReceivedOK					GET,

aFrameCheckSequenceErrors					GET,

aAlignmentErrors					GET,

aMACCapabilities					GET,

aDuplexStatus					GET-REPLACE,

aRateControlAbility					GET,

aRateControlStatus					GET-REPLACE,

aDeferControlAbility					GET,

aDeferControlStatus					GET-REPLACE;

REGISTERED AS		{iso(1) std(0) iso8802(8802) csma(3) csmacdmgt(30) package(4) macMandatoryPkg(1)};

PRESENT IF		Conformance to DTE Management is desired. Attributes aMACCapabilities and aDuplexStatus are mandatory in systems that can operate in full duplex mode and are recommended in systems that can only operate in half duplex mode.,

pRecommended		PACKAGE

ATTRIBUTES		aOctetsTransmittedOK					GET,

aFramesWithDeferredXmissions					GET,

aLateCollisions					GET,

aFramesAbortedDueToXSColls					GET,

aFramesLostDueToIntMACXmitError					GET,

aCarrierSenseErrors					GET,

aOctetsReceivedOK					GET,

aFramesLostDueToIntMACRcvError					GET,

aPromiscuousStatus					GET-REPLACE,

aReadMulticastAddressList					GET;

ACTIONS		acAddGroupAddress,

acDeleteGroupAddress;

REGISTERED AS		{iso(1) std(0) iso8802(8802) csma(3) csmacdmgt(30) package(4) macRecommendedPkg(2)};

PRESENT IF		The Recommended Package is implemented.,

pOptional			PACKAGE

ATTRIBUTES		aMulticastFramesXmittedOK					GET,

aBroadcastFramesXmittedOK					GET,

aMulticastFramesReceivedOK					GET,

aBroadcastFramesReceivedOK					GET,

aInRangeLengthErrors					GET,

aOutOfRangeLengthField					GET,

aFrameTooLongErrors					GET,

aMaxFrameLength					GET,

aMACEnableStatus					GET-REPLACE,

aTransmitEnableStatus					GET-REPLACE,

aMulticastReceiveStatus					GET-REPLACE,

aReadWriteMACAddress					GET-REPLACE;

ACTIONS		acExecuteSelfTest;

REGISTERED AS		{iso(1) std(0) iso8802(8802) csma(3) csmacdmgt(30) package(4) optionalPkg(3)};

PRESENT IF		The Optional Package and the Recommended Package are implemented.,

pArray			PACKAGE

ATTRIBUTES		aCollisionFrames					GET;

REGISTERED AS		{iso(1) std(0) iso8802(8802) csma(3) csmacdmgt(30) package(4) arrayPkg(4)};

PRESENT IF		The Array Package and the Recommended Package are implemented.,

pExcessiveDeferral			PACKAGE

ATTRIBUTES		aFramesWithExcessiveDeferral					GET;

REGISTERED AS		{iso(1) std(0) iso8802(8802) csma(3) csmacdmgt(30) package(4) excessiveDeferralPkg(5)};

PRESENT IF		The ExcessiveDeferral Package and the Recommended Package are implemented.;

REGISTERED AS		{iso(1) std(0) iso8802(8802) csma(3) csmacdmgt(30) managedObjectClass(3) macObjectClass(1)};

nbMACName	NAME BINDING

SUBORDINATE OBJECT CLASS	oMACEntity;

NAMED BY SUPERIOR OBJECT CLASS
“ISO/IEC 10165-2”:system;

WITH ATTRIBUTE	aMACID;

REGISTERED AS	{iso(1) std(0) iso8802(8802) csma(3) csmacdmgt(30) nameBinding(6) macName(1)};

nbMACMonitor	NAME BINDING

SUBORDINATE OBJECT CLASS	“IEEE802.1F”:oEWMAMetricMonitor;

NAMED BY SUPERIOR OBJECT CLASS
“ISO/IEC 10165-2”:system;

WITH ATTRIBUTE	“IEEE802.1F”:aScannerId;

CREATE	WITH-AUTOMATIC-INSTANCE-NAMING;

DELETE	ONLY-IF-NO-CONTAINED-OBJECTS;

REGISTERED AS	{iso(1) std(0) iso8802(8802) csma(3) csmacdmgt(30) nameBinding(6) macMonitor(2)};

nbMAC-MACControl						NAME BINDING

SUBORDINATE OBJECT CLASS					oMACEntity;

NAMED BY SUPERIOR OBJECT CLASS					oMACControlEntity;

WITH ATTRIBUTE					aMACID;

REGISTERED AS		{iso(1) std(0) iso8802(8802) csma(3) csmacdmgt(30) nameBinding(6) nbMAC-MACControl(16)};

nbMAC-Aggregator						NAME BINDING

SUBORDINATE OBJECT CLASS					oMACEntity;

NAMED BY SUPERIOR OBJECT CLASS					oAggregator;

WITH ATTRIBUTE					aMACID;

REGISTERED AS		{iso(1) std(0) iso8802(8802) csma(3) csmacdmgt(30) nameBinding(6) nbMAC-Aggregator(17)};

aDeferControlAbility					ATTRIBUTE

WITH ATTRIBUTE SYNTAX				IEEE802Dot3-MgmtAttributeModule.TrueFalse;

MATCHES FOR				EQUALITY, ORDERING;

BEHAVIOUR 				bDeferControlAbility;

REGISTERED AS		{iso(1) std(0) iso8802(8802) csma(3) csmacdmgt(30) attribute(7) deferControlAbility(311)};

bDeferControlAbility				BEHAVIOUR

DEFINED AS 		See “BEHAVIOUR DEFINED AS” in 30.3.1.1.35;

aDeferControlStatus					ATTRIBUTE

WITH ATTRIBUTE SYNTAX				IEEE802Dot3-MgmtAttributeModule.DeferControl;

MATCHES FOR				EQUALITY, ORDERING;

BEHAVIOUR 				bDeferControlStatus;

REGISTERED AS		{iso(1) std(0) iso8802(8802) csma(3) csmacdmgt(30) attribute(7) deferControlStatus(312)};

bDeferControlStatus				BEHAVIOUR

DEFINED AS 		See “BEHAVIOUR DEFINED AS” in 30.3.1.1.36;

- DTE MAC entity attributes

aMACID     						ATTRIBUTE

WITH ATTRIBUTE SYNTAX	IEEE802Dot3-MgmtAttributeModule.OneOfName;

MATCHES FOR	EQUALITY;

BEHAVIOUR	bMACID;

REGISTERED AS	{iso(1) std(0) iso8802(8802) csma(3) csmacdmgt(30) attribute(7) macID(1)};

bMACID   						 BEHAVIOUR

DEFINED AS	See “BEHAVIOUR DEFINED AS” in 30.3.1.1.1;

aFramesTransmittedOK  						 ATTRIBUTE

DERIVED FROM	aCMCounter;

BEHAVIOUR	bFramesTransmittedOK;

REGISTERED AS 	{iso(1) std(0) iso8802(8802) csma(3) csmacdmgt(30) attribute(7) framesTransmittedOK(2)};

bFramesTransmittedOK						 BEHAVIOUR

DEFINED AS	See “BEHAVIOUR DEFINED AS” in 30.3.1.1.2

- The approximate minimum time between counter rollovers for 10 Mb/s operation is 80 h.

- This maps to framesSent (of the mandatory macPackage) in ISO/IEC 10742:1994.;

aSingleCollisionFrames 						 ATTRIBUTE

DERIVED FROM	aCMCounter;

BEHAVIOUR	bSingleCollisionFrames;

REGISTERED AS 	{iso(1) std(0) iso8802(8802) csma(3) csmacdmgt(30) attribute(7) singleCollisionFrames(3)};

bSingleCollisionFrames 						 BEHAVIOUR

DEFINED AS	See “BEHAVIOUR DEFINED AS” in 30.3.1.1.3

- The approximate minimum time between counter rollovers for 10 Mb/s operation is 103 h.;

aMultipleCollisionFrames 						 ATTRIBUTE

DERIVED FROM	aCMCounter;

BEHAVIOUR	bMultipleCollisionFrames;

REGISTERED AS 	{iso(1) std(0) iso8802(8802) csma(3) csmacdmgt(30) attribute(7) multipleCollisionFrames(4)};

bMultipleCollisionFrames  						 BEHAVIOUR

DEFINED AS	See “BEHAVIOUR DEFINED AS” in 30.3.1.1.4

- The approximate minimum time between counter rollovers for 10 Mb/s operation is 125 h.;

aFramesReceivedOK  						 ATTRIBUTE

DERIVED FROM	aCMCounter;

BEHAVIOUR	bFramesReceivedOK;

REGISTERED AS 	{iso(1) std(0) iso8802(8802) csma(3) csmacdmgt(30) attribute(7) framesReceivedOK(5)};

bFramesReceivedOK   						 BEHAVIOUR

DEFINED AS	See “BEHAVIOUR DEFINED AS” in 30.3.1.1.5

- The approximate minimum time between counter rollovers for 10 Mb/s operation is 80 h.

- This maps to framesReceived (of the mandatory macPackage) in ISO/IEC 10742:1994.;

aFrameCheckSequenceErrors 						 ATTRIBUTE

DERIVED FROM	aCMCounter;

BEHAVIOUR	bFrameCheckSequenceErrors;

REGISTERED AS 	{iso(1) std(0) iso8802(8802) csma(3) csmacdmgt(30) attribute(7) frameCheckSequenceErrors(6)};

bFrameCheckSequenceErrors 						 BEHAVIOUR

DEFINED AS	See “BEHAVIOUR DEFINED AS” in 30.3.1.1.6

NOTE—The approximate minimum time between counter rollovers for 10 Mb/s operation is 80 h.;

aAlignmentErrors  						 ATTRIBUTE

DERIVED FROM	aCMCounter;

BEHAVIOUR	bAlignmentErrors;

REGISTERED AS 	{iso(1) std(0) iso8802(8802) csma(3) csmacdmgt(30) attribute(7) alignmentErrors(7)};

bAlignmentErrors  						 BEHAVIOUR

DEFINED AS	See “BEHAVIOUR DEFINED AS” in 30.3.1.1.7

NOTE—The approximate minimum time between counter rollovers for 10 Mb/s operation is 80 h.;

aOctetsTransmittedOK 						 ATTRIBUTE

DERIVED FROM	aCMCounter;

BEHAVIOUR	bOctetsTransmittedOK;

REGISTERED AS 	{iso(1) std(0) iso8802(8802) csma(3) csmacdmgt(30) attribute(7) octetsTransmittedOK(8)};

bOctetsTransmittedOK						 BEHAVIOUR

DEFINED AS	See “BEHAVIOUR DEFINED AS” in 30.3.1.1.8

- The approximate minimum time between counter rollovers for 10 Mb/s operation is 58 min.

- This maps to octetsSent (of the mandatory macPackage) in ISO/IEC 10742:1994.;

aFramesWithDeferredXmissions						 ATTRIBUTE

DERIVED FROM	aCMCounter;

BEHAVIOUR	bFramesWithDeferredXmissions;

REGISTERED AS 	{iso(1) std(0) iso8802(8802) csma(3) csmacdmgt(30) attribute(7) framesWithDeferredXmissions(9)};

bFramesWithDeferredXmissions						 BEHAVIOUR

DEFINED AS	See “BEHAVIOUR DEFINED AS” in 30.3.1.1.9

NOTE—The approximate minimum time between counter rollovers for 10 Mb/s operation is 103 h.;

aLateCollisions  						 ATTRIBUTE

DERIVED FROM	aCMCounter;

BEHAVIOUR	bLateCollisions;

REGISTERED AS 	{iso(1) std(0) iso8802(8802) csma(3) csmacdmgt(30) attribute(7) lateCollisions(10)};

bLateCollisions 						 BEHAVIOUR

DEFINED AS	See “BEHAVIOUR DEFINED AS” in 30.3.1.1.10

NOTE—The approximate minimum time between counter rollovers for 10 Mb/s operation is 80 h.;

aFramesAbortedDueToXSColls 						 ATTRIBUTE

DERIVED FROM	aCMCounter;

BEHAVIOUR	bFramesAbortedDueToXSColls;

REGISTERED AS 	{iso(1) std(0) iso8802(8802) csma(3) csmacdmgt(30) attribute(7) framesAbortedDueToXSColls(11)};

bFramesAbortedDueToXSColls  						 BEHAVIOUR

DEFINED AS	See “BEHAVIOUR DEFINED AS” in 30.3.1.1.11

NOTE—The approximate minimum time between counter rollovers for 10 Mb/s operation is 53 days.;

aFramesLostDueToIntMACXmitError						 ATTRIBUTE

DERIVED FROM	aCMCounter;

BEHAVIOUR	bFramesLostDueToIntMACXmitError;

REGISTERED AS 	{iso(1) std(0) iso8802(8802) csma(3) csmacdmgt(30) attribute(7) framesLostDueToIntMACXmitError(12)};

bFramesLostDueToIntMACXmitError   						BEHAVIOUR

DEFINED AS	See “BEHAVIOUR DEFINED AS” in 30.3.1.1.12

NOTE—The approximate minimum time between counter rollovers for 10 Mb/s operation is 16 h.;

aCarrierSenseErrors  						 ATTRIBUTE

DERIVED FROM	aCMCounter;

BEHAVIOUR	bCarrierSenseErrors;

REGISTERED AS 	{iso(1) std(0) iso8802(8802) csma(3) csmacdmgt(30) attribute(7) carrierSenseErrors(13)};

bCarrierSenseErrors 						 BEHAVIOUR

DEFINED AS	See “BEHAVIOUR DEFINED AS” in 30.3.1.1.13

NOTE—The approximate minimum time between counter rollovers for 10 Mb/s operation is 80 h.;

aOctetsReceivedOK  						 ATTRIBUTE

DERIVED FROM	aCMCounter;

BEHAVIOUR	bOctetsReceivedOK;

REGISTERED AS 	{iso(1) std(0) iso8802(8802) csma(3) csmacdmgt(30) attribute(7) octetsReceivedOK(14)};

bOctetsReceivedOK   						 BEHAVIOUR

DEFINED AS	See “BEHAVIOUR DEFINED AS” in 30.3.1.1.14

- The approximate minimum time between counter rollovers for 10 Mb/s operation is 58 min.

- This maps to octetsReceived (of the mandatory macPackage) in ISO/IEC 10742:1994.;

aFramesLostDueToIntMACRcvError 						 ATTRIBUTE

DERIVED FROM	aCMCounter;

BEHAVIOUR	bFramesLostDueToIntMACRcvError;

REGISTERED AS 	{iso(1) std(0) iso8802(8802) csma(3) csmacdmgt(30) attribute(7) framesLostDueToIntMACRcvError(15)};

bFramesLostDueToIntMACRcvError  						BEHAVIOUR

DEFINED AS	See “BEHAVIOUR DEFINED AS” in 30.3.1.1.15

NOTE—The approximate minimum time between counter rollovers for 10 Mb/s operation is 80 h.;

aPromiscuousStatus  						 ATTRIBUTE

WITH ATTRIBUTE SYNTAX	IEEE802Dot3-MgmtAttributeModule.TrueFalse;

BEHAVIOUR	bPromiscuousStatus;

REGISTERED AS 	{iso(1) std(0) iso8802(8802) csma(3) csmacdmgt(30) attribute(7) promiscuousStatus(16)};

bPromiscuousStatus   						 BEHAVIOUR

DEFINED AS	See “BEHAVIOUR DEFINED AS” in 30.3.1.1.16;

aReadMulticastAddressList						 ATTRIBUTE

WITH ATTRIBUTE SYNTAX	IEEE802Dot3-MgmtAttributeModule.
MulticastAddressList;

BEHAVIOUR	bReadMulticastAddressList;

REGISTERED AS 	{iso(1) std(0) iso8802(8802) csma(3) csmacdmgt(30) attribute(7) readMulticastAddressList(17)};

bReadMulticastAddressList 						 BEHAVIOUR

DEFINED AS	See “BEHAVIOUR DEFINED AS” in 30.3.1.1.17;

aMulticastFramesXmittedOK						 ATTRIBUTE

DERIVED FROM	aCMCounter;

BEHAVIOUR	bMulticastFramesXmittedOK;

REGISTERED AS 	{iso(1) std(0) iso8802(8802) csma(3) csmacdmgt(30) attribute(7) multicastFramesXmittedOK(18)};

bMulticastFramesXmittedOK  						 BEHAVIOUR

DEFINED AS	See “BEHAVIOUR DEFINED AS” in 30.3.1.1.18

NOTE—The approximate minimum time between counter rollovers for 10 Mb/s operation is 80 h.;

aBroadcastFramesXmittedOK						 ATTRIBUTE

DERIVED FROM	aCMCounter;

BEHAVIOUR	bBroadcastFramesXmittedOK;

REGISTERED AS 	{iso(1) std(0) iso8802(8802) csma(3) csmacdmgt(30) attribute(7) broadcastFramesXmittedOK(19)};

bBroadcastFramesXmittedOK  						 BEHAVIOUR

DEFINED AS	See “BEHAVIOUR DEFINED AS” in 30.3.1.1.19

NOTE—The approximate minimum time between counter rollovers for 10 Mb/s operation is 80 h.;

aFramesWithExcessiveDeferral 						 ATTRIBUTE

DERIVED FROM	aCMCounter;

BEHAVIOUR	bFramesWithExcessiveDeferral;

REGISTERED AS 	{iso(1) std(0) iso8802(8802) csma(3) csmacdmgt(30) attribute(7) framesWithExcessiveDeferral(20)};

bFramesWithExcessiveDeferral 						 BEHAVIOUR

DEFINED AS	See “BEHAVIOUR DEFINED AS” in 30.3.1.1.20

NOTE—The approximate minimum time between counter rollovers for 10 Mb/s operation is 58 days.;

aMulticastFramesReceivedOK 						 ATTRIBUTE

DERIVED FROM	aCMCounter;

BEHAVIOUR	bMulticastFramesReceivedOK;

REGISTERED AS 	{iso(1) std(0) iso8802(8802) csma(3) csmacdmgt(30) attribute(7) multicastFramesReceivedOK(21)};

bMulticastFramesReceivedOK 						 BEHAVIOUR

DEFINED AS	See “BEHAVIOUR DEFINED AS” in 30.3.1.1.21

NOTE—The approximate minimum time between counter rollovers for 10 Mb/s operation is 80 h.;

aBroadcastFramesReceivedOK 						 ATTRIBUTE

DERIVED FROM	aCMCounter;

BEHAVIOUR	bBroadcastFramesReceivedOK;

REGISTERED AS 	{iso(1) std(0) iso8802(8802) csma(3) csmacdmgt(30) attribute(7) broadcastFramesReceivedOK(22)};

bBroadcastFramesReceivedOK						 BEHAVIOUR

DEFINED AS	See “BEHAVIOUR DEFINED AS” in 30.3.1.1.22

NOTE—The approximate minimum time between counter rollovers for 10 Mb/s operation is 80 h.;

aInRangeLengthErrors 						 ATTRIBUTE

DERIVED FROM	aCMCounter;

BEHAVIOUR	bInRangeLengthErrors;

REGISTERED AS 	{iso(1) std(0) iso8802(8802) csma(3) csmacdmgt(30) attribute(7) inRangeLengthErrors(23)};

bInRangeLengthErrors 						 BEHAVIOUR

DEFINED AS	See “BEHAVIOUR DEFINED AS” in 30.3.1.1.23

NOTE—The approximate minimum time between counter rollovers for 10 Mb/s operation is 80 h.;

aOutOfRangeLengthField 						 ATTRIBUTE

DERIVED FROM	aCMCounter;

BEHAVIOUR	bOutOfRangeLengthField;

REGISTERED AS 	{iso(1) std(0) iso8802(8802) csma(3) csmacdmgt(30) attribute(7) outOfRangeLengthField(24)};

bOutOfRangeLengthField  						 BEHAVIOUR

DEFINED AS	See “BEHAVIOUR DEFINED AS” in 30.3.1.1.24

- The approximate minimum time between counter rollovers for 10 Mb/s operation is 80 h.;

aFrameTooLongErrors 						 ATTRIBUTE

DERIVED FROM	aCMCounter;

BEHAVIOUR	bFrameTooLongErrors;

REGISTERED AS 	{iso(1) std(0) iso8802(8802) csma(3) csmacdmgt(30) attribute(7) frameTooLongErrors(25)};

bFrameTooLongErrors  						 BEHAVIOUR

DEFINED AS	See “BEHAVIOUR DEFINED AS” in 30.3.1.1.25

- The approximate minimum time between counter rollovers for 10 Mb/s operation is 61 days.;

aMACEnableStatus   						 ATTRIBUTE

WITH ATTRIBUTE SYNTAX	IEEE802Dot3-MgmtAttributeModule.TrueFalse;

BEHAVIOUR	bMACEnableStatus;

REGISTERED AS 	{iso(1) std(0) iso8802(8802) csma(3) csmacdmgt(30) attribute(7) mACEnableStatus(26)};

bMACEnableStatus   						 BEHAVIOUR

DEFINED AS	See “BEHAVIOUR DEFINED AS” in 30.3.1.1.26;

aTransmitEnableStatus 						 ATTRIBUTE

WITH ATTRIBUTE SYNTAX	IEEE802Dot3-MgmtAttributeModule.TrueFalse;

BEHAVIOUR	bTransmitEnableStatus;

REGISTERED AS 	{iso(1) std(0) iso8802(8802) csma(3) csmacdmgt(30) attribute(7) transmitEnableStatus(27)};

bTransmitEnableStatus 						 BEHAVIOUR

DEFINED AS	See “BEHAVIOUR DEFINED AS” in 30.3.1.1.27;

aMulticastReceiveStatus 						 ATTRIBUTE

WITH ATTRIBUTE SYNTAX	IEEE802Dot3-MgmtAttributeModule.TrueFalse;

BEHAVIOUR	bMulticastReceiveStatus;

REGISTERED AS 	{iso(1) std(0) iso8802(8802) csma(3) csmacdmgt(30) attribute(7) multicastReceiveStatus(28)};

bMulticastReceiveStatus  						 BEHAVIOUR

DEFINED AS	See “BEHAVIOUR DEFINED AS” in 30.3.1.1.28;

aReadWriteMACAddress  						 ATTRIBUTE

WITH ATTRIBUTE SYNTAX	IEEE802CommonDefinitions.MACAddress;

BEHAVIOUR	bReadWriteMACAddress;

REGISTERED AS 	{iso(1) std(0) iso8802(8802) csma(3) csmacdmgt(30) attribute(7) modifyMACAddress(29)};

bReadWriteMACAddress  						 BEHAVIOUR

DEFINED AS	See “BEHAVIOUR DEFINED AS” in 30.3.1.1.29

- This maps to localMACAddress (of the mandatory macPackage) in ISO/IEC 10742:1994.;

aCollisionFrames  						 ATTRIBUTE

WITH ATTRIBUTE SYNTAX	IEEE802Dot3-MgmtAttributeModule.AttemptArray;

BEHAVIOUR	bCollisionFrames;

REGISTERED AS 	{iso(1) std(0) iso8802(8802) csma(3) csmacdmgt(30) attribute(7) collisionFrames(30)};

bCollisionFrames   						 BEHAVIOUR

DEFINED AS	See “BEHAVIOUR DEFINED AS” in 30.3.1.1.30

- The approximate minimum time for any single counter rollover for 10 Mb/s operation is 103 h.;

aMACCapabilities 						ATTRIBUTE

WITH ATTRIBUTE SYNTAX	IEEE802Dot3-MgmtAttributeModule.MACCapabilitiesList;

MATCHES FOR	EQUALITY, ORDERING;

BEHAVIOUR	bMACCapabilities;

REGISTERED AS 	{iso(1) std(0) iso8802(8802) csma(3) csmacdmgt(30) attribute(7) aMACCapabilities(89)};

bMACCapabilities   						 BEHAVIOUR

DEFINED AS	See “BEHAVIOUR DEFINED AS” in 30.3.1.1.31;

aMaxFrameLength 						ATTRIBUTE

WITH ATTRIBUTE SYNTAX	IEEE802Dot3-MgmtAttributeModule.MaxFrameLengthList;

MATCHES FOR	EQUALITY, ORDERING;

BEHAVIOUR	bMaxFrameLength;

REGISTERED AS 	{iso(1) std(0) iso8802(8802) csma(3) csmacdmgt(30) attribute(7) aMaxFrameLength(357)};

bMaxFrameLength 						 BEHAVIOUR

DEFINED AS	See “BEHAVIOUR DEFINED AS” in 30.3.1.1.37;

aDuplexStatus   						 ATTRIBUTE

WITH ATTRIBUTE SYNTAX	IEEE802Dot3-MgmtAttributeModule.DuplexValues;

MATCHES FOR	EQUALITY;

BEHAVIOUR	bDuplexStatus;

REGISTERED AS 	{iso(1) std(0) iso8802(8802) csma(3) csmacdmgt(30) attribute(7) aDuplexStatus(90)};

bDuplexStatus   						BEHAVIOUR

DEFINED AS	See “BEHAVIOUR DEFINED AS” in 30.3.1.1.32;

aRateControlAbility						ATTRIBUTE

WITH ATTRIBUTE SYNTAX	IEEE802Dot3-MgmtAttributeModule.TrueFalse;

BEHAVIOUR	bRateControlAbility;

REGISTERED AS 	{iso(1) std(0) iso8802(8802) csma(3) csmacdmgt(30) attribute(7) rateControlAbility(179)};

bRateControlAbility						BEHAVIOUR

DEFINED AS	See “BEHAVIOUR DEFINED AS” in 30.3.1.1.33;

aRateControlStatus						ATTRIBUTE

WITH ATTRIBUTE SYNTAX	IEEE802Dot3-MgmtAttributeModule.RateValues;

MATCHES FOR	EQUALITY;

BEHAVIOUR	bRateControlStatus;

REGISTERED AS 	{iso(1) std(0) iso8802(8802) csma(3) csmacdmgt(30) attribute(7) rateControlStatus(180)};

bRateControlStatus						BEHAVIOUR

DEFINED AS	See “BEHAVIOUR DEFINED AS” in 30.3.1.1.34;

- DTE MAC entity actions

acInitializeMAC   						 ACTION

BEHAVIOUR	bInitializeMAC;

MODE 	CONFIRMED;

REGISTERED AS	{iso(1) std(0) iso8802(8802) csma(3) csmacdmgt(30) action(9) initializeMAC(1)};

bInitializeMAC     						BEHAVIOUR

DEFINED AS	See “BEHAVIOUR DEFINED AS” in 30.3.1.2.1;

acAddGroupAddress    						ACTION

BEHAVIOUR	bAddGroupAddress;

MODE 	CONFIRMED;

WITH INFORMATION SYNTAX	IEEE802CommonDefinitions.MACAddress;

REGISTERED AS	{iso(1) std(0) iso8802(8802) csma(3) csmacdmgt(30) action(9) addGroupAddress(2)};

bAddGroupAddress     						BEHAVIOUR

DEFINED AS	See “BEHAVIOUR DEFINED AS” in 30.3.1.2.2;

acDeleteGroupAddress   						ACTION

BEHAVIOUR	bDeleteGroupAddress;

MODE 	CONFIRMED;

WITH INFORMATION SYNTAX	IEEE802CommonDefinitions.MACAddress;

REGISTERED AS	{iso(1) std(0) iso8802(8802) csma(3) csmacdmgt(30) action(9) deleteGroupAddress(3)};

bDeleteGroupAddress     						BEHAVIOUR

DEFINED AS	See “BEHAVIOUR DEFINED AS” in 30.3.1.2.3;

acExecuteSelfTest    						ACTION

BEHAVIOUR	bExecuteSelfTestMAC;

MODE 	CONFIRMED;

REGISTERED AS	{iso(1) std(0) iso8802(8802) csma(3) csmacdmgt(30) action(9) executeSelfTestMAC(4)};

bExecuteSelfTestMAC     						BEHAVIOUR

DEFINED AS	See “BEHAVIOUR DEFINED AS” in 30.3.1.2.4;

- DTE physical entity managed object class

- DTE physical entity formal definition

oPHYEntity	MANAGED OBJECT CLASS

DERIVED FROM	'CCITT Rec. X.721 (1992) | ISO/IEC 10165-2 :1992':top;

CHARACTERIZED BY

pBasic	PACKAGE

ATTRIBUTES	aPHYID	GET,
aPHYType		GET,

aPHYTypeList	GET,

aMIIDetect	GET,

aPHYAdminState	GET;

;

;

CONDITIONAL PACKAGES

pRecommended	PACKAGE

ATTRIBUTES	aSQETestErrors	GET;

REGISTERED AS	{iso(1) std(0) iso8802(8802) csma(3) csmacdmgt(30) package(4) 
phyRecommendedPkg(6)};

PRESENT IF	The Recommended Package is implemented.,

pMultiplePhy	PACKAGE

ACTIONS	acPHYAdminControl;

REGISTERED AS	{iso(1) std(0) iso8802(8802) csma(3) csmacdmgt(30) package(4) 
phyMultiplePhyPkg(7)};

PRESENT IF	There is more than one PHY per MAC.,

p100MbpsMonitor	PACKAGE

ATTRIBUTES	aSymbolErrorDuringCarrier	GET;

REGISTERED AS	{iso(1) std(0) iso8802(8802) csma(3) csmacdmgt(30) package(4) 
phy100MbpsMonitor(8)};

PRESENT IF	The 100/1000 Mb/s Monitor capability is implemented.;

REGISTERED AS	{iso(1) std(0) iso8802(8802) csma(3) csmacdmgt(30) managedObjectClass(3) phyObjectClass(2)};

nbPHYName	NAME BINDING

SUBORDINATE OBJECT CLASS	oPHYEntity;

NAMED BY SUPERIOR OBJECT CLASS
oMACEntity;

WITH ATTRIBUTE	aPHYID;

REGISTERED AS	{iso(1) std(0) iso8802(8802) csma(3) csmacdmgt(30) nameBinding(6) phyName(3)};

nbPHYMonitor	NAME BINDING

SUBORDINATE OBJECT CLASS	“IEEE802.1F”:oEWMAMetricMonitor;

NAMED BY SUPERIOR OBJECT CLASS
“ISO/IEC 10165-2”:system;

WITH ATTRIBUTE	“IEEE802.1F”:aScannerId;

CREATE	WITH-AUTOMATIC-INSTANCE-NAMING;

DELETE	ONLY-IF-NO-CONTAINED-OBJECTS;

REGISTERED AS	{iso(1) std(0) iso8802(8802) csma(3) csmacdmgt(30) nameBinding(6) phyMonitor(4)};

- DTE physical entity attributes

aPHYID     						ATTRIBUTE

WITH ATTRIBUTE SYNTAX	IEEE802Dot3-MgmtAttributeModule.OneOfName;

MATCHES FOR	EQUALITY;

BEHAVIOUR	bPHYID;

REGISTERED AS	{iso(1) std(0) iso8802(8802) csma(3) csmacdmgt(30) attribute(7) phyID(31)};

bPHYID     						BEHAVIOUR

DEFINED AS	See “BEHAVIOUR DEFINED AS” in 30.3.2.1.1;

aPHYType     						ATTRIBUTE

WITH ATTRIBUTE SYNTAX	IEEE802Dot3-MgmtAttributeModule.PhyTypeValue;

MATCHES FOR	EQUALITY;

BEHAVIOUR	bPHYType;

REGISTERED AS	{iso(1) std(0) iso8802(8802) csma(3) csmacdmgt(30) attribute(7) phyType(32)};

bPHYType     						BEHAVIOUR

DEFINED AS	See “BEHAVIOUR DEFINED AS” in 30.3.2.1.2;

aPHYTypeList     						ATTRIBUTE

WITH ATTRIBUTE SYNTAX	IEEE802Dot3-MgmtAttributeModule.PhyTypeList;

MATCHES FOR	EQUALITY, ORDERING;

BEHAVIOUR	bPHYTypeList;

REGISTERED AS	{iso(1) std(0) iso8802(8802) csma(3) csmacdmgt(30) attribute(7) phyTypeList(33)};

bPHYTypeList     						BEHAVIOUR

DEFINED AS	See “BEHAVIOUR DEFINED AS” in 30.3.2.1.3;

aSQETestErrors    						ATTRIBUTE

DERIVED FROM	aCMCounter;

BEHAVIOUR	bSQETestErrors;

REGISTERED AS 	{iso(1) std(0) iso8802(8802) csma(3) csmacdmgt(30) attribute(7) sqeTestErrors(34)};

bSQETestErrors						BEHAVIOUR

DEFINED AS	See “BEHAVIOUR DEFINED AS” in 30.3.2.1.4

- The approximate minimum time between counter rollovers for 10 Mb/s operation is 80 h.;

aSymbolErrorDuringCarrier   						ATTRIBUTE

DERIVED FROM	aCMCounter;

BEHAVIOUR	bSymbolErrorDuringCarrier;

REGISTERED AS 	{iso(1) std(0) iso8802(8802) csma(3) csmacdmgt(30) attribute(7) symbolErrorDuringCarrier(35)};

bSymbolErrorDuringCarrier   						BEHAVIOUR

DEFINED AS	See “BEHAVIOUR DEFINED AS” in 30.3.2.1.5

- The approximate minimum time between counter rollovers for 10 Mb/s operation is 80 h.;

aMIIDetect      ATTRIBUTE

WITH ATTRIBUTE SYNTAX	IEEE802Dot3-MgmtAttributeModule.MIIDetect;

MATCHES FOR	EQUALITY;

BEHAVIOUR	bMIIDetect;

REGISTERED AS	{iso(1) std(0) iso8802(8802) csma(3) csmacdmgt(30) attribute(7) mIIDetect(36)};

bMIIDetect      						BEHAVIOUR

DEFINED AS	See “BEHAVIOUR DEFINED AS” in 30.3.2.1.6;

aPHYAdminState						ATTRIBUTE

WITH ATTRIBUTE SYNTAX	IEEE802Dot3-MgmtAttributeModule.
PortAdminState;

MATCHES FOR	EQUALITY, ORDERING;

BEHAVIOUR	bPHYAdminState;

REGISTERED AS	{iso(1) std(0) iso8802(8802) csma(3) csmacdmgt(30) attribute(7) phyAdminState(37)};

bPHYAdminState    						BEHAVIOUR

DEFINED AS	See “BEHAVIOUR DEFINED AS” in 30.3.2.1.7;

- DTE physical entity actions

acPHYAdminControl    						ACTION

BEHAVIOUR	bPHYAdminControl;

MODE 	CONFIRMED;

WITH INFORMATION SYNTAX	 IEEE802Dot3-MgmtAttributeModule.
PortAdminState;

REGISTERED AS	{iso(1) std(0) iso8802(8802) csma(3) csmacdmgt(30) action(9) phyAdminControl(5)};

bPHYAdminControl     						BEHAVIOUR

DEFINED AS	See “BEHAVIOUR DEFINED AS” in 30.3.2.2.1;

- DTE MAC control entity managed object class

- DTE MAC control entity formal definition

oMACControlEntity				MANAGED OBJECT CLASS

DERIVED FROM					'CCITT Rec. X.721 (1992) | ISO/IEC 10165-2 :1992':top;

CHARACTERIZED BY

pMandatory	PACKAGE

ATTRIBUTES	aMACControlFunctionsSupported	GET-REPLACE;

;

;

CONDITIONAL PACKAGES

pRecommended	PACKAGE

ATTRIBUTES 	aMACControlFramesTransmitted	GET,

aMACControlFramesReceived	GET,

aUnsupportedOpcodesReceived	GET;

REGISTERED AS	{iso(1) std(0) iso8802(8802) csma(3)
	csmacdmgt(30)

package(4) macControlRecommendedPkg(17)};

PRESENT IF	The Recommended Package is implemented.;

REGISTERED AS	{iso(1) std(0) iso8802(8802) csma(3)
	csmacdmgt(30)
	managedObjectClass(3) macControlObjectClass(8)};

nbMACControl-System					NAME BINDING

SUBORDINATE OBJECT CLASS					oMACControlEntity;

NAMED BY SUPERIOR OBJECT CLASS					“ISO/IEC 10165-2”:system;

WITH ATTRIBUTE					aMACControlID;

REGISTERED AS		{iso(1) std(0) iso8802(8802) csma(3) csmacdmgt(30) nameBinding(6) nbMACControl-System(18)};

nbMACControl-Aggregator					NAME BINDING

SUBORDINATE OBJECT CLASS					oMACControlEntity;

NAMED BY SUPERIOR OBJECT CLASS					oAggregator;

WITH ATTRIBUTE					aMACControlID;

REGISTERED AS		{iso(1) std(0) iso8802(8802) csma(3) csmacdmgt(30) nameBinding(6) nbMACControl-Aggregator(19)};

nbMACControlMonitor				NAME BINDING

SUBORDINATE OBJECT CLASS	“IEEE802.1F”:oEWMAMetricMonitor;

NAMED BY SUPERIOR OBJECT CLASS

“ISO/IEC 10165-2”:system;

WITH ATTRIBUTE					“IEEE802.1F”:aScannerId;

CREATE	WITH-AUTOMATIC-INSTANCE-NAMING;

DELETE	ONLY-IF-NO-CONTAINED-OBJECTS;

REGISTERED AS	{iso(1) std(0) iso8802(8802) csma(3)
	csmacdmgt(30) nameBinding(6) macControlMonitor(15)};

- DTE MAC Control entity attributes

aMACControlID					ATTRIBUTE

WITH ATTRIBUTE SYNTAX	IEEE802Dot3-MgmtAttributeModule.OneOfName;

MATCHES FOR	EQUALITY;

BEHAVIOUR	bMACControlID;

REGISTERED AS	{iso(1) std(0) iso8802(8802) csma(3)
csmacdmgt(30) attribute(7) aMACControlID(92)};

bMACControlID					BEHAVIOUR

DEFINED AS	See “BEHAVIOUR DEFINED AS” in 30.3.3.1;

aMACControlFunctionsSupported					ATTRIBUTE

WITH ATTRIBUTE SYNTAX	IEEE802Dot3-MgmtAttributeModule.

MACControlFunctionsList;

MATCHES FOR	EQUALITY, ORDERING;

BEHAVIOUR	bMACControlFunctionsSupported;

REGISTERED AS	{iso(1) std(0) iso8802(8802) csma(3)
csmacdmgt(30) attribute(7) aMACControlFunctionsSupported(93)};

bMACControlFunctionsSupported					BEHAVIOUR

DEFINED AS	See “BEHAVIOUR DEFINED AS” in 30.3.3.2;

aMACControlFramesTransmitted					ATTRIBUTE

DERIVED FROM	aCMCounter;

BEHAVIOUR	bMACControlFramesTransmitted;

REGISTERED AS	{iso(1) std(0) iso8802(8802) csma(3)
csmacdmgt(30) attribute(7) aMACControlFramesTransmitted(94)};

bMACControlFramesTransmitted					BEHAVIOUR

DEFINED AS	See “BEHAVIOUR DEFINED AS” in 30.3.3.3;

aMACControlFramesReceived					ATTRIBUTE

DERIVED FROM	aCMCounter;

BEHAVIOUR	bMACControlFramesReceived;

REGISTERED AS	{iso(1) std(0) iso8802(8802) csma(3)
csmacdmgt(30) attribute(7) aMACControlFramesReceived(95)};

bMACControlFramesReceived					BEHAVIOUR

DEFINED AS	See “BEHAVIOUR DEFINED AS” in 30.3.3.4;

aUnsupportedOpcodesReceived					ATTRIBUTE

DERIVED FROM	aCMCounter;

BEHAVIOUR	bUnsupportedOpcodesReceived;

REGISTERED AS	{iso(1) std(0) iso8802(8802) csma(3)
csmacdmgt(30) attribute(7) aUnsupportedOpcodesReceived(96)};

bUnsupportedOpcodesReceived					BEHAVIOUR

DEFINED AS	See “BEHAVIOUR DEFINED AS” in 30.3.3.5;

- DTE MAC Control function entity managed object class

- DTE MAC Control function entity formal definition

oMACControlFunctionEntity				MANAGED OBJECT CLASS

DERIVED FROM					'CCITT Rec. X.721 (1992) | ISO/IEC 10165-2 :1992':top;

CHARACTERIZED BY

pMandatory	PACKAGE

ATTRIBUTES	aPAUSELinkDelayAllowance	GET-REPLACE;

;

;

CONDITIONAL PACKAGES

pRecommended	PACKAGE

ATTRIBUTES	aPAUSEMACCtrlFramesTransmitted	GET,

aPAUSEMACCtrlFramesReceived	GET;

REGISTERED AS	{iso(1) std(0) iso8802(8802) csma(3)
	csmacdmgt(30)
	package(4) macControlFunctionRecomendedPkg(25)};

PRESENT IF	The Recommended Package is implemented.;

REGISTERED AS	{iso(1) std(0) iso8802(8802) csma(3)
csmacdmgt(30)
managedObjectClass(3)macControlFunctionObjectClass(9)};

oMPCP					MANAGED OBJECT CLASS

DERIVED FROM				'CCITT Rec. X.721 (1992) | ISO/IEC 10165-2:1992':top;

CHARACTERIZED BY

pMPCPBasic			PACKAGE

ATTRIBUTES		aMPCPID					GET,

aMPCPAdminState					GET,

aMPCPMode					GET,

aMPCPLinkID					GET,

aMPCPRemoteMACAddress					GET,

aMPCPRegistrationState					GET,

aMPCPMACCtrlFramesTransmitted					GET,

aMPCPMACCtrlFramesReceived					GET,

aMPCPTxGate					GET,

aMPCPTxRegAck					GET,

aMPCPTxRegister					GET,

aMPCPTxRegRequest					GET,

aMPCPTxReport					GET,

aMPCPRxGate					GET,

aMPCPRxRegAck					GET,

aMPCPRxRegister					GET,

aMPCPRxRegRequest					GET,

aMPCPRxReport					GET,

aMPCPTransmitElapsed					GET,

aMPCPReceiveElapsed					GET,

aMPCPRoundTripTime					GET,

aMPCPDiscoveryWindowsSent					GET,

aMPCPDiscoveryTimeout					GET,

aMPCPMaximumPendingGrants					GET;

ACTIONS		acMPCPAdminControl;

;

;

REGISTERED AS	{iso(1) std(0) iso8802(8802) csma(3) csmacdmgt(30) managedObjectClass(3) mpcpObjectClass(21)};

nbMPCP-macControl					NAME BINDING

SUBORDINATE OBJECT CLASS					oMPCP;

NAMED BY SUPERIOR OBJECT CLASS					oMACControlEntity AND SUBCLASSES;

WITH ATTRIBUTE					aMPCPID;

REGISTERED AS		{iso(1) std(0) iso8802(8802) csma(3) csmacdmgt(30) nameBinding(6) mpcp-macControl(34)};

aMPCPID					ATTRIBUTE

WITH ATTRIBUTE SYNTAX				IEEE802Dot3-MgmtAttributeModule.OneOfName;

MATCHES FOR				EQUALITY, ORDERING;

BEHAVIOUR 				bMPCPID;

REGISTERED AS		{iso(1) std(0) iso8802(8802) csma(3) csmacdmgt(30) attribute(7) mpcpID(351)};

bMPCPID				BEHAVIOUR

DEFINED AS 		See “BEHAVIOUR DEFINED AS” in 30.3.5.1.1;

aMPCPAdminState					ATTRIBUTE

WITH ATTRIBUTE SYNTAX				IEEE802Dot3-MgmtAttributeModule.PortAdminState;

MATCHES FOR				EQUALITY, ORDERING;

BEHAVIOUR 				bMPCPAdminState;

REGISTERED AS		{iso(1) std(0) iso8802(8802) csma(3) csmacdmgt(30) attribute(7) mpcpAdminState(278)};

bMPCPAdminState				BEHAVIOUR

DEFINED AS 		See “BEHAVIOUR DEFINED AS” in 30.3.5.1.2;

aMPCPMode					ATTRIBUTE

WITH ATTRIBUTE SYNTAX				IEEE802Dot3-MgmtAttributeModule.MPCPMode;

MATCHES FOR				EQUALITY, ORDERING;

BEHAVIOUR 				bMPCPMode;

REGISTERED AS		{iso(1) std(0) iso8802(8802) csma(3) csmacdmgt(30) attribute(7) mpcpMode(279)};

bMPCPMode				BEHAVIOUR

DEFINED AS 		See “BEHAVIOUR DEFINED AS” in 30.3.5.1.3;

aMPCPLinkID					ATTRIBUTE

WITH ATTRIBUTE SYNTAX 				IEEE802Dot3-MgmtAttributeModule.OneOfName;

MATCHES FOR				EQUALITY, ORDERING;

BEHAVIOUR 				bMPCPLinkID;

REGISTERED AS		{iso(1) std(0) iso8802(8802) csma(3) csmacdmgt(30) attribute(7) mpcpLinkID(282)};

bMPCPLinkID				BEHAVIOUR

DEFINED AS 		See “BEHAVIOUR DEFINED AS” in 30.3.5.1.4;

aMPCPRemoteMACAddress					ATTRIBUTE

WITH ATTRIBUTE SYNTAX 				IEEE802Dot3-MgmtAttributeModule.MACAddress;

MATCHES FOR				EQUALITY, ORDERING;

BEHAVIOUR 				bMPCPRemoteMACAddress;

REGISTERED AS		{iso(1) std(0) iso8802(8802) csma(3) csmacdmgt(30) attribute(7) mpcpRemoteMACAddress(283)};

bMPCPRemoteMACAddress				BEHAVIOUR

DEFINED AS 		See “BEHAVIOUR DEFINED AS” in 30.3.5.1.5;

aMPCPRegistrationState					ATTRIBUTE

WITH ATTRIBUTE SYNTAX 				IEEE802Dot3-MgmtAttributeModule.MPCPRegState;

MATCHES FOR				EQUALITY, ORDERING;

BEHAVIOUR 				bMPCPRegistrationState;

REGISTERED AS		{iso(1) std(0) iso8802(8802) csma(3) csmacdmgt(30) attribute(7) mpcpRegistrationState(284)};

bMPCPRegistrationState				BEHAVIOUR

DEFINED AS 		See “BEHAVIOUR DEFINED AS” in 30.3.5.1.6;

aMPCPMACCtrlFramesTransmitted					ATTRIBUTE

DERIVED FROM				aCMCounter;

MATCHES FOR				EQUALITY, ORDERING;

BEHAVIOUR 				bMPCPMACCtrlFramesTransmitted;

REGISTERED AS		{iso(1) std(0) iso8802(8802) csma(3) csmacdmgt(30) attribute(7) mpcpMACCtrlFramesTransmitted(280)};

bMPCPMACCtrlFramesTransmitted				BEHAVIOUR

DEFINED AS 		See “BEHAVIOUR DEFINED AS” in 30.3.5.1.7;

aMPCPMACCtrlFramesReceived					ATTRIBUTE

DERIVED FROM				aCMCounter;

MATCHES FOR				EQUALITY, ORDERING;

BEHAVIOUR 				bMPCPMACCtrlFramesReceived;

REGISTERED AS		{iso(1) std(0) iso8802(8802) csma(3) csmacdmgt(30) attribute(7) mpcpMACCtrlFramesReceived(281)};

bMPCPMACCtrlFramesReceived				BEHAVIOUR

DEFINED AS 		See “BEHAVIOUR DEFINED AS” in 30.3.5.1.8;

aMPCPTxGate					ATTRIBUTE

DERIVED FROM				aCMCounter;

MATCHES FOR				EQUALITY, ORDERING;

BEHAVIOUR 				bMPCPTxGate;

REGISTERED AS		{iso(1) std(0) iso8802(8802) csma(3) csmacdmgt(30) attribute(7) txGate(315)};

bMPCPTxGate				BEHAVIOUR

DEFINED AS 		See “BEHAVIOUR DEFINED AS” in 30.3.5.1.9;

aMPCPTxRegAck					ATTRIBUTE

DERIVED FROM				aCMCounter;

MATCHES FOR				EQUALITY, ORDERING;

BEHAVIOUR 				bMPCPTxRegAck;

REGISTERED AS		{iso(1) std(0) iso8802(8802) csma(3) csmacdmgt(30) attribute(7) txRegAck(316)};

bMPCPTxRegAck				BEHAVIOUR

DEFINED AS 		See “BEHAVIOUR DEFINED AS” in 30.3.5.1.10;

aMPCPTxRegister					ATTRIBUTE

DERIVED FROM				aCMCounter;

MATCHES FOR				EQUALITY, ORDERING;

BEHAVIOUR 				bMPCPTxRegister;

REGISTERED AS		{iso(1) std(0) iso8802(8802) csma(3) csmacdmgt(30) attribute(7) txRegister(317)};

bMPCPTxRegister				BEHAVIOUR

DEFINED AS 		See “BEHAVIOUR DEFINED AS” in 30.3.5.1.11;

aMPCPTxRegRequest					ATTRIBUTE

DERIVED FROM				aCMCounter;

MATCHES FOR				EQUALITY, ORDERING;

BEHAVIOUR 				bMPCPTxRegRequest;

REGISTERED AS		{iso(1) std(0) iso8802(8802) csma(3) csmacdmgt(30) attribute(7) txRegRegister(318)};

bMPCPTxRegRequest				BEHAVIOUR

DEFINED AS 		See “BEHAVIOUR DEFINED AS” in 30.3.5.1.12;

aMPCPTxReport					ATTRIBUTE

DERIVED FROM				aCMCounter;

MATCHES FOR				EQUALITY, ORDERING;

BEHAVIOUR 				bMPCPTxReport;

REGISTERED AS		{iso(1) std(0) iso8802(8802) csma(3) csmacdmgt(30) attribute(7) txReport(319)};

bMPCPTxReport				BEHAVIOUR

DEFINED AS 		See “BEHAVIOUR DEFINED AS” in 30.3.5.1.13;

aMPCPRxGate					ATTRIBUTE

DERIVED FROM				aCMCounter;

MATCHES FOR				EQUALITY, ORDERING;

BEHAVIOUR 				bMPCPRxGate;

REGISTERED AS		{iso(1) std(0) iso8802(8802) csma(3) csmacdmgt(30) attribute(7) rxGate(320)};

bMPCPRxGate				BEHAVIOUR

DEFINED AS 		See “BEHAVIOUR DEFINED AS” in 30.3.5.1.14;

aMPCPRxRegAck					ATTRIBUTE

DERIVED FROM				aCMCounter;

MATCHES FOR				EQUALITY, ORDERING;

BEHAVIOUR 				bMPCPRxRegAck;

REGISTERED AS		{iso(1) std(0) iso8802(8802) csma(3) csmacdmgt(30) attribute(7) rxRegAck(321)};

bMPCPRxRegAck				BEHAVIOUR

DEFINED AS 		See “BEHAVIOUR DEFINED AS” in 30.3.5.1.15;

aMPCPRxRegister					ATTRIBUTE

DERIVED FROM				aCMCounter;

MATCHES FOR				EQUALITY, ORDERING;

BEHAVIOUR 				bMPCPRxRegister;

REGISTERED AS		{iso(1) std(0) iso8802(8802) csma(3) csmacdmgt(30) attribute(7) rxRegister(322)};

bMPCPRxRegister				BEHAVIOUR

DEFINED AS 		See “BEHAVIOUR DEFINED AS” in 30.3.5.1.16;

aMPCPRxRegRequest					ATTRIBUTE

DERIVED FROM				aCMCounter;

MATCHES FOR				EQUALITY, ORDERING;

BEHAVIOUR 				bMPCPRxRegRequest;

REGISTERED AS		{iso(1) std(0) iso8802(8802) csma(3) csmacdmgt(30) attribute(7) rxRegRequest(323)};

bMPCPRxRegRequest				BEHAVIOUR

DEFINED AS 		See “BEHAVIOUR DEFINED AS” in 30.3.5.1.17;

aMPCPRxReport					ATTRIBUTE

DERIVED FROM				aCMCounter;

MATCHES FOR				EQUALITY, ORDERING;

BEHAVIOUR 				bMPCPRxReport;

REGISTERED AS		{iso(1) std(0) iso8802(8802) csma(3) csmacdmgt(30) attribute(7) rxReport(324)};

bMPCPRxReport				BEHAVIOUR

DEFINED AS 		See “BEHAVIOUR DEFINED AS” in 30.3.5.1.18;

aMPCPTransmitElapsed					ATTRIBUTE

WITH ATTRIBUTE SYNTAX 				IEEE802Dot3-MgmtAttributeModule.Integer32;

MATCHES FOR				EQUALITY, ORDERING;

BEHAVIOUR 				bMPCPTransmitElapsed;

REGISTERED AS		{iso(1) std(0) iso8802(8802) csma(3) csmacdmgt(30) attribute(7) mpcpTransmitElapsed(285)};

bMPCPTransmitElapsed				BEHAVIOUR

DEFINED AS 		See “BEHAVIOUR DEFINED AS” in 30.3.5.1.19;

aMPCPReceiveElapsed					ATTRIBUTE

WITH ATTRIBUTE SYNTAX 				IEEE802Dot3-MgmtAttributeModule.Integer32;

MATCHES FOR				EQUALITY, ORDERING;

BEHAVIOUR 				bMPCPReceiveElapsed;

REGISTERED AS		{iso(1) std(0) iso8802(8802) csma(3) csmacdmgt(30) attribute(7) mpcpReceiveElapsed(286)};

bMPCPReceiveElapsed				BEHAVIOUR

DEFINED AS 		See “BEHAVIOUR DEFINED AS” in 30.3.5.1.20;

aMPCPRoundTripTime					ATTRIBUTE

WITH ATTRIBUTE SYNTAX 				IEEE802Dot3-MgmtAttributeModule.Integer16;

MATCHES FOR				EQUALITY, ORDERING;

BEHAVIOUR 				bMPCPRoundTripTime;

REGISTERED AS		{iso(1) std(0) iso8802(8802) csma(3) csmacdmgt(30) attribute(7) mpcpRoundTripTime(287)};

bMPCPRoundTripTime				BEHAVIOUR

DEFINED AS 		See “BEHAVIOUR DEFINED AS” in 30.3.5.1.21;

aMPCPDiscoveryWindowsSent					ATTRIBUTE

DERIVED FROM				aCMCounter;

MATCHES FOR				EQUALITY, ORDERING;

BEHAVIOUR 				bMPCPDiscoveryWindowsSent;

REGISTERED AS		{iso(1) std(0) iso8802(8802) csma(3) csmacdmgt(30) attribute(7) mpcpDiscoveryWindowsSent(288)};

bMPCPDiscoveryWindowsSent				BEHAVIOUR

DEFINED AS 		See “BEHAVIOUR DEFINED AS” in 30.3.5.1.22;

aMPCPDiscoveryTimeout					ATTRIBUTE

DERIVED FROM				aCMCounter;

MATCHES FOR				EQUALITY, ORDERING;

BEHAVIOUR 				bMPCPDiscoveryTimeout;

REGISTERED AS		{iso(1) std(0) iso8802(8802) csma(3) csmacdmgt(30) attribute(7) mpcpDiscoveryTimeout(290)};

bMPCPDiscoveryTimeout				BEHAVIOUR

DEFINED AS 		See “BEHAVIOUR DEFINED AS” in 30.3.5.1.23;

aMPCPMaximumPendingGrants					ATTRIBUTE

WITH ATTRIBUTE SYNTAX 				IEEE802Dot3-MgmtAttributeModule.Integer8;

MATCHES FOR				EQUALITY, ORDERING;

BEHAVIOUR 				bMPCPMaximumPendingGrants;

REGISTERED AS		{iso(1) std(0) iso8802(8802) csma(3) csmacdmgt(30) attribute(7) mpcpMaximumPendingGrants(291)};

bMPCPMaximumPendingGrants					BEHAVIOUR

DEFINED AS 		See “BEHAVIOUR DEFINED AS” in 30.3.5.1.24;

acMPCPAdminControl 						 ACTION

BEHAVIOUR	bMPCPAdminControl;

MODE 	CONFIRMED;

WITH INFORMATION SYNTAX	IEEE802Dot3-MgmtAttributeModule.AdminState;

REGISTERED AS	{iso(1) std(0) iso8802(8802) csma(3) csmacdmgt(30) action(9) mpcpAdminCtrl(16)};

bMPCPAdminControl 						 BEHAVIOUR

DEFINED AS	See “BEHAVIOUR DEFINED AS” in 30.3.5.2.1;

- DTE MAC Control function entity attributes

aPAUSELinkDelayAllowance					ATTRIBUTE

WITH ATTRIBUTE SYNTAX	IEEE802Dot3-MgmtAttributeModule.LinkDelayAllowance;

BEHAVIOUR	bPAUSELinkDelayAllowance;

REGISTERED AS	{iso(1) std(0) iso8802(8802) csma(3)
	csmacdmgt(30) attribute(7) aPAUSELinkDelayAllowance(97)};

bPAUSELinkDelayAllowance					BEHAVIOUR

DEFINED AS	See “BEHAVIOUR DEFINED AS” in 30.3.4.1;

aPAUSEMACCtrlFramesTransmitted					ATTRIBUTE

DERIVED FROM	aCMCounter;

BEHAVIOUR	bPAUSEMACCtrlFramesTransmitted;

REGISTERED AS	{iso(1) std(0) iso8802(8802) csma(3)
	csmacdmgt(30) attribute(7) aPAUSEMACCtrlFramesTransmitted(98)};

bPAUSEMACCtrlFramesTransmitted					BEHAVIOUR

DEFINED AS	See “BEHAVIOUR DEFINED AS” in 30.3.4.2;

aPAUSEMACCtrlFramesReceived					ATTRIBUTE

DERIVED FROM	aCMCounter;

BEHAVIOUR	bPAUSEMACCtrlFramesReceived;

REGISTERED AS	{iso(1) std(0) iso8802(8802) csma(3)
	csmacdmgt(30) attribute(7) aPAUSEMACCtrlFramesReceived(99)};

bPAUSEMACCtrlFramesReceived					 BEHAVIOUR

DEFINED AS	See “BEHAVIOUR DEFINED AS” in 30.3.4.3;

- Repeater managed object class

- Repeater, formal definition

oRepeater	MANAGED OBJECT CLASS

DERIVED FROM	'CCITT Rec. X.721 (1992) | ISO/IEC 10165-2 1992':top;

CHARACTERIZED BY

pRepeaterBasicControl	PACKAGE

ATTRIBUTES	aRepeaterID	GET,
aRepeaterType		GET,
aRepeaterGroupCapacity		GET,
aGroupMap		GET,
aRepeaterHealthState		GET,
aRepeaterHealthText		GET,
aRepeaterHealthData		GET;

ACTIONS	acResetRepeater,
acExecuteNonDisruptiveSelfTest;

NOTIFICATIONS	nRepeaterHealth,
nRepeaterReset,
nGroupMapChange;

;

;

CONDITIONAL PACKAGES

pRepeaterPerfMonitor	PACKAGE

ATTRIBUTES	aTransmitCollisions	GET;

REGISTERED AS	{iso(1) std(0) iso8802(8802) csma(3) csmacdmgt(30) package(4) 
repeaterPerfMonitorPkg(9)};

PRESENT IF	The Performance Monitor Capability is implemented.;

REGISTERED AS	{iso(1) std(0) iso8802(8802) csma(3) csmacdmgt(30) managedObjectClass(3) repeaterObjectClass(3)};

nbRepeaterName	NAME BINDING

SUBORDINATE OBJECT CLASS	oRepeater;

NAMED BY SUPERIOR OBJECT CLASS
“ISO/IEC 10165-2”:system AND SUBCLASSES;

WITH ATTRIBUTE	aRepeaterID;

REGISTERED AS	{iso(1) std(0) iso8802(8802) csma(3) csmacdmgt(30) nameBinding(6) repeaterName(5)};

nbRepeaterMonitor	NAME BINDING

SUBORDINATE OBJECT CLASS	“IEEE802.1F”:oEWMAMetricMonitor;

NAMED BY SUPERIOR OBJECT CLASS
“ISO/IEC 10165-2”:system  AND SUBCLASSES;

WITH ATTRIBUTE	“IEEE802.1F”:aScannerId;

CREATE	WITH-AUTOMATIC-INSTANCE-NAMING;

DELETE	ONLY-IF-NO-CONTAINED-OBJECTS;

REGISTERED AS	{iso(1) std(0) iso8802(8802) csma(3) csmacdmgt(30) nameBinding(6) repeaterMonitor(6)};

- Repeater attributes

aRepeaterID 	ATTRIBUTE

WITH ATTRIBUTE SYNTAX	IEEE802Dot3-MgmtAttributeModule.OneOfName;

MATCHES FOR	EQUALITY;

BEHAVIOUR	bRepeaterID;

REGISTERED AS	{iso(1) std(0) iso8802(8802) csma(3) csmacdmgt(30) attribute(7) repeaterID(38)};

bRepeaterID     	BEHAVIOUR

DEFINED AS	See “BEHAVIOUR DEFINED AS” in 30.4.1.1.1.;

aRepeaterType     	ATTRIBUTE

WITH ATTRIBUTE SYNTAX	IEEE802Dot3-MgmtAttributeModule.RepeaterType;

MATCHES FOR	EQUALITY;

BEHAVIOUR	bRepeaterType;

REGISTERED AS	{iso(1) std(0) iso8802(8802) csma(3) csmacdmgt(30) attribute(7) repeaterType (39)};

bRepeaterType    	BEHAVIOUR

DEFINED AS	See “BEHAVIOUR DEFINED AS” in 30.4.1.1.2.;

aRepeaterGroupCapacity	 ATTRIBUTE

WITH ATTRIBUTE SYNTAX	IEEE802Dot3-MgmtAttributeModule.OneOfName;

MATCHES FOR	EQUALITY, ORDERING;

BEHAVIOUR	bRepeaterGroupCapacity;

REGISTERED AS	{iso(1) std(0) iso8802(8802) csma(3) csmacdmgt(30) attribute(7) repeaterGroupCapacity(40)};

bRepeaterGroupCapacity	 BEHAVIOUR

DEFINED AS	See “BEHAVIOUR DEFINED AS” in 30.4.1.1.3.;

aGroupMap	ATTRIBUTE

WITH ATTRIBUTE SYNTAX	IEEE802Dot3-MgmtAttributeModule.BitString;

MATCHES FOR	EQUALITY;

BEHAVIOUR	bGroupMap;

REGISTERED AS	{iso(1) std(0) iso8802(8802) csma(3) csmacdmgt(30) attribute(7) groupMap(41)};

bGroupMap     	BEHAVIOUR

DEFINED AS	See “BEHAVIOUR DEFINED AS” in 30.4.1.1.4.;

aRepeaterHealthState    	ATTRIBUTE

WITH ATTRIBUTE SYNTAX	IEEE802Dot3-MgmtAttributeModule.
RepeaterHealthState;

MATCHES FOR	EQUALITY;

BEHAVIOUR	bRepeaterHealthState;

REGISTERED AS	{iso(1) std(0) iso8802(8802) csma(3) csmacdmgt(30) attribute(7) repeaterHealthState(42)};

bRepeaterHealthState   	 BEHAVIOUR

DEFINED AS	See “BEHAVIOUR DEFINED AS” in 30.4.1.1.5.;

aRepeaterHealthText    	ATTRIBUTE

WITH ATTRIBUTE SYNTAX	IEEE802Dot3-MgmtAttributeModule.
RepeaterHealthText;

MATCHES FOR	EQUALITY;

BEHAVIOUR	bRepeaterHealthText;

REGISTERED AS	{iso(1) std(0) iso8802(8802) csma(3) csmacdmgt(30) attribute(7) repeaterHealthText(43)};

bRepeaterHealthText   	 BEHAVIOUR

DEFINED AS	See “BEHAVIOUR DEFINED AS” in 30.4.1.1.6.;

aRepeaterHealthData    	ATTRIBUTE

WITH ATTRIBUTE SYNTAX	IEEE802Dot3-MgmtAttributeModule.
RepeaterHealthData;

MATCHES FOR	EQUALITY;

BEHAVIOUR	bRepeaterHealthData;

REGISTERED AS	{iso(1) std(0) iso8802(8802) csma(3) csmacdmgt(30) attribute(7) repeaterHealthData(44)};

bRepeaterHealthData   	 BEHAVIOUR

DEFINED AS	See “BEHAVIOUR DEFINED AS” in 30.4.1.1.7.;

aTransmitCollisions	 ATTRIBUTE

DERIVED FROM	aCMCounter;

BEHAVIOUR	bTransmitCollisions;

REGISTERED AS	{iso(1) std(0) iso8802(8802) csma(3) csmacdmgt(30) attribute(7) transmitCollisions (45)};

bTransmitCollisions 	 BEHAVIOUR

DEFINED AS	See “BEHAVIOUR DEFINED AS” in 30.4.1.1.8

NOTE—The approximate minimum time for counter rollover for 10 Mb/s operation is 16 h.;

- Repeater actions

acResetRepeater    	ACTION

BEHAVIOUR	bResetRepeater;

MODE	CONFIRMED;

REGISTERED AS	{iso(1) std(0) iso8802(8802) csma(3) csmacdmgt(30) action(9) resetRepeater(6)};

bResetRepeater    	BEHAVIOUR

DEFINED AS	See “BEHAVIOUR DEFINED AS” in 30.4.1.2.1.;

acExecuteNonDisruptiveSelfTest  	ACTION

BEHAVIOUR	bExecuteNonDisruptiveSelfTest;

REGISTERED AS	{iso(1) std(0) iso8802(8802) csma(3) csmacdmgt(30) action(9) executeNonDisruptiveSelfTestAction(7)};

bExecuteNonDisruptiveSelfTest 	 BEHAVIOUR

DEFINED AS	See “BEHAVIOUR DEFINED AS” in 30.4.1.2.2.;

- Repeater notifications

nRepeaterHealth   	 NOTIFICATION

BEHAVIOUR	bRepeaterHealth;

WITH INFORMATION SYNTAX	IEEE802Dot3-MgmtAttributeModule.
RepeaterHealthInfo

AND ATTRIBUTE IDS	repeaterHealthState aRepeaterHealthState,
repeaterHealthText aRepeaterHealthText,
repeaterHealthData aRepeaterHealthData

;

REGISTERED AS	{iso(1) std(0) iso8802(8802) csma(3) csmacdmgt(30) notification(10) repeaterHealth(1)};

bRepeaterHealth   	 BEHAVIOUR

DEFINED AS	See “BEHAVIOUR DEFINED AS” in 30.4.1.3.1.;

nRepeaterReset  	 NOTIFICATION

BEHAVIOUR	bRepeaterReset;

WITH INFORMATION SYNTAX	IEEE802Dot3-MgmtAttributeModule.
RepeaterHealthInfo

AND ATTRIBUTE IDS	repeaterHealthState aRepeaterHealthState,
repeaterHealthText aRepeaterHealthText,
repeaterHealthData aRepeaterHealthData

;

REGISTERED AS	{iso(1) std(0) iso8802(8802) csma(3) csmacdmgt(30) notification(10) repeaterReset(2)};

bRepeaterReset   	 BEHAVIOUR

DEFINED AS	See “BEHAVIOUR DEFINED AS” in 30.4.1.3.2.;

nGroupMapChange	 NOTIFICATION

BEHAVIOUR	bGroupMapChange;

WITH INFORMATION SYNTAX	IEEE802Dot3-MgmtAttributeModule.BitString;

REGISTERED AS	{iso(1) std(0) iso8802(8802) csma(3) csmacdmgt(30) notification(10) groupMapChange(3)};

bGroupMapChange   	 BEHAVIOUR

DEFINED AS	See “BEHAVIOUR DEFINED AS” in 30.4.1.3.3.;

- Group managed object class

- Group, formal definition

oGroup	MANAGED OBJECT CLASS

DERIVED FROM	'CCITT Rec. X.721 (1992) | ISO/IEC 10165-2 1992':top;

CHARACTERIZED BY

pGroupBasicControl	PACKAGE

ATTRIBUTES	aGroupID	GET,
	aGroupPortCapacity	GET,
	aPortMap	GET;

NOTIFICATIONS	nPortMapChange;

;

;

REGISTERED AS	{iso(1) std(0) iso8802(8802) csma(3) csmacdmgt(30) managedObjectClass(3) groupObjectClass(4)};

nbGroupName	NAME BINDING

SUBORDINATE OBJECT CLASS	oGroup;

NAMED BY SUPERIOR OBJECT CLASS
oRepeater  AND SUBCLASSES;

WITH ATTRIBUTE	aGroupID;

REGISTERED AS	{iso(1) std(0) iso8802(8802) csma(3) csmacdmgt(30) nameBinding(6) groupName(7)};

- Group attributes

aGroupID    						 ATTRIBUTE

WITH ATTRIBUTE SYNTAX	IEEE802Dot3-MgmtAttributeModule.OneOfName;

MATCHES FOR	EQUALITY;

BEHAVIOUR	bGroupID;

REGISTERED AS	{iso(1) std(0) iso8802(8802) csma(3) csmacdmgt(30) attribute(7) groupID(46)};

bGroupID    						 BEHAVIOUR

DEFINED AS	See “BEHAVIOUR DEFINED AS” in 30.4.2.1.1;

aGroupPortCapacity   						 ATTRIBUTE

WITH ATTRIBUTE SYNTAX	IEEE802Dot3-MgmtAttributeModule.OneOfName;

MATCHES FOR	EQUALITY, ORDERING;

BEHAVIOUR	bGroupPortCapacity;

REGISTERED AS	{iso(1) std(0) iso8802(8802) csma(3) csmacdmgt(30) attribute(7) groupPortCapacity(47)};

bGroupPortCapacity    						BEHAVIOUR

DEFINED AS	See “BEHAVIOUR DEFINED AS” in 30.4.2.1.2;

aPortMap   						 ATTRIBUTE

WITH ATTRIBUTE SYNTAX	IEEE802Dot3-MgmtAttributeModule.BitString;

MATCHES FOR	EQUALITY;

BEHAVIOUR	bPortMap;

REGISTERED AS	{iso(1) std(0) iso8802(8802) csma(3) csmacdmgt(30) attribute(7) portMap(48)};

bPortMap  						 BEHAVIOUR

DEFINED AS	See “BEHAVIOUR DEFINED AS” in 30.4.2.1.3;

- Group notifications

nPortMapChange   						 NOTIFICATION

BEHAVIOUR	bPortMapChange;

WITH INFORMATION SYNTAX	IEEE802Dot3-MgmtAttributeModule.BitString;

REGISTERED AS	{iso(1) std(0) iso8802(8802) csma(3) csmacdmgt(30) notification(10) portMapChange(4)};

bPortMapChange  						 BEHAVIOUR

DEFINED AS	See “BEHAVIOUR DEFINED AS” in 30.4.2.2.1;

- Repeater port managed object class

- Port, formal definition

oRepeaterPort	MANAGED OBJECT CLASS

DERIVED FROM	'CCITT Rec. X.721 (1992) | ISO/IEC 10165-2 1992':top;

CHARACTERIZED BY

pPortBasicControl	PACKAGE

ATTRIBUTES	aPortID	GET,
	aPortAdminState	GET,
	aAutoPartitionState	GET;

ACTIONS	acPortAdminControl;

;

;

CONDITIONAL PACKAGES

pPortPerfMonitor	PACKAGE

ATTRIBUTES	aReadableFrames	GET,
	aReadableOctets	GET,
	aFrameCheckSequenceErrors	GET,
	aAlignmentErrors	GET,
	aFramesTooLong	GET,
	aShortEvents	GET,
	aRunts	GET,
	aCollisions	GET,
	aLateEvents	GET,
	aVeryLongEvents	GET,
	aDataRateMismatches	GET,
	aAutoPartitions	GET;

REGISTERED AS	{iso(1) std(0) iso8802(8802) csma(3) csmacdmgt(30) package(4) 
portPerfMonitorPkg(10)};

PRESENT IF	The Performance Monitor Capability is implemented.,

pPortAddrTracking	PACKAGE

ATTRIBUTES	aLastSourceAddress	GET,
	aSourceAddressChanges	GET;

REGISTERED AS	{iso(1) std(0) iso8802(8802) csma(3)
csmacdmgt(30) package(4) portAddrTrackPkg(11)};

PRESENT IF	The Address Tracking and Performance Monitor capabilities are implemented.,

p100MbpsMonitor	PACKAGE

ATTRIBUTES	aIsolates	GET,

aSymbolErrorDuringPacket	GET;

REGISTERED AS	{iso(1) std(0) iso8802(8802) csma(3) 
csmacdmgt(30) package(4) 
port100MbpsMonitor(12)};

PRESENT IF	The 100/1000 Mb/s Monitor capability is implemented,

pBurst	PACKAGE

ATTRIBUTES	aBursts	GET;

REGISTERED AS	{iso(1) std(0) iso8802(8802) csma(3) 
csmacdmgt(30) package(4) 
portBurst(18)};

PRESENT IF	The 1000 Mb/s Burst Monitor capability is implemented;

REGISTERED AS	{iso(1) std(0) iso8802(8802) csma(3) csmacdmgt(30) managedObjectClass(3) repeaterPortObjectClass(5)};

nbPortName	NAME BINDING

SUBORDINATE OBJECT CLASS	oRepeaterPort;

NAMED BY SUPERIOR OBJECT CLASS
oGroup AND SUBCLASSES;

WITH ATTRIBUTE	aPortID;

REGISTERED AS	{iso(1) std(0) iso8802(8802) csma(3) csmacdmgt(30) nameBinding(6) portName(8)};

- Port attributes

aPortID      						ATTRIBUTE

WITH ATTRIBUTE SYNTAX	IEEE802Dot3-MgmtAttributeModule.OneOfName;

BEHAVIOUR	bPortID;

REGISTERED AS 	{iso(1) std(0) iso8802(8802) csma(3) csmacdmgt(30) attribute(7) portID(49)};

bPortID  						 BEHAVIOUR

DEFINED AS	See “BEHAVIOUR DEFINED AS” in 30.4.3.1.1;

aPortAdminState 						 ATTRIBUTE

WITH ATTRIBUTE SYNTAX	IEEE802Dot3-MgmtAttributeModule.
PortAdminState;

MATCHES FOR	EQUALITY;

BEHAVIOUR	bPortAdminState;

REGISTERED AS	{iso(1) std(0) iso8802(8802) csma(3) csmacdmgt(30) attribute(7) portAdminState(50)};

bPortAdminState 						 BEHAVIOUR

DEFINED AS	See “BEHAVIOUR DEFINED AS” in 30.4.3.1.2;

aAutoPartitionState  						 ATTRIBUTE

WITH ATTRIBUTE SYNTAX	IEEE802Dot3-MgmtAttributeModule.
AutoPartitionState;

MATCHES FOR	EQUALITY;

BEHAVIOUR	bAutoPartition;

REGISTERED AS	{iso(1) std(0) iso8802(8802) csma(3) csmacdmgt(30) attribute(7) autoPartitionState(51)};

bAutoPartition  						 BEHAVIOUR

DEFINED AS	See “BEHAVIOUR DEFINED AS” in 30.4.3.1.3;

aReadableFrames  						 ATTRIBUTE

DERIVED FROM	aCMCounter;

BEHAVIOUR	bReadableFrames;

REGISTERED AS	{iso(1) std(0) iso8802(8802) csma(3) csmacdmgt(30) attribute(7) readableFrames(52)};

bReadableFrames 						 BEHAVIOUR

DEFINED AS	See “BEHAVIOUR DEFINED AS” in 30.4.3.1.4

- The approximate minimum time between counter rollovers for 10 Mb/s operation is 80 h.;

aReadableOctets 						 ATTRIBUTE

DERIVED FROM	aCMCounter;

BEHAVIOUR	bReadableOctets;

REGISTERED AS	{iso(1) std(0) iso8802(8802) csma(3) csmacdmgt(30) attribute(7) readableOctets(53)};

bReadableOctets						 BEHAVIOUR

DEFINED AS	See “BEHAVIOUR DEFINED AS” in 30.4.3.1.5

- The approximate minimum time between counter rollovers for 10 Mb/s operation is 58 min.;

aFrameCheckSequenceErrors 						 ATTRIBUTE

DERIVED FROM	aCMCounter;

BEHAVIOUR	bFCSErrors;

REGISTERED AS	{iso(1) std(0) iso8802(8802) csma(3) csmacdmgt(30) attribute(7) frameCheckSequenceErrors(54)};

bFCSErrors    						 BEHAVIOUR

DEFINED AS	See “BEHAVIOUR DEFINED AS” in 30.4.3.1.6

- The approximate minimum time between counter rollovers for 10 Mb/s operation is 80 h.;

aAlignmentErrors  						 ATTRIBUTE

DERIVED FROM	aCMCounter;

BEHAVIOUR	bAlignmentErrors;

REGISTERED AS	{iso(1) std(0) iso8802(8802) csma(3) csmacdmgt(30) attribute(7) alignmentErrors(55)};

bAlignmentErrors  						 BEHAVIOUR

DEFINED AS	See “BEHAVIOUR DEFINED AS” in 30.4.3.1.7

- The approximate minimum time between counter rollovers for 10 Mb/s operation is 80 h.;

aFramesTooLong  						 ATTRIBUTE

DERIVED FROM	aCMCounter;

BEHAVIOUR	bFramesTooLong;

REGISTERED AS	{iso(1) std(0) iso8802(8802) csma(3) csmacdmgt(30) attribute(7) framesTooLong(56)};

bFramesTooLong 						 BEHAVIOUR

DEFINED AS	See “BEHAVIOUR DEFINED AS” in 30.4.3.1.8

- The approximate minimum time between counter rollovers for 10 Mb/s operation is 61 days.;

aShortEvents						 ATTRIBUTE

DERIVED FROM	aCMCounter;

BEHAVIOUR	bShortEvents;

REGISTERED AS	{iso(1) std(0) iso8802(8802) csma(3) csmacdmgt(30) attribute(7) shortEvents(57)};

bShortEvents  						 BEHAVIOUR

DEFINED AS	See “BEHAVIOUR DEFINED AS” in 30.4.3.1.9

- The approximate minimum time between counter rollovers for 10 Mb/s operation is 16 hours;

aRunts  						 ATTRIBUTE

DERIVED FROM	aCMCounter;

BEHAVIOUR	bRunts;

REGISTERED AS	{iso(1) std(0) iso8802(8802) csma(3) csmacdmgt(30) attribute(7) runts(58)};

bRunts     						 BEHAVIOUR

DEFINED AS	See “BEHAVIOUR DEFINED AS” in 30.4.3.1.10

- The approximate minimum time for counter rollover for 10 Mb/s operation is 16 h.;

aCollisions     						ATTRIBUTE

DERIVED FROM	aCMCounter;

BEHAVIOUR	bCollisions;

REGISTERED AS	{iso(1) std(0) iso8802(8802) csma(3) csmacdmgt(30) attribute(7) collisions(59)};

bCollisions    						 BEHAVIOUR

DEFINED AS	See “BEHAVIOUR DEFINED AS” in 30.4.3.1.11

- The approximate minimum time for counter rollover for 10 Mb/s operation is 16 h.;

aLateEvents    						 ATTRIBUTE

DERIVED FROM	aCMCounter;

BEHAVIOUR	bLateEvents;

REGISTERED AS	{iso(1) std(0) iso8802(8802) csma(3) csmacdmgt(30) attribute(7) lateEvents(60)};

bLateEvents     						BEHAVIOUR

DEFINED AS	See “BEHAVIOUR DEFINED AS” in 30.4.3.1.12

- The approximate minimum time between counter rollovers for 10 Mb/s operation is 81 h.;

aVeryLongEvents   						 ATTRIBUTE

DERIVED FROM	aCMCounter;

BEHAVIOUR	bVeryLongEvents;

REGISTERED AS	{iso(1) std(0) iso8802(8802) csma(3) csmacdmgt(30) attribute(7) veryLongEvents(61)};

bVeryLongEvents    						BEHAVIOUR

DEFINED AS	See “BEHAVIOUR DEFINED AS” in 30.4.3.1.13

- The approximate minimum time between counter rollovers for 10 Mb/s operation is 198 days.;

aDataRateMismatches   						 ATTRIBUTE

DERIVED FROM	aCMCounter;

BEHAVIOUR	bDataRateMismatches;

REGISTERED AS	{iso(1) std(0) iso8802(8802) csma(3) csmacdmgt(30) attribute(7) dataRateMismatches(62)};

bDataRateMismatches    						BEHAVIOUR

DEFINED AS	See “BEHAVIOUR DEFINED AS” in 30.4.3.1.14;

aAutoPartitions    						ATTRIBUTE

DERIVED FROM	aCMCounter;

BEHAVIOUR	bAutoPartitions;

REGISTERED AS	{iso(1) std(0) iso8802(8802) csma(3) csmacdmgt(30) attribute(7) autoPartitions(63)};

bAutoPartitions    						BEHAVIOUR

DEFINED AS	See “BEHAVIOUR DEFINED AS” in 30.4.3.1.15;

aIsolates     						ATTRIBUTE

DERIVED FROM	aCMCounter;

BEHAVIOUR	bIsolates;

REGISTERED AS	{iso(1) std(0) iso8802(8802) csma(3) csmacdmgt(30) attribute(7) isolates(64)};

bIsolates   						 BEHAVIOUR

DEFINED AS	See “BEHAVIOUR DEFINED AS” in 30.4.3.1.16;

aSymbolErrorDuringPacket   						ATTRIBUTE

DERIVED FROM	aCMCounter;

BEHAVIOUR	bSymbolErrorDuringPacket;

REGISTERED AS	{iso(1) std(0) iso8802(8802) csma(3) csmacdmgt(30) attribute(7) symbolErrorDuringPacket(65)};

bSymbolErrorDuringPacket  						 BEHAVIOUR

DEFINED AS	See “BEHAVIOUR DEFINED AS” in 30.4.3.1.17;

aLastSourceAddress   						 ATTRIBUTE

WITH ATTRIBUTE SYNTAX	IEEE802CommonDefinitions.MACAddress;

MATCHES FOR	EQUALITY;

BEHAVIOUR	bLastSourceAddress;

REGISTERED AS	{iso(1) std(0) iso8802(8802) csma(3) csmacdmgt(30) attribute(7) lastSourceAddress(66)};

bLastSourceAddress 						 BEHAVIOUR

DEFINED AS	See “BEHAVIOUR DEFINED AS” in 30.4.3.1.18;

aSourceAddressChanges  						 ATTRIBUTE

DERIVED FROM	aCMCounter;

BEHAVIOUR	bSourceAddressChanges;

REGISTERED AS	{iso(1) std(0) iso8802(8802) csma(3) csmacdmgt(30) attribute(7) sourceAddressChanges(67)};

bSourceAddressChanges						 BEHAVIOUR

DEFINED AS	See “BEHAVIOUR DEFINED AS” in 30.4.3.1.19

- The approximate minimum time for counter rollover for 10 Mb/s operation is 81 h.;

aBursts						ATTRIBUTE

DERIVED FROM	aCMCounter;

BEHAVIOUR	bBursts;

REGISTERED AS	{iso(1) std(0) iso8802(8802) csma(3) csmacdmgt(30) attribute(7) bursts(100)};

bBursts						BEHAVIOUR

DEFINED AS	See “BEHAVIOUR DEFINED AS” in 30.4.3.1.20;

- Port actions

acPortAdminControl 						ACTION

BEHAVIOUR	bPortAdminControl;

WITH INFORMATION SYNTAX	IEEE802Dot3-MgmtAttributeModule.
PortAdminState;

REGISTERED AS	{iso(1) std(0) iso8802(8802) csma(3) csmacdmgt(30) action(9) portAdminControl(8)};

bPortAdminControl    						BEHAVIOUR

DEFINED AS	See “BEHAVIOUR DEFINED AS” in 30.4.3.2.1;

- MAU managed object class

- MAU, formal definition

oMAU	MANAGED OBJECT CLASS

DERIVED FROM				'CCITT Rec. X.721 (1992) | ISO/IEC 10165-2 :1992':top;

CHARACTERIZED BY

pMAUBasic			PACKAGE

ATTRIBUTES			aMAUID					GET,

aMAUType					GET-REPLACE,

aMAUTypeList					GET,

aMediaAvailable					GET,

aJabber					GET,

aMAUAdminState					GET;

NOTIFICATIONS			nJabber;

;

;

CONDITIONAL PACKAGES

pMAUControl			PACKAGE

ACTIONS			acResetMAU,

acMAUAdminControl;

REGISTERED AS		{iso(1) std(0) iso8802(8802) csma(3) csmacdmgt(30) package(4) 
mauControlPkg(13)};

PRESENT IF		The pMAUControl package is implemented.,

pMediaLossTracking			PACKAGE

ATTRIBUTES	aLoseMediaCounter	GET;

REGISTERED AS		{iso(1) std(0) iso8802(8802) csma(3) csmacdmgt(30) package(4) mediaLossTrackingPkg(14)};

PRESENT IF		MAU TypeValue = AUI or if the pMediaLossTracking package is implemented.,

pBroadbandDTEMAU			PACKAGE

ATTRIBUTES			aBbMAUXmitRcvSplitType					GET,

aBroadbandFrequencies					GET;

REGISTERED AS		{iso(1) std(0) iso8802(8802) csma(3) csmacdmgt(30) package(4) broadbandMAUPkg(15)};

PRESENT IF		The MAU is of type 10BROAD36.,

p100MbpsMonitor				PACKAGE

ATTRIBUTES				aFalseCarriers				GET,

aIdleErrorCount				GET;

REGISTERED AS		{iso(1) std(0) iso8802(8802) csma(3) csmacdmgt(30) package(4) mau100MbpsMonitor(16)};

PRESENT IF		The MAU is capable of 100 Mb/s operation.,

pPHY10GBTMargin				PACKAGE

ATTRIBUTES				aSNROpMarginChnlA				GET,

aSNROpMarginChnlB				GET,

aSNROpMarginChnlC				GET,

aSNROpMarginChnlD				GET;

REGISTERED AS		{iso(1) std(0) iso8802(8802) csma(3) csmacdmgt(30) package(4) phy10GBTMargin(39)};

PRESENT IF		The MAU is capable of 10GBASE-T operation.,

pPCSMonitor				PACKAGE

ATTRIBUTES				aPCSCodingViolation				GET;

REGISTERED AS		{iso(1) std(0) iso8802(8802) csma(3) csmacdmgt(30) package(4) pcsMonitor(35)};

PRESENT IF		The MAU PCS code error monitoring capability is implemented.,

pFECMonitor				PACKAGE

ATTRIBUTES				aFECAbility				GET,

aFECmode				GET-REPLACE,

aFECCorrectedBlocks				GET,

aFECUncorrectableBlocks				GET;

REGISTERED AS		{iso(1) std(0) iso8802(8802) csma(3) csmacdmgt(30) package(4) fecMonitor(30)};

PRESENT IF		The MAU is capable of FEC operation.;

REGISTERED AS		{iso(1) std(0) iso8802(8802) csma(3) csmacdmgt(30) managedObjectClass(3) mauObjectClass(6)};

nbMAU-repeaterName	NAME BINDING

SUBORDINATE OBJECT CLASS	oMAU;

NAMED BY SUPERIOR OBJECT CLASS			--(of oRepeaterPort)
oRepeaterPort   AND SUBCLASSES;
			--{1.0.8802.3.30.3.5}

WITH ATTRIBUTE	aMAUID;

REGISTERED AS	{iso(1) std(0) iso8802(8802) csma(3) csmacdmgt(30) nameBinding(6) mau-repeaterName(9)};

nbMAU-dteName	NAME BINDING

SUBORDINATE OBJECT CLASS	oMAU;

NAMED BY SUPERIOR OBJECT CLASS			--(of oPHYEntity)
oPHYEntity AND SUBCLASSES;
			--{1.0.8802.3.30.3.2}

WITH ATTRIBUTE	aMAUID;

REGISTERED AS	{iso(1) std(0) iso8802(8802) csma(3) csmacdmgt(30) nameBinding(6) mau-dteName(10)};

- MAU attributes

aMAUID     						ATTRIBUTE

WITH ATTRIBUTE SYNTAX	IEEE802Dot3-MgmtAttributeModule.OneOfName;

MATCHES FOR	EQUALITY;

BEHAVIOUR	bMAUID;

REGISTERED AS	{iso(1) std(0) iso8802(8802) csma(3) csmacdmgt(30) attribute(7) mauID(68)};

bMAUID    						 BEHAVIOUR

DEFINED AS	See “BEHAVIOUR DEFINED AS” in 30.5.1.1.1;

aMAUType    						 ATTRIBUTE

WITH ATTRIBUTE SYNTAX	IEEE802Dot3-MgmtAttributeModule.TypeValue;

MATCHES FOR	EQUALITY, ORDERING;

BEHAVIOUR	bMAUType;

REGISTERED AS	{iso(1) std(0) iso8802(8802) csma(3) csmacdmgt(30) attribute(7) mauType(69)};

bMAUType     						BEHAVIOUR

DEFINED AS	See “BEHAVIOUR DEFINED AS” in 30.5.1.1.2;

aMAUTypeList    						 ATTRIBUTE

WITH ATTRIBUTE SYNTAX	IEEE802Dot3-MgmtAttributeModule.TypeList;

MATCHES FOR	EQUALITY, ORDERING;

BEHAVIOUR	bMAUTypeList;

REGISTERED AS	{iso(1) std(0) iso8802(8802) csma(3) csmacdmgt(30) attribute(7) mauTypeList(70)};

bMAUTypeList    						 BEHAVIOUR

DEFINED AS	See “BEHAVIOUR DEFINED AS” in 30.5.1.1.3;

aMediaAvailable 						 ATTRIBUTE

WITH ATTRIBUTE SYNTAX	IEEE802Dot3-MgmtAttributeModule.
MediaAvailState;

MATCHES FOR	EQUALITY, ORDERING;

BEHAVIOUR	bMediaAvailable;

REGISTERED AS	{iso(1) std(0) iso8802(8802) csma(3) csmacdmgt(30) attribute(7) mauMediaAvailable(71)};

bMediaAvailable   						 BEHAVIOUR

DEFINED AS	See “BEHAVIOUR DEFINED AS” in 30.5.1.1.4;

aLoseMediaCounter   						 ATTRIBUTE

DERIVED FROM	aCMCounter;

MATCHES FOR	EQUALITY, ORDERING;

BEHAVIOUR	bLoseMediaCounter;

REGISTERED AS	{iso(1) std(0) iso8802(8802) csma(3) csmacdmgt(30) attribute(7) mauLoseMediaCounter(72)};

bLoseMediaCounter   						 BEHAVIOUR

DEFINED AS	See “BEHAVIOUR DEFINED AS” in 30.5.1.1.5;

aJabber  						 ATTRIBUTE

WITH ATTRIBUTE SYNTAX	IEEE802Dot3-MgmtAttributeModule.Jabber;

MATCHES FOR	EQUALITY, ORDERING;

BEHAVIOUR	bJabberAttribute;

REGISTERED AS	{iso(1) std(0) iso8802(8802) csma(3) csmacdmgt(30) attribute(7) jabber(73)};

bJabberAttribute   						 BEHAVIOUR

DEFINED AS	See “BEHAVIOUR DEFINED AS” in 30.5.1.1.6;

aMAUAdminState  						 ATTRIBUTE

WITH ATTRIBUTE SYNTAX	IEEE802Dot3-MgmtAttributeModule.AdminState;

MATCHES FOR	EQUALITY, ORDERING;

BEHAVIOUR	bMAUAdminState;

REGISTERED AS	{iso(1) std(0) iso8802(8802) csma(3) csmacdmgt(30) attribute(7) mauAdminState(74)};

bMAUAdminState   						 BEHAVIOUR

DEFINED AS	See “BEHAVIOUR DEFINED AS” in 30.5.1.1.7;

aBbMAUXmitRcvSplitType  						 ATTRIBUTE

WITH ATTRIBUTE SYNTAX	IEEE802Dot3-MgmtAttributeModule.
BbandXmitRcvSplitType;

MATCHES FOR	EQUALITY;

BEHAVIOUR	bBbMAUXmitRcvSplitType;

REGISTERED AS	{iso(1) std(0) iso8802(8802) csma(3) csmacdmgt(30) attribute(7) bBandSplitType(75)};

bBbMAUXmitRcvSplitType  						 BEHAVIOUR

DEFINED AS	See “BEHAVIOUR DEFINED AS” in 30.5.1.1.8;

aBroadbandFrequencies  						 ATTRIBUTE

WITH ATTRIBUTE SYNTAX	IEEE802Dot3-MgmtAttributeModule.
BbandFrequency;

MATCHES FOR	EQUALITY;

BEHAVIOUR	bBroadbandFrequencies;

REGISTERED AS	{iso(1) std(0) iso8802(8802) csma(3) csmacdmgt(30) attribute(7) bBandFrequencies(76)};

bBroadbandFrequencies  						 BEHAVIOUR

DEFINED AS	See “BEHAVIOUR DEFINED AS” in 30.5.1.1.9;

aFalseCarriers    						 ATTRIBUTE

DERIVED FROM	aCMCounter;

MATCHES FOR	EQUALITY, ORDERING;

BEHAVIOUR	bFalseCarriers;

REGISTERED AS	{iso(1) std(0) iso8802(8802) csma(3) csmacdmgt(30) attribute(7) falseCarriers(77)};

bFalseCarriers    						 BEHAVIOUR

DEFINED AS	See “BEHAVIOUR DEFINED AS” in 30.5.1.1.10;

aIdleErrorCount						ATTRIBUTE

WITH ATTRIBUTE SYNTAX	IEEE802Dot3-MgmtAttributeModule.RegisterEight;

BEHAVIOUR	bIdleErrorCount;

REGISTERED AS	{iso(1) std(0) iso8802(8802) csma(3) csmacdmgt(30) attribute(7) idleErrorCount(91)};

bIdleErrorCount						BEHAVIOUR

DEFINED AS	See “BEHAVIOUR DEFINED AS” in 30.5.1.1.11;

aSNROpMarginChnlA 						ATTRIBUTE

WITH ATTRIBUTE SYNTAX					IEEE802Dot3-MgmtAttributeModule.Integer16;

MATCHES FOR					EQUALITY, ORDERING;

BEHAVIOUR					bSNROpMarginChnlA;

REGISTERED AS		{iso(1) std(0) iso8802(8802) csma(3) csmacdmgt(30) attribute(7) sNROpMarginChnlA(353)};

bSNROpMarginChnlA 						 BEHAVIOUR

DEFINED AS			See “BEHAVIOUR DEFINED AS” in 30.5.1.1.17;

aSNROpMarginChnlB 						ATTRIBUTE

WITH ATTRIBUTE SYNTAX					IEEE802Dot3-MgmtAttributeModule.Integer16;

MATCHES FOR					EQUALITY, ORDERING;

BEHAVIOUR					bSNROpMarginChnlB;

REGISTERED AS		{iso(1) std(0) iso8802(8802) csma(3) csmacdmgt(30) attribute(7) sNROpMarginChnlB(354)};

bSNROpMarginChnlB 						 BEHAVIOUR

DEFINED AS			See “BEHAVIOUR DEFINED AS” in 30.5.1.1.18;

aSNROpMarginChnlC 						ATTRIBUTE

WITH ATTRIBUTE SYNTAX					IEEE802Dot3-MgmtAttributeModule.Integer16;

MATCHES FOR					EQUALITY, ORDERING;

BEHAVIOUR					bSNROpMarginChnlC;

REGISTERED AS		{iso(1) std(0) iso8802(8802) csma(3) csmacdmgt(30) attribute(7) sNROpMarginChnlC(355)};

bSNROpMarginChnlC 						 BEHAVIOUR

DEFINED AS			See “BEHAVIOUR DEFINED AS” in 30.5.1.1.19;

aSNROpMarginChnlD 						ATTRIBUTE

WITH ATTRIBUTE SYNTAX					IEEE802Dot3-MgmtAttributeModule.Integer16;

MATCHES FOR					EQUALITY, ORDERING;

BEHAVIOUR					bSNROpMarginChnlD;

REGISTERED AS		{iso(1) std(0) iso8802(8802) csma(3) csmacdmgt(30) attribute(7) sNROpMarginChnlD(356)};

bSNROpMarginChnlD 						 BEHAVIOUR

DEFINED AS			See “BEHAVIOUR DEFINED AS” in 30.5.1.1.20;

aPCSCodingViolation					ATTRIBUTE

DERIVED FROM				aCMCounter;

MATCHES FOR				EQUALITY, ORDERING;

BEHAVIOUR 				bPCSCodingViolation;

REGISTERED AS		{iso(1) std(0) iso8802(8802) csma(3) csmacdmgt(30) attribute(7) pcsCodingViolation(292)};

bPCSCodingViolation				BEHAVIOUR

DEFINED AS 		See “BEHAVIOUR DEFINED AS” in 30.5.1.1.12;

aFECAbility					ATTRIBUTE

WITH ATTRIBUTE SYNTAX				IEEE802Dot3-MgmtAttributeModule.FECMode;

MATCHES FOR				EQUALITY, ORDERING;

BEHAVIOUR 				bFECAbility;

REGISTERED AS		{iso(1) std(0) iso8802(8802) csma(3) csmacdmgt(30) attribute(7) fecAbility(313)};

bFECAbility				BEHAVIOUR

DEFINED AS 		See “BEHAVIOUR DEFINED AS” in 30.5.1.1.13;

aFECmode					ATTRIBUTE

WITH ATTRIBUTE SYNTAX				IEEE802Dot3-MgmtAttributeModule.FECMode;

MATCHES FOR				EQUALITY, ORDERING;

BEHAVIOUR 				bFECmode;

REGISTERED AS		{iso(1) std(0) iso8802(8802) csma(3) csmacdmgt(30) attribute(7) fecMode(314)};

bFECmode				BEHAVIOUR

DEFINED AS 		See “BEHAVIOUR DEFINED AS” in 30.5.1.1.14;

aFECCorrectedBlocks					ATTRIBUTE

DERIVED FROM				aCMCounter;

MATCHES FOR				EQUALITY, ORDERING;

BEHAVIOUR 				bFECCorrectedBlocks;

REGISTERED AS		{iso(1) std(0) iso8802(8802) csma(3) csmacdmgt(30) attribute(7) fecCorrectedBlocks(293)};

bFECCorrectedBlocks				BEHAVIOUR

DEFINED AS 		See “BEHAVIOUR DEFINED AS” in 30.5.1.1.15;

aFECUncorrectableBlocks					ATTRIBUTE

DERIVED FROM				aCMCounter;

MATCHES FOR				EQUALITY, ORDERING;

BEHAVIOUR 				bFECUncorrectableBlocks;

REGISTERED AS		{iso(1) std(0) iso8802(8802) csma(3) csmacdmgt(30) attribute(7) fecUncorrectableBlocks(294)};

bFECUncorrectableBlocks				BEHAVIOUR

DEFINED AS 		See “BEHAVIOUR DEFINED AS” in 30.5.1.1.16;

- MAU actions

acResetMAU   						 ACTION

BEHAVIOUR	bResetMAU;

MODE 	CONFIRMED;

REGISTERED AS	{iso(1) std(0) iso8802(8802) csma(3) csmacdmgt(30) action(9) resetMAU(9)};

bResetMAU     						BEHAVIOUR

DEFINED AS	See “BEHAVIOUR DEFINED AS” in 30.5.1.2.1;

acMAUAdminControl  						 ACTION

BEHAVIOUR	bMAUAdminControl;

MODE 	CONFIRMED;

WITH INFORMATION SYNTAX	IEEE802Dot3-MgmtAttributeModule.AdminState;

REGISTERED AS	{iso(1) std(0) iso8802(8802) csma(3) csmacdmgt(30) action(9) mauAdminCtrl(10)};

bMAUAdminControl    						 BEHAVIOUR

DEFINED AS	See “BEHAVIOUR DEFINED AS” in 30.5.1.2.2;

- MAU notifications

nJabber   						 NOTIFICATION

BEHAVIOUR	bJabberNotification;

WITH INFORMATION SYNTAX	IEEE802Dot3-MgmtAttributeModule.Jabber;

REGISTERED AS	{iso(1) std(0) iso8802(8802) csma(3) csmacdmgt(30) notification(10) jabber(5)};

bJabberNotification						BEHAVIOUR

DEFINED AS	See “BEHAVIOUR DEFINED AS” in 30.5.1.3.1;

- AutoNegotiation managed object class

- AutoNegotiation, formal definition

oAutoNegotiation	MANAGED OBJECT CLASS

DERIVED FROM	'CCITT Rec. X.721 (1992) | ISO/IEC 10165-2 :1992':top;

CHARACTERIZED BY

pAutoNeg	PACKAGE

ATTRIBUTES	aAutoNegID	GET,
	aAutoNegAdminState	GET,
	aAutoNegRemoteSignaling	GET,
	aAutoNegAutoConfig	GET-REPLACE,
	aAutoNegLocalTechnologyAbility	GET,
	aAutoNegAdvertisedTechnologyAbility	GET-REPLACE,
	aAutoNegReceivedTechnologyAbility	GET,
	aAutoNegLocalSelectorAbility	GET,
	aAutoNegAdvertisedSelectorAbility	GET-REPLACE,
	aAutoNegReceivedSelectorAbility	GET;

ACTIONS
acAutoNegRestartAutoConfig,
acAutoNegAdminControl;

;

;

REGISTERED AS	{iso(1) std(0) iso8802(8802) csma(3) csmacdmgt(30) managedObjectClass(3) autoNegObjectClass(7)};

nbAutoNeg-mauName	NAME BINDING

SUBORDINATE OBJECT CLASS	oAutoNegotiation;

NAMED BY SUPERIOR OBJECT CLASS			--(of oMAU)
oMAU   AND SUBCLASSES;
			--{1.0.8802.3.30.3.6}

WITH ATTRIBUTE	aMAUID;

REGISTERED AS	{iso(1) std(0) iso8802(8802) csma(3) csmacdmgt(30) nameBinding(6) autoNeg-mauName(11)};

- Auto-Negotiation attributes

aAutoNegID  						 ATTRIBUTE

WITH ATTRIBUTE SYNTAX	IEEE802Dot3-MgmtAttributeModule.OneOfName;

MATCHES FOR	EQUALITY;

BEHAVIOUR	bAutoNegID;

REGISTERED AS	{iso(1) std(0) iso8802(8802) csma(3) csmacdmgt(30) attribute(7) autoNegID(78)};

bAutoNegID   						 BEHAVIOUR

DEFINED AS	See “BEHAVIOUR DEFINED AS” in 30.6.1.1.1;

aAutoNegAdminState  						 ATTRIBUTE

WITH ATTRIBUTE SYNTAX	IEEE802Dot3-MgmtAttributeModule.
AutoNegAdminState;

MATCHES FOR	EQUALITY;

BEHAVIOUR	bAutoNegAdminState;

REGISTERED AS	{iso(1) std(0) iso8802(8802) csma(3) csmacdmgt(30) attribute(7) autoNegAdminState(79)};

bAutoNegAdminState						 BEHAVIOUR

DEFINED AS	See “BEHAVIOUR DEFINED AS” in 30.6.1.1.2;

aAutoNegRemoteSignaling 						 ATTRIBUTE

WITH ATTRIBUTE SYNTAX	IEEE802Dot3-MgmtAttributeModule.
AutoNegRemoteSignalingDetect;

MATCHES FOR	EQUALITY;

BEHAVIOUR	bAutoNegRemoteSignaling;

REGISTERED AS	{iso(1) std(0) iso8802(8802) csma(3) csmacdmgt(30) attribute(7) autoNegRemoteSignaling(80)};

bAutoNegRemoteSignaling 						 BEHAVIOUR

DEFINED AS	See “BEHAVIOUR DEFINED AS” in 30.6.1.1.3;

aAutoNegAutoConfig  						 ATTRIBUTE

WITH ATTRIBUTE SYNTAX	IEEE802Dot3-MgmtAttributeModule.
AutoNegAutoConfig;

MATCHES FOR	EQUALITY;

BEHAVIOUR	bAutoNegAutoConfig;

REGISTERED AS	{iso(1) std(0) iso8802(8802) csma(3) csmacdmgt(30) attribute(7) autoNegAutoConfig(81)};

bAutoNegAutoConfig  						 BEHAVIOUR

DEFINED AS	See “BEHAVIOUR DEFINED AS” in 30.6.1.1.4;

aAutoNegLocalTechnologyAbility						 ATTRIBUTE

WITH ATTRIBUTE SYNTAX	IEEE802Dot3-MgmtAttributeModule.
AutoNegTechnologyList;

MATCHES FOR	EQUALITY, ORDERING;

BEHAVIOUR	bAutoNegLocalTechnologyAbility;

REGISTERED AS	{iso(1) std(0) iso8802(8802) csma(3) csmacdmgt(30) attribute(7) autoNegLocalTechnologyAbility(82)};

bAutoNegLocalTechnologyAbility						 BEHAVIOUR

DEFINED AS	See “BEHAVIOUR DEFINED AS” in 30.6.1.1.5;

aAutoNegAdvertisedTechnologyAbility						 ATTRIBUTE

WITH ATTRIBUTE SYNTAX	IEEE802Dot3-MgmtAttributeModule.
AutoNegTechnologyList;

MATCHES FOR	EQUALITY, ORDERING;

BEHAVIOUR	 bAutoNegAdvertisedTechnologyAbility;

REGISTERED AS	{iso(1) std(0) iso8802(8802) csma(3) csmacdmgt(30) attribute(7) autoNegAdvertisedTechnologyAbility(83)};

bAutoNegAdvertisedTechnologyAbility 						BEHAVIOUR

DEFINED AS	See “BEHAVIOUR DEFINED AS” in 30.6.1.1.6;

aAutoNegReceivedTechnologyAbility						 ATTRIBUTE

WITH ATTRIBUTE SYNTAX	IEEE802Dot3-MgmtAttributeModule.
AutoNegTechnologyList;

MATCHES FOR	EQUALITY, ORDERING;

BEHAVIOUR	 bAutoNegReceivedTechnologyAbility;

REGISTERED AS	{iso(1) std(0) iso8802(8802) csma(3) csmacdmgt(30) attribute(7) autoNegReceivedTechnologyAbility(84)};

bAutoNegReceivedTechnologyAbility 						BEHAVIOUR

DEFINED AS	See “BEHAVIOUR DEFINED AS” in 30.6.1.1.7;

aAutoNegLocalSelectorAbility  						ATTRIBUTE

WITH ATTRIBUTE SYNTAX	IEEE802Dot3-MgmtAttributeModule.
AutoNegSelectorList;

MATCHES FOR	EQUALITY, ORDERING;

BEHAVIOUR	bAutoNegLocalSelectorAbility;

REGISTERED AS	{iso(1) std(0) iso8802(8802) csma(3) csmacdmgt(30) attribute(7) autoNegLocalSelectorAbility(85)};

bAutoNegLocalSelectorAbility  						BEHAVIOUR

DEFINED AS	See “BEHAVIOUR DEFINED AS” in 30.6.1.1.8;

aAutoNegAdvertisedSelectorAbility ATTRIBUTE

WITH ATTRIBUTE SYNTAX	IEEE802Dot3-MgmtAttributeModule.
AutoNegSelectorList;

MATCHES FOR	EQUALITY, ORDERING;

BEHAVIOUR	bAutoNegAdvertisedSelectorAbility;

REGISTERED AS	{iso(1) std(0) iso8802(8802) csma(3) csmacdmgt(30) attribute(7) autoNegAdvertisedSelectorAbility(86)};

bAutoNegAdvertisedSelectorAbility						 BEHAVIOUR

DEFINED AS	See “BEHAVIOUR DEFINED AS” in 30.6.1.1.9;

aAutoNegReceivedSelectorAbility 						ATTRIBUTE

WITH ATTRIBUTE SYNTAX	IEEE802Dot3-MgmtAttributeModule.
AutoNegSelectorList;

MATCHES FOR	EQUALITY, ORDERING;

BEHAVIOUR	bAutoNegReceivedSelectorAbility;

REGISTERED AS	{iso(1) std(0) iso8802(8802) csma(3) csmacdmgt(30) attribute(7) autoNegReceivedSelectorAbility(87)};

bAutoNegReceivedSelectorAbility BEHAVIOUR

DEFINED AS	See “BEHAVIOUR DEFINED AS” in 30.6.1.1.10;

- AutoNegotiation actions

acAutoNegRestartAutoConfig						 ACTION

BEHAVIOUR	bAutoNegRestartAutoConfig;

MODE 	CONFIRMED;

REGISTERED AS	{iso(1) std(0) iso8802(8802) csma(3) csmacdmgt(30) action(9) autoNegRestartAutoConfig(11)};

bAutoNegRestartAutoConfig						 BEHAVIOUR

DEFINED AS	See “BEHAVIOUR DEFINED AS” in 30.6.1.2.1;

acAutoNegAdminControl						 ACTION

BEHAVIOUR	bAutoNegAdminControl;

MODE 	CONFIRMED;

WITH INFORMATION SYNTAX	IEEE802Dot3-MgmtAttributeModule.
AutoNegAdminState;

REGISTERED AS	{iso(1) std(0) iso8802(8802) csma(3) csmacdmgt(30) action(9) autoNegAdminCtrl(12)};

bAutoNegAdminControl						 BEHAVIOUR

DEFINED AS	See “BEHAVIOUR DEFINED AS” in 30.6.1.2.2;

- ResourceTypeID managed object class

- ResourceTypeID, formal definition

-- Implementation of this managed object in accordance with the definition contained in IEEE Std

-- 802.1F-1993 is a conformance requirement of this standard.

-- NOTE—A single instance of the Resource Type ID managed object exists within the oMACEntity

-- managed object class, a single instance of the Resource Type ID managed object exists within

-- the oRepeater managed object class, and a single instance of the Resource Type ID managed

-- object exists within the oMAU managed object class conditional on the presence of an MII.

-- The managed object itself is contained in IEEE Std 802.1F-1993, therefore only name bindings

-- appear in this standard;

nbResourceTypeID-mac	NAME BINDING

SUBORDINATE OBJECT CLASS	“IEEE802.1F”:oResourceTypeID;

NAMED BY SUPERIOR OBJECT CLASS
oMACEntity;

WITH ATTRIBUTE	“IEEE802.1F”:aResourceTypeIDName;

REGISTERED AS	{iso(1) std(0) iso8802(8802) csma(3) csmacdmgt(30) nameBinding(6) resourceTypeID-mac(12)};

nbResourceTypeID-repeater	NAME BINDING

SUBORDINATE OBJECT CLASS	“IEEE802.1F”:oResourceTypeID;

NAMED BY SUPERIOR OBJECT CLASS
oRepeater AND SUBCLASSES;

WITH ATTRIBUTE	“IEEE802.1F”:aResourceTypeIDName;

REGISTERED AS	{iso(1) std(0) iso8802(8802) csma(3) csmacdmgt(30) nameBinding(6) resourceTypeID-repeater(13)};

nbResourceTypeID-mau	NAME BINDING

SUBORDINATE OBJECT CLASS	“IEEE802.1F”:oResourceTypeID;

NAMED BY SUPERIOR OBJECT CLASS
oMAU AND SUBCLASSES;

WITH ATTRIBUTE	“IEEE802.1F”:aResourceTypeIDName;

REGISTERED AS	{iso(1) std(0) iso8802(8802) csma(3) csmacdmgt(30) nameBinding(6) resourceTypeID-mau(14)};

nbResourceTypeID-midSpan	NAME BINDING

SUBORDINATE OBJECT CLASS	“IEEE802.1F”:oResourceTypeID;

NAMED BY SUPERIOR OBJECT CLASS	oMidSpan AND SUBCLASSES;

WITH ATTRIBUTE	aMidSpanID;

REGISTERED AS	{iso(1) std(0) iso8802(8802) csma(3) csmacdmgt(30) nameBinding(6) resourceTypeID-midSpan(25)};

- Aggregator managed object class

-- NOTE—This managed object class has been deprecated by IEEE Std 802.1AX-200X.

- Aggregator, formal definition

oAggregator					MANAGED OBJECT CLASS

DERIVED FROM				“CCITT Rec. X.721 (1992) | ISO/IEC 10165-2 :1992”:top;

CHARACTERIZED BY

pAggregatorBasic			PACKAGE

ATTRIBUTES		aAggID					GET;

;

;

CONDITIONAL PACKAGES

pAggregatorMandatory			PACKAGE

ATTRIBUTES		aAggDescription					GET,

aAggName					GET-REPLACE,

aAggActorSystemID					GET-REPLACE,

aAggActorSystemPriority					GET-REPLACE,

aAggAggregateOrIndividual					GET,

aAggActorAdminKey 					GET-REPLACE,

aAggActorOperKey 					GET,

aAggMACAddress 					GET,

aAggPartnerSystemID 					GET,

aAggPartnerSystemPriority					GET,

aAggPartnerOperKey 					GET,

aAggAdminState 					GET-REPLACE,

aAggOperState 					GET,

aAggTimeOfLastOperChange 					GET,

aAggDataRate 					GET,

aAggFramesTxOK 					GET,

aAggFramesRxOK 					GET,

aAggLinkUpDownNotificationEnable 					GET-REPLACE,

aAggCollectorMaxDelay 					GET-REPLACE;

NOTIFICATIONS

nAggLinkUpNotification,

nAggLinkDownNotification;

REGISTERED AS

{iso(1) std(0) iso8802(8802) csma(3) csmacdmgt(30) package(4) pAggregatorMandatory(19)};

PRESENT IF		Conformance to Link Aggregation management is desired,

pAggregatorRecommended	PACKAGE

ATTRIBUTES		aAggOctetsTxOK 					GET,

aAggOctetsRxOK 					GET,

aAggFramesDiscardedOnTx 					GET,

aAggFramesDiscardedOnRx 					GET,

aAggFramesWithTxErrors 					GET,

aAggFramesWithRxErrors 					GET,

aAggUnknownProtocolFrames 					GET,

aAggPortList 					GET;

REGISTERED AS

{iso(1) std(0) iso8802(8802) csma(3) csmacdmgt(30) package(4) pAggregatorRecommended(20)};

PRESENT IF		The recommended package is implemented,

pAggregatorOptional			PACKAGE

ATTRIBUTES		aAggMulticastFramesTxOK 					GET,

aAggMulticastFramesRxOK 					GET,

aAggBroadcastFramesTxOK					GET,

aAggBroadcastFramesRxOK 					GET;

REGISTERED AS

{iso(1) std(0) iso8802(8802) csma(3) csmacdmgt(30) package(4) pAggregatorOptional(21)};

PRESENT IF		The optional package is implemented;

REGISTERED AS		{iso(1) std(0) iso8802(8802) csma(3) csmacdmgt(30) managedObjectClass(3) oAggregator(10)};

nbAggregatorName					NAME BINDING

SUBORDINATE OBJECT CLASS					oAggregator;

NAMED BY SUPERIOR OBJECT CLASS					“ISO/IEC 10165-2”:system;

WITH ATTRIBUTE					aAggID;

REGISTERED AS		{iso(1) std(0) iso8802(8802) csma(3) csmacdmgt(30) nameBinding(6) nbAggregatorName(20)};

- Aggregator attributes

aAggID 					ATTRIBUTE

WITH ATTRIBUTE SYNTAX 				IEEE802Dot3-MgmtAttributeModule.AggID;

MATCHES FOR				EQUALITY;

BEHAVIOUR 				bAggID;

REGISTERED AS		{iso(1) std(0) iso8802(8802) csma(3) csmacdmgt(30) attribute(7) aAggID(101)};

bAggID 					BEHAVIOUR

DEFINED AS 		See “BEHAVIOUR DEFINED AS” in 30.7.1.1.1;

aAggDescription 					ATTRIBUTE

WITH ATTRIBUTE SYNTAX 				IEEE802Dot3-MgmtAttributeModule.DescriptionString;

MATCHES FOR				EQUALITY;

BEHAVIOUR 				bAggDescription;

REGISTERED AS		{iso(1) std(0) iso8802(8802) csma(3) csmacdmgt(30) attribute(7) aAggDescription(102)};

bAggDescription 					BEHAVIOUR

DEFINED AS 		See “BEHAVIOUR DEFINED AS” in 30.7.1.1.2;

aAggName 					ATTRIBUTE

WITH ATTRIBUTE SYNTAX 				IEEE802Dot3-MgmtAttributeModule.DescriptionString;

MATCHES FOR				EQUALITY;

BEHAVIOUR 				bAggName;

REGISTERED AS		{iso(1) std(0) iso8802(8802) csma(3) csmacdmgt(30) attribute(7) aAggName(103)};

bAggName 					BEHAVIOUR

DEFINED AS 		See “BEHAVIOUR DEFINED AS” in 30.7.1.1.3;

aAggActorSystemID					ATTRIBUTE

WITH ATTRIBUTE SYNTAX 				IEEE802Dot3-MgmtAttributeModule.MACAddress;

MATCHES FOR				EQUALITY, ORDERING;

BEHAVIOUR 				bAggActorSystemID;

REGISTERED AS		{iso(1) std(0) iso8802(8802) csma(3) csmacdmgt(30) attribute(7) aAggActorSystemID(104)};

bAggActorSystemID					BEHAVIOUR

DEFINED AS 		See “BEHAVIOUR DEFINED AS” in 30.7.1.1.4;

aAggActorSystemPriority					ATTRIBUTE

WITH ATTRIBUTE SYNTAX 				IEEE802Dot3-MgmtAttributeModule.PriorityValue;

MATCHES FOR				EQUALITY, ORDERING;

BEHAVIOUR 				bAggActorSystemPriority;

REGISTERED AS		{iso(1) std(0) iso8802(8802) csma(3) csmacdmgt(30) attribute(7) aAggActorSystemPriority(105)};

bAggActorSystemPriority					BEHAVIOUR

DEFINED AS 		See “BEHAVIOUR DEFINED AS” in 30.7.1.1.5;

aAggAggregateOrIndividual					ATTRIBUTE

WITH ATTRIBUTE SYNTAX 				IEEE802Dot3-MgmtAttributeModule.AggOrInd;

MATCHES FOR				EQUALITY;

BEHAVIOUR 				bAggAggregateOrIndividual;

REGISTERED AS		{iso(1) std(0) iso8802(8802) csma(3) csmacdmgt(30) attribute(7) aAggAggregateOrIndividual(106)};

bAggAggregateOrIndividual					BEHAVIOUR

DEFINED AS 		See “BEHAVIOUR DEFINED AS” in 30.7.1.1.6;

aAggActorAdminKey					ATTRIBUTE

WITH ATTRIBUTE SYNTAX 				IEEE802Dot3-MgmtAttributeModule.KeyValue;

MATCHES FOR				EQUALITY;

BEHAVIOUR 				bAggActorAdminKey;

REGISTERED AS		{iso(1) std(0) iso8802(8802) csma(3) csmacdmgt(30) attribute(7) aAggActorAdminKey(107)};

bAggActorAdminKey 					BEHAVIOUR

DEFINED AS 		See “BEHAVIOUR DEFINED AS” in 30.7.1.1.7;

aAggActorOperKey					ATTRIBUTE

WITH ATTRIBUTE SYNTAX 				IEEE802Dot3-MgmtAttributeModule.KeyValue;

MATCHES FOR				EQUALITY;

BEHAVIOUR 				bAggActorOperKey;

REGISTERED AS		{iso(1) std(0) iso8802(8802) csma(3) csmacdmgt(30) attribute(7) aAggActorOperKey(108)};

bAggActorOperKey				 	BEHAVIOUR

DEFINED AS 		See “BEHAVIOUR DEFINED AS” in 30.7.1.1.8;

aAggMACAddress					ATTRIBUTE

WITH ATTRIBUTE SYNTAX 				IEEE802Dot3-MgmtAttributeModule.MACAddress;

MATCHES FOR				EQUALITY, ORDERING;

BEHAVIOUR 				bAggMACAddress;

REGISTERED AS		{iso(1) std(0) iso8802(8802) csma(3) csmacdmgt(30) attribute(7) aAggMACAddress(109)};

bAggMACAddress 				 	BEHAVIOUR

DEFINED AS 		See “BEHAVIOUR DEFINED AS” in 30.7.1.1.9;

aAggPartnerSystemID					ATTRIBUTE

WITH ATTRIBUTE SYNTAX 				IEEE802Dot3-MgmtAttributeModule.MACAddress;

MATCHES FOR				EQUALITY, ORDERING;

BEHAVIOUR 				bAggPartnerSystemID;

REGISTERED AS		{iso(1) std(0) iso8802(8802) csma(3) csmacdmgt(30) attribute(7) aAggPartnerSystemID(110)};

bAggPartnerSystemID					 BEHAVIOUR

DEFINED AS 		See “BEHAVIOUR DEFINED AS” in 30.7.1.1.10;

aAggPartnerSystemPriority					ATTRIBUTE

WITH ATTRIBUTE SYNTAX 				IEEE802Dot3-MgmtAttributeModule.PriorityValue;

MATCHES FOR				EQUALITY, ORDERING;

BEHAVIOUR 				bAggPartnerSystemPriority;

REGISTERED AS		{iso(1) std(0) iso8802(8802) csma(3) csmacdmgt(30) attribute(7) aAggPartnerSystemPriority(111)};

bAggPartnerSystemPriority					BEHAVIOUR

DEFINED AS 		See “BEHAVIOUR DEFINED AS” in 30.7.1.1.11;

aAggPartnerOperKey					ATTRIBUTE

WITH ATTRIBUTE SYNTAX 				IEEE802Dot3-MgmtAttributeModule.KeyValue;

MATCHES FOR				EQUALITY;

BEHAVIOUR 				bAggPartnerOperKey;

REGISTERED AS		{iso(1) std(0) iso8802(8802) csma(3) csmacdmgt(30) attribute(7) aAggPartnerOperKey(112)};

bAggPartnerOperKey				 	BEHAVIOUR

DEFINED AS 		See “BEHAVIOUR DEFINED AS” in 30.7.1.1.12;

aAggAdminState					ATTRIBUTE

WITH ATTRIBUTE SYNTAX 				IEEE802Dot3-MgmtAttributeModule.AggState;

MATCHES FOR				EQUALITY;

BEHAVIOUR 				bAggAdminState;

REGISTERED AS		{iso(1) std(0) iso8802(8802) csma(3) csmacdmgt(30) attribute(7) aAggAdminState(113)};

bAggAdminState				 	BEHAVIOUR

DEFINED AS 		See “BEHAVIOUR DEFINED AS” in 30.7.1.1.13;

aAggOperState					ATTRIBUTE

WITH ATTRIBUTE SYNTAX 				IEEE802Dot3-MgmtAttributeModule.AggState;

MATCHES FOR				EQUALITY;

BEHAVIOUR 				bAggOperState ;

REGISTERED AS		{iso(1) std(0) iso8802(8802) csma(3) csmacdmgt(30) attribute(7) aAggOperState(114)};

bAggOperState				 	BEHAVIOUR

DEFINED AS 		See “BEHAVIOUR DEFINED AS” in 30.7.1.1.14;

aAggTimeOfLastOperChange					ATTRIBUTE

WITH ATTRIBUTE SYNTAX 				IEEE802Dot3-MgmtAttributeModule.Integer32;

MATCHES FOR				EQUALITY, ORDERING;

BEHAVIOUR 				bAggTimeOfLastOperChange;

REGISTERED AS		{iso(1) std(0) iso8802(8802) csma(3) csmacdmgt(30) attribute(7) aAggTimeOfLastOperChange(115)};

bAggTimeOfLastOperChange				 	BEHAVIOUR

DEFINED AS 		See “BEHAVIOUR DEFINED AS” in 30.7.1.1.15;

aAggDataRate					ATTRIBUTE

WITH ATTRIBUTE SYNTAX 				IEEE802Dot3-MgmtAttributeModule.AggDataRate;

MATCHES FOR				EQUALITY, ORDERING;

BEHAVIOUR 				bAggDataRate;

REGISTERED AS		{iso(1) std(0) iso8802(8802) csma(3) csmacdmgt(30) attribute(7) aAggDataRate(116)};

bAggDataRate				 	BEHAVIOUR

DEFINED AS 		See “BEHAVIOUR DEFINED AS” in 30.7.1.1.16;

aAggOctetsTxOK					ATTRIBUTE

DERIVED FROM				aCMCounter;

MATCHES FOR				EQUALITY, ORDERING;

BEHAVIOUR 				bAggOctetsTxOK;

REGISTERED AS		{iso(1) std(0) iso8802(8802) csma(3) csmacdmgt(30) attribute(7) aAggOctetsTxOK(117)};

bAggOctetsTxOK				 	BEHAVIOUR

DEFINED AS 		See “BEHAVIOUR DEFINED AS” in 30.7.1.1.17

NOTE—This counter has a maximum increment rate of 1 230 000 counts per second at 
10 Mb/s.;

aAggOctetsRxOK					ATTRIBUTE

DERIVED FROM				aCMCounter;

MATCHES FOR				EQUALITY, ORDERING;

BEHAVIOUR 				bAggOctetsRxOK;

REGISTERED AS		{iso(1) std(0) iso8802(8802) csma(3) csmacdmgt(30) attribute(7) aAggOctetsRxOK(118)};

bAggOctetsRxOK				 	BEHAVIOUR

DEFINED AS 		See “BEHAVIOUR DEFINED AS” in 30.7.1.1.18

NOTE—This counter has a maximum increment rate of 1 230 000 counts per second at 
10 Mb/s.;

aAggFramesTxOK					ATTRIBUTE

DERIVED FROM				aCMCounter;

MATCHES FOR				EQUALITY, ORDERING;

BEHAVIOUR 				bAggFramesTxOK;

REGISTERED AS		{iso(1) std(0) iso8802(8802) csma(3) csmacdmgt(30) attribute(7) aAggFramesTxOK(119)};

bAggFramesTxOK				 	BEHAVIOUR

DEFINED AS 		See “BEHAVIOUR DEFINED AS” in 30.7.1.1.19

NOTE—This counter has a maximum increment rate of 16 000 counts per second at 
10 Mb/s.;

aAggFramesRxOK					ATTRIBUTE

DERIVED FROM				aCMCounter;

MATCHES FOR				EQUALITY, ORDERING;

BEHAVIOUR 				bAggFramesRxOK;

REGISTERED AS		{iso(1) std(0) iso8802(8802) csma(3) csmacdmgt(30) attribute(7) aAggFramesRxOK(120)};

bAggFramesRxOK				 	BEHAVIOUR

DEFINED AS 		See “BEHAVIOUR DEFINED AS” in 30.7.1.1.20

NOTE—This counter has a maximum increment rate of 16 000 counts per second at 
10 Mb/s.;

aAggMulticastFramesTxOK					ATTRIBUTE

DERIVED FROM				aCMCounter;

MATCHES FOR				EQUALITY, ORDERING;

BEHAVIOUR 				bAggMulticastFramesTxOK;

REGISTERED AS		{iso(1) std(0) iso8802(8802) csma(3) csmacdmgt(30) attribute(7) aAggMulticastFramesTxOK(121)};

bAggMulticastFramesTxOK				 	BEHAVIOUR

DEFINED AS 		See “BEHAVIOUR DEFINED AS” in 30.7.1.1.21

NOTE—This counter has a maximum increment rate of 16 000 counts per second at 
10 Mb/s.;

aAggMulticastFramesRxOK					ATTRIBUTE

DERIVED FROM				aCMCounter;

MATCHES FOR				EQUALITY, ORDERING;

BEHAVIOUR 				bAggMulticastFramesRxOK;

REGISTERED AS		{iso(1) std(0) iso8802(8802) csma(3) csmacdmgt(30) attribute(7) aAggMulticastFramesRxOK(122)};

bAggMulticastFramesRxOK					BEHAVIOUR

DEFINED AS 		See “BEHAVIOUR DEFINED AS” in 30.7.1.1.22

NOTE—This counter has a maximum increment rate of 16 000 counts per second at 
10 Mb/s.;

aAggBroadcastFramesTxOK					ATTRIBUTE

DERIVED FROM				aCMCounter;

MATCHES FOR				EQUALITY, ORDERING;

BEHAVIOUR 				bAggBroadcastFramesTxOK;

REGISTERED AS		{iso(1) std(0) iso8802(8802) csma(3) csmacdmgt(30) attribute(7) aAggBroadcastFramesTxOK(123)};

bAggBroadcastFramesTxOK				 	BEHAVIOUR

DEFINED AS 		See “BEHAVIOUR DEFINED AS” in 30.7.1.1.23

NOTE—This counter has a maximum increment rate of 16 000 counts per second at 
10 Mb/s.;

aAggBroadcastFramesRxOK					ATTRIBUTE

DERIVED FROM				aCMCounter;

MATCHES FOR				EQUALITY, ORDERING;

BEHAVIOUR 				bAggBroadcastFramesRxOK;

REGISTERED AS		{iso(1) std(0) iso8802(8802) csma(3) csmacdmgt(30) attribute(7) aAggBroadcastFramesRxOK(124)};

bAggBroadcastFramesRxOK				 	BEHAVIOUR

DEFINED AS 		See “BEHAVIOUR DEFINED AS” in 30.7.1.1.24

NOTE—This counter has a maximum increment rate of 16 000 counts per second at 
10 Mb/s.;

aAggFramesDiscardedOnTx					ATTRIBUTE

DERIVED FROM				aCMCounter;

MATCHES FOR				EQUALITY, ORDERING;

BEHAVIOUR 				bAggFramesDiscardedOnTx;

REGISTERED AS		{iso(1) std(0) iso8802(8802) csma(3) csmacdmgt(30) attribute(7) aAggFramesDiscardedOnTx(125)};

bAggFramesDiscardedOnTx				 	BEHAVIOUR

DEFINED AS 		See “BEHAVIOUR DEFINED AS” in 30.7.1.1.25

NOTE—This counter has a maximum increment rate of 16 000 counts per second at 
10 Mb/s.;

aAggFramesDiscardedOnRx					ATTRIBUTE

DERIVED FROM				aCMCounter;

MATCHES FOR				EQUALITY, ORDERING;

BEHAVIOUR 				bAggFramesDiscardedOnRx;

REGISTERED AS		{iso(1) std(0) iso8802(8802) csma(3) csmacdmgt(30) attribute(7) aAggFramesDiscardedOnRx(126)};

bAggFramesDiscardedOnRx				 	BEHAVIOUR

DEFINED AS 		See “BEHAVIOUR DEFINED AS” in 30.7.1.1.26

NOTE—This counter has a maximum increment rate of 16 000 counts per second at 
10 Mb/s.;

aAggFramesWithTxErrors					ATTRIBUTE

DERIVED FROM				aCMCounter;

MATCHES FOR				EQUALITY, ORDERING;

BEHAVIOUR 				bAggFramesWithTxErrors;

REGISTERED AS		{iso(1) std(0) iso8802(8802) csma(3) csmacdmgt(30) attribute(7) aAggFramesWithTxErrors(127)};

bAggFramesWithTxErrors				 	BEHAVIOUR

DEFINED AS 		See “BEHAVIOUR DEFINED AS” in 30.7.1.1.27

NOTE—This counter has a maximum increment rate of 16 000 counts per second at 
10 Mb/s.;

aAggFramesWithRxErrors					ATTRIBUTE

DERIVED FROM				aCMCounter;

MATCHES FOR				EQUALITY, ORDERING;

BEHAVIOUR 				bAggFramesWithRxErrors;

REGISTERED AS		{iso(1) std(0) iso8802(8802) csma(3) csmacdmgt(30) attribute(7) aAggFramesWithRxErrors(128)};

bAggFramesWithRxErrors				 	BEHAVIOUR

DEFINED AS 		See “BEHAVIOUR DEFINED AS” in 30.7.1.1.28

NOTE—This counter has a maximum increment rate of 16 000 counts per second at 
10 Mb/s.;

aAggUnknownProtocolFrames					ATTRIBUTE

DERIVED FROM				aCMCounter;

MATCHES FOR				EQUALITY, ORDERING;

BEHAVIOUR 				bAggUnknownProtocolFrames;

REGISTERED AS		{iso(1) std(0) iso8802(8802) csma(3) csmacdmgt(30) attribute(7) aAggUnknownProtocolFrames(129)};

bAggUnknownProtocolFrames				 	BEHAVIOUR

DEFINED AS 		See “BEHAVIOUR DEFINED AS” in 30.7.1.1.29.

NOTE—This counter has a maximum increment rate of 16 000 counts per second at 
10 Mb/s.;

aAggLinkUpDownNotificationEnable					ATTRIBUTE

WITH ATTRIBUTE SYNTAX 				IEEE802Dot3-MgmtAttributeModule.NotificationEnable;

MATCHES FOR				EQUALITY;

BEHAVIOUR 				bAggLinkUpDownNotificationEnable;

REGISTERED AS		{iso(1) std(0) iso8802(8802) csma(3) csmacdmgt(30) attribute(7) aAggLinkUpDownNotificationEnable(130)};

bAggLinkUpDownNotificationEnable				 	BEHAVIOUR

DEFINED AS 		See “BEHAVIOUR DEFINED AS” in 30.7.1.1.31;

aAggPortList					ATTRIBUTE

WITH ATTRIBUTE SYNTAX 				IEEE802Dot3-MgmtAttributeModule.AggPortList;

MATCHES FOR				EQUALITY;

BEHAVIOUR 				bAggPortList;

REGISTERED AS		{iso(1) std(0) iso8802(8802) csma(3) csmacdmgt(30) attribute(7) aAggPortList(131)};

bAggPortList				 	BEHAVIOUR

DEFINED AS 		See “BEHAVIOUR DEFINED AS” in 30.7.1.1.30;

aAggCollectorMaxDelay					ATTRIBUTE

WITH ATTRIBUTE SYNTAX 				IEEE802Dot3-MgmtAttributeModule.CollectorMaxDelay;

MATCHES FOR				EQUALITY;

BEHAVIOUR 				bAggCollectorMaxDelay;

REGISTERED AS		{iso(1) std(0) iso8802(8802) csma(3) csmacdmgt(30) attribute(7) aAggCollectorMaxDelay(132)};

bAggCollectorMaxDelay				 	BEHAVIOUR

DEFINED AS 		See “BEHAVIOUR DEFINED AS” in 30.7.1.1.32;

- Aggregator notifications

nAggLinkUpNotification					NOTIFICATION

BEHAVIOUR 				bAggLinkUpNotification;

WITH INFORMATION SYNTAX 				IEEE802Dot3-MgmtAttributeModule.AggID;

REGISTERED AS		{iso(1) std(0) iso8802(8802) csma(3) csmacdmgt(30) notification(10) nAggLinkUpNotification(6)};

bAggLinkUpNotification				 	BEHAVIOUR

DEFINED AS 		See “BEHAVIOUR DEFINED AS” in 30.7.1.2.1;

nAggLinkDownNotification 					NOTIFICATION

BEHAVIOUR 				bAggLinkDownNotification;

WITH INFORMATION SYNTAX 				IEEE802Dot3-MgmtAttributeModule.AggID;

REGISTERED AS		{iso(1) std(0) iso8802(8802) csma(3) csmacdmgt(30) notification(10) nAggLinkDownNotification(7)};

bAggLinkDownNotification				 	BEHAVIOUR

DEFINED AS 		See “BEHAVIOUR DEFINED AS” in 30.7.1.2.2;

- Aggregation Port managed object class

-- NOTE—This managed object class has been deprecated by IEEE Std 802.1AX-200X.

- Aggregation Port, formal definition

oAggregationPort					MANAGED OBJECT CLASS

DERIVED FROM				“CCITT Rec. X.721 (1992) | ISO/IEC 10165-2 :1992”:top;

CHARACTERIZED BY

pAggregationPortBasic			PACKAGE

ATTRIBUTES		aAggPortID 					GET;

;

;

CONDITIONAL PACKAGES

pAggregationPortMandatory PACKAGE

ATTRIBUTES		aAggPortActorSystemPriority 					GET-REPLACE,

aAggPortActorSystemID 					GET,

aAggPortActorAdminKey 					GET-REPLACE,

aAggPortActorOperKey 					GET,

aAggPortPartnerAdminSystemPriority 					GET-REPLACE,

aAggPortPartnerOperSystemPriority 					GET,

aAggPortPartnerAdminSystemID 					GET-REPLACE,

aAggPortPartnerOperSystemID 					GET,

aAggPortPartnerAdminKey 					GET-REPLACE,

aAggPortPartnerOperKey 					GET,

aAggPortSelectedAggID 					GET,

aAggPortAttachedAggID 					GET,

aAggPortActorPort 					GET,

aAggPortActorPortPriority 					GET-REPLACE,

aAggPortPartnerAdminPort 					GET-REPLACE,

aAggPortPartnerOperPort 					GET,

aAggPortPartnerAdminPortPriority 					GET-REPLACE,

aAggPortPartnerOperPortPriority 					GET,

aAggPortActorAdminState 					GET-REPLACE,

aAggPortActorOperState 					GET,

aAggPortPartnerAdminState 					GET-REPLACE,

aAggPortPartnerOperState 					GET,

aAggPortAggregateOrIndividual 					GET;

REGISTERED AS

{iso(1) std(0) iso8802(8802) csma(3) csmacdmgt(30) package(4) pAggregationPortMandatory(22)};

PRESENT IF		Conformance to Link Aggregation management is desired;

REGISTERED AS		{iso(1) std(0) iso8802(8802) csma(3) csmacdmgt(30) managedObjectClass(3) oAggregationPort(11)};

nbAggregationPort					NAME BINDING

SUBORDINATE OBJECT CLASS					oAggregationPort;

NAMED BY SUPERIOR OBJECT CLASS					oAggregator;

WITH ATTRIBUTE					aAggPortID;

REGISTERED AS		{iso(1) std(0) iso8802(8802) csma(3) csmacdmgt(30) nameBinding(6) nbAggregationPortName(21)};

- Aggregation Port attributes

aAggPortID					ATTRIBUTE

WITH ATTRIBUTE SYNTAX 				IEEE802Dot3-MgmtAttributeModule.AggPortID;

MATCHES FOR				EQUALITY;

BEHAVIOUR 				bAggPortID;

REGISTERED AS		{iso(1) std(0) iso8802(8802) csma(3) csmacdmgt(30) attribute(7) aAggPortID(133)};

bAggPortID				 	BEHAVIOUR

DEFINED AS 		See “BEHAVIOUR DEFINED AS” in 30.7.2.1.1;

aAggPortActorSystemPriority					ATTRIBUTE

WITH ATTRIBUTE SYNTAX 				IEEE802Dot3-MgmtAttributeModule.PriorityValue;

MATCHES FOR				EQUALITY, ORDERING;

BEHAVIOUR 				bAggPortActorSystemPriority;

REGISTERED AS		{iso(1) std(0) iso8802(8802) csma(3) csmacdmgt(30) attribute(7) aAggPortActorSystemPriority(134)};

bAggPortActorSystemPriority				 	BEHAVIOUR

DEFINED AS 		See “BEHAVIOUR DEFINED AS” in 30.7.2.1.2;

aAggPortActorSystemID					ATTRIBUTE

WITH ATTRIBUTE SYNTAX 				IEEE802Dot3-MgmtAttributeModule.MACAddress;

MATCHES FOR				EQUALITY, ORDERING;

BEHAVIOUR 				bAggPortActorSystemID;

REGISTERED AS		{iso(1) std(0) iso8802(8802) csma(3) csmacdmgt(30) attribute(7) aAggPortActorSystemID(135)};

bAggPortActorSystemID				 	BEHAVIOUR

DEFINED AS 		See “BEHAVIOUR DEFINED AS” in 30.7.2.1.3;

aAggPortActorAdminKey					ATTRIBUTE

WITH ATTRIBUTE SYNTAX 				IEEE802Dot3-MgmtAttributeModule.KeyValue;

MATCHES FOR				EQUALITY;

BEHAVIOUR 				bAggPortActorAdminKey;

REGISTERED AS		{iso(1) std(0) iso8802(8802) csma(3) csmacdmgt(30) attribute(7) aAggPortActorAdminKey(136)};

bAggPortActorAdminKey					 BEHAVIOUR

DEFINED AS 		See “BEHAVIOUR DEFINED AS” in 30.7.2.1.4;

aAggPortActorOperKey					ATTRIBUTE

WITH ATTRIBUTE SYNTAX 				IEEE802Dot3-MgmtAttributeModule.KeyValue;

MATCHES FOR				EQUALITY;

BEHAVIOUR 				bAggPortActorOperKey;

REGISTERED AS		{iso(1) std(0) iso8802(8802) csma(3) csmacdmgt(30) attribute(7) aAggPortActorOperKey(137)};

bAggPortActorOperKey				 	BEHAVIOUR

DEFINED AS 		See “BEHAVIOUR DEFINED AS” in 30.7.2.1.5;

aAggPortPartnerAdminSystemPriority					ATTRIBUTE

WITH ATTRIBUTE SYNTAX 				IEEE802Dot3-MgmtAttributeModule.PriorityValue;

MATCHES FOR				EQUALITY, ORDERING;

BEHAVIOUR 				bAggPortPartnerAdminSystemPriority;

REGISTERED AS		{iso(1) std(0) iso8802(8802) csma(3) csmacdmgt(30) attribute(7) aAggPortPartnerAdminSystemPriority(138)};

bAggPortPartnerAdminSystemPriority				 	BEHAVIOUR

DEFINED AS 		See “BEHAVIOUR DEFINED AS” in  30.7.2.1.6;

aAggPortPartnerOperSystemPriority					ATTRIBUTE

WITH ATTRIBUTE SYNTAX 				IEEE802Dot3-MgmtAttributeModule.PriorityValue;

MATCHES FOR				EQUALITY, ORDERING;

BEHAVIOUR 				bAggPortPartnerOperSystemPriority;

REGISTERED AS		{iso(1) std(0) iso8802(8802) csma(3) csmacdmgt(30) attribute(7) aAggPortPartnerOperSystemPriority(139)};

bAggPortPartnerOperSystemPriority					 BEHAVIOUR

DEFINED AS 		See “BEHAVIOUR DEFINED AS” in 30.7.2.1.7;

aAggPortPartnerAdminSystemID					ATTRIBUTE

WITH ATTRIBUTE SYNTAX 				IEEE802Dot3-MgmtAttributeModule.MACAddress;

MATCHES FOR				EQUALITY, ORDERING;

BEHAVIOUR 				bAggPortPartnerAdminSystemID;

REGISTERED AS		{iso(1) std(0) iso8802(8802) csma(3) csmacdmgt(30) attribute(7) aAggPortPartnerAdminSystemID(140)};

bAggPortPartnerAdminSystemID					 BEHAVIOUR

DEFINED AS 		See “BEHAVIOUR DEFINED AS” in 30.7.2.1.8;

aAggPortPartnerOperSystemID					ATTRIBUTE

WITH ATTRIBUTE SYNTAX 				IEEE802Dot3-MgmtAttributeModule.MACAddress;

MATCHES FOR				EQUALITY, ORDERING;

BEHAVIOUR 				bAggPortPartnerOperSystemID;

REGISTERED AS		{iso(1) std(0) iso8802(8802) csma(3) csmacdmgt(30) attribute(7) aAggPortPartnerOperSystemID(141)};

bAggPortPartnerOperSystemID					 BEHAVIOUR

DEFINED AS 		See “BEHAVIOUR DEFINED AS” in 30.7.2.1.9;

aAggPortPartnerAdminKey					ATTRIBUTE

WITH ATTRIBUTE SYNTAX 				IEEE802Dot3-MgmtAttributeModule.KeyValue;

MATCHES FOR				EQUALITY;

BEHAVIOUR 				bAggPortPartnerAdminKey;

REGISTERED AS		{iso(1) std(0) iso8802(8802) csma(3) csmacdmgt(30) attribute(7) aAggPortPartnerAdminKey(142)};

bAggPortPartnerAdminKey				 	BEHAVIOUR

DEFINED AS 		See “BEHAVIOUR DEFINED AS” in 30.7.2.1.10;

aAggPortPartnerOperKey					ATTRIBUTE

WITH ATTRIBUTE SYNTAX 				IEEE802Dot3-MgmtAttributeModule.KeyValue;

MATCHES FOR				EQUALITY;

BEHAVIOUR 				bAggPortPartnerOperKey;

REGISTERED AS		{iso(1) std(0) iso8802(8802) csma(3) csmacdmgt(30) attribute(7) aAggPortPartnerOperKey(143)};

bAggPortPartnerOperKey				 	BEHAVIOUR

DEFINED AS 		See “BEHAVIOUR DEFINED AS” in 30.7.2.1.11;

aAggPortSelectedAggID					ATTRIBUTE

WITH ATTRIBUTE SYNTAX 				IEEE802Dot3-MgmtAttributeModule.AggID;

MATCHES FOR				EQUALITY;

BEHAVIOUR 				bAggPortSelectedAggID;

REGISTERED AS		{iso(1) std(0) iso8802(8802) csma(3) csmacdmgt(30) attribute(7) aAggPortSelectedAggID(144)};

bAggPortSelectedAggID				 	BEHAVIOUR

DEFINED AS 		See “BEHAVIOUR DEFINED AS” in 30.7.2.1.12;

aAggPortAttachedAggID					ATTRIBUTE

WITH ATTRIBUTE SYNTAX 				IEEE802Dot3-MgmtAttributeModule.AggID;

MATCHES FOR				EQUALITY;

BEHAVIOUR 				bAggPortAttachedAggID;

REGISTERED AS		{iso(1) std(0) iso8802(8802) csma(3) csmacdmgt(30) attribute(7) aAggPortAttachedAggID(145)};

bAggPortAttachedAggID				 	BEHAVIOUR

DEFINED AS 		See “BEHAVIOUR DEFINED AS” in 30.7.2.1.13;

aAggPortActorPort					ATTRIBUTE

WITH ATTRIBUTE SYNTAX 				IEEE802Dot3-MgmtAttributeModule.PortNumber;

MATCHES FOR				EQUALITY, ORDERING;

BEHAVIOUR 				bAggPortActorPort;

REGISTERED AS		{iso(1) std(0) iso8802(8802) csma(3) csmacdmgt(30) attribute(7) aAggPortActorPort(146)};

bAggPortActorPort				 	BEHAVIOUR

DEFINED AS 		See “BEHAVIOUR DEFINED AS” in 30.7.2.1.14;

aAggPortActorPortPriority					ATTRIBUTE

WITH ATTRIBUTE SYNTAX 				IEEE802Dot3-MgmtAttributeModule.PriorityValue;

MATCHES FOR				EQUALITY, ORDERING;

BEHAVIOUR 				bAggPortActorPortPriority;

REGISTERED AS		{iso(1) std(0) iso8802(8802) csma(3) csmacdmgt(30) attribute(7) aAggPortActorPortPriority(147)};

bAggPortActorPortPriority				 	BEHAVIOUR

DEFINED AS 		See “BEHAVIOUR DEFINED AS” in 30.7.2.1.15;

aAggPortPartnerAdminPort					ATTRIBUTE

WITH ATTRIBUTE SYNTAX 				IEEE802Dot3-MgmtAttributeModule.PortNumber;

MATCHES FOR				EQUALITY, ORDERING;

BEHAVIOUR 				bAggPortPartnerAdminPort;

REGISTERED AS		{iso(1) std(0) iso8802(8802) csma(3) csmacdmgt(30) attribute(7) aAggPortPartnerAdminPort(148)};

bAggPortPartnerAdminPort				 	BEHAVIOUR

DEFINED AS 		See “BEHAVIOUR DEFINED AS” in 30.7.2.1.16;

aAggPortPartnerOperPort					ATTRIBUTE

WITH ATTRIBUTE SYNTAX 				IEEE802Dot3-MgmtAttributeModule.PortNumber;

MATCHES FOR				EQUALITY, ORDERING;

BEHAVIOUR 				bAggPortPartnerOperPort;

REGISTERED AS		{iso(1) std(0) iso8802(8802) csma(3) csmacdmgt(30) attribute(7) aAggPortPartnerOperPort(149)};

bAggPortPartnerOperPort				 	BEHAVIOUR

DEFINED AS 		See “BEHAVIOUR DEFINED AS” in 30.7.2.1.17;

aAggPortPartnerAdminPortPriority					ATTRIBUTE

WITH ATTRIBUTE SYNTAX 				IEEE802Dot3-MgmtAttributeModule.PriorityValue;

MATCHES FOR				EQUALITY, ORDERING;

BEHAVIOUR 				bAggPortPartnerAdminPortPriority;

REGISTERED AS		{iso(1) std(0) iso8802(8802) csma(3) csmacdmgt(30) attribute(7) aAggPortPartnerAdminPortPriority(150)};

bAggPortPartnerAdminPortPriority					 BEHAVIOUR

DEFINED AS 		See “BEHAVIOUR DEFINED AS” in 30.7.2.1.18;

aAggPortPartnerOperPortPriority					ATTRIBUTE

WITH ATTRIBUTE SYNTAX 				IEEE802Dot3-MgmtAttributeModule.PriorityValue;

MATCHES FOR				EQUALITY, ORDERING;

BEHAVIOUR 				bAggPortPartnerOperPortPriority;

REGISTERED AS		{iso(1) std(0) iso8802(8802) csma(3) csmacdmgt(30) attribute(7) aAggPortPartnerOperPortPriority(151)};

bAggPortPartnerOperPortPriority					 BEHAVIOUR

DEFINED AS 		See “BEHAVIOUR DEFINED AS” in 30.7.2.1.19;

aAggPortActorAdminState					ATTRIBUTE

WITH ATTRIBUTE SYNTAX 				IEEE802Dot3-MgmtAttributeModule.AggPortState;

MATCHES FOR				EQUALITY;

BEHAVIOUR 				bAggPortActorAdminState;

REGISTERED AS		{iso(1) std(0) iso8802(8802) csma(3) csmacdmgt(30) attribute(7) aAggPortActorAdminState(152)};

bAggPortActorAdminState				 	BEHAVIOUR

DEFINED AS 		See “BEHAVIOUR DEFINED AS” in 30.7.2.1.20;

aAggPortActorOperState					ATTRIBUTE

WITH ATTRIBUTE SYNTAX 				IEEE802Dot3-MgmtAttributeModule.AggPortState;

MATCHES FOR				EQUALITY;

BEHAVIOUR 				bAggPortActorOperState;

REGISTERED AS		{iso(1) std(0) iso8802(8802) csma(3) csmacdmgt(30) attribute(7) aAggPortActorOperState(153)};

bAggPortActorOperState				 	BEHAVIOUR

DEFINED AS 		See “BEHAVIOUR DEFINED AS” in 30.7.2.1.21;

aAggPortPartnerAdminState					ATTRIBUTE

WITH ATTRIBUTE SYNTAX 				IEEE802Dot3-MgmtAttributeModule.AggPortState;

MATCHES FOR				EQUALITY, ORDERING;

BEHAVIOUR 				bAggPortPartnerAdminState;

REGISTERED AS		{iso(1) std(0) iso8802(8802) csma(3) csmacdmgt(30) attribute(7) aAggPortPartnerAdminState(154)};

bAggPortPartnerAdminState				 	BEHAVIOUR

DEFINED AS 		See “BEHAVIOUR DEFINED AS” in 30.7.2.1.22;

aAggPortPartnerOperState					ATTRIBUTE

WITH ATTRIBUTE SYNTAX 				IEEE802Dot3-MgmtAttributeModule.AggPortState;

MATCHES FOR				EQUALITY, ORDERING;

BEHAVIOUR 				bAggPortPartnerOperState;

REGISTERED AS		{iso(1) std(0) iso8802(8802) csma(3) csmacdmgt(30) attribute(7) aAggPortPartnerOperState(155)};

bAggPortPartnerOperState				 	BEHAVIOUR

DEFINED AS 		See “BEHAVIOUR DEFINED AS” in 30.7.2.1.23;

aAggPortAggregateOrIndividual					ATTRIBUTE

WITH ATTRIBUTE SYNTAX 				IEEE802Dot3-MgmtAttributeModule.AggOrInd;

MATCHES FOR				EQUALITY;

BEHAVIOUR 				bAggPortAggregateOrIndividual;

REGISTERED AS		{iso(1) std(0) iso8802(8802) csma(3) csmacdmgt(30) attribute(7) aAggPortAggregateOrIndividual(156)};

bAggPortAggregateOrIndividual					BEHAVIOUR

DEFINED AS 		See “BEHAVIOUR DEFINED AS” in 30.7.2.1.24;

- Aggregation Port Statistics managed object class

-- NOTE—This managed object class has been deprecated by IEEE Std 802.1AX-200X.

- Aggregation Port Statistics, formal definition

oAggPortStats					MANAGED OBJECT CLASS

DERIVED FROM				“CCITT Rec. X.721 (1992) | ISO/IEC 10165-2 :1992”:top;

CONDITIONAL PACKAGES

pAggPortStats			PACKAGE

ATTRIBUTES		aAggPortStatsID					GET,

aAggPortStatsLACPDUsRx 					GET,

aAggPortStatsMarkerPDUsRx 					GET,

aAggPortStatsMarkerResponsePDUsRx 					GET,

aAggPortStatsUnknownRx 					GET,

aAggPortStatsIllegalRx 					GET,

aAggPortStatsLACPDUsTx 					GET,

aAggPortStatsMarkerPDUsTx 					GET,

aAggPortStatsMarkerResponsePDUsTx 					GET;

REGISTERED AS

{iso(1) std(0) iso8802(8802) csma(3) csmacdmgt(30) package(4) pAggPortStats(23)};

PRESENT IF		The Aggregation Port Statistics package is supported;

REGISTERED AS		{iso(1) std(0) iso8802(8802) csma(3) csmacdmgt(30) managedObjectClass(3) oAggPortStats(12)};

nbAggPortStats						NAME BINDING

SUBORDINATE OBJECT CLASS					oAggPortStats;

NAMED BY SUPERIOR OBJECT CLASS					oAggregationPort;

WITH ATTRIBUTE					aAggPortStatsID;

REGISTERED AS		{iso(1) std(0) iso8802(8802) csma(3) csmacdmgt(30) nameBinding(6) nbAggPortStats(22)};

- Aggregation Port Statistics attributes

aAggPortStatsID					ATTRIBUTE

WITH ATTRIBUTE SYNTAX 				IEEE802Dot3-MgmtAttributeModule.AggPortID;

MATCHES FOR				EQUALITY;

BEHAVIOUR 				bAggPortStatsID;

REGISTERED AS		{iso(1) std(0) iso8802(8802) csma(3) csmacdmgt(30) attribute(7) aAggPortStatsID(157)};

bAggPortStatsID				 	BEHAVIOUR

DEFINED AS 		See “BEHAVIOUR DEFINED AS” in 30.7.3.1.1;

aAggPortStatsLACPDUsRx					ATTRIBUTE

DERIVED FROM				aCMCounter;

MATCHES FOR				EQUALITY, ORDERING;

BEHAVIOUR 				bAggPortStatsLACPDUsRx;

REGISTERED AS		{iso(1) std(0) iso8802(8802) csma(3) csmacdmgt(30) attribute(7) aAggPortStatsLACPDUsRx(158)};

bAggPortStatsLACPDUsRx				 	BEHAVIOUR

DEFINED AS 		See “BEHAVIOUR DEFINED AS” in 30.7.3.1.2

NOTE—This counter has a maximum increment rate of 5 counts per second at 10 Mb/s.;

aAggPortStatsMarkerPDUsRx					ATTRIBUTE

DERIVED FROM				aCMCounter;

MATCHES FOR				EQUALITY, ORDERING;

BEHAVIOUR 				bAggPortStatsMarkerPDUsRx;

REGISTERED AS		{iso(1) std(0) iso8802(8802) csma(3) csmacdmgt(30) attribute(7) aAggPortStatsMarkerPDUsRx(159)};

bAggPortStatsMarkerPDUsRx				 	BEHAVIOUR

DEFINED AS 		See “BEHAVIOUR DEFINED AS” in 30.7.3.1.3

NOTE—This counter has a maximum increment rate of 5 counts per second at 10 Mb/s.;

aAggPortStatsMarkerResponsePDUsRx					ATTRIBUTE

DERIVED FROM				aCMCounter;

MATCHES FOR				EQUALITY, ORDERING;

BEHAVIOUR 				bAggPortStatsMarkerResponsePDUsRx;

REGISTERED AS		{iso(1) std(0) iso8802(8802) csma(3) csmacdmgt(30) attribute(7) aAggPortStatsMarkerResponsePDUsRx(160)};

bAggPortStatsMarkerResponsePDUsRx				 	BEHAVIOUR

DEFINED AS 		See “BEHAVIOUR DEFINED AS” in 30.7.3.1.4

NOTE—This counter has a maximum increment rate of 5 counts per second at 10 Mb/s.;

aAggPortStatsUnknownRx					ATTRIBUTE

DERIVED FROM				aCMCounter;

MATCHES FOR				EQUALITY, ORDERING;

BEHAVIOUR 				bAggPortStatsUnknownRx;

REGISTERED AS		{iso(1) std(0) iso8802(8802) csma(3) csmacdmgt(30) attribute(7) aAggPortStatsUnknownRx(161)};

bAggPortStatsUnknownRx				 	BEHAVIOUR

DEFINED AS 		See “BEHAVIOUR DEFINED AS” in 30.7.3.1.5

NOTE—This counter has a maximum increment rate of 50 counts per second at 10 Mb/s.;

aAggPortStatsIllegalRx					ATTRIBUTE

DERIVED FROM				aCMCounter;

MATCHES FOR				EQUALITY, ORDERING;

BEHAVIOUR 				bAggPortStatsIllegalRx;

REGISTERED AS		{iso(1) std(0) iso8802(8802) csma(3) csmacdmgt(30) attribute(7) aAggPortStatsIllegalRx(162)};

bAggPortStatsIllegalRx				 	BEHAVIOUR

DEFINED AS 		See “BEHAVIOUR DEFINED AS” in 30.7.3.1.6

NOTE—This counter has a maximum increment rate of 50 counts per second at 10 Mb/s.;

aAggPortStatsLACPDUsTx					ATTRIBUTE

DERIVED FROM				aCMCounter;

MATCHES FOR				EQUALITY, ORDERING;

BEHAVIOUR 				bAggPortStatsLACPDUsTx;

REGISTERED AS		{iso(1) std(0) iso8802(8802) csma(3) csmacdmgt(30) attribute(7) aAggPortStatsLACPDUsTx(163)};

bAggPortStatsLACPDUsTx					BEHAVIOUR

DEFINED AS 		See “BEHAVIOUR DEFINED AS” in 30.7.3.1.7

NOTE—This counter has a maximum increment rate of 5 counts per second at 10 Mb/s.;

aAggPortStatsMarkerPDUsTx					ATTRIBUTE

DERIVED FROM				aCMCounter;

MATCHES FOR				EQUALITY, ORDERING;

BEHAVIOUR 				bAggPortStatsMarkerPDUsTx;

REGISTERED AS		{iso(1) std(0) iso8802(8802) csma(3) csmacdmgt(30) attribute(7) aAggPortStatsMarkerPDUsTx(164)};

bAggPortStatsMarkerPDUsTx				 	BEHAVIOUR

DEFINED AS 		See “BEHAVIOUR DEFINED AS” in 30.7.3.1.8

NOTE—This counter has a maximum increment rate of 5 counts per second at 10 Mb/s.;

aAggPortStatsMarkerResponsePDUsTx					ATTRIBUTE

DERIVED FROM				aCMCounter;

MATCHES FOR				EQUALITY, ORDERING;

BEHAVIOUR 				bAggPortStatsMarkerResponsePDUsTx;

REGISTERED AS		{iso(1) std(0) iso8802(8802) csma(3) csmacdmgt(30) attribute(7) aAggPortStatsMarkerResponsePDUsTx(165)};

bAggPortStatsMarkerResponsePDUsTx				 	BEHAVIOUR

DEFINED AS 		See “BEHAVIOUR DEFINED AS” in 30.7.3.1.9

NOTE—This counter has a maximum increment rate of 5 counts per second at 10 Mb/s.;

- Aggregation Port Debug Information managed object class

-- NOTE—This managed object class has been deprecated by IEEE Std 802.1AX-200X.

- Aggregation Port Debug Information, formal definition

oAggPortDebugInformation					MANAGED OBJECT CLASS

DERIVED FROM				“CCITT Rec. X.721 (1992) | ISO/IEC 10165-2 :1992”:top;

CONDITIONAL PACKAGES

pLACPDebug			PACKAGE

ATTRIBUTES		aAggPortDebugInformationID 						GET,

aAggPortDebugRxState 						GET,

aAggPortDebugLastRxTime 						GET,

aAggPortDebugMuxState 						GET,

aAggPortDebugMuxReason 						GET,

aAggPortDebugActorChurnState 						GET,

aAggPortDebugPartnerChurnState 						GET,

aAggPortDebugActorChurnCount 						GET,

aAggPortDebugPartnerChurnCount 						GET,

aAggPortDebugActorSyncTransitionCount						GET,

aAggPortDebugPartnerSyncTransitionCount	 					GET,

aAggPortDebugActorChangeCount 	 					GET,

aAggPortDebugPartnerChangeCount 						GET;

REGISTERED AS

{iso(1) std(0) iso8802(8802) csma(3) csmacdmgt(30) package(4) pAggPortDebugInformation(24)};

PRESENT IF		The Aggregation Port Debug Information package is supported;

REGISTERED AS		{iso(1) std(0) iso8802(8802) csma(3) csmacdmgt(30) managedObjectClass(3) oAggPortDebugInformation(13)};

nbAggPortDebugInformation						NAME BINDING

SUBORDINATE OBJECT CLASS					oAggPortDebugInformation;

NAMED BY SUPERIOR OBJECT CLASS					oAggregationPort;

WITH ATTRIBUTE					aAggPortDebugInformationID;

REGISTERED AS		{iso(1) std(0) iso8802(8802) csma(3) csmacdmgt(30) nameBinding(6) nbAggPortDebugInformation(23)};

- Aggregation Port Debug Information attributes

aAggPortDebugInformationID					ATTRIBUTE

WITH ATTRIBUTE SYNTAX 				IEEE802Dot3-MgmtAttributeModule.AggPortID;

MATCHES FOR				EQUALITY, ORDERING;

BEHAVIOUR 				bAggPortDebugInformationID;

REGISTERED AS		{iso(1) std(0) iso8802(8802) csma(3) csmacdmgt(30) attribute(7) aAggPortDebugInformationID(166)};

bAggPortDebugInformationID				 	BEHAVIOUR

DEFINED AS 		See “BEHAVIOUR DEFINED AS” in 30.7.4.1.1;

aAggPortDebugRxState					ATTRIBUTE

WITH ATTRIBUTE SYNTAX 				IEEE802Dot3-MgmtAttributeModule.RxState;

MATCHES FOR				EQUALITY;

BEHAVIOUR 				bAggPortDebugRxState;

REGISTERED AS		{iso(1) std(0) iso8802(8802) csma(3) csmacdmgt(30) attribute(7) aAggPortDebugRxState(167)};

bAggPortDebugRxState				 	BEHAVIOUR

DEFINED AS 		See “BEHAVIOUR DEFINED AS” in 30.7.4.1.2;

aAggPortDebugLastRxTime					ATTRIBUTE

WITH ATTRIBUTE SYNTAX 				IEEE802Dot3-MgmtAttributeModule.Integer32;

MATCHES FOR				EQUALITY, ORDERING;

BEHAVIOUR 				bAggPortDebugLastRxTime;

REGISTERED AS		{iso(1) std(0) iso8802(8802) csma(3) csmacdmgt(30) attribute(7) aAggPortDebugLastRxTime(168)};

bAggPortDebugLastRxTime				 	BEHAVIOUR

DEFINED AS 		See “BEHAVIOUR DEFINED AS” in 30.7.4.1.3;

aAggPortDebugMuxState					ATTRIBUTE

WITH ATTRIBUTE SYNTAX 				IEEE802Dot3-MgmtAttributeModule.MuxState;

MATCHES FOR				EQUALITY;

BEHAVIOUR 				bAggPortDebugMuxState;

REGISTERED AS		{iso(1) std(0) iso8802(8802) csma(3) csmacdmgt(30) attribute(7) aAggPortDebugMuxState(169)};

bAggPortDebugMuxState				 	BEHAVIOUR

DEFINED AS 		See “BEHAVIOUR DEFINED AS” in 30.7.4.1.4;

aAggPortDebugMuxReason					ATTRIBUTE

WITH ATTRIBUTE SYNTAX 				IEEE802Dot3-MgmtAttributeModule.DescriptionString;

MATCHES FOR				EQUALITY;

BEHAVIOUR 				bAggPortDebugMuxReason;

REGISTERED AS		{iso(1) std(0) iso8802(8802) csma(3) csmacdmgt(30) attribute(7) aAggPortDebugMuxReason(170)};

bAggPortDebugMuxReason				 	BEHAVIOUR

DEFINED AS 		See “BEHAVIOUR DEFINED AS” in 30.7.4.1.5;

aAggPortDebugActorChurnState					ATTRIBUTE

WITH ATTRIBUTE SYNTAX 				IEEE802Dot3-MgmtAttributeModule.ChurnState;

MATCHES FOR				EQUALITY;

BEHAVIOUR 				bAggPortDebugActorChurnState;

REGISTERED AS		{iso(1) std(0) iso8802(8802) csma(3) csmacdmgt(30) attribute(7) aAggPortDebugActorChurnState(171)};

bAggPortDebugActorChurnState					 BEHAVIOUR

DEFINED AS 		See “BEHAVIOUR DEFINED AS” in 30.7.4.1.6;

aAggPortDebugPartnerChurnState					ATTRIBUTE

WITH ATTRIBUTE SYNTAX 				IEEE802Dot3-MgmtAttributeModule.ChurnState;

MATCHES FOR				EQUALITY;

BEHAVIOUR 				bAggPortDebugPartnerChurnState;

REGISTERED AS		{iso(1) std(0) iso8802(8802) csma(3) csmacdmgt(30) attribute(7) aAggPortDebugPartnerChurnState(172)};

bAggPortDebugPartnerChurnState					 BEHAVIOUR

DEFINED AS 		See “BEHAVIOUR DEFINED AS” in 30.7.4.1.7;

aAggPortDebugActorChurnCount					ATTRIBUTE

DERIVED FROM				aCMCounter;

MATCHES FOR				EQUALITY, ORDERING;

BEHAVIOUR 				bAggPortDebugActorChurnCount;

REGISTERED AS		{iso(1) std(0) iso8802(8802) csma(3) csmacdmgt(30) attribute(7) aAggPortDebugActorChurnCount(173)};

bAggPortDebugActorChurnCount				 	BEHAVIOUR

DEFINED AS 		See “BEHAVIOUR DEFINED AS” in 30.7.4.1.8.

NOTE—This counter has a maximum increment rate of 5 counts per second.;

aAggPortDebugPartnerChurnCount					ATTRIBUTE

DERIVED FROM				aCMCounter;

MATCHES FOR				EQUALITY, ORDERING;

BEHAVIOUR 				bAggPortDebugPartnerChurnCount;

REGISTERED AS		{iso(1) std(0) iso8802(8802) csma(3) csmacdmgt(30) attribute(7) aAggPortDebugPartnerChurnCount(174)};

bAggPortDebugPartnerChurnCount				 	BEHAVIOUR

DEFINED AS 		See “BEHAVIOUR DEFINED AS” in 30.7.4.1.9.

NOTE—This counter has a maximum increment rate of 5 counts per second.;

aAggPortDebugActorSyncTransitionCount					ATTRIBUTE

DERIVED FROM				aCMCounter;

MATCHES FOR				EQUALITY, ORDERING;

BEHAVIOUR 				bAggPortDebugActorSyncTransitionCount;

REGISTERED AS		{iso(1) std(0) iso8802(8802) csma(3) csmacdmgt(30) attribute(7) aAggPortDebugActorSyncTransitionCount(175)};

bAggPortDebugActorSyncTransitionCount				 	BEHAVIOUR

DEFINED AS 		See “BEHAVIOUR DEFINED AS” in 30.7.4.1.10

NOTE—This counter has a maximum increment rate of 5 counts per second.;

aAggPortDebugPartnerSyncTransitionCount					ATTRIBUTE

DERIVED FROM				aCMCounter;

MATCHES FOR				EQUALITY, ORDERING;

BEHAVIOUR 				bAggPortDebugPartnerSyncTransitionCount;

REGISTERED AS		{iso(1) std(0) iso8802(8802) csma(3) csmacdmgt(30) attribute(7) aAggPortDebugPartnerSyncTransitionCount(176)};

bAggPortDebugPartnerSyncTransitionCount				 		BEHAVIOUR

DEFINED AS 		See “BEHAVIOUR DEFINED AS” in 30.7.4.1.11

NOTE—This counter has a maximum increment rate of 5 counts per second.;

aAggPortDebugActorChangeCount						ATTRIBUTE

DERIVED FROM				aCMCounter;

MATCHES FOR				EQUALITY, ORDERING;

BEHAVIOUR 				bAggPortDebugActorChangeCount;

REGISTERED AS		{iso(1) std(0) iso8802(8802) csma(3) csmacdmgt(30) attribute(7) aAggPortDebugActorChangeCount(177)};

bAggPortDebugActorChangeCount				 		BEHAVIOUR

DEFINED AS 		See “BEHAVIOUR DEFINED AS” in 30.7.4.1.12

NOTE—This counter has a maximum increment rate of 5 counts per second.;

aAggPortDebugPartnerChangeCount						ATTRIBUTE

DERIVED FROM				aCMCounter;

MATCHES FOR				EQUALITY, ORDERING;

BEHAVIOUR 				bAggPortDebugPartnerChangeCount;

REGISTERED AS		{iso(1) std(0) iso8802(8802) csma(3) csmacdmgt(30) attribute(7) aAggPortDebugPartnerChangeCount(178)};

bAggPortDebugPartnerChangeCount				 	BEHAVIOUR

DEFINED AS 		See “BEHAVIOUR DEFINED AS” in 30.7.4.1.13

NOTE—This counter has a maximum increment rate of 5 counts per second.;

- WIS managed object class

- WIS, formal definition

oWIS					MANAGED OBJECT CLASS

DERIVED FROM				'CCITT Rec. X.721 (1992) | ISO/IEC 10165-2 :1992':top;

CHARACTERIZED BY

pWISBasic			PACKAGE

ATTRIBUTES		aWISID					GET,

aSectionStatus					GET,

aLineStatus					GET,

aPathStatus					GET,

aFarEndPathStatus					GET;

;

;

CONDITIONAL PACKAGES

pWISOptional			PACKAGE

ATTRIBUTES		aSectionSESThreshold					GET-REPLACE,

aSectionSESs					GET,

aSectionESs					GET,

aSectionSEFSs					GET,

aSectionCVs					GET,

aJ0ValueTX 					GET-REPLACE,

aJ0ValueRX 					GET,

aLineSESThreshold					GET-REPLACE,

aLineSESs 					GET,

aLineESs					GET,

aLineCVs					GET,

aFarEndLineSESs					GET,

aFarEndLineESs					GET,

aFarEndLineCVs					GET,

aPathSESThreshold					GET-REPLACE,

aPathSESs					GET,

aPathESs					GET,

aPathCVs					GET,

aJ1ValueTX					GET-REPLACE,

aJ1ValueRX					GET,

aFarEndPathSESs					GET,

aFarEndPathESs					GET,

aFarEndPathCVs					GET;

REGISTERED AS

{iso(1) std(0) iso8802(8802) csma(3) csmacdmgt(30) package(4) wisOptionalPkg(26)};

PRESENT IF		The optional package is implemented;

REGISTERED AS		{iso(1) std(0) iso8802(8802) csma(3) csmacdmgt(30) managedObjectClass(3) wisObjectClass(14)};

nbWIS-mauName					NAME BINDING

SUBORDINATE OBJECT CLASS					oWIS;

NAMED BY SUPERIOR OBJECT CLASS					oMAU AND SUBCLASSES;

WITH ATTRIBUTE					aWISID;

REGISTERED AS		{iso(1) std(0) iso8802(8802) csma(3) csmacdmgt(30) nameBinding(6) wis-mauName(24)};

- WIS attributes

aWISID					ATTRIBUTE

WITH ATTRIBUTE SYNTAX 				IEEE802Dot3-MgmtAttributeModule.OneOfName;

MATCHES FOR				EQUALITY;

BEHAVIOUR 				bWISID;

REGISTERED AS		{iso(1) std(0) iso8802(8802) csma(3) csmacdmgt(30) attribute(7) wisID(181)};

bWISID					BEHAVIOUR

DEFINED AS 		See “BEHAVIOUR DEFINED AS” in 30.8.1.1.1;

aSectionStatus					ATTRIBUTE

WITH ATTRIBUTE SYNTAX 				IEEE802Dot3-MgmtAttributeModule.SectionStatus;

MATCHES FOR				EQUALITY;

BEHAVIOUR 				bSectionStatus;

REGISTERED AS		{iso(1) std(0) iso8802(8802) csma(3) csmacdmgt(30) attribute(7) sectionStatus(182)};

bSectionStatus					BEHAVIOUR

DEFINED AS 		See “BEHAVIOUR DEFINED AS” in 30.8.1.1.2;

aSectionSESThreshold					ATTRIBUTE

WITH ATTRIBUTE SYNTAX 				IEEE802Dot3-MgmtAttributeModule.SESThreshold;

MATCHES FOR				EQUALITY, ORDERING;

BEHAVIOUR 				bSectionSESThreshold;

REGISTERED AS		{iso(1) std(0) iso8802(8802) csma(3) csmacdmgt(30) attribute(7) sectionSESThreshold(183)};

bSectionSESThreshold					BEHAVIOUR

DEFINED AS 		See “BEHAVIOUR DEFINED AS” in 30.8.1.1.3

NOTE—This counter has a maximum increment rate of 1 count per second.;

aSectionSESs					ATTRIBUTE

DERIVED FROM				aCMCounter;

MATCHES FOR				EQUALITY, ORDERING;

BEHAVIOUR 				bSectionSESs;

REGISTERED AS		{iso(1) std(0) iso8802(8802) csma(3) csmacdmgt(30) attribute(7) sectionSESs(184)};

bSectionSESs					BEHAVIOUR

DEFINED AS 		See “BEHAVIOUR DEFINED AS” in 30.8.1.1.4

NOTE—This counter has a maximum increment rate of 1 count per second.;

aSectionESs					ATTRIBUTE

DERIVED FROM				aCMCounter;

MATCHES FOR				EQUALITY, ORDERING;

BEHAVIOUR 				bSectionESs;

REGISTERED AS		{iso(1) std(0) iso8802(8802) csma(3) csmacdmgt(30) attribute(7) sectionESs(185)};

bSectionESs				BEHAVIOUR

DEFINED AS 		See “BEHAVIOUR DEFINED AS” in 30.8.1.1.5;

aSectionSEFSs					ATTRIBUTE

DERIVED FROM				aCMCounter;

MATCHES FOR				EQUALITY, ORDERING;

BEHAVIOUR 				bSectionSEFSs;

REGISTERED AS		{iso(1) std(0) iso8802(8802) csma(3) csmacdmgt(30) attribute(7) sectionSEFSs(186)};

bSectionSEFSs				BEHAVIOUR

DEFINED AS 		See “BEHAVIOUR DEFINED AS” in 30.8.1.1.6

NOTE—This counter has a maximum increment rate of 1 count per second.;

aSectionCVs					ATTRIBUTE

DERIVED FROM				aCMCounter;

MATCHES FOR				EQUALITY, ORDERING;

BEHAVIOUR 				bSectionCVs;

REGISTERED AS		{iso(1) std(0) iso8802(8802) csma(3) csmacdmgt(30) attribute(7) sectionCVs(187)};

bSectionCVs				BEHAVIOUR

DEFINED AS 		See “BEHAVIOUR DEFINED AS” in 30.8.1.1.7

NOTE—This counter has a maximum increment rate of 64 000 counts per second.;

aJ0ValueTX					ATTRIBUTE

WITH ATTRIBUTE SYNTAX 				IEEE802Dot3-MgmtAttributeModule.JValue;

MATCHES FOR				EQUALITY, ORDERING;

BEHAVIOUR 				bJ0ValueTX;

REGISTERED AS		{iso(1) std(0) iso8802(8802) csma(3) csmacdmgt(30) attribute(7) j0ValueTX(188)};

bJ0ValueTX					BEHAVIOUR

DEFINED AS 		See “BEHAVIOUR DEFINED AS” in 30.8.1.1.8;

aJ0ValueRX					ATTRIBUTE

WITH ATTRIBUTE SYNTAX 				IEEE802Dot3-MgmtAttributeModule.JValue;

MATCHES FOR				EQUALITY, ORDERING;

BEHAVIOUR 				bJ0ValueRX;

REGISTERED AS		{iso(1) std(0) iso8802(8802) csma(3) csmacdmgt(30) attribute(7) j0ValueRX(189)};

bJ0ValueRX					BEHAVIOUR

DEFINED AS 		See “BEHAVIOUR DEFINED AS” in 30.8.1.1.9;

aLineStatus					ATTRIBUTE

WITH ATTRIBUTE SYNTAX 				IEEE802Dot3-MgmtAttributeModule.LineStatus;

MATCHES FOR				EQUALITY;

BEHAVIOUR 				bLineStatus;

REGISTERED AS		{iso(1) std(0) iso8802(8802) csma(3) csmacdmgt(30) attribute(7) lineStatus(190)};

bLineStatus					BEHAVIOUR

DEFINED AS 		See “BEHAVIOUR DEFINED AS” in 30.8.1.1.10;

aLineSESThreshold 					ATTRIBUTE

WITH ATTRIBUTE SYNTAX 				IEEE802Dot3-MgmtAttributeModule.SESThreshold;

MATCHES FOR				EQUALITY, ORDERING;

BEHAVIOUR 				bLineSESThreshold;

REGISTERED AS		{iso(1) std(0) iso8802(8802) csma(3) csmacdmgt(30) attribute(7) lineSESThreshold(191)};

bLineSESThreshold					BEHAVIOUR

DEFINED AS 		See “BEHAVIOUR DEFINED AS” in 30.8.1.1.11;

aLineSESs					ATTRIBUTE

DERIVED FROM				aCMCounter;

MATCHES FOR				EQUALITY, ORDERING;

BEHAVIOUR 				bLineSESs;

REGISTERED AS		{iso(1) std(0) iso8802(8802) csma(3) csmacdmgt(30) attribute(7) lineSESs(192)};

bLineSESs					BEHAVIOUR

DEFINED AS 		See “BEHAVIOUR DEFINED AS” in 30.8.1.1.12

NOTE—This counter has a maximum increment rate of 1 count per second.;

aLineESs					ATTRIBUTE

DERIVED FROM				aCMCounter;

MATCHES FOR				EQUALITY, ORDERING;

BEHAVIOUR 				bLineESs;

REGISTERED AS		{iso(1) std(0) iso8802(8802) csma(3) csmacdmgt(30) attribute(7) lineESs(193)};

bLineESs				BEHAVIOUR

DEFINED AS 		See “BEHAVIOUR DEFINED AS” in 30.8.1.1.13

NOTE—This counter has a maximum increment rate of 1 count per second.;

aLineCVs					ATTRIBUTE

DERIVED FROM				aCMCounter;

MATCHES FOR				EQUALITY, ORDERING;

BEHAVIOUR 				bLineCVs;

REGISTERED AS		{iso(1) std(0) iso8802(8802) csma(3) csmacdmgt(30) attribute(7) lineCVs(194)};

bLineCVs				BEHAVIOUR

DEFINED AS 		See “BEHAVIOUR DEFINED AS” in 30.8.1.1.14

NOTE—This counter has a maximum increment rate of 12 288 000 counts per second.;

aFarEndLineSESs					ATTRIBUTE

DERIVED FROM				aCMCounter;

MATCHES FOR				EQUALITY, ORDERING;

BEHAVIOUR 				bFarEndLineSESs;

REGISTERED AS		{iso(1) std(0) iso8802(8802) csma(3) csmacdmgt(30) attribute(7) farEndLineSESs(195)};

bFarEndLineSESs					BEHAVIOUR

DEFINED AS 		See “BEHAVIOUR DEFINED AS” in 30.8.1.1.15

NOTE—This counter has a maximum increment rate of 1 count per second.;

aFarEndLineESs					ATTRIBUTE

DERIVED FROM				aCMCounter;

MATCHES FOR				EQUALITY, ORDERING;

BEHAVIOUR 				bFarEndLineESs;

REGISTERED AS		{iso(1) std(0) iso8802(8802) csma(3) csmacdmgt(30) attribute(7) farEndLineESs(196)};

bFarEndLineESs				BEHAVIOUR

DEFINED AS 		See “BEHAVIOUR DEFINED AS” in 30.8.1.1.16

NOTE—This counter has a maximum increment rate of 1 count per second.;

aFarEndLineCVs					ATTRIBUTE

DERIVED FROM				aCMCounter;

MATCHES FOR				EQUALITY, ORDERING;

BEHAVIOUR 				bFarEndLineCVs;

REGISTERED AS		{iso(1) std(0) iso8802(8802) csma(3) csmacdmgt(30) attribute(7) farEndLineCVs(197)};

bFarEndLineCVs				BEHAVIOUR

DEFINED AS 		See “BEHAVIOUR DEFINED AS” in 30.8.1.1.17

NOTE—This counter has a maximum increment rate of 2 040 000 counts per second.;

aPathStatus					ATTRIBUTE

WITH ATTRIBUTE SYNTAX 				IEEE802Dot3-MgmtAttributeModule.PathStatus;

MATCHES FOR				EQUALITY;

BEHAVIOUR 				bPathStatus;

REGISTERED AS		{iso(1) std(0) iso8802(8802) csma(3) csmacdmgt(30) attribute(7) pathStatus(198)};

bPathStatus					BEHAVIOUR

DEFINED AS 		See “BEHAVIOUR DEFINED AS” in 30.8.1.1.18;

aPathSESThreshold 					ATTRIBUTE

WITH ATTRIBUTE SYNTAX 				IEEE802Dot3-MgmtAttributeModule.SESThreshold;

MATCHES FOR				EQUALITY, ORDERING;

BEHAVIOUR 				bPathSESThreshold;

REGISTERED AS		{iso(1) std(0) iso8802(8802) csma(3) csmacdmgt(30) attribute(7) pathSESThreshold(199)};

bPathSESThreshold					BEHAVIOUR

DEFINED AS 		See “BEHAVIOUR DEFINED AS” in 30.8.1.1.19;

aPathSESs					ATTRIBUTE

DERIVED FROM				aCMCounter;

MATCHES FOR				EQUALITY, ORDERING;

BEHAVIOUR 				bPathSESs;

REGISTERED AS		{iso(1) std(0) iso8802(8802) csma(3) csmacdmgt(30) attribute(7) pathSESs(200)};

bPathSESs					BEHAVIOUR

DEFINED AS 		See “BEHAVIOUR DEFINED AS” in 30.8.1.1.20

NOTE—This counter has a maximum increment rate of 1 count per second.;

aPathESs					ATTRIBUTE

DERIVED FROM				aCMCounter;

MATCHES FOR				EQUALITY, ORDERING;

BEHAVIOUR 				bPathESs;

REGISTERED AS		{iso(1) std(0) iso8802(8802) csma(3) csmacdmgt(30) attribute(7) pathESs(201)};

bPathESs				BEHAVIOUR

DEFINED AS 		See “BEHAVIOUR DEFINED AS” in 30.8.1.1.21

NOTE—This counter has a maximum increment rate of 1 count per second.;

aPathCVs					ATTRIBUTE

DERIVED FROM				aCMCounter;

MATCHES FOR				EQUALITY, ORDERING;

BEHAVIOUR 				bPathCVs;

REGISTERED AS		{iso(1) std(0) iso8802(8802) csma(3) csmacdmgt(30) attribute(7) pathCVs(202)};

bPathCVs				BEHAVIOUR

DEFINED AS 		See “BEHAVIOUR DEFINED AS” in 30.8.1.1.22

NOTE—This counter has a maximum increment rate of 8000 counts per second.;

aJ1ValueTX 					ATTRIBUTE

WITH ATTRIBUTE SYNTAX 				IEEE802Dot3-MgmtAttributeModule.JValue;

MATCHES FOR				EQUALITY, ORDERING;

BEHAVIOUR 				bJ1ValueTX;

REGISTERED AS		{iso(1) std(0) iso8802(8802) csma(3) csmacdmgt(30) attribute(7) j1ValueTX(203)};

bJ1ValueTX					BEHAVIOUR

DEFINED AS 		See “BEHAVIOUR DEFINED AS” in 30.8.1.1.23;

aJ1ValueRX					ATTRIBUTE

WITH ATTRIBUTE SYNTAX 				IEEE802Dot3-MgmtAttributeModule.JValue;

MATCHES FOR				EQUALITY, ORDERING;

BEHAVIOUR 				bJ1ValueRX;

REGISTERED AS		{iso(1) std(0) iso8802(8802) csma(3) csmacdmgt(30) attribute(7) j1ValueRX(204)};

bJ1ValueRX					BEHAVIOUR

DEFINED AS 		See “BEHAVIOUR DEFINED AS” in 30.8.1.1.24;

aFarEndPathStatus					ATTRIBUTE

WITH ATTRIBUTE SYNTAX 				IEEE802Dot3-MgmtAttributeModule.FarEndPathStatus;

MATCHES FOR				EQUALITY;

BEHAVIOUR 				bFarEndPathStatus;

REGISTERED AS		{iso(1) std(0) iso8802(8802) csma(3) csmacdmgt(30) attribute(7) farEndPathStatus(205)};

bFarEndPathStatus					BEHAVIOUR

DEFINED AS 		See “BEHAVIOUR DEFINED AS” in 30.8.1.1.25;

aFarEndPathSESs					ATTRIBUTE

DERIVED FROM				aCMCounter;

MATCHES FOR				EQUALITY, ORDERING;

BEHAVIOUR 				bFarEndPathSESs;

REGISTERED AS		{iso(1) std(0) iso8802(8802) csma(3) csmacdmgt(30) attribute(7) farEndPathSESs(206)};

bFarEndPathSESs					BEHAVIOUR

DEFINED AS 		See “BEHAVIOUR DEFINED AS” in 30.8.1.1.26

NOTE—This counter has a maximum increment rate of 1 count per second.;

aFarEndPathESs					ATTRIBUTE

DERIVED FROM				aCMCounter;

MATCHES FOR				EQUALITY, ORDERING;

BEHAVIOUR 				bFarEndPathESs;

REGISTERED AS		{iso(1) std(0) iso8802(8802) csma(3) csmacdmgt(30) attribute(7) farEndPathESs(207)};

bFarEndPathESs				BEHAVIOUR

DEFINED AS 		See “BEHAVIOUR DEFINED AS” in 30.8.1.1.27

NOTE—This counter has a maximum increment rate of 1 count per second.;

aFarEndPathCVs 					ATTRIBUTE

DERIVED FROM				aCMCounter;

MATCHES FOR				EQUALITY, ORDERING;

BEHAVIOUR 				bFarEndPathCVs;

REGISTERED AS		{iso(1) std(0) iso8802(8802) csma(3) csmacdmgt(30) attribute(7) farEndPathCVs(208)};

bFarEndPathCVs				BEHAVIOUR

DEFINED AS 		See “BEHAVIOUR DEFINED AS” in 30.8.1.1.28

NOTE—This counter has a maximum increment rate of counts 8000 per second.;

- PSE managed object class

- PSE, formal definition

oPSE	MANAGED OBJECT CLASS

DERIVED FROM	'CCITT Rec. X.721 (1992) | ISO/IEC 10165-2 :1992':top;

CHARACTERIZED BY

pPSEBasic	PACKAGE

ATTRIBUTES	aPSEID	GET,
aPSEAdminState		GET,
aPSEPowerPairsControlAbility		GET,
aPSEPowerPairs		GET-REPLACE,
aPSEPowerDetectionStatus		GET;

ACTIONS	acPSEAdminControl;

;

;

CONDITIONAL PACKAGES

pPSERecommended	PACKAGE

ATTRIBUTES	aPSEPowerClassification		GET,
aPSEInvalidSignatureCounter		GET,
aPSEPowerDeniedCounter		GET,
aPSEOverLoadCounter		GET,
aPSEShortCounter		GET,
aPSEMPSAbsentCounter		GET;

REGISTERED AS	{iso(1) std(0) iso8802(8802) csma(3) csmacdmgt(30) package(4) pseRecommendedPkg(27)};

PRESENT IF	The recommended package is implemented;

REGISTERED AS	{iso(1) std(0) iso8802(8802) csma(3) csmacdmgt(30) managedObjectClass(3) pseObjectClass(15)};

pDLLPSEPowerClassification	PACKAGE

ATTRIBUTES	aDLLPowerType		GET,
aMirroredDLLPowerType		GET,
aDLLPowerSource		GET,
aMirroredDLLPowerSource		GET,
aMirroredDLLPDPowerPriority		GET-SET,
aReceivedDLLPDRequestedPowerValue		GET,
aMirroredDLLPDRequestedPowerValue		GET,
aDLLPSEAllocatedPowerValue		GET,
aEchoedDLLPSEAllocatedPowerValue		GET,
LostCommunication	GET
aMirroredLostCommunication	GET
aDLLPSEResponseTime	GET

REGISTERED AS	{iso(1) std(0) iso8802(8802) csma(3) csmacdmgt(30) package(4) dllPSEPowerClassificationPkg(nn)};

PRESENT IF	The DLL Power Classification package is implemented;

nbPSE-repeaterPortName	NAME BINDING

SUBORDINATE OBJECT CLASS	oPSE;

NAMED BY SUPERIOR OBJECT CLASS	oRepeaterPort AND SUBCLASSES;

WITH ATTRIBUTE	aPSEID;

REGISTERED AS	{iso(1) std(0) iso8802(8802) csma(3) csmacdmgt(30) nameBinding(6) pse-repeaterPortName(26)};

nbPSE-dteName	NAME BINDING

SUBORDINATE OBJECT CLASS	oPSE;

NAMED BY SUPERIOR OBJECT CLASS	oPHYEntity AND SUBCLASSES;

WITH ATTRIBUTE	aPSEID;

REGISTERED AS	{iso(1) std(0) iso8802(8802) csma(3) csmacdmgt(30) nameBinding(6) pse-dteName(27)};

nbPSE-pseGroupName	NAME BINDING

SUBORDINATE OBJECT CLASS	oPSE;

NAMED BY SUPERIOR OBJECT CLASS	oPSEGroup AND SUBCLASSES;

WITH ATTRIBUTE	aPSEID;

REGISTERED AS	{iso(1) std(0) iso8802(8802) csma(3) csmacdmgt(30) nameBinding(6) pse-pseGroupName(28)};

- PSE attributes

aPSEID	ATTRIBUTE

WITH ATTRIBUTE SYNTAX 	IEEE802Dot3-MgmtAttributeModule.OneOfName;

MATCHES FOR	EQUALITY;

BEHAVIOUR 	bPSEID;

REGISTERED AS	{iso(1) std(0) iso8802(8802) csma(3) csmacdmgt(30) attribute(7) pseID(209)};

bPSEID	BEHAVIOUR

DEFINED AS 	See “BEHAVIOUR DEFINED AS” in 30.9.1.1.1;

aPSEAdminState	ATTRIBUTE

WITH ATTRIBUTE SYNTAX 	IEEE802Dot3-MgmtAttributeModule.PortAdminState;

MATCHES FOR	EQUALITY;

BEHAVIOUR 	bPSEAdminState;

REGISTERED AS	{iso(1) std(0) iso8802(8802) csma(3) csmacdmgt(30) attribute(7) pseAdminState(210)};

bPSEAdminState	BEHAVIOUR

DEFINED AS 	See “BEHAVIOUR DEFINED AS” in 30.8.1.1.2;

aPSEPowerPairsControlAbility	ATTRIBUTE

WITH ATTRIBUTE SYNTAX 	IEEE802Dot3-MgmtAttributeModule.PairCtrlAbility;

MATCHES FOR	EQUALITY;

BEHAVIOUR 	bPSEPowerPairsControlAbility;

REGISTERED AS	{iso(1) std(0) iso8802(8802) csma(3) csmacdmgt(30) attribute(7) psePowerPairsControlAbility(211)};

bPSEPowerPairsControlAbility	BEHAVIOUR

DEFINED AS 	See “BEHAVIOUR DEFINED AS” in 30.8.1.1.3;

aPSEPowerPairs	ATTRIBUTE

WITH ATTRIBUTE SYNTAX 	IEEE802Dot3-MgmtAttributeModule.PSEPowerPairs;

MATCHES FOR	EQUALITY;

BEHAVIOUR 	bPSEPowerPairs;

REGISTERED AS	{iso(1) std(0) iso8802(8802) csma(3) csmacdmgt(30) attribute(7) psePowerPairs(212)};

bPSEPowerPairs	BEHAVIOUR

DEFINED AS 	See “BEHAVIOUR DEFINED AS” in 30.8.1.1.4;

aPSEPowerDetectionStatus	ATTRIBUTE

WITH ATTRIBUTE SYNTAX 	IEEE802Dot3-MgmtAttributeModule.DetectStatus;

MATCHES FOR	EQUALITY;

BEHAVIOUR 	bPSEPowerDetectionStatus;

REGISTERED AS	{iso(1) std(0) iso8802(8802) csma(3) csmacdmgt(30) attribute(7) psePowerDetectionStatus(214)};

bPSEPowerDetectionStatus	BEHAVIOUR

DEFINED AS 	See “BEHAVIOUR DEFINED AS” in 30.8.1.1.11;

aPSEPowerClassification	ATTRIBUTE

WITH ATTRIBUTE SYNTAX 	IEEE802Dot3-MgmtAttributeModule.PowerClass;

MATCHES FOR	EQUALITY;

BEHAVIOUR 	bPSEPowerClassification;

REGISTERED AS	{iso(1) std(0) iso8802(8802) csma(3) csmacdmgt(30) attribute(7) psePowerClassification(215)};

bPSEPowerClassification	BEHAVIOUR

DEFINED AS 	See “BEHAVIOUR DEFINED AS” in 30.9.1.1.6;

aPSEInvalidSignatureCounter	ATTRIBUTE

DERIVED FROM	aCMCounter;

MATCHES FOR	EQUALITY;

BEHAVIOUR	bPSEInvalidSignatureCounter;

REGISTERED AS	{iso(1) std(0) iso8802(8802) csma(3) csmacdmgt(30) attribute(7)pseInvalidSignatureCounter(227)};

bPSEInvalidSignatureCounter	BEHAVIOUR

DEFINED AS	See “BEHAVIOUR DEFINED AS” in 30.8.1.1.5;

aPSEPowerDeniedCounter	ATTRIBUTE

DERIVED FROM	aCMCounter;

MATCHES FOR	EQUALITY;

BEHAVIOUR	bPSEPowerDeniedCounter;

REGISTERED AS	{iso(1) std(0) iso8802(8802) csma(3) csmacdmgt(30) attribute(7)psePowerDeniedCounter(228)};

bPSEPowerDeniedCounter	BEHAVIOUR

DEFINED AS	See “BEHAVIOUR DEFINED AS” in 30.9.1.1.8;

aPSEOverLoadCounter	ATTRIBUTE

DERIVED FROM	aCMCounter;

MATCHES FOR	EQUALITY;

BEHAVIOUR	bPSEOverLoadCounter;

REGISTERED AS	{iso(1) std(0) iso8802(8802) csma(3) csmacdmgt(30) attribute(7)pseOverLoadCounter(229)};

bPSEOverLoadCounter	BEHAVIOUR

DEFINED AS	See “BEHAVIOUR DEFINED AS” in 30.9.1.1.9;

aPSEShortCounter	ATTRIBUTE

DERIVED FROM	aCMCounter;

MATCHES FOR	EQUALITY;

BEHAVIOUR	bPSEShortCounter;

REGISTERED AS	{iso(1) std(0) iso8802(8802) csma(3) csmacdmgt(30) attribute(7)pseShortCounter(230)};

bPSEShortCounter	BEHAVIOUR

DEFINED AS	See “BEHAVIOUR DEFINED AS” in 30.9.1.1.10;

aPSEMPSAbsentCounter	ATTRIBUTE

DERIVED FROM	aCMCounter;

MATCHES FOR	EQUALITY;

BEHAVIOUR	bPSEMPSAbsentCounter;

REGISTERED AS	{iso(1) std(0) iso8802(8802) csma(3) csmacdmgt(30) attribute(7) pseMPSAbsentCounter(217)};

bPSEMPSAbsentCounter	BEHAVIOUR

DEFINED AS	See “BEHAVIOUR DEFINED AS” in 30.9.1.1.11;

aDLLPowerType	ATTRIBUTE

WITH ATTRIBUTE SYNTAX 	IEEE802Dot3-MgmtAttributeModule.Where?;

MATCHES FOR	WHAT?;

BEHAVIOUR	bDLLPowerType;

REGISTERED AS	{iso(1) std(0) iso8802(8802) csma(3) csmacdmgt(30) attribute(7) dLLPowerType (nnn)};

bDLLPowerType	BEHAVIOUR

DEFINED AS	See “BEHAVIOUR DEFINED AS” in 30.[Wael to provide Pointer/Link];

aMirroredDLLPowerType	ATTRIBUTE

WITH ATTRIBUTE SYNTAX 	IEEE802Dot3-MgmtAttributeModule.Where?;

MATCHES FOR	WHAT?;

BEHAVIOUR	bMirroredDLLPowerType;

REGISTERED AS	{iso(1) std(0) iso8802(8802) csma(3) csmacdmgt(30) attribute(7) mirroredDLLPowerType (nnn)};

bMirroredDLLPowerType	BEHAVIOUR

DEFINED AS	See “BEHAVIOUR DEFINED AS” in 30.[Wael to provide Pointer/Link];

aDLLPowerSource	ATTRIBUTE

WITH ATTRIBUTE SYNTAX 	IEEE802Dot3-MgmtAttributeModule.Where?;

MATCHES FOR	WHAT?;

BEHAVIOUR	bDLLPowerSource;

REGISTERED AS	{iso(1) std(0) iso8802(8802) csma(3) csmacdmgt(30) attribute(7) dLLPowerSource (nnn)};

bDLLPowerSource	BEHAVIOUR

DEFINED AS	See “BEHAVIOUR DEFINED AS” in 30.[Wael to provide Pointer/Link];

aMirroredDLLPowerSource	ATTRIBUTE

WITH ATTRIBUTE SYNTAX 	IEEE802Dot3-MgmtAttributeModule.Where?;

MATCHES FOR	WHAT?;

BEHAVIOUR	bMirroredDLLPowerSource;

REGISTERED AS	{iso(1) std(0) iso8802(8802) csma(3) csmacdmgt(30) attribute(7) mirroredDLLPowerSource (nnn)};

bMirroredDLLPowerSource	BEHAVIOUR

DEFINED AS	See “BEHAVIOUR DEFINED AS” in 30.[Wael to provide Pointer/Link];

aMirroredDLLPDPowerPriority	ATTRIBUTE

WITH ATTRIBUTE SYNTAX 	IEEE802Dot3-MgmtAttributeModule.Where?;

MATCHES FOR	WHAT?;

BEHAVIOUR	bMirroredDLLPDPowerPriority;

REGISTERED AS	{iso(1) std(0) iso8802(8802) csma(3) csmacdmgt(30) attribute(7) mirroredDLLPDPowerPriority (nnn)};

bMirroredDLLPDPowerPriority	BEHAVIOUR

DEFINED AS	See “BEHAVIOUR DEFINED AS” in 30.[Wael to provide Pointer/Link];

aReceivedDLLPDRequestedPowerValue	ATTRIBUTE

WITH ATTRIBUTE SYNTAX 	IEEE802Dot3-MgmtAttributeModule.Where?;

MATCHES FOR	WHAT?;

BEHAVIOUR	bReceivedDLLPDRequestedPowerValue;

REGISTERED AS	{iso(1) std(0) iso8802(8802) csma(3) csmacdmgt(30) attribute(7) receivedDLLPDRequestedPowerValue (nnn)};

bReceivedDLLPDRequestedPowerValue	BEHAVIOUR

DEFINED AS	See “BEHAVIOUR DEFINED AS” in 30.[Wael to provide Pointer/Link];

aMirroredDLLPDRequestedPowerValue	ATTRIBUTE

WITH ATTRIBUTE SYNTAX 	IEEE802Dot3-MgmtAttributeModule.Where?;

MATCHES FOR	WHAT?;

BEHAVIOUR	bMirroredDLLPDRequestedPowerValue;

REGISTERED AS	{iso(1) std(0) iso8802(8802) csma(3) csmacdmgt(30) attribute(7) mirroredDLLPDRequestedPowerValue (nnn)};

bMirroredDLLPDRequestedPowerValue	BEHAVIOUR

DEFINED AS	See “BEHAVIOUR DEFINED AS” in 30.[Wael to provide Pointer/Link];

aDLLPSEAllocatedPowerValue	ATTRIBUTE

WITH ATTRIBUTE SYNTAX 	IEEE802Dot3-MgmtAttributeModule.Where?;

MATCHES FOR	WHAT?;

BEHAVIOUR	bDLLPSEAllocatedPowerValue;

REGISTERED AS	{iso(1) std(0) iso8802(8802) csma(3) csmacdmgt(30) attribute(7) dLLPSEAllocatedPowerValue (nnn)};

bDLLPSEAllocatedPowerValue	BEHAVIOUR

DEFINED AS	See “BEHAVIOUR DEFINED AS” in 30.[Wael to provide Pointer/Link];

aEchoedDLLPSEAllocatedPowerValue	ATTRIBUTE

WITH ATTRIBUTE SYNTAX 	IEEE802Dot3-MgmtAttributeModule.Where?;

MATCHES FOR	WHAT?;

BEHAVIOUR	bEchoedDLLPSEAllocatedPowerValue;

REGISTERED AS	{iso(1) std(0) iso8802(8802) csma(3) csmacdmgt(30) attribute(7) echoedDLLPSEAllocatedPowerValue (nnn)};

bEchoedDLLPSEAllocatedPowerValue	BEHAVIOUR

DEFINED AS	See “BEHAVIOUR DEFINED AS” in 30.[Wael to provide Pointer/Link];

aLostCommunication	ATTRIBUTE

WITH ATTRIBUTE SYNTAX 	IEEE802Dot3-MgmtAttributeModule.Where?;

MATCHES FOR	WHAT?;

BEHAVIOUR	bLostCommunication;

REGISTERED AS	{iso(1) std(0) iso8802(8802) csma(3) csmacdmgt(30) attribute(7) lostCommunication (nnn)};

bLostCommunication	BEHAVIOUR

DEFINED AS	See “BEHAVIOUR DEFINED AS” in 30.[Wael to provide Pointer/Link];

aMirroredLostCommunication	ATTRIBUTE

WITH ATTRIBUTE SYNTAX 	IEEE802Dot3-MgmtAttributeModule.Where?;

MATCHES FOR	WHAT?;

BEHAVIOUR	bMirroredLostCommunication;

REGISTERED AS	{iso(1) std(0) iso8802(8802) csma(3) csmacdmgt(30) attribute(7) mirroredLostCommunication (nnn)};

bMirroredLostCommunication	BEHAVIOUR

DEFINED AS	See “BEHAVIOUR DEFINED AS” in 30.[Wael to provide Pointer/Link];

aDLLPSEResponseTime	ATTRIBUTE

WITH ATTRIBUTE SYNTAX 	IEEE802Dot3-MgmtAttributeModule.Where?;

MATCHES FOR	WHAT?;

BEHAVIOUR	bDLLPSEResponseTime;

REGISTERED AS	{iso(1) std(0) iso8802(8802) csma(3) csmacdmgt(30) attribute(7) dLLPSEResponseTime (nnn)};

bDLLPSEResponseTime	BEHAVIOUR

DEFINED AS	See “BEHAVIOUR DEFINED AS” in 30.[Wael to provide Pointer/Link];

- PSE actions

acPSEAdminControl	ACTION

BEHAVIOUR	bPSEAdminControl;

MODE	CONFIRMED;

WITH INFORMATION SYNTAX 	IEEE802Dot3-MgmtAttributeModule.PortAdminState;

REGISTERED AS	{iso(1) std(0) iso8802(8802) csma(3) csmacdmgt(30) action(9) pseAdminControl(13)};

bPSEAdminControl	BEHAVIOUR

DEFINED AS 	See “BEHAVIOUR DEFINED AS” in 30.9.1.2.1;

- Midspan managed object class

oMidSpan	MANAGED OBJECT CLASS

DERIVED FROM	'CCITT Rec. X.721 (1992) | ISO/IEC 10165-2 :1992':top;

CHARACTERIZED BY

pMidSpanBasic	PACKAGE

ATTRIBUTES	aMidSpanID	GET,
aMidSpanPSEGroupCapacity		GET,
aMidSpanPSEGroupMap		GET;

NOTIFICATIONS 	nMidSpanPSEGroupMapChange;

;

;

REGISTERED AS	{iso(1) std(0) iso8802(8802) csma(3) csmacdmgt(30) managedObjectClass(3) midSpanObjectClass(17)};

nbMidSpanName	NAME BINDING

SUBORDINATE OBJECT CLASS	oMidSpan;

NAMED BY SUPERIOR OBJECT CLASS	“ISO/IEC 10165-2”:system AND SUBCLASSES;

WITH ATTRIBUTE	aMidSpanID;

REGISTERED AS	{iso(1) std(0) iso8802(8802) csma(3) csmacdmgt(30) nameBinding(6) midSpanName(31)};

nbMidSpanMonitor	NAME BINDING

SUBORDINATE OBJECT CLASS	“IEEE802.1F ”:oEWMAMetricMonitor;

NAMED BY SUPERIOR OBJECT CLASS	“ISO/IEC 10165-2”:system AND SUBCLASSES;

WITH ATTRIBUTE	“IEEE802.1F”:aScannerId;

CREATE	WITH-AUTOMATIC-INSTANCE-NAMING;

DELETE	ONLY-IF-NO-CONTAINED-OBJECTS;

REGISTERED AS	{iso(1) std(0) iso8802(8802) csma(3) csmacdmgt(30) nameBinding(6) midSpanMonitor(32)};

- Midspan attributes

aMidSpanID	ATTRIBUTE

WITH ATTRIBUTE SYNTAX 	IEEE802Dot3-MgmtAttributeModule.OneOfName;

MATCHES FOR	EQUALITY;

BEHAVIOUR 	bMidSpanID;

REGISTERED AS	{iso(1) std(0) iso8802(8802) csma(3) csmacdmgt(30) attribute(7) midSpanID(221)};

bMidSpanID	BEHAVIOUR

DEFINED AS 	See “BEHAVIOUR DEFINED AS” in 30.10.1.1.1;

aMidSpanPSEGroupCapacity	ATTRIBUTE

WITH ATTRIBUTE SYNTAX 	IEEE802Dot3-MgmtAttributeModule.OneOfName;

MATCHES FOR	EQUALITY,ORDERING;

BEHAVIOUR 	bMidSpanPSEGroupCapacity;

REGISTERED AS	{iso(1) std(0) iso8802(8802) csma(3) csmacdmgt(30) attribute(7) midSpanPSEGroupCapacity(222)};

bMidSpanPSEGroupCapacity	BEHAVIOUR

DEFINED AS 	See “BEHAVIOUR DEFINED AS” in 30.10.1.1.2;

aMidSpanPSEGroupMap	ATTRIBUTE

WITH ATTRIBUTE SYNTAX 	IEEE802Dot3-MgmtAttributeModule.BitString;

MATCHES FOR	EQUALITY;

BEHAVIOUR 	bMidSpanPSEGroupMap;

REGISTERED AS	{iso(1) std(0) iso8802(8802) csma(3) csmacdmgt(30) attribute(7) midSpanPSEGroupMap(223)};

bMidSpanPSEGroupMap	BEHAVIOUR

DEFINED AS 	See “BEHAVIOUR DEFINED AS” in 30.10.1.1.3;

- Midspan notifications

nMidSpanPSEGroupMapChange	NOTIFICATION

BEHAVIOUR 	bMidSpanPSEGroupMapChange;

WITH INFORMATION SYNTAX	IEEE802Dot3-MgmtAttributeModule.BitString;

REGISTERED AS	{iso(1) std(0) iso8802(8802) csma(3) csmacdmgt(30) notification(10)midSpanPSEGroupMapChange(8)};

bMidSpanPSEGroupMapChange	BEHAVIOUR

DEFINED AS 	See “BEHAVIOUR DEFINED AS” in 30.10.1.2.1;

- PSE Group managed object class

oPSEGroup	MANAGED OBJECT CLASS

DERIVED FROM	'CCITT Rec. X.721 (1992) | ISO/IEC 10165-2 :1992':top;

CHARACTERIZED BY

pPSEGroupBasic	PACKAGE

ATTRIBUTES	aPSEGroupID	GET,
aPSECapacity		GET,
aPSEMap		GET;

NOTIFICATIONS	nPSEMapChange;

;

;

REGISTERED AS	{iso(1) std(0) iso8802(8802) csma(3) csmacdmgt(30) managedObjectClass(3) midSpanGroupObjectClass(18)};

nbPSEGroupName	NAME BINDING

SUBORDINATE OBJECT CLASS	oPSEGroup;

NAMED BY SUPERIOR OBJECT CLASS	oMidSpan AND SUBCLASSES;

WITH ATTRIBUTE	aPSEGroupID;

REGISTERED AS	{iso(1) std(0) iso8802(8802) csma(3) csmacdmgt(30) nameBinding(6) pseGroupName(33)};

- PSE Group attributes

aPSEGroupID	ATTRIBUTE

WITH ATTRIBUTE SYNTAX 	IEEE802Dot3-MgmtAttributeModule.OneOfName;

MATCHES FOR	EQUALITY;

BEHAVIOUR 	bPSEGroupID;

REGISTERED AS	{iso(1) std(0) iso8802(8802) csma(3) csmacdmgt(30) attribute(7) pseGroupID(224)};

bPSEGroupID	BEHAVIOUR

DEFINED AS 	See “BEHAVIOUR DEFINED AS” in 30.10.2.1.1;

aPSECapacity	ATTRIBUTE

WITH ATTRIBUTE SYNTAX 	IEEE802Dot3-MgmtAttributeModule.OneOfName;

MATCHES FOR	EQUALITY,ORDERING;

BEHAVIOUR 	bPSECapacity;

REGISTERED AS	{iso(1) std(0) iso8802(8802) csma(3) csmacdmgt(30) attribute(7) pseCapacity(225)};

bPSECapacity	BEHAVIOUR

DEFINED AS 	See “BEHAVIOUR DEFINED AS” in 30.10.2.1.2;

aPSEMap	ATTRIBUTE

WITH ATTRIBUTE SYNTAX 	IEEE802Dot3-MgmtAttributeModule.BitString;

MATCHES FOR	EQUALITY;

BEHAVIOUR 	bPSEMap;

REGISTERED AS	{iso(1) std(0) iso8802(8802) csma(3) csmacdmgt(30) attribute(7) pseMap(226)};

bPSEMap	BEHAVIOUR

DEFINED AS 	See “BEHAVIOUR DEFINED AS” in 30.10.2.1.3;

- PSE Group notifications

nPSEMapChange	NOTIFICATION

BEHAVIOUR 	bPSEMapChange;

WITH INFORMATION SYNTAX	IEEE802Dot3-MgmtAttributeModule.BitString;

REGISTERED AS	{iso(1) std(0) iso8802(8802) csma(3) csmacdmgt(30) notification(10)pseMapChange(9)};

bPSEMapChange	BEHAVIOUR

DEFINED AS 	See “BEHAVIOUR DEFINED AS” in 30.10.2.2.1;

- OMP entity formal definition

oOMP					MANAGED OBJECT CLASS

DERIVED FROM				'CCITT Rec. X.721 (1992) | ISO/IEC 10165-2:1992':top;

CHARACTERIZED BY

pOMPBasic			PACKAGE

ATTRIBUTES		aOMPEmulationID					GET,

aOMPEmulationType					GET,

aSLDErrors					GET,

aCRC8Errors					GET;

;

;

CONDITIONAL PACKAGES

pOMPOLTMonitor			PACKAGE

ATTRIBUTES		aGoodLLID					GET,

aONUPONcastLLID					GET,

aOLTPONcastLLID					GET,

aBadLLID					GET;

REGISTERED AS		{iso(1) std(0) iso8802(8802) csma(3) csmacdmgt(30) package(4) oMPError(37)};

PRESENT IF		This package is mandatory for a OLT and optional for a ONU.;

REGISTERED AS		{iso(1) std(0) iso8802(8802) csma(3) csmacdmgt(30) managedObjectClass(3) ompObjectClass(19)};

nbOMP-MACEntity					NAME BINDING

SUBORDINATE OBJECT CLASS					oOMP;

NAMED BY SUPERIOR OBJECT CLASS					oMACEntity;

WITH ATTRIBUTE					aOMPEmulationID;

REGISTERED AS		{iso(1) std(0) iso8802(8802) csma(3) csmacdmgt(30) nameBinding(6) omp-macEntity(36)};

- OMP attributes

aOMPEmulationID					ATTRIBUTE

WITH ATTRIBUTE SYNTAX				IEEE802Dot3-MgmtAttributeModule.OneOfName;

MATCHES FOR				EQUALITY, ORDERING;

BEHAVIOUR 				bOMPEmulationID;

REGISTERED AS		{iso(1) std(0) iso8802(8802) csma(3) csmacdmgt(30) attribute(7) ompEmulationID(231)};

bOMPEmulationID				BEHAVIOUR

DEFINED AS 		See “BEHAVIOUR DEFINED AS” in 30.3.7.1.1;

aOMPEmulationType					ATTRIBUTE

WITH ATTRIBUTE SYNTAX				IEEE802Dot3-MgmtAttributeModule.OMPEmulationType;

MATCHES FOR				EQUALITY, ORDERING;

BEHAVIOUR 				bOMPEmulationType;

REGISTERED AS		{iso(1) std(0) iso8802(8802) csma(3) csmacdmgt(30) attribute(7) ompEmulationType(232)};

bOMPEmulationType				BEHAVIOUR

DEFINED AS 		See “BEHAVIOUR DEFINED AS” in 30.3.7.1.2;

aSLDErrors					ATTRIBUTE

DERIVED FROM				aCMCounter;

MATCHES FOR				EQUALITY, ORDERING;

BEHAVIOUR 				bSLDErrors;

REGISTERED AS		{iso(1) std(0) iso8802(8802) csma(3) csmacdmgt(30) attribute(7) sldErrors(233)};

bSLDErrors				BEHAVIOUR

DEFINED AS 		See “BEHAVIOUR DEFINED AS” in 30.3.7.1.3;

aCRC8Errors					ATTRIBUTE

DERIVED FROM				aCMCounter;

MATCHES FOR				EQUALITY, ORDERING;

BEHAVIOUR 				bCRC8Errors;

REGISTERED AS		{iso(1) std(0) iso8802(8802) csma(3) csmacdmgt(30) attribute(7) crc8Errors(234)};

bCRC8Errors				BEHAVIOUR

DEFINED AS 		See “BEHAVIOUR DEFINED AS” in 30.3.7.1.4;

aGoodLLID					ATTRIBUTE

DERIVED FROM				aCMCounter;

MATCHES FOR				EQUALITY, ORDERING;

BEHAVIOUR 				bGoodLLID;

REGISTERED AS		{iso(1) std(0) iso8802(8802) csma(3) csmacdmgt(30) attribute(7) goodLLID(341)};

bGoodLLID				BEHAVIOUR

DEFINED AS 		See “BEHAVIOUR DEFINED AS” in 30.3.7.1.5;

aONUPONcastLLID					ATTRIBUTE

DERIVED FROM				aCMCounter;

MATCHES FOR				EQUALITY, ORDERING;

BEHAVIOUR 				bONUPONcastLLID;

REGISTERED AS		{iso(1) std(0) iso8802(8802) csma(3) csmacdmgt(30) attribute(7) onuPONcastLLID(342)};

bONUPONcastLLID				BEHAVIOUR

DEFINED AS 		See “BEHAVIOUR DEFINED AS” in 30.3.7.1.6;

aOLTPONcastLLID					ATTRIBUTE

DERIVED FROM				aCMCounter;

MATCHES FOR				EQUALITY, ORDERING;

BEHAVIOUR 				bOLTPONcastLLID;

REGISTERED AS		{iso(1) std(0) iso8802(8802) csma(3) csmacdmgt(30) attribute(7) oltPONcastLLID(343)};

bOLTPONcastLLID				BEHAVIOUR

DEFINED AS 		See “BEHAVIOUR DEFINED AS” in 30.3.7.1.7;

aBadLLID					ATTRIBUTE

DERIVED FROM				aCMCounter;

MATCHES FOR				EQUALITY, ORDERING;

BEHAVIOUR 				bBadLLID;

REGISTERED AS		{iso(1) std(0) iso8802(8802) csma(3) csmacdmgt(30) attribute(7) badLLID(235)};

bBadLLID				BEHAVIOUR

DEFINED AS 		See “BEHAVIOUR DEFINED AS” in 30.3.7.1.8;

- OAM entity managed object class

- OAM entity formal definition

oOAM					MANAGED OBJECT CLASS

DERIVED FROM				'CCITT Rec. X.721 (1992) | ISO/IEC 10165-2:1992':top;

CHARACTERIZED BY

pOAMBasic			PACKAGE

ATTRIBUTES		aOAMID					GET,

aOAMAdminState					GET,

aOAMMode					GET-REPLACE,

aOAMDiscoveryState					GET,

aOAMRemoteMACAddress					GET,

aOAMLocalConfiguration					GET,

aOAMRemoteConfiguration					GET,

aOAMLocalPDUConfiguration					GET,

aOAMRemotePDUConfiguration					GET,

aOAMLocalFlagsField					GET,

aOAMRemoteFlagsField					GET,

aOAMLocalRevision					GET,

aOAMRemoteRevision					GET,

aOAMLocalState					GET,

aOAMRemoteState					GET,

aOAMRemoteVendorOUI					GET,

aOAMRemoteVendorSpecificInfo					GET,

aOAMUnsupportedCodesTx					GET,

aOAMUnsupportedCodesRx					GET,

aOAMInformationTx					GET,

aOAMInformationRx					GET,

aOAMUniqueEventNotificationTx					GET,

aOAMDuplicateEventNotificationTx					GET,

aOAMUniqueEventNotificationRx					GET,

aOAMDuplicateEventNotificationRx					GET,

aOAMLoopbackControlTx					GET,

aOAMLoopbackControlRx					GET,

aOAMVariableRequestTx					GET,

aOAMVariableRequestRx					GET,

aOAMVariableResponseTx					GET,

aOAMVariableResponseRx					GET,

aOAMOrganizationSpecificTx					GET,

aOAMOrganizationSpecificRx					GET,

aOAMLocalErrSymPeriodConfig					GET,

aOAMLocalErrSymPeriodEvent					GET,

aOAMLocalErrFrameConfig					GET,

aOAMLocalErrFrameEvent					GET,

aOAMLocalErrFramePeriodConfig					GET,

aOAMLocalErrFramePeriodEvent					GET,

aOAMLocalErrFrameSecsSummaryConfig					GET,

aOAMLocalErrFrameSecsSummaryEvent					GET,

aOAMRemoteErrSymPeriodEvent					GET,

aOAMRemoteErrFrameEvent					GET,

aOAMRemoteErrFramePeriodEvent					GET,

aOAMRemoteErrFrameSecsSummaryEvent					GET,

aFramesLostDueToOAMError					GET;

ACTIONS		acOAMAdminControl;

;

;

REGISTERED AS		{iso(1) std(0) iso8802(8802) csma(3) csmacdmgt(30) managedObjectClass(3) oamObjectClass(20)};

nbOAM-aggregator					NAME BINDING

SUBORDINATE OBJECT CLASS					oOAM;

NAMED BY SUPERIOR OBJECT CLASS					oAggregator;

WITH ATTRIBUTE					aOAMID;

REGISTERED AS		{iso(1) std(0) iso8802(8802) csma(3) csmacdmgt(30) nameBinding(6) oam-aggregator(35)};

- OAM entity attributes

aOAMID					ATTRIBUTE

WITH ATTRIBUTE SYNTAX				IEEE802Dot3-MgmtAttributeModule.OneOfName;

MATCHES FOR				EQUALITY, ORDERING;

BEHAVIOUR 				bOAMID;

REGISTERED AS		{iso(1) std(0) iso8802(8802) csma(3) csmacdmgt(30) attribute(7) oamID(236)};

bOAMID				BEHAVIOUR

DEFINED AS 		See “BEHAVIOUR DEFINED AS” in 30.3.6.1.1;

aOAMAdminState					ATTRIBUTE

WITH ATTRIBUTE SYNTAX				IEEE802Dot3-MgmtAttributeModule.PortAdminState;

MATCHES FOR				EQUALITY, ORDERING;

BEHAVIOUR 				bOAMAdminState;

REGISTERED AS		{iso(1) std(0) iso8802(8802) csma(3) csmacdmgt(30) attribute(7) oamAdminState(237)};

bOAMAdminState				BEHAVIOUR

DEFINED AS 		See “BEHAVIOUR DEFINED AS” in 30.3.6.1.2;

aOAMMode					ATTRIBUTE

WITH ATTRIBUTE SYNTAX				IEEE802Dot3-MgmtAttributeModule.OAMMode;

MATCHES FOR				EQUALITY, ORDERING;

BEHAVIOUR 				bOAMMode;

REGISTERED AS		{iso(1) std(0) iso8802(8802) csma(3) csmacdmgt(30) attribute(7) oamMode(238)};

bOAMMode				BEHAVIOUR

DEFINED AS 		See “BEHAVIOUR DEFINED AS” in 30.3.6.1.3;

aOAMDiscoveryState					ATTRIBUTE

WITH ATTRIBUTE SYNTAX 				IEEE802Dot3-MgmtAttributeModule.OAMDiscoveryState;

MATCHES FOR				EQUALITY, ORDERING;

BEHAVIOUR 				bOAMDiscoveryState;

REGISTERED AS		{iso(1) std(0) iso8802(8802) csma(3) csmacdmgt(30) attribute(7) oamDiscoveryState(333)};

bOAMDiscoveryState				BEHAVIOUR

DEFINED AS 		See “BEHAVIOUR DEFINED AS” in 30.3.6.1.4;

aOAMRemoteMACAddress					ATTRIBUTE

WITH ATTRIBUTE SYNTAX 				IEEE802Dot3-MgmtAttributeModule.MACAddress;

MATCHES FOR				EQUALITY, ORDERING;

BEHAVIOUR 				bOAMRemoteMACAddress;

REGISTERED AS		{iso(1) std(0) iso8802(8802) csma(3) csmacdmgt(30) attribute(7) oamRemoteMACAddress(239)};

bOAMRemoteMACAddress				BEHAVIOUR

DEFINED AS 		See “BEHAVIOUR DEFINED AS” in 30.3.6.1.5;

aOAMLocalConfiguration					ATTRIBUTE

WITH ATTRIBUTE SYNTAX 				IEEE802Dot3-MgmtAttributeModule.OAMConfig;

MATCHES FOR				EQUALITY, ORDERING;

BEHAVIOUR 				bOAMLocalConfiguration;

REGISTERED AS		{iso(1) std(0) iso8802(8802) csma(3) csmacdmgt(30) attribute(7) oamLocalConfiguration(334)};

bOAMLocalConfiguration				BEHAVIOUR

DEFINED AS 		See “BEHAVIOUR DEFINED AS” in 30.3.6.1.6;

aOAMRemoteConfiguration					ATTRIBUTE

WITH ATTRIBUTE SYNTAX 				IEEE802Dot3-MgmtAttributeModule.OAMConfig;

MATCHES FOR				EQUALITY, ORDERING;

BEHAVIOUR 				bOAMRemoteConfiguration;

REGISTERED AS		{iso(1) std(0) iso8802(8802) csma(3) csmacdmgt(30) attribute(7) oamRemoteConfiguration(240)};

bOAMRemoteConfiguration				BEHAVIOUR

DEFINED AS 		See “BEHAVIOUR DEFINED AS” in 30.3.6.1.7;

aOAMLocalPDUConfiguration					ATTRIBUTE

WITH ATTRIBUTE SYNTAX 				IEEE802Dot3-MgmtAttributeModule.Integer16;

MATCHES FOR				EQUALITY, ORDERING;

BEHAVIOUR 				bOAMLocalPDUConfiguration;

REGISTERED AS		{iso(1) std(0) iso8802(8802) csma(3) csmacdmgt(30) attribute(7) oamLocalPDUConfiguration(335)};

bOAMLocalPDUConfiguration				BEHAVIOUR

DEFINED AS 		See “BEHAVIOUR DEFINED AS” in 30.3.6.1.8;

aOAMRemotePDUConfiguration					ATTRIBUTE

WITH ATTRIBUTE SYNTAX 				IEEE802Dot3-MgmtAttributeModule.Integer16;

MATCHES FOR				EQUALITY, ORDERING;

BEHAVIOUR 				bOAMRemotePDUConfiguration;

REGISTERED AS		{iso(1) std(0) iso8802(8802) csma(3) csmacdmgt(30) attribute(7) oamRemotePDUConfiguration(241)};

bOAMRemotePDUConfiguration				BEHAVIOUR

DEFINED AS 		See “BEHAVIOUR DEFINED AS” in 30.3.6.1.9;

aOAMLocalFlagsField					ATTRIBUTE

WITH ATTRIBUTE SYNTAX 				IEEE802Dot3-MgmtAttributeModule.OAMFlagsField;

MATCHES FOR				EQUALITY, ORDERING;

BEHAVIOUR 				bOAMLocalFlagsField;

REGISTERED AS		{iso(1) std(0) iso8802(8802) csma(3) csmacdmgt(30) attribute(7) oamLocalFlagsField(242)};

bOAMLocalFlagsField				BEHAVIOUR

DEFINED AS 		See “BEHAVIOUR DEFINED AS” in 30.3.6.1.10;

aOAMRemoteFlagsField					ATTRIBUTE

WITH ATTRIBUTE SYNTAX 				IEEE802Dot3-MgmtAttributeModule.OAMFlagsField;

MATCHES FOR				EQUALITY, ORDERING;

BEHAVIOUR 				bOAMRemoteFlagsField;

REGISTERED AS		{iso(1) std(0) iso8802(8802) csma(3) csmacdmgt(30) attribute(7) oamRemoteFlagsField(243)};

bOAMRemoteFlagsField				BEHAVIOUR

DEFINED AS 		See “BEHAVIOUR DEFINED AS” in 30.3.6.1.11;

aOAMLocalRevision					ATTRIBUTE

WITH ATTRIBUTE SYNTAX 				IEEE802Dot3-MgmtAttributeModule.Integer16;

MATCHES FOR				EQUALITY, ORDERING;

BEHAVIOUR 				bOAMLocalRevision;

REGISTERED AS		{iso(1) std(0) iso8802(8802) csma(3) csmacdmgt(30) attribute(7) oamLocalRevision(336)};

bOAMLocalRevision				BEHAVIOUR

DEFINED AS 		See “BEHAVIOUR DEFINED AS” in 30.3.6.1.12;

aOAMRemoteRevision					ATTRIBUTE

WITH ATTRIBUTE SYNTAX 				IEEE802Dot3-MgmtAttributeModule.Integer16;

MATCHES FOR				EQUALITY, ORDERING;

BEHAVIOUR 				bOAMRemoteRevision;

REGISTERED AS		{iso(1) std(0) iso8802(8802) csma(3) csmacdmgt(30) attribute(7) oamRemoteRevision(244)};

bOAMRemoteRevision				BEHAVIOUR

DEFINED AS 		See “BEHAVIOUR DEFINED AS” in 30.3.6.1.13;

aOAMLocalState					ATTRIBUTE

WITH ATTRIBUTE SYNTAX 				IEEE802Dot3-MgmtAttributeModule.OAMState;

MATCHES FOR				EQUALITY, ORDERING;

BEHAVIOUR 				bOAMLocalState;

REGISTERED AS		{iso(1) std(0) iso8802(8802) csma(3) csmacdmgt(30) attribute(7) oamLocalState(337)};

bOAMLocalState				BEHAVIOUR

DEFINED AS 		See “BEHAVIOUR DEFINED AS” in 30.3.6.1.14;

aOAMRemoteState					ATTRIBUTE

WITH ATTRIBUTE SYNTAX 				IEEE802Dot3-MgmtAttributeModule.OAMState;

MATCHES FOR				EQUALITY, ORDERING;

BEHAVIOUR 				bOAMRemoteState;

REGISTERED AS		{iso(1) std(0) iso8802(8802) csma(3) csmacdmgt(30) attribute(7) oamRemoteState(245)};

bOAMRemoteState				BEHAVIOUR

DEFINED AS 		See “BEHAVIOUR DEFINED AS” in 30.3.6.1.15;

aOAMRemoteVendorOUI					ATTRIBUTE

WITH ATTRIBUTE SYNTAX 				IEEE802Dot3-MgmtAttributeModule.OUI;

MATCHES FOR				EQUALITY, ORDERING;

BEHAVIOUR 				bOAMRemoteVendorOUI;

REGISTERED AS		{iso(1) std(0) iso8802(8802) csma(3) csmacdmgt(30) attribute(7) oamRemoteVendorOUI(246)};

bOAMRemoteVendorOUI				BEHAVIOUR

DEFINED AS 		See “BEHAVIOUR DEFINED AS” in 30.3.6.1.16;

aOAMRemoteVendorSpecificInfo					ATTRIBUTE

WITH ATTRIBUTE SYNTAX 				IEEE802Dot3-MgmtAttributeModule.Integer32;

MATCHES FOR				EQUALITY, ORDERING;

BEHAVIOUR 				bOamRemoteVendorSpecificInfo;

REGISTERED AS		{iso(1) std(0) iso8802(8802) csma(3) csmacdmgt(30) attribute(7) oamRemoteVendorSpecificInfo(247)};

bOamRemoteVendorSpecificInfo				BEHAVIOUR

DEFINED AS 		See “BEHAVIOUR DEFINED AS” in 30.3.6.1.17;

aOAMUnsupportedCodesTx					ATTRIBUTE

DERIVED FROM				aCMCounter;

MATCHES FOR				EQUALITY, ORDERING;

BEHAVIOUR 				bOAMUnsupportedCodesTx;

REGISTERED AS		{iso(1) std(0) iso8802(8802) csma(3) csmacdmgt(30) attribute(7) oamUnsupportedCodesTx(338)};

bOAMUnsupportedCodesTx				BEHAVIOUR

DEFINED AS 		See “BEHAVIOUR DEFINED AS” in 30.3.6.1.18;

aOAMUnsupportedCodesRx					ATTRIBUTE

DERIVED FROM				aCMCounter;

MATCHES FOR				EQUALITY, ORDERING;

BEHAVIOUR 				bOAMUnsupportedCodesRx;

REGISTERED AS		{iso(1) std(0) iso8802(8802) csma(3) csmacdmgt(30) attribute(7) oamUnsupportedCodesRx(250)};

bOAMUnsupportedCodesRx				BEHAVIOUR

DEFINED AS 		See “BEHAVIOUR DEFINED AS” in 30.3.6.1.19;

aOAMInformationTx					ATTRIBUTE

DERIVED FROM				aCMCounter;

MATCHES FOR				EQUALITY, ORDERING;

BEHAVIOUR 				bOAMInformationTx;

REGISTERED AS		{iso(1) std(0) iso8802(8802) csma(3) csmacdmgt(30) attribute(7) oamInformationTx(251)};

bOAMInformationTx				BEHAVIOUR

DEFINED AS 		See “BEHAVIOUR DEFINED AS” in 30.3.6.1.20;

aOAMInformationRx					ATTRIBUTE

DERIVED FROM				aCMCounter;

MATCHES FOR				EQUALITY, ORDERING;

BEHAVIOUR 				bOAMInformationRx;

REGISTERED AS		{iso(1) std(0) iso8802(8802) csma(3) csmacdmgt(30) attribute(7) oamInformationRx(252)};

bOAMInformationRx				BEHAVIOUR

DEFINED AS 		See “BEHAVIOUR DEFINED AS” in 30.3.6.1.21;

aOAMUniqueEventNotificationTx					ATTRIBUTE

DERIVED FROM				aCMCounter;

MATCHES FOR				EQUALITY, ORDERING;

BEHAVIOUR 				bOAMUniqueEventNotificationTx;

REGISTERED AS		{iso(1) std(0) iso8802(8802) csma(3) csmacdmgt(30) attribute(7) oamUniqueEventNotificationTx(339)};

bOAMUniqueEventNotificationTx				BEHAVIOUR

DEFINED AS 		See “BEHAVIOUR DEFINED AS” in 30.3.6.1.22;

aOAMDuplicateEventNotificationTx					ATTRIBUTE

DERIVED FROM				aCMCounter;

MATCHES FOR				EQUALITY, ORDERING;

BEHAVIOUR 				bOAMDuplicateEventNotificationTx;

REGISTERED AS		{iso(1) std(0) iso8802(8802) csma(3) csmacdmgt(30) attribute(7) oamDuplicateEventNotificationTx(340)};

bOAMDuplicateEventNotificationTx				BEHAVIOUR

DEFINED AS 		See “BEHAVIOUR DEFINED AS” in 30.3.6.1.23;

aOAMUniqueEventNotificationRx					ATTRIBUTE

DERIVED FROM				aCMCounter;

MATCHES FOR				EQUALITY, ORDERING;

BEHAVIOUR 				bOAMUniqueEventNotificationRx;

REGISTERED AS		{iso(1) std(0) iso8802(8802) csma(3) csmacdmgt(30) attribute(7) oamUniqueEventNotificationRx(254)};

bOAMUniqueEventNotificationRx				BEHAVIOUR

DEFINED AS 		See “BEHAVIOUR DEFINED AS” in 30.3.6.1.24;

aOAMDuplicateEventNotificationRx					ATTRIBUTE

DERIVED FROM				aCMCounter;

MATCHES FOR				EQUALITY, ORDERING;

BEHAVIOUR 				bOAMDuplicateEventNotificationRx;

REGISTERED AS		{iso(1) std(0) iso8802(8802) csma(3) csmacdmgt(30) attribute(7) oamDuplicateEventNotificationRx(255)};

bOAMDuplicateEventNotificationRx				BEHAVIOUR

DEFINED AS 		See “BEHAVIOUR DEFINED AS” in 30.3.6.1.25;

aOAMLoopbackControlTx					ATTRIBUTE

DERIVED FROM				aCMCounter;

MATCHES FOR				EQUALITY, ORDERING;

BEHAVIOUR 				bOAMLoopbackControlTx;

REGISTERED AS		{iso(1) std(0) iso8802(8802) csma(3) csmacdmgt(30) attribute(7) oamLoopbackControlTx(256)};

bOAMLoopbackControlTx				BEHAVIOUR

DEFINED AS 		See “BEHAVIOUR DEFINED AS” in 30.3.6.1.26;

aOAMLoopbackControlRx					ATTRIBUTE

DERIVED FROM				aCMCounter;

MATCHES FOR				EQUALITY, ORDERING;

BEHAVIOUR 				bOAMLoopbackControlRx;

REGISTERED AS		{iso(1) std(0) iso8802(8802) csma(3) csmacdmgt(30) attribute(7) oamLoopbackControlRx(257)};

bOAMLoopbackControlRx				BEHAVIOUR

DEFINED AS 		See “BEHAVIOUR DEFINED AS” in 30.3.6.1.27;

aOAMVariableRequestTx					ATTRIBUTE

DERIVED FROM				aCMCounter;

MATCHES FOR				EQUALITY, ORDERING;

BEHAVIOUR 				bOAMVariableRequestTx;

REGISTERED AS		{iso(1) std(0) iso8802(8802) csma(3) csmacdmgt(30) attribute(7) oamVariableRequestTx(258)};

bOAMVariableRequestTx				BEHAVIOUR

DEFINED AS 		See “BEHAVIOUR DEFINED AS” in 30.3.6.1.28;

aOAMVariableRequestRx					ATTRIBUTE

DERIVED FROM				aCMCounter;

MATCHES FOR				EQUALITY, ORDERING;

BEHAVIOUR 				bOAMVariableRequestRx;

REGISTERED AS		{iso(1) std(0) iso8802(8802) csma(3) csmacdmgt(30) attribute(7) oamVariableRequestRx(259)};

bOAMVariableRequestRx				BEHAVIOUR

DEFINED AS 		See “BEHAVIOUR DEFINED AS” in 30.3.6.1.29;

aOAMVariableResponseTx					ATTRIBUTE

DERIVED FROM				aCMCounter;

MATCHES FOR				EQUALITY, ORDERING;

BEHAVIOUR 				bOAMVariableResponseTx;

REGISTERED AS		{iso(1) std(0) iso8802(8802) csma(3) csmacdmgt(30) attribute(7) oamVariableResponseTx(260)};

bOAMVariableResponseTx				BEHAVIOUR

DEFINED AS 		See “BEHAVIOUR DEFINED AS” in 30.3.6.1.30;

aOAMVariableResponseRx					ATTRIBUTE

DERIVED FROM				aCMCounter;

MATCHES FOR				EQUALITY, ORDERING;

BEHAVIOUR 				bOAMVariableResponseRx;

REGISTERED AS		{iso(1) std(0) iso8802(8802) csma(3) csmacdmgt(30) attribute(7) oamVariableResponseRx(261)};

bOAMVariableResponseRx				BEHAVIOUR

DEFINED AS 		See “BEHAVIOUR DEFINED AS” in 30.3.6.1.31;

aOAMOrganizationSpecificTx					ATTRIBUTE

DERIVED FROM				aCMCounter;

MATCHES FOR				EQUALITY, ORDERING;

BEHAVIOUR 				bOAMOrganizationSpecificTx;

REGISTERED AS		{iso(1) std(0) iso8802(8802) csma(3) csmacdmgt(30) attribute(7) oamOrganizationSpecificTx(262)};

bOAMOrganizationSpecificTx				BEHAVIOUR

DEFINED AS 		See “BEHAVIOUR DEFINED AS” in 30.3.6.1.32;

aOAMOrganizationSpecificRx					ATTRIBUTE

DERIVED FROM				aCMCounter;

MATCHES FOR				EQUALITY, ORDERING;

BEHAVIOUR 				bOAMOrganizationSpecificRx;

REGISTERED AS		{iso(1) std(0) iso8802(8802) csma(3) csmacdmgt(30) attribute(7) oamOrganizationSpecificRx(263)};

bOAMOrganizationSpecificRx				BEHAVIOUR

DEFINED AS 		See “BEHAVIOUR DEFINED AS” in 30.3.6.1.33;

aOAMLocalErrSymPeriodConfig					ATTRIBUTE

WITH ATTRIBUTE SYNTAX 				IEEE802Dot3-MgmtAttributeModule.OAMErrorConf;

MATCHES FOR				EQUALITY, ORDERING;

BEHAVIOUR 				bOAMLocalErrSymPeriodConfig;

REGISTERED AS		{iso(1) std(0) iso8802(8802) csma(3) csmacdmgt(30) attribute(7) oamLocalErrSymPeriodConfig(264)};

bOAMLocalErrSymPeriodConfig				BEHAVIOUR

DEFINED AS 		See “BEHAVIOUR DEFINED AS” in 30.3.6.1.34;

aOAMLocalErrSymPeriodEvent					ATTRIBUTE

WITH ATTRIBUTE SYNTAX 				IEEE802Dot3-MgmtAttributeModule.OAMErrorEvent;

MATCHES FOR				EQUALITY, ORDERING;

BEHAVIOUR 				bOAMLocalErrSymPeriodEvent;

REGISTERED AS		{iso(1) std(0) iso8802(8802) csma(3) csmacdmgt(30) attribute(7) oamLocalErrSymPeriodEvent(265)};

bOAMLocalErrSymPeriodEvent				BEHAVIOUR

DEFINED AS 		See “BEHAVIOUR DEFINED AS” in 30.3.6.1.35;

aOAMLocalErrFrameConfig					ATTRIBUTE

WITH ATTRIBUTE SYNTAX 				IEEE802Dot3-MgmtAttributeModule.OAMErrorConf;

MATCHES FOR				EQUALITY, ORDERING;

BEHAVIOUR 				bOAMLocalErrFrameConfig;

REGISTERED AS		{iso(1) std(0) iso8802(8802) csma(3) csmacdmgt(30) attribute(7) oamLocalErrFrameConfig(266)};

bOAMLocalErrFrameConfig				BEHAVIOUR

DEFINED AS 		See “BEHAVIOUR DEFINED AS” in 30.3.6.1.36;

aOAMLocalErrFrameEvent					ATTRIBUTE

WITH ATTRIBUTE SYNTAX 				IEEE802Dot3-MgmtAttributeModule.OAMErrorEvent;

MATCHES FOR				EQUALITY, ORDERING;

BEHAVIOUR 				bOAMLocalErrFrameEvent;

REGISTERED AS		{iso(1) std(0) iso8802(8802) csma(3) csmacdmgt(30) attribute(7) oamLocalErrFrameEvent(267)};

bOAMLocalErrFrameEvent				BEHAVIOUR

DEFINED AS 		See “BEHAVIOUR DEFINED AS” in 30.3.6.1.37;

aOAMLocalErrFramePeriodConfig					ATTRIBUTE

WITH ATTRIBUTE SYNTAX 				IEEE802Dot3-MgmtAttributeModule.OAMErrorConf;

MATCHES FOR				EQUALITY, ORDERING;

BEHAVIOUR 				bOAMLocalErrFramePeriodConfig;

REGISTERED AS		{iso(1) std(0) iso8802(8802) csma(3) csmacdmgt(30) attribute(7) oamLocalErrFramePeriodConfig(268)};

bOAMLocalErrFramePeriodConfig				BEHAVIOUR

DEFINED AS 		See “BEHAVIOUR DEFINED AS” in 30.3.6.1.38;

aOAMLocalErrFramePeriodEvent					ATTRIBUTE

WITH ATTRIBUTE SYNTAX 				IEEE802Dot3-MgmtAttributeModule.OAMErrorEvent;

MATCHES FOR				EQUALITY, ORDERING;

BEHAVIOUR 				bOAMLocalErrFramePeriodEvent;

REGISTERED AS		{iso(1) std(0) iso8802(8802) csma(3) csmacdmgt(30) attribute(7) oamLocalErrFramePeriodEvent(269)};

bOAMLocalErrFramePeriodEvent				BEHAVIOUR

DEFINED AS 		See “BEHAVIOUR DEFINED AS” in 30.3.6.1.39;

aOAMLocalErrFrameSecsSummaryConfig					ATTRIBUTE

WITH ATTRIBUTE SYNTAX 				IEEE802Dot3-MgmtAttributeModule.OAMErrorConf;

MATCHES FOR				EQUALITY, ORDERING;

BEHAVIOUR 				bOAMLocalErrFrameSecsSummaryConfig;

REGISTERED AS		{iso(1) std(0) iso8802(8802) csma(3) csmacdmgt(30) attribute(7) oamLocalErrFrameSecsSummaryConfig(270)};

bOAMLocalErrFrameSecsSummaryConfig				BEHAVIOUR

DEFINED AS 		See “BEHAVIOUR DEFINED AS” in 30.3.6.1.40;

aOAMLocalErrFrameSecsSummaryEvent					ATTRIBUTE

WITH ATTRIBUTE SYNTAX 				IEEE802Dot3-MgmtAttributeModule.OAMErrorEvent;

MATCHES FOR				EQUALITY, ORDERING;

BEHAVIOUR 				bOAMLocalErrFrameSecsSummaryEvent;

REGISTERED AS		{iso(1) std(0) iso8802(8802) csma(3) csmacdmgt(30) attribute(7) oamLocalErrFrameSecsSummaryEvent(271)};

bOAMLocalErrFrameSecsSummaryEvent				BEHAVIOUR

DEFINED AS 		See “BEHAVIOUR DEFINED AS” in 30.3.6.1.41;

aOAMRemoteErrSymPeriodEvent					ATTRIBUTE

WITH ATTRIBUTE SYNTAX 				IEEE802Dot3-MgmtAttributeModule.OAMErrorEvent;

MATCHES FOR				EQUALITY, ORDERING;

BEHAVIOUR 				bOAMRemoteErrSymPeriodEvent;

REGISTERED AS		{iso(1) std(0) iso8802(8802) csma(3) csmacdmgt(30) attribute(7) oamRemoteErrSymPeriodEvent(272)};

bOAMRemoteErrSymPeriodEvent				BEHAVIOUR

DEFINED AS 		See “BEHAVIOUR DEFINED AS” in 30.3.6.1.42;

aOAMRemoteErrFrameEvent					ATTRIBUTE

WITH ATTRIBUTE SYNTAX 				IEEE802Dot3-MgmtAttributeModule.OAMErrorEvent;

MATCHES FOR				EQUALITY, ORDERING;

BEHAVIOUR 				bOAMRemoteErrFrameEvent;

REGISTERED AS		{iso(1) std(0) iso8802(8802) csma(3) csmacdmgt(30) attribute(7) oamRemoteErrFrameEvent(273)};

bOAMRemoteErrFrameEvent				BEHAVIOUR

DEFINED AS 		See “BEHAVIOUR DEFINED AS” in 30.3.6.1.43;

aOAMRemoteErrFramePeriodEvent					ATTRIBUTE

WITH ATTRIBUTE SYNTAX 				IEEE802Dot3-MgmtAttributeModule.OAMErrorEvent;

MATCHES FOR				EQUALITY, ORDERING;

BEHAVIOUR 				bOAMRemoteErrFramePeriodEvent;

REGISTERED AS		{iso(1) std(0) iso8802(8802) csma(3) csmacdmgt(30) attribute(7) oamRemoteErrFramePeriodEvent(274)};

bOAMRemoteErrFramePeriodEvent				BEHAVIOUR

DEFINED AS 		See “BEHAVIOUR DEFINED AS” in 30.3.6.1.44;

aOAMRemoteErrFrameSecsSummaryEvent					ATTRIBUTE

WITH ATTRIBUTE SYNTAX 				IEEE802Dot3-MgmtAttributeModule.OAMErrorEvent;

MATCHES FOR				EQUALITY, ORDERING;

BEHAVIOUR 				bOAMRemoteErrFrameSecsSummaryEvent;

REGISTERED AS		{iso(1) std(0) iso8802(8802) csma(3) csmacdmgt(30) attribute(7) oamRemoteErrFrameSecsSummaryEvent(275)};

bOAMRemoteErrFrameSecsSummaryEvent				BEHAVIOUR

DEFINED AS 		See “BEHAVIOUR DEFINED AS” in 30.3.6.1.45;

aFramesLostDueToOAMError					ATTRIBUTE

DERIVED FROM				aCMCounter;

MATCHES FOR				EQUALITY, ORDERING;

BEHAVIOUR 				bFramesLostDueToOAMError;

REGISTERED AS		{iso(1) std(0) iso8802(8802) csma(3) csmacdmgt(30) attribute(7) framesLostDueToOAMError(276)};

bFramesLostDueToOAMError				BEHAVIOUR

DEFINED AS 		See “BEHAVIOUR DEFINED AS” in 30.3.6.1.46;

- OAM actions

acOAMAdminControl 						 ACTION

BEHAVIOUR	bOAMAdminControl;

MODE 	CONFIRMED;

WITH INFORMATION SYNTAX	IEEE802Dot3-MgmtAttributeModule.AdminState;

REGISTERED AS	{iso(1) std(0) iso8802(8802) csma(3) csmacdmgt(30) action(9) oamAdminControl(15)};

bOAMAdminControl 						 BEHAVIOUR

DEFINED AS	See “BEHAVIOUR DEFINED AS” in 30.3.6.2.1;

- PAF managed object class

- PAF formal definition

oPAF					MANAGED OBJECT CLASS

DERIVED FROM				'CCITT Rec. X.721 (1992) | ISO/IEC 10165-2:1992':top;

CHARACTERIZED BY

pPAFBasic			PACKAGE

ATTRIBUTES		aPAFID					GET,

aPhyEnd					GET,

aPHYCurrentStatus					GET,

aPAFSupported					GET;

;

;

CONDITIONAL PACKAGES

pPAFAggregation PACKAGE

ATTRIBUTES		aPAFAdminState					GET-REPLACE,

aLocalPAFCapacity					GET,

aLocalPMEAvailable					GET,

aLocalPMEAggregate					GET,

aRemotePAFSupported					GET,

aRemotePAFCapacity					GET,

aRemotePMEAggregate					GET;

REGISTERED AS		{iso(1) std(0) iso8802(8802) csma(3) csmacdmgt(30) package(4) pafAggregation(38)};

PRESENT IF		The PAF aggregation capability is implemented.;

REGISTERED AS		{iso(1) std(0) iso8802(8802) csma(3) csmacdmgt(30) managedObjectClass(3) pafObjectClass(24)};

nbPAF-PHYEntity					NAME BINDING

SUBORDINATE OBJECT CLASS					oPAF;

NAMED BY SUPERIOR OBJECT CLASS					oPHYEntity;

WITH ATTRIBUTE					aPAFID;

REGISTERED AS		{iso(1) std(0) iso8802(8802) csma(3) csmacdmgt(30) nameBinding(6) paf-phyEntity(39)};

aPAFID					ATTRIBUTE

WITH ATTRIBUTE SYNTAX				IEEE802Dot3-MgmtAttributeModule.OneOfName;

MATCHES FOR				EQUALITY, ORDERING;

BEHAVIOUR 				bPAFID;

REGISTERED AS		{iso(1) std(0) iso8802(8802) csma(3) csmacdmgt(30) attribute(7) pafID(344)};

bPAFID				BEHAVIOUR

DEFINED AS 		See “BEHAVIOUR DEFINED AS” in 30.11.1.1.1;

aPhyEnd					ATTRIBUTE

WITH ATTRIBUTE SYNTAX				IEEE802Dot3-MgmtAttributeModule.PHYEnd;

MATCHES FOR				EQUALITY, ORDERING;

BEHAVIOUR 				bPhyEnd;

REGISTERED AS		{iso(1) std(0) iso8802(8802) csma(3) csmacdmgt(30) attribute(7) phyEnd(326)};

bPhyEnd				BEHAVIOUR

DEFINED AS 		See “BEHAVIOUR DEFINED AS” in 30.11.1.1.2;

aPHYCurrentStatus					ATTRIBUTE

WITH ATTRIBUTE SYNTAX				IEEE802Dot3-MgmtAttributeModule.PHYCurrentState;

MATCHES FOR				EQUALITY, ORDERING;

BEHAVIOUR 				bPHYCurrentStatus;

REGISTERED AS		{iso(1) std(0) iso8802(8802) csma(3) csmacdmgt(30) attribute(7) phyCurrentStatus(296)};

bPHYCurrentStatus				BEHAVIOUR

DEFINED AS 		See “BEHAVIOUR DEFINED AS” in 30.11.1.1.3;

aPAFSupported					ATTRIBUTE

WITH ATTRIBUTE SYNTAX				IEEE802Dot3-MgmtAttributeModule.TrueFalse;

MATCHES FOR				EQUALITY, ORDERING;

BEHAVIOUR 				bPAFSupported;

REGISTERED AS		{iso(1) std(0) iso8802(8802) csma(3) csmacdmgt(30) attribute(7) pAFSupported(304)};

bPAFSupported				BEHAVIOUR

DEFINED AS 		See “BEHAVIOUR DEFINED AS” in 30.11.1.1.4;

aPAFAdminState					ATTRIBUTE

WITH ATTRIBUTE SYNTAX				IEEE802Dot3-MgmtAttributeModule.PortAdminState;

MATCHES FOR				EQUALITY, ORDERING;

BEHAVIOUR 				bPAFAdminState;

REGISTERED AS		{iso(1) std(0) iso8802(8802) csma(3) csmacdmgt(30) attribute(7) pAFAdminState(305)};

bPAFAdminState				BEHAVIOUR

DEFINED AS 		See “BEHAVIOUR DEFINED AS” in 30.11.1.1.5;

aLocalPAFCapacity					ATTRIBUTE

WITH ATTRIBUTE SYNTAX				IEEE802Dot3-MgmtAttributeModule.Integer8;

MATCHES FOR				EQUALITY, ORDERING;

BEHAVIOUR 				 bLocalPAFCapacity;

REGISTERED AS		{iso(1) std(0) iso8802(8802) csma(3) csmacdmgt(30) attribute(7) localPAFCapacity(327)};

bLocalPAFCapacity				BEHAVIOUR

DEFINED AS 		See “BEHAVIOUR DEFINED AS” in 30.11.1.1.6;

aLocalPMEAvailable					ATTRIBUTE

WITH ATTRIBUTE SYNTAX				IEEE802Dot3-MgmtAttributeModule.Integer32;

MATCHES FOR				EQUALITY, ORDERING;

BEHAVIOUR 				 bLocalPMEAvailable;

REGISTERED AS		{iso(1) std(0) iso8802(8802) csma(3) csmacdmgt(30) attribute(7) localPMEAvailable(306)};

bLocalPMEAvailable				BEHAVIOUR

DEFINED AS 		See “BEHAVIOUR DEFINED AS” in 30.11.1.1.7;

aLocalPMEAggregate					ATTRIBUTE

WITH ATTRIBUTE SYNTAX				IEEE802Dot3-MgmtAttributeModule.Integer32;

MATCHES FOR				EQUALITY, ORDERING;

BEHAVIOUR 				 bLocalPMEAggregate;

REGISTERED AS		{iso(1) std(0) iso8802(8802) csma(3) csmacdmgt(30) attribute(7) localPMEAggregate(307)};

bLocalPMEAggregate				BEHAVIOUR

DEFINED AS 		See “BEHAVIOUR DEFINED AS” in 30.11.1.1.8;

aRemotePAFSupported					ATTRIBUTE

WITH ATTRIBUTE SYNTAX				IEEE802Dot3-MgmtAttributeModule.TrueFalse;

MATCHES FOR				EQUALITY, ORDERING;

BEHAVIOUR 				bRemotePAFSupported;

REGISTERED AS		{iso(1) std(0) iso8802(8802) csma(3) csmacdmgt(30) attribute(7) remotePAFSupported(328)};

bRemotePAFSupported				BEHAVIOUR

DEFINED AS 		See “BEHAVIOUR DEFINED AS” in 30.11.1.1.9;

aRemotePAFCapacity					ATTRIBUTE

WITH ATTRIBUTE SYNTAX				IEEE802Dot3-MgmtAttributeModule.Integer8;

MATCHES FOR				EQUALITY, ORDERING;

BEHAVIOUR 				 bRemotePAFCapacity;

REGISTERED AS		{iso(1) std(0) iso8802(8802) csma(3) csmacdmgt(30) attribute(7) remotePAFCapacity(329)};

bRemotePAFCapacity				BEHAVIOUR

DEFINED AS 		See “BEHAVIOUR DEFINED AS” in 30.11.1.1.10;

aRemotePMEAggregate					ATTRIBUTE

WITH ATTRIBUTE SYNTAX				IEEE802Dot3-MgmtAttributeModule.Integer32;

MATCHES FOR				EQUALITY, ORDERING;

BEHAVIOUR 				bRemotePMEAggregate;

REGISTERED AS		{iso(1) std(0) iso8802(8802) csma(3) csmacdmgt(30) attribute(7) remotePMEAggregate(310)};

bRemotePMEAggregate				BEHAVIOUR

DEFINED AS 		See “BEHAVIOUR DEFINED AS” in 30.11.1.1.11;

- PME managed object class

- PME formal definition

oPME					MANAGED OBJECT CLASS

DERIVED FROM				'CCITT Rec. X.721 (1992) | ISO/IEC 10165-2:1992':top;

CHARACTERIZED BY

pPMEBasic			PACKAGE

ATTRIBUTES		aPMEID					GET,

aPMEAdminState					GET-REPLACE,

aPMEStatus					GET,

aPMESNRMgn					GET,

aTCCodingViolations					GET,

aProfileSelect					GET-REPLACE,

aOperatingProfile					GET,

aPMEFECCorrectedBlocks					GET,

aPMEFECUncorrectableBlocks					GET,

aTCCRCErrors					GET;

;

;

REGISTERED AS		{iso(1) std(0) iso8802(8802) csma(3) csmacdmgt(30) managedObjectClass(3) pmeObjectClass(25)};

nbPME-PAF					NAME BINDING

SUBORDINATE OBJECT CLASS					oPME;

NAMED BY SUPERIOR OBJECT CLASS					oPAF;

WITH ATTRIBUTE					aPAFID;

REGISTERED AS		{iso(1) std(0) iso8802(8802) csma(3) csmacdmgt(30) nameBinding(6) pme-paf(40)};

aPMEID					ATTRIBUTE

WITH ATTRIBUTE SYNTAX				IEEE802Dot3-MgmtAttributeModule.OneOfName;

MATCHES FOR				EQUALITY, ORDERING;

BEHAVIOUR 				bPMEID;

REGISTERED AS		{iso(1) std(0) iso8802(8802) csma(3) csmacdmgt(30) attribute(7) pmeID(330)};

bPMEID				BEHAVIOUR

DEFINED AS 		See “BEHAVIOUR DEFINED AS” in 30.11.2.1.1;

aPMEAdminState					ATTRIBUTE

WITH ATTRIBUTE SYNTAX				IEEE802Dot3-MgmtAttributeModule.PortAdminState;

MATCHES FOR				EQUALITY, ORDERING;

BEHAVIOUR 				bPMEAdminState;

REGISTERED AS		{iso(1) std(0) iso8802(8802) csma(3) csmacdmgt(30) attribute(7) pmeAdminState(345)};

bPMEAdminState				BEHAVIOUR

DEFINED AS 		See “BEHAVIOUR DEFINED AS” in 30.11.2.1.2;

aPMEStatus					ATTRIBUTE

WITH ATTRIBUTE SYNTAX				IEEE802Dot3-MgmtAttributeModule.PMEStatus;

MATCHES FOR				EQUALITY, ORDERING;

BEHAVIOUR 				bPMEStatus;

REGISTERED AS		{iso(1) std(0) iso8802(8802) csma(3) csmacdmgt(30) attribute(7) pmeStatus(346)};

bPMEStatus				BEHAVIOUR

DEFINED AS 		See “BEHAVIOUR DEFINED AS” in 30.11.2.1.3;

aPMESNRMgn					ATTRIBUTE

WITH ATTRIBUTE SYNTAX				IEEE802Dot3-MgmtAttributeModule.Integer8;

MATCHES FOR				EQUALITY, ORDERING;

BEHAVIOUR 				bPMESNRMgn;

REGISTERED AS		{iso(1) std(0) iso8802(8802) csma(3) csmacdmgt(30) attribute(7) pmeSNRMgn(331)};

bPMESNRMgn				BEHAVIOUR

DEFINED AS 		See “BEHAVIOUR DEFINED AS” in 30.11.2.1.4;

aTCCodingViolations					ATTRIBUTE

DERIVED FROM				aCMCounter;

MATCHES FOR				EQUALITY, ORDERING;

BEHAVIOUR 				bTCCodingViolations;

REGISTERED AS		{iso(1) std(0) iso8802(8802) csma(3) csmacdmgt(30) attribute(7) tcCodingViolations(332)};

bTCCodingViolations				BEHAVIOUR

DEFINED AS 		See “BEHAVIOUR DEFINED AS” in 30.11.2.1.5;

aProfileSelect					ATTRIBUTE

WITH ATTRIBUTE SYNTAX				IEEE802Dot3-MgmtAttributeModule.PMEProfile;

MATCHES FOR				EQUALITY, ORDERING;

BEHAVIOUR 				bProfileSelect;

REGISTERED AS		{iso(1) std(0) iso8802(8802) csma(3) csmacdmgt(30) attribute(7) profileSelect(347)};

bProfileSelect				BEHAVIOUR

DEFINED AS 		See “BEHAVIOUR DEFINED AS” in 30.11.2.1.6;

aOperatingProfile				ATTRIBUTE

WITH ATTRIBUTE SYNTAX				IEEE802Dot3-MgmtAttributeModule.PMEOperatingProfile;

MATCHES FOR				EQUALITY, ORDERING;

BEHAVIOUR 				bOperatingProfile;

REGISTERED AS		{iso(1) std(0) iso8802(8802) csma(3) csmacdmgt(30) attribute(7) operatingProfile(348)};

bOperatingProfile				BEHAVIOUR

DEFINED AS 		See “BEHAVIOUR DEFINED AS” in 30.11.2.1.7;

aPMEFECCorrectedBlocks					ATTRIBUTE

DERIVED FROM				aCMCounter;

MATCHES FOR				EQUALITY, ORDERING;

BEHAVIOUR 				bPMEFECCorrectedBlocks;

REGISTERED AS		{iso(1) std(0) iso8802(8802) csma(3) csmacdmgt(30) attribute(7) pmeFECCorrectedBlocks(349)};

bPMEFECCorrectedBlocks				BEHAVIOUR

DEFINED AS 		See “BEHAVIOUR DEFINED AS” in 30.11.2.1.8;

aPMEFECUncorrectableBlocks					ATTRIBUTE

DERIVED FROM				aCMCounter;

MATCHES FOR				EQUALITY, ORDERING;

BEHAVIOUR 				bPMEFECUncorrectableBlocks;

REGISTERED AS		{iso(1) std(0) iso8802(8802) csma(3) csmacdmgt(30) attribute(7) pmeFECUncorrectableBlocks(350)};

bPMEFECUncorrectableBlocks				BEHAVIOUR

DEFINED AS 		See “BEHAVIOUR DEFINED AS” in 30.11.2.1.9;

aTCCRCErrors					ATTRIBUTE

DERIVED FROM 				aCMCounter;

MATCHES FOR				EQUALITY, ORDERING;

BEHAVIOUR				bTCCRCErrors;

REGISTERED AS		{iso(1) std(0) iso8802(8802) csma(3) csmacdmgt(30) attribute(7) tcCRCErrors(352)};

bTCCRCErrors BEHAVIOUR

DEFINED AS		See “BEHAVIOUR DEFINED AS” in 30.11.2.1.10;

GDMO and ASN.1 definitions for management

- Common attributes template

This template defines generic facilities that are used in the standard.

aCMCounter      ATTRIBUTE

DERIVED FROM	“ISO/IEC 10165-5”:genericWrappingCounter;

BEHAVIOUR	bCMCounter;

REGISTERED AS	{iso(1) std(0) iso8802(8802) csma(3) csmacdmgt(30) attribute(7) cmCounter(88)};

bCMCounter     BEHAVIOUR

DEFINED AS	Wraps at one of two sizes. Size is conditional.
Wraps at 32 bits, that is this counter reaches its maximum value at 232 –1 (i.e., approximately 4.294109) and then rolls over to zero on the next increment, if maximum increment rate from zero causes a rollover in 58 min or more.
Wraps at 64 bits, that is this counter reaches its maximum value at 264 –1 (i.e., approximately 1.844 . . . 1019) and then rolls over to zero on the next increment, if maximum increment rate from zero would cause a 32 bit counter to roll over in less than 58 min.
The counter that this is derived from initializes to zero. Initialization to zero is not a requirement of this standard;

- ASN.1 module for CSMA/CD managed objects

This ASN.1 module defines the ASN.1 types and subtypes that are referred to immediately after the WITH ATTRIBUTE SYNTAX construct in this clause’s uses of the attribute template defined in ISO/IEC 10165-4:1992, Guidelines for the definition of managed objects (GDMO).

IEEE802Dot3-MgmtAttributeModule {iso(1) std(0) iso8802(8802) csma(3) global(1) asn1Module(2) commonDefinitions(0) version(2)} DEFINITIONS IMPLICIT TAGS::= BEGIN

EXPORTS	--everything

IMPORTS	--implicitly imports ISO 8824:1990

MACAddress	
FROM IEEE802CommonDefinitions
{iso(1) member-body(2) us(840) ieee802dot1partF(10011) 
asn1Module(2) commonDefinitions(0) version1(0)};

AdminState::= ENUMERATED {
other	(1),	--undefined
unknown	(2),	--initializing, true state not yet known
operational	(3),	--powered and connected
standby	(4),	--inactive but on
shutdown	(5)	--similar to power down
}

AggDataRate ::= INTEGER (0..2^32-1)		--The data rate of an Aggregation

AggID ::= INTEGER (0..2^32-1)

AggOrInd ::= BOOLEAN

AggPortID ::= INTEGER (0..2^32-1)

AggPortList ::= SEQUENCE OF AggPortID

AggPortState ::= BIT STRING (SIZE (1..8))

AggState ::= ENUMERATED {
up	(0), 	--operational
down	(1) 	--disabled
}

AttemptArray::= SEQUENCE OF aCMCounter	--array [1..attempt limit - 1]

AutoNegAdminState::= ENUMERATED {
disabled	(1),
enabled	(2)
}

AutoNegAutoConfig::= ENUMERATED {
other	(1),
configuring	(2),
complete	(3),
disabled	(4),
parallel detect fail	(5)
}

AutoNegRemoteSignalingDetect::= ENUMERATED {
detected	(1),
notdetected	(2)
}

AutoNegSelector::= ENUMERATED {
other	(1),	--undefined
ethernet	(2),	--802.3
isoethernet	(3)	--802.9
}

AutoNegSelectorList::= SEQUENCE OF AutoNegSelector

AutoNegTechnology::= ENUMERATED {
global	(0),	--reserved for future use.
other	(1),	--undefined
unknown	(2),	--initializing, true ability not yet known.
10BASE-T	(14),	--10BASE-T as defined in Clause 14
10BASE-TFD	(142),	--Full duplex 10BASE-T as defined in Clauses 14 and 31
100BASE-T4	(23),	--100BASE-T4 as defined in Clause 23
100BASE-TX	(25),	--100BASE-TX as defined in Clause 25
100BASE-TXFD	(252),	--Full duplex 100BASE-TX as defined in Clauses 25 and 31
10GBASE-KX4	(483),	--10GBASE-KX4 PHY as defined in Clause 71
10GBASE-KR	(495),	--10GBASE-KR PHY as defined in Clause 72
10GBASE-T	(55),	--10GBASE-T PHY as defined in Clause 55
FDX PAUSE	(312),	--PAUSE operation for full duplex links as defined in
		Annex 31B
FDX APAUSE	(313),	--Asymmetric PAUSE operation for full duplex links as defined 
		 in Clause 37 and Annex 31B
FDX SPAUSE	(314),	--Symmetric PAUSE operation for full duplex links as defined 
		 in Clause 37 and Annex 31B
FDX BPAUSE	(315),	--Asymmetric and Symmetric PAUSE operation for full duplex 
		links as defined in Clause 37 and Annex 31B
100BASE-T2	(32),	--100BASE-T2 as defined in Clause 32
100BASE-T2FD	(322),	--Full duplex 100BASE-T2 as defined in Clauses 31 and 32
1000BASE-X	(36),	--1000BASE-X as defined in Clause 36
1000BASE-XFD	(362),	--Full duplex 1000BASE-X as defined in Clause 36
1000BASE-KX	(393),	--1000BASE-KX PHY as defined in Clause 70
1000BASE-T	(40),	--1000BASE-T UTP PHY as defined in Clause 40
1000BASE-TFD	(402),	--Full duplex 1000BASE-T UTP PHY to be defined in Clause 40
Rem Fault1	(37),	--Remote fault bit 1 (RF1) as specified in Clause 37
Rem Fault2	(372),	--Remote fault bit 2 (RF1) as specified in Clause 37
isoethernet	(8029)	--802.9 ISLAN-16T
}

AutoNegTechnologyList::= SEQUENCE OF AutoNegTechnology

AutoPartitionState::= ENUMERATED {
autoPartitioned	(1),
notAutoPartitioned	(2)
}

BbandFrequency::= SEQUENCE {
xmitCarrierFrequency	[1]	INTEGER --Frequency in MHz times 4 (250 kHz resolution)
translationFrequency	[2]	INTEGER   --Frequency in MHz times 4 (250 kHz resolution)
}

BbandXmitRcvSplitType::= ENUMERATED {
other	(1),	--undefined
single	(2),	--single-cable system
dual	(3)	--dual-cable system, offset normally zero
}

BitString::= BIT STRING (SIZE (1..1024))

ChurnState ::= ENUMERATED {
noChurn	 (0),	--NO_ACTOR/PARTNER_CHURN
		--or ACTOR/PARTNER_CHURN_MONITOR
churn	(1)	--ACTOR/PARTNER_CHURN
	}

CollectorMaxDelay ::= INTEGER		--16 bit value, tens of microseconds
		--(max = 0.65535 seconds)

CurrentStatus ::= ENUMERATED {
MPSAbsent	(0),	-- MPS absent
ok 	(1)	-- MPS present and over current not detected
}

DeferControl::= ENUMERATED {
unknown	(1),	--defer control mode unknown
defer control off	(2),	--defer control mode disabled
defer control on	(3)	--defer control mode enabled
}

DescriptionString ::= PrintableString (SIZE 0..255))

DetectStatus ::= ENUMERATED {
disabled	(0),	-- PSE disabled
searching	(1),	-- PSE searching
deliveringPower	(2),	-- PSE delivering power
test	(3),	-- PSE test mode
fault	(4),	-- PSE fault detected
otherFault	(5)	-- PSE implementation specific fault detected
}

DuplexValues::= ENUMERATED {
half duplex	(1),	--capable of operating in half duplex mode
full duplex	(2),	--capable of operating in full duplex mode
unknown	(3)	--unknown duplex capability
}

FarEndPathStatus ::= BIT STRING (SIZE (2))

FECMode::= ENUMERATED {
unknown	(1),	--initializing, true state not yet known
supported	(2),	--FEC supported
not supported	(3)	--FEC not supported
}

Integer8::= INTEGER (0..2^8-1)		--8 bit value

Integer16::= INTEGER (0..2^16-1)		--16 bit value

Integer32 ::= INTEGER (0..2^32-1)		--32 bit value

Integer48::= INTEGER (0..2^48-1) 		--48 bit value

Integer64::= INTEGER (0..2^64-1) 		--64 bit value

Jabber::= SEQUENCE {
jabberFlag	[1]	JabberFlag,
jabberCounter	[2]	JabberCounter
}

JabberFlag::= ENUMERATED {
other	(1),	--undefined
unknown	(2),	--initializing, true state not yet known
normal	(3),	--state is true or normal
fault	(4)	--state is false, fault or abnormal
}

JabberCounter::= INTEGER (0..2^32- 1)

JValue::= OCTET STRING (SIZE (16))

KeyValue ::= INTEGER (0..2^16-1) 		--16 bit value; range 0-65535

LACPActivity ::= ENUMERATED {
active 	(0), 	--Port is Active LACP
passive	(1) 	--Port is Passive LACP
}

LACPTimeout ::= ENUMERATED {
short	(0),	--Timeouts are Short
long	(1)	--Timeouts are Long
}

LineStatus ::= BIT STRING (SIZE (2))

LinkDelayAllowance::= INTEGER (0..2^32 - 1)

MACControlFunctions::= ENUMERATED {
PAUSE	(312)	--PAUSE command implemented
}

MACControlFunctionsList::= SEQUENCE OF MACControlFunctions

MACCapabilitiesList::= SEQUENCE OF DuplexValues

MauTypeList::= SEQUENCE OF TypeValue

MaxFrameLengthList::= ENUMERATED {
unknown	(1),	--Frame length capability unknown
basicFrame	(2),	--Capable of supporting maxBasicFrameSize 
		(1518 octet frames)
qTaggedFrame	(3),	--Capable of supporting maxBasicFrameSize + qTagPrefixSize
		(1522 octet frames)
envelopeFrame	(4)	--Capable of supporting maxEnvelopeFrameSize 
		(2000 octet frames)
}

MediaAvailState::= ENUMERATED {
other	(1),	--undefined
unknown	(2),	--initializing, true state not yet known
available	(3),	--link or light normal, loopback normal
not available	(4),	--link loss or low light, no loopback
remote fault	(5),	--remote fault with no detail
invalid signal	(6),	--invalid signal, applies only to 10BASE-FB
remote jabber	(7),	--remote fault, reason known to be jabber
remote link loss	(8),	--remote fault, reason known to be far-end link loss
remote test	(9),	--remote fault, reason known to be test
offline	(10),	--offline, applies only to Clause 37 Auto-Negotiation
auto neg error	(11)	--Auto-Negotiation error, applies only to Clause 37 Auto- 
		Negotiation
PMD link fault	(12)	--PMD/PMD receive link fault
WIS frame loss	(13)	--WIS loss of frame, applies only to 10GBASE-W
WIS signal loss	(14)	--WIS loss of signal, applies only to 10GBASE-W
PCS link fault	(15)	--PCS receive link fault
excessive BER	(16)	--PCS Bit Error Ratio monitor reporting excessive error ratio
DXS link fault	(17)	--DTE XS receive link fault, applies only to XAUI 
PXS link fault	(18)	--PHY XS transmit link fault, applies only to XAUI
}

MIIDetect::= ENUMERATED {
unknown	(1),
presentNothingConnected	(2),
presentConnected	(3),
absent	(4)
}

MPCPMode::= ENUMERATED {
OLT	(1),	--sublayer operating in OLT mode
ONU	(2)	--sublayer operating in ONU mode
}

MPCPRegState::= ENUMERATED {
unregistered	(1),	--unregistered
registering	(2),	--registering
registered	(3)	--registered
}

MulticastAddressList::= SEQUENCE OF MACAddress

MuxState ::= ENUMERATED {
detached	(0),	--DETACHED
waiting	(1),	--WAITING
attached	(2),	--ATTACHED
collecting	(3),	--COLLECTING
distributing	(4),	--DISTRIBUTING
collecting_distributing	(5)	--COLLECTING_DISTRIBUTING
}

NotificationEnable ::= ENUMERATED {
enabled 	(0), 	--Notifications enabled
disabled	(1)	--Notifications disabled

OAMConfig::= BIT STRING (SIZE (5))

OAMDiscoveryState::= ENUMERATED {
link fault	(1),
active send local	(2),
passive wait	(3),
send local remote	(4),
send any	(5),
send local remote ok	(6)
}

OAMErrorConf::= SEQUENCE {
OAMEventWindow	[1]	Integer64,
OAMEventThreshold	[2]	Integer64
}

OAMErrorEvent::= SEQUENCE {
OAMEventTimeStamp	[1]	Integer16,
OAMEventWindow	[2]	Integer64,
OAMEventThreshold	[3]	Integer64,
OAMErrors	[4]	Integer64,
OAMErrorRunTotal	[5]	Integer64,
OAMEventRunTotal	[6]	Integer32
}

OAMFlagsField::= BIT STRING (SIZE (7))

OAMMode::= ENUMERATED {
passive	(1),	--passive 
active	(2)	--active
}

OAMState::= BIT STRING (SIZE (3))

OMPEmulationType::= ENUMERATED {
unknown	(1),	--Initializing, true state or type not yet known 
OLT	(2),	--Sublayer operating in OLT mode
ONU	(3)	--Sublayer operating in ONU mode
}

OneOfName::= INTEGER (1..1024)

OUI::= OCTETSTRING 		-- Length 3 octets

PairCtrlAbility ::=BOOLEAN

PathStatus ::= BIT STRING (SIZE (4))

PHYCurrentState::= ENUMERATED {
noPMEAssigned	(1),	--no PME assigned in case of PME aggregation
lossOfFraming	(2), 	--one or more PME in aggregation indicates Loss of Framing
lossOfSignal	(3), 	--one or more PME in aggregation indicates Loss of Signal
lossOfPower	(4), 	--one or more PME in aggregation indicates Loss of Power,
			10PASS-TS only
configInitFailure	(5),	--configuration initialization failure
noPeerPMEPresent	(6),	--one or more PME in aggregation indicates no peer PME
			present
snrMarginViolation	(7),	--one or more PME in aggregation indicates SNR Margin
			Violation
lineAttenViolation	(8)	--one or more PME in aggregation indicates Loop Attenuation
			Violation
}

PHYEnd::= ENUMERATED {
subscriber	(1),	--Subscriber mode of operation 
office	(2)	--Office mode of operation
}

PhyTypeList::= SEQUENCE OF PhyTypeValue

PhyTypeValue::= ENUMERATED {
other	(1),	--undefined 
unknown	(2),	--initializing, true state or type not yet known
none	(3),	--MII present and nothing connected
10 Mb/s	(7),	--Clause 7 10 Mb/s Manchester
100BASE-T4	(23),	--Clause 23 100 Mb/s 8B/6T
100BASE-X	(24),	--Clause 24 100 Mb/s 4B/5B
100BASE-T2	(32),	--Clause 32 100 Mb/s PAM5x5
1000BASE-X	(36),	--Clause 36 1000 Mb/s 8B/10B
1000BASE-T	(40)	--Clause 40 1000 Mb/s 4D-PAM5
10GBASE-X	(48)	--Clause 48 10 Gb/s 4
10GBASE-R	(49)	--Clause 49 10 Gb/s 64B/66B
10GBASE-W	(50),	--Clause 49 10 Gb/s 64B/66B and Clause 50 WIS
10GBASE-T	(55),	--Clause 55 10 Gb/s DSQ128
10PASS-TS	(62),	--Clause 61 2.5 Mb/s to 100 Mb/s 64/65-octet
2BASE-TL	(63)	--Clause 61 0.5 Mb/s to 5.5 Mb/s 64/65-octet
}

PMEOperatingProfile::= SEQUENCE {
PMEProfileState	[1] 	PMEProfileState,
PMEProfileNumber	[2] 	Integer8
}

PMEProfile::= SEQUENCE {
profile1	[1]	Integer8,
profile2	[2]	Integer8,
profile3	[3]	Integer8,
profile4	[4]	Integer8,
profile4	[5]	Integer8,
profile4	[6]	Integer8
}

PMEProfileState::= ENUMERATED {
no link	(1),	--link is not up
match	(2),	--link up using a profile
no match	(3),	--link up not using a profile
activate failure	(4)	--link activate failure
}

PMEStatus::= ENUMERATED {
down not ready	(1),	--link is down, not ready
down ready	(2),	--link is down, ready
initializing	(3),	--link is initializing
10PASS-TS	(4),	--link is up as 10PASS-TS
2BASE-TL	(5)	--link is up as 2BASE-TL
}

PortAdminState::= ENUMERATED {
disabled	(1),
enabled	(2)
}

PortNumber ::= INTEGER (0..2^16-1)

PowerClass ::= ENUMERATED {
class0	(0),	-- Class 0 PD
class1	(1),	-- Class 1 PD
class2	(2),	-- Class 2 PD
class3	(3),	-- Class 3 PD
class4	(4)	-- Class 4 PD
}

PriorityValue ::= INTEGER (0..2^16-1) 	--16 bits

PSEPowerPairs ::= ENUMERATED {
signal	(0),	-- PSE Pinout Alternative A
spare	(1)	-- PSE Pinout Alternative B
}

RateValues ::= ENUMERATED {
rate control off	(1),	-- rate control enabled
rate control on	(2),	-- rate control disabled
unknown	(3)	-- rate control mode unknown
}

RegisterEight::= INTEGER (0..255)

RepeaterHealthData::= OCTET STRING (SIZE (0..255))

RepeaterHealthInfo::= SEQUENCE {
repeaterHealthState	[1]	RepeaterHealthState,
repeaterHealthText	[2]	RepeaterHealthText  OPTIONAL,
repeaterHealthData	[3]	RepeaterHealthData  OPTIONAL
}

RepeaterHealthState::= ENUMERATED {
other	(1),	--undefined or unknown
ok	(2),	--no known failures
repeaterFailure	(3),	--known to have a repeater-related failure
groupFailure	(4),	--known to have a group-related failure
portFailure	(5),	--known to have a port-related failure
generalFailure	(6)	--has a failure condition, unspecified type
}

RepeaterType::= ENUMERATED {
other	(1),	--See 30.2.5: 
unknown	(2),	--initializing, true state or type not yet known
10 Mb/s	(9),	--Clause 9 10 Mb/s Baseband repeater
100 Mb/sClassI	(271),	--Clause 27 class I 100 Mb/s Baseband repeater
100 Mb/sClassII	(272),	--Clause 27 class II 100 Mb/s Baseband repeater
1000 Mb/s	(41),	--Clause 41 1000 Mb/s Baseband repeater
802.9a	(99)	--Integrated services repeater
}

RepeaterHealthText::= PrintableString (SIZE (0..255))

RxState ::= ENUMERATED {
current	(0),
expired	(1),
defaulted	(2),
initialize	(3),
lacpDisabled	(4),
portDisabled	(5)
}

SectionStatus ::= BIT STRING (SIZE (2))

SESThreshold ::= INTEGER (0..2^32-1)

TrueFalse::= BOOLEAN

TypeList::= SEQUENCE OF TypeValue

TypeValue::= ENUMERATED {
global	(0),	--undefined
other	(1),	--undefined
unknown	(2),	--initializing, true state not yet known
AUI	(7),	--no internal MAU, view from AUI
2BASE-TL	(63),	--Voice grade UTP PHY as specified in Clause 61 and 63
10BASE5	(8),	--Thick coax MAU as specified in Clause 8
FOIRL	(9),	--FOIRL MAU as specified in 9.9
10BASE2	(10),	--Thin coax MAU as specified in Clause 10
10BROAD36	(11),	--Broadband DTE MAU as specified in Clause 11
10BASE-T	(14),	--UTP MAU as specified in Clause 14, duplex mode
		   unknown
10BASE-THD	(141),	--UTP MAU as specified in Clause 14, half duplex mode
10BASE-TFD	(142),	--UTP MAU as specified in Clause 14, full duplex mode
10PASS-TS	(62),	--Voice grade UTP PHY as specified in Clause 61 and 62
10BASE-FP	(16),	--Passive fiber MAU as specified in Clause 16
10BASE-FB	(17),	--Synchronous fiber MAU as specified in Clause 17
10BASE-FL	(18),	--Asynchronous fiber MAU as specified in Clause 18, duplex
		   mode unknown
10BASE-FLHD	(181),	--Asynchronous fiber MAU as specified in Clause 18, half
		   duplex mode
10BASE-FLFD	(182),	--Asynchronous fiber MAU as specified in Clause 18, full
		   duplex mode
100BASE-T4	(23),	--Four-pair Category 3 UTP as specified in Clause 23
100BASE-TX	(25),	--Two-pair Category 5 UTP as specified in Clause 25, duplex
		   mode unknown
100BASE-TXHD	(251),	--Two-pair Category 5 UTP as specified in Clause 25, half
		duplex mode
100BASE-TXFD	(252),	--Two-pair Category 5 UTP as specified in Clause 25, full
		   duplex mode
100BASE-BX10D	(581),	--One-fiber OLT PHY as specified in Clause 58
100BASE-BX10U	(582),	--One-fiber ONU PHY as specified in Clause 58
100BASE-FX	(26),	--X fiber over PMD as specified in Clause 26, duplex mode
		   unknown
100BASE-FXHD	(261),	--X fiber over PMD as specified in Clause 26, half duplex mode
100BASE-FXFD	(262),	--X fiber over PMD as specified in Clause 26, full duplex mode
100BASE-LX10	(58),	--Two-fiber PHY as specified in Clause 58
100BASE-T2	(32),	--Two-pair Category 3 UTP as specified in Clause 32, duplex
		   mode unknown
100BASE-T2HD	(321),	--Two-pair Category 3 UTP as specified in Clause 32, half
		   duplex mode
100BASE-T2FD	(322),	--Two-pair Category 3 UTP as specified in Clause 32, full
		   duplex mode
1000BASE-X	(36),	--X PCS/PMA as specified in Clause 36 over unknown PMD,
		duplex mode unknown
1000BASE-BX10D	(591),	--One-fiber OLT PHY as specified in Clause 59
1000BASE-BX10U	(592),	--One-fiber ONU PHY as specified in Clause 59
1000BASE-XHD	(361),	--X PCS/PMA as specified in Clause 36 over unknown PMD, 
		half duplex mode
1000BASE-XFD	(362),	--X PCS/PMA as specified in Clause 36 over unknown PMD,
		 full duplex mode
1000BASE-LX	(381),	--X fiber over long-wavelength laser PMD as specified in 
		Clause 38, duplex mode unknown
1000BASE-LXHD	(382),	--X fiber over long-wavelength laser PMD as specified in 
		Clause 38, half duplex mode
1000BASE-LXFD	(383),	--X fiber over long-wavelength laser PMD as specified in 
		Clause 38, full duplex mode
1000BASE-LX10	(59),	--Two-fiber 10 km PHY as specified in Clause 59
1000BASE-PX10D	(601),	--One-fiber OMP OLT 10 km PHY as specified in
		   Clause 60
1000BASE-PX10U	(602),	--One-fiber OMP ONU 10 km PHY as specified in
		   Clause 60
1000BASE-PX20D	(603),	--One-fiber OMP OLT 20 km PHY as specified in
		   Clause 60
1000BASE-PX20U	(604),	--One-fiber OMP ONU 20 km PHY as specified in
		   Clause 60
1000BASE-SX	(384),	--X fiber over short-wavelength laser PMD as specified in 
		Clause 38, duplex mode unknown
1000BASE-SXHD	(385),	--X fiber over short-wavelength laser PMD as specified in
		Clause 38, half duplex mode
1000BASE-SXFD	(386),	--X fiber over short-wavelength laser PMD as specified in
		Clause 38, full duplex mode
1000BASE-CX	(39),	--X copper over 150-Ohmbalanced cable PMD as specified in
		Clause 39, duplex mode unknown
1000BASE-CXHD	(391),	--X copper over 150-Ohmbalanced cable PMD as specified in
		Clause 39, half duplex mode
1000BASE-CXFD	(392),	--X copper over 150-Ohmbalanced cable PMD as specified in
		Clause 39, full duplex mode
1000BASE-KX	(393),	--X PCS/PMA over an electrical backplane PMD as specified 
		in Clause 70
1000BASE-T	(40),	--Four-pair Category 5 UTP PHY as specified in Clause 40,
		duplex mode unknown
1000BASE-THD	(401),	--Four-pair Category 5 UTP PHY as specified in Clause 40,
		half duplex mode 
1000BASE-TFD	(402),	--Four-pair Category 5 UTP PHY as specified in Clause 40,
		full duplex mode 
10GBASE-X	(48),	--X PCS/PMA as specified in Clause 48 over undefined PMD
10GBASE-LX4	(481),	--X fiber over WWDM optics as specified in Clause 53
10GBASE-CX4	(482),	--X copper over 8 pair 100-Ohm balanced cable as specified in
		Clause 54
10GBASE-KX4	(483),	--X PCS/PMA over an electrical backplane PMD as specified 
		in Clause 71
10GBASE-R	(49),	--R PCS/PMA as specified in Clause 49 over undefined PMD
10GBASE-ER	(491),	--R fiber over 1550 nm optics as specified in Clause 52
10GBASE-LR	(492),	--R fiber over 1310 nm optics as specified in Clause 52
10GBASE-SR	(493),	--R fiber over 850 nm optics as specified in Clause 52
10GBASE-LRM	(494),	--R fiber over 1310 nm optics as specified in Clause 68
10GBASE-KR	(495),	--R PCS/PMA over an electrical backplane PMD as specified 
		in Clause 72
10GBASE-W	(50),	--W PCS/PMA as specified in Clauses 49 and 50 over
		   undefined PMD
10GBASE-EW	(501),	--W fiber over 1550 nm optics as specified in Clause 52
10GBASE-LW	(502),	--W fiber over 1310 nm optics as specified in Clause 52
10GBASE-SW	(503),	--W fiber over 850 nm optics as specified in Clause 52
10GBASE-T	(55),	--Four-pair twisted-pair balanced copper cabling PHY as
		specified in Clause 55
802.9a	(99)	--Integrated services MAU as specified in IEEE Std 802.9
		   ISLAN-16T
}

END