PROPOSED REMEDY FOR COMMENT # 251

Replace lines 47-50 p. 77 with the following text:

33.3.7.1 Input Voltage

The specification for VPort_PD in Table 33–18 is for the input voltage range after startup (see 33.3.7.3), and accounts for loss in the cabling plant.

Note, VPort_PD = VPSE – (RChan × IPort).