IEEE 802.3 Ethernet Working Group
DRAFT Liaison Communication

| Source: | IEEE 802.3 Working Group |  |
| --- | --- | --- |
| To: | Frank Effenberger | Rapporteur Q2/15, ITU-T frank.effenberger@futurewei.com |
|  | Jun-ichi Kani | Associate Rapporteur Q2/15, ITU-T kani.junichi@lab.ntt.co.jp |
| CC: | Hiroshi Ota | Advisor, ITU-T Study Group 15 hiroshi.ota@itu.int |
|  | Steve Trowbridge | Chair, ITU-T Study Group 15 steve.trowbridge@nokia.com |
|  | Konstantinos Karachalios | Secretary, IEEE-SA Standards Board Secretary, IEEE-SA Board of Governors sasecretary@ieee.org |
|  | Paul Nikolich | Chair, IEEE 802 LMSC p.nikolich@ieee.org |
|  | Adam Healey | Vice-chair, IEEE 802.3 Ethernet Working Group adam.healey@broadcom.com |
|  | Jon Lewis | Secretary, IEEE 802.3 Ethernet Working Group jon.lewis@dell.com |
|  | Frank Effenberger | Chair, IEEE P802.3cp Task Force frank.effenberger@futurewei.com |
| From: | David Law | Chair, IEEE 802.3 Ethernet Working Group dlaw@hpe.com |
| Subject: | Liaison reply to ITU-T SG15 on coordination of bidirectional optical for access |  |

Approval: Agreed to at IEEE 802.3 interim meeting, Geneva, CH,  23rd Jan 2020

Dear Dr. Effenberger,

We want to thank the members of Q2/15 for providing your comments against our recent drafts.  Many of these were very useful and have been implemented.  We would inform you that the IEEE P802.3cp 10, 25, & 50G Bidirectional optical access PHYs project has progressed its draft to version 1.2, and has reached consensus on the various optical parameter choices for the loss budgets and wavelength plans.  The most up-to-date values are summarized in the following tables, where the updates are shown in red.

The optical path loss budgets are as follows:

| Reach class | Total loss budget range |
| --- | --- |
| 10 km | 0 to 6.3 dB |
| 20 km | 0 to 15 dB |
| 40 km | 5 to 18 dB |
| 40 km + | 10 to 23 dB |

As before, there are two loss budgets for 40 km reach, reflecting the traditional ER budget (18 dB) and a budget suitable for non-engineered links (23 dB).

The center wavelengths are as follows:

| Down / Up wavelength (nm) | Reach class |  |  |
| --- | --- | --- | --- |
| Speed | 10 km | 20 km | 40 km |
| 10Gb/s NRZ | 1330 / 1270 ±10nm | 1330 / 1270 ±10nm | 1330 / 1270 ±10nm |
| 25Gb/s NRZ | 1330 / 1270 ±10nm | 1314 / 1289 ±8nm | 1314 / 1289 ±8nm |
| 50Gb/s PAM4 | 1330 / 1270 ±10nm | 1314 / 1289 ±8nm | 1314 / 1289 ±8nm |

This is described in the next version of our draft which is attached.  It would be appreciated if you could review the draft and have an interested party submit comments for our continued improvement of the text.

For future communications, please note that our next face to face meeting will be 16-19 March 2020.  We also plan to have a conference call 15:00-16:00 UTC 27 Feb 2020.  The reflector subscription information can be found at http://www.ieee802.org/3/cp/reflector.html.

We appreciate your review and look forward to continued coordination on the development of bidirectional optical access PHYs.

Sincerely,

David Law

Chair, IEEE 802.3 Ethernet Working Group

Attachment: 8023cp_D1p2.pdf

This document solely represents the views of the IEEE 802.3 Working Group, and does not necessarily represent a position of the IEEE, the IEEE Standards Association, or IEEE 802.