If the number of classes should be between 30 and 40, my scheme (Backward Compatible Enhanced Classification) can be easily adapted.

This scheme uses 4 ping-pong cycles (fingers) but needs only 3 pins.

I propose to define 9 sub-classes for each .af class and 9 classes in the new range from 12.95 to xx W. (36 classes total)

Cycle One: Macro-Classes definition

- Classes 1 to 3 cover .af power range

  - .af PSE classifies .at PD under 13W properly

- Class 4 covers 12.95W to xx power range

Cycle two: .at classification signature

- .at PD sinks

  - class1 current if it’s a > 6.49W PD (classes 3 or 4 on Cycle1)

  - class4 current if it’s a <6.49W PD  (classes 1 or 2 on Cycle2)

- no potential false .at PD indication, because Cycle2 current is different from Cycle1)

- Fixed current, no pin needed.

Cycles three and four: class refining

- Use only 3 classes: Class1 to Class3 to divide each class into 3 sub-classes (cycle 3) and each sub-class into 3 sub-sub-classes (cycle4).

- No potential heating of the system

- 4x3x3= 36 classes

- No avoided class codes.

- If the PSE doesn’t perform 4 class cycles, it is a .af PSE