33.4.8.2 Midspan signal path requirements

When an Alternative A Midspan is connected to a 100BASE-TX PHY, the Midspan transfer function gain

shall be greater than the below expression from 100 kHz to 1 MHz at the pins of the PI used as 100BASETX transmit pins:

−c+37.5⋅LOG10a⋅f1+b⋅f2a=22.4b=520.5c=0.10.1MHz≤f<1MHz

33-14

where

f is the frequency expressed in MHz.

Additionally, the requirements will be met with a DC bias current, Ibias  between 0 mA and {TBD+0.5*Iunb mA}. Iunb is defined in  Table 33–9.

33.4.8.2.1     Alternative A Midspan PSE Compliance test setup

Compliance testing shall be performed by applying test signal to the Midspan signal input through a source impedance of 100 Ω ± 1 % . The Midspan signal input and output may be connected to a 0.5 m max length  CAT5 cable,

terminated with 100 Ω ± 1%.

The transfer function  shall be measured from the output termination to the the signal input. See figure 33-24-1.

Figure 33-24-1. Midspan Transfer Function for draft D3.0