(contribution number)

TIA TECHNICAL COMMITTEE TR-42

User-Premises Telecommunications Infrastructure

Title:	IEC specs relevant to DTE power

Source:      Wayne Larsen

AVAYA INC

2855 North Franklin Road

Indianapolis, Indiana 46219

Phone:  317-917-5591

Fax:  317-917-5656

Email:  wlarsen@avaya.com

Date:	October 29, 2002

Location:

Distribution to:	DTE power task group

Abstract:	IEC specs relevant to DTE power

Recommendation:

For information.

|  |  | Voltage rating | Current capacity | DC resistance | DC resistance unbalance |
| --- | --- | --- | --- | --- | --- |
| Connecting hardware | Spec spec | IEC 60603-7-7 section 6.4.1 | IEC 60603-7-7 section 6.4.2 | IEC 60603-7-7 section 6.4.4 | IEC 60603-7-7 section 6.4.4 |
|  | Value | 1000 VDC or AC peak, between signal conductors. 1500 VDC or AC peak, between signal conductors and the panel, or the shield, if any. | According to a de-rating curve versus temperature, is 0.8 Amps at 60 degrees C, with “all contacts, connected in series”. | 0.2 Ohms maximum | - Ohms maximum (will be 0.05 Ohms in 60603-7-4). |
|  | Test spec | IEC 60512-2 test 4a, method a. | IEC 60512-3, test 5b | IEC 60512-2, test 2a | IEC 60512-2, test 2a |
| Solid cable | Spec spec | IEC 61156-5 3.2.3 | Controlled by design; details given in 61156-5 | IEC 61156-5 3.2.1 | IEC 61156-5 3.2.2 |
|  | Value | 1000 VDC | >0.175 Amps per conductor | 19 Ohms on a 100 meter loop | 2 % |
|  | Test spec | IEC61156-5 3.2.3 |  | IEC 60189-1 | IEC 60189-1 |
| Stranded cable | Spec spec | IEC61156-6 3.2.3 | Controlled by design; details given in 61156-6 | IEC 61156-6 3.2.1 | IEC 61156-6 3.2.2 |
|  | Value | 1000 VDC | 0.175 Amps per conductor | 29 Ohms on a 100 meter loop | 2 % |
|  | Test spec | IEC61156-6 3.2.3 |  | IEC 60189-1 | IEC 60189-1 |