3GPP TSG-RAN WG4 Meeting #79 	R4-163325

- Nanjing, P. R. of China, 23rd-27th May 2016

| CR-Form-v11.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.133 | CR | 3516 | Rev | - | Current version: | 13.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Editorial corrections for LAA |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | RAN WG4 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_LAA-Core |  |  |  |  |  |  | Date: |  | 2016-05-13 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-13 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) |  |
| Reason for change: |  | Currently the RSRP, SCH, and CSI-RSRP conditions apply in all subframes, even when eNodeB does not transmit |  |  |  |  |  |  |  |  |
| Summary of change: |  | It is clarified that the conditions apply only in the available DRS subframes |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Confusing requirements |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.11.2.1.1.1, 8.11.2.1.1.2, 8.11.3.1.1.1, 8.11.3.1.1.2, 8.12.2.4.1, 8.12.2.4.2, 8.12.3.4.1, 8.12.3.4.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  |  |  | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

###### --- Start of Change 1 ----

###### 8.11.2.1.1.1	Requirements when no DRX is used

When no DRX is in use the UE shall be able to identify a new detectable FS3 intra-frequency cell within Tidentify_intra_ FS3,

Tidentify_intra_FS3 = Tdetect intra_FS3 + Tmeasure intra_FS3_CRS,

where:

Tdetect intra_FS3 is the intra-frequency period for cell detection as specified in Table 8.11.2.1.1.1-1,

Tmeasure_intra_FS3_CRS is the intra-frequency period for measurements as shown in Table 8.11.2.1.1.1-2,

TDMTC_periodicity is the discovery signal measurement timing configuration periodicity of higher layer,

L is the number of configured discovery signal occasions which are not available during Tdetect intra_FS3 for cell detection at the UE due to the absence of the necessary radio signals,

M is the number of configured discovery signal occasions which are not available during Tmeasure_intra_FS3_CRS for the measurements at the UE due to the absence of the necessary radio signals.

Table 8.11.2.1.1.1-1: Intra-frequency cell detection under operation with frame structure 3

| SCH Ês/Iot | Tdetect intra_FS3, [ms] |
| --- | --- |
| [0] ≤ SCH Ês/Iot | ([1]+L) * TDMTC_periodicity |
| [-6] ≤ SCH Ês/Iot < [0] | ([4]+L) * TDMTC_periodicity |

A cell shall be considered detectable when the following conditions are met during the discovery signal occasions which are available during Tidentify_intra_FS3:

-	RSRP related side conditions given in Section 9.1.18.2 are fulfilled for a corresponding Band,

-	RSRQ related side conditions given in Section 9.1.18.3 are fulfilled for a corresponding Band,

-	SCH_RP is according to Annex B.2.12 for a corresponding Band and SCH Ês/Iot is according to Table 8.11.2.1.1.1-1.

Identification of a cell shall include detection of the cell and additionally performing a single measurement with measurement period of Tmeasure_intra_FS3_CRS when no DRX is used. If higher layer filtering is used, an additional cell identification delay can be expected.

In the RRC_CONNECTED state the measurement period for intra-frequency measurements is Tmeasure_intra_FS3_CRS as shown in Table 8.11.2.1.1.1-2, when no DRX is in use. The UE shall be capable of performing RSRP and RSRQ measurements for 3 identified intra-frequency cells, and the UE physical layer shall be capable of reporting measurements to higher layers within the measurement period of Tmeasure_intra_FS3_CRS.

Table 8.11.2.1.1.1-2: Intra-frequency measurement requirements under operation with frame structure 3

| Measurement bandwidth [RB] | CRS Ês/Iot | Discovery signal occasion duration (ds-OccasionDuration) [ms] | Tmeasure_intra_FS3_CRS [ms] |
| --- | --- | --- | --- |
| 6 | [0] ≤ CRS Ês/Iot | 1 | ([3]+M) * TDMTC_periodicity |
| 6 | [-6] ≤ CRS Ês/Iot < [0] | 1 | ([5]+M) * TDMTC_periodicity |
| 25 | [0] ≤ CRS Ês/Iot | 1 | ([1]+M) * TDMTC_periodicity |
| 25 | [-6] ≤ CRS Ês/Iot < [0] | 1 | ([3]+M) * TDMTC_periodicity |

The RSRP measurement accuracy for all measured cells shall be as specified in Section 9.1.18.2, and the RSRQ measurement accuracy for all measured cells shall be as specified in Section 9.1.18.3.

###### --- End of Change 1 ----

###### --- Start of Change 2 ----

###### 8.11.2.1.1.2	Requirements when DRX is used

When DRX is in use the UE shall be able to identify a new detectable FS3 intra-frequency cell within Tidentify_intra_FS3_DRX.

Tidentify_intra_FS3_DRX = Tdetect_intra_FS3_DRX + Tmeasure_intra_FS3_CRS_DRX,

where:

Tdetect_intra_FS3_DRX is the intra-frequency period for cell detection as shown in Table 8.11.2.1.1.2-1,

Tmeasure_intra_FS3_CRS_DRX is the intra-frequency period for measurements as shown in Table 8.11.2.1.1.2-2,

TDMTC_periodicity is the discovery signal measurement timing configuration periodicity of higher layer,

L is the number of configured discovery signal occasions during ON DURATION and which are not available during Tdetect intra_FS3_DRX for cell detection at the UE due to the absence of the necessary radio signals,

M is the number of configured discovery signal occasions during ON DURATION and which are not available during Tmeasure_intra_FS3_CRS_DRX for the measurements at the UE due to the absence of the necessary radio signals.

Table 8.11.2.1.1.2-1: Intra-frequency cell detection under operation with frame structure 3

| SCH Ês/Iot | Tdetect intra_FS3_CRS_DRX, [ms] |
| --- | --- |
| [0] ≤ SCH Ês/Iot | (TBD+L) * Max{ TDMTC_periodicity, DRX cycle length } |
| [-6] ≤ SCH Ês/Iot < [0] | (TBD+L) * Max{ TDMTC_periodicity, DRX cycle length } |

A cell shall be considered detectable when the following conditions are met during the discovery signal occasions which are available during Tidentify_intra_FS3_DRX:

-	RSRP related side conditions given in Section 9.1.18.2 are fulfilled for a corresponding Band,

-	RSRQ related side conditions given in Section 9.1.18.3 are fulfilled for a corresponding Band,

-	SCH_RP is according to Annex B.2.12 for a corresponding Band and SCH Ês/Iot is according to Table 8.11.2.1.1.2-1.

Identification of a cell shall include detection of the cell and additionally performing a single measurement with measurement period of Tmeasure_intra_FS3_ CRS_DRX when DRX is used. If higher layer filtering is used, an additional cell identification delay can be expected.

In the RRC_CONNECTED state the measurement period for intra-frequency measurements is Tmeasure_ intra_FS3_CRS_DRX as shown in Table 8.11.2.1.1.2-2, when DRX is in use. The UE shall be capable of p performing RSRP and RSRQ measurements for 3 identified intra-frequency cells, and the UE physical layer shall be capable of reporting measurements to higher layers within the measurement period of Tmeasure_intra_FS3_CRS_DRX.

Table 8.11.2.1.1.2-2: Intra-frequency measurement requirements under operation with frame structure 3

| Measurement bandwidth [RB] | CRS Ês/Iot | Discovery signal occasion duration (ds-OccasionDuration) [ms] | Tmeasure_intra_FS3_CRS_DRX, [ms] |
| --- | --- | --- | --- |
| 6 | [0] ≤ CRS Ês/Iot | 1 | ([3]+M) * Max{ TDMTC_periodicity, DRX cycle length } |
| 6 | [-6] ≤ CRS Ês/Iot < [0] | 1 | ([5]+M) * Max{ TDMTC_periodicity, DRX cycle length } |
| 25 | [0] ≤ CRS Ês/Iot | 1 | ([1]+M) * Max{ TDMTC_periodicity, DRX cycle length } |
| 25 | [-6] ≤ CRS Ês/Iot < [0] | 1 | ([3]+M) * Max{ TDMTC_periodicity, DRX cycle length } |

The RSRP measurement accuracy for all measured cells shall be as specified in Section 9.1.18.2, and the RSRQ measurement accuracy for all measured cells shall be as specified in Section 9.1.18.3.

###### --- End of Change 2 ----

###### --- Start of Change 3 ----

###### 8.11.3.1.1.1	Requirements when no DRX is used

When no DRX is in use the UE shall be able to identify a new detectable FS3 intra-frequency TP within Tidentify_intra_TP_FS3.

Tidentify_intra_TP_FS3 = Tidentify_intra_FS3 + Tmeasure_intra_FS3_CSI-RS,

where

Tidentify_intra_FS3 is the intra-frequency period for cell identification in Section 8.11.2.1.1.1,

Tmeasure_ intra_FS3_CSI-RS is the intra-frequency period for TP measurement as shown in Table 8.11.3.1.1.1-1,

TDMTC_periodicity is the discovery signal measurement timing configuration periodicity of higher layer,

M is the number of configured discovery signal occasions which are not available during Tmeasure_ intra_FS3_CSI-RS for the measurements at the UE due to the absence of the necessary radio signals.

A TP shall be considered detectable when the following conditions are met during the discovery signal occasions which are available during Tidentify_intra_TP_FS3:

-	CSI-RSRP related side conditions given in Section 9.1.18.4 are fulfilled for a corresponding Band,

-	SCH_RP is according to Annex B.2.12 for a corresponding Band and SCH Ês/Iot is according to Section 8.11.2.1.1.1.

Identification of a TP shall include identification of the cell and additionally performing a single measurement on the TP within the measurement period of Tmeasure_ intra_FS3_CSI-RS when no DRX is used. If higher layer filtering is used, an additional TP identification delay can be expected.

In the RRC_CONNECTED state the measurement period for intra-frequency measurements is Tmeasure_intra_FS3_CSI-RS as shown in table 8.11.3.1.1.1-1, when no DRX is in use. The UE shall be capable of performing CSI-RSRP measurements for 3 identified intra-frequency TPs, and the UE physical layer shall be capable of reporting measurements to higher layers with the measurement period of Tmeasure_intra_FS3_CSI-RS

Table 8.11.3.1.1.1-1: Intra-frequency TP measurement requirements under operation with frame structure 3

| Measurement bandwidth [RB] | CSI-RS Ês/Iot | Discovery signal occasion duration (ds-OccasionDuration) [ms] | Tmeasure_intra_FS3_CSI-RS, [ms] |
| --- | --- | --- | --- |
| 6 | [0] ≤ CSI-RS Ês/Iot | 1 | ([3]+M) * TDMTC_periodicity |
| 6 | [-6] ≤ CSI-RS Ês/Iot < [0] | 1 | ([5]+M) * TDMTC_periodicity |
| 25 | [0] ≤ CSI-RS Ês/Iot | 1 | ([1]+M) * TDMTC_periodicity |
| 25 | [-6] ≤ CSI-RS Ês/Iot < [0] | 1 | ([3]+M) * TDMTC_periodicity |

The CSI-RSRP measurement accuracy for all measured TPs shall be as specified in Section 9.1.18.4.

###### --- End of Change 3 ----

###### --- Start of Change 4 ----

###### 8.11.3.1.1.2	Requirements when DRX is used

When DRX is in use the UE shall be able to identify a new detectable FS3 intra-frequency TP within Tidentify_intra_TP_FS3_DRX.

Tidentify_intra_TP_FS3_DRX = Tidentify_intra_FS3_DRX + Tmeasure_ intra_FS3_CSI-RS_DRX,

where:

Tidentify_intra_FS3_DRX is the intra-frequency period for cell identification in Section 8.11.2.1.1.2.

Tmeasure_ intra_FS3_CSI-RS_DRX is the intra-frequency period for TP measurement as shown in Table 8.11.3.1.1.2-1, where

TDMTC_periodicity is the discovery signal measurement timing configuration periodicity of higher layer,

M is the number of configured discovery signal occasions during ON DURATION and which are not available duting Tmeasure_intra_FS3_CSI-RS_DRX for the measurements at the UE due to the absence of the necessary radio signals.

A TP shall be considered detectable when the following conditions are met during the discovery signal occasions which are available during Tidentify_intra_TP_FS3_DRX:

-	CSI-RSRP related side conditions given in Section 9.1.18.4 are fulfilled for a corresponding Band,

-	SCH_RP is according to Annex B.2.12 for a corresponding Band and SCH Ês/Iot is according to Section 8.11.2.1.1.2.

Identification of a TP shall include identification of the cell and additionally performing a single measurement on the TP within measurement period of Tmeasure_intra_FS3_CSI-RS_DRX when DRX is used. If higher layer filtering is used, an additional TP identification delay can be expected.

In the RRC_CONNECTED state the measurement period for intra frequency measurements is Tmeasure_intra_FS3_CSI-RS_DRX as shown in Table 8.11.3.1.1.2-1, when DRX is in use. The UE shall be capable of performing CSI-RSRP measurements for 3 identified intra-frequency TPs, and the UE physical layer shall be capable of reporting measurements to higher layers with the measurement period of Tmeasure_intra_FS3_CSI-RS_DRX.

Table 8.11.3.1.1.2-1: Intra-frequency TP measurement requirements under operation with frame structure 3

| Measurement bandwidth [RB] | CSI-RS Ês/Iot | Discovery signal occasion duration (ds-OccasionDuration) [ms] | Tmeasure_intra_FS3_CSI-RS_DRX, [ms] |
| --- | --- | --- | --- |
| 6 | [0] ≤ CSI-RS Ês/Iot | 1 | ([3]+M) * Max{ TDMTC_periodicity, DRX cycle length } |
| 6 | [-6]≤ CSI-RS Ês/Iot<[0] | 1 | ([5]+M) * Max{ TDMTC_periodicity, DRX cycle length } |
| 25 | [0] ≤ CSI-RS Ês/Iot | 1 | ([1]+M) * Max{ TDMTC_periodicity, DRX cycle length } |
| 25 | [-6]≤ CSI-RS Ês/Iot<[0] | 1 | ([3]+M) * Max{ TDMTC_periodicity, DRX cycle length } |

The CSI-RSRP measurement accuracy for all measured TPs shall be as specified in Section 9.1.18.4.

###### --- End of Change 4 ----

###### --- Start of Change 5 ----

##### 8.12.2.4.1	E-UTRAN secondary component carrier measurements when no common DRX is used

When no DRX is in use the UE shall be able to identify a new detectable cell on a secondary component carrier within Tidentify_SCC_FS3, according to the parameter measCycleSCell.

Tidentify_SCC_FS3 = Tdetect_SCC_FS3 + Tmeasure_SCC_FS3_CRS,

where:

Tdetect_SCC_FS3 is the time period for cell detection as specified in Table 8.12.2.4.1-1,

Tmeasure_SCC_FS3_CRS is thetime period for measurements as shown in Table 8.12.2.4.1-2,

TDMTC_periodicity is the discovery signal measurement timing configuration periodicity of higher layer,

L is the number of configured discovery signal occasions which are not available during Tdetect_SCC_FS3 for cell detection at the UE due to the absence of the necessary radio signals,

M is the number of configured discovery signal occasions which are not available during Tmeasure_SCC_FS3_CRS for the measurements at the UE due to the absence of the necessary radio signals.

Table 8.12.2.4.1-1: Intra-frequency cell detection on SCC under operation with frame structure 3 with deactivated SCell

| SCH Ês/Iot | Tdetect SCC_FS3, [ms] |
| --- | --- |
| [0] ≤ SCH Ês/Iot | ([1]+L) * measCycleSCell |
| [-6] ≤ SCH Ês/Iot < [0] | ([5]+L) * measCycleSCell |

A cell shall be considered detectable when the following conditions are met during the discovery signal occasions which are available during Tidentify_SCC_FS3:

-	RSRP related side condition given in Section 9.1.19 are fulfilled for a corresponding Band,

-	 SCH_RP|dBm is according to Annex B.2.12 for a corresponding Band and SCH Ês/Iot is according to Table 8.12.2.4.1-1.

The measurement period for deactivated SCell measurements is Tmeasure_SCC_FS3_CRS according to the parameter measCycleSCell shown in Table 8.12.2.4.1-1.

Table 8.12.2.4.1-2: Intra-frequency cell measurement requirements on SCC under operation with frame structure 3 with deactivated SCell

| Measurement bandwidth [RB] | CRS Ês/Iot | Discovery signal occasion duration (ds-OccasionDuration) [ms] | Tmeasure_SCC_FS3_CRS [ms] |
| --- | --- | --- | --- |
| 6 | [0] ≤ CRS Ês/Iot | 1 | ([3]+M) * measCycleSCell |
| 6 | [-6] ≤ CRS Ês/Iot < [0] | 1 | ([5]+M) * measCycleSCell |
| 25 | [0] ≤ CRS Ês/Iot | 1 | ([1]+M) * measCycleSCell |
| 25 | [-6] ≤ CRS Ês/Iot < [0] | 1 | ([3]+M) * measCycleSCell |

The UE shall be capable of performing RSRP and RSRQ measurements for 3 identified cells on a secondary component carrier, and the UE physical layer shall be capable of reporting measurements to higher layers within the measurement period of Tmeasure_SCC_FS3_CRS.

The measurement accuracy for all measured cells shall be as specified in Section 9.1.19.

A UE may reconfigure the receiver bandwidth or turn on/off one of the RF chains when performing measurements on one or two SCCs with deactivated SCell. This may cause interruptions on PCell or an activated SCell that are specified in Section 7.8.

###### --- End of Change 5 ----

###### --- Start of Change 6 ----

##### 8.12.2.4.2	E-UTRAN secondary component carrier measurements when common DRX is used

When DRX is in use the UE shall be able to identify a new detectable FS3 cell on a secondary component carrier within Tidentify_SCC_FS3_DRX, according to the parameter measCycleSCell.

Tidentify_SCC_FS3_DRX = Tdetect_SCC_FS3_ DRX + Tmeasure_SCC_FS3_CRS_DRX,

where:

Tdetect_SCC_FS3_DRX is the time period for cell detection as shown in Table 8.12.2.4.2-1,

Tmeasure_SCC_FS3_CRS_DRX is the time period for measurements as shown in Table 8.12.2.4.2-2,

TDMTC_periodicity is the discovery signal measurement timing configuration periodicity of higher layer,

L is the number of configured discovery signal occasions during ON DURATION and which are not available during Tdetect_SCC_FS3_DRX for cell detection at the UE due to the absence of the necessary radio signals,

M is the number of configured discovery signal occasions during ON DURATION and which are not available during Tmeasure_SCC_FS3_CRS_DRX for the measurements at the UE due to the absence of the necessary radio signals.

Table 8.12.2.4.2-1: Cell detection on SCC under operation with frame structure 3

| SCH Ês/Iot | Tdetect SCC_FS3_ DRX, [ms] |
| --- | --- |
| [0] ≤ SCH Ês/Iot | (TBD+L) * Max{ measCycleSCell, DRX cycle length } |
| [-6] ≤ SCH Ês/Iot < [0] | (TBD +L) * Max{ measCycleSCell, DRX cycle length } |

A cell shall be considered detectable when the following conditions are met during the discovery signal occasions which are available during Tidentify_SCC_ FS3_DRX:

-	RSRP related side condition given in Section 9.1.19 are fulfilled for a corresponding Band,

-	 SCH_RP|dBm is according to Annex B.2.12 for a corresponding Band and SCH Ês/Iot is according to Table 8.12.2.4.2-1.

The measurement period for deactivated scell measurements is Tmeasure_SCC_FS3_CRS_DRX according to the parameter measCycleSCell shown in Table 8.12.2.4.2-2.

Table 8.12.2.4.2-2: Measurement requirements on SCC under operation with frame structure 3

| Measurement bandwidth [RB] | CRS Ês/Iot | Discovery signal occasion duration (ds-OccasionDuration) [ms] | Tmeasure_SCC_FS3_CRS_DRX, [ms] |
| --- | --- | --- | --- |
| 6 | [0] ≤ CRS Ês/Iot | 1 | ([3]+M) * Max{ measCycleSCell, DRX cycle length } |
| 6 | [-6] ≤ CRS Ês/Iot < [0] | 1 | ([5]+M) * Max{ measCycleSCell, DRX cycle length } |
| 25 | [0] ≤ CRS Ês/Iot | 1 | ([1]+M) * Max{ measCycleSCell, DRX cycle length } |
| 25 | [-6] ≤ CRS Ês/Iot < [0] | 1 | ([3]+M) * Max{ measCycleSCell, DRX cycle length } |

The UE shall be capable of performing RSRP and RSRQ measurements for 3 identified cells on a secondary component carrier, and the UE physical layer shall be capable of reporting measurements to higher layers with the measurement period of Tmeasure_SCC_FS3_CRS_DRX.

The measurement accuracy for all measured cells shall be as specified in Section 9.1.19.

A UE may reconfigure receiver bandwidth taking into account the SCell activation/deactivation status, and when making measurements of cells on one or two SCCs with deactivated SCell. This may cause interruptions (packet drops) to a PCell or an activated SCell. No interruptions while the On Duration timer is running shall be allowed when common DRX is used. The requirement considers only interruptions due to reconfiguration of the receiver bandwidth, and not due to other causes such as RF impairments or channel conditions.

###### --- End of Change 6 ----

###### --- Start of Change 7 ----

##### 8.12.3.4.1	E-UTRAN secondary component carrier measurements when no common DRX is used

When no DRX is in use the UE shall be able to identify a new detectable FS3 TP on a secondary component carrier within Tidentify_SCC_TP_FS3, according to the parameter measCycleSCell.

Tidentify_SCC_TP_FS3 = Tidentify_SCC_FS3 + Tmeasure_SCC_FS3_CSI-RS,

where:

Tidentify_SCC_FS3 is the time period for cell identification in Section 8.12.2.4.1,

Tmeasure_SCC_FS3_CSI-RS is the time period for TP measurement in Table 8.12.3.4.1-1,

M is the number of configured discovery signal occasions which are not available for the measurements at the UE during Tmeasure_SCC_FS3_CSI-RS due to the absence of the necessary radio signals.

Table 8.12.3.4.1-1: Measurement requirements for a TP on SCC under operation with frame structure 3

| Measurement bandwidth [RB] | CSI-RS Ês/Iot | Discovery signal occasion duration (ds-OccasionDuration) [ms] | Tmeasure_SCC_FS3_CSI-RS [ms] |
| --- | --- | --- | --- |
| 6 | [0] ≤ CSI-RS Ês/Iot | 1 | ([3]+M) * measCycleSCell |
| 6 | [-6] ≤ CSI-RS Ês/Iot < [0] | 1 | ([5]+M) * measCycleSCell |
| 25 | [0] ≤ CSI-RS Ês/Iot | 1 | ([1]+M) * measCycleSCell |
| 25 | [-6] ≤ CSI-RS Ês/Iot < [0] | 1 | ([3]+M) * measCycleSCell |

A cell shall be considered detectable when the following conditions are met during the discovery signal occasions which are available during Tidentify_SCC_TP_FS3:

-	CSI-RSRP related side condition given in Section 9.1.19 are fulfilled for a corresponding Band,

-	 SCH_RP|dBm is according to Annex B.2.12 for a corresponding Band and SCH  Ês/Iot is accordin g to Table 8.12.2.4.1-1.

The UE shall be capable of performing RSRP measurements for 3 identified TPs on a secondary component carrier, and the UE physical layer shall be capable of reporting measurements to higher layers with the measurement period of Tmeasure_SCC_FS3_CSI-RS.

The measurement accuracy for all measured TPs shall be as specified in Section 9.1.19.

A UE may reconfigure the receiver bandwidth or turn on/off one of the RF chains when performing measurements on one or two SCCs with deactivated SCell. This may cause interruptions on PCell or an activated SCell that are specified in Section 7.8.

###### --- End of Change 7 ----

###### --- Start of Change 8 ----

##### 8.12.3.4.2	E-UTRAN secondary component carrier measurements when common DRX is used

When DRX is in use the UE shall be able to identify a new detectable FS3 TP on a secondary component carrier within Tidentify_SCC_TP_FS3_DRX, according to the parameter measCycleSCell.

Tidentify_SCC_TP_FS3_DRX = Tidentify_SCC_FS3_DRX + Tmeasure_SCC_FS3_CSI-RS_DRX,

where:

Tidentify_SCC_FS3_DRX is the time period for cell identification in Section 8.12.2.4.2,

Tmeasure_SCC_FS3_CSI-RS_DRX is the time period for TP measurement in Table 8.12.3.4.2-1,

M is the number of configured discovery signal occasions during ON DURATION and which are not available during Tmeasure_SCC_FS3_CSI-RS_DRX for the measurements at the UE due to the absence of the necessary radio signals.

Table 8.12.3.4.2-1: Measurement requirements for a TP on SCC under operation with frame structure 3

| Measurement bandwidth [RB] | CSI-RS Ês/Iot | Discovery signal occasion duration (ds-OccasionDuration) [ms] | Tmeasure_SCC_FS3_CSI-RS_DRX, [ms] |
| --- | --- | --- | --- |
| 6 | [0] ≤ CSI-RS Ês/Iot | 1 | ([3]+M) * Max{ measCycleSCell, DRX cycle length } |
| 6 | [-6] ≤ CSI-RS Ês/Iot < [0] | 1 | ([5]+M) * Max{ measCycleSCell, DRX cycle length } |
| 25 | [0] ≤ CSI-RS Ês/Iot | 1 | ([1]+M) * Max{ measCycleSCell, DRX cycle length } |
| 25 | [-6] ≤ CSI-RS Ês/Iot < [0] | 1 | ([3]+M) * Max{ measCycleSCell, DRX cycle length } |

A cell shall be considered detectable when the following conditions are met during the discovery signal occasions which are available during Tidentify_SCC_TP_ FS3_DRX:

-	CSI-RSRP related side condition given in Section 9.1.19 are fulfilled for a corresponding Band,

-	SCH_RP|dBm is according to Annex B.2.12 for a corresponding Band and SCH Ês/Iot  is according to Table 8.12.2.4.2-1.

The UE shall be capable of performing CSI-RSRP measurements for 3 identified TPs on a secondary component carrier, and the UE physical layer shall be capable of reporting measurements to higher layers with the measurement period of Tmeasure_SCC_FS3_CSI-RS_DRX.

The measurement accuracy for all measured TPs shall be as specified in Section 9.1.19.

A UE may reconfigure receiver bandwidth taking into account the SCell activation/deactivation status, and when making measurements of TPs on one or two SCCs with deactivated SCell. This may cause interruptions (packet drops) to a PCell or an activated SCell. No interruptions while the On Duration timer is running shall be allowed when common DRX is used. The requirement considers only interruptions due to reconfiguration of the receiver bandwidth, and not due to other causes such as RF impairments or channel conditions.

###### --- End of Change 8 ----